3GPP TSG RAN WG1 #115																R1-23XXXX

Chicago, USA, November 13th – November 17th, 2023

Source:	Thales

Title:	TP on mobility for TR 37.911

Agenda Item:	8.15.2

Document for:	Agreement

# Introduction

This is a TP on mobility TPR for TR 37.911 “Study on self-evaluation toward s the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology” V0.0.1.

# Proposed changes

<Unchanged text is omitted>

# 4	Self-evaluation of eMBB-s technical performance

## 4.9	Mobility

As defined in in [2, Rep ITU-R  M.2514-0], mobility is the maximum device speed at which a defined QoS can be achieved (in km/h). The QoS is defined as normalized traffic channel link data rate.

Both configurations with frequency reuse factor equal to 1 (FRF1) and frequency reuse factor equal to 3 (FRF3) are considered for mobility evaluation of the Rural-eMBB-s test environment. Detailed evaluation assumptions and results can be found in Annex A.3.

The evaluation results of mobility for NR satellite access for both evaluation configuration with FRF1 and FRF3 are provided in Table 4.9.1.

It is observed that NR satellite access fulfils the mobility requirement under 250 km/h.

Table 4.9.1 Evaluation results of NR satellite access mobility under 250 km/h

| Frequency reuse factor | ITU Requirement |  |  | Number of samples |
| --- | --- | --- | --- | --- |
| FRF1 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.07 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.16%] 0.18% | 4 |
| FRF3 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.14 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.34%] 0.33% | 4 |

# Annex A:	Simulation models and assumptions

## A.3 	Evaluation assumption for mobility for NR satellite access

The detailed assumptions and results for mobility can be found in the attached document "A.3_Mobility.zip"

<End of change >