3GPP TSG RAN WG1 #115																R1-2310940

Chicago, USA, November 13th – November 17th, 2023

Source:	Thales

Title:	TP on reliability for TR 37.911

Agenda Item:	8.15.2

Document for:	Agreement

# Introduction

This is a TP on reliability for TR 37.911 “Study on self-evaluation toward s the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology” V0.0.1.

# Proposed changes

<Unchanged text is omitted>

# 6	Self-evaluation of HTC-s technical performance

## 6.1	Reliability

As defined in [2, Rep ITU-R  M.2514-0], reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

The evaluation is conducted in the Rural-HRC-s test environment, applicable to handheld devices. Both downlink and uplink are evaluated. Detailed assumptions and results are provided in Annex A.4.

## 6.1.1 DL reliability

For downlink reliability, both evaluation configuration with frequency reuse factor equal to 1 and evaluation configuration with frequency reuse factor equal to 3 are evaluated.

The evaluation results of NR FDD for downlink reliability are provided in Table 6.1.1.

- It is observed that NR satellite access fulfils the reliability requirement for downlink.

Table 6.1.1 Evaluation results of DL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | DL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.98% | 4 |
| FRF 3 | 99.9% | 99.96% | 5 |

## 6.1.1 UL reliability

For uplink reliability, both evaluation configuration with frequency reuse factor equal to 1 and evaluation configuration with frequency reuse factor equal to 3 are evaluated.

The evaluation results of NR FDD for uplink reliability are provided in Table 6.1.2.

- It is observed that NR satellite access fulfils the reliability requirement for uplink.

Table 6.1.2 Evaluation results of UL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | UL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.97% | 4 |
| FRF 3 | 99.9% | 99.97% | 5 |

# Annex A:	Simulation models and assumptions

## A.4 	Detailed assumptions and results for reliability for NR satellite access

The detailed assumptions and results for reliability can be found in the attached document "A.4_Reliability.zip"

<End of change >