3GPP TSG RAN WG1 #115		R1-2310941

Chicago , USA, November 13th – November 17th, 2023

Agenda Item:	8.15.2

Source:	Moderator (Thales)

Title:	Feature Lead Summary #1 on self-evaluation results for NR NTN

Document for:	Discussion

# Introduction

This feature lead summary document aims to collect and discuss the evaluation results for NR NTN and align companies views on the self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology. It contains a summary of the contributions under 8.15.2 at TSG-RAN WG1 #115 together with identified key issues and a set of proposed TPs for the TR 37.911.

A total of 12 TDocs have been submitted to current meeting for discussion. The source contributions are cited in references [1]-[12]: Please see the appendix I for the details, with all the proposals.

The self-evaluation results against technical performance requirements (TPR) for eMBB-s and HRC-s are discussed, including:

  - TPR#1: Peak spectral efficiency

  - TPR#2: Peak data rate

  - TPR#3: 5th percentile user spectral efficiency

  - TPR#4: Average spectral efficiency

  - TPR#5: User experienced data rate

  - TPR#6: Area traffic capacity

  - TPR#7: Mobility

  - TPR#8: Reliability

  - TPR#9: Connection density

  - TPR#10: Energy efficiency, including both network and device

The following files are attached to this feature lead summary:

- A.2_eMBB_SE_ UserExpDataRate_AreaTrafCap

- A.3_Mobility

- A.4_Reliability

- A.5 ConnectionDensity

- B.1_ Calibration

- Att  TP for TR37.911- Spectral efficiency_user data rate_ area traffic capacity

- Att  TP for TR37.911- Mobility

- Att  TP for TR37.911- Reliability

- Att  TP for TR37.911- Energy efficiency

# Calibration results

RAN1#113 made the following agreement:

Companies are encouraged to provide calibration curves aligned with TR 38.821 calibration case 9 or case 10 (depending on whether frequency reuse factor one or three is used) for system-level simulation. Then there is no need for additional cross-company calibration.

Further, RAN1#114bis made the following observations and agreements:

Observation:

The results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions reported within the attached template are aligned with TR 38.821 calibration case 9 and case 10.

Attachment:

R1-2308869 1 Att Calibration_results v06

Agreement

The results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions reported within the attached template will be captured in the TR 37.911.

Attachment:

R1-2308869 1 Att Calibration_results v06

## Companies’ contributions summary

Refer to appendix I

## First round

### Proposal  1-1

The attached template B.1_ Calibration is proposed for calibration results collection:

Proposal 1-1

The updated results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions reported within the attached template will be captured in the TR 37.911.

Attachment:

B.1_ Calibration

Companies are encouraged to provide comments on the above proposal and the proposed template:

| Companies | Comments |
| --- | --- |
| OPPO | We support the proposal and upload our calibration results to the attached template. |

# [CLOSED] TPR#1-2: Peak spectral efficiency and peak data rate

RAN1#114bis made the following agreement:

Agreement

Confirm the values of NR NTN peak spectral efficiency and Peak data rate reported in the pCR in RP-231946 for TR 37.911 with the following correction:

- Peak data rate (Mbit/s) in uplink should be 2.67 instead of 2.76

Thereby, there will be no further discussion on TPR#1-2 in current meeting.

# TPR#3-4: Spectral efficiency

## Companies’ contributions summary

On TPR#3-4, companies made the following observations and proposals:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 2: NR NTN with 4Rx and 0/2.2 dB scintillation loss can fulfil DL average spectral efficiency and 5th percentile spectral efficiency requirements, while NR NTN with 2Rx and 0/2.2 dB scintillation loss cannot fulfil DL average spectral efficiency and 5th percentile spectral efficiency requirements. Observation 3: NR NTN with FRF=3 and 0 dB scintillation loss can fulfil UL average spectral efficiency and 5th percentile spectral efficiency requirements. Observation 4: NR NTN with 4Rx UE antenna elements and 0/2.2 dB scintillation loss can fulfil DL user experienced data rate requirements, while NR NTN with 2Rx UE antenna elements and 0/2.2 dB scintillation loss cannot fulfil DL user experienced data rate requirements. Observation 5: NR NTN with FRF=3 and 0 dB scintillation loss can fulfil UL user experienced data rate requirements. Observation 6: NR NTN can fulfil DL and UL area traffic capacity requirements. Proposal 1: Capture the evaluation assumptions in Table 1 and evaluation results in Table 2/Table 3 in TR 37.911. Proposal 2: Capture the evaluation results in Table 4 in TR 37.911. Proposal 3: Capture the evaluation results in Table 5 in TR 37.911. |
| THALES | Observation 1 NR NTN can fulfill the 5th percentile user spectral efficiency requirements for both DL and UL Observation 2 NR NTN can fulfill the average spectral efficiency requirements for both DL and UL Observation 3 NR NTN can fulfill the user experienced data rate requirements for both DL and UL Observation 4 NR NTN can fulfill the area traffic capacity requirements for both DL and UL requirements |
| vivo | Observation 2: NR NTN with FRF=1 and 2Rx/4Rx can fulfil DL average spectral efficiency ITU requirement, but cannot fulfil DL 5th percentile user spectral efficiency ITU requirement. Observation 3: NR NTN with FRF=3 and 2Rx/4Rx can fulfil DL average spectral efficiency and 5th percentile user spectral efficiency ITU requirements. Observation 4: NR NTN with FRF=1 and FRF=3 can fulfil UL average spectral efficiency and 5th percentile user spectral efficiency ITU requirements. Observation 5: NR NTN with 4Rx and FRF=3 can fulfil DL user experienced data rate ITU requirement, while NR NTN with 2Rx or with 4Rx and FRF=1 cannot fulfil DL user experienced data rate ITU requirement. Observation 6: NR NTN with FRF=1 and with FRF=3 can fulfil UL user experienced data rate ITU requirement. Observation 7: NR NTN with 2Rx/4Rx and FRF=1/FRF=3 can fulfil DL area traffic capacity ITU requirement. Observation 8: NR NTN with FRF=1 and FRF=3 can fulfil UL area traffic capacity ITU requirement. |
| ZTE | Observation 1: All metrics meet the ITU requirements if 4 UE antennas are used Proposal 1: Capture the simulation results in Table 1~Table 4 into TR 37.911. |
| OPPO | Observation 1: For scintillation of 2.2dB, the average spectral efficiency and 5th percentile user spectral efficiency can meet the ITU-R requirements with little margin: - For DL, the average spectral efficiency is 0.56 bit/s/Hz for FRF1 and 0.5 bit/s/Hz for FRF3, and the 5th percentile user spectral efficiency is 0.043 bit/s/Hz for FRF1 and 0.043 bit/s/Hz for FRF3. - For UL, the average spectral efficiency is 0.17 bit/s/Hz for FRF1 and 0.23 bit/s/Hz for FRF3, and the 5th percentile user spectral efficiency is 0.0052 bit/s/Hz for FRF1 and 0.0067 bit/s/Hz for FRF3. Observation 2: For scintillation of 0dB, the average spectral efficiency and 5th percentile user spectral efficiency can meet the ITU-R requirements with little margin: - For DL, the average spectral efficiency is 0.57 bit/s/Hz for FRF1 and 0.53 bit/s/Hz for FRF3, and the 5th percentile user spectral efficiency is 0.044 bit/s/Hz for FRF1 and 0.046 bit/s/Hz for FRF3. - For UL, the average spectral efficiency is 0.25 bit/s/Hz for FRF1 and 0.27 bit/s/Hz for FRF3, and the 5th percentile user spectral efficiency is 0.0078 bit/s/Hz for FRF1 and 0.0113 bit/s/Hz for FRF3. Observation 3: For scintillation of 2.2dB, the requirement on user experience data rate can be satisfied, where the DL user experience data rate is 1.24 Mbit/s for FRF = 1 and 1.23 Mbit/s for FRF = 3, and the UL user experience data rate is 149.69 kbit/s for FRF = 1 and 193.22 kbit/s for FRF = 3. Observation 4: For scintillation of 0dB, the requirement on user experience data rate can be satisfied, where the DL user experience data rate is 1.28 Mbit/s for FRF = 1 and 1.32 Mbit/s for FRF = 3, and the UL user experience data rate is 225.81 kbit/s for FRF = 1 and 327.97 kbit/s for FRF = 3. Observation 5: For scintillation of 2.2dB, the area traffic capacity can meet ITU-R requirement, where the DL area traffic capacity is 11.31 kbit/s/km2 for FRF = 1 and 10.15 kbit/s/km2 for FRF = 3, and the UL area traffic capacity is 3.46 kbit/s/km2 for FRF = 1 and 4.6 kbit/s/km2 for FRF = 3. Observation 6: For scintillation of 0dB, the area traffic capacity can meet ITU-R requirement, where the DL area traffic capacity is 11.61 kbit/s/km2 for FRF = 1 and 10.84 kbit/s/km2 for FRF = 3, and the UL area traffic capacity is 4.99 kbit/s/km2 for FRF = 1 and 5.53 kbit/s/km2 for FRF = 3 |
| Panasonic | - 5%-tile user spectral efficiency, User experienced data rate, Average spectral efficiency and Area traffic capacity Downlink DL; 5%-tile user spectral efficiency (bps/Hz); User experienced data rate (Mbps); Average user spectral efficiency (bps/Hz); Area traffic capacity (kbps/km2). Requirement; 0.03; 1; 0.5; 8. FRF 1; 0.043; 1.28; 0.80; 16.96. FRF 3; 0.047; 1.41; 0.65; 13.78. *FRF: Frequency Reuse Factor Uplink UL; 5%-tile user spectral efficiency (bps/Hz); User experienced data rate (kbps); Average user spectral efficiency (bps/Hz); Area traffic capacity (kbps/km2). Requirement; 0.003; 100; 0.1; 1.5. FRF 1; 0.007; 209; 0.13; 2.76. FRF 3; 0.012; 360; 0.21; 4.45. |
| Xiaomi | Observation 1: At least for the FRF= 3 case, the ITU-R required user experienced data rate, 5th percentile user spectral efficiency, average spectral efficiency and area traffic capacity can be satisfied. |
| CCU, ITRI | Observation 1: For NTN Self-evaluation, the ITU requirement on average spectral efficiency can be satisfied. Observation 2: For NTN Self-evaluation, the ITU requirement on 5th percentile user spectral efficiency can be satisfied. Observation 3: For NTN Self-evaluation with 2.2 dB scintillation loss, the ITU requirement on DL user experienced data rate can be satisfied. Observation 4: For NTN Self-evaluation with 2.2 dB scintillation loss, the ITU requirement on UL user experienced data rate cannot be satisfied. Observation 5: For NTN Self-evaluation with 0 dB scintillation loss, the ITU requirement on user experienced data rate can be satisfied. Observation 6: For NTN Self-evaluation, the ITU requirement on area traffic capacity can be satisfied. |
| Qualcomm | Proposal 1 (eMBB-s): Capture the evaluation results in Table 1 in TR 37.911 Proposal 2 (eMBB-s): Capture the UL evaluation results in Table 2 in TR 37.911 Proposal 3 (eMBB-s): Capture the evaluation assumptions in Table 3 and evaluation results in Table 4 in TR 37.911. |
| Nokia, Nokia Shanghai Bell | Observation 1: Under the EFC scenario the system will experience better performance in terms of DL throughput compared to the EMC scenario. Observation 2: Under the EFC scenario the system will provide better performance in terms of DL area capacity compared to EMC scenario, while in contrast the opposite is the case when it comes to the UL area capacity. Observation 3: The main contributing factor for the relatively big difference in the achieved uplink mean SINR between EMC and EFC scenarios is the elevation angle span. Proposal 1: Both EMC and EFC performance results should be included in the evaluation excel sheets in [9] for more representative results in the final report. |

## Evaluation results summary

The following Table provides the evaluation results of TPR#3 and TPR#4 based on companies inputs submitted to RAN1#115:

Refer to: A.2_eMBB_SE_ UserExpDataRate_AreaTrafCap.

## First round

5th percentile user spectral efficiency is assessed jointly with average spectral efficiency using the same simulation

### Proposal  3-1

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil downlink spectral efficiency requirement

Table  Evaluation results of DL spectral efficiency for NR NTN

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | DL Spectral efficiency | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.500 | 0.573 | 8 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.047 | 8 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.500 | 0.536 | 7 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.037 | 7 |
|  | 4 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.500 | 0.746 | 2 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.036 | 2 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.500 | 0.608 | 4 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.040 | 4 |
| 0 dB | 2 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.500 | 0.617 | 3 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.033 | 3 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.500 | 0.554 | 3 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.039 | 3 |
|  | 4 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.500 | 0.813 | 1 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.044 | 1 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.500 | 0.531 | 1 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.048 | 1 |

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |
| OPPO | We support the proposal, but the number of samples needs to be updated in some rows. |

### Proposal  3-2

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil uplink spectral efficiency requirement

Table  Evaluation results of UL spectral efficiency for NR NTN

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | DL Spectral efficiency | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.100 | 0.136 | 7 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.003 | 0.006 | 7 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.100 | 0.187 | 6 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.003 | 0.008 | 6 |
| 0 dB | 2 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.100 | 0.225 | 2 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.003 | 0.008 | 2 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.100 | 0.223 | 3 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.003 | 0.008 | 3 |

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |
| OPPO | We support the proposal, but the number of samples needs to be updated in some rows. |

# TPR#5: User experienced data rate

The user experienced data rate should be derived from the 5th percentile user spectral efficiency through equation.

## Companies’ contributions summary

On TPR #5, companies made the following observations and proposals:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 4: NR NTN with 4Rx UE antenna elements and 0/2.2 dB scintillation loss can fulfil DL user experienced data rate requirements, while NR NTN with 2Rx UE antenna elements and 0/2.2 dB scintillation loss cannot fulfil DL user experienced data rate requirements. Observation 5: NR NTN with FRF=3 and 0 dB scintillation loss can fulfil UL user experienced data rate requirements. Proposal 2: Capture the evaluation results in Table 4 in TR 37.911. Proposal 3: Capture the evaluation results in Table 5 in TR 37.911. |
| THALES | Observation 3 NR NTN can fulfill the user experienced data rate requirements for both DL and UL |
| vivo | Observation 5: NR NTN with 4Rx and FRF=3 can fulfil DL user experienced data rate ITU requirement, while NR NTN with 2Rx or with 4Rx and FRF=1 cannot fulfil DL user experienced data rate ITU requirement. Observation 6: NR NTN with FRF=1 and with FRF=3 can fulfil UL user experienced data rate ITU requirement. |
| ZTE | Observation 1: All metrics meet the ITU requirements if 4 UE antennas are used Proposal 1: Capture the simulation results in Table 1~Table 4 into TR 37.911. |
| OPPO | Observation 3: For scintillation of 2.2dB, the requirement on user experience data rate can be satisfied, where the DL user experience data rate is 1.24 Mbit/s for FRF = 1 and 1.23 Mbit/s for FRF = 3, and the UL user experience data rate is 149.69 kbit/s for FRF = 1 and 193.22 kbit/s for FRF = 3. Observation 4: For scintillation of 0dB, the requirement on user experience data rate can be satisfied, where the DL user experience data rate is 1.28 Mbit/s for FRF = 1 and 1.32 Mbit/s for FRF = 3, and the UL user experience data rate is 225.81 kbit/s for FRF = 1 and 327.97 kbit/s for FRF = 3. |
| Panasonic | - 5%-tile user spectral efficiency, User experienced data rate, Average spectral efficiency and Area traffic capacity Downlink DL; 5%-tile user spectral efficiency (bps/Hz); User experienced data rate (Mbps); Average user spectral efficiency (bps/Hz); Area traffic capacity (kbps/km2). Requirement; 0.03; 1; 0.5; 8. FRF 1; 0.043; 1.28; 0.80; 16.96. FRF 3; 0.047; 1.41; 0.65; 13.78. *FRF: Frequency Reuse Factor Uplink UL; 5%-tile user spectral efficiency (bps/Hz); User experienced data rate (kbps); Average user spectral efficiency (bps/Hz); Area traffic capacity (kbps/km2). Requirement; 0.003; 100; 0.1; 1.5. FRF 1; 0.007; 209; 0.13; 2.76. FRF 3; 0.012; 360; 0.21; 4.45. |
| Xiaomi | Observation 1: At least for the FRF= 3 case, the ITU-R required user experienced data rate, 5th percentile user spectral efficiency, average spectral efficiency and area traffic capacity can be satisfied. |
| CCU, ITRI | Observation 3: For NTN Self-evaluation with 2.2 dB scintillation loss, the ITU requirement on DL user experienced data rate can be satisfied. Observation 4: For NTN Self-evaluation with 2.2 dB scintillation loss, the ITU requirement on UL user experienced data rate cannot be satisfied. Observation 5: For NTN Self-evaluation with 0 dB scintillation loss, the ITU requirement on user experienced data rate can be satisfied. |
| Qualcomm | Proposal 1 (eMBB-s): Capture the evaluation results in Table 1 in TR 37.911 Proposal 2 (eMBB-s): Capture the UL evaluation results in Table 2 in TR 37.911 Proposal 3 (eMBB-s): Capture the evaluation assumptions in Table 3 and evaluation results in Table 4 in TR 37.911. |
| Nokia, Nokia Shanghai Bell | Observation 1: Under the EFC scenario the system will experience better performance in terms of DL throughput compared to the EMC scenario. Observation 2: Under the EFC scenario the system will provide better performance in terms of DL area capacity compared to EMC scenario, while in contrast the opposite is the case when it comes to the UL area capacity. Observation 3: The main contributing factor for the relatively big difference in the achieved uplink mean SINR between EMC and EFC scenarios is the elevation angle span. Proposal 1: Both EMC and EFC performance results should be included in the evaluation excel sheets in [9] for more representative results in the final report. |

## Evaluation results summary

The following Table provides the preliminary evaluation results of TPR#5 based on companies inputs submitted to RAN1#115:

Refer to: A.2_eMBB_SE_ UserExpDataRate_AreaTrafCap.

## First round

### Proposal 4-1

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil DL user experienced data rate requirement

Table Evaluation results of DL user experienced data rate for NR NTN

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | DL user experienced data rate | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 3 | User experienced data rate (Mbit/s) | 1 | 1,08 | 6 |
|  | 4 | FRF = 1 | User experienced data rate (Mbit/s) | 1 | 1,07 | 1 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 1 | 1,21 | 3 |
| 0 dB | 2 | FRF = 3 | User experienced data rate (Mbit/s) | 1 | 1,13 | 2 |
|  | 4 | FRF = 1 | User experienced data rate (Mbit/s) | 1 | 1,33 | 1 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 1 | 1,45 | 1 |

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |
| OPPO | We support the proposal, but the number of samples needs to be updated in some rows. |
| Huawei/HiSilicon | Any particular reason why results for 2.2dB Scintillation loss 2 Rx FRF=1 are not captured in the table above though there are 6 sources? |

### Proposal 4-2

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil UL user experienced data rate requirement

Table Evaluation results of UL user experienced data rate for NR NTN

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | UL user experienced data rate | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | User experienced data rate (Mbit/s) | 0,1 | 0,25 | 5 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 0,1 | 0,23 | 5 |
| 0 dB | 2 | FRF = 1 | User experienced data rate (Mbit/s) | 0,1 | 0,16 | 1 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 0,1 | 0,22 | 2 |

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |
| OPPO | We support the proposal, but the number of samples needs to be updated in some rows. |

# TPR#6: Area traffic capacity

Area traffic capacity is derived assuming one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

## Companies’ contributions summary

On TPR#6, companies made the following observations and proposals:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 6: NR NTN can fulfil DL and UL area traffic capacity requirements. Proposal 3: Capture the evaluation results in Table 5 in TR 37.911. |
| THALES | Observation 4 NR NTN can fulfill the area traffic capacity requirements for both DL and UL requirements |
| vivo | Observation 7: NR NTN with 2Rx/4Rx and FRF=1/FRF=3 can fulfil DL area traffic capacity ITU requirement. Observation 8: NR NTN with FRF=1 and FRF=3 can fulfil UL area traffic capacity ITU requirement. |
| ZTE | Observation 1: All metrics meet the ITU requirements if 4 UE antennas are used Proposal 1: Capture the simulation results in Table 1~Table 4 into TR 37.911. |
| OPPO | Observation 5: For scintillation of 2.2dB, the area traffic capacity can meet ITU-R requirement, where the DL area traffic capacity is 11.31 kbit/s/km2 for FRF = 1 and 10.15 kbit/s/km2 for FRF = 3, and the UL area traffic capacity is 3.46 kbit/s/km2 for FRF = 1 and 4.6 kbit/s/km2 for FRF = 3. Observation 6: For scintillation of 0dB, the area traffic capacity can meet ITU-R requirement, where the DL area traffic capacity is 11.61 kbit/s/km2 for FRF = 1 and 10.84 kbit/s/km2 for FRF = 3, and the UL area traffic capacity is 4.99 kbit/s/km2 for FRF = 1 and 5.53 kbit/s/km2 for FRF = 3 |
| Panasonic | - 5%-tile user spectral efficiency, User experienced data rate, Average spectral efficiency and Area traffic capacity Downlink DL; 5%-tile user spectral efficiency (bps/Hz); User experienced data rate (Mbps); Average user spectral efficiency (bps/Hz); Area traffic capacity (kbps/km2). Requirement; 0.03; 1; 0.5; 8. FRF 1; 0.043; 1.28; 0.80; 16.96. FRF 3; 0.047; 1.41; 0.65; 13.78. *FRF: Frequency Reuse Factor Uplink UL; 5%-tile user spectral efficiency (bps/Hz); User experienced data rate (kbps); Average user spectral efficiency (bps/Hz); Area traffic capacity (kbps/km2). Requirement; 0.003; 100; 0.1; 1.5. FRF 1; 0.007; 209; 0.13; 2.76. FRF 3; 0.012; 360; 0.21; 4.45. |
| Xiaomi | Observation 1: At least for the FRF= 3 case, the ITU-R required user experienced data rate, 5th percentile user spectral efficiency, average spectral efficiency and area traffic capacity can be satisfied. |
| CCU, ITRI | Observation 6: For NTN Self-evaluation, the ITU requirement on area traffic capacity can be satisfied. |
| Qualcomm | Proposal 1 (eMBB-s): Capture the evaluation results in Table 1 in TR 37.911 Proposal 2 (eMBB-s): Capture the UL evaluation results in Table 2 in TR 37.911 Proposal 3 (eMBB-s): Capture the evaluation assumptions in Table 3 and evaluation results in Table 4 in TR 37.911. |
| Nokia, Nokia Shanghai Bell | Observation 1: Under the EFC scenario the system will experience better performance in terms of DL throughput compared to the EMC scenario. Observation 2: Under the EFC scenario the system will provide better performance in terms of DL area capacity compared to EMC scenario, while in contrast the opposite is the case when it comes to the UL area capacity. Observation 3: The main contributing factor for the relatively big difference in the achieved uplink mean SINR between EMC and EFC scenarios is the elevation angle span. Proposal 1: Both EMC and EFC performance results should be included in the evaluation excel sheets in [9] for more representative results in the final report. |

## Evaluation results summary

The following Table provides the preliminary evaluation results of TPR#6 based on companies inputs submitted to RAN1#115:

Refer to: A.2_eMBB_SE_ UserExpDataRate_AreaTrafCap.

## First round

### Proposal  5-1

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil DL area traffic capacity requirement

Table  Evaluation results of DL area traffic capacity for NR NTN

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | DL area traffic capacity | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 11,82 | 8 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 8 | 11,27 | 7 |
|  | 4 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 15,81 | 2 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 8 | 12,81 | 4 |
| 0 dB | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 12,95 | 3 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 8 | 11,64 | 3 |
|  | 4 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 17,23 | 1 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 8 | 11,26 | 1 |

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |
| OPPO | We support the proposal, but the number of samples needs to be updated in some rows. |

### Proposal  5-2

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil UL area traffic capacity requirement

Table  Evaluation results of UL area traffic capacity for NR NTN

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | UL area traffic capacity | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 1,5 | 2,96 | 8 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 1,5 | 3,98 | 5 |
| 0 dB | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 1,5 | 4,75 | 3 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 1,5 | 4,56 | 2 |

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |
| OPPO | We support the proposal, but the number of samples needs to be updated in some rows. |
| Huawei/HiSilicon | According to Huawei contribution R1-2310856, UL area traffic capacity 3.67 and 5.03 are for FRF=3 instead of FRF=1. The excel spreadsheet need to be updated |

# TPR#7: Mobility

RAN1#114bis made the following agreement:

Agreement

Capture the following observation and table in the TR 37.911:

It is observed that NR NTN fulfils the mobility requirement under 250 km/h.

Table Evaluation results of NR NTN mobility under 250 km/h

| Frequency reuse factor | ITU Requirement |  |  | Number of samples |
| --- | --- | --- | --- | --- |
| FRF1 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | [0.07] | [4] |
|  | Residual decoded packet error ratio | 1% | [0.16%] | [3] |
| FRF3 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | [0.14] | [4] |
|  | Residual decoded packet error ratio | 1% | [0.34%] | [3] |

## Companies’ contributions summary

On TPR#7, companies made the following observations and proposals:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 7: NR NTN can fulfil the requirements of mobility with 250km/h. |

## Evaluation results summary

The following Table provides the preliminary evaluation results of TPR#7 based on companies inputs submitted to RAN1#115:

Refer to: R1-230886x 5 Att eMBB-s_Mobility _v00

|  | FRF | Req. |  | Huawei | ZTE | CATT | Qualcomm | Ericsson | Mean | Var | # samples |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL Mobility (250 km/h) | FRF1 | Normalized traffic channel link data rate (bit/s/Hz) | 0,005 | 0,044 | 0,089 | 0,059 |  | 0,088 | 0,07 | 0,02 | 4 |
|  |  | Residual decoded packet error ratio | 1% | 0,47% | <0.1% | 0,010% |  | 0,06% | 0,18% | 0,00 | 3 |
|  | FRF3 | Normalized traffic channel link data rate (bit/s/Hz) | 0,005 | 0,044 | 0,089 | 0,234 | 0,203 |  | 0,14 | 0,09 | 4 |
|  |  | Residual decoded packet error ratio | 1% | 0,01% | <0.1% | 0,37% | 0,62% |  | 0,33% | 0,00 | 3 |

## First round

Based on the updated results submitted to RAN1#115, the following proposal is made:

### Proposal 6-1

Confirm the values of the evaluation results of NR satellite access mobility under 250 km/h agreed in RAN1#114bis with the following modification.

Table  Evaluation results of NR NTN mobility under 250 km/h

| Frequency reuse factor | ITU Requirement |  |  | Number of samples |
| --- | --- | --- | --- | --- |
| FRF1 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.07 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.16%] 0.18% | 3 |
| FRF3 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.14 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.34%] 0.33% | 3 |

- Companies are encouraged to provide views/comments within the following table:

# TPR#8: Reliability

RAN1#114bis made the following observations and agreements:

Agreement

Capture the following observation and table in the TR 37.911:

It is observed that NR NTN fulfils the reliability requirement for downlink.

- Table 1 Evaluation results of DL reliability for NR NTN

| Frequency Reuse Factor | ITU Requirement | DL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | [99.98%] | [4] |
| FRF 3 | 99.9% | [99.96%] | [5] |

Agreement

Capture the following observation and table in the TR 37.911:

It is observed that NR NTN fulfils the reliability requirement for uplink.

- Table Evaluation results of UL reliability for NR NTN

| Frequency Reuse Factor | ITU Requirement | UL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | [99.97%] | [4] |
| FRF 3 | 99.9% | [99.97%] | [5] |

## Companies’ contributions summary

On TPR#8, companies made the following observations and proposals:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 9: NR NTN can fulfil DL and UL reliability requirements. |
| ZTE | The maximum delay is assumed to be 34ms |
| Panasonic | - Reliability Downlink DL; 5%-tile SINR (dB); Repetition; HARQ; Success rate; Max delay. Requirement; 99.9 %; -. FRF 1; -3.17; 2; Enable (up to 8Tx); 99.98 %; 160 ms. FRF 3; 6.61; 8; Disable; 99.92 %; 12 ms. Uplink UL; 5%-tile SINR (dB); Repetition; HARQ; Success rate; Max delay. Requirement; 99.9 %; -. FRF 1; -4.02; 32; Enable (up to 8Tx); 99.95 %; 400 ms. FRF 3; 6.72; 8; Enable (up to 4Tx); 99.92 %; 92 ms. |
| Qualcomm | Proposal 4 (HRC-s): Capture the evaluation assumptions in Table 5 and evaluation results in Table 6 in TR 37.911. |

## Evaluation results summary

The following Table provides the evaluation results of TPR#8 based on companies inputs submitted to RAN1#115:

Refer to: A.4_Reliability

## First round

### Proposal  7-1

Confirm the following evaluation results of DL and UL reliability for NR satellite access:

Table 1 Evaluation results of DL reliability for NR NTN

| Frequency Reuse Factor | ITU Requirement | DL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.98% | 4 |
| FRF 3 | 99.9% | 99.96% | 5 |

Table Evaluation results of UL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | UL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.97% | 4 |
| FRF 3 | 99.9% | 99.97% | 5 |

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |

# Topic#9: Connection density

RAN1#114bis made the following observations and agreements:

Agreement

Capture the following observation and table in the TR 37.911:

It is observed that NR NTN fulfils connection density requirement.

- Table Evaluation results of connection density for NR NTN

| Traffic model | Frequency reuse factor | ITU Requirement | Connection density(/km2) | Bandwidth (kHz) | Number of samples |
| --- | --- | --- | --- | --- | --- |
| 1 message/day/device | FRF1 | 500 | [7334] | 180 | [4] |
|  | FRF3 | 500 | [28115] | 540 | [4] |
| 1 message/2 hours/device | FRF1 | 500 | [611] | 180 | [4] |
|  | FRF3 | 500 | [2340] | 540 | [4] |

## Companies’ contributions summary

On TPR#9. companies made the following observations and proposals:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Observation 8: NR NTN can fulfil the connection density requirements with full-buffer system-level simulation followed by link-level simulation method |
| Ericsson | Observation 1 NR NTN meets the IMT-2020 requirement for connection density of 500 devices per km2 as our system-level simulations indicate around 700 devices per km2 can be served. |

## Evaluation results summary

The following Table provides the evaluation results of TPR#9 based on companies inputs submitted to RAN1#115:

Refer to:

|  | RIT | Traffic model | FRF | Req. |  | Huawei | ZTE | Qualcomm | CATT | Mean | Var | # samples |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Connection density |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR NTN | 1 message/day/device | FRF1 | Connection density (/km2) | 500 | 6 273 | 6 125 | 8 966 | 731 | 5523,75 | 3451,69 | 4 |
|  |  |  |  | Bandwidth (kHz) |  | 180 | 180 | 180 | 180 |  |  |  |
|  |  |  | FRF3 | Connection density (/km2) | 500 | 20 331 | 32 556 | 27 972 | 4501 | 21340,00 | 12306,53 | 4 |
|  |  |  |  | Bandwidth (kHz) |  | 540 | 540 | 540 | 540 |  |  |  |
|  |  | 1 message/2 hours/device | FRF1 | Connection density (/km2) | 500 | 523 | 510 | 747 |  | 593,33 | 133,24 | 3 |
|  |  |  |  | Bandwidth (kHz) |  | 180 | 180 | 180 |  |  |  |  |
|  |  |  | FRF3 | Connection density (/km2) | 500 | 1 694 | 2 713 | 2 322 |  | 2243,00 | 514,07 | 3 |
|  |  |  |  | Bandwidth (kHz) |  | 540 | 540 | 540 |  |  |  |  |

## First round

Based on the updated evaluation results submitted to RAN1#115, the following proposal is made:

### Proposal 8-1

The evaluation results of connection density for NR satellite access are updated as follow:

Table  Evaluation results of connection density for NR satellite access

| Traffic model | Frequency reuse factor | ITU Requirement | Connection density(/km2) | Bandwidth (kHz) | Number of samples |
| --- | --- | --- | --- | --- | --- |
| 1 message/day/device | FRF1 | 500 | 5394 [7334] | 180 | 4 |
|  | FRF3 | 500 | 20582 [28115] | 180 | 4 |
| 1 message/2 hours/device | FRF1 | 500 | 720 [611] | 180 | 4 |
|  | FRF3 | 500 | 2277 [2340] | 180 | 4 |

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |

# TPR#10: Energy efficiency

RAN1#114bis made the following observations and agreements:

Agreement

The same energy efficiency aspects from the terrestrial self-evaluation in the report TR 37.910 apply for NTN as well on network side and device side. Thereby the text on the energy efficiency requirement from TR 37.910 can be reused for TR 37.911 with changes as necessary, considering only 15 kHz subcarrier spacing.

The TP on TPR#10 should be discussed and agreed in current meeting. Refer to section 11.6

# Text Proposals for TR 37.911

## [CLOSED]TP on TPR#1-2

RAN1#114bis made the following agreement:

Agreement

Confirm the values of NR NTN peak spectral efficiency and Peak data rate reported in the pCR in RP-231946 for TR 37.911 with the following correction:

- Peak data rate (Mbit/s) in uplink should be 2.67 instead of 2.76

Thereby, there will be no further discussion on TPR#1-2 in current meeting.

## TP on TPR#3-4-5-6

TP on spectral efficiency, user data rate and area traffic capacity for TR 37.911 is provided within the companion document:  Att  TP for TR37.911- Spectral efficiency_user data rate_ area traffic capacity.

### Proposal  10-1

Adopt the TP for clauses 4.3, 4.4, 4.5 and 4.6 of TR 37.911 provided within the attached document

Attachment:

Att  TP for TR37.911- Spectral efficiency_user data rate_ area traffic capacity

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |
| OPPO | We support the TP, but the number of samples needs to be updated in some rows. |

## TP on TPR#7

TP on mobility TPR for clause 4.9 of TR 37.911 is provided within the companion document:  TP for TR37.911- Mobility.

### Proposal  10-2

Adopt the TP for clause 4.9 of TR 37.911 on Mobility evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Mobility

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |

## TP on TPR#8

TP on Reliability TPR for clause 6.1 of TR 37.911 is provided within the companion document:  TP for TR37.911- Reliability.

### Proposal  10-3

Adopt the TP for clause 6.1 of TR 37.911 on Reliability evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Reliability

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |
| Huawei/HiSilicon | There are few typos: The evaluation results of NR FDD for downlink reliability are provided in Table 6.1.1 The evaluation results of NR FDD for downuplink reliability are provided in Table 6.1.2. |

## TP on TPR#9

The TP on connection density is discussed within the agenda item 8.15.3.

## TP on TPR#10

TP on Energy efficiency TPR for clause 4.8 of TR 37.911 is provided within the companion document:  TP for TR37.911- Energy efficiency.

### Proposal  10-4

Adopt the TP for clause 4.8 of TR 37.911 on Energy efficiency evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Energy efficiency

- Companies are encouraged to provide views/comments within the following table:

| Companies | Comments and Views |
| --- | --- |

## TP on calibration

TBC (during the meeting week)

# Proposals for offline on day 2

## Calibration results

Proposal 1-1

The updated results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions reported within the attached template will be captured in the TR 37.911.

Attachment:

B.1_ Calibration

## TPR#7: Mobility

### Proposal 6-1

Confirm the values of the evaluation results of NR satellite access mobility under 250 km/h agreed in RAN1#114bis with the following modification.

Table  Evaluation results of NR satellite access mobility under 250 km/h

| Frequency reuse factor | ITU Requirement |  | ? | Number of samples |
| --- | --- | --- | --- | --- |
| FRF1 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.07 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.16%] 0.18% | 4 |
| FRF3 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.14 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.34%] 0.33% | 4 |

### Proposal  10-2

Adopt the TP for clause 4.9 of TR 37.911 on Mobility evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Mobility

## TPR#8: Reliability

### Proposal  7-1

Confirm the following evaluation results of DL and UL reliability for NR satellite access:

Table 1 Evaluation results of DL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | DL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.98% | 4 |
| FRF 3 | 99.9% | 99.96% | 5 |

Table Evaluation results of UL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | UL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.97% | 4 |
| FRF 3 | 99.9% | 99.97% | 5 |

### Proposal  10-3

Adopt the TP for clause 6.1 of TR 37.911 on Reliability evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Reliability

## Topic#9: Connection density

### Proposal 8-1

The evaluation results of connection density for NR satellite access are updated as follow:

Table  Evaluation results of connection density for NR satellite access

| Traffic model | Frequency reuse factor | ITU Requirement | Connection density(/km2) | Bandwidth (kHz) | Number of samples |
| --- | --- | --- | --- | --- | --- |
| 1 message/day/device | FRF1 | 500 | 7205 [7334] | 180 | 4 |
|  | FRF3 | 500 | 27357 [28115] | 540 | 4 |
| 1 message/2 hours/device | FRF1 | 500 | 600 [611] | 180 | 4 |
|  | FRF3 | 500 | 2277 [2340] | 540 | 4 |

## TPR#10: Energy efficiency

### Proposal  10-4

Adopt the TP for clause 4.8 of TR 37.911 on Energy efficiency evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Energy efficiency

## TPR#3-4: Spectral efficiency

### Proposal  3-1

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil downlink spectral efficiency requirement

Table  Evaluation results of DL spectral efficiency for NR satellite access

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | DL Spectral efficiency | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.500 | 0.573 | 9 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.047 | 9 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.500 | 0.536 | 8 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.037 | 8 |
|  | 4 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.500 | 0.746 | 2 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.036 | 2 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.500 | 0.608 | 4 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.040 | 4 |
| 0 dB | 2 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.500 | 0.589 | 6 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.029 | 6 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.500 | 0.562 | 6 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.038 | 6 |
|  | 4 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.500 | 0.783 | 3 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.041 | 3 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.500 | 0.659 | 3 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.030 | 0.047 | 3 |

### Proposal  3-2

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil uplink spectral efficiency requirement

Table  Evaluation results of UL spectral efficiency for NR satellite access

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | DL Spectral efficiency | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.100 | 0.136 | 8 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.003 | 0.006 | 8 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.100 | 0.187 | 7 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.003 | 0.008 | 7 |
| 0 dB | 2 | FRF = 1 | Average [bit/s/Hz/TRxP] | 0.100 | 0.233 | 4 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.003 | 0.006 | 4 |
|  |  | FRF = 3 | Average [bit/s/Hz/TRxP] | 0.100 | 0.230 | 5 |
|  |  |  | 5th percentile [bit/s/Hz] | 0.003 | 0.009 | 5 |

## TPR#5: User experienced data rate

### Proposal 4-1

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil DL user experienced data rate requirement

Table Evaluation results of DL user experienced data rate for NR satellite access

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | DL user experienced data rate | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | User experienced data rate (Mbit/s) | 1 | 0.97 | 9 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 1 | 1.08 | 8 |
|  | 4 | FRF = 1 | User experienced data rate (Mbit/s) | 1 | 1.07 | 2 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 1 | 1.21 | 4 |
| 0 dB | 2 | FRF = 3 | User experienced data rate (Mbit/s) | 1 | 0.85 | 6 |
|  | 4 | FRF = 1 | User experienced data rate (Mbit/s) | 1 | 1.12 | 6 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 1 | 1.24 | 3 |

### Proposal 4-2

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil UL user experienced data rate requirement

Table Evaluation results of UL user experienced data rate for NR satellite access

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | UL user experienced data rate | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | User experienced data rate (Mbit/s) | 0.1 | 0.14 | 8 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 0.1 | 0.23 | 7 |
| 0 dB | 2 | FRF = 1 | User experienced data rate (Mbit/s) | 0.1 | 0.13 | 5 |
|  |  | FRF = 3 | User experienced data rate (Mbit/s) | 0.1 | 0.25 | 6 |

## TPR#6: Area traffic capacity

### Proposal  5-1

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil DL area traffic capacity requirement

Table  Evaluation results of DL area traffic capacity for NR satellite access

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | DL area traffic capacity | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 11.82 | 9 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 8 | 11.27 | 8 |
|  | 4 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 15.81 | 2 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 8 | 12.81 | 4 |
| 0 dB | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 12.41 | 6 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 8 | 11.85 | 6 |
|  | 4 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 8 | 16.60 | 3 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 8 | 13.97 | 3 |

### Proposal  5-2

Capture the following observation and table in the TR 37.911:

It is observed that NR satellite access can fulfil UL area traffic capacity requirement

Table  Evaluation results of UL area traffic capacity for NR satellite access

| Scintillation loss | Number of UE antennas | Frequency reuse factor | Req. |  | UL area traffic capacity | Number of samples |
| --- | --- | --- | --- | --- | --- | --- |
| 2.2 dB | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 1.5 | 2.87 | 8 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 1.5 | 3.93 | 7 |
| 0 dB | 2 | FRF = 1 | Area traffic capacity (kbit/s/km2) | 1.5 | 4.87 | 4 |
|  |  | FRF = 3 | Area traffic capacity (kbit/s/km2) | 1.5 | 4.84 | 5 |

## TP on TPR#3-4-5-6

TP on spectral efficiency, user data rate and area traffic capacity for TR 37.911 is provided within the companion document:  Att  TP for TR37.911- Spectral efficiency_user data rate_ area traffic capacity.

### Proposal  10-1

Adopt the TP for clauses 4.3, 4.4, 4.5 and 4.6 of TR 37.911 provided within the attached document

Attachment:

Att  TP for TR37.911- Spectral efficiency_user data rate_ area traffic capacity

# Proposals for online on day 2

Proposal 1-1

The updated results of CL, Geometry SIR and Geometry SINR simulated on DL and UL transmissions reported within the attached template will be captured in the TR 37.911.

Attachment:

B.1_ Calibration

Proposal 6-1

Confirm the values of the evaluation results of NR satellite access mobility under 250 km/h agreed in RAN1#114bis with the following modification.

Table  Evaluation results of NR satellite access mobility under 250 km/h

| Frequency reuse factor | ITU Requirement |  | ? | Number of samples |
| --- | --- | --- | --- | --- |
| FRF1 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.07 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.16%] 0.18% | 4 |
| FRF3 | Normalized traffic channel link data rate (bit/s/Hz) | 0.005 | 0.14 | 4 |
|  | Residual decoded packet error ratio | 1% | [0.34%] 0.33% | 4 |

Proposal  10-2

Adopt the TP for clause 4.9 of TR 37.911 on Mobility evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Mobility

Proposal  7-1

Confirm the following evaluation results of DL and UL reliability for NR satellite access:

Table 1 Evaluation results of DL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | DL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.98% | 4 |
| FRF 3 | 99.9% | 99.96% | 5 |

Table Evaluation results of UL reliability for NR satellite access

| Frequency Reuse Factor | ITU Requirement | UL Reliability | Number of samples |
| --- | --- | --- | --- |
| FRF 1 | 99.9% | 99.97% | 4 |
| FRF 3 | 99.9% | 99.97% | 5 |

Proposal  10-3

Adopt the TP for clause 6.1 of TR 37.911 on Reliability evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Reliability

Proposal 8-1

The evaluation results of connection density for NR satellite access are updated as follow:

Table  Evaluation results of connection density for NR satellite access

| Traffic model | Frequency reuse factor | ITU Requirement | Connection density(/km2) | Bandwidth (kHz) | Number of samples |
| --- | --- | --- | --- | --- | --- |
| 1 message/day/device | FRF1 | 500 | 7205 [7334] | 180 | 4 |
|  | FRF3 | 500 | 27357 [28115] | 540 | 4 |
| 1 message/2 hours/device | FRF1 | 500 | 600 [611] | 180 | 4 |
|  | FRF3 | 500 | 2277 [2340] | 540 | 4 |

Proposal  10-4

Adopt the TP for clause 4.8 of TR 37.911 on Energy efficiency evaluation in NR satellite access provided within the attached document

Attachment:

Att  TP for TR37.911- Energy efficiency

# Conclusion

TBC

# Appendix I: Summary of proposals

| TDoc | Source | Proposals and observations |
| --- | --- | --- |
| R1-2310856 | Huawei, HiSilicon | Observation 1: Calibration curves for FRF=1 and FRF=3 are aligned with TR 38.821 calibration case 9 and case 10, respectively. Observation 2: NR NTN with 4Rx and 0/2.2 dB scintillation loss can fulfil DL average spectral efficiency and 5th percentile spectral efficiency requirements, while NR NTN with 2Rx and 0/2.2 dB scintillation loss cannot fulfil DL average spectral efficiency and 5th percentile spectral efficiency requirements. Observation 3: NR NTN with FRF=3 and 0 dB scintillation loss can fulfil UL average spectral efficiency and 5th percentile spectral efficiency requirements. Observation 4: NR NTN with 4Rx UE antenna elements and 0/2.2 dB scintillation loss can fulfil DL user experienced data rate requirements, while NR NTN with 2Rx UE antenna elements and 0/2.2 dB scintillation loss cannot fulfil DL user experienced data rate requirements. Observation 5: NR NTN with FRF=3 and 0 dB scintillation loss can fulfil UL user experienced data rate requirements. Observation 6: NR NTN can fulfil DL and UL area traffic capacity requirements. Observation 7: NR NTN can fulfil the requirements of mobility with 250km/h. Observation 8: NR NTN can fulfil the connection density requirements with full-buffer system-level simulation followed by link-level simulation method. Observation 9: NR NTN can fulfil DL and UL reliability requirements. Proposal 1: Capture the evaluation assumptions in Table 1 and evaluation results in Table 2/Table 3 in TR 37.911. Proposal 2: Capture the evaluation results in Table 4 in TR 37.911. Proposal 3: Capture the evaluation results in Table 5 in TR 37.911. Proposal 4: Capture the evaluation assumptions in Table 6 and evaluation results in Table 7 in TR 37.911. Proposal 5: Capture the evaluation assumptions in Table 8 and evaluation results in Table 9 in TR 37.911. Proposal 6: Capture the evaluation assumptions in Table 10 and evaluation results in Table 11 in TR 37.911. |
| R1-2310940 | THALES | Observation 1 NR NTN can fulfill the 5th percentile user spectral efficiency requirements for both DL and UL Observation 2 NR NTN can fulfill the average spectral efficiency requirements for both DL and UL Observation 3 NR NTN can fulfill the user experienced data rate requirements for both DL and UL Observation 4 NR NTN can fulfill the area traffic capacity requirements for both DL and UL requirements Observation 5 NR NTN network can achieve high sleep ratio and long sleep duration in unloaded case Observation 6 NR NTN meets network side energy efficiency requirement Observation 7 NR NTN device can achieve very long sleep duration in both idle mode and connected mode Observation 8 NR NTN meets device side energy efficiency requirement |
| R1-2311116 | vivo | Observation 1: All of scenarios and channels for NR NTN can meet the coverage requirements. Observation 2: NR NTN with FRF=1 and 2Rx/4Rx can fulfil DL average spectral efficiency ITU requirement, but cannot fulfil DL 5th percentile user spectral efficiency ITU requirement. Observation 3: NR NTN with FRF=3 and 2Rx/4Rx can fulfil DL average spectral efficiency and 5th percentile user spectral efficiency ITU requirements. Observation 4: NR NTN with FRF=1 and FRF=3 can fulfil UL average spectral efficiency and 5th percentile user spectral efficiency ITU requirements. Observation 5: NR NTN with 4Rx and FRF=3 can fulfil DL user experienced data rate ITU requirement, while NR NTN with 2Rx or with 4Rx and FRF=1 cannot fulfil DL user experienced data rate ITU requirement. Observation 6: NR NTN with FRF=1 and with FRF=3 can fulfil UL user experienced data rate ITU requirement. Observation 7: NR NTN with 2Rx/4Rx and FRF=1/FRF=3 can fulfil DL area traffic capacity ITU requirement. Observation 8: NR NTN with FRF=1 and FRF=3 can fulfil UL area traffic capacity ITU requirement. Proposal 1: Capture the evaluation results in Table 4~5 and Table 7~12 into the TR37.911. |
| R1-2311205 | ZTE | Observation 1: All metrics meet the ITU requirements if 4 UE antennas are used. Proposal 1: Capture the simulation results in Table 1~Table 4 into TR 37.911. |
| R1-2311258 | OPPO | Observation 1: For scintillation of 2.2dB, the average spectral efficiency and 5th percentile user spectral efficiency can meet the ITU-R requirements with little margin: - For DL, the average spectral efficiency is 0.56 bit/s/Hz for FRF1 and 0.5 bit/s/Hz for FRF3, and the 5th percentile user spectral efficiency is 0.043 bit/s/Hz for FRF1 and 0.043 bit/s/Hz for FRF3. - For UL, the average spectral efficiency is 0.17 bit/s/Hz for FRF1 and 0.23 bit/s/Hz for FRF3, and the 5th percentile user spectral efficiency is 0.0052 bit/s/Hz for FRF1 and 0.0067 bit/s/Hz for FRF3. Observation 2: For scintillation of 0dB, the average spectral efficiency and 5th percentile user spectral efficiency can meet the ITU-R requirements with little margin: - For DL, the average spectral efficiency is 0.57 bit/s/Hz for FRF1 and 0.53 bit/s/Hz for FRF3, and the 5th percentile user spectral efficiency is 0.044 bit/s/Hz for FRF1 and 0.046 bit/s/Hz for FRF3. - For UL, the average spectral efficiency is 0.25 bit/s/Hz for FRF1 and 0.27 bit/s/Hz for FRF3, and the 5th percentile user spectral efficiency is 0.0078 bit/s/Hz for FRF1 and 0.0113 bit/s/Hz for FRF3. Observation 3: For scintillation of 2.2dB, the requirement on user experience data rate can be satisfied, where the DL user experience data rate is 1.24 Mbit/s for FRF = 1 and 1.23 Mbit/s for FRF = 3, and the UL user experience data rate is 149.69 kbit/s for FRF = 1 and 193.22 kbit/s for FRF = 3. Observation 4: For scintillation of 0dB, the requirement on user experience data rate can be satisfied, where the DL user experience data rate is 1.28 Mbit/s for FRF = 1 and 1.32 Mbit/s for FRF = 3, and the UL user experience data rate is 225.81 kbit/s for FRF = 1 and 327.97 kbit/s for FRF = 3. Observation 5: For scintillation of 2.2dB, the area traffic capacity can meet ITU-R requirement, where the DL area traffic capacity is 11.31 kbit/s/km2 for FRF = 1 and 10.15 kbit/s/km2 for FRF = 3, and the UL area traffic capacity is 3.46 kbit/s/km2 for FRF = 1 and 4.6 kbit/s/km2 for FRF = 3. Observation 6: For scintillation of 0dB, the area traffic capacity can meet ITU-R requirement, where the DL area traffic capacity is 11.61 kbit/s/km2 for FRF = 1 and 10.84 kbit/s/km2 for FRF = 3, and the UL area traffic capacity is 4.99 kbit/s/km2 for FRF = 1 and 5.53 kbit/s/km2 for FRF = 3. |
| R1-2311562 | Panasonic | The evaluation results are summarized below. - Peak spectral efficiency and Peak data rate Downlink DL; Peak spectral efficiency [bit/s/Hz]; Peak data rate (Mbps). Requirement; 3; 70. DL; 3.71; 111.3. Uplink UL; Peak spectral efficiency [bit/s/Hz]; Peak data rate (Mbps). Requirement; 1.5; 2. UL; 1.58; 2.27. - 5%-tile user spectral efficiency, User experienced data rate, Average spectral efficiency and Area traffic capacity Downlink DL; 5%-tile user spectral efficiency (bps/Hz); User experienced data rate (Mbps); Average user spectral efficiency (bps/Hz); Area traffic capacity (kbps/km2). Requirement; 0.03; 1; 0.5; 8. FRF 1; 0.043; 1.28; 0.80; 16.96. FRF 3; 0.047; 1.41; 0.65; 13.78. *FRF: Frequency Reuse Factor Uplink UL; 5%-tile user spectral efficiency (bps/Hz); User experienced data rate (kbps); Average user spectral efficiency (bps/Hz); Area traffic capacity (kbps/km2). Requirement; 0.003; 100; 0.1; 1.5. FRF 1; 0.007; 209; 0.13; 2.76. FRF 3; 0.012; 360; 0.21; 4.45. - Reliability Downlink DL; 5%-tile SINR (dB); Repetition; HARQ; Success rate; Max delay. Requirement; 99.9 %; -. FRF 1; -3.17; 2; Enable (up to 8Tx); 99.98 %; 160 ms. FRF 3; 6.61; 8; Disable; 99.92 %; 12 ms. Uplink UL; 5%-tile SINR (dB); Repetition; HARQ; Success rate; Max delay. Requirement; 99.9 %; -. FRF 1; -4.02; 32; Enable (up to 8Tx); 99.95 %; 400 ms. FRF 3; 6.72; 8; Enable (up to 4Tx); 99.92 %; 92 ms. |
| R1-2311566 | Beijing Xiaomi Mobile Software | Observation 1: At least for the FRF= 3 case, the ITU-R required user experienced data rate, 5th percentile user spectral efficiency, average spectral efficiency and area traffic capacity can be satisfied. |
| R1-2311641 | NTT DOCOMO, INC. | Observation 1: DL calibration results of case 9 and case 10 including coupling loss, geometry SIR and geometry SINR are aligned with other companies. Observation 2: UL calibration results of case 9 and case 10 including coupling loss, geometry SIR and geometry SINR are aligned with other companies. Observation 3: The SNR for peak spectral efficiency and peak data rate are 16.95dB for DL with 30MHz, 7.08 dB for UL with 1.44MHz, respectively. Observation 4: DL and UL peak spectral efficiency and peak data rate can fulfil Requirements. |
| R1-2311750 | CCU, ITRI | Observation 1: For NTN Self-evaluation, the ITU requirement on average spectral efficiency can be satisfied. Observation 2: For NTN Self-evaluation, the ITU requirement on 5th percentile user spectral efficiency can be satisfied. Observation 3: For NTN Self-evaluation with 2.2 dB scintillation loss, the ITU requirement on DL user experienced data rate can be satisfied. Observation 4: For NTN Self-evaluation with 2.2 dB scintillation loss, the ITU requirement on UL user experienced data rate cannot be satisfied. Observation 5: For NTN Self-evaluation with 0 dB scintillation loss, the ITU requirement on user experienced data rate can be satisfied. Observation 6: For NTN Self-evaluation, the ITU requirement on area traffic capacity can be satisfied. |
| R1-2312059 | Qualcomm Incorporated | Proposal 1 (eMBB-s): Capture the evaluation results in Table 1 in TR 37.911 Proposal 2 (eMBB-s): Capture the UL evaluation results in Table 2 in TR 37.911 Proposal 3 (eMBB-s): Capture the evaluation assumptions in Table 3 and evaluation results in Table 4 in TR 37.911. Proposal 4 (HRC-s): Capture the evaluation assumptions in Table 5 and evaluation results in Table 6 in TR 37.911. Proposal 5 (mMTC-s): Capture the evaluation assumptions in Table 7 and evaluation results in Table 8 in TR 37.911. Proposal 6 (Energy Efficiency): Endorse the text proposal on energy efficiency in Section 7. |
| R1-2312140 | Nokia, Nokia Shanghai Bell | Observation 1: Under the EFC scenario the system will experience better performance in terms of DL throughput compared to the EMC scenario. Observation 2: Under the EFC scenario the system will provide better performance in terms of DL area capacity compared to EMC scenario, while in contrast the opposite is the case when it comes to the UL area capacity. Observation 3: The main contributing factor for the relatively big difference in the achieved uplink mean SINR between EMC and EFC scenarios is the elevation angle span. Proposal 1: Both EMC and EFC performance results should be included in the evaluation excel sheets in [9] for more representative results in the final report. |
| R1-2312162 | Ericsson | Observation 1 NR NTN meets the IMT-2020 requirement for connection density of 500 devices per km2 as our system-level simulations indicate around 700 devices per km2 can be served. Proposal 1 The simulated results of CL, Geometry SIR and Geometry SINR reported within the attached template (R1- R1-2312162_Calibration_results_v00_Ericsson.xlsx) are aligned with TR 38.821 calibration case 9 and case 10. Proposal 2 NR NTN supports a maximum transmission bandwidth of 30 MHz, meeting the IMT-2020 requirement. |

# References

- R1-2310856	Self-evaluation results for NR NTN	Huawei, HiSilicon

- R1-2310940	Self-evaluation results for NR NTN	THALES

- R1-2311116	Discussions on self-evaluation results for NR NTN	vivo

- R1-2311205	Discussion on the Self-evaluation results for NR NTN	ZTE

- R1-2311258	Self-evaluation results for NR NTN	OPPO

- R1-2311562	Evaluation results of NR-NTN for IMT-2020 satellite radio interface technology	Panasonic

- R1-2311566	Self-evaluation results for NR NTN	Beijing Xiaomi Mobile Software

- R1-2311641	Self-evaluation results for NR NTN	NTT DOCOMO, INC.

- R1-2311750	Self-evaluation results for NR NTN	CCU, ITRI

- R1-2312059	Self-evaluation results for NR NTN	Qualcomm Incorporated

- R1-2312140	Comparison results related to IMT-2020 Satellite for NR over NTN for Earth Fixed Cell Vs. Earth Moving Cell	Nokia, Nokia Shanghai Bell

- R1-2312162	Satellite IMT-2020 self-evaluation results for NR NTN	Ericsson