- 3GPP TSG RAN WG1 #115                                                                                   R1-2311206

Chicago, USA, November 13th - November 17th, 2023

Source:	ZTE

Title:	Discussion on the Self-evaluation results for IoT NTN

Agenda Item:	8.15.3

Document for:    Discussion

- Introduction

In RAN1#114bis meeting, following agreement has been achieved for IoT NTN self-evaluation [1]. In this contribution, the simulation results for IoT-NTN self-evaluation are provided.

Agreement For eMTC evaluations, companies to report the TBS size used.

- Discussion on connection density evaluation

The SLS and LLS simulation parameters marked with “to be reported” are quoted below for quick reference.

Table 1 Simulation parameters to be reported

| Parameter | Value |
| --- | --- |
| Simulation bandwidth | 15kHz (Single tone) |
| TBS | 256 |
| Modulation order | BPSK-π/2, QPSK-π/4 |
| Number of Resource units | 2,3,4,5,6,8,10 |
| Number of repetition | 1,2,4 |

The connection density of NB-IoT are evaluated via the methodology (i.e., using full buffer system level simulation with link level simulation) as defined in Report ITU-R M.2412. The UL SINR distribution obtained by SLS for FRF=1 and FRF=3 is shown in Figure 1. In the simulation, UL power control is not modeled, i.e. all users with full transmission power.

(a)Without fasting fading                              (b) With fasting fading (in [2])

Figure 1 the Uplink SINR distribution of NB-IoT

The 99th percentile of the delay per user D and connection density are shown in Table 2 to Table 3, respectively. The traffic model of packet arrival rate of 1 message / 2 hours / device is used.

Table 2 99th percentile delay recorded in Step 3 under SINRi (s) without fasting fading

| Physical channel | Configuration |  |  | 99th percentile delay | Requirement |
| --- | --- | --- | --- | --- | --- |
|  | FRF | repetitions | Modulation order |  |  |
| NB-IoT | 1 | 1,2,4 | MCS sweeping with target BLER @10% | 0.2066s | <10s |
|  | 1 | 1 | BPSK-π/2 with fixed MCS | 0.1797s | <10s |
|  | 3 | 1 | QPSK-π/4, MCS sweeping with target BLER @10% | 0.0358s | <10s |

Table 3 Connection density recorded in Step 7 under SINRi (devices per km2) without fasting fading

| Physical channel | Configuration |  |  | Connection density | Requirement |
| --- | --- | --- | --- | --- | --- |
|  | FRF | repetitions | Modulation order |  |  |
| NB-IoT | 1 | 1,2,4 | MCS sweeping with target BLER @10% | 436 | >500 |
|  | 1 | 1 | BPSK-π/2 with fixed MCS | 558 | >500 |
|  | 3 | 1 | QPSK-π/4, MCS sweeping with target BLER @10% | 2250 | >500 |

For FRF=3, different number of RU is used with MCS sweeping to achieve the target BLER of 10%. And the density requirement can be fulfilled.

For FRF=1, 10RU with BPSK-π/2 is used to achieve satisfactory connection density, which does not take the target BLER of 10% restriction into account. MCS sweeping with different repetition number is also used in the simulation with the target BLER of 10% restriction. In this case, the connection density is slightly lower than the requirement due to more time domain resource used in repetition.

Based on the CDF of pre-processing SINR, the spectral efficiency is calculated by the following formula with the corresponding BLER by LLS. In the formula, 256 is the TBS in bit. N is the number of RU, and 1RU is 8ms*15kHz.

The SINR - SE mapping graphs for FRF=1 and FRF=3 are shown in Figure 2.

Figure 2 Spectral efficiency for NB-IoT

Observation 1: For FRF=3, the connection density requirement can be fulfilled with target BLER of 10%.

Observation 2: For FRF=1, the connection density requirement can be fulfilled without target BLER of 10% restriction.

Proposal 1: Capture the evaluation results in Table 2 ~ Table 3 into the TR 37.911.

- Conclusions

In this contribution, the evaluation including analytical study and simulations are presented with following proposals:

Observation 1: For FRF=3, the connection density requirement can be fulfilled with target BLER of 10%.

Observation 2: For FRF=1, the connection density requirement can be fulfilled without target BLER of 10% restriction.

Proposal 1: Capture the evaluation results in Table 2 ~ Table 3 into the TR 37.911.

Reference

- RAN1 Chair's notes, RAN1#114bis.

- R1-2309155 Discussion on the Self-evaluation results for IoT NTN