3GPP TSG RAN WG1 #115		R1-2311233

Chicago, USA, November 13th – 17th, 2023

Source:	OPPO

Title:	Discussion on LS on L1 measurements for LTM

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

RAN2 sent Reply LS [1] to ask one question on L1 measurement for LTM.

- Discussion

In Reply LS [1], RAN2 explain they made the following agreements:

1. For the model of CSI report configuration, RAN2 to implement Option2 (i.e., introduce ltm-CSI-ReportConfig and not reuse CSI-ReportConfig) as in current RRC running CR. 2. For the model of RS configuration, RAN2 to follow what is indicated by RAN1 in the parameter list. 3. The LTM CSI resource configuration is generated by the CU. 4. The list of LTM CSI resource configuration is common for all the LTM candidate cells (as in current RRC running CR). 5. RAN2 assumes that network can include the field spCellInclusion only if the SpCell is an LTM candidate cell.

And RAN2 then asked the following question related with L1 measurement:

Question: Does the UE need to know the SMTC of LTM candidate cells in order to perform L1 measurements (so that it needs to be included in the RS configuration)?

In our view, the SMTC for L1 measurement of LTM candidate cells shall be provided. The UE only needs to measure the SSBs of LTM candidate cells within the SMTC. That is useful to reduce the system complexity for supporting LTM L1  measurement. Furthermore, RAN1 previous agreements has include the SMTC as part of the configuration information of candidate cell SSB, for example the following RAN1 agreement:

Agreement - For L1-RSRP measurement RS configuration - For SSB based L1-RSRP measurement: - As a starting point, at least the following information needs to be provided to a UE, e.g. - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst) - For inter-frequency: frequency domain location (e.g. center frequency), SCS - FFS: transmission power (for pathloss calculation) - Note: other parameters included in the configuration can be further discussed - Including above agreement into the LS

Proposal 1: Answer RAN2’s question as follows:

- The SMTC shall be included in the configuration of SSB of LTM candidate cells for L1 measurement.

- Conclusions

In this contribution, we present our views on LTM candidate cells L1 measurement and the following proposal is made:

Proposal 1: Answer RAN2’s question as follows:

- The SMTC shall be included in the configuration of SSB of LTM candidate cells for L1 measurement.

- Reference

R1-2310809 LS on L1 measurements for LTM