3GPP TSG RAN WG1 #115		                                                       R1-2311277

Chicago, USA, November 13th – November 17th, 2023

Agenda item:	8.16.3

Source: 	OPPO

Title: 	Discussion on UE features for expanded and improved NR positioning

Document for:	Discussion and decision

- Introduction

This contribution includes our views on RAN1 UE features list for Rel-18 positioning on top of the one agreed in the last meeting[1].

- UE feature list for SL positioning

Regarding FG 41-1-1, component 5 and corresponding discritption in Note column should be removed, as in SL positioning UE must support slot level buffering to receiver SCI. In component 4 we prefer to remove “and preparing the positioning measurement report” as this operation is not necessary for UE based positioning, and it can be regarded as part of SL-PRS resource processing if needed. We prefer this FG to be reported per band.

| 41. NR_pos_enh2 | 41-1-1 | Common SL PRS Processing Capability in a SL BWP | 1. Maximum SL PRS bandwidth in MHz in a resource pool for positioning, which is supported and reported by UE for SL-PRS measurement 2. Maximum number of active SL PRS resources across all configured RPs assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE 3. Maximum number of slots with active SL PRS resources across all configured RPs assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE 4. Minimum time after the end of a slot carrying the active SL-PRS resource(s) assuming maximum number of symbols and maximum bandwidth for a UE finish the SL-PRS resource processing [and preparing the positioning measurement report] [assuming the active SL-PRS resources during this time haven’t exceeded the reported capabilities] which is supported and reported by UE] [5. SL PRS buffering capability] |  | Yes | No | The UE does not support the reception and processing of SL PRS | [Per FS/Per Band/Per FCPC] | n/a | n/a | n/a | Component 1 candidate values: FR1 bands: {5, 10, 20, 40, 50, 80, 100} FR2 bands: {50, 100, 200, 400} Component 2 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16[, 24, 32, 48, 64, 128]} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64, 128} for each SCS: 60kHz, 120kHz Component 3 candidate values: FFS Component 4 candidate values: {[30ms, 40ms, 50ms, 100ms]} [Component 5 candidate values: {Type 1 – sub-slot/symbol level buffering, Type 2 – slot level buffering}] Note: a SL PRS resource is considered as active starting at the end of the last symbol of the PSCCH carrying the SCI trigger and the occupancy is released at the end of timeline indicated in component 4 Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 41-1-2, FG 15-4 (Synchronization sources for NR sidelink) should be a prerequisite. “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” should be Yes, such that the transmitter can select resource pool for SL PRS transmission accordingly. The yellow highlighted part in the Note column can be adopted.

| 41. NR_pos_enh2 | 41-1-2 | Receiving SL-PRS in a shared resource pool | 1. Support SL-PRS in shared resource pool 2. Support receiving SCI format 2D | [15-1, 15-4, 41-1-1] | Yes | [No] Yes | Receiving SL-PRS in a shared resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported [UE indicating support of FG 41-1-1 must indicate either this feature group or feature group 41-1-3 is supported or both are supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Similar comments as that for FG 41-1-2.

| 41. NR_pos_enh2 | 41-1-3 | Receiving SL-PRS in a dedicated resource pool | 1. Support SL-PRS in dedicated resource pool 2. Support receiving SCI format 1B | [15-1, 15-4 41-1-1] | Yes | [No] Yes | Receiving SL-PRS in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported [UE support of FG 41-1-1 must indicate either this feature group or feature group 41-1-2 is supported or both are supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

We propose to define separate FG for scheme 1 and scheme 2, the components and prerequisites for Scheme 1 and Scheme 2 are different, and a UE may not support both.  DL pathloss based power control for PSSCH and SL PRS are different.

| 41. NR_pos_enh2 | 41-1-4a-1 | Transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool | 1. Support of transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool 2. Support transmitting SCI format 2D [3. Support downlink pathloss based open loop power control for SL PRS] 4. UE can monitor DCI format 3_0 for dynamic scheduling and configured grant type 2 for SL PRS on the same carrier as sidelink. | [15-2, 41-1-2] | Yes | FFS | Transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool is not supported | [Per band] | n/a | n/a | n/a | [Need for location server to know if the feature is supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 41. NR_pos_enh2 | 41-1-4a-2 | Transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool | 1. Support of transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool 2. Support transmitting SCI format 2D [3. Support downlink pathloss based open loop power control for SL PRS] | [15-2, 41-1-2] 15-3 | Yes | FFS | Transmitting SL-PRS [scheme 1 and scheme 2] in a shared resource pool is not supported | [Per band] | n/a | n/a | n/a | [Need for location server to know if the feature is supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Transmitting SL-PRS scheme 1 does not rely on the support of 15-2 or 41-1-3, there is no common components between this feature group and 15-2, and an anchor UE in TDOA positioning can support SL PRS transmission but not support SL PRS reception. But 15-4 has to be a prerequisite.

| 41. NR_pos_enh2 | 41-1-4b | Transmitting SL-PRS scheme 1 in a dedicated resource pool | 1. UE can transmit SL-PRS and PSCCH within a slot without PSSCH in dedicated resource pool 2. UE can transmit SL-PRS according to the mapping rule between PSCCH and SL-PRS 3. Support transmitting SCI format 1B 4. Support receiving DCI format [3_x2] [5. Support open loop power control of SL-PRS] | [15-2, 41-1-3] 15-4 | Yes | FFS | Transmitting SL-PRS scheme 1 in a dedicated resource pool is not supported | [Per band] | n/a | n/a | n/a | [Need for location server to know if the feature is supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

A UE supports mode 2 should support 41-1-3 for sensing.

| 41. NR_pos_enh2 | 41-1-4c | Transmitting SL-PRS mode 2 in a dedicated resource pool | 1. UE can transmit SL-PRS and PSCCH within a slot without PSSCH in dedicated resource pool 2. UE can transmit SL-PRS according to the mapping rule between PSCCH and SL-PRS 3. Support transmitting SCI format 1B 4. UE support mode 2 sensing-based resource selection in dedicated SL PRS resource pool. | [15-[x], 41-1-3], at least one of {41-1-8, 41-1-10} | Yes | FFS | Transmitting SL-PRS mode 2 in a dedicated resource pool is not supported | [Per band] | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

At least one of 41-1-4b or 41-1-4c should be a prerequisite of the feature group 41-1-5.

| 41. NR_pos_enh2 | 41-1-5 | SL-PRS congestion control in a dedicated resource pool | 1) UE can report CBR measurement to gNB when operating in Scheme 1 and Scheme 2 [2) UE can adjust its radio parameters based on CBR measurement and CRlimit.] 3) UE can process CBR and CR within the time it indicates | FFS at least one of 41-1-4b and 41-1-4c | Yes | No | SL-PRS congestion control in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | [Component-3 candidate value set {Congestion process time 1, Congestion process time 2} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing] Note: component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

UE can only transmit one SL PRS in a slot, TDMed multiplexing from UEs does not impact SL PRS transmission of an individual UE. This should be a basic feature to avoid resource fragmentation.

| 41. NR_pos_enh2 | 41-1-11 | TDM-based multiplexing of SL-PRS [transmission/reception] from different UEs in the same slot in dedicated resource pool | Support of TDM-based multiplexing of SL-PRS [transmission/reception] from different UEs in the same slot in dedicated resource pool | FFS | No | FFS | TDM-based multiplexing of SL-PRS [transmission/reception] from different UEs in the same slot is not supported in dedicated resource pool | [Per band] | n/a | n/a | n/a | [Need for location server to know if the feature is supported] | Optional with capability signaling. This is the basic FG for NR sidelink positioning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Similarly, comb-based multiplexing for SL PRS should also be a basic FG.

| 41. NR_pos_enh2 | 41-1-12 | Comb-based multiplexing for SL-PRS reception from different UEs in the same slot in dedicated resource pool | Support of comb-based multiplexing for SL-PRS reception from different UEs in the same slot in dedicated resource pool | FFS | No | FFS | Comb-based multiplexing for SL-PRS reception from different UEs in the same slot is not supported in dedicated resource pool | [Per band] | n/a | n/a | n/a | [Need for location server to know if the feature is supported] | Optional with capability signaling. This is the basic FG for NR sidelink positioning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Regarding the FFS in FG 41-1-17, as a UE needs to support SL communication to set up unicast link, for such a UE it should be able to support RSRP report.

| 41. NR_pos_enh2 | 41-1-17 | Open loop SL pathloss based power control and SL RSRP report [for dedicated resource pool] | FFS: merge with 41-1-4 for SL PC and 41-1-3 for RSRP report Support of open loop SL pathloss based power control and SL RSRP report [for dedicated resource pool] for unicast transmissions | FFS 15-1 and at least one of 15-2 or 15-3 | Yes | FFS | Open loop SL power control and SL RSRP report [for dedicated resource pool] is not supported for unicast transmissions FFS: whether/how to address UEs that only support SL PRS reception but not sidelink RSRP reporting | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- UE features list for NR DL and UL carrier phase positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs. | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 |  |  |  |  |  |  |  |  |
|  | 41-2-1b | DL PRS processing capability for DL RSCP in RRC_CONNECTED | 1.Maximum number of DL PRS resources within a give time duration for support of DL reference carrier phase difference measurement 2.(N,T) maximum number of DL PRS resource for DL refernce carrier phase difference measurement within given time duration. |  |  |  |  |  |
|  | 41-2-1c | DL PRS processing capability for DL RSCPD in RRC_CONNECTED | 1.Maximum number of DL PRS resources within a give time duration for support of DL reference carrier phase measurement 2.(N,T) maximum number of DL PRS resource for DL reference carrier phase measurement within given time duration. |  |  |  |  |  |
|  | 41-2-2b | DL PRS processing capability for DL RSCP in RRC_INACTIVE | 1.Maximum number of DL PRS resources within a give time duration for support of DL reference carrier phase difference measurement 2.(N,T) maximum number of DL PRS resource for DL refernce carrier phase difference measurement within given time duration. |  |  |  |  |  |
|  | 41-2-2c | DL PRS processing capability for DL RSCPD in RRC_INACTIVE | 1.Maximum number of DL PRS resources within a give time duration for support of DL reference carrier phase measurement 2.(N,T) maximum number of DL PRS resource for DL reference carrier phase measurement within given time duration. |  |  |  |  |  |
|  | 41-2-4a | DL PRS processing capability for UE-based Carrier Phase Positioning |  |  |  |  |  |  |

- UE features list for LPHAP (Low Power High Accuracy Positioning)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs. | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-3-1 | SRS for positioning pre-configuration for UEs in RRC_INACTIVE state in validity region | 1. Support of SRS for positioning transmission in RRC_INACTIVE state for multiple cells 2. Maximum number of pre-configured cells for SRS for positioning in RRC_INACTIVE state |  | Yes | No | 1) Per UE |  |
|  | 41-3-2 | UE autonomous UL timing determinaton for SRS for positioning in RRC_INACTIVE | 1. Support of UE autonomous UL timing determinaton for SRS for positioning in RRC_INACTIVE | Y-1 | Yes | No | 1) Per UE |  |
|  | 41-3-3 | Spatial relation information across multiple cells for SRS for positioning in RRC_INACTIVE | 1. Support of spatial relation information for multiple cells for SRS for positioning in RRC_INACTIVE 2. Maximum number of candidate spatial relation information Values = [1, 2, 4, 8] | Y-1 | Yes | No | 2) Per Band |  |
|  | 41-3-4 | PL RS across multiple cells for SRS for positioning in RRC_INACTIVE | 1. Support of PL RS for multiple cells for SRS for positioning in RRC_INACTIVE 2. Maximum number of candidate PL RS Values = [1, 2, 4, 8] | Y-1 | Yes | No | 2) Per Band |  |

- UE features list for Bandwidth aggregation for positioning measurements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs. | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y. NR_pos_enh2 |  |  |  |  |  |  |  |  |
|  | 41-4-7 | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED | [1. The maximum number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE] [3. Support of periodic/semi-persistent/aperiodic UL SRS for UL bandwidth aggregation] 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of [periodic/semi-persistent/aperiodic] aggregated SRS resources for bandwidth aggregation 7. Maximum number of [periodic/semi-persistent/aperiodic] aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers [9. Guard period] |  |  |  |  |  |
|  | 41-4-7a | Support semi-persistent aggregated SRS resources for bandwidth aggregation |  |  |  |  |  |  |
|  | 41-4-7b | Support aperiodic aggregated SRS resources for bandwidth aggregation |  |  |  |  |  |  |
|  | 41-4-8 | Positioning SRS bandwidth aggregation in RRC_INACTIVE | [1. The maximum number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE] [3. Support of periodic/semi-persistent UL SRS for UL bandwidth aggregation] 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of [periodic/semi-persistent] aggregated SRS resources for bandwidth aggregation 7. Maximum number of [periodic/semi-persistent] aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers |  |  |  |  |  |
|  | 41-4-7c | Support same power prioritization for aggregated SRS resources for positioning | 1.Support of applying same power prioritization for SRS resources for positioning for bandwidth aggregation |  |  |  |  | If not supported, legacy power prioritization rule is applied. |

- UE features list for Positioning for RedCap UEs

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs. | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-5-1 | PRS with Rx frequency hopping for RedCap UEs | 1. Support of RedCap UEs Rx frequency hopping for PRS reception within a MG 2. Maximum of PRS bandwidth (in MHz) per hop FR1: {5, 10, 20} FR2: {50, 100} 3. Switch times for PRS between consecutive hops FR1: {70us, 140us} FR2: {35us, 70us, 140us} 4. Number of PRB(s) overlapped between hops |  | Yes | No | 2) Per Band |  |
|  | 41-5-2 | SRS for positioning frequency hopping for RedCap UEs | 1. Support of RedCap UEs Tx frequency hopping for SRS for positioning in RRC_CONNECTED/RRC_INACTIVE 2. Maximum of SRS for positioning bandwidth (in MHz) per hop FR1: {5, 10, 20} FR2: {50, 100} 3. Switch times for SRS for positioning between consecutive hops FR1: {70us, 140us} FR2: {35us, 70us, 140us} 4. Number of PRB(s) overlapped between hops |  | Yes | No | 2) Per Band |  |

- References

- R1-2310382, Session Notes of AI 8.16.3;