3GPP TSG RAN WG1 #115			                              R1-2311489

Chicago, USA, November 13th – November 17th, 2023

Source: 	CMCC

Title:	Remaining issues on XR-specific capacity enhancements

Agenda item:	8.6.1

Document for:	Discussion & Decision

# Introduction

The last RAN1#114bis meeting has well addressed some remaining issues on XR-specific capacity enhancements for Rel-18 Work Item (WI) “XR Enhancement for NR” [1], which includes the following two objectives from RAN1’s perspective:

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

- Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE

In detail, some TPs were approved to capture the description of an invalid CG PUSCH transmission occasion (TO), to clarify the applicability condition of joint encoding of UTO-UCI and HARQ-ACK, to include joint encoding of UTO-UCI and HARQ-ACK with polar codes when applicable, and so on. Based on these TPs, three draft change requests (CR) on specifications TS 38.212, 38.213, and 38.214 were endorsed to improve the support of XR enhancements for NR from the specifications’ perspective.

In the last meeting, it was concluded in the UE feature session that UTO-UCI is applicable to both of multi-PUSCHs CG and legacy CGs [2]. For multi-PUSCHs CG, there is a clear conclusion that neither PUSCH repetition nor TB processing over multiple slots (TBoMS) is supported in Rel-18. However, when the UTO-UCI is applied in legacy CGs, it should be discussed whether the mapping rule between UTO-UCI bitmap and CG PUSCH TOs needs to be reconsidered because PUSCH repetition and TBoMS can be supported in this case.

So, in this contribution, we provide our views on the above topics and propose to change Clause 9.3.1 “UE procedure for reporting UTO-UCI” in TS 38.213 to clarify the mapping rule between a UTO-UCI bitmap and CG PUSCH TOs on top of the changes captured in the draft CR R1-2310740 endorsed after the last meeting.

# Discussion

## 2.1 UTO-UCI mapping rule in legacy CGs

It was concluded in the UE feature session of the last RAN1 meeting that UTO-UCI is also applicable to legacy CGs. However, when repetition of the transmitted transport block (TB) or TB processing over multiple slots (TBoMS) is supported for a legacy CG configuration, we may need to consider new designs on the mapping rule between UTO-UCI bitmap and CG PUSCH TOs. For example, when the high layer parameter nrof_UTO_UCI is configured for a legacy CG configuration, if the parameter repK is configured or numberOfRepetitions is present in the indexed row in the TDRA table and the number of repetitions is K, then the following two options can be considered:

- Option 1: The CG PUSCH TO corresponding to one repetition of a TB is mapped to 1 bit in the bitmap of UTO-UCI.

- Option 2: The CG PUSCH TOs corresponding to K repetitions of a TB are mapped to 1 bit in the bitmap of UTO-UCI.

Actually, Option 1 is consistent with what is captured in the latest version of specification, that is, a bit in the bitmap of UTO-UCI corresponds to a valid CG PUSCH TO and each repetition of a TB is transmitted in a valid CG PUSCH TO. However, if a CG PUSCH TO is used to transmit a TB, the following K-1 valid CG PUSCH TOs may be used to transmit the repetitions of the transmitted TB, as illustrated in Figure 1, where the number of repetitions is configured as repK=2 and the CG periodicity equals to the duration of 4 slots. Therefore, from our understanding, it is unnecessary for a UE to report that the K CG PUSCH TOs corresponding to the K repetitions of a TB are “NOT unused” through K bits, especially when the number of repetitions is configured as 4 or even 8.

For Option 2, the K CG PUSCH TOs corresponding to K repetitions of a TB are regarded as a bundle of CG PUSCH TOs, which is only mapped to 1 bit in the bitmap of UTO-UCI, as shown in Figure 2. It should be noted that if UE transmits the first repetition of a TB and UTO-UCI in a CG PUSCH TO, then UE does not need to indicate the CG PUSCH TOs corresponding to the remaining K-1 repetitions of this TB are “unused” or “NOT unused” because they may be used. Compared to Option 1, Option 2 can indicate more CG PUSCH TOs are “unused” or not through the bitmap in UTO-UCI. Considering that the maximum length of a UTO-UCI bitmap is only 8, it appears that Option 2 provides a more meaningful approach to the use of the UTO-UCI, especially when the number of repetitions is configured as 4 or even 8.

Figure 1. Illustration of Option 1 regarding the mapping rule between a UTO-UCI bitmap and CG PUSCH TOs

Figure 2. Illustration of Option 2 regarding the mapping rule between a UTO-UCI bitmap and CG PUSCH TOs

Besides, there is another issue that we need to consider, that is, whether to transmit UTO-UCI in all PUSCH repetitions or only in the first PUSCH repetition. Figure 2 provides an example for the former case, i.e., transmitting UTO-UCI in all PUSCH repetitions. The following Figure 3 gives an example of the latter case, transmitting UTO-UCI only in the first PUSCH repetition. Obviously, the latter case can reduce the signaling overhead of UE.

Figure 3. UTO-UCI is only transmitted in the CG PUSCH TOs corresponding to the first repetition of a transmitted TB

Similar to PUSCH repetition, if TBoMS is supported for a legacy CG configuration and the number of slots allocated for TBoMS is N, the CG PUSCH TOs corresponding to N consecutive slots allocated for TBoMS can be mapped to 1 bit in the bitmap of UTO-UCI.

Proposal 1. Regarding the mapping rule between a UTO-UCI bitmap and CG PUSCH TOs when repetition of a transmitted TB and UTO-UCI are supported in a legacy CG configuration, support that the CG PUSCH TOs corresponding to K repetitions of a TB are mapped to 1 bit in the bitmap of UTO-UCI.

Proposal 2. If repetition of a transmitted TB and UTO-UCI are supported in a legacy CG configuration, only transmit UTO-UCI in the CG PUSCH TOs corresponding to the first repetition of a transmitted TB.

Proposal 3. Regarding the mapping rule between a UTO-UCI bitmap and CG PUSCH TOs when TBoMS and UTO-UCI are supported in a legacy CG configuration, support that the CG PUSCH TOs corresponding to N consecutive slots allocated for TBoMS are mapped to 1 bit in the bitmap of UTO-UCI.

## 2.2 Text proposal to clause 9.3.1 of TS 38.213

Based on the analysis in the previous section, clause 9.3.1 of TS 38.213 should be modified to clarify the mapping rule between a UTO-UCI bitmap and CG PUSCH TOs when the high layer parameter nrof_UTO_UCI is configured for a legacy CG configuration.

Proposal 4. Adopt the following text proposal to Clause 9.3.1 of TS 38.213.

## *** Unchanged text is omitted *** 9.3.1 UE procedure for reporting UTO-UCI If the UE is provided nrof_UTO_UCI with value equal to in configuredGrantConfig of a CG-PUSCH configuration, the UE multiplexes UTO-UCI represented by a bitmap of bits in each CG-PUSCH transmission for the CG-PUSCH configuration. The bits of UTO-UCI, , have a one-to-one mapping to subsequent CG-PUSCH TOs in ascending order of start time. For unpaired spectrum operation, the subsequent CG-PUSCH TOs exclude invalid ones where a UE does not transmit a PUSCH due to collision of the PUSCH with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or with symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst, based on the procedures in Clause 11.1. A bit value of ‘0’ indicates that the UE may transmit CG-PUSCH, and a bit value of ‘1’ indicates that the UE will not transmit CG-PUSCH, in a corresponding CG-PUSCH TO. When the UE indicates by UTO-UCI a value of ‘1’ for a CG-PUSCH TO, the UE continues to indicate the value of ‘1’ for the CG-PUSCH TO by UTO-UCI multiplexed in subsequent CG-PUSCH transmissions, and the UE does not transmit CG-PUSCH in the CG-PUSCH TO. If the UE is also provided repK with a value equal to K in configuredGrantConfig of the CG-PUSCH configuration or numberOfRepetitions with a value equal to K is present in the indexed row in the time domain resource allocation table, follow the above mapping procedure by treating the CG-PUSCH TOs corresponding to K repetitions of a TB as a bundle of CG-PUSCH TOs, which is mapped to one bit of UTO-UCI. If numberOfSlotsTBoMS with a value equal to N is present in the indexed row in the time domain resource allocation table, follow the above mapping procedure by treating the CG-PUSCH TOs corresponding to N consecutive slots allocated for TBoMS as a bundle of CG-PUSCH TOs, which is mapped to one bit in the bitmap of UTO-UCI. ## *** Unchanged text is omitted ***

# Conclusions

In this contribution, considering that the PUSCH repetition and TBoMS can be supported for legacy CG configurations, we propose to reconsider the mapping rule between UTO-UCI bitmap and CG PUSCH TOs and change Clause 9.3.1 “UE procedure for reporting UTO-UCI” of TS 38.213. The following proposals are made.

Proposal 1. Regarding the mapping rule between a UTO-UCI bitmap and CG PUSCH TOs when repetition of a transmitted TB and UTO-UCI are supported in a legacy CG configuration, support that the CG PUSCH TOs corresponding to K repetitions of a TB are mapped to 1 bit in the bitmap of UTO-UCI.

Proposal 2. If repetition of a transmitted TB and UTO-UCI are supported in a legacy CG configuration, only transmit UTO-UCI in the CG PUSCH TOs corresponding to the first repetition of a transmitted TB.

Proposal 3. Regarding the mapping rule between a UTO-UCI bitmap and CG PUSCH TOs when TBoMS and UTO-UCI are supported in a legacy CG configuration, support that the CG PUSCH TOs corresponding to N consecutive slots allocated for TBoMS are mapped to 1 bit in the bitmap of UTO-UCI.

Proposal 4. Adopt the following text proposal to Clause 9.3.1 of TS 38.213.

## *** Unchanged text is omitted *** 9.3.1 UE procedure for reporting UTO-UCI If the UE is provided nrof_UTO_UCI with value equal to in configuredGrantConfig of a CG-PUSCH configuration, the UE multiplexes UTO-UCI represented by a bitmap of bits in each CG-PUSCH transmission for the CG-PUSCH configuration. The bits of UTO-UCI, , have a one-to-one mapping to subsequent CG-PUSCH TOs in ascending order of start time. For unpaired spectrum operation, the subsequent CG-PUSCH TOs exclude invalid ones where a UE does not transmit a PUSCH due to collision of the PUSCH with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or with symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst, based on the procedures in Clause 11.1. A bit value of ‘0’ indicates that the UE may transmit CG-PUSCH, and a bit value of ‘1’ indicates that the UE will not transmit CG-PUSCH, in a corresponding CG-PUSCH TO. When the UE indicates by UTO-UCI a value of ‘1’ for a CG-PUSCH TO, the UE continues to indicate the value of ‘1’ for the CG-PUSCH TO by UTO-UCI multiplexed in subsequent CG-PUSCH transmissions, and the UE does not transmit CG-PUSCH in the CG-PUSCH TO. If the UE is also provided repK with a value equal to K in configuredGrantConfig of the CG-PUSCH configuration or numberOfRepetitions with a value equal to K is present in the indexed row in the time domain resource allocation table, follow the above mapping procedure by treating the CG-PUSCH TOs corresponding to K repetitions of a TB as a bundle of CG-PUSCH TOs, which is mapped to one bit of UTO-UCI. If numberOfSlotsTBoMS with a value equal to N is present in the indexed row in the time domain resource allocation table, follow the above mapping procedure by treating the CG-PUSCH TOs corresponding to N consecutive slots allocated for TBoMS as a bundle of CG-PUSCH TOs, which is mapped to one bit in the bitmap of UTO-UCI. ## *** Unchanged text is omitted ***

# References

- Chairman’s notes, RAN1#114bis, October 9th - October 13th, 2023.

- R1-2310618, Summary on UE features for XR enhancements, NTT DOCOMO.