3GPP TSG RAN WG1 #115			R1-2311577

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.16.5

Source:	Google

Title:	UE feature on NR MIMO evolution

Document for:	Discussion/Decision

# Introduction

In RAN1 #114 and RAN1 #114b, most of the UE FGs for network energy saving have been agreed. In this contribution, we provided some discussion on the remaining issues for each UE FG.

# Joint spatial domain and power domain adaptation

Currently joint spatial domain and power domain adaptation are supported. Therefore, it is necessary to consider a new FG for joint spatial domain and power domain adaptation, where the component can be similar to FG 42-1.

Proposal 3: Introduce the following UE FG for joint spatial domain and power domain adaptation.

| 42. Netw_Energy_NR | 42-3 | Joint spatial domain and power domain adaptation with CSI feedback based on CSI report sub-configuration(s) | 1. The max number of sub-configurations L in one CSI report configuration 2. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration. 3. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 4. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 5. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 6. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7. Supported configuration for each sub-configuration |  | Yes |  | UE does not support joint spatial domain and power domain adaptation | Per band | No | No | N/A | Component 1 candidate value {2,3,4} Component 2 candidate value(s): {1,2,3,4} Component 3 candidate value(s): {1,2,3,4} Component 4 candidate value(s): {8, 16, 24, 32} Component 5 candidate value(s): {8, 16, 24, 32} Component 6 candidate value: {1,2,3,4} Component 7 candidate value: {port subset, list of CSI-RS IDs, both} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Cell DTX/DRX

For cell DTX/DRX, in RAN1 #114b, it is agreed that this is applicable only for sTRP operation as follows:

Agreement Cell DTX/DRX operation is only supported for sTRP.

Therefore, the FG 42-4 and 42-5 should be clarified that it is for cell DTX/DRX for sTRP operation. If the mTRP is supported in future, there will be a separate FG, and the FG 42-4/5 can be the pre-requisite FGs. In addition, there is one FFS on whether to introduce the supported number of cell DTX/DRX patters per cell group, which be benefitial for UE power saving quite a lot. Therefore, it is necessary to introduce this component in 42-4.

Proposal 4: Support the following modification for FG 42-4/5.

| 42. Netw_Energy_NR | 42-4 | Cell DTX or DRX operation based on RRC configuration [with one DTX/DRX configuration per cell] for single-TRP operation | 1. Support of cell DTX/DRX operation [with one DTX/DRX configuration per cell] by RRC configuration 2. supported maximum number of cell DTX/DRX patterns per cell group |  | Yes |  | UE does not support Cell DTX/DRX operation [with one DTX/DRX configuration per cell] | FFS | No | No | N/A | Candidate values for component 1: {cell DTX only, cell DRX only, both} Candidate values for component 2: {1, 2, 3, 4} Note: RAN2 may add additional details | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. Netw_Energy_NR | 42-5 | Cell DTX/DRX operation triggered by DCI format 2_9x for single-TRP operation | 1) Support of Cell DTX/DRX configuration activation and deactivation via DCI 2_9x | 42-4 | Yes |  | UE does not support dynamic Cell DTX/DRX operation triggered by L1 signalling DCI format 2_9 | FFS | No | No | N/A |  | Optional with capability signaling |