# 3GPP TSG RAN WG1 #115							R1-231xxxx

# Chicago, USA, November 13th – November 17th, 2023

Agenda item:	6

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Discussion on R1-2312308

Document for:	Discussion and Decision

# Issue #1: Applicability of UE-specific K-offset during random access in connected mode

<Unchanged parts are omitted>

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers , otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

If the UE is configured with higher layer parameter k-Offset, for a PUSCH (re)transmission associated with the TC-RNTI, Koffset= k-Offset.

The following change is proposed in several sections of TS 36.213 to address the issue of applicability of UE-specific kOffset during random access in connected mode:

During the online discussion, companies had some comments on the following points:

- Whether we should use “temporary C-RNTI” or “scheduled by RAR”.

- Whether the change was needed at all (which hopefully was resolved during the session).

Companies are encouraged to provide feedback on the draft CR above.

| Company | Comment |
| --- | --- |
| Ericsson | We are ok with the current wording of the draft CR above. |
| Qualcomm | Based on some offline discussion, we would like to provide the spec information that indeed the (N)PUSCH associated with MSG3 RAR during CBRA is using TC-RNTI (36.213, NB-IoT) Note that the TC-RNTI is only set by higher layers if the UE initiates CBRA (36.321): |