3GPP TSG RAN WG1 #116			R1-2400904

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.6.1

Source:	Panasonic

Title:	Discussion on the LP-WUS and LP-SS design

Document for:	Discussion/Decision

# Introduction

In RAN #102 meeting, the new work item of LP-WUS/WUR (low-power wake-up signal and receiver) for NR was approved. The objective of the WI [RP-234056] includes the design of the LP-WUS, LP-SS and the procedures in RRC IDLE/INACTIVE and CONNCTED modes. The detailed scope of the objectives can be found in the appendix.

In this contribution, we share our views on the waveform design of the LP-WUS and LP-SS (low-power synchronization signal).

# Discussion

## Waveform design

As described in the WID,

- LP-WUS based on one or both OOK-1/4 waveform with overlaid OFDM sequences will be specified for both idle/inactive and connected modes. Whether to choose one of or both OOK-1 and OOK-4 needs to be discussed and decided.

- LP-SS based on one or both OOK-1/4 waveform will be specified for idle/inactive mode. Whether to choose one of or both OOK-1 and OOK-4 and whether to support overlaid OFDM sequence needs to be discussed and decided.

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

(a) OOK-1: M bits using 2M OFDM symbols with Manchester code

(b) OOK-4: M bits using 2M OOK-4 segments within an OFDM symbol

Figure.1 Illustration of the time domain comparison of the OOK-1 and OOK-4

As can be seen in Figure.1, when the OFDM symbol lengths are matched such than OOK-4 OFDM symbol length is X times of that of OOK-1, where X is the number of the segments within an OOK-4 OFDM symbol, in terms of the theoretical data rate per OFDM symbol, OOK-1 and OOK-4 can be same. In Figure.1, X = 2M, where Manchester code is assumed and thus M bit payload is assumed. That also means that the SCS of OOK-1 would be X times of that of OOK-4. For example, for OOK-1 with 30kHz SCS, 2 symbols may carry 2 bits, while for OOK-4 with 15kHz SCS, 1 symbol may also carry 2 bits using 2 OOK-4 segments with the same time duration. It is noted that, within an OFDM symbol of OOK-4, the number of and length of each OOK-4 segments could be more flexibly chosen to carry more variable number of bits, unlike OOK-1 that one OFDM symbol always carries one bit.

Therefore, depending on the parameter design, OOK-4 would achieve better flexibility. However, to achieve such merit, the LP-WUR would also need more comprehensive design to adapt more option of symbol rate and more variable length of segments, each of which is mapped with overlaid sequence. Such additional complexity and tradeoff with flexibility merit should be discussed and carefully checked whether necessary for LP-WUR and LP-WUS design. Thus, in the below table, some observations of the comparison of OOK-1 and OOK-4 waveform is provided.

Observation 1: The pros/cons of OOK-1 and OOK4 waveform are summarized in Table.1

Table.1 Pros/Cons of OOK-1 and OOK-4.

| Comparison of OOK-1 and OOK-4 | OOK-1 | OOK-4 |
| --- | --- | --- |
| Pros | - Less impact to the gNB waveform generation - Simpler design of overlaid sequence, whose length is decided by the bandwidth and SCS of LP-WUS | - More flexibility on the payload size thus more adaptive to more functionalities and better forward compatibility. - More flexible symbol rate means potentially better LP-WUS resource efficiency. - More adaptive to design for idle/inactive and connected modes, respecitively under the unified framework. - Single bit rate design of LP-WUR irrespective of NR SCS would be possible, which simplifies LP-WUR. |
| Cons | - Less flexibility on support the payload size. Only OFDM symbol number is varying factor. - In order to address the support of multiple NR SCS, LP-WUR is also required to support multiple bit rate, which is more complex LP-WUR design. - Forward compatibility of more payload bits relies on extended number of OFDM symbols. - Less adaptive to the required payload size and functionality means less LP-WUS resource efficiency. | - More impact and complexity to the gNB waveform generation, e.g., pre-storing of the generated frequency domain samples - More standardization efforts on defining the overlaid sequence, whose length is decided by the number of segements within a symbol, bandwidth and SCS of LP-WUS |

For more efficient standardization efforts and product implementation, LP-WUS should be designed and commonly applied for both RRC CONNECTED and IDLE/INACTIVE mode UEs with limited options of numerology and channel structure. A fixed symbol rate of LP-WUS should be supported regardless of NR side SCS in order that UE is not required to support variable bandwidth filter. For example, 30 kHz symbol rate of LP-WUS means OOK-4, M=2 in 15 kHz NR SCS and OOK-1 in 30 kHz NR SCS. This means the same symbol rate for LP-WUS can be seen as OOK-1 or OOK-4 depending on the NR SCS.

Proposal 1: LP-WUS should be designed and commonly applied for both RRC CONNECTED and IDLE/INACTIVE mode UEs with fixed symbol rate, irrespective of the SCS used by NR waveform.

Moreover, for easier commercialization, LP-SS waveform should always follow the LP-WUS. Even if more than one options of waveform of LP-WUS and LP-SS would be supported and defined eventually, in the supported configurations, LP-SS and LP-WUS should follow the same waveform and numerology.

Proposal 2: Regardless which one or both waveforms would be supported for LP-WUS and LP-SS, LP-SS and LP-WUS should always follow the same waveform and numerology in the applicable configuration.

## Evaluations

- In this section, we provide LLS evaluation results to check the performance of OOK-1 and OOK-4 waveform for both LP-WUS and LP-SS in below aspects:

- Impact of timing error

- Impact of carrier frequency error

- Impact of different number of OFDM symbols in a block.

- Symbol timing estimation performance by LP-SS

### Simulation assumptions

The general evaluation assumptions are shown in Table.2 and some specific assumptions are listed as follows:

- OOK-1 and OOK-4 based signal with overlaid OFDM sequence over OOK symbol are assumed.

- In OOK-4, DFT-spreading is employed for transformation in time domain.

- At transmitter side, the raw bits are encoded by Manchester code with coding rate of 1/2, and ZC sequence-based modulated signal is transmitted when coded bit is ‘1’ at each segment.

- At receiver side, received bits are detected from Manchester coded signal by using envelope detection for both OOK-1 and OOK-4 in time domain. Envelope detection is performed in symbol-level for OOK-1, and time-segment-level for OOK-4. In the process, blind boundary estimation is not performed in either time domain or frequency domain for LP-WUS evaluation.

- Regarding SCS, different SCSs are used in OOK-1 and OOK-4, respectively for assuming same amount of time-domain resource usage and focusing on fair comparison with same bitrate, such as OOK-1 with 60kHz SCS and OOK-4, M=2 with 30kHz SCS.

- For the evaluation on impact of timing error and carrier frequency error shown in Section 2.2.2 and 2.2.3, number of OFDM symbols for one block is 14.. For the evaluation with different number of symbols in Section 2.2.4, the number of symbols varies from 4 to 56. In the case with more than 14 symbols, multiple slots are used for the transmission of  block. For the symbol timing estimation performance by LP-SS in section 2.2.5, 2 symbols are used for OOK-1 and OOK-4, M=1 and 1 symbol is used for OOK-4, M=2.

- To model interference from in-band emission, PDSCH is mapped on the whole system bandwidth other than resource used by LP-WUS and GB. PDSCH power is modelled as constant.

- Sampling rate at Tx and channel are modelled by 30.72 MHz sampling rate, and at Rx, operation after BPF performs 7.68 MHz sampling rate. Further lower rate is not used at Rx to focus on the investigation of only time / frequency impairments or difference on number of symbols without any impact from down sampling, such as loss of energy and aliasing.

Table. 2 Simulation Assumptions

| Carrier Frequency | 2.6 GHz |
| --- | --- |
| Channel structure | Sequence modulation (ZC sequence-based signal) - information bit length: 7*M bits |
| Waveform | OOK-1, OOK-4 with overlaid OFDM sequence - Manchester coding is applied with R=1/2 |
| SCS of OFDM generator for NR signal | same as LP-WUS signal |
| Configuration for LP-WUS signal | For OOK-1: M=1 and SCSs = 30/60/120 kHz For OOK-4: M=1,2,4 and SCSs = 15/30 kHz |
| LP-WUS/LP-SS duration | Number of OFDM symbols: - For Section 2.2.2 and 2.2.3, 14 symbols in 1 block - For Section 2.2.4, Described in the legend - For Section 2.2.5, 2 symbols for OOK-1, OOK-4, M=1, 1 symbol for OOK-4, M=2 in 1 block |
| gNB Channel BW | 20 MHz |
| LP-WUS BW | 5.04 MHz (14 PRBs for 30 kHz SCS*) for LP-WUS transmission including both inner and outer GB - LP-WUS transmission: 12 PRBs including inner GB (6 SCs) for 30 kHz SCS - Outer GB between LP-WUS and adjacent PDSCH: 15 SCs for 30 kHz SCS * For cases other than 30 kHz SCS, number of PRBs/SCs is adjusted to be same bandwidth for LP-WUS and inner/outer GB in the case with 30 kHz SCS. |
| Filter | 5-th Order Butterworth filter with Y MHz bandwidth - Y is same as BW for 1 segment of LP-WUS including GB |
| Adjacent subcarrier interference | PDSCH mapped on resources other than that for WUS and outer GB in Channel BW; EPRE of PDSCH is same with that of LP-WUS=ρ, where ρ=0 dB |
| Sampling Rate | - Tx: 30.72 MHz - Rx: 7.68 MHz |
| ADC | Ideal |
| Channel Model | TDL-C - Delay spread: 300 ns - UE velocity: 3 km/h |
| Time / Frequency impairments | - Residual time offset (TO): 0,1,2,4,10 (us) - Residual frequency error offset (CFO): 0,50,100 (ppm) |
| Detection at receiver | Envelope detection in time domain |

### The impact of timing error

The impact of constant timing error on OOK-1 and OOK-4 is evaluated. The result is shown in Figure.2 and each graph represents 14, 28, and 56 kbps bitrates. We can observe the following points:

- In Figure.2 (a), as expected, OOK-4, M=1 has same performance as OOK-1, as segment allocation of OOK-4, M=1 is exactly the same as OOK-1 in time/frequency domain.

- In both OOK-1 and OOK-4 under 30 kHz SCS, 1% BLER SNR degradation compared with no timing error is negligible for a timing error up to 4 us.means

- In 60 kHz SCS and 120 kHz SCS shown in Figure. 2 (b) and (c) respectively, both OOK-1 and OOK-4 are more sensitive to timing error, and 1% BLER SNR degradation compared with no timing error can be within 2 dB for timing error up to 2 us in 28 kbps and 1 us in 56 kbps. This is because shorter OFDM symbol / time segment length decrease robustness to timing error.

- In the case of 56 kbps, OOK-4 has slightly better performance than OOK-1 up to TO=2. OOK-1 is higher SCS and thus shorter CP length becomes more sensitive to inter-symbol interference.

- Observation 2: Both OOK-1 and OOK-4 become more sensitive to timing error with higher bitrate due to shorter symbol / time segment length.

- Observation 3: In the case of higher bitrate such as 56 kbps, OOK-4 has slightly better performance than OOK-1 with wider SCS, as OOK-1 with shorter CP is more sensitive to inter-symbol interference.

(a) OOK-1 with 30kHz SCS vs OOK-4, M=1, 30kHz SCS	(b) OOK-1 with 60kHz SCS vs OOK-4, M=2, 30kHz SCS

(Bitrate: 14 kbps)						(Bitrate: 28 kbps)

(c) OOK-1 with 120kHz SCS vs OOK-4, M=4, 30kHz SCS

(Bitrate: 56 kbps)

Figure. 2 The LLS performance for timing error impact investigation

### The impact of carrier frequency error

The impact of constant carrier frequency error on OOK-1 and OOK-4 is shown in Figure.3. Same as timing error impact investigation, 3 results with different bitrates are shown. We can observe the following points:

- In all cases, both OOK-1 and OOK-4 of a carrier frequency error up to 100 ppm can be within 2 dB degradation of 1% BLER SNR compared with no timing error. Both waveforms use the same bandwidth, which is sufficiently wide to handle the amount of frequency offset. We also see GB between LP-WUS and interfering PDSCH contributes to the robustness against the carrier frequency error.

- In the case of 56 kbps shown in Figure.3 (c), slight performance gap between OOK-1 and OOK-4 is observed and its amount  is same as the result of no timing difference in Figure.2 (c). This would be caused by the ISI and shorter CP length in OOK-1.

- Observation 4: With carrier frequency offset, there is no significant performance difference between OOK-1 and OOK-4 and both are robust to even large carrier frequency error. But the robustness also relies on the bandwidth of LP-WUS and GB.

(a) OOK-1 with 30kHz SCS vs OOK-4, M=1, 30kHz SCS	(b) OOK-1 with 60kHz SCS vs OOK-4, M=2, 30kHz SCS

(Bitrate: 14 kbps)						(Bitrate: 28 kbps)

(c) OOK-1 with 120kHz SCS vs OOK-4, M=4, 30kHz SCS

(Bitrate: 56 kbps)

Figure.3 The LLS performance for carrier frequency error impact investigation

### The impact of different number of OFDM symbols in a block

The impact of different number of symbols constituting one LP-WUS/LP-SS block is shown in Figure.4 and each curve represent different symbol length . In Figure.4, no time/frequency impairments are assumed to focus only on the difference of number of symbols In Figure.5, the impact of timing error are evaluated in the case of 8 symbols and 56 symbols.

From Figure.4, following points are observed:

- In both OOK-1 and OOK-4, BLER performances degrade with larger number of OFDM symbols to constitute a block. At 1%BLER, working points is >=2 dB.

- In Figure.4 (b), OOK-4 is slightly better than OOK-1with large number of OFDM symbol over 32.

From Figure.5, following points are observed:

- Slight performance difference between OOK-1 and OOK-4 due to timing error is observed.

- With smaller number of OFDM symbols, OOK-1 performs slightly better than OOK-4, whereas, with larger number of OFDM symbols to constitute a block, OOK-4 performs better than OOK-1 and performance gap is further increased, as similar as shown in Figure. 2 (c).

- Observation 5: BLER performances in both OOK-1 and OOK-4 get worse with larger number of OFDM symbols constituting a block.

- Observation6: In the case of 28 kbps with smaller payload size, i.e., lower number of OFDM symbols, OOK-1 has better performance than OOK-4 with timing error.

- Observation 7: In the case of 56 kbps with larger payload size, i.e., higher number of OFDM symbols, OOK-4 is slightly better than OOK-1 with large number of OFDM symbol over 32, regardless of timing error or not.

(a) OOK-1 with 60kHz SCS vs OOK-4, M=2, 30kHz SCS   (b) OOK-1 with 120kHz SCS vs OOK-4, M=4, 30kHz SCS

(Bitrate: 28 kbps)						(Bitrate: 56 kbps)

Figure.4 The LLS performance for different number of OFDM symbols in a block, without timing error

(a) OOK-1 with 60kHz SCS vs OOK-4, M=2, 30kHz SCS   (b) OOK-1 with 120kHz SCS vs OOK-4, M=4, 30kHz SCS

(Bitrate: 28 kbps)						(Bitrate: 56 kbps)

Figure.5 The LLS performance for different number of OFDM symbols in a block, with timing error

### Symbol timing estimation by LP-SS

Symbol timing estimation performance by using LP-SS for synchronization is evaluated in TDL-C channel. Timing estimation is performed by energy-based sliding window in time domain for OOK-1 and OOK-4. For OOK-1 and OOK-4 with M=1, 2 OFDM symbols are used for timing estimation at receiver side, assuming simplified symbol timing estimation with LP-SS. For OOK-4 with M=2, only first 2 segments in the OFDM symbol are used for timing estimation. The performances are evaluated for different SNR without timing error and without frequency error. The results in Figure.6 shows CDF vs absolute values of estimation timing error.

From the figure, following points are observed:

- Symbol timing estimation performance decreases with lower SNR.

- With higher SNR, estimation performance with OOK-4 is slightly better than OOK-1, such as in OOK-4, M=2 with 15kHz SCS and OOK-4, M=2 with 30 kHz SCS, due to that OOK-1 with shorter CP length is less robust against ISI and may lead to ambiguity when detecting the symbol boundary.

- Observation 8: For OOK-1, different SCSs such as 30kHz and 60kHz does not obviously impact symbol timing estimation performance in different SNR.

- Observation 9: For OOK-4, different SCSs such as 15kHz and 30kHz, and different M such as M=1,2 does not obviously impact symbol timing estimation performance in different SNR.

Observation 10: Due to the shorter CP length that is less robust to delay in the fading channel, timing estimation performance of OOK-1 with higher SCS is slightly worse than that of OOK-4, especially in higher SNR.

(a) OOK-1 with 30 kHz SCS		(b) OOK-4, M=1 with 30 kHz SCS	(c) OOK-4, M=2 with 15kHz SCS

(d) OOK-1 with 60 kHz SCS		  (e) OOK-4, M=2 with 30 kHz SCS

Figure.6 Impact of SNR on CDF of the absolute timing estimation error with LP-SS

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Observation 1: The pros/cons of OOK-1 and OOK4 waveform are summarized in Table.1

Table.1 Pros/Cons of OOK-1 and OOK-4.

| Comparison of OOK-1 and OOK-4 | OOK-1 | OOK-4 |
| --- | --- | --- |
| Pros | - Less impact to the gNB waveform generation - Simpler design of overlaid sequence, whose length is decided by the bandwidth and SCS of LP-WUS | - More flexibility on the payload size thus more adaptive to more functionalities and better forward compatibility. - More flexible symbol rate means potentially better LP-WUS resource efficiency. - More adaptive to design for idle/inactive and connected modes, respecitively under the unified framework. - Single bit rate design of LP-WUR irrespective of NR SCS would be possible, which simplifies LP-WUR. |
| Cons | - Less flexibility on support the payload size. Only OFDM symbol number is varying factor. - In order to address the support of multiple NR SCS, LP-WUR is also required to support multiple bit rate, which is more complex LP-WUR design. - Forward compatibility of more payload bits relies on extended number of OFDM symbols. - Less adaptive to the required payload size and functionality means less LP-WUS resource efficiency. | - More impact and complexity to the gNB waveform generation, e.g., pre-storing of the generated frequency domain samples - More standardization efforts on defining the overlaid sequence, whose length is decided by the number of segements within a symbol, bandwidth and SCS of LP-WUS |

Proposal 1: LP-WUS should be designed and commonly applied for both RRC CONNECTED and IDLE/INACTIVE mode UEs with fixed symbol rate, irrespective of the SCS used by NR waveform.

Proposal 2: Regardless which one or both waveforms would be supported for LP-WUS and LP-SS, LP-SS and LP-WUS should always follow the same waveform and numerology in the applicable configuration.

- Observation 2: Both OOK-1 and OOK-4 become more sensitive to timing error with higher bitrate due to shorter symbol / time segment length.

- Observation 3: In the case of higher bitrate such as 56 kbps, OOK-4 has slightly better performance than OOK-1 with wider SCS, as OOK-1 with shorter CP is more sensitive to inter-symbol interference.

- Observation 4: With carrier frequency offset, there is no significant performance gap between OOK-1 and OOK-4 and both are robust to even large carrier frequency error. But the robustness also relies on the bandwidth of LP-WUS and GB.

- Observation 5: BLER performances in both OOK-1 and OOK-4 get worse with larger number of OFDM symbols constituting a block.

- Observation 6: In the case of 28 kbps with smaller payload size, i.e., lower number of OFDM symbols, OOK-1 has better performance than OOK-4 with timing error.

- Observation 7: In the case of 56 kbps with larger payload size, i.e., higher number of OFDM symbols, OOK-4 is slightly better than OOK-1 with large number of OFDM symbol over 32, regardless of timing error or not.

- Observation 8: For OOK-1, different SCSs such as 30kHz and 60kHz does not obviously impact symbol timing estimation performance in different SNR.

- Observation 9: For OOK-4, different SCSs such as 15kHz and 30kHz, and different M such as M=1,2 does not obviously impact symbol timing estimation performance in different SNR.

Observation 10: Due to the shorter CP length that is less robust to delay in the fading channel, timing estimation performance of OOK-1 with higher SCS is slightly worse than that of OOK-4, especially in higher SNR.

# Reference

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

# Appendix

From the WID [1], the objectives of the work item are the following:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - This objective is to be further refined in RAN#103