3GPP TSG RAN WG1 #117                                           R1-2405455

Fukuoka City, Fukuoka, Japan, May 20th – 24th,, 2024

Agenda item:    8.1

Source:         Moderator (ZTE)

Title:           Summary #2 for BW aggregation positioning

Document for:   Discussion and Decision

Introduction

- Background

So far, it is in the maintenance stage of NR Rel-18 positioning. The individual draft CRs or TPs are submitted in this meeting.

The focus of this contribution is to summarize the TPs or draft CRs for PRS/SRS bandwidth aggregation, and try to get consensus to solve the remaining problems in the current RAN1 specifications for TS 38.214.

- References

The following papers are provided for PRS/SRS bandwidth aggregation in RAN1#117 meeting. These do not belong to alignment draft CRs.

R1-2403959	Remaining issues of Rel-18 positioning	Huawei, HiSilicon

R1-2405320	Correction to the impact from DL reception on the affected bands	Huawei, HiSilicon

R1-2404052	Maintenance on Expanded and Improved NR Positioning	Nokia

R1-2404154	Draft CR on SRS bandwidth aggregation for positioning measurements	vivo

R1-2404988	Draft CR for dropping rule on SRS bandwidth aggregation	ZTE

R1-2404989	Draft CR on PRS processing for bandwidth aggregation	ZTE

The following contributions for corrections will be handled by Rapporteur.

R1-2404986	Correction on PRS bandwidth aggregation for 38.214	ZTE

R1-2404987	Correction on SRS bandwidth aggregation in RRC_INACTIVE for 38.214	ZTE

Affected bands for SRS

Based on the submitted contributions, the following statements/proposals are identified to be related to this topic:

| Huawei [R1-2403959] [R1-2405320] | Proposal 2: Towards UE transmitting aggregated SRS, RAN1 to clarify that the time duration for UE not expecting to transmit or receive other signals or channels on the affected bands includes both the guard period and the SRS symbol. - Send an LS to RAN4. CR in R1-2405320 in TS 38.214 - 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements <omitted text> When an SRS resource configured in a CC without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of another CC in the same band, there is a guard period subject to the UE capability. During the guard period and the SRS symbols, the UE is not expected to transmit or receive other signals or channels in this band, or any other affected band(s), subject to UE capability. For the linked SRS resource sets for bandwidth aggregation across CCs, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol in the affected band(s) and if the SRS in that symbol is dropped, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol. <omitted text>. |
| --- | --- |

Round 1

FL comments:

We can check if the draft CR in R1-2405320 can be agreed. Further, in the discussion paper R1-2403959, it suggests to send an LS to RAN4 since it is related to RAN4’s questions in previous LS.

    - FL Proposal 2.1-1:

- Towards UE transmitting aggregated SRS, RAN1 to clarify that the time duration for UE not expecting to transmit or receive other signals or channels on the affected bands includes both the guard period and the SRS symbol.

- Send an LS to RAN4.

- Endorse the draft CR in R1-2405320 for TS 38.214 with fixing the typo of ‘decription’ in the cover page.

| Company | Comments |
| --- | --- |
| vivo | We wonder the modification is right that if no collision is expected for SRS transmission why the drop rule for SRS transmission needs to be considered in the next paragraph. In our view, the previous version is “ UE is not expected to transmit or receive other signals or channels for guard period only”. And whether the guard period is needed or not which is dependent on the SRS transmission and SRS priority in the next paragraph. In this case, the modification is not needed. |
| Huawei, HiSillicon | Support. Reply to vivo: We understand the organization would be First paragraph says that network does not expect UE to transmit anything in gap+SRS (if UE transmits SRS). Second paragraph says that if network would ask UE to transmit anything else in gap+SRS, UE will drop part of SRS. |
| CATT | One question on the change of the second paragraph: “if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol in the affected band(s)”, we are wondering a) why only mention “in the affected band(s)” but not include “the same band”, which is included in the previous paragraph; and b) how does the UE determines the affected band(s)”? |
| Qualcomm | Similar view with vivo. |
| Samsung | The revision seems unnecessary in our understanding. The intention of the original spec is to define a guard period between PUSCH or PUCCH and SRS transmission. Based on HW’s explanation, we think a new limitation is introduced as “network does not expect UE to transmit anything in gap+SRS”, which is no need to restrict in the spec. In our understanding, the original spec has already said that if UE transmit other signals or channels, if SRS is dropped in the collide symbol, the SRS symbols is dropped across all CCs. |
| Intel | Share the same view as vivo. Since we define UE behavior for handling of collisions (via SRS dropping), it would not be desirable to further constrain NW scheduling with this current proposal. |
| Moderator | There is no consensus to support this FL proposal. It is supposed no more discussion for this proposal in the future meetings. |

SRS priority

Based on the submitted contributions, the following statements/proposals are identified to be related to this topic:

| vivo [ R1-2404154] | In the last meeting, the priority rule of SRS frequency hopping follows the rule in Clause 6.2.1, but the priority rule of SRS bandwidth aggregation is still unclear. - 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements <omitted text> When an SRS resource configured in a CC without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of another CC in the same band, there is a guard period during which the UE is not expected to transmit or receive other signals or channels in this band, or any other affected band(s), subject to UE capability. For the linked SRS resource sets for bandwidth aggregation across CCs, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol and if the SRS in that symbol is dropped based on priority rules as defined in Clause 6.2.1, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol. <omitted text>. |
| --- | --- |
| ZTE [R1-2404988] | For Rel-17 positioning SRS in RRC_INACTIVE state, SRS is deprioritized (i.e. dropped) if the SRS along with the switching period collides with other signals/channels as shown below. Rel-17 TS 38.214: If an SRS symbol for positioning outside the initial BWP in RRC_INACTIVE mode including the switching time, indicated in higher layer parameter switchingTimeSRS-TX-OtherTX, in unpaired spectrum, subject to UE capability, collides in time domain with other DL signals or channels or UL signals or channels, the colliding SRS symbol for positioning is dropped.. For Rel-18 positioning SRS bandwidth aggregation in RRC_INACTIVE state, SRS should be also deprioritized and dropped if the SRS along with the guard period (instead of switching period) collides with other signals/channels. However, such priority rule is missed in the current TS 38.214. The following is the draft revision: - 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements The UE is expected to be configured with linkage information SRS-PosResourceSetLinkedForAggBWList on SRS resource sets for positioning across two or three CCs which are linked for bandwidth aggregation. For the linked SRS resource sets, the UE is expected to be configured with the same values of startPosition, nrofSymbols, periodicityAndOffset, slotOffset, alpha, p0, spatialRelationInfoPos, resourceType, subcarrier spacing, CP, and comb size, and the UE is expected to maintain phase continuity for the SRS transmission on the same symbol(s). The UE assumes that SRS resources across the linked SRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, otherwise, the UE does not assume that SRS resources of the linked SRS resource sets are linked for bandwidth aggregation. If the UE is configured with dci-TriggeringPosResourceSetLink, and if the UE receives a DCI 0_1, 0_2, 1_1, or 1_2 triggering an aperiodic SRS resource set for positioning linked for bandwidth aggregation in a CC, subject to UE capability, UE transmits SRS of the linked SRS resource sets across all CCs. A UE in RRC_INACTIVE mode is expected to be configured with freqInfoAdditionalCcList on additional component carrier(s) with respective SRS configuration(s) for bandwidth aggregation. When an SRS resource configured in a CC without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of another CC in the same band, there is a guard period during which the UE is not expected to transmit or receive other signals or channels in this band, or any other affected band(s), subject to UE capability. For the linked SRS resource sets for bandwidth aggregation across CCs in RRC_CONNECTED state, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol and if the SRS in that symbol is dropped, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol. For the linked SRS resource sets for bandwidth aggregation in RRC_INACTIVE state, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol, the SRS transmission in all linked SRS resource sets is dropped on that symbol. If the UE receives an activation or deactivation command of semi-persistent SRS resource set(s) for positioning in up to three aggregated carriers or SRS resource set(s) for positioning in up to two aggregated carriers as specified in [10, TS 38.321] and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation or deactivation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission or cessation corresponding to the SRS resource set(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where µ is the SCS configuration for the PUCCH. For positioning SRS resources on multiple carriers linked for aggregation, the channel over which a symbol on one carrier for SRS transmission is conveyed can be inferred from the channel over which the same symbol of another carrier or the aggregated carrier is conveyed. <omitted text>. |

Round 1

FL comments:

From FL perspective, the revisions from vivo and ZTE are valid. So the revisions can be combined as below.

    - FL Proposal 3.1-1:

- Endorse the TP 3.1-1 in section 3.1 of R1-24xxxxx for TS 38.214 clause 6.2.1.4.2

- The draft CR will be prepared accordingly if the TP is agreeable

    - TP#3.1-1:

| Reason for change | For Rel-17 positioning SRS in RRC_INACTIVE state, SRS is deprioritized (i.e. dropped) if the SRS along with the switching period collides with other signals/channels as shown below. If an SRS symbol for positioning outside the initial BWP in RRC_INACTIVE mode including the switching time, indicated in higher layer parameter switchingTimeSRS-TX-OtherTX, in unpaired spectrum, subject to UE capability, collides in time domain with other DL signals or channels or UL signals or channels, the colliding SRS symbol for positioning is dropped.. For Rel-18 positioning SRS bandwidth aggregation in RRC_INACTIVE state, SRS should be also deprioritized and dropped if the SRS along with the guard period (instead of switching period) collides with other signals/channels. However, such priority rule is missed in the current TS 38.214. Further, in the RAN1#116bis meeting, the priority rule of SRS frequency hopping follows the rule in Clause 6.2.1, but the priority rule of SRS bandwidth aggregation is still unclear including for RRC_CONNECTED state. |
| --- | --- |
| Summary of change | Add a new sentence to clarify that for positioning SRS bandwidth aggregation in RRC_INACTIVE state, SRS should be dropped if the SRS along with the guard period collides with other signals/channels. Add ‘Clause 6.2.1’ as a reference to the SRS bandwidth aggregation priority rule for RRC_CONNECTED state. |
| Consequences if not approved | The priority rule of SRS bandwidth aggregation is unclear. |
| Text proposal | ---------------------------- Start of Text Proposal for TS 38.214 ---------------------------- - 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements <omitted text> When an SRS resource configured in a CC without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of another CC in the same band, there is a guard period during which the UE is not expected to transmit or receive other signals or channels in this band, or any other affected band(s), subject to UE capability. For the linked SRS resource sets for bandwidth aggregation across CCs in RRC_CONNECTED state, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol and if the SRS in that symbol is dropped based on priority rules as defined in Clause 6.2.1, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol. For the linked SRS resource sets for bandwidth aggregation in RRC_INACTIVE state, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol, SRS transmission of the all linked SRS resource sets is dropped on that symbol. <omitted text> ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |

| Company | Comments |
| --- | --- |
| vivo | OK |
| Huawei, HiSilicon | No need to split into two paragraphs, otherwise, it’s lengthy and duplicate. |
| CATT | No strong view on split into two paragraphs. The original requirements seem covers already “RRC_CONNECTED” and “RRC_INACTIVE” status. For adding “based on priority rules as defined in Clause 6.2.1”, we are wondering if there is any other cases when “SRS in that symbol is dropped”, but UE still transmit the linked SRS resource sets across all CCs. If not, adding “based on priority rules as defined in Clause 6.2.1” also may not be needed. |
| Qualcomm | We don’t think the “based on priority rules as defined in Clause 6.2.1” is necessary and in the first paragraph we could add just the “in RRC_CONNECTED state” |
| Samsung | The revision is not needed in our understanding. Since we only agree the condition as “if the SRS in that symbol is dropped” in the WI stage, we believe the dropping rule can be applied to any collision cases as long as the SRS is dropped. In fact, we never agreed that SRS should be always determined as low priority in RRC CONNECTED state. Considering that there are many collision rules defined for SRS transmission in TS 38.213 and TS 38.214, a clarification question is why only the priority rules defined in clause 6.2.1 should be adopted here. |
| Intel | The CR is not an essential correction in our view. For the first change, the current text is consistent with the intention – in RRC_INACTIVE, if SRS is to be dropped, that’s fine. Also, agree with Samsung that the rules in Clause 6.2.1 (priority handling) are not the only ones to be taken into account – e.g., there is also handling of DL symbols with SSB, etc. Thus, just a reference to Clause 6.2.1 would not be accurate. |
| Nokia | We are okay with the proposal. |

    - Agreement

Agreement

Endorse the below for TS 38.214 clause 6.2.1.4.2.

| Reason for change | For Rel-17 positioning SRS in RRC_INACTIVE state, SRS is deprioritized (i.e. dropped) if the SRS along with the switching period collides with other signals/channels as shown below. If an SRS symbol for positioning outside the initial BWP in RRC_INACTIVE mode including the switching time, indicated in higher layer parameter switchingTimeSRS-TX-OtherTX, in unpaired spectrum, subject to UE capability, collides in time domain with other DL signals or channels or UL signals or channels, the colliding SRS symbol for positioning is dropped.. For Rel-18 positioning SRS bandwidth aggregation in RRC_INACTIVE state, SRS should be also deprioritized and dropped if the SRS along with the guard period collides with other signals/channels. However, such priority rule is missed in the current TS 38.214. |
| --- | --- |
| Summary of change | Add a new sentence to clarify that for positioning SRS bandwidth aggregation in RRC_INACTIVE state, SRS should be dropped if the SRS along with the guard period collides with other signals/channels. |
| Consequences if not approved | The priority rule of SRS bandwidth aggregation is unclear. |
| Text proposal | ---------------------------- Start of Text Proposal for TS 38.214 ---------------------------- - 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements <omitted text> When an SRS resource configured in a CC without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of another CC in the same band, there is a guard period during which the UE is not expected to transmit or receive other signals or channels in this band, or any other affected band(s), subject to UE capability. For the linked SRS resource sets for bandwidth aggregation across CCs in RRC_CONNECTED state, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol and if the SRS in that symbol is dropped, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol. For the linked SRS resource sets for bandwidth aggregation in RRC_INACTIVE state, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol, SRS transmission of all linked SRS resource sets is dropped on that symbol. <omitted text> ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |

The final CR with tdoc number R1-2405606 has been uploaded.

LOS/NLOS indicator

Based on the submitted contributions, the following statements/proposals are identified to be related to this topic:

| Nokia [R1-2404052] | Proposal 6: Support the following text proposal of Clause 5.1.6.5.3 of TS 38.214 - 5.1.6.5.3 PRS bandwidth aggregation for positioning measurements <omitted text> If the UE reports a DL PRS-RSRP or a DL PRS-RSRPP with aggregated DL RSTD measurement(s) or aggregated UE Rx-Tx time difference measurement(s), the DL PRS-RSRP or the DL PRS-RSRPP correspond to the aggregated DL PRS resources across two or three DL PRS positioning frequency layers. The UE is expected to report an aggregated LoS/NLoS indicator associated with the aggregated DL RSTD measurement(s) or aggregated UE Rx-T time difference measurement(s), if an LoS/NloS indicator is reported. <omitted text>. Reason for change:; RAN1 introduced aggregated positioning measurements using bandwidth aggregation across multiple PFLs. However, it is unclear from the LMF side if reported LoS/NloS indicator(s) was derived from a single DL PRS resource or from aggregated DL PRS resources, as the reported LoS/NloS indicator could be derived from the aggregated DL PRS resources.. Summary of change:; Add the clarification on LoS/NloS measurement reporting for DL PRS bandwidth aggregation.. Consequences if not approved:; It is unclear from the LMF whether the reported LoS/NloS indicator was derived from a single PFL or from aggregated PFLs.. |
| --- | --- |

Round 1

FL comments:

Nokia thinks the current spec is unclear from the LMF side if reported LoS/NloS indicator(s) was derived from a single DL PRS resource or from aggregated DL PRS resources, as the reported LoS/NloS indicator could be derived from the aggregated DL PRS resources. From FL perspective, the LoS/NloS indicator should be shared for the linked PRS resources. Anyway, lets try the TP.

    - FL Proposal 4.1-1:

- Endorse the TP 4.1-1 in section 4.1 of R1-24xxxxx for TS 38.214 clause 5.1.6.5.3

- The draft CR will be prepared accordingly if the TP is agreeable.

    - TP#4.1-1:

| Reason for change | RAN1 introduced aggregated positioning measurements using bandwidth aggregation across multiple PFLs. However, it is unclear from the LMF side if reported LoS/NloS indicator(s) was derived from a single DL PRS resource or from aggregated DL PRS resources, as the reported LoS/NloS indicator could be derived from the aggregated DL PRS resources. |
| --- | --- |
| Summary of change | Add the clarification on LoS/NloS measurement reporting for DL PRS bandwidth aggregation. |
| Consequences if not approved | It is unclear from the LMF whether the reported LoS/NloS indicator was derived from a single PFL or from aggregated PFLs. |
| Text proposal | ---------------------------- Start of Text Proposal for TS 38.214 ---------------------------- - 5.1.6.5.3 PRS bandwidth aggregation for positioning measurements <omitted text> If the UE reports a DL PRS-RSRP or a DL PRS-RSRPP with aggregated DL RSTD measurement(s) or aggregated UE Rx-Tx time difference measurement(s), the DL PRS-RSRP or the DL PRS-RSRPP correspond to the aggregated DL PRS resources across two or three DL PRS positioning frequency layers. The UE is expected to report an aggregated LoS/NloS indicator associated with the aggregated DL RSTD measurement(s) or aggregated UE Rx-T time difference measurement(s), if an LoS/NloS indicator is reported. <omitted text> ---------------------------- End of Text Proposal for TS 38.214 ---------------------------- |

| Company | Comments |
| --- | --- |
| vivo | aggregated LoS/NloS indicator is unclear to us |
| Huawei, HiSilicon | No need. The aggregated time based measurement is based on the aggregated PRS resources. What the aggregated LOS/NLOS indicator is unclear. |
| CATT | Consider aggregated DL PRS resources are sent from the same DL channel, it is unclear to us how to separate LOS/NLOS indicators for individual DL PRS resources and aggregated DL PRS resources for the measurement reporting. |
| Qualcomm | Not needed; there is no requirement, nor a specified way of a UE to derive the LOS/NLOS. Yes, the LOS/NLOS flag is associated with a measurement, and a measurement can be aggregated or non-aggregated. We don’t believe the additional sentence is essential. |
| Intel | Agree with others that it is not necessary to introduce a notion of “aggregated LOS/NLOS indicator”. |
| Nokia | We understand the majority view, but it is unclear from the LMF perspective if the LoS/NloS indicator reported by the UE is made from aggregated resources. We think a minimum clarification could be that “If the UE report an LoS/NloS indicator and aggregated DL RSTD measruement(s) or aggregated Rx-Tx time difference measurement(s), the LoS/NloS indicator should be made from the aggregated DL PRS resources” |
| Moderator | There is no consensus to support this FL proposal. It is supposed no more discussion for this proposal in the future meetings. |

PRS processing

Based on the submitted contributions, the following statements/proposals are identified to be related to this topic:

| ZTE [R1-2404989] | In the current TS 38.214, the description of PRS processing does not include the case of PRS bandwidth aggregation - 5.1.6.5 PRS reception procedure <omitted text> Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed: - Up to 64 NR-SelectedDL-PRS-IndexPerTRP of the DL PRS positioning frequency layer are sorted according to priority if nr-SelectedDL-PRS-IndexListPerFreq is provided, or up to 64 NR-DL-PRS-AssistanceDataPerTRP of the frequency layer are sorted according to priority; except when the UE is requested to perform aggregated measurement(s), in which case: - A DL PRS ID associated with DL PRS bandwidth aggregation linkage has higher priority than a DL PRS ID not associated with DL PRS bandwidth aggregation linkage. If multiple DL PRS ID are associated with DL PRS bandwidth aggregation linkage, they are sorted according to priority. - Up to 2 DL-SelectedPRS-ResourceSetIndex per dl-PRS-ID of the DL PRS positioning frequency layer are sorted according to priority if dl-SelectedPRS-ResourceSetIndexList is provided, or up to 2 NR-DL-PRS-ResourceSet per dl-PRS-ID of the DL PRS positioning frequency layer are sorted according to priority except when the UE is requested to perform aggregated measurement(s), in which case: - A DL PRS resource set linked for a DL PRS bandwidth aggregation has higher priority than a DL PRS resource set not linked for DL PRS bandwidth aggregation. If multiple DL PRS resource sets are linked for DL PRS bandwidth aggregation, then they are sorted according to priority. The UE DL PRS processing capability is defined in [TS 37.355]. For the purpose of DL PRS processing capability, the duration K msec of DL PRS symbols within P msec window, is calculated by - Type 1 duration calculation with UE symbol level buffering capability K=s∈SKs Ks=Tsend−Tsstart - Type 2 duration calculation with UE slot level buffering capability K=12μS - S is the set of slots based on the numerology of the DL PRS of a serving cell within the P msec window in the positioning frequency layer or across positioning frequency layers for bandwidth aggregation that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets. - For Type 1, Tsstart, Tsend is the smallest interval in msec within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential DL PRS symbols and determines the DL PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference). - For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S. <omitted text>. |
| --- | --- |

Round 1

FL comments:

The CR is valid from FL perspective since the current description on PRS processing in TS 38.214 seems only for single frequency layer. We can try if the draft CR in R1-2405320 can be agreed.

    - FL Proposal 5.1-1:

- Endorse the draft CR in R1-2404989 for TS 38.214.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | No need. Since all the slots are aligned across the aggregated PFL, so S is the same for either positioning frequency layer. |
| Qualcomm | Not needed; It doesn’t look to us that the current spec is written only from “one PFL perspective”. In PRS aggregation, the PRS resources across PFLs occupy the same exact symbols so we don’t see the need to clarify further this aspect |
| Samsung | Not needed. It should be noticed that the PRS processing capability mentioned above can be applied to both MG and PPW. However, for PRS bandwidth aggregation, we have already concluded that PPW is not supported in Rel-18. When the revision is applied, it may imply that PPW can be supported for PRS bandwidth aggregation. |
| Intel | Not necessary as explained by others due to same SCS – so, the same footprint in time domain applies. |
| Nokia | We have a similar view with Huawei. The aggregated DL PRS measurement is made from aggregated PFLs on the same slot. In addition, in our understanding, the measurement should be made from the received DL PRS resources on the same symbols. |
| Moderator | There is no consensus to support this FL proposal. It is supposed no more discussion for this proposal in the future meetings. |

Previous agreement

  - RAN1#112

Agreement To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs Agreement To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers - In the same slot, in same symbols, from the same antenna, this implies - FFS: The same gNB Rx TEG and the same UE Tx TEG - The same spatial relation - The same startPosition, nrofSymbols - FFS: periodicityAndOffset, and slotOffset - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of SRS resource sets and resources - The same Tx PSD (power per subcarrier) - FFS whether to need the same pathloss RS, Po and alpha - Note: the Tx PSD is not captured in RAN1 specifications - FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones - Phase continuity between aggregated SRS in different carriers Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. - FFS whether the link is for all TRPs or per TRP basis - FFS whether the link is per PRS resource set basis or per PRS resource basis. Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs - FFS: RSRP, RSRPP - FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement - FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs - FFS RSTD reference configuration or report should be enhanced Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked - SRS resources are per BWP per carrier configuration - FFS whether the link is per SRS resource set basis or per SRS resource basis. Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details Agreement From RAN1 perspective, support UE performs PRS measurement across multiple aggregated PFLs in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE state. Agreement Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods - Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers - FFS: RSRP or RSRPP - FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement - Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs Agreement At least support periodic positioning SRS and semi-persistent positioning SRS for bandwidth aggregation - Support single MAC CE activating positioning SRS resource sets across the linked carriers - FFS whether support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. Study a single DCI scheduling positioning SRS across the linked carriers, and check whether the conclusion/agreements in agenda of multi-cell PUSCH/PDSCH scheduling with a single DCI can be reused - FFS MIMO SRS can be supported for bandwidth aggregation, e.g. with UE transparent way Agreement Study potential power control enhancement of simultaneous transmission of SRS for SRS bandwidth aggregation especially in the case when the total uplink transmission power across multiple carriers exceeds P_c,max Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e. SRS outside BWP and across carriers

  - RAN1#112bis-e

Agreement Study whether single TRP Tx TEG ID or UE Rx TEG ID is applied across PRSs in aggregated PFLs for TEG information reporting, i.e. single TEG ID is reported across the aggregated PRS resources for TRP Tx TEG association reporting, or for UE Rx TEG ID reporting in the measurement reporting Agreement For PRS bandwidth aggregation across PFLs, select one of the following options in RAN1#113 - Option 2: Per TRP basis and per PRS resource set basis. - For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked. - It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied. - Option 3: Per TRP basis and per PRS resource basis. - For each TRP, support new signaling to indicate which PRS resource(s) across PFLs are linked. - For the non-linked PRS resources, no aggregation is assumed even if the conditions are satisfied. Conclusion The legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference is reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same. - Note: multiple PRS/SRS resources which can be used to determine the start of subframe can be from multiple intra-band continuous carriers, - Note: no RAN1 spec impact - Send an LS to RAN4 to confirm RAN1’s understanding Agreement Draft LS to RAN4 is endorsed in R1-2304081. Final LS in R1-2304082. Agreement Support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. - FFS the details Agreement For PRS resources aggregated across PFLs for DL-TDOA and multi-RTT positioning methods, use similar signaling as the existing Rel-16/Rel-17 DL PRS measurement of single PFL with the necessary update. - FFS: In a measurement report element, single RSRP or single RSRPP is reported - In a measurement report element, PFL aggregation indication is supported to indicate whether/which measurement is aggregated - Support new signaling in location information request message to indicate UE whether to perform joint measurement across aggregated PFLs - Single RSTD reference in assistance data and measurement report is used for PRS bandwidth aggregation measurement - FFS RSTD reference is aggregated or not Conclusion The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3. Agreement For SRS bandwidth aggregation between SRS in two or three carriers, the aggregated SRS resources are of the same SRS resource-Type. Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA. Agreement Support the same power prioritization between the aggregated carriers in the case when total UE transmit power in a transmission occasion I exceeds - The UE allocates power to the multiple SRS resources in the transmission occasion i of the aggregated carriers such that the UE’s transmit power in each transmitted resource element is equal. - FFS further details, e.g. power scaling between aggregated carriers Agreement Introduce new UE capability(-ies) to support PRS bandwidth aggregation measurement - FFS the details include the processing capability (N, T), the maximum number of PRS resources that can be process in a slots over the aggregation - FFS the details on the PFL bandwidth combinations, including maximum number of PFLs, the total aggregated bandwidth, etc. - This is applicable for DL-TDOA and Multi-RTT positioning methods Agreement Study whether single UE Tx TEG ID or TRP Rx TEG ID is applied across SRSs in aggregated carriers for TEG information reporting, i.e. single UE Tx TEG ID is reported across the aggregated SRS resources for UE Tx TEG association reporting, or for TRP Rx TEG ID reporting in measurement reporting Agreement Positioning SRS bandwidth aggregation is supported for UEs in RRC_CONNECTED. Positioning SRS bandwidth aggregation is supported for UEs in RRC_INACTIVE state. - For the details, Rel-17 positioning SRS configuration for UE in RRC_INACTIVE state outside initial UL BWP can be the starting point Agreement From RAN1 perspective, MG-based bandwidth aggregation measurement is supported. Decide whether PPW is supported for PRS bandwidth aggregation measurement in RAN1#113 meeting. - FFS the details for PPW if supported Agreement For the case when PRS in one of aggregated PFL is dropped, e.g. because of collision with SSB, select one of the following solutions for LMF based positioning - Alt. 1: Drop positioning measurement in all aggregated PFLs in the same symbol(s) - Alt. 2: Still perform positioning measurement based on the remaining PRSs in other PFL(s) - FFS the details and the difference between MG and PPW if PPW is supported - Note: Up to RAN4 to discuss impact on requirements, if any, for such cases Agreement For SRS bandwidth aggregation across two or three carriers, select one of the following options in RAN1#113 meeting - Option 2: Per SRS resource set basis. - Support new signaling to indicate which SRS resource sets across carriers are linked. - It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied. - Option 3: Per SRS resource basis. - Support new signaling to indicate which SRS resources across carriers are linked. - For the non-linked SRS resources, no aggregation is assumed even if the conditions are satisfied Agreement For the SRS resources across aggregated carriers for UL-TDOA and Multi-RTT positioning methods, use similar signaling as the existing Rel-16/Rel-17 SRS measurement of single carrier with the necessary update - FFS: Single RSRP or RSRPP is reported for the SRS resources across aggregated carriers - SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which measurement is aggregated Agreement For positioning SRS aggregation across CCs, if SRS in one of aggregated carriers is dropped in a symbol, select one of the following two options: - Alt. 1: Stop SRS transmission in all aggregated carriers in the same symbol - Alt. 2: SRS is still transmitted in other carriers in the same symbol - FFS: The UE may not be expected to maintain phase continuity across the remaining carriers - FFS the applicable scenario, e.g. the positioning SRS collides with another higher priority SRS or others Agreement For SRS bandwidth aggregation between SRS in two or three carriers, decide whether one or more of the following are needed for the aggregated SRS resources in RAN1#113 meeting - The same timing advance offset or the same TAG - The same periodicityAndOffset, and slotOffset - The same number of SRS resource sets and/or the same number of SRS resources per set - The configuration of same pathloss RS, Po and alpha to ensure the same Tx PSD (power per subcarrier) - FFS the details, e.g. UE determines the transmit power for SRS transmission in a reference carrier and applies the same Tx PSD for SRS transmission in other carriers, or configure a common parameter set for the aggregated carriers - The same antenna port from RAN1 specification perspective - Note: this is to achieve phase continuity between carriers - UE is expected to be configured with SRS resources that maintain a per-symbol uniformly spaced SRS pattern across aggregated bandwidths - Others if any Agreement For PRS bandwidth aggregation between PRS in two or three different PFLs, decide whether one or more of the following are needed for the aggregated PRS resources from a TRP in RAN1#113 meeting: - The same antenna port from RAN1 perspective - Note: this is to achieve phase continuity between PFLs - The same periodicity and slot offset - The same muting pattern - The same number of PRS resource sets and/or resources per set for a TRP - The same NR-DL-PRS-SFN0-Offset value - UE is expected to be configured with PRS resources that maintain a per-symbol uniformly spaced PRS pattern across aggregated bandwidths - FFS: a per-symbol uniformly spaced PRS pattern across aggregated bandwidths does not preclude dropping some REs in the guardband between two PFLs - Others if any

  - RAN1#113

Agreement For PRS bandwidth aggregation between PRS in two or three different PFLs, the following are needed for the aggregated PRS resources for a TRP: - The same periodicity and slot offset - The same muting pattern - The same NR-DL-PRS-SFN0-Offset value - UE expects to be configured with PRS resources that maintain a per-symbol uniformly spaced PRS pattern across aggregated bandwidths in frequency domain (Note: It does not preclude dropping some REs in the guardband between two PFLs). - FFS same antenna port from RAN1 perspective Agreement For PRS bandwidth aggregation across PFLs, support - Option 2: Per TRP basis and per PRS resource set basis. - For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked. - It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied. Agreement For PRS bandwidth aggregation across PFLs, in a measurement report element, support - Single RSRP or single RSRPP - FFS: the single RSRP/RSRPP is based on aggregated PRS resources across aggregated PFLs - The aggregated reference RSTD - The used PRS resource set IDs for the aggregated measurement which are shared for RSRP/RSRPP and/or timing measurement results Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions. - Send an LS to RAN4 with the above information and a request to provide the retuning time values needed. Agreement The draft LS in R1-2306215 is endorsed. Final LS in R1-2306216. Agreement For PRS bandwidth aggregation, with regards to the signaling in the location information request message, introduce the following: - A request to indicate UE which two or three PFLs to be used for performing joint measurement - A new ReportingGranularityfactor smaller than 0 which can be applicable at least when the LMF requests aggregated measurements - Support at least the values of k={-1,-2} - FFS other values e.g. -3, -4, -5, -6 - Send RAN4 an LS to confirm the feasibility Conclusion For PRS bandwidth aggregation, PPW is not supported in Rel-18. Agreement When the UE receives a request to perform aggregated measurements, - TRP(s) that include PRS aggregation have higher priority than the TRPs that do not include PRS aggregation - If 2 or more TRPs include linked resources, then their priority follows the legacy priority, i.e., sorted in the configuration according to priority - If a PRS resource set is linked for aggregation, then it has higher priority compared to the PRS resource set not linked for aggregation. - If both sets in a PFL are linked for aggregation, then their priority follows the legacy priority, i.e., sorted in the configuration according to priority Agreement For SRS bandwidth aggregation between SRS in two or three carriers, the following is needed for the aggregated SRS resources - The same periodicityAndOffset, and slotOffset - The configuration of pathloss RS, Po and alpha to ensure the same Tx PSD (power per subcarrier) - The same configuration of Po and alpha. - Note: UE may either perform pathloss RS measurement across CCs and form a single path loss value to apply across CCs or perform pathloss RS measurement in a single CC and apply across CCs Agreement For SRS bandwidth aggregation across two or three carriers, support - Option 2: Per SRS resource set basis. - Support new signaling to indicate which SRS resource sets across carriers are linked. - It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied. Agreement To support intra-band contiguous SRS bandwidth aggregation for UE in RRC_INACTIVE state, frequency information (e.g. point A, offset to carrier) of one or two additional carriers with respective SRS configurations should be provided to the UE, where the newly introduced carrier(s) and the carrier of the initial BWP should be intra-band contiguous carriers. Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. Send an LS to RAN2 to confirm the feasibility. Agreement Draft LS in R1-2306213 is endorsed with the following change: ACTION: RAN1 respectfully asks RAN2 to check the feasibility of the working assumption and inform RAN1 of RAN2’s conclusion on the feasibility. Final LS in R1-2306214. Agreement For positioning SRS aggregation transmission in RRC_INACTIVE state, reuse Rel-17 prioritization rule of SRS outside initial BWP, i.e. SRS is dropped in the symbol(s) of all aggregated carriers where collision occurs. Agreement For a carrier including positioning SRS for aggregation, - Positioning SRS can be transmitted only when the carrier is activated - This is also applicable for the carrier only including positioning SRS for aggregation Agreement With regard to support of aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, at least the existing Rel-17 DCI framework (i.e. use multiple DCIs schedule SRSs in multiple carriers) can be reused - FFS: whether Rel-18 DCI framework for multi-cell PDSCH/PUSCH scheduling with a single DCI (i.e. single DCI schedules SRSs in multiple carriers) can also be reused with or without specification work in RAN1. Agreement For SRS bandwidth aggregation across carriers, support - Single RSRP or RSRPP is reported - FFS: the single RSRP/RSRPP is based on aggregated SRS resources across aggregated carriers - The used SRS resource IDs for the aggregated measurement are shared for RSRP/RSRPP and/or timing measurement results

  - RAN1#114

Agreement For PRS/SRS bandwidth aggregation between two or three different PFLs/carriers, send a reply LS to request RAN4 to capture the condition of ‘the same RF chain (same antenna)’ in RAN4 specification. Agreement For the case when PRS in one of aggregated PFL is dropped because of collision with other signals, for LMF based positioning, it is up to UE implementation to perform positioning measurement based on one or more of the PRS resources in the aggregated PFLs. - Note: it is up to RAN4 whether or not to define performance requirements for this case of collision with other signals Agreement In RRC_CONNECTED state, for positioning SRS aggregation across CCs, if SRS in one of aggregated carriers is dropped in a symbol, stop SRS transmission in all aggregated carriers in the same symbol Agreement Send an LS to RAN2 with the following content With regards to higher layer parameter dl-PRS-ID, RAN1 understands that the current RAN2 specification support two interpretations: - Interpretation 1: PRS resource sets in different PFLs of a TRP are configured with the same dl-PRS-ID - Interpretation 2: PRS resource sets in different PFLs of a TRP can be configured with different dl-PRS-ID For PRS bandwidth aggregation, RAN1’s agreement is that the linked PRS resource sets from two or three PFLs should be from the same TRP. RAN1 kindly requests RAN2 to capture the condition of the same TRP in RAN2 specifications for PRS bandwidth aggregation. Agreement Endorse the draft LS in R1-2308645 with the following modification to the action: ACTION: RAN1 respectfully ask RAN2 to take the above information into consideration for their future work, and asks RAN2 to capture the condition of the same TRP in RAN2 specifications for PRS bandwidth aggregation. Final LS in R1-2308646. Agreement With regard to aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, support both Option 2 and Option1. - Option 2: Support to use a DCI format 0_3 or 1_3 for multi-cell PDSCH/PUSCH scheduling to trigger SRS resources for bandwidth aggregation in multiple CCs. - Option 1: Support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers, as a separate UE capability. - Reuse Rel-17 DCI framework without modification. - If a single DCI indicates transmission of an aperiodic positioning SRS resource set, UE transmits aperiodic positioning SRS resource sets across all linked carriers for bandwidth aggregation. Reply LS to RAN4 LS in R1-2306363 on Bandwidth Aggregation Agreement The draft LS in R1-2308448 is endorsed as a reply to RAN2. Final LS in R1-2308449.

  - RAN1#114bis

Agreement Configuring up to two PFL combinations is supported (e.g. PFL1 aggregated with PFL2 and PFL3 aggregated with PFL4). - Send an LS to RAN4 (CC to RAN2 and RAN3) to inform them with the above agreement and specify corre-sponding requirements. - Note: more than one combinations are measured in TDMed manner Agreement The draft LS to RAN4 in R1-2310477 is endorsed. Final LS in R1-2310478. Agreement Endorse the TP in section 3.2 of R1-2309227 for TS 38.214 clause 6.2.1.4. Agreement Endorse TP 6.2-2 in section 6.2.2 of R1-2309227 for TS 38.214 clause 5.1.6.5. Agreement Endorse TP 5.1-1 in section 5.1-1 of R1-2309227 for TS 38.214 clause 5.1.6.5. Agreement For positioning SRS bandwidth aggregation, introduce a new RRC signaling to indicate whether to enable Rel-17 single DCI-triggering SRS resource sets across the linked carriers. Agreement Confirm the following WA: Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources.. Agreement Endorse the TP in section 8.1.1 of R1-2309228 for TS 38.214 clause 5.1.6.5 and 6.1.2.4 Agreement With regards to the bandwidth aggregation measurement for positioning, suggest to the editor of TS38.214 to align the terminology between “joint measurement” and “aggregated measurement” by using only “aggregated measurement”. Agreement Endorse the TP in section 9.1.1 of R1-2309228 for TS 38.213 clause 7.3.1. Agreement When the LMF requests aggregated measurements, the following existing requested fields can also be applicable: - A request for reduced sample processing for aggregated measurement - Reuse the existing field: reducedDL-PRS-ProcessingSamples-r17 - A request for lower Rx beam sweeping factor for FR2 that is applicable for aggregated measurements - Reuse the existing field: lowerRxBeamSweepingFactor-FR2 - A request for the maximum number of aggregated UE-Rx-Tx / RSTD measurements for different DL-PRS Resources or DL-PRS Resource Sets per TRP - Reuse the existing field: maxDL-PRS-RSTD-MeasurementsPerTRPPair

  - RAN1#115

Agreement The new ReportingGranularityfactor also supports k = {-3, -4, -5, -6} in addition to {-1, -2} - These k values are applicable for timing measurements for all applicable positioning methods - Support for both DL and UL - Support for both FR1 and FR2 - Reply the RAN4 LS R1-2310797, and CC to RAN2 and RAN3. Agreement Draft LS reply to RAN4 in R1-2312394 is endorsed. Final LS in R1-2312395. Conclusion With regards to TEG reporting for PRS/SRS bandwidth aggregation, for Rx, a single Rx or RxTx TEG ID is reported for the aggregated measurement. Agreement When the LMF requests aggregated measurements, the following existing requested fields can also be applicable: - A request for configuring the UE to measure the same aggregated DL-PRS Resources of a TRP with N different UE Rx TEGs - Reuse the existing IE: measureSameDL-PRS-ResourceWithDifferentRxTEGs - A request for configuring the UE to measure the same aggregated DL-PRS Resources of a TRP with N different UE RxTx TEGs - Reuse the existing IE: measureSameDL-PRS-ResourceWithDifferentRxTxTEGs Agreement Endorse the TP 2.1-2 in section 2.1.2 of R1-2311464 for TS 38.212 clause 7.3.1.1.2. Agreement Endorse the TP 3.1-1 in section 3.1.1 of R1-2311464 for TS 38.214 clause 6.2.1.4.2. Agreement If the UE/gNB reports aggregated timing measurement, the single reported RSRP/RSRPP (if reported) is based on aggregated PRS/SRS resources across aggregated PFLs/carriers. - Note1: it is up to RAN4 whether to define a corresponding requirement - Note2: for UL, measured SRS signals refer to aggregated SRS resources. For DL, measured PRS signals refer to aggregated PRS resources. Agreement Endorse the TP 2.1-1 in section 2.1.1 of R1-2311465 for TS 38.214 clause 6.2.1.4.2 Agreement Endorse the TP 4.1-1 in section 4.1.1 of R1-2311465 for TS 38.214 clause 5.1.6.5.3 Agreement The TP below is endorsed for TS38.214 TP 10.1-2. Reason for change; In the current RAN1 specification, it specifies that the UE may assume that the PRS/SRS resources across the linked PRS/SRS resource sets are linked for bandwidth aggregation. If the linked PRS/SRS resource sets satisfy the listed conditions, the UE should assume these resource sets are linked for the bandwidth aggregation. The current “may assume” is not a clear wording.. Summary of change; The UE should assume or should determine that DL PRS resources across the linked DL PRS resource sets which satisfy the above conditions are linked for bandwidth aggregation.. Consequences if not approved; The current “may assume” is not a clear wording.. Text proposal; -------------------------------------- TS 38.214 ----------------------------------------------------- < Unchanged text omitted > ##### 5.1.6.5.3 PRS bandwidth aggregation for positioning measurements When the UE is expected to perform aggregated measurements for bandwidth aggregation across DL PRS positioning frequency layers, the UE expects to be configured with linkage information, via higher layer parameter [linkage], between DL PRS resource sets across DL PRS positioning frequency layers. For the linked DL PRS resource sets, the UE is expected to be configured with the same values of QCL, dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-NumSymbols, dl-PRS-ResourceTimeGap, dl-PRS-ResourceRepetitionFactor, dl-PRS-ResourceSymbolOffset, dl-prs-MutingBitRepetitionFactor, dl-PRS-CyclicPrefix, comb size, power per subcarrier, NR-MutingPattern, and NR-DL-PRS-SFN0-Offset, and the UE is expected to be configured with DL PRS resources that maintain uniformly spaced DL PRS RE pattern within a symbol across aggregated DL PRS positioning frequency layers. The UE may assumes that DL PRS resources across the linked DL PRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, and the UE may assume phase continuity on the DL PRS resources on same symbol(s); otherwise, the UE does not assume that PRS resources from the linked DL PRS resource sets are linked for bandwidth aggregation. < Unchanged text omitted > 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements The UE is expected to be configured with linkage information [linkage] on SRS resource sets for positioning across two or three CCs which are linked for bandwidth aggregation. For the linked SRS resource sets, the UE is expected to be configured with the same values of startPosition, nrofSymbols, periodicityAndOffset, slotOffset, alpha, p0, subcarrier spacing, CP, and comb size, and the UE is expected to maintain phase continuity for the SRS transmission. The UE may assumes that SRS resources across the linked SRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, otherwise, the UE does not assume that SRS resources of the linked SRS resource sets are linked for bandwidth aggregation. For the linked SRS resource sets for bandwidth aggregation across CCs, if an SRS configured by the higher layer parameter SRS-PosResource, along with the [switching period] when applicable, collides with other signals or channels on a symbol and if the SRS in that symbol is dropped, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol. < Unchanged text omitted >.

  - RAN1#116

Agreement For PRS/SRS bandwidth aggregation, capture the “single Tx chain” (same Tx antenna) assumption in RAN1 specification. Endorse the TP 2.1-2 in section 2.1 of R1-2401594 for TS 38.214 clause 5.1.6.5.3 and 6.2.1.4.2. Agreement RAN1 understands that the current RRC ASN.1 only supports single “aggregated combination”, in which only one SRS resource set from each of the 2 or 3 carriers are aggregated, e.g. CC#1 SRS resource set 1 + CC#2 SRS resource set 2 + CC#3 SRS resource set 3. RAN1 suggests to extend the number of such “aggregated combinations” to up to 32. Send an LS to RAN2 and RAN3. Conclusion It is supported that the SCell not configured with DL but contain only positioning SRS in the UL to be aggregated with positioning SRS on another PCell/SCell. Send LS to RAN2. Agreement The draft LS to RAN2 and RAN3 in R1-2401707 is endorsed. Final LS in R1-2401708. Agreement Endorse the TP below for TS 38.214 clause 6.2.1.4.2 Reason for change; There are brackets in the specification. Summary of change; Remove the whole bracket and make the spec complete. Consequences if not approved; The specification is not complete. Text proposal; ---------------------------- Start of Text Proposal for TS 38.214 ---------------------------- - 6.2.1.4.2 SRS bandwidth aggregation for positioning measurements < Unchanged parts are omitted > When an SRS resource configured in a CC without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of another CC, there is a guard period during which the UE is not expected to transmit or receive other signals or channels, subject to UE capability. ---------------------------- End of Text Proposal for TS 38.214 ----------------------------. Agreement Endorse the TP 9.1-2 in section 9.1 of R1-2401594 for TS 38.214 clause 5.1.6.5.3 and 6.2.1.4.2 Agreement Endorse the TP 10.1-1 in section 10.1 of R1-2401594 for TS 38.214 clauses 5.1.6.5. Agreement Endorse the TP 10.1-2 in section 10.1 of R1-2401594 for TS 38.214 clauses 5.1.6.5.3 and 6.2.1.4.2. Agreement TP 6.2-1 in section 6.2 of R1-2401595 for TS 38.214 clause 5.1.6.5.3 is endorsed. This does not have LPP impact Agreement TP 8.1-1 in section 8.1 of R1-2401595 for TS 38.214 clause 5.1.6.5.3 is endorsed.

  - RAN1#117

Agreement TP 2.1-1a in section 2.1 of R1-2403426 for TS 38.214 clause 6.2.1.4.2 is endorsed. Agreement TP 5.1-1 in section 5.1 of R1-2403426 for TS 38.214 clause 5.1.6.5 is endorsed as editorial correction. Agreement Send an LS to RAN2 indicating that at least one PRS resource ID from one of aggregated resource sets may be reported in one measurement report element for the main and additional measurements, respectively. The existing PRS resource ID can be reused, details up to RAN2. Agreement The draft LS in R1-2403717 is endorsed with the following revision: - However, from RAN1 perspective, PRS resource ID can be optionally reported for PRS bandwidth aggregation. Hence, RAN1 made the following agreement. Final LS in R1-2403728.