3GPP TSG RAN WG1 Meeting #118				                                   R1-2405943

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:  	9.2.2

Source:             	Fraunhofer IIS, Fraunhofer HHI

Title:                  	CSI enhancements for Rel. 19 MIMO

Document for: 		Decision

# Introduction

In RAN1#102, a new work item for NR MIMO phase 5 has been agreed. In this contribution, we discuss our view on the remaining issues related to CSI enhancements.

# CSI enhancements

Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, - N1⋅N2X1⋅X2-bit group-based CBSR - the following (X1, X2) values are supported: P; (N1, N2); (X1, X2). 48; (8,3); (1,1), (2,1), (4,1). (6,4); (1,1), (2,1), (2,2),. 64; (16,2); (1,1), (2,1), (2,2), (4,1), (4,2). (8,4); (1,1), (2,1), (2,2), (4,1), (4,2). 128; (16,4); (1,1), (2,1), (2,2), (4,1), (4,2). (8,8); (1,1), (2,1), (2,2), (4,1), (4,2). - FFS: (1,2), (1,4), (2,4), when applicable

Type-II CBSR RRC overhead reduction

Codebook subset restriction has been supported since Rel. 15 NR and is configured to the UE via information element (IE) n1-n2-codebookSubsetRestriction.The IE n1-n2-codebookSubsetRestriction forms the bit-sequence B= B1B2, where the two-bit sequences are concatenated to form bit sequence B. The UE is configured with restrictions for 4 groups out of O1O2 groups, wherein each group comprises N1N2  vectors.  The remaining O1O2-4 groups are not restricted. The 4 groups are ordered according to the equation gk=O1r2k+r1k, where r1∈{0,…, O1-1} and r2∈{0,…, O2-1}, and wherein gk is the group index and k=0,…, 3 denotes the index of the 4 groups configured with restrictions and the indices are assigned such that gk increases as k increases. The bit-sequence B1 is a log2O1O24-bit indicator and indicates the 4 groups configured with restrictions. The second bit sequence B2 is the concatenation of bit sequences B2k, k = 0,1,…, K-1  given by B2=B20B21…B23 corresponding to the group indices gk. The 4 restricted groups are determined by the wireless device based on the bit-sequence B1. The bit sequence B2k is of length 2N1N2 and is defined as B2k=b2(k,2N1N2-1)…b2k,0. Bits b2(k,2N1N2-1)…b2k,0. Bits b2(k,2N1x2+x1+1)b2k,2N1x2+x1 indicate the maximum allowed amplitude coefficient pl,i1 for the vector in group gk indexed by x1, x2, where the maximum amplitude coefficients are given in the Table 1.

Table 1: Maximum allowed amplitude coefficients for restricted vectors

| Bits b2(k,2N1x2+x1+1)b2k,2N1x2+x1 | Maximum Amplitude coefficient pl,i1 |
| --- | --- |
| 00 | 0 |
| 01 | 0.25 |
| 10 | 0.5 |
| 11 | 1 |

For the agreed value of N1 and N2, the overhead associated with the RRC configured IE n1-n2-codebookSubsetRestriction is very high. This becomes notable as the second bit sequence B2 scales proportionally with the values of N1 and N2. Therefore, it was agreed in RAN1#116-bis to support group based CBSR, where each bit in the CBSR is associated with a set of X1X2 SD basis vectors, where the set includes X1 adjacent SD basis vectors along the N1 direction and/or X2 adjacent SD bases along the N2 direction. The ordering of the SD basis vectors in each group is given by N1x2+x1, wherein x1=0,…,N1-1 and x2=0,…,N2-1. Hence, the N1N2 SD basis vectors in each group are ordered with respect to N1 dimension first and N2 dimension second. This means, the first N1 SD vectors in each group are associated with index x2=0, the second N2 SD vectors in each group are associated with index x2=1, and so on and the last N2 SD vectors in each group are associated with index x2=N2-1. Based on the ordering of the SD basis vectors, several combinations of X1,X2 have been agreed for each agreed combination of (N1,N2). However, the agreed values are too small to reduce the configuration overhead for large values of (N1,N2). Hence, to fully utilize the potential of group-based CBSR, additional combinations of X1 and X2 need to be supported. The proposed values of  X1,X2 for different combinations are provided in the following table.

Table 2: Allowable combinations of X1,X2 for different combinations of (N1,N2).

| P | (N1,N2) | Supported values of X1,X2 | Proposed values of X1,X2 |
| --- | --- | --- | --- |
| 48 | (8,3) | (1,1), (2,1), (4,1) | (1,3), (8,1) |
|  | (6,4) | (1,1), (2,1), (2,2), | (1,4) |
| 64 | (16,2) | (1,1), (2,1), (2,2), (4,1), (4,2) | (1,2), (8,1), (8,2) |
|  | (8,4) | (1,1), (2,1), (2,2), (4,1), (4,2) | (1,4), (2,4), (8,1) |
| 128 | (16,4) | (1,1), (2,1), (2,2), (4,1), (4,2) | (2,4), (4,4), (8,2) |
|  | (8,8) | (1,1), (2,1), (2,2), (4,1), (4,2) | (2,4), (1,8), (8,1) |

Observation: To fully utilize the potential of group-based CBSR, additional combinations of X1,X2 can be supported for different combinations of (N1,N2) to further reduce the configuration overhead.

Proposal: Support the following combinations of X1,X2 and (N1,N2)

- For (N1,N2) =(8,3), support X1 ,X2 =1,3,(8,1)

- For (N1,N2) =(6,4), support (X1 ,X2 )=(1,4)

- For (N1,N2) =(16,2), support X1 ,X2 =1,2,8,1

- For (N1,N2) =(8,4), support X1 ,X2 =1,4, 2,4,8,1

- For (N1,N2) =(16,4), support X1 ,X2 =2,4, 4,4, (8,1)

- For (N1,N2) =(8,8), support X1 ,X2 =2,4, 4,4, (8,1)

Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, support the following schemes: - The same O1=O2 value(s) as RI=1-4 are supported - Scheme-A (based on Scheme3 described in RAN1#116bis): - W1 structure: - The 1st SD basis vector is freely selected and subsequent 2 (RI=5-6) or 3 SD basis vectors (RI=7-8) are freely selected such that they are orthogonal in at least one dimension (horizontal or vertical). - The v layers are mapped to the selected SD basis vectors following legacy Rel-15 Type-I for RI=5-8. - W2 structure: - Following legacy Rel-15 Type-I RI=5-8 - Scheme-B (based on Scheme2 described in RAN1#116bis): - W1 structure: - Independent selection of different ceil(v/2) SD basis vectors for RI = v, where each SD basis vector is applied to two respective layers following legacy Rel-15 Type-I for RI=5-8, except that, if v is odd, the last SD basis vector is applied to the orphan layer. - FFS: mapping between the orphan layer and its selected SD basis vector and, if needed, UE reporting of the selection - FFS: support of 4 selected SD basis vectors for RI=5-6 - The SD basis vectors are freely selected from a group of N1N2 orthogonal SD DFT basis vectors via combinatorial indication, as well as a layer-common (q1,q2) - W2 structure: - For the orphan layer, the inter-polarization co-phasing is selected from {1, j, -1, -j} - For two layers sharing a same SD basis vector, the inter-polarization co-phasing between two layers is selected from the following pairs {(1, -1), (j, -j)} to achieve inter-layer orthogonality. - Only Scheme-A (RI=1-4+RI=5-8) and Scheme-B (RI=1-4+RI=5-8) are supported in Rel-19 Proposal [1.A.6]: For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, regarding Scheme-B, reuse the legacy Rel-15 Type-I layer pairing scheme, and decide (by RAN1#118) from the following alternatives (red texts are new alternatives mentioned offline in Round-4): - Alt1 (fixed mapping between SD basis vectors and layers): - The k-th SD basis vector is associated with the k-th layer-group. - Note: This should be the baseline based on joint SD basis selection/indication using the agreed combinatorial indication (logCLGN1N2 bits). - Alt2 (UE-selected SD basis vector for the orphan layer): - In addition to the agreed combinatorial indication (logCLGN1N2 bits), The SD basis vector associated with the orphan layer is selected from the v/2 SD basis vectors and indicated with log⁡(v/2) bits - Except for the orphan layer and the associated SD basis vector, the j-th SD basis vector from the remaining SD basis vectors is associated with the j-th layer group from the remaining layer-groups. - Note: The proponents claim that the ordering information across all layer-groups is not needed, except for the orphan layer (cf. SLS from ZTE showing gain with orphan layer indication). This claim was questioned by a number of companies. - Alt3: - Indication of the 2 SD basis vectors associated with the first and second layer-groups, respectively, from the selected v/2 SD basis vectors, with 2*log⁡(v/2) bits - Except for the first 2 layer-groups and the associated SD basis vectors, the j-th SD basis vector from the remaining SD basis vector(s) is associated with the j-th layer group from the remaining layer-group(s). - Alt4: replace the agreed joint selection/indication using combinatorial indication (logCLGN1N2 bits) with per-layer-group selection/indication, i.e. with log⁡(N1N2) bits per layer-group, where LG denotes the number of layer groups - Note: In this case, full information on ordering across all layer-groups is not needed since each layer-group freely selects an SD basis vector - Note: It can be argued that this reverts a previous agreement on “via combinatorial indication”. Despite its simplicity, this is not preferred from FL perspective unless there is consensus - Alt5: replace the agreed joint selection/indication using combinatorial indication (logCLGN1N2 bits) with permutational indication (logPLGN1N2 bits) where LG denotes the number of layer groups - Note: In this case, full information on ordering across all layer-groups is included in permutation - Note: It can be argued that this reverts a previous agreement on “via combinatorial indication”. Despite its elegance, this is not preferred from FL perspective unless there is consensus - Alt6: add log⁡(LG!)-bit ordering information (in addition to the agreed combinatorial indication), where LG denotes the number of layer groups - Note: In this case, full information on ordering across all layer-groups is included in the additional indicator. While it is not as simple/elegant as Alt4/Alt5, this alternative doesn’t revert previous agreement and is functionally equivalent to Alt5 (with a bit more overhead in some cases)

In RAN1#117, for RI = 5-8, two codebook schemes namely scheme-A and scheme-B were agreed. For scheme-B, for RI=v, the UE freely determines v/2 SD basis vectors from a group of N1N2 orthogonal SD DFT basis vectors. From the determined v/2 SD basis vectors, a single SD basis vector is applied to a layer group comprising two consecutive layers, and it was also agreed to report the selected v/2 SD basis via a combinatorial indicator. Based on the agreement, a log2CLGN1N2  bit indicator can be used to report the selected LG= v/2 SD basis vectors to the UE. However, the network lacks information of the SD basis vector selected by the UE for each layer group. In the following, six alternatives listed by the FL in the offline discussion are discussed in detail.

With Alt 1, a log2CLGN1N2-bit indicator is used to indicate the selected LG SD basis vectors associated with the LG layer groups. From the reported LG SD basis vectors, the k-th SD basis vector is applied to the k-th layer group. As this alternative is based on a fixed mapping between the reported LG SD basis vectors and the LG layers, a single log2CLGN1N2-bit indicator is only reported by the UE. Alt 2 is similar to Alt 1 with an exception that in addition to the log2CLGN1N2-bit indicator, the selected SD basis vector associated with the orphan layer from the selected LG basis vectors is indicated to the UE using a log2(v/2)-bit indicator resulting in a total feedback overhead of log2CLGN1N2+log2(v/2) bits. Alt 3 is similar to Alt 1 with an exception that in addition to the log2CLGN1N2-bit indicator, the selected SD basis vectors associated with the first and the second layer groups from the selected LG SD basis vectors are indicated to the UE separately using two log2(v/2)-bit indicators resulting in a total feedback overhead of log2CLGN1N2+2⋅ log⁡(v/2)-bits. The motivation of Alt 2 and Alt3 is not clear as the selected SD basis vectors is reported for only a subset of layer groups is reported by the UE. With Alt 4, the selected SD basis per layer group is indicated using a log2(N1N2)-bit indicator resulting in total feedback of LG⋅log2(N1N2) bits. With Alt 5, the selection of the LG SD basis vectors from the group of N1N2 orthogonal SD basis vectors and the mapping of the selected LG SD basis vectors to the LG layer groups is indicated to the UE using a single permutation indicator log2(PLGN1N2)-bit indicator, where PLGN1N2= N1N2! N1N2-LG!. Alt 6 is based on adding the mapping information i.e., the mapping of the selected LG SD basis vectors to the LG layer groups to Alt 1, Alt 2 and Alt 3.

Alt 1 with fixed mapping destroys the purpose of free SD basis selection as the selected SD basis vector of each layer group selection is not free but rather restricted due to the fixed mapping between the selected LG SD basis vectors to the LG layer groups. On the other hand, UE can still perform free SD basis selection per layer group and indicate the LG SD basis vectors for the LG layer groups using the combinatorial indicator of Alt 1. Additionally, a log2(LG!)-bit indicator can be reported by the UE to indicate the mapping of the selected LG SD basis vectors to the LG layer groups. With this approach there is no need to revert the previous agreement of using a combinatorial indicator to jointly indicate the LG SD basis vectors of LG layer groups and the network can be provided with full information regarding the selected SD basis vector associated with each layer group. Among the six alternatives, only Alt 4, Alt 5 and Alt 1 in combination with Alt 6 provides full information of the selected SD basis vector per layer group to the network. Compared to Alt 5, Alt 4 is simple and straight forward followed by Alt 1+Alt 6, whereas Alt 5 is complex due to the permutation involved. Therefore, for simplicity, Alt 4 or Alt 1+Alt 6 needs to be supported. Alt 1 can also be considered if Alt 4 or Alt 1+Alt 6 does not provide a good trade-off between feedback overhead and throughput gains.

Observation: Alt 1 with fixed mapping destroys the purpose of free SD basis selection as it does not support indication of the selected SD basis vector per layer group.

Observation: Alt 4 is simple and straightforward among the six alternatives.

Observation: Alt 1 in combination with Alt 6 provides mapping of the LG SD basis vectors to the LG layer groups to the network.

Proposal: Support Alt 4 (first preference) or Alt 1 (second preference).

In RAN1#117, For scheme B, it was already agreed that a single SD basis vector is associated with a layer group comprising two layers. Therefore, supporting 4 SD basis vectors can be seen as violating the agreement from RAN1#117.

Proposal: Do not support selection of 4 SD basis vector for RI = 5-6.

# Conclusion

In this contribution, we have the following observations and proposals.

Observation: To fully utilize the potential of group-based CBSR, additional combinations of X1,X2 can be supported for different combinations of (N1,N2) to further reduce the configuration overhead.

Proposal: Support the following combinations of X1,X2 and (N1,N2)

- For (N1,N2) =(8,3), support X1 ,X2 =1,3,(8,1)

- For (N1,N2) =(6,4), support (X1 ,X2 )=(1,4)

- For (N1,N2) =(16,2), support X1 ,X2 =1,2,8,1

- For (N1,N2) =(8,4), support X1 ,X2 =1,4, 2,4,8,1

- For (N1,N2) =(16,4), support X1 ,X2 =2,4, 4,4, (8,1)

- For (N1,N2) =(8,8), support X1 ,X2 =2,4, 4,4, (8,1)

Observation: Alt 1 with fixed mapping destroys the purpose of free SD basis selection as it does not support indication of the selected SD basis vector per layer group.

Observation: Alt 4 is simple and straightforward among the six alternatives.

Observation: Alt 1 in combination with Alt 6 provides mapping of the LG SD basis vectors to the LG layer groups to the network.

Proposal: Support Alt 4 (first preference) or Alt 1 (second preference).

Proposal: Do not support selection of 4 SD basis vector for RI = 5-6.

# References

- 3GPP RAN#102, New WID: NR MIMO Phase 5, RP-234007

- Chair Notes, 3GPP RAN1#117, Fukuoka, Japan