3GPP TSG RAN WG1 #118																	R1-2406097

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.5.3

Source:	China Telecom

Title:	Discussion on common channel signal adaptation

Document for:	Discussion and decision

- Introduction

In the RAN #102 meeting, a new WID for Rel-19 enhancement of network energy savings was approved [1]. The objectives are listed as follows:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In last meeting, multiple potential techniques were proposed by companies and discussed for common channels/signals adaptation, more details can be found in [2]. In this contribution, we provide our views on SSB and PRACH adaptation.

- Considerations on SSB adaptation

In RAN #117 meeting [1], the following proposal was discussed with respect to SSB adaptation:

Proposal 2.1.1-updated For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

In this section, we further discuss corresponding issues based on the above proposal.

- 2.1 Applicable scenario of SSB adaptation

In RAN1 #116bis meeting, the following agreement was reached.

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

And in last meeting, there is no more discussion progress on the issue. When look back to the options, we found that A3, B3 are actually the subsets of A2, A3. Which means, if Option A2/B2 are supported, Option A3/B3 will be supported too. Thus, down selections should be made between A2 and A3/B2 and B3.

Observation 1: Options A3/B3 are subset of Option A2/A3, thus, if A2/B2 are support, A3/B3 will be supported naturally.

In the common understanding, the SSB adaptation can be applied to the SCell in multi-carrier scenario, since the legacy SSB in PCell can always guarantee the network performance. Besides, for UE who are in connected mode, there is no need to decode SIB1 for initial random access, which is not transmitted in SSB, the SSB adaptation can also be applied. However, even if UE is in connected mode, UE may still need to perform DL timing and RLM/RRM measurement on PCell, the adaptation on CD-SSB of PCell may result in the degradation of performance. Thus, we think Option A1 should not be supported for NES-capable UE’s PCell.

Proposal 1: Deprioritize Option A1, i.e., adaptation for CD-SSB of PCell (connected mode), for Rel-19 NES-capable UEs.

For the other Options, since there are more than one UEs camping on one Cell, which means SCell for one UE may be PCell for other UEs. Thus, if SSB adaptation is applied to CD-SSBs on SCell, it may have impact on the CD-SSBs on PCell also, which should be further discussed. Even if such mechanism is supported, network may need to think twice before configuring the SSB adaptation in practical network. But for the other Options, we think they can be supported firstly.

Proposal 2: At least support Option A2 and B2 for SSB adaptation for Rel-NES capable UEs.

Also, if A2 can be supported, for connected UEs with single carrier, SSB adaptation can also be supported, at least for the low load network in Rel-19 NES. The spec impact of NCD-SSB adaptation for connected on single carrier and PCell of multi-carrier scenario can be similar. Thus, introducing NCD-SSB adaptation for single carrier scenario is feasible.

Proposal 3: NCD-SSB adaptation for UE in connected mode in single carrier scenario can also be supported if A2 is supported.

Adaptation of SSB periodicity is the one of the most straightforward methods to achieve the SSB adaptation in time domain. If so, as long as the SSB periodicity can be adjusted in time, the longer periodicity of SSB can be considered to achieve larger NES gain with SSB adaptation. For the scenarios supported with SSB adaptation, the dormancy duration of corresponding cell can be larger and the energy efficiency is improved. Thus, it is beneficial to adopt a longer SSB periodicity at the same time.

Proposal 4: Support introducing SSB with longer periodicity for SSB adaptation.

- 2.2 Cell DTX for connected mode UEs for SSB

By entering Cell-DTX mode, the NES gain can be harvested by allowing the network to adapt to varying traffic loads and operational conditions. However, for connected mode UEs, the need for SSB is generally tied to mobility management and measurement. In high-mobility scenarios, for instance, UEs rely on frequent SSB transmissions for reliable handovers and measurements. If DTX periods are not managed effectively, this could compromise the performance and reliability of these critical functions.

Proposal 5: Support to enhance Cell DTX for SSB for connected mode UEs

- FFS: Management of DTX periods for connected mode UEs in high-mobility scenarios.

- Considerations on PRACH adaptation

- 3.1 The additional conditions for additional PRACH resources

In RAN #117 meeting [1], the following agreements were achieved about the PRACH overlap for PRACH adaptation as follows:

Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

For Case 1 and Case 3, no additional conditions are needed, as the non-overlapping nature of additional PRACH resources and legacy PRACH resources in the time domain simplifies the mapping and resource allocation processes. However, in Case 2, where additional PRACH resources overlap with legacy PRACH resources, additional conditions are needed to avoid potential detection issue. Because the overlapping ROs in time domain may be associated with different SSBs, thus more SSBs are associated with ROs in one time instance. Then, gNB needs to detect more directions for preamble reception.

Proposal 6: Additional conditions need to be considered for Case 2, i.e., time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs.

- 3.2 Adaptation mechanism for additional PRACH resources

In RAN1 #117 meeting, the following agreements were reached on the adaptation mechanism for PRACH adaptation.

Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

PRACH is crucial for enabling initial access for UE and acquiring resources from Msg 3. The time domain resources of PRACH are configured by the parameter prach-ConfigurationIndex specified in TS 38.211 Table 6.3.3.2-2 [2]. The parameter prach-ConfigurationIndex decides the PRACH periodicity. Configuring PRACH with different periodicity can meet different network requirements and performance criteria.

However, PRACH periodicity can only be changed semi-statically in current NR specification. Considering PRACH resources cannot be dynamically adjusted based on traffic load, the energy saving benefit cannot be acquired at the gNB side. Therefore, dynamic PRACH configuration that adjusts the PRACH periodicity and the distribution of ROs can be supported to reduce the network load. For example, using longer PRACH periodicity instead of short periods to reduce the network load and achieve the energy saving effect, as shown in Figure 1.

Figure 1. illustration of PRACH adaptation

Proposal 7: Support to configure PRACH periodicity dynamically and indicated with L1 signals, via DCI.

- Details on indication signalling and mechanism design should be further discussed.

What’s more, when the periodicity of PRACH is adapted dynamically, consider centralized ROs in anticipated high-demand scenarios and enlarge the PRACH periodicity in low-load scenarios to enable the network to enter longer sleep states can be considered without affecting UE access opportunities. For example, adopting a centralized ROs distribution pattern can minimize the network's active scanning period, which will bring more NES gain, as shown in Figure 2.

Figure 2. illustration of PRACH adaptation

Proposal 8: Support the non-uniformly transmission of PRACH with centralized ROs scheme adopted for PRACH adaptation in time-domain.

- Conclusions

In this contribution, we discuss NES and have following observations and proposals:

Observation 1: Options A3/B3 are subset of Option A2/A3, thus, if A2/B2 are support, A3/B3 will be supported naturally.

Proposal 1: Deprioritize Option A1, i.e., adaptation for CD-SSB of PCell (connected mode), for Rel-19 NES-capable UEs.

Proposal 2: At least support Option A2 and B2 for SSB adaptation for Rel-NES capable UEs.

Proposal 3: NCD-SSB adaptation for UE in connected mode in single carrier scenario can also be supported if A2 is supported.

Proposal 4: Support introducing SSB with longer periodicity for SSB adaptation.

Proposal 5: Support to enhance Cell DTX for SSB for connected mode UEs

- FFS: Management of DTX periods for connected mode UEs in high-mobility scenarios.

Proposal 6: Additional conditions should be considered for Case 2, i.e., time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs.

Proposal 7: Support to configure PRACH periodicity dynamically and indicated with L1 signals, via DCI.

- Details on indication signalling and mechanism design should be further discussed.

Proposal 8: Support the non-uniformly transmission of PRACH with centralized ROs scheme adopted for PRACH adaptation in time-domain.

- References

- RP-234065, New WID: New WID: Enhancements of network energy savings for NR, 3GPP TSG-RAN Meeting #102, Edinburgh, UK, December 11th – December 15th, 2023.

- “Chair notes RAN1 #117”, May, 2024.