3GPP TSG RAN WG1 #118			R1-2406276

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	9.11.2

Source:		Xiaomi

Title:	Discussion on the support of Redcap UE in NR-NTN

Document for:	Discussion

Introduction

In the 114th RAN plenary meeting, the working scope related to the support of Redcap UE with NR NTN operating in FR1-NTN bands is approved as follows [1]:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1]

- For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4]

- For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1]

  - Enhancements are targeted for the following HD collisions cases (cases 3 and 4):

    - Semi-statically configured DL reception collides with semi-statically configured UL transmission

    - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

  - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded.

  - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution.

In this contribution, we present the discussion on the potential RAN1 impact to support (e)Redcap UEs with NR NTN operating in FR1-NTN bands.

Discussion

A half-duplex UE (HD-UE) in paired spectrum is not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum. The collision issues between DL semi-static configured or dynamic scheduled transmissions and UL semi-static configured or dynamic scheduled transmissions should be resolved.

- Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission

For this case, the collision issue is basically resolved by relying on the gNB’s implementation. A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols.

- Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

For this case, the collision issue is also resolved by relying on the gNB’s implementation. It is specified that “A HD-UE does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols.”

Section 5.4.8	Timing Advance Reporting

RRC controls Timing Advance reporting by configuring the following parameters:

-	offsetThresholdTA;

-	timingAdvanceSR.

A Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured.

For the case 3 and 4, UE would treat them as error cases in the legacy design and the gNB can handle the collision cases by implementation. However, this needs further assessment due to the TA mismatch between actual TA used by UE and assumed TA at the gNB. The collision issue can be resolved by either gNB’s implementation or defining additional rule. In the legacy design, the UE’s TA reporting is optionally supported to assist the gNB to make configuration/scheduling decisions. It is specified in [2] that the value range for “offsetThresholdTA” can be configured from 0.5ms to 15ms which means that the TA mismatch value between actual TA used by UE and assumed TA at the gNB is expected to be less than 15ms. Even when the TA reporting is not supported, the TA mismatch value should be less than 10.3ms [3]. Thus, it seems not necessary to enhance the TA reporting resolve the collision issue.

Proposal 1: It is suggested to define additional rule to resolve the collision issue for case 3 and case 4.

Conclusions

In this contribution, we discuss the potential collision issues to support the HD-FDD Redcap in the NTN scenario. Based on our analysis, we have the following proposals:

Proposal 1: It is suggested to define additional rule to resolve the collision issue for case 3 and case 4.

References

- 3GPP RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Thales, CATT, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024

- 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”, V18.2.0 (2024-06)

- 3GPP TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN)”, V16.2.0 (2023-03)