____________________________________________________________________

3GPP TSG RAN WG1 Meeting #118 	                                                                    R1-2406452

Maastricht, NL, Aug 19th – Aug 23rd, 2024

Agenda item:	9.11.1

Source: 	IIT Kharagpur, CEWiT

Title: 	Discussion on NR NTN Downlink Coverage Enhancements

Document for:	Discussion & Decision

____________________________________________________________________

# Introduction

In RAN#102 meeting, a new Work Item on Non-Terrestrial Networks (NTN) for NR Phase 3 was approved [1]. A revised Work Item [2] is approved in RAN#103 meeting. Several objectives were discussed for NR phase 3 enhancements. This discussion document is targeted towards NR-NTN downlink coverage enhancements.

Detailed objectives of downlink coverage enhancements for NR NTN include:

- Offer optimized performance especially when addressing handset terminals (including smartphones with -5.5 dBi antenna gain) w.r.t. downlink coverage considering the NTN deployment constraints such as payload power limitation, large satellite foot print and limited feeder link bandwidth. DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the « nominal » EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth, while maximizing the number of beams that can be activated simultaneously, and ensuring that all user terminals can be served across the satellite foot print while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS. DL coverage enhancements can be considered at both Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design. System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

This contribution focuses on enhancements at the system level, specifically towards beamforming strategies.

## Discussion

The following agreements are made in RAN1#116bis [3] regarding the system level enhancements

Agreement

Confirm the Satellite phased-array antenna parameters for LEO 600km in FR1 defined in RAN1#116.

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65 for H 65 for V |
| Antenna element spacing | 0.667 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 400 elements (20 x 20) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | 4 dB |

Al least the above model is considered for SLS to ease the alignment between evaluation results. The model below can be optionally considered:

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | TR38.820 section 7.2.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| Antenna element spacing | 0.5 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 676 elements (26 x 26) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi (Note 1) |
| Steering loss at 30° elevation angle | 2.5 dB |

Note 1: The maximum antenna gain is determined by considering an overall array efficiency [of 50%.]

Agreement

For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1:

| LEO600km Set1-1 FR1 (i.e., S-band) |  |
| --- | --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size(Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 Digital beamforming |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set1-2 FR1 (i.e., S-band) |  |
| --- | --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set 1-3 FR1 (i.e., S-band) |  |
| --- | --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |  |
| --- | --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Agreement

RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level:

At least:

- CDF of the received SINR

- The dwell time and revisit time interval for each beam illumination across the coverage

- Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio

Other metrics may be reported such as

- CDF of the cell throughput

- CDF of user perceived throughput (UPT)

- CDF of Latency

- Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889)

For system level study based on analytical evaluation:

- N1 beam footprints are in state “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints are also served the necessary information for cell discovery and initial access

- N1 + N2 + N3 = “Total number of beam footprints “

- N1, N2, N3, X are to be reported by companies.

- Resource utilization obtained under the assumptions above is to be reported by companies.

- Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Agreement

For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following:

- -5.5 dBi antenna gain is assumed

- at least 2Rx are considered at the UE

  - 4Rx can be optionally considered and reported

Note: Redcap device is not considered in the scope of DL coverage study

Agreement

The following traffic models are considered for system level evaluation of DL coverage:

- FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time

- FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time

- VoIP can be considered in the evaluation.

It is up to company report which traffic model is used among the discussed traffic models in their evaluations.

- Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in Rel-18 NTN CE. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

This contribution focuses on enhancements at the system level.

## System level enhancements

System Level proposals are summarised in [7] for NR NTN Downlink Enhancements phase 3.

The maximum number of SSB blocks in an SSB burst as per current standard are

Table 1 Max number of SSB blocks in SSB burst

| Frequency | L (Max number of SSB blocks) |
| --- | --- |
| Sub 3GHz | 4 |
| 3 GHz – 7.125 GHz | 8 |
| mmW (FR2) | 64 |

The agreed total number of beam footprints and total number of simultaneously active beams are

|  | LEO600km Set1-1 FR1 (i.e., S-band) | LEO600km Set1-2 FR1 (i.e., S-band) | LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- | --- | --- | --- |
| Total number of beam footprints | 1058 | 1058 | 1058 |
| Total number of simultaneously active beams | 106 | 16 | 106 |

Proposed solutions to increase the coverage are

- Increase the SSB periodicity

- For Set 1-1, 1-3: With SSB periodicity of 40ms,80ms,160ms,320ms.

N2+N3 = 4 * (40/20) * 106 = 848; N1 = 1058-848 = 210;

Coverage (%) = (N2+N3)/(N1+N2+N3) = 848/1058 = 80.1%.

SSB periodicity of 80ms,160ms,320ms will make the coverage to be more than 100%

- For Set 1-2: With SSB periodicity of 320ms.

N2+N3 = 4 * (320/20) * 16 = 1024; N1 = 1058-1024 = 34;

Coverage (%) = (N2+N3)/(N1+N2+N3) = 1024/1058 = 96.79%

- Increase the number of SSB blocks in SSB Burst

- For Set 1-1, 1-3: With SSB periodicity of 20ms. Increase L in Table 1 for FR1 to 10

  - N2+N3 = 10 * (20/20) * 106 = 1060; N1 = 0;

Coverage (%) = (N2+N3)/(N1+N2+N3) = 1060/1058 = >100%

- For Set 1-2: With SSB periodicity of 20ms. Increase L in Table 1 for FR1 to 10

N2+N3 = 10 * (20/20) * 16 = 160; N1 = 1058-160=898;

Coverage (%) = (N2+N3)/(N1+N2+N3) = 160/1058 = 15.12%

Increasing the number of SSB blocks in a SSB burst to 10 alone will not enhance the coverage    for Set 1-2 with 20ms SSB periodicity

[3] Joint Increase of SSB periodicity & number of SSB blocks in SSB Burst

Increase the SSB periodicity to same values in Set 1-1/1-3 & Set 1-2 along with L to 8

- For Set 1-1, 1-3: With SSB periodicity of 160ms. Increase L in Table 1 for FR1 to 8

N2+N3 = 8 * (160/20) * 106 = 6784; N1 = 0;

Coverage (%) = (N2+N3)/(N1+N2+N3) = 6784/1058 = >100%

- For Set 1-2: With SSB periodicity of 160ms. Increase L in Table 1 for FR1 to 8

N2+N3 = 8* (160/20) * 16 = 1024; N1 =0;

Coverage (%) = (N2+N3)/(N1+N2+N3) = 1024/1058 = 96.79%

Observation 1: Increasing the number of SSB blocks in a SSB burst to twice the existing (8) along with SSB periodicity to 160ms will achieve the required coverage enhancement for the 3 sets of LEO-600 parameters in FR1.

Proposal 1: For NTN system level DL coverage enhancements, a joint approach of identical SSB blocks increase in SSB burst and unified SSB periodicity increase can be arrived instead of diversified periodicity increase for the 3 sets of LEO-600 parameters in FR1.

[4] Wider beam size for SSB transmission, narrow beam size for SIB/Common PDCCH/Data transmission

[8] reported improved Average SE (bps/Hz/ beam footprint) compared to fixed beam footprint for

wider beam size for SSB transmission, narrower beam size for SIB/Common PDCCH/Data transmission. However, the beam switching time/ reconfiguration time need to be considered.

Proposal 2: Beam switching time/ reconfiguration time need to be considered when dealing with dynamic beam sizes.

# Conclusion

In this contribution, we shared our views on improving NR-NTN DL coverage. Below are our observations and proposals:

Observation 1: Increasing the number of SSB blocks in a SSB burst to twice the existing (8) along with SSB periodicity to 160ms will achieve the required coverage enhancement for the 3 sets of LEO-600 parameters in FR1.

Proposal 1: For NTN system level DL coverage enhancements, a joint approach of identical SSB blocks increase in SSB burst and unified SSB periodicity increase can be arrived instead of diversified periodicity increase for the 3 sets of LEO-600 parameters in FR1.

Proposal 2: Beam switching time/ reconfiguration time need to be considered when dealing with dynamic beam sizes.

# References

[1] RP-233108, New WI: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#102, Dec. 2023.

[2] RP-240775, Revised WI: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#103, Mar. 2024.

[3] RP-2401993 FL Summary #3: NR-NTN downlink coverage enhancements, RAN1 #116bis, Apr, 2024.

[4] Chair notes RAN1#116bis, Apr. 2024.

[5] 3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks (Release 15)".

[6] 3GPP TR 38.821: “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”.

[7] R1-2404204: “FL Summary #3: NR-NTN downlink coverage enhancements” RAN1#117, May 2024.

[8] R1-2404194: “Discussions on NR-NTN downlink coverage enhancement” RAN1#117, May 2024.