3GPP TSG RAN WG1 #118									R1-2406474

Maastricht, Netherlands,  19-23 August 2024

Agenda Item 	:	9.4.2.1

Source 	:	Sony

Title 	:	General aspects of Ambient IoT physical layer

Document for 	:	Discussion

# Introduction

In RANP#103 Maastricht, the study item on Ambient IoT was updated [1], including the following objective:

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

Agreements from previous meetings are listed in the appendix.

In this document we discuss the following aspects of Ambient IoT:

- Waveform and modulation

- Multiple access

# Waveform and modulation

AIoT devices have very limited power consumption targets, in the range of 1 uW to 100s of uW. To allow for the use of low-power low-complexity designs at both transmit and receive sides, simple non-coherent modulation schemes such as on-off keying (OOK) are being considered.  In the following we discuss waveform and modulation schemes for both R2D and D2R.

  - R2D waveform and modulation

Considering the AIoT receiver architecture block diagram for device types 1 and 2, agreed during the RAN1#116 and RAN1#116bis meetings, OOK modulation can be considered as a candidate modulation for the AIoT R2D signal.

When transmitting the R2D signals, it is important that OOK modulation, that is received by a non-OFDM-based receiver, does not create any interference with other NR OFDM-based signals. This means the AIoT R2D signals needs to be compliant with the transmission principles of NR at gNBs and UEs, i.e., based on OFDM transmission. One aspect to study is whether/how to handle CP at transmitter/device/design. In the previous RAN1 meetings it is agreed to down-select between the following two methods.

Agreement For R2D CP handling for OFDM based OOK waveform: - For potential down-selection, study among the following candidate methods - Method Type 1: Removal of CP at device without specified transmit-side - FFS: How device determines the CP location - FFS: Impact on feasibility of device SFO - FFS: relation to M, if any - Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. - FFS: Whether/how to arrange that OOK chips have equal length after CP insertion - FFS: relation to M, if any - FFS: Detail of relationship to line code codewords - FFS: Impact on feasibility of device SFO - [Other method types are not precluded] - Study of the methods should include e.g.: - CP impact on R2D timing acquisition, and decoding & performance of PRDCH - Reader and device implementation complexities - Interference between R2D and NR DL/UL if in the same NR band - Spectrum efficiency

    - Method type 1: Removal of CP at device without transmit-side specification

The following was agreed for method type 1 at RAN1#117:

Agreement Study the following regarding CP location/length determination for Method Type 1: - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Companies are encouraged to clarify the CP removal method used and implementation aspects for the device - Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device - Companies should report the values of SFO, and SFO detection methods used in evaluations

The signal transmission is assigned with a certain bandwidth, e.g., 180 kHz, for the R2D link. The reader which conforms to the OFDM transmitter architecture generates the R2D signal through an OOK-4 scheme.

The following figure illustrates the block diagram of the R2D signal generation through OOK-4 process at the reader side. The time-domain OOK-modulated R2D signal is first segmented into a certain number of blocks, where each block is fed as the input of the DFT operation. Note that the number of samples per R2D signal block is denoted by N which needs to be configured consistent with the NR OFDM system.

Then the frequency-domain (FD) representation of the R2D signal is modified with the aim of fitting the FD R2D signal into the allocated frequency band while preserving the square wave-like shape in time domain as much as possible. The modification is realized by first shifting most of the energy to the corresponding frequency band that is assigned for R2D and then nulling out the frequency-domain values of the FD R2D signal in other frequency band.

The modified R2D signal is then transformed into time domain by an N-IDT operation. Depending on the system configuration, the reader may or may not need to perform the CP insertion to the signal.

The R2D signal generated through such method generally follows the OOK-4 principle and is therefore compatible to the NR OFDM system. Please note that there exist other R2D signal generation methods that also obey the OOK-4 principle.

The R2D signal transmission bandwidth could be assigned as 180 kHz, which means 12 subcarriers are assigned if the subcarrier spacing is 15 kHz. We note that this is true for the OOK signal transmission with higher number of OOK chips per OFDM symbol block. In the scenario where the data rate is relatively low, e.g., lower than 7 kbps, the time-domain representation of the R2D signal is flat (either ‘ON’ or ‘OFF’ state) and the resulting FD R2D signal can be sufficiently represented by a single subcarrier. Therefore, the R2D signal transmission bandwidth does not need to be as large as 180kHz. Moreover, the OOK-4 scheme is simplified into OOK-1 scheme under this condition.

Figure 1. Block diagram of R2D signal generation through OOK-4 scheme.

Benefiting from the flexibility of signal approximation through OOK-4 scheme, we propose to modify the time-domain R2D signal by introducing a carefully designed tail-CP pattern, such that the AIoT device can recognize the pattern and conducts its clock acquisition.

Our proposal lies in the insertion of a carefully designed tail pattern to each time-domain R2D signal, as illustrated in Figure 2. The tail pattern, e.g., an Off-On-Off pattern that spans a certain number of samples, along with the main data part are first concatenated and fed as the input of the N-FFT operation. This would help yield the frequency domain representation. With some modification/rearranging operations, the time-domain R2D signal is generated. It is visualised that the tail pattern is incorporated in the generated R2D signal.

Note that the symbol duration of the final R2D signal including the tail pattern remains the same as that of an R2D signal without tail pattern insertion. This also means that the On duration of the data sequence is compressed proportionally.

Figure 2. Proposed tail pattern insertion and corresponding signal generation through OOK-4 scheme.

An example of the signal structure according to this proposal is shown in the figure below. It shows a single OFDM symbol of the R2D signal. The main portion of the signal is the “baseline R2D signal”. In this example, the baseline R2D signal is a Manchester-encoded signal. A tail portion with a known sequence (in this case an upward pulse) is appended to the baseline R2D signal. According to the cyclic prefix addition rules, this tail portion’s duration is equal to the CP duration and is copied as the cyclic prefix of the whole OFDM symbol. In this example, the CP-tail portions of the OFDM symbol contain a known pattern that can be used by the tag for demodulation purposes.

Figure 3. An example of the time-domain R2D signal structure with CP-tail pattern inserted

The digital baseband signal processing conducted by the AIoT devices generally follows the block diagram illustrated in Figure 4.

Pattern determination

Upon the reception of the R2D signal that incorporates the inserted CP-tail pattern, the AIoT device may conduct the correlation calculation to determine the location of CP and the tail. In one example, the correlation method is an autocorrelation method which uses the CP sampled according to the tag’s clock, as the typical sequence to calculate the sequence-wise correlation with the rest of the sampled sequence. Since the highest value of autocorrelation occurs when the CP and tail are aligned in the autocorrelation process, the tag then knows where the tail portion is. Based on the detected CP-tail, the tag can extract the signal remaining in the middle of the OFDM symbol as the data-carrying signal.

Figure 4. Block diagram of the digital baseband processing

On the other hand, the correlation calculation method can be a correlation of the received sequence with the known transmitted sequence.

SFO Estimation & Clock Frequency Correction

Given that the tag successfully detects the CP-tail of a single OFDM symbol, it can estimate the time drift caused by the SFO by comparing the counted number of samples of the data-carrying sequence with the reference number of samples. This counting operation is equivalent to counting the number of samples between the CP and the tail and comparing the counted number with the reference number of samples.

The data-carrying sequence length can be straightforwardly derived based on the counted index of the largest autocorrelation value and the length of CP. The reference number of samples could be quantified based on each OOK data chip length and the total number of OOK data chips incorporated in a single OFDM symbol. Such knowledge is required by the tag and needs to be either predefined by the specifications or transmitted by the reader through control signalling, e.g., the preamble signal or system information.

Note that for the tag to successfully estimate and correct the SFO, the reader needs to be aware of essential information about the R2D signal, including the chip duration of the data-carrying portion of the R2D signal (the “baseline R2D signal”), the CP duration and the number of chips in the data-carrying portion. For example, the reference number of samples of the data-carrying portion is 238 samples; this knowledge is acquired by the tag. Based on the CP-tail detection process, the tag has counted the actual number of sampled data-carrying sequence as 240. This means that the clock frequency possessed by the tag is (240-238)/238 = 0.84% higher than the reference frequency. Then the tag reduces its clock frequency by 0.84% based on the estimate. Note that the tag can oversample the signal to get more accurate SFO estimation and thus improved clock correction.

Design flexibility

If the tag failed to detect at the CP-tail pattern in the current OFDM symbol, it keeps its clock frequency unchanged. We note that this could be caused by the poor SNR condition of the received signal. Thanks to the OOK-4 scheme, one can increase the amplitude of the CP-tail pattern to yield more accurate CP-tail determination and SFO estimation performance. Correspondingly, the amplitude of the CP-tail pattern can also be reduced to improve the transmission energy efficiency if the AIoT device is synchronised.

Note that only one possible pattern of the CP-tail part is considered above, although other patterns are not precluded.

Proposal 1: For Method Type 1 of CP handling, within an OFDM symbol, the R2D signal consists of a CP, a data portion and a tail portion. The tail portion contains a known signal shape and is copied to the CP (as per legacy OFDM transmission). The Ambient IoT device can use the CP-tail pair to aid in clock acquisition.

    - Method type 2:  CP handling at transmission side while maintaining orthogonality

The CP removal can also be handled at the transmitter and through the R2D signal design. When handling CP, to allow for low-complexity detection, it is important not only to keep the total length of the resulting signal the same as the original target R2D AIoT signal, but also to maintain a regular R2D AIoT signal bit period.

Observation 1 - To allow for low-complexity detection, it is important both to keep the total length of the resulting signal the same as the original target R2D AIoT signal and to maintain a regular R2D AIoT signal bit period.

In RAN1#117, the following was agreed.

Agreement Study the following options regarding subcarrier orthogonality for Method Type 2: - Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) - Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same - FFS: whether this alternative applies if CP length is longer than the chip duration - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP - Other potential methods are not precluded - Alt 2: Method Type 2 does not retain subcarrier orthogonality - Proponents to bring further details to RAN1#118 - Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24. - Companies should report the values of SFO, and SFO detection methods used in evaluations

In the following we discuss two schemes, where both schemes maintain subcarrier orthogonality:

- a potential solution ensuring the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n, as in Alt1-1 and Alt 1-2.

- our proposed solution - which is an alternative to Alt 1-1 and Alt 1-2 method type 2 and does not necessarily follow preventing false rising/falling edge when inserting CP.

In our discussion we highlight the advantage and drawback of each scheme as well as the relation of M, the number of bits per OFDM symbol. In both schemes, to multiplex the AIoT OOK-based R2D signal, a similar scheme to the OOK-4 transmission technique, used for LP-WUS transmission, can be adopted. We assume the lengths of the AIoT R2D signals (in PRDCH) are longer than an OFDM symbol duration, but the bit lengths are shorter than one OFDM symbol.

Solution preventing false rising/falling edge when inserting CP – We first create sections of the target R2D AIoT signal. To allow the total resulting signal length after CP insertion remain the same, in each section the total number of bits/chips per OFDM symbol, M, times the number of samples per bit/chip, K, needs to match the OFDM symbol duration including CP,

M . K=NIFFT+NCP, (1)

To prevent false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n, two conditions need to be fulfilled.

- The start and the end information bit/chip in one OFDM symbol need to have the same status. For instance, assuming M=4, only bit sequence of "0b1b20" and "1b1b21"  are basically possible. This means that a parity bit carrying the same status as the first bit needs to be inserted.

- Additionally, the bit/chip length needs to be longer than the CP length, this translates to

K>NCP. (2)

Fulfilling the above conditions results in a restriction in the number of bits that can be used per OFDM symbol and thereby restriction in the total data rate. Assuming SCS of 15 kHz, NIFFT of 2048 and NCP of 144, K at least needs to be 144 samples. This means with an OOK modulation M will be limited to 8, 4, 2 and 1. If we apply Manchester coding, the number of samples per bit K needs to at least be 288 samples which changes M being restricted to 4, 2 and 1.

Another important drawback of this approach is that we lose on the correlation properties of the signal due to insertion of the parity bit in each OFDM symbol. Sequences with auto- and cross-correlation properties are important particularly when we design preamble sequences as they are typically used for synchronization purposes and need to be selected from good correlation probabilities.

Observation 2 – An important drawback of Alt1-1 and Alt1-2 is that the correlation properties of the sequences cannot be kept due to parity bit insertion making this method not suitable for mapping sequences where their correlation properties need to be maintained.

In the following we propose a solution where we can maintain the properties of the AIoT R2D signal.

Our proposed solution –A version of OOK-4 based R2D transmission is illustrated in Figure 5. The first block (A) is a sectioning block creating sections of the target R2D AIoT signal excluding CPs and is only used if the length of the R2D AIoT signal becomes longer than one OFDM symbol. The shaping block (B) then shapes each R2D AIoT signal section so that the resulting signal as output from the OFDM modulator becomes as close to the target AIoT R2D signal (OOK) as possible using a least-square approach. Step (C) to (D) adds CPs to maintain orthogonality in channels with time dispersion.

Figure 5 – Block diagram of modified OFDM transmitter (for OOK-4 transmission mechanism)

Figure 6 illustrates the resulting signals after each block. No line-coding coding is applied in this example. By creating the R2D AIoT signal in this manner, not only the total length of the resulting signal remains the same as the original target R2D AIoT signal, but we also maintain a regular R2D AIoT signal bit period. This allows for simple low-power detection, without complex DBB processing or synchronization with the OFDM structure. The transmitted AIoT R2D signal is entirely orthogonal to the other signals transmitted in the same OFDM symbols and does not create any interference.

Figure 6 - Illustration of input/output signals of each block in the transmitter of Fig. 1, using a simple example.

An important advantage of our proposed approach over method type 2 is that we can choose the entire OFDM symbol length effectively, this is particularly if we have a signal that spans over more than one OFDM signal and the entire bit-sequence is detected with straight forward correlation rather than one with periodic shift of sampling instant at each CP location. This means that the number of bits per OFDM symbol, M, is not limited to a certain value and it can be freely decided based on AIoT DL bit rate and according to channel conditions, e.g., delay spread and Doppler and the tolerable time/frequency errors by the AIoT DL receiver.

However, the drawback with our proposed approach is an error floor resulting from enforcing the CP property of the signal to avoid interference with other NR transmission, i.e., removing of CP portions of the signals in step A to B). To compensate for this, two potential techniques can be used: (i) the re-built version of DL AIoT signal, signal D, can be used as the known signal when correlating with the received signal, (ii) low complexity error correcting codes may be used when decoding the received signal. This type of signal generation is particularly beneficial when generating preamble signals that are used for clock-acquisition and synchronization purposes. Unlike Alt-1 and Alt-2, by generating the signal with our proposed scheme, the correlation properties of the signal can be kept, an important property necessary when designing signal/sequences for synchronization purposes.

Observation 3 – With our proposed method, we can choose the entire OFDM symbol length effectively, this is particularly if we have a signal that spans over more than one OFDM signal and the entire bit-sequence is detected with straight forward correlation rather than one with periodic shift of sampling instant at each CP location.

Proposal 2 – For Method Type 2 of CP handling, to maintain a regular R2D AIoT signal bit period and to allow for low-power detection, the CPs portion of the DL AIoT signal can be excluded before it is fed to the OFDM-based transmitter.

Proposal 3 – For Method Type 2 of CP handling, support CP handing at OFDM transmitter where the correlation properties of the AIoT R2D signal can be kept.

Using the above technique for DL AIoT, both OFDM-CP and DFT-S-OFDM transmitters can be used to multiplex the OOK-based DL AIoT signal. Considering gNBs and UEs used are AIoT readers:

- For gNBs and UEs transmitting based on OFDM-CP, two additional blocks, i.e., the sectioning block excluding CPs and shaping/least square block, need to be added.

- For UEs transmitting based on DFT-S-OFDM, only an extra block for sectioning and excluding CPs is needed as the DFT block can be used as the shaping/least-square block.

Proposal 4 – Both CP-OFDM and DFT-S-OFDM transmitters can be used for the AIoT R2D signal transmission.

  - D2R waveform and modulation

The following agreement has been made on device type in RAN1#116.

Agreement For the purpose of the study, RAN1 uses the following terminologies: - Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

A passive device, as in “device 1” and “device 2a”, cannot generate its own RF signal and the device’s UL transmission is backscattered on a carrier wave provided externally. Architectural block diagrams of the two devices operating based on backscattering have also been agreed during RAN1#116. An impedance switching approach is used as a backscatter modulator. An example backscatter modulator is shown in Figure 7. In the following, we give examples on how to realize OOK and BFSK type modulations using the switching pattern in the backscatter modulator.

Figure 7 – Illustration of a simple backscatter modulator with two states.

OOK modulation – In OOK, the data is represented by the presence / absence of the backscattered carrier wave. The presence of the backscattered signal represents ‘1’ and the absence of the backscattered signal represents ‘0’. The frequency of the switching pattern determines the length of each OOK bit. Figure 8 illustrates a switching pattern for transmission of “010010110101010100111”.

Figure 8 - Switching pattern to generate an example OOK signal.

BFSK modulation – BFSK scheme modulates the data by varying the frequency of switching pattern. The backscatter modulator toggles between two different impedances with frequencies f1 or f2; this is equivalent to the generation of two square waves with frequency f1 and f2 representing ‘1’ and ’0’ respectively. Figure 9 illustrates a switching pattern for transmission of “0100”.

Figure 9 - Switching pattern to generate an example BFSK signal.

Similar to the above, by adjusting the switching pattern and the load, we can generate backscatter signals based on other modulation schemes such as PPM and PWM.

Proposal 5 – RAN1 to study on the switching pattern when realizing low-power low-complexity modulations.

As an active device, Device 2b can generate its own UL transmission. Having the same power consumption budget as device 2a, conventional circuity can be used to modulate based on OOK, BFSK or other waveforms.

# Multiple Access

A non-backscattering Type 2b device can be scheduled to transmit within certain frequency resources. It can then tune its oscillator or configure its signal processing to transmit at that scheduled frequency. Hence, these devices can be readily frequency division multiplexed.

A Type 1 passive device or Type 2a backscattering device can shift the frequency of its backscattered transmission by rapidly switching the load impedance of its antenna in order to frequency modulate its reflection coefficient and hence frequency shift its backscattered transmission, as discussed in section 2.2. The rate at which the load impedance can be switched needs to be investigated as this will affect the range of frequency division multiplexing that is possible.

Proposal 6: Frequency division multiplexing is studied. The amount of frequency division multiplexing depends on the maximum possible switching rate of the load impedance.

The frequency accuracy of the device clock is dependent on the device type. Ranges for the frequency accuracy for evaluation were captured in [5]. The evaluation assumptions suggest that (1) the SFO of passive devices is significantly greater than that of active devices and (2) the frequency error of the active device can be reduced through synchronization and calibration procedures. We hence assume the following clock accuracies:

| Device type | SFO | After calibration |
| --- | --- | --- |
| 1 | up to 105 ppm | 104 / 105 ppm |
| 2a | up to 104 ppm | 104 / 105 ppm Note: depending on SYNC / preamble design, this value might be reduced |
| 2b (active) | up to 103 | 20ppm [5] |

The amount of multiplexing depends on the clock accuracy. As the clock accuracy of the device worsens, larger guard bands between the frequency division multiplexed devices will be required (see also our discussion in [6]). The sizes of the guard bands increase as the “subcarrier frequency” of the D2R transmission increases for passive devices. For example, considering a passive device with an SFO of 105ppm, a passive device operating at a switching rate of 100kHz requires a guard band of 10kHz (or 20kHz accounting for the accuracy of a device operating in a neighbouring sub-band) while a passive device operating at a switching rate of 1MHz would require a guard band of 100kHz (or 200kHz accounting for a neighbouring device). An active device operating at a carrier frequency of 1GHz would only require a guard band of 20kHz, assuming a frequency accuracy of 20ppm after calibration. Given that active devices have greater frequency stability, it is preferable if those devices can use frequency resources that are further from the centre of the A-IoT system bandwidth. Guard band requirements for passive and active devices are illustrated in Figure 10. It is also advantageous for passive devices to occupy frequencies close to the Ambient IoT carrier frequency (or CW frequency) since the power consumption of passive devices increases with switching rate.

Figure 10 – Guard band requirements for passive and active devices. Passive devices need larger guard bands in general and their transmit frequencies should be closer to the A-IoT carrier frequency

Observation 4: The device clock accuracy impacts the size of the guard band required between frequency division multiplexed devices. The guard band size increases for higher switching frequencies for passive devices.

Observation 5: The frequency accuracy of active devices is better than that of passive devices. The frequency error of active devices is a function of the carrier frequency.

Observation 6: The power consumption of passive devices increases with the rate of load impedance switching.

Proposal 7: The reader should be able to assign frequency resources as a function of device type. E.g. passive devices are assigned lower frequency resource and active devices are assigned higher frequency resources.

# Conclusion

This document has considered General aspects of physical layer design for Ambient IoT. The following observations and proposals are made:

Observation 1 - To allow for low-complexity detection, it is important both to keep the total length of the resulting signal the same as the original target R2D AIoT signal and to maintain a regular R2D AIoT signal bit period.

Observation 2 – An important drawback of Alt1-1 and Alt1-2 is that the correlation properties of the sequences cannot be kept due to parity bit insertion making this method not suitable for mapping sequences where their correlation properties need to be maintained.

Observation 3 – With our proposed method, we can choose the entire OFDM symbol length effectively, this is particularly if we have a signal that spans over more than one OFDM signal and the entire bit-sequence is detected with straight forward correlation rather than one with periodic shift of sampling instant at each CP location.

Observation 4: The device clock accuracy impacts the size of the guard band required between frequency division multiplexed devices. The guard band size increases for higher switching frequencies for passive devices.

Observation 5: The frequency accuracy of active devices is better than that of passive devices. The frequency error of active devices is a function of the carrier frequency.

Observation 6: The power consumption of passive devices increases with the rate of load impedance switching.

Proposal 1: For Method Type 1 of CP handling, within an OFDM symbol, the R2D signal consists of a CP, a data portion and a tail portion. The tail portion contains a known signal shape and is copied to the CP (as per legacy OFDM transmission). The Ambient IoT device can use the CP-tail pair to aid in clock acquisition.

Proposal 2 – For Method Type 2 of CP handling, to maintain a regular R2D AIoT signal bit period and to allow for low-power detection, the CPs portion of the DL AIoT signal can be excluded before it is fed to the OFDM-based transmitter.

Proposal 3 – For Method Type 2 of CP handling, support CP handing at OFDM transmitter where the correlation properties of the AIoT R2D signal can be kept.

Proposal 4 – Both CP-OFDM and DFT-S-OFDM transmitters can be used for the AIoT R2D signal transmission.

Proposal 5 – RAN1 to study on the switching pattern when realizing low-power low-complexity modulations.

Proposal 6: Frequency division multiplexing is studied. The amount of frequency division multiplexing depends on the maximum possible switching rate of the load impedance.

Proposal 7: The reader should be able to assign frequency resources as a function of device type. E.g. passive devices are assigned lower frequency resource and active devices are assigned higher frequency resources.

References

[1]	RP-240826. “Study on solutions for Ambient IoT (Internet of Things) in NR”. RANP#103. Maastricht, Netherlands. March 2024.

[2]	Chair notes. RAN1#116. Athens, Greece, February 2024.

[3]	Chair notes. RAN1#116bis. Changsha, Hunan Province, China, April 2024.

[4]	Chair notes. RAN1#117. Fukuoka, Japan. May 2024.

[5]	R1-2405772. “Email discussion on Ambient IoT evaluation assumptions”. Moderator (CMCC). RAN1#117. Fukuoka, Japan. May 2024.

[6]	R1-2407175. “Ambient IoT device architecture”. Sony. RAN1#118. Maastricht, Netherlands. August 2024.

Appendix: Agreements from previous meetings

In RAN1 #116 [2], the following were agreed.

Agreement A-IoT DL study includes an OFDM-based waveform from A-IoT R2D (reader-to-device) perspective. - Depending on what modulation(s) are decided to be studied: - Study whether/how to handle CP at transmitter/device/design - Study other characteristics of the OFDM waveform, e.g.: - CP-OFDM - DFT-s-OFDM - Etc. - The type of OFDM waveform is transparent to A-IoT device. Other waveforms from DL transmitter’s perspective can be proposed, and further discussion will consider whether or not they are included in the study. Agreement A-IoT DL study includes OOK from DL transmitter’s perspective. - For an OFDM waveform, assume OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869. - FFS value(s) of M. - FFS: Any changes needed from the definitions in TR 38.869. - FFS: Exact definition of chip - If other DL waveforms are included, further elaboration of the transmitter’s OOK generation would be needed. Agreement For R2D, line codes studied are: Manchester encoding and pulse-interval encoding (PIE). - FFS: Mapping(s) from bit(s) to line-code codewords - FFS: Time domain definition of e.g., chips and relation to OFDM symbols, resource allocation unit, etc. Agreement Regarding FEC, R2D with no forward error-correction code (FEC) is studied as baseline. - Evaluations would be by comparison to this baseline Agreement R2D study assumes use of CRC. FFS which CRC generator polynomial(s) are assumed, and if any cases are included with no CRC. - FFS: Association, if any, between down-selected CRC(s) and message size, considering at least false-alarm rate target Agreement D2R study assumes use of CRC. FFS which CRC generator polynomial(s) are assumed, and if any cases are included with no CRC. - FFS: Association, if any, between down-selected CRC(s) and message size, considering at least false-alarm rate target Agreement At least the following bandwidths for R2D are defined for the purpose of the study: - Transmission bandwidth, Btx,R2D from a Reader perspective: The frequency resources used for transmitting R2D - Occupied bandwidth, Bocc,R2D from a Reader perspective: The frequency resources used for transmitting R2D, and potential guard band - Bocc,R2D ≥ Btx,R2D - FFS: Further constraint(s) e.g. Bocc,R2D = Btx,R2D. - Possible values of each bandwidth are FFS

Further, in RAN1#116bis [3], the following were agreed.

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS. Agreement The following bandwidths for D2R are defined for the purpose of the study: - Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device. - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined - Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R - Note: this guard band is not for coexistence with NR/LTE - If/how to define guard band for coexistence between A-IoT D2R and NR/LTE is up to RAN4. - Bocc,D2R >= Btx,D2R - Possible values of each bandwidth are FFS Agreement For D2R, study: Manchester encoding, FM0 encoding, Miller encoding, no line coding. - FFS: Mapping(s) from bit(s) to line-code codewords - FFS: How to achieve small frequency shift in baseband and/or FDM(A) among devices - Aspects to study include: - Spectrum shape - Complexity - Power consumption - BER, BLER - Resilience to SFO - If there is any relation to CFO Agreement A-IoT D2R study of FEC includes at least convolutional codes. - Comparisons are encouraged to compare to the case of no FEC - FFS details of convolutional codes, such as polynomial(s), shift-register termination, etc. - FFS if other FEC candidates/methods will be studied. Agreement Study - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PRDCH - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PDRCH - FFS: details when different CRC lengths or no CRC may be used - FFS: other 6 bits and 16 bits CRC with different polynomials than from TS 38.212 Agreement Study D2R transmission in the physical layer using repetition - Note: Discussions regarding higher-layer repetitions are up to RAN2. Agreement R2D study includes subcarrier spacing of 15 kHz, from the reader perspective, for OFDM-based waveform. - Inclusion in the study of subcarrier spacing of 30 kHz is FFS. Agreement For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among: - Alt 1: Including 180 kHz, 360 kHz, and FFS other values - Alt 2: Integer multiple(s) of 180 kHz (FFS: what integer(s)) - Alt 3: Integer multiple(s) of the subcarrier spacing (FFS: what integer(s)) Agreement For R2D CP handling for OFDM based OOK waveform: - For potential down-selection, study among the following candidate methods - Method Type 1: Removal of CP at device without specified transmit-side - FFS: How device determines the CP location - FFS: Impact on feasibility of device SFO - FFS: relation to M, if any - Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. - FFS: Whether/how to arrange that OOK chips have equal length after CP insertion - FFS: relation to M, if any - FFS: Detail of relationship to line code codewords - FFS: Impact on feasibility of device SFO - [Other method types are not precluded] - Study of the methods should include e.g.: - CP impact on R2D timing acquisition, and decoding & performance of PRDCH - Reader and device implementation complexities - Interference between R2D and NR DL/UL if in the same NR band - Spectrum efficiency Agreement Study for all devices the following for D2R baseband modulation, for potential down-selection: - OOK - Binary PSK - Binary FSK - Strive to identify one variant of Binary FSK to study further

Further, in RAN1#117 [4], the following were agreed.

Agreement Study the following regarding CP location/length determination for Method Type 1: - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Companies are encouraged to clarify the CP removal method used and implementation aspects for the device - Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement Study the following options regarding subcarrier orthogonality for Method Type 2: - Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) - Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same - FFS: whether this alternative applies if CP length is longer than the chip duration - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP - Other potential methods are not precluded - Alt 2: Method Type 2 does not retain subcarrier orthogonality - Proponents to bring further details to RAN1#118 - Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24. - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times Agreement For D2R, study at least block-level and bit-level repetition type 1 and type 2. Agreement For R2D evaluation purposes, the R2D waveform for DFT-s-OFDM is generated as follows: - The time domain OOK signal is the M chips of one OFDM symbol. - A chip is represented (e.g. upsampled) by L samples - Companies to report L - An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal. - Companies to report N’, e.g. N’=128 or equal to X - Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D. - Companies report how to map and report X - An N-points IDFT is performed to obtain the time domain signal. - Companies to report N, and how value was selected Note: companies report whether/how CP samples are added. Agreement The study assumes the following bit to chip mapping for Manchester encoding: - bit 0→chips{10}, bit 1→chips{01} - FFS: Variant of the above for CP handling