3GPP TSG RAN WG1 #118		R1-2406841

Maastricht, NL, August 19th – 23rd, 2024

Source: 	Apple Inc.

Title:                     Frame structure and timing aspects for Ambient IoT

Agenda item:	9.4.2.2

Document for:	Discussion and Decision

# 1 Introduction

A new study item on solutions for Ambient IoT (Internet of Things) was approved in [1]. In the RAN1 117 meeting, the following was agreed for frame structure, random access and timing relation aspects [2]:

Agreement

Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

Agreement

Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

Conclusion

RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2.

Agreement

Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it:

- Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max].

  - FFS: maximum time is common or different for different A-IoT devices

  - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)

- Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min

  - FFS the maximum value(s) for TR2D

In this contribution, we discuss the open issues of frame structure, the necessity of synchronization operation, random access procedure and the timing relation for Ambient IOT devices based on the latest progress.

# 2.  Discussions

## 2.1 Frame structure

For DL transmission, it was agreed to align NR and Ambient IoT at least on a symbol boundary to simplify the gNB imiplementation e.g., single FFT etc. However, it remains FFS on UL transmission for Ambient IOT devices.

Due to low-cost and low-complexity, the initial SFO of an Ambient IoT device is relaxed to be as large as ~105 ppm and the exact value depends on the oscillator implementation. Due to the significant ~105  ppm SFO target and extreme low power consumption target (~1µW) for low-CAT device, it is quite challenging and costly in terms of power consumption to align Uplink timing between NW and Ambient IoT on a symbol or slot boundary.

We therefore propose:

Proposal 1:  For UL transmission, no need of timing alignment on symbol or slot boundary between NR and A-IOT, due to time drift at device

The following was agreed in RAN1 116 meeting [3]:

Agreement At least the following time domain frame structure is studied for A-IoT R2D and D2R transmission. - For R2D transmission, - A R2D timing acquisition signal (e.g. R2D preamble) is included at least for timing acquisition and for indicating the start of the R2D transmission in time domain. - For D2R transmission, - A D2R timing acquisition signal (e.g. D2R preamble) is included at least for timing acquisition and for indicating the start of the D2R transmission in time domain. - FFS other necessary component(s), e.g. midamble, postamble, periodic sync signal, control fields, guard period

In the RFID spec, the reference time interval (Tari) for DL is implicitly indicated by the preamble preceding a DL communicaiton.  For R2D transmission, a R2D timing acqusition signal is always included and can also provide the chip length for the following PRDCH transmission.

Proposal 2: The R2D timing acquisition part preceding the PRDSCH indicates the chip length used for the following PRDCH transmission.

For the D2R transmissioin, three options were identified for further study during RAN1 116bis meeting [4]:

How to indicate/determine the D2R chip length? - Option 1: by R2D preamble - Option 2: by R2D control information - Option 3: by D2R transmission bandwidth, e.g. chip length =2double sideband transmission bandwidth

Compared to Opt.1, Opt.2 enhances the scheduling flexibility and enables the configuration of varied chip lengths for different device types, even when they share the same R2D preamble seqeunce. For instance, a shorter chip length can be used for advance Ambident IoT devices i.e., Type 2a or 2b to achieve higher data rates. In our view, Opt.2 still opens door for Opt.3, depending on how the control information is defined. If the bandwidth of DSB signal is specified, Opt.2 is equivalent to Opt.3.

Therefore, we propose:

Proposal 3: The chip length of PDRCH is explicitly indicated by R2D control information.

## 2.2 Scheduling and timing relations

In RAN1 116 meeting, the following was agreed [5]:

Agreement For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device. - The study should consider at least following aspects - Implementation restrictions for the existing BS/UE - [Processing time is common or different for different A-IoT devices] - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)] - FFS other timing aspects

The following was agreed in RAN1 117 meeting for further study [2]:

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

The TR2D_min is primarily determined by the processing capabilities of A-IOT hardware, making it feasible to define different TR2D_min values for various A-IoT device. The TR2D_max in Opt.1 is analogous to the time alignment error (TAE) defined in RAN4 specification, which is induced by A-IOT SFO. This helps determine when the receiver should stop monitoring the reponse, thereby minimizing the power consumption. Opt.2 intends to offer better flexibility and efficient scheduling by supporting TDMed PDRCH transmissions from multiple devices using a single R2D command. However, there are significant concerns on the resoruce overehead in Opt.2 due to substantial SFO. Since timing drift is a cumulative effect of the SFO over time, the overhead and feasibility of Opt.2 depends on several factors, such as a time gap between the R2D command and the last PDRCH in the intended use case, and a sustainable timing accuracy of sampling clock after adjusting it based on the received R2D command. These factors need to be clarified first before making decision on Opt.2.

Proposal 4: Support Opt.1 as baseline (i.e., defining a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it). FFS on Opt.2.

## 2.3 Random access for Inventory procedure

The following was agreed in RAN2 126 meeting [6]:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

The coverage targetted by Ambient device is up to 50m in indoor deplyement scenario and the one way shorter end-to-end maximum latency including query/triggering time targets to 1 second.  As discussed earlier, one drawback of anti-collision procotol used in Query procedure for RFID is the high collision rate in case of dense use case and eventually causes wastage valuable enery and large latency as device needs to wait for the next Query round. To achive the design goal of extended coverage and low latency, it is imporant to enhance the anti-collision protocol for Ambient IoT devices. The collision rate of Query responses can be reduced by supporting multiple frequency points for high-CAT device to transmit Query responses (i.e., PRACHs signal), regardless of backscattering-based or UE-genrated signals. In addition, the fequency-shifted signal can improve the detection performance and link budget of Query reponse due to shifting the backscatter signal away from the incoming carrier wave frequency to mitigate the interfence at the gNB receiver. The frequency-shifted operation will cause extra power consumption. While, considering the benefit of increased Query response capacity and the reduced latency for Query procedure, support of ‘frequency-shifted’ backscattering signal for high-CAT devices is imporant to explorit the signficantly higher peak power i.e., a few hundred µW.

Proposal 5: For Msg1 and Msg3 transmission, support parallel FDMed resources by frequency shifting operation to cope with multiplexing a high number of A-IoT devices

## 2.4 RF Energy havest impact on device availablilty

The following was agreed in RAN 103 meeting [7]:

Agreement The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary

In RAN1 117 meeting, concensus was reached on the agreement below [2]:

Agreement For study purpose, assume that A-IoT device has a single antenna for both communication (tx/rx) and RF energy harvesting purposes. Conclusion RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2.

In RAN plenary 104 meeting, there was a heated debate on how to model the charging and discharging process to address the impact of device unavailability on the latency of inventory procedure due to energy havesting operation. More specifically, different power states were brought up to model the varying level of energy usage under various device behaviors. To reflect the dynamic nature of power consumption for Ambient IoT devices and design more effective charging management strategies, the following power states can be considered to faciliate the discussions:

- ‘ON’ state: device is assumed to support both transmission and reception operations.

- ‘Sleep’ state:

  - Maintaining a memory content from ‘ON’ state.

  - Maintaining a timer that meets a certain timing accuracy requirement.

  - Not support transmission and reception.

- RF eneregy havesting is conducted at least during ‘Sleep’ state.

Figure 1: A simplified power model for study of RF energy havesting and device unaviliability

Proposal 6: Define at least two power states for the study of charging and discharging period in RF energy havesting operation

- ‘ON’ state: support transmission/reception.

- ‘Sleep’ state: maintain a memory and timer to count the samples.

- Energy havesting is conducted at least during ‘Sleep’ State.

In general, the time required for RF energy harvesting is determined by several key factors, including the capacitor value, the receiver RF signal strength and RF energy conversion rate. Table 1 provides two examples of energy harvesting times for Ambient IOT device #1 and #2 at different RF signal strengths:

Table 1: Device charging time calculation for different received RF signal strengths

|  | Received RF signal strength | Energy efficicy rate | Energy havest target (uJ) | Charging time (second) |
| --- | --- | --- | --- | --- |
| Device #1 | -20dBm | 10% | 0.5 | 0.5 |
| Device #2 | -32dBm | 10% |  | ~8 |

The sustainable time varies based on power state. For instance, a longer sustainable time is expected for ‘Sleep’ state due to less power consumption compared to ‘ON’ state. Additionally, different device types have various sustainable duration for ‘ON’ state due to different ‘Tx/Rx’ power consumptions (i.e., 1uW for device 1 and a few 100uW for device 2). Due to the signficant varation in unavailability time for devices caused by RF energy havesting operations as analysized above, it is essential to design the inventory procedure to ensure the device availiability times align with the transmission occasions of inventory commands. This alignement minimizes inventory latency and NW power consumption.

A duty-cycle based solution has been proposed to achieve the ‘alignment’ goal between the reader and devices. Specifically, each duty-cycle is divided into two parts, one is ‘monitoring’ part and the other is ‘nonActive’ part. During a ‘monitoring’ period, the inventory command transmission is allowed to be transmitted from reader, and Ambient IOT devices are expected to monitor if they wish to participate in the potenital inventory processes (i.e., ‘ON’ period for Ambient IOT devices). If no inventory command is detected during monitoring period, Ambient IOT devices enter ‘Sleep’ state for RF energy havesting, with a running time detemining when to wake up for the next ‘monitoring’ period in next duty cycle.

Proposal 7: Study a duty-cycle based approach where each cycle include ‘monitoring period’ and ‘nonActive period’, where reader tranmits paging command within ‘monitoring’ period to trigger inventory procedure.

# 3. Conclusion

In this contribution, we have presented our views on the frame structure, synchronization, random access and timing relation aspects. We proposed the following:

Proposal 1:  For UL transmission, no need of timing alignment on symbol or slot boundary between NR and A-IOT, due to time drift at device

Proposal 2: The R2D timing acquisition part preceding the PRDSCH indicates the chip length used for the following PRDCH transmission.

Proposal 3: The chip length of PDRCH is explicitly indicated by R2D control information.

Proposal 4: Support Opt.1 as baseline (i.e., defining a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it). FFS on Opt.2.

Proposal 5: For Msg1 and Msg3 transmission, support parallel FDMed resources by frequency shifting operation to cope with multiplexing a high number of A-IoT devices

Proposal 6: Define at least two power states for the study of charging and discharging period in RF energy havesting operation

- ‘ON’ state: support transmission/reception.

- ‘Sleep’ state: maintain a memory and timer to count the samples.

- Energy havesting is conducted at least during ‘Sleep’ State.

Proposal 7: Study a duty-cycle based approach where each cycle include ‘monitoring period’ and ‘nonActive period’, where reader tranmits paging command within ‘monitoring’ period to trigger inventory procedure.

# References

RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), RAN#102, December 2023.

RAN1 117 meeting Chairman notes

RAN1 116bis meeting Chairman notes

R1-2403776, “Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT”, Moderator (Vivo), RAN1#116bis, April 2024.

RAN1 116 meeting Chairman notes

RAN2 126 meeting Chairman notes

RAN 103 meeting Chairman notes