3GPP TSG RAN WG1 #118                                                                                          R1-2407294

Maastricht, NL, August 19th – 23rd, 2024

Source:	Moderator(vivo)

Title:	Summary of alignment on the indication of number of HARQ processes

Agenda Item:	8.1

Document for:	Discussion and Decision

- Introduction

This feature lead summary (FLS) document aims to collect company views on draft CR on the indication of number of HARQ processes

R1-2406163	Draft CR on alignment on the indication of number of HARQ processes for PUSCH	vivo

1st round discussion: please provide your feedback before Monday 17:00

- Discussions

According to the latest TS38.214 v18.3.0 after RAN#104, it is observed that the RRC parameter nrofHARQ-ProcessesForPUSCH used to indicate the number of HARQ processes for supporting 32 HARQ processes is duplicated twice. It is also noted that the similar issue was fixed in the latest TS38.214 v17.10.0 after RAN#104 by removing the duplicated parameter name, but the change to TS38.214 v18.3.0 section 6.1 is missing. Therefore, the following change is proposed in [1]:

| Reason for change: | According to the latest TS38.214 v18.3.0 after RAN#104, it is observed that the RRC parameter nrofHARQ-ProcessesForPUSCH used to indicate the number of HARQ processes for supporting 32 HARQ processes is duplicated twice. It is also noted that the similar issue was fixed in the latest TS38.214 v17.10.0 after RAN#104 by removing the duplicated parameter name, but the change to TS38.214 v18.3.0 section 6.1 is missing. |
| --- | --- |
| Summary of change: | Delete one of nrofHARQ-ProcessesForPUSCH to align with TS38.214 v17.10.0. |
| Consequences if not approved: | Redundancy remains in the text on how to determine the number of HARQ process. |

6 Physical uplink shared channel related procedure 6.1 UE procedure for transmitting the physical uplink shared channel <<omitted>> For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell as defined in [13, TS 38.306]. The number of processes the UE may assume will at most be used for the uplink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPUSCH, or nrofHARQ-ProcessesForPUSCH, and when no configuration is provided the UE may assume a default number of 16 processes.

- Collection of Companies’ views

Q1: Do you agree with the draft CR proposed in R1-2406163?

Companies are invited to provide their views on the questions in the following table.

| Company | Y/N | Comments |
| --- | --- | --- |
| Lenovo | Yes | We are fine with the CR. |
| Ericsson | Yes | Perhaps the CR should be category D (editorial modification). |
| Nokia | Yes | Agree with Ericsson that CR should probably be editorial. Additionally, the reason for change contains the statement “duplicated twice”, which does not sound correct. Suggest to remove the “twice”. |
| Huawei, HiSilicon |  | Considering the agreement made in RAN1#117, seems no need to agree again in RAN1. Just need remind editor when we do the alignment CR review. Agreement The draft CR in R1-2404228 is agreed in principle for the spec editor’s alignment Rel-17 CR for TS38.214 and for a Rel-18 shadow alignment CR, without NR_newRAT-Core WI code, by keeping the parameter name nrofHARQ-ProcessesForPUSCH instead of nrofHARQ-ProcessesForPUSCH-r17. |

- Proposal for online discussion

According to the inputs, all companies agreed that the proposed change is correct and necessary. Two companies commented that this is editorial change, and one company commented this can be up to editor during the alignment CR review.

Thus, we have the following proposal:

Proposal: TP#1 in section 3 of R1-2407294 is endorsed for TS 38.214 alignment CR.

TP#1

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on alignment on the indication of number of HARQ processes for PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (vivo) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the latest TS38.214 v18.3.0 after RAN#104, it is observed that the RRC parameter nrofHARQ-ProcessesForPUSCH used to indicate the number of HARQ processes for supporting 32 HARQ processes is duplicated. It is also noted that the similar issue was fixed in the latest TS38.214 v17.10.0 after RAN#104 by removing the duplicated parameter name, but the change to TS38.214 v18.3.0 section 6.1 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete one of nrofHARQ-ProcessesForPUSCH to align with TS38.214 v17.10.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundancy remains in the text on how to determine the number of HARQ process. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6	Physical uplink shared channel related procedure

6.1	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts omitted>

For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell as defined in [13, TS 38.306]. The number of processes the UE may assume will at most be used for the uplink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPUSCH, and when no configuration is provided the UE may assume a default number of 16 processes.

<Unchanged parts omitted>

- Conclusion

TBD

- References

- R1-2406163	Draft CR on alignment on the indication of number of HARQ processes for PUSCH	vivo

## 10 UE procedure for receiving control information <<unchanged parts omitted>> ## 10.1 UE procedure for determining physical downlink control channel assignment <<unchanged parts omitted>> The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB, the DM-RS antenna port associated with corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable [6, TS 38.214], if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. For operation without shared spectrum channel access in FR1 and FR2-1 and FR2-NTN, a SCS is provided by subCarrierSpacingCommon in MIB. For operation with shared spectrum channel access in FR1 and for operation in FR2-2, a SCS is same as the SCS of a corresponding SS/PBCH block. <<unchanged parts omitted>> # 13 UE procedure for monitoring Type0-PDCCH CSS sets If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-0 through 13-10, for operation without shared spectrum channel access in FR1 and FR2-1 and FR2-NTN, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Table 13-10A for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15A. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix. In Table 13-0, configurations with index 0 to 9 are applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in [8-1, TS 38.101-1], configurations with index 10 to 11 are applicable when an associated SS/PBCH block is located according to NOTE 12 of Table 5.4.3.3-1 in [8-1, TS 38.101-1], and non-interleaved CCE-to-REG mapping applies for configurations with index 6 to 9. In Table 13-1, the associated SS/PBCH block is not located according to NOTE 12 of Table 5.4.3.3-1 in [8-1, TS 38.101-1]. For operation with shared spectrum channel access in FR2-2 and for operation without shared spectrum channel access, a UE assumes that the offset in Tables 13-0 through 13-10A is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block, after puncturing if any [4, TS 38.211]. The SCS of the CORESET for Type0-PDCCH CSS set is provided by subCarrierSpacingCommon for FR1 and FR2-1 and FR2-NTN, and same as the SCS of the corresponding SS/PBCH block for FR2-2. In Tables 13-7, 13-8, and 13-10, kSSB is defined in [4, TS 38.211]. <<unchanged parts omitted>> Table 13-8: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz for FR2-1 and FR2-NTN Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 14. 3; 1; 48; 2; 14. 4; 3; 24; 2; -20 if kSSB=0 -21 if kSSB>0. 5; 3; 24; 2; 24. 6; 3; 48; 2; -20 if kSSB=0 -21 if kSSB>0. 7; 3; 48; 2; 48. 8; Reserved. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. <<unchanged parts omitted>> Table 13-12: Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR2-1 and FR2-NTN, or SS/PBCH block and CORESET multiplexing pattern 1 and {SS/PBCH block, PDCCH} SCS {120, 120} kHz in FR2-2 Index; O; Number of search space sets per slot; M; First symbol index. 0; 0; 1; 1; 0. 1; 0; 2; 1/2; {0, if i is even}, {7, if i is odd}. 2; 2.5; 1; 1; 0. 3; 2.5; 2; 1/2; {0, if i is even}, {7, if i is odd}. 4; 5; 1; 1; 0. 5; 5; 2; 1/2; {0, if i is even}, {7, if i is odd}. 6; 0; 2; 1/2; {0, if i is even}, {NsymbCORESET, if i is odd}. 7; 2.5; 2; 1/2; {0, if i is even}, {NsymbCORESET, if i is odd}. 8; 5; 2; 1/2; {0, if i is even}, {NsymbCORESET, if i is odd}. 9; 7.5; 1; 1; 0. 10; 7.5; 2; 1/2; {0, if i is even}, {7, if i is odd}. 11; 7.5; 2; 1/2; {0, if i is even}, {NsymbCORESET, if i is odd}. 12; 0; 1; 2; 0. 13; 5; 1; 2; 0. 14; Reserved. 15; Reserved. <<unchanged parts omitted>> If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for 24≤kSSB≤29 for FR1 or for 12≤kSSB≤13 for FR2, the UE may determine the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a CORESET for an associated Type0-PDCCH CSS set as NGSCNReference+NGSCNSize⋅NGSCNOffset. NGSCNReference is the GSCN of the first SS/PBCH block, NGSCNSize=1 in FR1 and FR2-1 and FR2-NTN, NGSCNSize= 3 in FR2-2, and NGSCNOffset is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If the UE detects the second SS/PBCH block and the second SS/PBCH block does not provide a CORESET for Type0-PDCCH CSS set, as described in clause 4.1, the UE may ignore the information related to GSCN of SS/PBCH block locations for performing cell search.