3GPP TSG RAN WG1 #118		R1-2407366

Maastricht, NL, August 19th – 23rd, 2024

Source:	Moderator (CATT)

Title:	Summary #1 of UE procedure for applying transform precoding on PUSCH

Agenda Item:	7

Document for:	Decision

Introduction

In RAN1#118, the following discussion paper in R1-2406324 and draft CR in R1-2406325 are submitted on UE procedure for applying transform precoding on PUSCH from Rel-17.

R1-2406324	UE procedure for applying transform precoding on PUSCH from Rel-17	CATT, Nokia

R1-2406325	Clarification on UE procedure for applying transform precoding on PUSCH	CATT, Nokia

This contribution collects companies’ views on the issues and proposals discussed in the above Tdocs.

Round 1 discussion

In [1], the signaling structure of RACH configurations from Rel-17 is illustrated in Figure 1. A list of AdditionalRACH-Configs can be configured in additionalRACH-ConfigList to provide additional RACH resources for different features/feature combinations.

Figure 1: signaling structure of RACH configurations from Rel-17

Q1: Do you agree with the issue/Observation 1 in R1-2406324?

Observation 1: With introduction of additionalRACH-ConfigList in Rel-17, which msg3-transformPrecoder/ msgA-TransformPrecoder are used for PUSCH waveform determination is unclear because multiple configurations of msg3-transformPrecoder/msgA-TransformPrecoder can be provided.

| Company | Agree or Not Agree | Comments |
| --- | --- | --- |
| Apple | Agree |  |
| QC |  | Most cases discussed in the following section seem resolvable in a straightforward manner. Not sure if there are any differing interpretations. To agree that there is an issue seems like a strong statement. |
| MTK | Agree |  |
| ZTE | No | As we commented below, the UE behavior is clear for the cases under discussion. |
| Huawei, HiSilicon | agree | Some unclearness exists and we are open for discussion. |
| Ericsson |  | We are open for discussion. But it seems the 331 spec is clear as ZTE pointed out, so it is a matter of if we want to capture same text in RAN1 spec. |

To address the issue, two alternatives were discussed in [1].

On a high level, there are two ways to resolve the problem.

- Alt 1: gNB ensures aligned configurations of msg3-transformPrecoder and msgA-TransformPrecoder to avoid any potential misalignment.

- Alt 2: Clarify which msg3-transformPrecoder and msgA-TransformPrecoder are used to determine PUSCH waveform

It was noted in [1] that in TS 38.331, for CG configuration, there is clarification in the field description highlighted in green below, which is in line with Alt 2 assuming that msg3-transformPrecoder can be different in rach-ConfigCommon included directly within BWP configuration and in additionalRACH-ConfigList.

transformPrecoder Enables or disables transform precoding for type1 and type2. If the field is absent, the UE enables or disables transform precoding in accordance with the field msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList), see TS 38.214 [19], clause 6.1.3.

Q2: Do you agree to address the issues assuming msg3-transformPrecoder can be different in rach-ConfigCommon included directly within BWP configuration and in rach-ConfigCommon included in additionalRACH-ConfigList?

| Company | Agree or Not Agree | Comments |
| --- | --- | --- |
| Apple | Agree |  |
| QC |  | This question touches upon an even more basic question: Why will a gNB not use DFT-S-OFDM for all Msg3/MsgA transmission across all RACH procedures. Note that DFT-S-OFDM is a mandatory feature and all UEs support this waveform. Can the proponents identify any scenarios where there is a clear incentive for the gNB to set the transformPrecoder parameter to be different in rach-ConfigCommon and additionalRACH-ConfigList(s) |
| Xiaomi |  | From the RRC signaling design perspective, the gNB has the flexibility to configure different waveform for msg3/MsgA transmission across all PRACH procedures. But, we share the same view as QUALCOMM that whether there is any reasonable case for the gNB to configure different waveform for different msg3/MsgA transmissions? |
| MTK | Yes |  |
| ZTE |  | This becomes irrelevant as the UE behavior is clear for the cases under discussion. |
| Huawei, HiSilicon | agree | And wonder whether the parameters are mandatorily configurable, or in real case would naturally be configured? |
| Samsung |  | We tend to agree with Qualcomm. We find it difficult to identify a practical use case where the gNB would configure them differently. |
| Ericsson |  | If we want to clarify it, Alt1 should be the way. |

Q3: Do you agree with Proposal 1 in R1-2406324 and the corresponding text proposal in R1-2406325?

Proposal 1: For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI, msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration is used to determine whether transform precoding is enabled or disabled when applicable, as for PUSCH transmission with a configured grant. For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration.

| Company | Agree or Not Agree | Comments |
| --- | --- | --- |
| Apple | Agree |  |
| QC | Agree, but there is no need for TP. | Lets wait to see if there are any companies with differing views. If there are none, there is nothing to be done. |
| Xiaomi | Agree, but slightly prefer not changing the current spec | We are fine with the logic, but we slightly prefer not to change the specification. In addition, if the CR is necessary from most companies’ view, we have some suggestions for minor changes to the newly added restrictive words as follows: in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration |
| MTK | Agree | At least the TP is a good clarification. |
| ZTE | Agree but the current spec is already clear | According to transformPrecoder in PUSCH-Config, it is already clear. It is noted that, the following highlighted texts are added for 38.331 in Rel-17. transformPrecoder The UE specific selection of transformer precoder for PUSCH (see TS 38.214 [19], clause 6.1.3). When the field is absent the UE applies the value of the field msg3- transformPrecoder from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList).. |
| Huawei, HiSilicon |  | For DCI dynamic scheduling, 331 already clarifies. |
| Samsung |  | TR 38.331 is already clear on this matter. TP is not needed. |
| Ericsson |  | We are fine with the TP. |

Q4-1: Do you agree with Proposal 2 and 3 in R1-2406324? Q4-2: Do you agree that no spec change is needed for the proposals?

Proposal 2: For a PUSCH scheduled by RAR UL grant and for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, UE enables or disables transform precoding according to msg3-transformPrecoder in the RACH-ConfigCommon corresponding to the RACH procedure. Proposal 3: For a MsgA PUSCH, UE enables or disables transform precoding according to msgA-TransformPrecoder in the MsgA-ConfigCommon corresponding to the RACH procedure.

| Company | Agree or Not Agree | Comments |
| --- | --- | --- |
| Apple | Agree | One question is whether it is clear to everyone that “according to the higher layer configured parameter msg3-transformPrecoder” automatically means the parameter corresponding to the RACH procedure. If not, this may need clarification. |
| QC | Agree, also agree that there is no need for a TP. | Same comment as above. |
| Xiaomi | Agree, but slightly prefer not changing the current spec | Same comment as above. There is an additional question: how to deal with the following case: the RRC parameter msg3-transformPrecoder in the RACH-ConfigCommon corresponding to the RACH procedure is not provided? Do we just follow the following specification in TS 38.331. msg3-transformPrecoder Enables the transform precoder for Msg3 transmission according to clause 6.1.3 of TS 38.214 [19]. If the field is absent, the UE disables the transformer precoder (see TS 38.213 [13], clause 8.3).. Or, could have other understanding? For example, just like msgA PUSCH shares the same waveform as for Msg3 PUSCH if msgA-transformPrecoder is absent, the msg3 PUSCH with FC could also share the waveform of msg3 PUSCH configured for the legacy RACH if the msg3-transformPrecoder corresponding to the FC is not provided. In our view, both directions are fine but a further clarification is necessary, if Q1 proposed by FL is valid. |
| MTK | Agree |  |
| ZTE | Agree the proposal but no spec change is needed | We don’t think there could be other interpretation. For PUSCH in a RACH procedure, it should be clear that the RRC parameters in RACH-ConfigCommon corresponding to the RACH procedure should be used. |
| Samsung |  | We agree with the UE's behavior, but it is commonly understood, and no specification change is necessary. |

Q5: For a PUSCH scheduled by fallbackRAR UL grant and for a MsgA PUSCH when msgA-TransformPrecoder is not configured, which Alt do you prefer?

- Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration. - Alt 2: - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration. - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config.

| Company | Alt 1 or Alt 2 | Comments |
| --- | --- | --- |
| Apple | Alt 2 | We think Alt 2 makes more sense logically. However, for the 2nd sub-bullet, the case when msg3-transformPrecoder is not configured in the same AdditionalRACH-Config is still missing. |
| QC | Neither | The issue doesn’t arise as long as the gNB provides UE with the necessary parameters. gNB to therefore ensure the relevant parameters for a procedure are provided. In effect “not configuring” should be an error case. |
| Xiaomi | Both are ok | For the second bullet of alt 2, we have the same question as in Q4. How to deal with the same FC for 4-step RACH is configured in the same AdditionalRACH-Config? Or, the the same FC for 4-step RACH is configured, but the msg3-transformPrecoder is not included? Besides, maybe there is another solution for msgA-configCommon included in additionalRACH-ConfigList. That is, if the field is absent, the UE disables the transformer precoder, which is a simple way in our view with minor or no spec change. |
| MTK | Alt 1 | Alt 1 seems more straightforward to us, and also a simpler solution. |
| ZTE | Alt 2. | Based on the current RAN2 framework, a UE will first select the corresponding RACH configuration for a certain UE feature/feature combination, and the decide whether to use 2-step or 4 step RACH. When fallback happens, the UE is still under the RACH procedure for that UE feature combination, and Alt 2 in proposal 4 is the only way. So, this is clear based on RAN2 spec and no spec change is needed. |
| Huawei, HiSilicon | Case by case | For the case that a MsgA PUSCH when msgA-TransformPrecoder is not configured, it is straightforward to take Alt 1 since the rach-ConfigCommon is supposed to always include necessary parameters. However, for the case of fallback RAR, it is more natural that the UE check the configuration of corresponding 4-step RACH that the UE would fallback from 2stepRACH to. (similar to what ZTE commented). |
| Samsung | - | We believe that discussing practical use cases should take priority. |

Q6: Any other comments/questions?

| Company | Comments |
| --- | --- |
| Xiaomi | As shown in Q4 and Q5 |

Summary

The following cases are discussed in [1]. Companies’ views are summarized below.

- Case 1: For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI, if the DCI with the scheduling grant was not received with DCI format 0_0 and the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config

ZTE pointed out that it is already clear from the field description of transformPrecoder in PUSCH-Config in TS38.331.

transformPrecoder The UE specific selection of transformer precoder for PUSCH (see TS 38.214 [19], clause 6.1.3). When the field is absent the UE applies the value of the field msg3-transformPrecoder from rach-ConfigCommon included directly within BWP configuration (i.e., not included in additionalRACH-ConfigList).

- Case 2: For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI, If the DCI with the scheduling grant was received with DCI format 0_0

All companies agree that msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration is used to determine whether transform precoding is enabled or disabled.

Apple, MediaTek, Huawei and Ericsson agree the following TP in principle.

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

- Case 3: For a PUSCH scheduled by RAR UL grant and for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI and for a MsgA PUSCH

Companies agree that parameters corresponding to the RACH procedure should be used and no spec impact is needed.

- Case 4: For a PUSCH scheduled by fallbackRAR UL grant and for a MsgA PUSCH when msgA-TransformPrecoder is not configured

  - Alt 1: It is always msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - Support: xiaomi, MediaTek, Huawei (for MsgA PUSCH)

  - Alt 2:

    - For msgA-ConfigCommon included directly within BWP configuration, it is msg3-transformPrecoder in rach-ConfigCommon included directly within BWP configuration.

    - For msgA-ConfigCommon included in additionalRACH-ConfigList, it is msg3-transformPrecoder included in the same AdditionalRACH-Config.

    - Support: Apple, xiaomi, ZTE, Huawei (for fallback RAR)

Qualcomm, xiaomi and Samsung asked scenarios where gNB set the transformPrecoder parameter to be different in rach-ConfigCommon and additionalRACH-ConfigList(s).

However, the current spec supports different configuration clearly from the field descriptions in TS38.331.

Proposal 1: Agree the following text proposal for TS38.214 clause 6.1.3 for Case 2.

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder in RACH-ConfigCommon from rach-ConfigCommon included directly within BWP configuration. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

Proposal 2: Discuss and decide between the two alternatives for Case 4 and the corresponding text proposal.

Reference

- R1-2406324	UE procedure for applying transform precoding on PUSCH from Rel-17	CATT, Nokia

- R1-2406325	Clarification on UE procedure for applying transform precoding on PUSCH	CATT, Nokia