3GPP TSG RAN WG1 #118bis	R1-xxxxx

# Hefei, China, October 14 – 18, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of parameter names |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Ambiguous description of intermediate resources for antenna port mapping. (R1-2407789) - Enhanced DM-RS pattern for PDSCH should, according to RAN1 agreements, not be supported for DCI formats 1_0, 4_0, and 4_1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification of the intermediate resoruce. - Clarificatio of the DM-RS pattern generation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Ambiguous specification. - Specifications not in line with the RAN1 agreements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1.3. 7.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.4.1.1.3	Precoding and mapping to physical resources

The sequence  shall be mapped to the intermediate quantity ak,l(pj,μ) according to

-	if transform precoding is not enabled,

-	if the higher-layer parameter dmrs-TypeEnh is configured

ak,lpjpj,μ=wfk'wtl'r4n+k'
k=8n+2k'+Δconfiguration type 112n+k'+Δconfiguration type 2, k'=0,112n+k'+Δ+4configuration type 2, k'=2,3
k'=0,1,2,3
l=l+l'
n=0,1,…
j=0,1,…,υ-1

-	otherwise

ak,lpjpj,μ=wfk'wtl'r2n+k'
k=4n+2k'+Δconfiguration type 16n+k'+Δconfiguration type 2
k'=0,1
l=l+l'
n=0,1,…
j=0,1,…,υ-1

-	if transform precoding is enabled

ak,lp0p0,μ=wfk'wtl'r2n+k'
k=4n+2k'+Δ
k'=0,1
l=l+l'
n=0,1,…

where wfk', wtl', and Δ are given by Tables 6.4.1.1.3-1 and 6.4.1.1.3-2 and the configuration type is given by the higher-layer parameter DMRS-UplinkConfig, and both k' and Δ correspond to p0, …, pν-1. The intermediate quantity ak,l(pj,μ)=0 if Δ corresponds to any other antenna ports than pj.

The intermediate quantity ak,l(pj,μ) shall be precoded, multiplied with the amplitude scaling factor βPUSCHDMRS in order to conform to the transmit power specified in [6, TS 38.214], and mapped to physical resources according to

ak,lp0,μ⋮ak,lpρ-1,μ=βPUSCHDMRSWak,lp0,μ⋮ak,lpυ-1,μ

where

-	the precoding matrix W is given by clause 6.3.1.5,

-	the set of antenna ports p0,…,pρ-1 is given by clause 6.3.1.5, and

-	the set of antenna ports p0,…,pρ-1 is given by [6, TS 38.214];

and the following conditions are fulfilled:

-	the resource elements ak,l(pj,μ) are within the common resource blocks allocated for PUSCH transmission.

The reference point for k is

-	subcarrier 0 in common resource block 0 if transform precoding is not enabled, and

-	subcarrier 0 of the lowest-numbered resource block of the scheduled PUSCH allocation if transform precoding is enabled.

The reference point for l and the position  of the first DM-RS symbol depends on the mapping type:

-	for PUSCH mapping type A:

-	 is defined relative to the start of the slot if frequency hopping is disabled and relative to the start of each hop in case frequency hopping is enabled

-	 is given by the higher-layer parameter dmrs-TypeA-Position

-	for PUSCH mapping type B:

-	 is defined relative to the start of the scheduled PUSCH resources if frequency hopping is disabled and relative to the start of each hop in case frequency hopping is enabled

-

The position(s) of the DM-RS symbols is given by  and duration ld where

-	ld is the duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PUSCH resources in the slot for PUSCH mapping type A according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if intra-slot frequency hopping is not used, or

-	ld is the duration of scheduled PUSCH resources for PUSCH mapping type B according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if intra-slot frequency hopping is not used, or

-	ld is the duration per hop according to Table 6.4.1.1.3-6 if intra-slot frequency hopping is used.

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is not configured, or for a msgA transmission msgA-MaxLength in msgA-DMRS-Config is not configured, the tables shall be used according to single-symbol DM-RS

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 'len2', the associated DCI or configured grant configuration determines whether single-symbol or double-symbol DM-RS shall be used

-	if the higher-layer parameter msgA-MaxLength in msgA-DMRS-Config is equal to 'len2', double-symbol DM-RS shall be used

-	if the higher-layer parameter dmrs-AdditionalPosition is not set to 'pos0' and intra-slot frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212] and by higher layer, Tables 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is equal to 'pos1' for each hop.

For PUSCH mapping type A,

-	the case dmrs-AdditionalPosition is equal to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2';

-	ld=4 symbols in Table 6.4.1.1.3-4 is only applicable when dmrs-TypeA-Position is equal to 'pos2'.

For msgA transmitted using PUSCH mapping type A,

-	the case msgA-DMRS-AdditionalPosition is equal to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2';

-	'dmrs-AdditionalPosition' in Tables 6.4.1.1.3-3 to 6.4.1.1.3-6 shall be replaced by msgA-DMRS-AdditionalPosition;

-	only PUSCH DM-RS configuration type 1 is supported;

-	only basic DM-RS multiplexing in Table 6.4.1.1.3-5 is supported.

For msgA transmitted using PUSCH mapping type B,

-	'dmrs-AdditionalPosition' in Tables 6.4.1.1.3-3 to 6.4.1.1.3-6 shall be replaced by msgA-DMRS-AdditionalPosition;

-	only PUSCH DM-RS configuration type 1 is supported;

-	only basic DM-RS multiplexing in Table 6.4.1.1.3-5 is supported.

The time-domain index l', and the supported antenna ports pj are given by Table 6.4.1.1.3-5.

Table 6.4.1.1.3-1: Parameters for PUSCH DM-RS configuration type 1.

| p | CDM group λ | Δ | wf(0)…wf(3) | wt(0)wt(1) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | +1+1+1+1 | +1+1 |
| 1 | 0 | 0 | +1-1+1-1 | +1+1 |
| 2 | 1 | 1 | +1+1+1+1 | +1+1 |
| 3 | 1 | 1 | +1-1+1-1 | +1+1 |
| 4 | 0 | 0 | +1+1+1+1 | +1-1 |
| 5 | 0 | 0 | +1-1+1-1 | +1-1 |
| 6 | 1 | 1 | +1+1+1+1 | +1-1 |
| 7 | 1 | 1 | +1-1+1-1 | +1-1 |
| 8 | 0 | 0 | +1+j-1-j | +1+1 |
| 9 | 0 | 0 | +1-j-1+j | +1+1 |
| 10 | 1 | 1 | +1+j-1-j | +1+1 |
| 11 | 1 | 1 | +1-j-1+j | +1+1 |
| 12 | 0 | 0 | +1+j-1-j | +1-1 |
| 13 | 0 | 0 | +1-j-1+j | +1-1 |
| 14 | 1 | 1 | +1+j-1-j | +1-1 |
| 15 | 1 | 1 | +1-j-1+j | +1-1 |

Table 6.4.1.1.3-2: Parameters for PUSCH DM-RS configuration type 2.

| p | CDM group λ | Δ | wf(0)…wf(3) | wt(0)wt(1) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | +1+1+1+1 | +1+1 |
| 1 | 0 | 0 | +1-1+1-1 | +1+1 |
| 2 | 1 | 2 | +1+1+1+1 | +1+1 |
| 3 | 1 | 2 | +1-1+1-1 | +1+1 |
| 4 | 2 | 4 | +1+1+1+1 | +1+1 |
| 5 | 2 | 4 | +1-1+1-1 | +1+1 |
| 6 | 0 | 0 | +1+1+1+1 | +1-1 |
| 7 | 0 | 0 | +1-1+1-1 | +1-1 |
| 8 | 1 | 2 | +1+1+1+1 | +1-1 |
| 9 | 1 | 2 | +1-1+1-1 | +1-1 |
| 10 | 2 | 4 | +1+1+1+1 | +1-1 |
| 11 | 2 | 4 | +1-1+1-1 | +1-1 |
| 12 | 0 | 0 | +1+j-1-j | +1+1 |
| 13 | 0 | 0 | +1-j-1+j | +1+1 |
| 14 | 1 | 2 | +1+j-1-j | +1+1 |
| 15 | 1 | 2 | +1-j-1+j | +1+1 |
| 16 | 2 | 4 | +1+j-1-j | +1+1 |
| 17 | 2 | 4 | +1-j-1+j | +1+1 |
| 18 | 0 | 0 | +1+j-1-j | +1-1 |
| 19 | 0 | 0 | +1-j-1+j | +1-1 |
| 20 | 1 | 2 | +1+j-1-j | +1-1 |
| 21 | 1 | 2 | +1-j-1+j | +1-1 |
| 22 | 2 | 4 | +1+j-1-j | +1-1 |
| 23 | 2 | 4 | +1-j-1+j | +1-1 |

Table 6.4.1.1.3-3: PUSCH DM-RS positions  within a slot for single-symbol DM-RS and intra-slot frequency hopping disabled.

| ld in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  | PUSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | pos0 | pos1 | pos2 | pos3 | pos0 | pos1 | pos2 | pos3 |
| <4 | - | - | - | - |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 6 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 7 |  |  |  |  |  | , 4 | , 4 | , 4 |
| 8 |  | , 7 | , 7 | , 7 |  | , 6 | , 3, 6 | , 3, 6 |
| 9 |  | , 7 | , 7 | , 7 |  | , 6 | , 3, 6 | , 3, 6 |
| 10 |  | , 9 | , 6, 9 | , 6, 9 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 11 |  | , 9 | , 6, 9 | , 6, 9 |  | , 8 | , 4, 8 | , 3, 6, 9 |
| 12 |  | , 9 | , 6, 9 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |
| 13 |  | , 11 | , 7, 11 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |
| 14 |  | , 11 | , 7, 11 | , 5, 8, 11 |  | , 10 | , 5, 10 | , 3, 6, 9 |

Table 6.4.1.1.3-4: PUSCH DM-RS positions  within a slot for double-symbol DM-RS and intra-slot frequency hopping disabled.

| ld in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  | PUSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | pos0 | pos1 | pos2 | pos3 | pos0 | pos1 | pos2 | pos3 |
| <4 | - | - |  |  | - | - |  |  |
| 4 |  |  |  |  | - | - |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  | , 5 |  |  |
| 9 |  |  |  |  |  | , 5 |  |  |
| 10 |  | , 8 |  |  |  | , 7 |  |  |
| 11 |  | , 8 |  |  |  | , 7 |  |  |
| 12 |  | , 8 |  |  |  | , 9 |  |  |
| 13 |  | , 10 |  |  |  | , 9 |  |  |
| 14 |  | , 10 |  |  |  | , 9 |  |  |

Table 6.4.1.1.3-5: PUSCH DM-RS time index l'.

| DM-RS multiplexing | DM-RS duration | l' | Supported antenna ports p |  |
| --- | --- | --- | --- | --- |
|  |  |  | Configuration type 1 | Configuration type 2 |
| Basic | single-symbol DM-RS | 0 | 0 – 3 | 0 – 5 |
|  | double-symbol DM-RS | 0, 1 | 0 – 7 | 0 – 11 |
| Enhanced | single-symbol DM-RS | 0 | 0 – 3, 8 – 11 | 0 – 5, 12 – 17 |
|  | double-symbol DM-RS | 0, 1 | 0 – 15 | 0 – 23 |

Table 6.4.1.1.3-6: PUSCH DM-RS positions  within a slot for single-symbol DM-RS and intra-slot frequency hopping enabled.

| ld in symbols | DM-RS positions l |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PUSCH mapping type A |  |  |  |  |  |  |  | PUSCH mapping type B l0=0 |  |  |  |
|  | l0=2 |  |  |  | l0=3 |  |  |  |  |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | pos0 |  | pos1 |  | pos0 |  | pos1 |  | pos0 |  | pos1 |  |
|  | 1st hop | 2nd hop | 1st hop | 2nd hop | 1st hop | 2nd hop | 1st hop | 2nd hop | 1st hop | 2nd hop | 1st hop | 2nd hop |
| ≤3 | - | - | - | - | - | - | - | - | 0 | 0 | 0 | 0 |
| 4 | 2 | 0 | 2 | 0 | 3 | 0 | 3 | 0 | 0 | 0 | 0 | 0 |
| 5, 6 | 2 | 0 | 2 | 0, 4 | 3 | 0 | 3 | 0, 4 | 0 | 0 | 0, 4 | 0, 4 |
| 7 | 2 | 0 | 2, 6 | 0, 4 | 3 | 0 | 3 | 0, 4 | 0 | 0 | 0, 4 | 0, 4 |

##### 7.4.1.1.2	Mapping to physical resources

The UE shall assume the PDSCH DM-RS being mapped to physical resources according to configuration type 1 or configuration type 2 as given by the higher-layer parameter dmrs-Type.

The UE shall assume the sequence  is scaled by a factor βPDSCHDMRS to conform with the transmission power specified in [6, TS 38.214] and mapped to resource elements k,lp,μ according to

-	if the higher-layer parameter dmrs-TypeEnh is configured and the PDSCH is not scheduled by DCI format 1_0, 4_0, or 4_1

ak,lpj,μ=βPDSCHDMRSwfk'wtl'r4n+k'
k=8n+2k'+Δconfiguration type 112n+k'+Δconfiguration type 2, k'=0,112n+k'+Δ+4configuration type 2, k'=2,3
k'=0,1,2,3
l=l+l'
n=0,1,…
j=0,1,…,υ-1

-	otherwise

ak,lpj,μ=βPDSCHDMRSwfk'wtl'r2n+k'
k=4n+2k'+Δconfiguration type 16n+k'+Δconfiguration type 2
k'=0,1
l=l+l'
n=0,1,…
j=0,1,…,υ-1

where , , and Δ are given by Tables 7.4.1.1.2-1 and 7.4.1.1.2-2 and the following conditions are fulfilled:

-	the resource elements are within the common resource blocks allocated for PDSCH transmission

The reference point for k is

-	subcarrier 0 of the lowest-numbered resource block in CORESET 0 if the corresponding PDCCH is associated with CORESET 0 and Type0-PDCCH common search space and is addressed to SI-RNTI;

-	otherwise, subcarrier 0 in common resource block 0

The reference point for  and the position  of the first DM-RS symbol depends on the mapping type:

-	for PDSCH mapping type A:

-	 is defined relative to the start of the slot

-	if the higher-layer parameter dmrs-TypeA-Position is equal to 'pos3' and  otherwise

-	for PDSCH mapping type B:

-	 is defined relative to the start of the scheduled PDSCH resources

-

The position(s) of the DM-RS symbols is given by  and duration ld where

-	for PDSCH mapping type A, ld is the duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot

-	for PDSCH mapping type B, ld is the duration of the scheduled PDSCH resources

and according to Tables 7.4.1.1.2-3 and 7.4.1.1.2-4.

For PDSCH mapping type A

-	the case dmrs-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2';

-	ld=3 and ld=4 symbols in Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 respectively is only applicable when dmrs-TypeA-Position is equal to 'pos2';

-	single-symbol DM-RS, l1=11 except if all of the following conditions are fulfilled in which case l1=12:

-	the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4 is configured; and

-	the higher-layer parameter dmrs-AdditionalPosition is equal to 'pos1' and l0=3; and

-	the UE has indicated it is capable of additionalDMRS-DL-Alt

For PDSCH mapping type B

-	if the PDSCH duration ld ∈2,3,4,5,6,7,8,9,10,11,12,13 OFDM symbols for normal cyclic prefix or ld∈2,4,6 OFDM symbols for extended cyclic prefix, and the front-loaded DM-RS of the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET,  shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and until no collision with any CORESET occurs, and

-	if the PDSCH duration ld is 2 symbols, the UE is not expected to receive a DM-RS symbol beyond the second symbol;

-	if the PDSCH duration ld is 5 symbols and if one additional single-symbol DMRS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th symbol when the front-loaded DM-RS symbol is in the 1st symbol of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted;

-	if the PDSCH duration ld is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix:

-	if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted;

-	if the PDSCH duration ld ∈5,6,7,8,9,10,11,12,13 OFDM symbols, the UE is not expected to receive the front-loaded DM-RS beyond the 4th symbol;

-	if the PDSCH duration ld is 12 or 13 symbols, the UE is not expected to receive DM-RS mapped to symbol 12 or later in the slot;

-	for all values of the PDSCH duration ld other than 2, 5, and 7 symbols, the UE is not expected to receive DM-RS beyond the (ld-1):th symbol;

-	if the PDSCH duration ld is less than or equal to 4 OFDM symbols, only single-symbol DM-RS is supported.

-	if the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4 is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4, then l shall be incremented by one in all slots.

The time-domain index l' and the supported antenna ports p are given by Table 7.4.1.1.2-5 where

-	single-symbol DM-RS is used if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is not configured;

-	single-symbol or double-symbol DM-RS is determined by the associated DCI if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is equal to 'len2';

-	basic or enhanced DM-RS multiplexing is controlled by the higher-layer parameter dmrs-TypeEnh.

In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. Unless specified otherwise, the UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx (when applicable). The UE may assume that DMRS ports associated with a TCI state as described in clause 5.1.6.2 of [6, TS 38.214] of a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain.

The UE may assume that no DM-RS collides with the SS/PBCH block.

Table 7.4.1.1.2-1: Parameters for PDSCH DM-RS configuration type 1.

| p | CDM group λ | Δ | wf(0)…wf(3) | wt(0)wt(1) |
| --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1+1+1+1 | +1+1 |
| 1001 | 0 | 0 | +1-1+1-1 | +1+1 |
| 1002 | 1 | 1 | +1+1+1+1 | +1+1 |
| 1003 | 1 | 1 | +1-1+1-1 | +1+1 |
| 1004 | 0 | 0 | +1+1+1+1 | +1-1 |
| 1005 | 0 | 0 | +1-1+1-1 | +1-1 |
| 1006 | 1 | 1 | +1+1+1+1 | +1-1 |
| 1007 | 1 | 1 | +1-1+1-1 | +1-1 |
| 1008 | 0 | 0 | +1+1-1-1 | +1+1 |
| 1009 | 0 | 0 | +1-1-1+1 | +1+1 |
| 1010 | 1 | 1 | +1+1-1-1 | +1+1 |
| 1011 | 1 | 1 | +1-1-1+1 | +1+1 |
| 1012 | 0 | 0 | +1+1-1-1 | +1-1 |
| 1013 | 0 | 0 | +1-1-1+1 | +1-1 |
| 1014 | 1 | 1 | +1+1-1-1 | +1-1 |
| 1015 | 1 | 1 | +1-1-1+1 | +1-1 |

Table 7.4.1.1.2-2: Parameters for PDSCH DM-RS configuration type 2.

| p | CDM group λ | Δ | wf(0)…wf(3) | wt(0)wt(1) |
| --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1+1+1+1 | +1+1 |
| 1001 | 0 | 0 | +1-1+1-1 | +1+1 |
| 1002 | 1 | 2 | +1+1+1+1 | +1+1 |
| 1003 | 1 | 2 | +1-1+1-1 | +1+1 |
| 1004 | 2 | 4 | +1+1+1+1 | +1+1 |
| 1005 | 2 | 4 | +1-1+1-1 | +1+1 |
| 1006 | 0 | 0 | +1+1+1+1 | +1-1 |
| 1007 | 0 | 0 | +1-1+1-1 | +1-1 |
| 1008 | 1 | 2 | +1+1+1+1 | +1-1 |
| 1009 | 1 | 2 | +1-1+1-1 | +1-1 |
| 1010 | 2 | 4 | +1+1+1+1 | +1-1 |
| 1011 | 2 | 4 | +1-1+1-1 | +1-1 |
| 1012 | 0 | 0 | +1+1-1-1 | +1+1 |
| 1013 | 0 | 0 | +1-1-1+1 | +1+1 |
| 1014 | 1 | 2 | +1+1-1-1 | +1+1 |
| 1015 | 1 | 2 | +1-1-1+1 | +1+1 |
| 1016 | 2 | 4 | +1+1-1-1 | +1+1 |
| 1017 | 2 | 4 | +1-1-1+1 | +1+1 |
| 1018 | 0 | 0 | +1+1-1-1 | +1-1 |
| 1019 | 0 | 0 | +1-1-1+1 | +1-1 |
| 1020 | 1 | 2 | +1+1-1-1 | +1-1 |
| 1021 | 1 | 2 | +1-1-1+1 | +1-1 |
| 1022 | 2 | 4 | +1+1-1-1 | +1-1 |
| 1023 | 2 | 4 | +1-1-1+1 | +1-1 |

Table 7.4.1.1.2-3: PDSCH DM-RS positions  for single-symbol DM-RS.

| ld in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type A |  |  |  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | pos0 | pos1 | pos2 | pos3 | pos0 | pos1 | pos2 | pos3 |
| 2 | - | - | - | - |  |  | l0 | l0 |
| 3 |  |  |  |  | l0 | l0 | l0 | l0 |
| 4 |  |  |  |  |  |  | l0 | l0 |
| 5 |  |  |  |  | l0 | l0,4 | l0,4 | l0,4 |
| 6 |  |  |  |  |  |  | l0,4 | l0,4 |
| 7 |  |  |  |  |  |  | l0,4 | l0,4 |
| 8 |  | , 7 | , 7 | , 7 | l0 | l0, 6 | l0,3, 6 | l0,3, 6 |
| 9 |  | , 7 | , 7 | , 7 | l0 | l0,7 | l0, 4, 7 | l0, 4, 7 |
| 10 |  | , 9 | , 6, 9 | , 6, 9 | l0 | l0, 7 | l0, 4, 7 | l0, 4, 7 |
| 11 |  | , 9 | , 6, 9 | , 6, 9 | l0 | l0, 8 | l0, 4, 8 | l0, 3, 6, 9 |
| 12 |  | , 9 | , 6, 9 | , 5, 8, 11 | l0 | l0,9 | l0,5, 9 | l0, 3, 6, 9 |
| 13 |  | , l1 | , 7, 11 | , 5, 8, 11 | l0 | l0,9 | l0,5, 9 | l0, 3, 6, 9 |
| 14 |  | , l1 | , 7, 11 | , 5, 8, 11 | - | - | - | - |

Table 7.4.1.1.2-4: PDSCH DM-RS positions  for double-symbol DM-RS.

| ld in symbols | DM-RS positions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type A |  |  | PDSCH mapping type B |  |  |
|  | dmrs-AdditionalPosition |  |  | dmrs-AdditionalPosition |  |  |
|  | pos0 | pos1 | pos2 | pos0 | pos1 | pos2 |
| <4 |  |  |  | - | - |  |
| 4 |  |  |  | - | - |  |
| 5 |  |  |  | l0 | l0 |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  | l0 | l0,5 |  |
| 9 |  |  |  | l0 | l0,5 |  |
| 10 |  | , 8 |  | l0 | l0,7 |  |
| 11 |  | , 8 |  | l0 | l0,7 |  |
| 12 |  | , 8 |  | l0 | l0,8 |  |
| 13 |  | , 10 |  | l0 | l0,8 |  |
| 14 |  | , 10 |  | - | - |  |

Table 7.4.1.1.2-5: PDSCH DM-RS time index l' and antenna ports p.

| DM-RS multiplexing | DM-RS duration | l' | Supported antenna ports p |  |
| --- | --- | --- | --- | --- |
|  |  |  | Configuration type 1 | Configuration type 2 |
| Basic | single-symbol DM-RS | 0 | 1000 – 1003 | 1000 – 1005 |
|  | double-symbol DM-RS | 0, 1 | 1000 – 1007 | 1000 – 1011 |
| Enhanced | single-symbol DM-RS | 0 | 1000 – 1003, 1008 – 1011 | 1000 – 1005, 1012 – 1017 |
|  | double-symbol DM-RS | 0, 1 | 1000 – 1015 | 1000 – 1023 |