R1- 2500106

3GPP TSG RAN WG1 Meeting #120

Athens, Greece, February 17th – 21st, 2025

| Agenda Item (AI): | 9.2.3 |
| --- | --- |
| AI Description: | UL 3Tx |
| Source: | MediaTek Inc. |
| Title: | Enhancements for 3-antenna-port transmissions |
| Document for: | Discussion |

- Introduction

In the RAN (Plenary) Meeting #102, a new Work Item (WI) was approved targeting MIMO evolution for Downlink (DL) and Uplink (UL). The 4th objective of the aformentioned WI is:

The lagacy specifications include support of up to 1/2/4 UL TX ports, allowing up to maximum of 4 trasnmission layers with Codebook and Non-Codebook based schemes.

- The codebook transmission scheme entails providing the UE with a specific Transmit Precoder Matrix Indicator (TPMI) chosen from a predefined codebook, based on non-precoded SRS transmission.

- The non-codebook scheme, on the other hand, relies on channel reciprocity, to deduce general precoding weights. Those weights are in turn used to precode SRS. Up to 4 SRS resources in one SRS Resource Set are configured, with only one port per SRS resource. The Network (NW) implicitly indicates to UE which precoding weights to use, in addition to the trasnsmission rank, through SRS Resource Indicators (SRI).

However, the commercial UEs in the market only support a maximum of 2Tx ports with 2Rx or 4Rx capability. To enhance the commercial handset UEs from 2Tx capability, 3Tx in the UL is explored in this work item without significant modifications from the existing 4Tx design structure. Thanks to the innovations in RF, which allows a UE to support 3 parallel RF chains for UL transmission. Thus 3Tx is being viewed as trade-off between the improved performance with optimized RF architecture.

The agreements till the previous RAN1 meetings try to achieve majority of the objectives in the WID. Thus in the RAN plenary (RAN#105), it is agreed to update the WID to enhance the 3Tx with non codebook based transmission and SRS antenna switching features. RAN4 had proposed to study the SRS antenna switching for 3Tx UE and on the other side the specification impact for enabling non codebook based transmission is low. Considering these aspects, both of the features are incorporated into the WID objective as below.  In this document, we try to discuss our proposals on the updated WID objective.

WID Objective (updated as in RP#105): Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource.

- Note: UL full power transmission mode 1 and 2 are not supported.

- Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

Further it was discussed in RP#106 on configuring 3T3R as the WID objective specifies to not support any enhancement other than UE capability signaling. In order to avoid broken spec, further discussion on 3T3R is allowed.

- 3T3R operation.

It was discussed in RP#105 on configuring 3T3R as the WID objective specifies to not support any enhancement other than UE capability signaling. However, this would result in broken specification as there is no SRS configuration. It was discussed among the companies to relook in to the WID objective and further updated in RANP#106. The UE capability of “'t3r3' for 3T3R” has been agreed in RAN1#119, but without any SRS configuration association. To have a meaningful spec, the configuration of SRS resource set(s) with 3T3R operation is essential and has been agreed in RANP#106 to further discuss and complete 3T3R.

RP#106 outcome: RAN clarifies that for ‘3T3R’ antenna switching, SRS configuration i.e., SRS resource/set definition for ‘3T3R’ antenna switching can be discussed and supported as part of Rel-19 MIMO in RAN1.

- where ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports; No RAN4 impact.

UE capability without corresponding procedure update result in broken spec on how to configure or fall back to 3T3R. The objective also assumes that 3T3R should be supported by UEs with 4Tx/6Rx/8Rx as these are the common configurations in the market. With muted last port of 4-port SRS resource, UE even with 4Rx/6Rx/8Rx allows to support 3T3R with very limited impact on specifications. Similar to all legacy xTxR configurations, 3T3R is also expected to have the UE capability and SRS configuration association. To support 3T3R, corresponding procedure update can be done by allowing RRC parameter to indicate the SRS resource set usage as “antennaSwitching”. Existing agreement on 3T6R for RRC parameter can be reused for 3T3R signaling.

Agreement (RAN1#119)

For a UE operating with 3TX,

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as codebook.

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antenna switching.

- Note: The RRC parameter mentioned in the above two bullets is configured per set

A UE does not expect an SRS resource to be configured in both codebook and antenna switching resource sets with conflicting (disabled state) configuration for the port 1003.

Proposal 1: Support 3T3R by configuring SRS resource set according to the existing agreement on RRC parameter signaling on 3T6R in RAN1#119.

- Conclusions

The document discussed enhancememnts targeting codebook transmission to enable up to 3 Tx UL operation. Based on the observations, the proposals are presented as below.

Proposal 1: Support 3T3R by configuring SRS resource set according to the existing agreement on RRC parameter signaling for 3T6R in RAN1#119.

- References

[1] RP-23400 - New WID: NR MIMO Phase 5, Edinburgh, Scotland, December 11-15, 2023.