3GPP TSG-RAN WG1 Meeting #120	  R1-2500151

Athens, Greece, February 17 – 21, 2025

Agenda Item:	9.1.4.1

Source:	Huawei, HiSilicon

Title:	Discussion on AI/ML for CSI compression

Document for:	Discussion and Decision

# Introduction

In RAN#106 plenary meeting [1], a new SID on AI/ML for air-interface was approved for the remaining Release 19, where the study objectives on AI/ML for CSI compression are given below:

The following areas will be further studied as part of this project: - CSI feedback enhancement [RAN1]: - For CSI temporal prediction: further update TR 38.843 with additional evaluations. - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. ….

This contribution mainly discusses issues related with alleviating inter-vendor training collaboration and the necessary specification impact for CSI compression cases. In addition, this contribution would provide some introduction of extending the CSI compression use case for TDD scenarios.

# Inter-vendor training collaboration issues

In the RAN1#118bis meeting, the following agreements and conclusion on directions for on-device operation and UE side offline engineering are agreed to be studied further.

Conclusion For issues listed for inter-vendor collaboration Direction A, conclude the following - Answer to Issue 2: From RAN1 perspective, at least, - For option 4-1, there is no proprietary information concern of disclosing encoder input/output from NW side to UE side - For option 3a-1, there is no proprietary information concern of disclosing encoder parameters from NW side to UE side - Note: If some additional information is disclosed from network side to UE side, proprietary concerns for option 4-1 and 3a-1 can not be precluded by default. Agreement For issues listed for inter-vendor collaboration direction B, further study the overhead in Direction B based on the size of the encoder (i.e., number of parameters and quantization level), the number of encoders, and how often the parameters need to be transferred. Agreement For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary. Agreement - For Direction C: - It is clarified that, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side. - Study whether the fully specified reference models or their compatible models may provide reasonable performance for deployment at NW and UE - Including performance evaluation of training/finetuning the models by modeling data distribution mismatch between synthetic and field data, and/or by using actual field data - Including performance evaluation of training/finetuning the models by modelling data distribution mismatch between NW-side field data and UE-side field data, and/or by using actual field data from both NW-side and UE-side - Note: Training/finetuning is done at either or both sides without inter-vendor collaboration. - FFS whether the reference model is the encoder (1-1) or decoder (1-2) or both (1-3) - FFS whether the reference model is the same as RAN4 specified model - FFS whether a single or multiple reference (and how many) models need to be specified Agreement For the evaluation studies of Direction A, - For encoder parameter exchange from NW-side to UE-side of 3a-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder against the exchanged encoder. - Step 2: UE-side develops actual encoder against the nominal decoder - Alt. 2: UE-side directly develops and trains the actual encoder. - Note: Target CSI is available at UE side - For dataset exchange from NW-side to UE-side {target CSI, CSI feedback} of 4-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder using the exchanged dataset. - Step 2: UE-side develops an actual encoder against the nominal decoder. - Alt. 2: UE-side directly develops and trains the actual encoder.

Some of the above issues for different directions are discussed in the following sections.

## Analysis on training methods subject to Direction A

As the further discussions of Direction A focus on Option 4-1 and Option 3a-1, we only provide analysis on these two options. Before analyzing the specific issues, we will first demonstrate the procedure and methods for training under Direction A.

Step 1: Training at NW side. This step is common to all Option 3a-1/4-1 of Direction A. NW side would first train the end-to-end model and obtain the corresponding dataset as showed in the below figure, including: Encoder 1, Decoder 1, Target CSI V1, CSI feedback C1, Reconstructed Target CSI V1.

Step 2: Training at UE side. This procedure is different for different options, and will be analyzed separately in a later part.

Step 3: Deploy models for inference. Encoder 2 at UE and Decoder 1 at NW are deployed to the network for inference.

Figure 1 Example of steps for training collaboration subject to Direction A

In the following, we will elaborate the methods for Step 2 under different options.

Methods for training at UE side under Option 4-1

The dataset exchanged from the NW side to UE side may consist of (Target CSI V1, CSI feedback C1). The following 2 alternatives may be considered for UE side training.

- Alt.1 (train nominal Decoder first): Step 1: UE side trains a nominal Decoder 2 with <C1, V1> as the <model input, label>. Step 2: UE side trains its own Encoder 2 jointly with the frozen Decoder 2. For Alt.1, since the NW side training in Step 1 also adopts V1 as the label, the trained nominal Decoder 2 could achieve similar mapping relationship of model input and output as NW side Decoder 1. Regarding the dataset used for training Step 2, Target CSI V' can be adopted, and there can be two sub alternatives for V'.

  - Alt.1-1 (NW side dataset): Adopt NW side delivered V1 as V'.

  - Alt.1-2 (UE side dataset): UE side adopts a new dataset of V2 as V', which may include new data generated from UE side. E.g., UE side may generate the new dataset for new UEs which reflects particular distribution of UE side additional conditions while cannot be represented by the NW side dataset V1.

- Alt.2 (train Encoder directly): UE side trains its own Encoder 2 with <V1, C1> as the <model input, label>.

Figure 2 Example of training at UE side under Option 4-1

Observation 1: For inter-vendor collaboration Option 4-1, Alt. 1: UE-side first trains a nominal decoder, after receiving the dataset of (Target CSI, CSI feedback) from NW side, UE side may first train a nominal decoder using (CSI feedback, Target CSI) as (input, label) and then train its encoder jointly with the nominal decoder using Target CSI as input and label.

In the following, we simulate the performance of Alt.2 where one NW part decoder pairing with M=2 different UE part encoders. UE side trains its own Encoder 2 with either the same dataset as the NW side Dataset A or Dataset B which is a subset of Dataset A. UE part Encoder 2 has the same TF model backbone but different model structure from the NW part Encoder 1. It can be seen that the actual UE part Encoder 2 can well pair with the NW part actual Decoder 1.

Table 1 Evaluation results for various training methods

| Cases | Dataset used at UE | Encoder at UE | Decoder at NW | SGCS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 60bits | 120bits | 240bits |
| E1-D1 joint training | Dataset A (300K) | Encoder 1 | Decoder 1 | 0.767 | 0.861 | 0.918 |
| E2-D1 separate training, Opt 4-1 Alt.2 | Dataset A (300K) | Encoder 2 | Decoder 1 | 0.765 | 0.860 | 0.918 |
| E2-D1 separate training, Opt 4-1 Alt.2 | Dataset B (100K) | Encoder 2 | Decoder 1 | 0.762 | 0.858 | 0.917 |

Observation 2: For inter-vendor collaboration Option 4-1, Alt. 2: UE-side directly develops and trains the actual encoder, without considering dataset mismatch issue between NW side and UE side, the actual encoder trained by UE side can pair with the actual decoder trained by NW side.

Methods for training at UE side under Option 3a-1

The parameters exchanged from the NW side to UE side correspond to Encoder 1 generated by the NW side. The following 2 alternatives may be considered for UE side training.

- Alt.1 (train nominal Decoder first): Step 1: UE side trains a nominal Decoder 2 jointly with the frozen Encoder 1 using dataset of Target CSI V'. Step 2: UE side trains its own Encoder 2 jointly with the frozen Decoder 2 using Target CSI V'. Regarding Target CSI V', similar to Option 4-1, there can be two sub alternatives.

  - Alt.1-1 (NW side dataset): NW side delivers the NW side dataset of V1 to UE as V'.

  - Alt.1-2 (UE side dataset): UE side adopts a new dataset of V2 as V', which may include new data generated from UE side.

- Alt.2 (train Encoder directly): Step 1: UE side first generates a dataset consisting of <Target CSI V', CSI feedback C2> as the <model input, model output> by using Encoder 1. Step 2: UE side trains its own Encoder 2 with <V', C2> as the <model input, label>. Regarding Target CSI V', similar to Option 4-1, there can be two sub alternatives.

  - Alt.2-1 (NW side dataset): NW side delivers the NW side dataset of V1 to UE as V'.

  - Alt.2-2 (UE side dataset): UE side adopts a new dataset of V2 as V', which may include new data generated from UE side.

Figure 3 Example of training at UE side under Option 3a-1

## Issues related with the directions

### Additional information issues for Direction A

In the RAN1#117 meeting, the following agreements was achieved:

Agreement - For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact. - Exchange - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a) - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4) - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4) - Performance target (3a/5a/4) - Dataset or information related to collecting dataset (3a/5a) - Any other additional information - Model pairing (3a/3b/4/5a) - UE capability (3a/3b/4/5a) - Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a) - Quantization of feedback (3a/3b/4/5a) - Model structure details (3a/3b) Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both. - Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters.

In the following, we will elaborate the performance target and dataset related information.

Performance target (Option 3a-1/4-1)

Since the UE side will perform offline engineering for the training of the UE-part model, it needs to know the anticipated performance of the NW-side so that UE side may choose appropriate model backbone/structure and training method. E.g., for relatively low performance, UE side may choose a model structure with low complexity (e.g., CNN backbone), while for relatively high performance, UE side should choose a model structure with high complexity (e.g., Transformer backbone). From NW perspective, it needs to ensure the trained UE part model can achieve decent performance target before it enables the CSI compression feature.

Such performance target could be end-to-end performance metric if UE side prefers to train Encoder 2 and Decoder 2 with end-to-end manner (e.g., Alt.1 of Option 3a-1/4-1), encoder only performance metric if UE side prefers to directly train Encoder 2 only (e.g., Alt.2 of Option 3a-1/4-1), or decoder only performance metric if UE side prefers to first train a nominal Decoder 2 (e.g., Alt.1 of Option 4-1).

For the type of performance metric, we may consider at least SGCS/NMSE which are widely evaluated, or study other metrics such as MSE, MAE.

In addition, it should be noted that the performance target is only valid for using the testing dataset shared from NW, e.g., Target CSI V1, CSI feedback C1. For the training methods where UE side adopts new dataset of V2 for training and testing, the performance target is not applicable, since there is no insurance or awareness to NW side on the size, distribution, and quality for the UE side new dataset which are probably collected by UE side implementation. E.g., if UE side selects a small overfitting testing dataset to calculate the performance target, the reported performance target may be over-estimated compared with its real performance during the inference with the NW.

Proposal 1: For additional information of Direction A (Option 3a-1/4-1), study the performance target including at least the following aspects:

- End-to-end performance metric (e.g., Alt.1 of Option 3a-1/4-1), encoder only performance metric (e.g., Alt.2 of Option 3a-1/4-1), and/or decoder only performance metric (e.g., Alt.1 of Option 4-1).

- The type of performance metric.

- Testing dataset used for metric calculation to test the performance target.

  - Note: the performance target is only applicable for the testing dataset shared from NW.

Model backbone information (Option 4-1)

Regarding the model backbone or model structure information, as analyzed in the “Performance target” part, such information is not needed since the UE side can autonomously select appropriate model structure based on the guidance of performance target information delivered by NW side.

Proposal 2: For additional information of Direction A Option 4-1, model backbone information is not needed since the UE side can autonomously select appropriate model structure based on the guidance of performance target information indicated by NW side.

Dataset including Target CSI (Option 3a-1)

As analyzed in the above procedures, for Option 3a-1, there are two candidates for dataset generation: Alt.1-1/2-1 (NW side dataset) and Alt.1-2/2-2 (UE side dataset). One controversial issue discussed during the previous meeting is whether the NW side Target CSI V1 is needed for Option 3a-1.

From UE side training perspective, if V1 is not provided to UE side, i.e., the dataset V2 for UE side training is totally collected by UE side, that means the UE side dataset training the UE side actual encoder may be mismatched with the NW side dataset training the actual decoder. E.g., V1 may include data (namely V1 which is non-overlapped with V2) for which the property is unseen from V2, so that the data distribution of V1 cannot be learnt by the UE side actual encoder. Consequently, the UE trained Encoder 2 may fail the pairing with Decoder 1 of the gNB subject to the data of during the inference phase. Taking an extreme example, if the UE side V2 is collected to be a small overfitting dataset, Encoder 2 and Decoder 2 (if any) are trained based on this overfitting dataset, the trained Encoder 2 may also be overfitting even though the delivered Encoder 1 is generalized. Such mismatched dataset can be understood as a generalization issue (i.e., generalization Case 2) as has been widely evaluated in Rel-18.

From performance testing perspective, the Target CSI V1 can include the testing dataset served for testing the performance target as analyzed previously. If V1 is not provided to UE side, it is meaningless to test the target performance of the UE side trained model based on the UE side’s own dataset which is not known by the NW side. E.g., UE side may adopt an overfitting testing dataset in V2 to derive good performance target, while the trained actual encoder may still fail the pairing the NW side decoder.

From the discussion of the last meeting, it was raised by some company that the training dataset and/or testing dataset used for UE side training/testing can be autonomously performed by UE side data collection using dataset ID. However, it is our view that the data samples in the training dataset/testing dataset generated at the NW side is not only collected from the field, but also performed with data processing such as data cleaning, data augmentation, etc., so that improved quality of the data samples is achieved to optimize the performance. In contrast, if the training and/or testing dataset is autonomously collected by UE side, it would be more complex from both spec perspective and UE side implementation perspective to align the dataset construction information, e.g., list of cells for collecting data, number of data samples for per cell, quality of data, methods for data processing, etc.

In summary, it is our understanding that the dataset of Target CSI should be shared from NW side to UE side for Option 3a-1 to ensure model pairing and target performance testing.

Proposal 3: For additional information of Direction A Option 3a-1, the dataset of at least Target CSI should be also shared from NW side to UE side to ensure:

- Aligned dataset for training NW part Decoder and UE part Encoder to avoid generalization problem.

- Aligned dataset for metric calculation to test the performance target at UE side.

- Note: it would be more complex from both spec perspective and UE side implementation perspective to align the dataset construction information if assuming UE side to autonomously collect training and/or testing dataset.

### Data distribution mismatch issues for both Direction A and B

For Direction A, an issue of dataset mismatch was discussed during the previous meetings. Taking Option 4-1 for example, for Alt.1-2 (UE side dataset), UE side may want to adopt a new UE side dataset V2 generated from new UEs for which the data has not been collected by NW side and therefore is not part of the NW side dataset V1. Thus, the trained UE part Encoder 2 is more overfitted to the new UEs.

However, even though the UE side trains its Encoder 2 with the new UE side dataset V2, (assuming V2 does not include V1) the trained Encoder 2 may fail the pairing with NW side Decoder 1 since the NW side Decoder 1 was not trained based on V2. In addition, the newly collected UE side dataset V2, due to insufficient NW sites from where it is generated, may not be able to cover the data property of the NW side dataset V1. That means V2 may not be generalized to the NW sites at the inference phase.

In addition, from evaluation perspective, how to model the UE side factor to the channel is not quite clear. The impact factors of UE side additional condition include RF modules, baseband processing, etc. For the TxRU mapping of UE, it is possible to model with different receiving antenna spacing, but as we evaluated over NW side TxRU mappings in Rel-18, generalized performance can be achieved by dataset mixing; for UE side TxRU, a similar observation is expected to be also applicable. For baseband processing, since the methods of channel estimation, SVD/EVD decomposition, etc., are strongly subject to implementation, it seems difficult over companies to set up aligned EVM as far as we know. Thus, the impact to the data distribution can hardly be cross checked.

Actually, as long as the NW side collects sufficiently diverse data, the dataset V1 would be generalized enough to the new UE side data regardless of the UE side additional conditions.

Last, it should be noted that the mismatch issue can be alleviated to a large extent considering the NW side would persistently perform NW side data collection. E.g., after new UEs with new UE side additional condition are launched, NW side data collection can be performed to collect the new data and incorporate them into the NW side training dataset V1'. After the NW side training with the updated dataset V1', the UE side can obtain the updated V1' (and its corresponding CSI feedback/reconstructed Target CSI) for training the UE part model. In that way, both the UE part model and the NW part model can be fit to the new UE side dataset.

For Direction B, it is similar understanding that the necessity and feasibility to resolve the data distribution mismatch issue may be questionable.

Proposal 4: Regarding the data distribution mismatch issue of Direction A/B, the necessity to resolve it may need careful justification, considering:

- The mismatch issue may not exist considering NW side collect sufficiently diverse training dataset which could be generalized to the new UE side data.

- The mismatch issue (if exists) can be alleviated to a large extent with timely dataset update by NW, where the NW side would perform NW side data collection from the new UEs and update the delivered dataset/model/parameters to represent the new UE side data.

### Overhead concern issues for both Direction A and B

As analyzed previously, the following information may at least be delivered for Direction A:

- For Option 4-1, the delivered information includes (Target CSI V1, CSI feedback C1).

- For Option 3a-1, the delivered information includes Target CSI V1 and parameters corresponding to the CSI generation part.

The following information may at least be delivered for Direction B (Option 3b): the CSI generation part.

Overhead alleviation for dataset delivery of Option 3a-1/4-1

As analyzed earlier, besides Option 4-1, Option 3a-1 also need dataset delivery of at least Target CSI from NW side to UE side. Therefore, we will analyze the dataset delivery as a common issue for Direction A.

In RAN1#117 meeting, it has been agreed that dataset delivery is considered at RAN1 by only focusing on air-interface delivery, i.e., from NW to UE. Thus, we only discuss over-the-air solution.

Agreement From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset

From the discussions in Rel-18, one concern of dataset delivery over-the-air is the enormous size of the training dataset that may lead to an excessive UE power consumption and air-interface overhead.

However, it should be clarified that the overall dataset is not necessarily sent from a single gNB to a single UE. On the other hand, the original dataset can be split into subsets each with a limited number of data samples. Considering that the model training currently at UE side is usually performed at a non-3GPP entity belonging to the UE vendor, each UE may only need to receive one subset of the original dataset and the non-3GPP entity can recombine all subsets received and uploaded by many UEs to recover the original dataset, which is then used for model training. All subsets of the original dataset are associated with a common dataset ID to facilitate the dataset re-combination.

As shown in Figure 4, for NW first training, the NW side can split the overall dataset into K*N subsets, each of which is delivered from a gNB to a UE, that is, the overall dataset can be delivered by K gNBs, each sending N subsets to N UEs. Thus, the delivered dataset size per UE can be reduced by K*N times. Assuming K*N=10000 UEs are used to share this dataset delivery and the size of the overall dataset is 40 MB by using Rel-16 eType II-like quantization method, the per UE overhead is only 4KB which is comparable to a piece of RRC message; it contributes a negligible overall overhead especially when considering per month/per week/per day level dataset delivery frequency.

Figure 4 Dataset delivery over massive UEs

Regarding the procedure for dataset delivery, since the dataset delivery and retraining at UE side is non real time and overhead non-sensitive, there is no need to specify sophisticated procedure to particularly avoid redundant transmission of subsets, or handle retransmission of subsets. For a dataset with a unique dataset ID, the UE side can combine its component subsets at the OTT server; if any component subsets are still missed from its OTT server, the UE side can (via its UEs) request the NW to transmit them, until all the component subsets have been completely received at the OTT server.

Proposal 5: For the dataset sharing for Option 3a-1/4-1 over air-interface, study the solution to relieve the overhead. E.g.,

- NW splits the overall dataset into many subsets each with a limited number of data samples (e.g., with an overhead affordable by the RRC signaling). The subsets can be separately sent to different UEs, and all subsets are associated with a common dataset ID for the UE side re-combination.

Regarding the dataset sharing from UE to OTT server, it can be also achieved with non real time manner. Thus, the UEs can upload the received subset of dataset shared by NW to its own OTT server via offline manner, e.g., WiFi when available, without additionally consuming uplink air-interface resources. This behavior is similar as UE-sided model for beam management or positioning.

Observation 3: For the dataset sharing for Option 3a-1/4-1 over air-interface, UEs can upload the received subset of the dataset shared by NW to its own OTT server via offline manner without additionally consuming UL air-interface resources, which is similar to other use cases of UE-sided model.

Overhead analysis between Option 3a-1 and Option 4-1

From the above analysis on delivered information for options, Option 3a-1 needs to deliver the parameter as well as Target CSI, while Option 4-1 needs to deliver the CSI feedback as well as Target CSI. Thus, to compare the overhead between parameter delivery and dataset delivery, we only need to compare the parameter part and the CSI feedback part and omit the common part of Target CSI.

To compare the payload size of parameters and CSI feedback, we assume the parameters are quantized in FP32, and the number of parameters is around 5M (referring to the complexity results of Rel-18 CSI compression from TR 38.843), then the total payload size is around 160Mbits; on the other hand, for CSI feedback, we assume the number of data samples is around 500k (an averaged value referring to the spread sheet of CSI_Table 1 of TR 38.843), while the payload size is 300bits per data sample (corresponding to payload size Z), then the total payload size is around 150Mbits, which is comparable with parameter delivery.

Observation 4: For the overhead analysis of Direction A, the overhead between Option 3a-1 and Option 4-1 may be comparable.

Overhead analysis between Direction A and Direction B

A difference between Direction A and Direction B is that, the parameter and/or dataset delivery subject to Direction A only transmitted once for each UE side for per trained model, while the parameter delivery subject to Direction B needs to be repeatedly performed for each UE supporting Option 3b for per trained model. As a typical example of Direction A, after NW side trains a new model or generates a new dataset, it can deliver the parameter/dataset to the UE side via over-the-air manner for this single model; after that, UE side can train the UE part model and download the trained model to UEs with offline implementation manner, which does not consume air-interface resources.

In contrast, Direction B works with plug-and-play manner, so after NW side trains a new model, it needs to deliver the parameters of the new model to each single UE that has not received it. That means, the overall consumed overhead under Direction B is proportional with the number UEs to deploy the trained model, and is much larger than Direction A.

Observation 5: For the overhead analysis of Direction A/B, the overall consumed air-interface overhead subject to Direction A (Option 3a-1/4-1) is much less than Direction B, since parameter delivery of Direction B needs to be repeatedly performed to each UE for per trained model.

### Proprietary disclosure issues for both Direction A and B

For parameter delivery (Option 3a-1/3b), the exact model is directly exposed to the other side. In contrast, dataset delivery (Option 4-1) shares the information of input/output mapping relationship, from which it cannot directly deduce the exact model – the UE side still needs its own proprietary training to derive the model for inference. Compared with data information, the model is kind of algorithm and more proprietary in our understanding.

Observation 6: For the proprietary disclosure analysis of Direction A/B, sharing the model information (Option 3a-1/3b) incurs more risk of NW side proprietary disclosure than sharing the dataset information (Option 4-1).

### Issues for Direction C

Dataset for reference model(s) training

One issue related with Direction C is whether to consider statistical channel model or field data for training the reference model(s). From our understanding, it spends super large effort to manually collect field data compared with generating the data from a simulator following 3GPP channel model. In addition, different NW vendors and test UEs may have different configurations and implementations, which makes it infeasible to calibrate the collected dataset.

In addition, as far as we know, the evaluation on the testing model performed at RAN4 is also based on 3GPP statistical channel.

Observation 7: For the dataset for reference model(s) training of Direction C, it is not practical to collect field data for training the reference model(s).

Data distribution mismatch issue

Another issue is, whether there is mismatch between the distribution of the training dataset and inference dataset. From our understanding, this issue exists due to the following two reasons.

On one hand, considering the dataset used for training the reference models(s) is possibly the synthetic data rather than field data, but the data for model inference in real network is field data with different NW side conditions/additional conditions such as TxRU mapping, deployment scenario, port number, CSI payload size, etc., the training dataset may not cover the property of the field data, so the trained model may not be generalized to the field inference data.

On the other hand, the update of the reference model(s) depends on the evolution of specifications, which means that before the frozen of the next release, the old model based on out-of-date training dataset has to be used by the NW and UE, and therefore the mismatch issue cannot be resolved. Even though the NW side and UE side can train their proprietary model to separately pair with the specified models, it should be noted that the performance is upper bounded by the end-to-end specified models. In contrast, for Direction A/B, the model/parameter or dataset can be timely updated based on collected field data.

Observation 8: For Direction C, the performance is negatively impacted due to mismatch between the synthetic dataset used for reference model(s) training and the field data for inference.

- The mismatch issue persistently exists till the reference model(s) are updated based on the evolution of specifications.

## Potential standard discussions for Option 3/4

In RAN1#118 meeting, the model input and output related parameters are considered in the following agreement.

Agreement For studying the standardization of model structure RAN1, RAN1 assumes at least the following: - Precoding matrix as an input (as opposed to raw channel matrix) - Per-layer processing, with common structure across ranks and layers (corresponding to Option 2-1, 2-2, 3-1, or 3-2 for handling rank ≥ 1) - For temporal domain aspects Case 2 and Case 3, strive to reuse the model structure of Case 0 where appropriate, with additional layers or operations either at the input/output domain or at the latent domain. - For Case 0, use precoding matrix, e.g., eigen vector in spatial-frequency domain, and/or angular, delay domain representation such as eType-II W2. - For Case 2 and Case 3, use precoding matrix, e.g., spatial-frequency domain for each CSI observation instance, and/or angular, delay, and Doppler domain representation such as eType-II W2. - Consider scalability approaches over numbers of Tx ports, CSI feedback payload sizes, and bandwidths.

### Model structure related standard discussion on Option 3

In RAN1#119 meeting [2], the guidance to study model structure has been agreed.

Agreement For studying the standardized model structure, - For temporal domain Case 0, - In case of spatial-frequency domain input, adopt Transformer as the backbone structure. - Further study angular-frequency and angular-delay domain input - For temporal domain Case 2, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation with additional layers - Conv-LSTM - LSTM - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots - Note: other options are not precluded - Further study angular-frequency and angular-delay domain input - For temporal domain Case 3, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation - Option 2: Latent adaptation - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input. - Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure. - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered. - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used.

Deriving optimized model structure(s) of Option 3 needs evaluations efforts to compare and cross check on the performance of model structure(s). To save effort on the evaluation and future product development, we should strive to perform down selection to these parameters so that a single generalized and scalable model structure along with pre/post processing approach could be achieved.

- For rank>1, there 4 options: layer specific and rank common (Option 2-1), layer specific and rank specific (Option 2-2), layer common and rank common (Option 3-1), and layer common and rank specific (Option 3-2). From the specification perspective, we do not need to specify the type of rank>1 option, but from evaluation perspective, we need to align on one solution for cross check. Our preference is to prioritize Option 3-1 for simplicity, since the data distribution over layers are not so diverse.

- For the generalization aspects, at least various UE indoor/outdoor distributions and various UE speeds should be considered for the mixed dataset, as a unified model is preferred at least within a cell to be generalized to these factors which may vary over UEs and over time.

- For the scalability aspects, truncation/padding and adaptation layer are adopted in the Rel-18 evaluations.

  - For various CSI payload sizes, the number of companies adopting adaptation layer and truncation/padding in Rel-18 is half/half from TR 38.843; our flavor is to adopt adaptation layer. The specific number of candidate payload sizes should also be determined, and partial omission should be considered.

  - For various bandwidths, we should consider some typical bandwidth values ranging from 5MHz to 100MHz.

  - For various Tx ports, the number of companies adopting truncation/padding in Rel-18 is the majority from TR 38.843, so we prefer to consider truncation/padding as the pre-processing.

- For the quantization/dequantization method for CSI feedback, we have evaluated scalar quantization (SQ) and vector quantization (VQ) in Rel-18. Our flavor is to adopt VQ with quantization-aware training for better performance.

Proposal 6: For studying the standardized model structure of inter-vendor collaboration Option 3, strive to perform down selection to the factors (e.g., rank>1 options, generalization aspects, scalability approaches, quantization/dequantization method, etc.) so that a single model structure along with pre/post processing approach is aligned before starting the evaluation.

### Dataset related standard discussion on Option 4

For the potential spec impact of Option 4, we may further study the following aspects.

- Data sample format related aspects:

  - The type of ground-truth CSI, e.g., spatial frequency domain or angular-delay domain eigenvectors, channel matrix (as will be analyzed in Section 4), etc.

  - The dimension of input (port number, bandwidth, etc.)/output (CSI payload size).

  - CSI feedback related information: 1) The type of the CSI feedback as the label for training encoder or as the input for training decoder, e.g., the CSI feedback is before or after quantization. 2) Segmentation information of the CSI feedback with respect to different layers and segmentation of per layer.

- Dataset construction related aspects: (1) Number of data samples in the dataset. (2) Dataset ID. (3) Dataset split/segmentation for overhead saving, as discussed in Section 2.2.3. (4) Association between ground-truth CSI and CSI feedback which serve as input/label and label/input, respectively, for training encoder/decoder. (5) For temporal domain Case 2 and Case 3, the observation window/prediction window information.

- Scalability information: The construction information of data samples for a set of input/output scalability configuration, which would be preferred by NW during the later inference phase.

- Performance target information, as discussed in Section 2.2.1.

Proposal 7: For studying the standardized dataset format of inter-vendor collaboration Option 4, consider at least the following aspects:

- Data sample format related aspects, e.g., type of ground-truth CSI, the dimension of input/output, CSI feedback related information.

- Dataset construction related aspects, e.g., number of data samples, dataset ID, dataset split/segmentation, association between ground-truth CSI and CSI feedback subject to a data sample, observation window/prediction window information for temporal domain cases.

- Scalability information.

- Target performance information.

# Potential specification impact for CSI compression cases

Aspects of data collection, monitoring, inference and the necessary specification impact for the prioritized temporal domain CSI compression Case 2 and Case 3 have been discussed in previous meetings, but the necessity/feasibility/details of some issues are still controversial. In this section, we will continue the discussion of these controversial issues.

## NW side data collection

In the RAN1#119 meeting, the following agreements on NW side data collection are agreed to be studied further.

Agreement For NW to collect data for training, study following spec impacts - Data format: codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction. - FFS if enhancement is needed - FFS number of samples in the report. - FFS whether channel or precoder is needed for temporal Cases 3 - Configuration of rank/layer, number of subbands - Mechanism for ground-truth reporting - FFS: Report additional information regarding the samples, e.g., data quality, FFS the definition of data quality and corresponding parameters. - FFS if enhancements in CSI-RS and SRS configuration is needed. - FFS: Report associated information that captures UE side additional condition - FFS: Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI - FFS: details of CSI measurement For UE to collect data for training, study following spec impacts - NW configuration or UE request, e.g., RS configuration/transmission for data collection - Whether enhancements in CSI-RS configuration is needed. - Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI - FFS: Need of configuration of ID, and configuration of ID

From the Rel-18 observation captured in TR 38.843, it can be seen that the majority of companies have considered precoding matrix as the model input type (which can be also regarded as ground-truth CSI/Target CSI). Besides, considering the model input type for the temporal domain CSI compression Case 3 which is prioritized in Rel-19 as well as the model input type for supporting the fusion of SRS measurement and CSI feedback under TDD as analyzed in Section 4, raw channel matrix can also be considered as ground-truth CSI type for the NW side data collection.

Regarding the quantization format, both Rel-16 eType II CB based quantization with legacy/new parameters and scalar quantization have been evaluated in Rel-18:

- For model training, majority companies show that ground-truth CSI format of Rel-16 eType II CB with new/larger parameter(s) outperforms Rel-16 eType II CB with legacy parameter (PC8) by obvious gain (0.7%~5.4% SGCS gain), and is almost close to the upper-bound performance of Float32. On the other hand, the overhead of Rel-16 eType II CB with new/larger parameter(s) is significantly reduced (94%~97.5%) than unquantized ground-truth CSI of Float32. Considering the training data collection is non-real time (days/months level frequency), the average overhead for training data collection is negligible.

- For model monitoring, majority companies show that ground-truth CSI format of Rel-16 eType II CB with new/larger parameter(s) outperform legacy parameter (PC8) in general, e.g., by 12.2%~76% monitoring accuracy gain under threshold 0.02. As opposed to scalar quantization, the spec effort of expanding the Rel-16 eType II CB for quantization has relatively smaller spec effort and smaller overhead. On the other hand, considering the monitoring data collection may be infrequently triggered, the average overhead for monitoring data collection is also negligible.

With respect to the above analysis, the ground-truth CSI format of Rel-16 eType II CB with new/larger parameter(s) could be considered for model training and model performance monitoring. The candidate parameters for expansion can refer to the Rel-18 evaluation.

It should be noted that, though the Rel-18 evaluations on quantization format focused on the ground-truth CSI type of precoding matrix, the quantization to the raw channel matrix can follow the similar approach of angular-delay domain conversion. For example, the dimension of precoding matrix to be quantized is [Tx ports, Subbands, Layers], while the dimension of the channel matrix is [Tx ports, Subbands, Rx antennas]. Considering full layer report of precoding matrix, it has the same dimension as the raw channel matrix, and thus the “Layers” dimension can be simply replaced with the “Rx antennas” dimension.

Proposal 8: For the NW side data collection, confirm the necessity and feasibility of UE report of the ground-truth CSI.

- For the data type, consider the following:

  - Precoding matrix

  - Channel matrix

- For the data format, prioritize Rel-16 eType II CB based quantization with new parameters for both above data types, and take the following new parameters as candidates for discussion:

  - L= 8, 10, 12; pv = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits.

## Monitoring

In the RAN1#117 meeting, the following agreements on model monitoring are agreed to be studied further.

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

In the following of this section, we will discuss our views on the above monitoring types as illustrated in below.

| Case 1 NW-side monitoring based on the Target CSI reported by the UE |  | Case 2-0 UE-side monitoring based on the output of the CSI reconstruction model indicated by the NW |  |
| --- | --- | --- | --- |
| Case 2-1 UE-side monitoring based on the output of the CSI reconstruction model at the UE | Case 2-2 UE-side monitoring based on direct estimation of intermediate KPI/other KPIs |  | Case 2-3 UE-side monitoring based on precoded RS |

Figure 5 Illustration on the procedures for the monitoring cases

### NW-side monitoring

Ground-truth CSI based monitoring (Case 1)

For NW side monitoring, NW can calculate the intermediate KPI between the ground-truth CSI (i.e., Target CSI with realistic channel estimation) reported by UE and the recovery CSI. As the accuracy of measured ground-truth CSI is testable at RAN4, the reliability of the monitored results could be guaranteed regardless of the change of environment.

For the reporting format, similar to NW side data collection, eT2-like high-resolution codebook is preferred due to better monitoring performance. By using eT2-like high-resolution codebook rather than legacy eT2 codebook, the ground-truth CSI is reported more precisely.

For the reporting mode, both per sample reporting and reporting of a number of monitored samples can be considered. For the type of intermediate KPI, the preference between SGCS and NMSE may be related to the flavor of the input type between precoding matrix and channel matrix, however, as the metric is calculated at the NW side, there is no need to specify the metric type.

Regarding the manner of reporting, we can further study per sample reporting, or the reporting of a number of samples of ground-truth CSIs.

Proposal 9: For NW-side monitoring, consider ground-truth CSI based monitoring in Rel-19.

- eT2-like high-resolution codebook for reporting format with higher priority.

- Further discuss the reporting mode, e.g., per sample reporting and reporting of a number of monitored samples.

### UE-side monitoring

Reconstructed CSI based monitoring (Case 2-0)

This case is denoted as Case 2-0. For UE side monitoring, UE can calculate the intermediate KPI between the ground-truth CSI and the recovery CSI (i.e., output of the CSI reconstruction model) indicated by NW. Similar as analyzed in the above NW-side ground-truth CSI based monitoring, eT2-like high-resolution codebook for reporting format and SGCS for the type of intermediate KPI are likely used to deliver the recovery CSIs.

Regarding the manner of reporting, as the indication signaling is highly likely to be RRC signaling, we may consider the indication of a number of samples of recovery CSIs. Therefore, a down side of this solution is the large latency compared with NW-side monitoring.

Moreover, as the inter-vendor collaboration Option 4-2 is deprioritized, Case 2-0 may have the same problem of proprietary disclosure risk of the NW side decoder related information.

Proxy model based monitoring (Case 2-1/2-2)

This type of monitoring includes Case 2-1 and Case 2-2, where both the direct estimation of intermediate KPI and other KPIs are categorized to Case 2-2. Both of Case 2-1 and Case 2-2 are based on a proxy model at UE. It should be noted that the proxy model has the following drawbacks:

- Generalization performance of the proxy model. Since the proxy CSI reconstruction model is different from the actual NW part CSI reconstruction model, e.g., with smaller size/simpler architecture and weaker learning capability, the generalization performance of the proxy model will be different from (e.g., worse than) the actual NW part CSI reconstruction model. For example, when the test scenario changes, the actual CSI reconstruction model may still work well (even though it has not been trained with the unseen data subject to the test scenario) but the proxy model will degrade significantly and therefore fail to mimic the actual model. As shown in the Rel-18 evaluation, the proxy model has an imbalanced generalization performance from the NW side CSI reconstruction part model and its monitoring accuracy would degrade when the scenario changes from UMa to InH. That is to say, the reliability of the proxy model cannot be guaranteed.

- Additional LCM of the UE side proxy model. Considering the proxy model is used to monitor the performance of actual model, then a coming up issue is how to monitor and manage the proxy model. Since the proxy model is one-sided model at UE side, one potential way is that it is up to UE implementation and transparent to the NW side; otherwise, for a CSI compression feature, it will bring a huge burden at the NW side to additionally identify/monitor/manage the proxy model besides the actual two-sided models being monitored. Due to this, even if the proxy model is adopted, it is very likely that it will operate under level x collaboration manner, i.e., the UE side proxy model is transparent to NW side.

  - In particular, different from other use cases with one-sided model (such as BM and PoS) where the UE can monitor the UE side model by using the measured label and model input, for CSI compression, the UE cannot obtain the end-to-end SGCS without receiving the recovery CSI from the gNB; it means the UE cannot monitor the proxy model either, due to lack of labels.

Observation 9: The imbalanced generalization performances between the proxy model (Case 2-1/2-2) at UE and the actual CSI reconstruction part at NW will lead to a degraded monitoring accuracy at the UE side when the channel environment changes.

Observation 10: UE side proxy model (Case 2-1/2-2) is likely to operate under collaboration level x, since its additional LCM will impose huge burden on NW, including model/functionality identification, monitoring, activation/deactivation/switching/fallback, etc., of the UE side proxy model. Without such additional LCM, the performance and robustness of the proxy model are not trustable at NW.

- In particular, how to monitor the performance of the UE side proxy model is not clear.

In summary, from monitoring perspective, the proxy model (either to mimic the CSI reconstruction part at NW or to directly output the intermediate KPI) can be operated under level x manner, and there is no need to consider specification impact on the proxy model.

Proposal 10: There is no strong motivation for specifying the UE side proxy model (Case 2-1/2-2) for monitoring.

Precoded RS based monitoring (Case 2-3)

This case is denoted in our contribution as Case 2-3. UE can use the precoded RS to calculate the eventual KPI (e.g., L1-RSRP, hypothetical BLER, etc.). On top of that, UE may compare the AI/ML-based eventual KPI with a legacy non-AI/ML solution applied with the legacy precoder/PMI.

This option is overhead friendly, as it may avoid delivery of ground-truth CSI from UE to NW, or recovery CSI from NW to UE. On the other hand, it may reflect the eventual performance of hypothetical BLER, but may not reflect the intermediate KPI (e.g., SGCS) of the model - note that the eventual performance may not only be impacted by the accuracy of the model but also other factors such as the channel status.

Proposal 11: For UE-side monitoring, consider precoded RS (transmitted from NW based on the output of the CSI reconstruction model) based monitoring in Rel-19.

- Further study the type of monitoring KPI calculated from the precoded RS.

## Inference

### Quantization methods for CSI report

In Rel-18, it has been identified that the quantization alignment is needed between UE and NW, either via model pairing or via standardized quantization scheme, and it has been studied with two quantization schemes: scalar quantization (SQ) and vector quantization (VQ). For quantization alignment using standardized quantization scheme, it includes configuration/reporting/updating of the quantization related codebook/parameters. For SQ, the configuration of quantization granularity and the quantization range can be further studied. For VQ, as it is usually trained together with the AI/ML models and it is hard to specify a fixed dictionary, a realistic way for vector quantization is to indicate its dictionary (including format and size) to the other side. E.g., for training collaboration Option 3a-1/3b, the NW side can send the dictionary to the UE side in together with the delivered model. For training collaboration Option 4-1, the NW side performing the first step training can send the dictionary to the UE side which performs the second step training, while the opposite side only trains the model but keeps the dictionary unchanged. As the dimension of the direct output of the encoder (i.e., the latent space) could be large, it is unlikely to generate the VQ dictionary with super long VQ vector/super large codebook size to directly map the whole latent space; in contrast, it would be beneficial to further study the segmentation of the latent space, so that the long latent space can be split to segments, each of which is then mapped to the VQ dictionary with short VQ vectors.

In addition, considering the varying channel status, it is also possible that the AI/ML model is unchanged but the dictionary is updated and indicated to the other side to adapt to the varying channel, e.g., only reconfigure quantization granularity for SQ, or only re-indicate the dictionary for VQ.

Proposal 12: For quantization methods of the CSI report, further study potential specification impact on quantization alignment using standardized quantization scheme.

- For vector quantization,

  - Configuration/reporting/updating of the quantization dictionary.

  - Segmentation of the CSI generation model output to map with short VQ vector.

- For scalar quantization,

  - The configuration of the quantization granularity/range.

### CQI determination

For AI/ML-based CSI compression, UE does not have the CSI reconstruction part and thus does not know the output channel matrix/eigenvector at the NW. Therefore, Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) should be the starting point. A straightforward way is that the UE adopts the original eigenvectors for CQI calculation which is different from what will be recovered by NW. Such misalignment between the original eigenvectors and recovered eigenvectors would lead to a CQI misalignment between NW and UE, and the CQI calculated by UE would be overestimated. To report a more accurate CQI, a simple way is that UE compensates the CQI calculated with the original CSI before being input to the model. As the UE may not have information of the recovery CSI, the CQI compensation can be derived based on some NW indicated assistance, e.g., NW to indicate a previous output-CSI-UE to UE for calculating the compensation value.

Proposal 13: For the study of CQI determination in inference, consider Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) as a starting point.

### CSI reporting principles

In legacy CSI reporting framework, the priority rules among different CSI report is based on periodicity, type of content, cell ID, report configuration ID, etc. If AI/ML based CSI feedback is specified, the priority rule may need to differ whether it is AI/ML based CSI report or legacy codebook based CSI report. Moreover, for AI/ML based CSI feedback, some AI/ML specific contents should be considered for the priority rule, e.g., ground-truth CSI, inference CSI report, monitoring metrics, etc. In addition, for the priority rules within single AI/ML based CSI report for model inference, since the content is a bit sequence without any physical meaning, how to specify the priority rules of this bit sequence to achieve a similar effect of Group 0/1/2 of legacy CSI codebook needs further study.

For CSI processing unit, the legacy determination method is based on the reportQuantity and number of CSI-RS resources in the CSI-RS resource set; e.g., different OCPU values are specified for the quantity types of ‘none’, RSRP/SINR, and CQI/PMI/RI. For AI/ML based CSI feedback, as different UE part models may have different complexity or equivalently required CPU values, it may hardly specify a fixed value of CPU for the AI/ML based CSI compression report. As a candidate method, the UE can report and calculate the needed OCPU values for per UE part model basis. For those UE part models without sufficient CPU, the corresponding CSI reports could be dropped.

For CSI mapping, it has been agreed that the framework of two parts CSI is considered.  For inference CSI report, CSI part 1 includes information representing the part 2 size, while CSI part 2 includes the content of the CSI generation part output. From our view, as the size of the of the CSI generation part output is impacted by selected rank value, quantization granularity, selected CSI payload size, etc., these factors can be carried by CSI part 1. For the mapping of CSI part 2, as the content of the CSI generation part output is a bit sequence without physical meaning, how to map CSI part 2 should be further studied.

Proposal 14: For CSI report in inference, on top of the legacy CSI reporting principles, the following AI/ML specific aspects may be additionally studied:

- The CSI priority rules, e.g., priority rules by considering the AI/ML specific reporting type, priority rules within the bit sequence of per AI/ML specific inference CSI report.

- The CSI processing unit (CPU), e.g., the required CPU value may consider difference of UE part model complexity.

- The CSI mapping, e.g., factors representing the part 2 size in CSI part 1, mapping of the CSI generation part output in CSI part 2, etc.

### Rank>1 options

In Rel-18, 6 Rank>1options have been discussed: Option 1-1 (rank specific), Option 1-2 (rank common), Option 2-1 (layer specific and rank common), Option 2-2 (layer specific and rank specific), Option 3-1 (layer common and rank common), and Option 3-2 (layer common and rank specific).

From the perspective of CSI feedback format, since the content is a bit sequence without any physical meaning, there is no need to differentiate which part of the sequence is corresponding to which layer. Thus, a unified solution can be applicable irrespective of the Rank>1 options.

From the LCM perspective, it is straightforward that Option 3-1 (layer common and rank common) adopts only a single model for all ranks/layers, so the LCM (e.g., activation/deactivation/monitoring) is applied to this single model. For Option 3-2 (layer common and rank specific), Option 2-1 (layer specific and rank common), and Option 2-2 (layer specific and rank specific), it needs to further discuss whether these layer/rank specific models are handled separately or as handled commonly/as a whole. Though different models may be applied for handling of different layers and/or rank values, from NW perspective, it mainly needs to be aware of the impact to the CSI timeline, which could be reported accompanied with the model during the model pairing phase; if different models are applied for the inference to different layer/rank situations, as implementation, UE may simply report the maximum timeline. Therefore, from specification perspective, the model(s) can be handled as a whole, and a unified solution is considered irrespective of the Rank>1 options.

From the inter-vendor training collaboration perspective, standardized model/structure (i.e., inter-vendor training collaboration Option 1/3, respectively) may need to take into account Rank>1 options to the model structure discussion. As analyzed in earlier section, Option 3-1 can be prioritized for simplicity. For standardized dataset (i.e., inter-vendor training collaboration Option 4), on the other hand, as long as the physical meaning (e.g., rank value, layer index, Rx index) of the Target CSI as well as the corresponding CSI feedback is indicated, how to develop the Rank>1 option can be UE implementation.

Proposal 15: For Rank>1 options, further study the following LCM aspects:

- From inference perspective, there is no need to specify the Rank>1 option.

- From LCM perspective (e.g., activation/deactivation/monitoring), there seems no need to specify the Rank>1 option.

- From inter-vendor training collaboration perspective,

  - For Option 1/3, a specific Rank>1 option should be adopted for specifying the model structure, e.g., Option 3-1 (layer common and rank common).

  - For Option 4, there is no need to specify the Rank>1 option.

## Potential specification impact for the temporal domain cases

In the RAN1#118 meeting, the following agreement is achieved for prioritizing temporal domain Case 2 and Case 3.

Agreement For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3.

In the following, we will analyze the potential spec impact to Case 2 and Case 3.

### Temporal domain CSI compression Case 2

Similar to the principle of discussing EVM, the potential spec impact of the extended cases should focus on the additional aspects on top of Rel-18 AI/ML based SF CSI compression for which the potential spec impacts have been widely discussed.

The performance of CSI compression Case 2 (namely AI-TSF) may degrade if no special handling is performed to cope with the UCI missing. One candidate to resolve this issue is to specify the signaling to re-align the accumulated CSI, e.g., triggering the reset of accumulated CSI.

As analyzed in the previous section, the accumulated CSI information between the encoder and the decoder will be mismatched if the gNB fails to receive the UCI reported from UE, and such mismatched CSI accumulation may impact the inference for the next/future time instance(s). To evaluate how large impact such mismatch will cause to the eventual performance of throughput, we consider the following cases.

- Case 1 (benchmark#1): Rel-16 eType II CB. We do not model UCI missing for this case. The throughput of Case 1 is assumed as 100%, while throughputs of other cases are normalized by taking Case 1 as the baseline.

- Case 2 (benchmark#2): Rel-18 AI-SF. This benchmark is adopted to evaluate whether the performance loss of AI-TSF would degrade the performance to be lower than the benchmark of SF domain CSI compression.

- Case 3: AI-TSF without considering UCI missing. This can be achieved when UL link is robust, e.g., when the TPC and beta-offset are configured with reliable values.

- Case 4: AI-TSF with UCI missing, and no special handling is performed. As mentioned in the previous section, 10% UCI missing rate is assumed for each individual UL CSI feedback. The NW side accumulated CSI and UE side accumulated CSI would be mismatched after UCI missing occurs.

- Case 5: AI-TSF with UCI missing, and solution to re-align the accumulated CSI for both sides are considered. In realistic network, the re-alignment can be triggered by gNB, as the gNB can identify the UCI missing by verifying the CRC of UCI. In this simulation, CSI buffer resetting is considered.

Table 2 illustrates that AI-TSF with 10% UCI missing rate will cause performance loss compared with the AI-TSF without UCI missing (but would still slightly outperform AI-SF). On the other hand, by introducing solutions for re-aligning the accumulated CSI between NW side and UE side, the performance loss can be significantly alleviated.

Table 2 Evaluation results for AI-TSF considering UCI missing (Rank 1, full buffer, CSI payload = 240bits)

| Cases | Throughput |
| --- | --- |
| Case 1: Rel-16 eType II CB, no UCI missing | 100% |
| Case 2: Rel-18 AI-SF, no UCI missing | 112% |
| Case 3: AI-TSF, no UCI missing | 136% |
| Case 4: AI-TSF with UCI missing (10% missing rate), no special handling | 113% |
| Case 5: AI-TSF with UCI missing (10% missing rate), reset accumulated CSI information at both sides after UCI missing | 127% |

Observation 11: For the evaluation of temporal domain CSI compression Case 2:

- When UCI missing is considered, performance loss is observed compared with the case without modeling UCI missing due to mismatched accumulated CSI information between NW side and UE side.

- When adopting solutions to re-align the accumulated CSI information for both sides, the performance loss due to the UCI missing can be largely alleviated.

Other aspects for AI-TSF can reuse the potential spec impact discussed for Rel-18 SF CSI compression to a large extent, e.g., data collection, inference, monitoring, etc.

Proposal 16: For the additional potential spec impact of temporal domain CSI compression Case 2 on top of Rel-18 SF domain CSI compression, consider methods to handle the misalignment of the accumulated CSI between NW part model and UE part model due to UCI missing, e.g., NW indicating the reset of historical CSI, or indicating CSI retransmission.

### Temporal domain CSI compression Case 3

In the RAN1#119 meeting, two options are agreed for further study for data collection and monitoring of Case 3.

Agreement For temporal domain aspects Case 3, study LCM aspects and specification impacts, consider the following options for training data collection - Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s). - Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s). - Note: During inference, the input to the CSI generation part is derived based on the predicted CSI. consider following options for the monitoring labels - Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s). - CSI prediction output is used as input to CSI generation part. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. - Option 2: The monitoring label is derived based on the measured CSI of the future slot(s) - Option 2a: CSI prediction output is used as input to CSI generation part. - Note: This corresponds to end-to-end monitoring of CSI prediction and compression. - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

From inter-vendor training collaboration perspective, the potential solutions for JPC (joint prediction and compression) and SPC (separate prediction and compression) are analyzed. For JPC, it is not clear whether we would assume a single model to perform prediction and compression (i.e., with input of measured CSI and output of compressed prediction CSI) or assume two separate models to perform prediction and compression, respectively. The former case may lead to higher complexity in implementation, while the latter case may cause higher standard effort as it needs to specify model structures and/or dataset format separately for the compression model and the prediction model. For SPC, on the other hand, the development of the CSI prediction model can be left to UE implementation, while the standard efforts of inter-vendor training collaboration may only focus on the compression model. Therefore, we may take SPC with higher priority in the remaining study.

From training data collection perspective, for SPC, Option 1 (based on future predicted CSI) can be considered with higher priority as the type of Target CSI. Considering the CSI compression model takes future predicted CSI as model input during inference phase, taking the predicted CSI as Target CSI for training data collection can ensure consistent type of Target CSI between training and inference, and therefore avoid generalization issue in case the data distribution of the predicted CSI bias from the measured CSI.

From monitoring perspective, for SPC, similar to the training data collection, to avoid mismatch between monitoring label and inference input, we can consider Option 1 (based on future predicted CSI) with higher priority as the type of label.

Proposal 17: For the additional potential spec impact of temporal domain CSI compression Case 3 on top of Rel-18 SF domain CSI compression, consider SPC (separate prediction and compression) with higher priority.

- Regarding the type of Target CSI for training data collection, consider Option 1 (based on future predicted CSI) with higher priority, which can ensure consistent type of Target CSI between training and inference.

- Regarding the type of monitoring label for monitoring, consider Option 1 (based on future predicted CSI) with higher priority, which can ensure consistent type of Target CSI between monitoring label and inference input.

# Extension of CSI compression for TDD scenarios

The past study of Rel-18/19 two-sided CSI compression has been mainly focusing on the FDD massive MIMO case. E.g., the evaluation assumes 2GHz carrier frequency, 10MHz/20MHz bandwidth, and 16/32 Tx ports. Considering the wide deployment of TDD massive MIMO in realistic network, it will strongly drive the commercialization of the AI/ML CSI compression feature if it can provide promising gain to TDD massive MIMO.

## Motivation

In legacy TDD massive MIMO system, the DL precoder can be generated based on either CSI feedback or SRS measurement.

DL precoder based on SRS measurement

SRS measurement has been widely used to generate DL precoder in legacy. However, in realistic network, the SRS resource for per UE is typically configured with sparse manner in time domain considering the following factors:

- SRS frequency hopping. To ensure SRS coverage, frequency hopping is typically configured, so that only partial bandwidth is occupied for SRS transmission at per hop. E.g., assuming 17 hops are configured, gNB needs 17 periods to obtain the full DL CSI by SRS measurement.

- Antenna switching for DL CSI acquisition. Considering more than 1 DL Rx antenna (e.g., 2R/4R), multiple SRS resources in different symbols are needed for per UE.

- SRS multiplexing over UEs. Frequency domain/CS domain/comb domain multiplexing have been adopted to enable SRS multiplexing over UEs. However, considering large number of UEs in per cell and inter-cell SRS interference avoidance, time domain multiplexing is also needed, thus per UE SRS resources are not densely configured.

With respect to the sparse time domain SRS resource configuration, it may take long time duration for DL full bandwidth channel acquisition at gNB. E.g., for 100MHz BWP, assuming SRS is configured with 40ms period and 17 frequency hops, gNB needs 680ms to obtain the full bandwidth CSI, which causes severe channel aging issue. Increasing SRS resource density can shorten the latency and alleviate channel aging issue, but it will occupy the UL resources which could be used for other UL channels such as PUSCH/PUCCH, and lead to low efficient UL resource usage under TDD.

DL precoder based on CSI feedback

Another legacy approach for DL CSI acquisition is CSI feedback by UE report. As shown in the table in 38.214, the frequency granularity of the PMI feedback is more coarse under large BWP size. For FDD system with typically 10MHz/20MHz BWP, precise PMI can be addressed with small subband size of 4 PRBs. However, if CSI feedback is used for TDD scenarios with up to 100MHz BWP, precision of DL precoding is limited under more coarse PMI report granularity of 16/32 PRBs.

Table 3 Frequency granularity for PMI feedback for various bandwidths

| Bandwidth part (PRBs) | CQI/PMI(R=1) subband size (PRBs) |
| --- | --- |
| 24 – 72 | 4, 8 |
| 73 – 144 | 8, 16 |
| 145 – 275 | 16, 32 |

Introducing smaller frequency granularity of PMI report can improve precision of CSI obtained at NW, but the price is higher burden of UE complexity, power consumption and larger CSI overhead.

Based on the analysis above, either SRS measurement based manner or CSI feedback based manner has downsides to achieve optimized DL precoder accuracy.

Observation 12: For the approaches to generate DL precoding in legacy

- DL precoder based on SRS measurement may suffer severe channel aging issue considering long latency for full bandwidth DL CSI acquisition in real network due to sparse SRS resources for per UE.

  - Configuring dense SRS resources to shorten the latency can alleviate channel aging issue, but it will lead to low efficient UL resource usage.

- DL precoder based on CSI feedback may have limited precision due to coarse frequency domain CSI granularity under large TDD bandwidth.

  - Introducing smaller frequency granularity of CSI measurement can improve precision of CSI obtained at NW, but that incurs higher UE complexity, CSI overhead, and power consumption as the price.

## AI/ML solution description

From the analysis above, we observe the CSI feedback approach and SRS measurement approach can be complementary to each other:

- From latency perspective, CSI feedback can provide full/large bandwidth CSI with shorter latency, while SRS measurement suffers channel aging issue due to long latency for full bandwidth acquisition.

- From frequency domain CSI precision perspective, SRS based measurement can provide CSI with RE level granularity, while CSI feedback only provides coarse granularity.

To take advantage of the pros of both approaches and make use of CSI information from both approaches, fusion of SRS measurement and AI/ML compressed CSI feedback is brought up to extend AI/ML CSI compression to well improve TDD scenarios.

The model description is depicted in Figure 6. In general, the model consists of an Encoder at UE side and a Decoder at NW side both with Transformer backbone, which is similar to the architecture for CSI compression in FDD. The main differences are shown as following.

- Model input of the Decoder. To enable fusion, the Decoder includes two model inputs: SRS measurement and CSI feedback.

  - The input type of SRS measurement is channel matrix of per PRB. Measured CSIs from all frequency hops are combined to constitute the full bandwidth SRS measurement. Thus, the input dimension of SRS measurement is [PRB number, Tx port number, 2].

  - The CSI feedback is used as another input. Since it is generated by the compression of the Encoder, the type of CSI feedback is a bit sequence without physical meaning, and the dimension of the CSI feedback depends on the configured CSI payload size. To enable the fusion with the SRS measurement, the dimension of the CSI feedback is further processed inside the Decoder before performing the multi-head attention.

- Model input of Encoder. To enable the fusion of SRS measurement and AI/ML compressed CSI, the two information should have the same type. Hence, channel matrix is also considered as the input type of the Encoder, instead of precoding matrix which is widely used for AI/ML CSI compression for FDD scenarios. Note that UE side precoding matrix calculated from CSI-RS measurement may largely bias from the NW side precoding matrix calculated from SRS measurement, so using precoding matrix as model input may lead to failed fusion. In addition, legacy subband size is reused for CSI feedback. Thus, the input dimension of Target CSI for Encoder is [Subband number, Tx port number, 2].

- Model output of Decoder. Since the model input of Encoder and Decoder are all subject to channel matrix, the output of Decoder is also channel matrix. The output dimension is [PRB number, Tx port number, 2]. Based on the recovered channel matrix, NW will further perform SVD decomposition to derive the DL precoder.

Figure 6 Model structure of CSI compression for TDD scenarios

From the above elaborations, fusion is performed at Decoder, while the impact to the Encoder is similar to the FDD case other than that the input type is replaced with channel matrix.

Observation 13: For TDD scenario, fusion of SRS measurement and AI/ML compressed CSI feedback can be performed to optimize the accuracy of the DL precoder.

- For Decoder, fusion is performed to the SRS measurement and the CSI feedback (i.e., output of Encoder) before performing multi-head attention.

- For Encoder, channel matrix is assumed as the model input to enable the fusion with SRS measurement, while other impacts are similar to the AI/ML CSI compression for FDD.

## Evaluation assumptions and evaluation result

The TDD specific evaluation assumptions are provided in the following table. Other evaluation assumptions follow TR38.843. We consider 4 cases:

- Case 1 (Benchmark#1): SRS measurement only. gNB generates the DL precoder based on only the SRS measurement as legacy.

- Case 2 (Benchmark#2): PMI feedback only with Rel-16 eType II CB. gNB generates the DL precoder based on only the PMI fed back from UE as legacy. We consider Rel-16 eType II CB as the benchmark, since other AI/ML CSI compression methods (e.g., SF domain CSI compression, TSF domain CSI compression Case 2/3) are still under study. The CB parameters follow the Rel-19 MIMO agreement for up to 128 ports.

- Case 3: Fusion of SRS measurement and AI/ML compressed CSI feedback. For alignment with the two benchmarks, the SRS configuration (i.e., period and hops) is the same as Case 1, and the CSI feedback payload size is the same as Case 2.

- Case 4 (Upper bound): Ideal CSI. As ideal assumption, gNB generates the DL precoder based on the genie-aided CSI at the time of precoder generation. This case is used to assess the margin of potential performance improvement.

Table 4 TDD specific evaluation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 3.7 GHz |
| Antenna configuration of gNB | 64 Tx ports: (12,8,2,1,1,4,8) |
| Antenna configuration of UE | 4Rx: (1,2,2,1,1,1,2) |
| Simulation bandwidth | 100MHz, 272PRBs |
| SRS configuration | 40ms period, 17 hops |

The evaluation results are shown as follow, including the 4 cases mentioned above. Numerical spectral efficiency is used as intermedia KPI for fair comparison between recovery CSI types of precoding matrix and channel matrix.

It can be observed that the performance of SRS measurement only and PMI feedback only is largely restricted in comparison with the ideal CSI case (53% and 44% of the ideal CSI performance, respectively) due to the downsides analyzed earlier. In contrast, fusion of SRS measurement and AI/ML compressed CSI feedback can improve the performance significantly, which achieves 30% gain over SRS measurement only and 60% gain over PMI feedback only.

Figure 7 Evaluation results for fusion of SRS measurement and AI/ML CSI feedback under TDD

Observation 14: From the evaluation results under TDD scenario, DL CSI acquisition based on fusion of SRS measurement and AI/ML compressed CSI feedback can significantly improve performance with

- 30% gain over legacy method based on SRS measurement only, and

- 60% gain over legacy method based on PMI feedback only with Rel-16 eType II CB.

## Potential specification impact

As mentioned in Section 4.2, fusion of SRS measurement and AI/ML compressed CSI feedback is performed at the Decoder which is NW side implementation. Compared with AI/ML CSI compression for FDD scenario where precoding matrix may be used as the type of Target CSI, the major difference for the fusion solution under TDD scenario is to specify channel matrix as the type of Target CSI.

Based on the analysis above, we propose to enable the fusion solution for TDD scenario by supporting channel matrix as the type of Target CSI for AI/ML CSI compression.

Proposal 18: To enable the fusion of SRS measurement and AI/ML compressed CSI for performance improvement of TDD scenario, support channel matrix as the type of Target CSI for AI/ML CSI compression.

# Conclusions

In this contribution, we discussed issues related with alleviating inter-vendor training collaboration, the specification impact for CSI compression cases, and fusion of SRS measurement and AI/ML compressed CSI for TDD scenarios. Based on the discussions, we have the following observations and proposals.

Observation 1: For inter-vendor collaboration Option 4-1, Alt. 1: UE-side first trains a nominal decoder, after receiving the dataset of (Target CSI, CSI feedback) from NW side, UE side may first train a nominal decoder using (CSI feedback, Target CSI) as (input, label) and then train its encoder jointly with the nominal decoder using Target CSI as input and label.

Observation 2: For inter-vendor collaboration Option 4-1, Alt. 2: UE-side directly develops and trains the actual encoder, without considering dataset mismatch issue between NW side and UE side, the actual encoder trained by UE side can pair with the actual decoder trained by NW side.

Observation 3: For the dataset sharing for Option 3a-1/4-1 over air-interface, UEs can upload the received subset of the dataset shared by NW to its own OTT server via offline manner without additionally consuming UL air-interface resources, which is similar to other use cases of UE-sided model.

Observation 4: For the overhead analysis of Direction A, the overhead between Option 3a-1 and Option 4-1 may be comparable.

Observation 5: For the overhead analysis of Direction A/B, the overall consumed air-interface overhead subject to Direction A (Option 3a-1/4-1) is much less than Direction B, since parameter delivery of Direction B needs to be repeatedly performed to each UE for per trained model.

Observation 6: For the proprietary disclosure analysis of Direction A/B, sharing the model information (Option 3a-1/3b) incurs more risk of NW side proprietary disclosure than sharing the dataset information (Option 4-1).

Observation 7: For the dataset for reference model(s) training of Direction C, it is not practical to collect field data for training the reference model(s).

Observation 8: For Direction C, the performance is negatively impacted due to mismatch between the synthetic dataset used for reference model(s) training and the field data for inference.

- The mismatch issue persistently exists till the reference model(s) are updated based on the evolution of specifications.

Observation 9: The imbalanced generalization performances between the proxy model (Case 2-1/2-2) at UE and the actual CSI reconstruction part at NW will lead to a degraded monitoring accuracy at the UE side when the channel environment changes.

Observation 10: UE side proxy model (Case 2-1/2-2) is likely to operate under collaboration level x, since its additional LCM will impose huge burden on NW, including model/functionality identification, monitoring, activation/deactivation/switching/fallback, etc., of the UE side proxy model. Without such additional LCM, the performance and robustness of the proxy model are not trustable at NW.

- In particular, how to monitor the performance of the UE side proxy model is not clear.

Observation 11: For the evaluation of temporal domain CSI compression Case 2:

- When UCI missing is considered, performance loss is observed compared with the case without modeling UCI missing due to mismatched accumulated CSI information between NW side and UE side.

- When adopting solutions to re-align the accumulated CSI information for both sides, the performance loss due to the UCI missing can be largely alleviated.

Observation 12: For the approaches to generate DL precoding in legacy

- DL precoder based on SRS measurement may suffer severe channel aging issue considering long latency for full bandwidth DL CSI acquisition in real network due to sparse SRS resources for per UE.

  - Configuring dense SRS resources to shorten the latency can alleviate channel aging issue, but it will lead to low efficient UL resource usage.

- DL precoder based on CSI feedback may have limited precision due to coarse frequency domain CSI granularity under large TDD bandwidth.

  - Introducing smaller frequency granularity of CSI measurement can improve precision of CSI obtained at NW, but that incurs higher UE complexity, CSI overhead, and power consumption as the price.

Observation 13: For TDD scenario, fusion of SRS measurement and AI/ML compressed CSI feedback can be performed to optimize the accuracy of the DL precoder.

- For Decoder, fusion is performed to the SRS measurement and the CSI feedback (i.e., output of Encoder) before performing multi-head attention.

- For Encoder, channel matrix is assumed as the model input to enable the fusion with SRS measurement, while other impacts are similar to the AI/ML CSI compression for FDD.

Observation 14: From the evaluation results under TDD scenario, DL CSI acquisition based on fusion of SRS measurement and AI/ML compressed CSI feedback can significantly improve performance with

- 30% gain over legacy method based on SRS measurement only, and

- 60% gain over legacy method based on PMI feedback only with Rel-16 eType II CB.

Proposal 1: For additional information of Direction A (Option 3a-1/4-1), study the performance target including at least the following aspects:

- End-to-end performance metric (e.g., Alt.1 of Option 3a-1/4-1), encoder only performance metric (e.g., Alt.2 of Option 3a-1/4-1), and/or decoder only performance metric (e.g., Alt.1 of Option 4-1).

- The type of performance metric.

- Testing dataset used for metric calculation to test the performance target.

  - Note: the performance target is only applicable for the testing dataset shared from NW.

Proposal 2: For additional information of Direction A Option 4-1, model backbone information is not needed since the UE side can autonomously select appropriate model structure based on the guidance of performance target information indicated by NW side.

Proposal 3: For additional information of Direction A Option 3a-1, the dataset of at least Target CSI should be also shared from NW side to UE side to ensure:

- Aligned dataset for training NW part Decoder and UE part Encoder to avoid generalization problem.

- Aligned dataset for metric calculation to test the performance target at UE side.

- Note: it would be more complex from both spec perspective and UE side implementation perspective to align the dataset construction information if assuming UE side to autonomously collect training and/or testing dataset.

Proposal 4: Regarding the data distribution mismatch issue of Direction A/B, the necessity to resolve it may need careful justification, considering:

- The mismatch issue may not exist considering NW side collect sufficiently diverse training dataset which could be generalized to the new UE side data.

- The mismatch issue (if exists) can be alleviated to a large extent with timely dataset update by NW, where the NW side would perform NW side data collection from the new UEs and update the delivered dataset/model/parameters to represent the new UE side data.

Proposal 5: For the dataset sharing for Option 3a-1/4-1 over air-interface, study the solution to relieve the overhead. E.g.,

- NW splits the overall dataset into many subsets each with a limited number of data samples (e.g., with an overhead affordable by the RRC signaling). The subsets can be separately sent to different UEs, and all subsets are associated with a common dataset ID for the UE side re-combination.

Proposal 6: For studying the standardized model structure of inter-vendor collaboration Option 3, strive to perform down selection to the factors (e.g., rank>1 options, generalization aspects, scalability approaches, quantization/dequantization method, etc.) so that a single model structure along with pre/post processing approach is aligned before starting the evaluation.

Proposal 7: For studying the standardized dataset format of inter-vendor collaboration Option 4, consider at least the following aspects:

- Data sample format related aspects, e.g., type of ground-truth CSI, the dimension of input/output, CSI feedback related information.

- Dataset construction related aspects, e.g., number of data samples, dataset ID, dataset split/segmentation, association between ground-truth CSI and CSI feedback subject to a data sample, observation window/prediction window information for temporal domain cases.

- Scalability information.

- Target performance information.

Proposal 8: For the NW side data collection, confirm the necessity and feasibility of UE report of the ground-truth CSI.

- For the data type, consider the following:

  - Precoding matrix

  - Channel matrix

- For the data format, prioritize Rel-16 eType II CB based quantization with new parameters for both above data types, and take the following new parameters as candidates for discussion:

  - L= 8, 10, 12; pv = 0.8, 0.9, 0.95; reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits.

Proposal 9: For NW-side monitoring, consider ground-truth CSI based monitoring in Rel-19.

- eT2-like high-resolution codebook for reporting format with higher priority.

- Further discuss the reporting mode, e.g., per sample reporting and reporting of a number of monitored samples.

Proposal 10: There is no strong motivation for specifying the UE side proxy model (Case 2-1/2-2) for monitoring.

Proposal 11: For UE-side monitoring, consider precoded RS (transmitted from NW based on the output of the CSI reconstruction model) based monitoring in Rel-19.

- Further study the type of monitoring KPI calculated from the precoded RS.

Proposal 12: For quantization methods of the CSI report, further study potential specification impact on quantization alignment using standardized quantization scheme.

- For vector quantization,

  - Configuration/reporting/updating of the quantization dictionary.

  - Segmentation of the CSI generation model output to map with short VQ vector.

- For scalar quantization,

  - The configuration of the quantization granularity/range.

Proposal 13: For the study of CQI determination in inference, consider Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) as a starting point.

Proposal 14: For CSI report in inference, on top of the legacy CSI reporting principles, the following AI/ML specific aspects may be additionally studied:

- The CSI priority rules, e.g., priority rules by considering the AI/ML specific reporting type, priority rules within the bit sequence of per AI/ML specific inference CSI report.

- The CSI processing unit (CPU), e.g., the required CPU value may consider difference of UE part model complexity.

- The CSI mapping, e.g., factors representing the part 2 size in CSI part 1, mapping of the CSI generation part output in CSI part 2, etc.

Proposal 15: For Rank>1 options, further study the following LCM aspects:

- From inference perspective, there is no need to specify the Rank>1 option.

- From LCM perspective (e.g., activation/deactivation/monitoring), there seems no need to specify the Rank>1 option.

- From inter-vendor training collaboration perspective,

  - For Option 1/3, a specific Rank>1 option should be adopted for specifying the model structure, e.g., Option 3-1 (layer common and rank common).

  - For Option 4, there is no need to specify the Rank>1 option.

Proposal 16: For the additional potential spec impact of temporal domain CSI compression Case 2 on top of Rel-18 SF domain CSI compression, consider methods to handle the misalignment of the accumulated CSI between NW part model and UE part model due to UCI missing, e.g., NW indicating the reset of historical CSI, or indicating CSI retransmission.

Proposal 17: For the additional potential spec impact of temporal domain CSI compression Case 3 on top of Rel-18 SF domain CSI compression, consider SPC (separate prediction and compression) with higher priority.

- Regarding the type of Target CSI for training data collection, consider Option 1 (based on future predicted CSI) with higher priority, which can ensure consistent type of Target CSI between training and inference.

- Regarding the type of monitoring label for monitoring, consider Option 1 (based on future predicted CSI) with higher priority, which can ensure consistent type of Target CSI between monitoring label and inference input.

Proposal 18: To enable the fusion of SRS measurement and AI/ML compressed CSI for performance improvement of TDD scenario, support channel matrix as the type of Target CSI for AI/ML CSI compression.

# References

RP-243245, “New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Madrid, Spain, December 9-12, 2024.

RAN1 Chair’s Notes, RAN1#119, Orlando, US, November 18-22, 2024.