3GPP TSG-RAN WG1 Meeting #120	  R1-2500152

Athens, Greece, February 17 – 21, 2025

Agenda Item:	9.1.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on other aspects of the additional study for AI/ML

Document for:	Discussion and Decision

- Introduction

In RAN#106 plenary meeting [1], a new SID on AI/ML for air-interface was approved for the remaining Release 19, where the study objectives on AI/ML general framework are given below:

Study objectives: … - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study …

Therefore, this contribution discusses the 3 study objectives: model identification for two-sided models, model transfer/delivery, and UE side training data collection.

- Model identification for two-sided models

In the RAN1#116 meeting, the following agreement and observation have been achieved for categories of model identification options.

Agreement - To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases. - To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose - Note: other options are not precluded Observation The other options are proposed for model identification type B by companies during the discussion: - MI-Option 4. Model identification via standardization of reference models. (for CSI compression) - MI-Option 5. Model identification via model monitoring

From the following agreement from RAN1#116bis, MI-Option 1 with AI-Example1 is applicable to one-sided model use case, e.g., beam management, where associated ID is introduced later to alleviate the consistency between training and inference.

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) ……

For two-sided model, however, from the agreements of the CSI compression agenda, the model is trained at the NW side (e.g., for MI-Option 3), or first trained at the NW side (e.g., for MI-Option 2), so the data collection and data categorization is to be performed at the NW side rather than the UE side. UE side receives the model/parameter/dataset delivered from the NW side which already represent the NW-side additional condition. Then, UE side may either directly perform inference or retrain the UE part model, neither of which needs the obtainment of associated ID.

From the WID, model identification is revised to be discussed only for two-sided model, so there is no need to further discuss MI-Option 1 in 9.1.4.2.

Proposal 1: There is no need to further discuss MI-Option 1 in 9.1.4.2, since MI-Option 1 is applicable only to one-sided model cases, while the scope of model identification in revised WID is limit to two-sided case, for which the UE side does not need to be provided with NW-side additional condition since the data collection is performed by NW side for NW-side training or NW-first training.

And in the RAN1#118 meeting, the following conclusions have been achieved.

Conclusion The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work. Conclusion The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work.

In the following, we will analyze the MI-Option 2, 3 and 4 separately, for two-sided model.

  - MI-Option 2: Two-sided model identification with dataset transfer

In the RAN1#117 meeting, the following example of MI-Option 2 are agreed to be studied further.

Agreement From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset

To facilitate RAN2 discussion on the procedure, RAN1 may be able to provide some inputs on the content of information to be interacted between NW and UE.

Taking NW first training for example, the NW side trains the NW side decoder and the nominal encoder (i.e., NW side CSI generation part). Afterwards, the NW will deliver the content of the dataset for training the UE part model, including the input and output of the nominal encoder of the NW side. The input and output of the NW side nominal encoder can be used to train the UE part encoder as input and label, respectively.

Besides the content of the dataset, other meta information may at least include: dataset ID, size of dataset, type/format of data samples, association between ground-truth CSI (e.g., serves as input) and CSI feedback (e.g., serves as label), model scalability information, quantization method for CSI feedback, dataset split/segmentation information, and target performance information.

In RAN1#119 meeting [2], the following agreement on ID-X is agreed for the further study of model pairing.

Agreement For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, - ID-X can be used for pairing the UE-part and the NW-part of a two-sided model - FFS: other information needed for pairing

For the procedure of MI-Option 2, the model identification is achieved when the dataset ID (i.e., “ID-X”) is delivered in together with the delivered dataset during the inter-vendor collaboration phase. During the inference phase, the UE can report the supported dataset ID(s) to the gNB to achieve the pairing. E.g., UE may report one or multiple dataset ID(s) to the gNB which could be paired with the CSI reconstruction part at the gNB, and gNB will activate one dataset ID among them for inference. Alternatively, gNB may provide the information of preferred dataset ID(s) to UE, and UE reports the applicability of the dataset ID(s); afterwards, gNB will activate one dataset ID among them for inference.

It is our understanding that the “ID-X” is interpreted as dataset ID, and there is no need to redundantly introduce other notations such as model ID or pairing ID.

Proposal 2: For the transmitted information of MI-Option 2, if the dataset is delivered from NW side to UE side, the following information may be needed:

- Content of model input and label for training the UE side CSI generation part.

- Other meta information, including at least: dataset ID, size of dataset, type/format of data samples, association between ground-truth CSI and CSI feedback subject to a data sample, scalability information, quantization method for CSI feedback, dataset split/segmentation information, and target performance information.

Proposal 3: For the procedure of MI-Option 2, the model identification, the “ID-X” could be interpreted as dataset ID.

  - MI-Option 3: Model identification with model transfer from NW to UE

In the RAN1#117 meeting, the following example of MI-Option 3 are agreed to be studied further.

Agreement From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

As we analyzed for the applicable case, model identification is applicable to two-sided model. The feasibility and specific case for model delivery from NW to UE can refer to our discussion at Section 3.1. This section focuses on the content and procedure of model transfer.

Taking Case z4 for example, the content includes the list of model parameters. Besides the content of parameters, other meta information may at least include: model ID, format of the parameters, model structure information, quantization method and parameters. Note that the model structure information is applicable when multiple model structures are aligned between NW side and UE side, either with standardized manner or offline manner as analyzed in Section 3.1.

For the procedure of MI-Option 3, the model identification is achieved when the model ID is delivered in together with the delivered model. During the inference phase, similar to MI-Option 2, one alternative is that the UE reports the supported model ID(s) to the gNB to achieve the pairing, and another alternative is that gNB provides the information of preferred model ID(s) to UE, and UE reports the applicability of the model ID(s).

Regarding Alt. A (UE proactive) and Alt. B (UE reactive) for the model delivery procedure, both alternatives can enable the end-to-end system workable. Considering the detailed signaling and handshake between UE and NW may mostly higher layer based, it may be better discussed at RAN2 before making potential down-selection.

Proposal 4: For the transmitted information of MI-Option 3, taking Case z4 for example, the following information may be needed:

- Model parameters.

- Other meta information, including at least: model ID, format of the parameters, model structure information, quantization method and parameters.

Proposal 5: For the procedure of MI-Option 3, the model identification is achieved when the model ID is delivered in together with the delivered model.

MI-Option 4: Model identification via standardization of reference models/model monitoring

In the RAN1#116 meeting, MI-Option 4 are also raised by some company as candidates. For MI-Option 4, to indicate the reference model (e.g., a standardized encoder for pairing with the decoder), a sort of indexing mechanism may be introduced to refer to the standardized model, and such index could also be understood as globally unique model ID. However, there seems no need to have a particular model identification procedure to align the common understanding of the model between NW side and UE side, e.g., for initiating the procedure or assigning model ID, as captured in the definition of Type A/B1/B2.

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signalling - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signalling after model identification. - Type B: Model is identified via over-the-air signalling, - Type B1: - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - Type B2: - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification

Therefore, if the indication of standardized model would need to be classified to model identification, we may need to revisit the definition of the Type A such as “The model may be assigned/specified with a model ID during the model identification”.

Observation 1: If MI-Option 4 needs to be classified to model identification, the definition of model identification may need to be revisited.

- Model transfer/delivery

In TR 38.843, the following observations for model transfer/delivery cases are provided.

## 4.3 Collaboration levels … For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models): - Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement. - Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale. - Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE. - For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model. - For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden. - Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage. - Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue.

From the highlighted parts in Rel-18, the analysis could be performed for model trained at NW side (Class 1) and trained at UE side (Class 2), respectively.

- Class 1: The delivered model is trained at NW side, including MNOs (mobile network operators). This class includes Case y, z2, z4, and z5. In this class, non-trivial cross vendor alignment is needed, as the training dataset or trained model may need to be delivered across vendors with offline contract agreement.

Figure 1 Class 1: AI/ML model transfer/delivery to UE is trained at NW side

- Class 2: The delivered model is trained at UE side, including neutral site which is affiliated with the UE vendor. This class includes Case y, z1, and z3. Note that model trained at neutral site is categorized to Class 2, since it is assumed that there is no obstacle between the UE vendor and its affiliated neutral site in productization level for model development, dataset, training/validation/testing approaches, etc.

Figure 2 Class 2: AI/ML model transfer/delivery to UE is trained at UE side/neutral site

  - Analysis when the delivered model is trained at NW side (Class 1)

In RAN1#116 meeting, Case z5 has been deprioritized in Rel-19. In RAN1#116bis meeting, Case z2 has been deprioritized at least for one-sided model in Rel-19. In RAN1#118bis meeting, Case z2 has been deprioritized for two-sided model in Rel-19.

Conclusion [RAN1#116] From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. Conclusion [RAN1#116bis] From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration Conclusion [RAN1#118bis] From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized for two-sided model in Rel-19.

Therefore, for model trained at NW side, only Case z4 is left for further study. For Case z4, one of the key issues for realizing the model transfer/delivery is whether/how to achieve the alignment of the model structures between NW side and UE side. Two candidates may include:

- Candidate 1: Offline alignment between NW side and UE side. This candidate faces the cross-vendor collaboration challenge to align the model structure; e.g., UE vendor needs to ensure the model structure is compatible with the software/hardware environment of the modem to smoothly run the model, while NW vendor may also need to ensure the model structure can be efficiently run during the training at the NW side training entity; therefore, it is probable that the UE vendor and the NW vendor need to jointly develop the model structure during the productization phase. Moreover, as the preferred model structure for each UE vendor may presumably be different, the NW vendor may need to separately train/maintain/store the UE side/UE part models for different UE vendors, as the UEs subject to various UE vendors may all camp in the same cell; this causes the burden of maintenance/storage of multiple models at the NW side, similar as highlighted in the observation of TR 38.843.

- Candidate 2: 3GPP specified model structure. This candidate can relieve the cross-vendor collaboration challenge and the model storage/maintenance challenge, since both NW vendors and UE vendors may develop/train/run the model with the common specified model structure: UE vendor can develop the software/hardware environment to run the common model structure to ensure highly efficient model inference; NW vendor can train the UE side/UE part model based on the common model structure. However, the drawback of this candidate is that the model structure is generally deemed as implementation and proprietary; companies may have different flavours on the model structure, especially for the sophisticated backbone such as Transformer; therefore, whether it is possible to achieve agreed-upon model structure at 3GPP level may be questionable, and the principle to down-select the exact model structure proposed by companies may also need to be discussed (e.g., considering the trade-off of complexity, performance, etc.). Moreover, the common model structure may limit the upper bound of the achievable performance of the model, since more advanced model structure cannot be used if not supported by specification; therefore, the motivation for vendors to improve proprietary model structure is crippled.

Observation 2: For model transfer/delivery Case z4, how to align the model structure between NW side and UE side may need further study, e.g., 2 candidates are listed in below:

- Candidate 1: Offline alignment between NW side and UE side.

  - The burden of cross-vendor collaboration still exists.

  - It causes burden of maintenance/storage of multiple models to different UE vendors at the NW side.

- Candidate 2: 3GPP specified model structure.

  - Avoid the burden of cross-vendor collaboration and the burden of maintaining/storing multiple models at NW.

  - Whether it is possible to achieved agreed-upon model structure at 3GPP level may be questionable.

  - The common specified model structure may limit the upper bound of the achievable performance of the model.

In RAN1#118bis and #119 meetings [2], the following agreements are achieved for Case z4:

Agreement [RAN1#118bis] Regarding model transfer/delivery Case z4 for inference, further study the following aspects: - Whether/when UE needs the transfer of new parameters for a known model structure. - Whether/when the AI model with the transferred parameters is ready for inference. - The transfer of partial of the parameters for a known model structure. Agreement [RAN1#118bis] Regarding the study of model transfer/delivery Case z4, for a given known model structure, network can transmit the following information along with the parameters: - a first indication referring to the known model structure at least for the case there are multiple known model structures - a second indication referring to the transmitted parameters - FFS: relationship of model ID, first indication, and second indication - FFS: transfer partial parameters Agreement [RAN1#119] Regarding the relationship of model ID, first indication, and second indication for model transfer/delivery Case z4, further study the following options: - Opt.1: model ID consists of the information of the first indication and the second indication - E.g., model ID is a combination of the first and second indications - Opt.2: The second indication is assumed as the model ID - Opt.3: Model ID is assigned by network and is separated from the first indication and the second indication Conclusion [RAN1#119] For the study of model delivery/transfer Case z4, if the model delivery/transfer is directly used for inference, the following options are identified as the candidate solutions to determine the readiness of AI model with the transferred parameters for inference (either or combination of the following options) - Option 1: UE sends signaling to network to notify that the AI model with the transferred parameters is ready for activation - Option 2: The AI model with the transferred parameters can be assumed ready for activation from a minimum applicable time after the completion of model delivery/transfer

For the issues of the first agreement, regarding whether/when UE needs the transfer of new parameters for a known model structure, it can be reported by UE via some higher layer signaling, e.g., UAI, similar to the RAN2 agreement to the report of applicable functionalities of the one-sided model. Such discussion is more appropriate to be handled by RAN2.

Regarding whether/when the AI/ML model with the transferred parameters is ready for inference, it is relevant to the UE capability to receive the parameters and deploy the parameters to the model structure. Some additional processing may be needed based on UE implementation such as quantization, pruning, etc., so that the timeline before the model is ready for inference could be relatively long time scale, e.g., second or minute level. Similar to the first bullet, UE may consider higher layer signaling to report the readiness of the received parameters. Such discussion is more appropriate to be handled by RAN2.

Regarding the transfer of partial of the parameters, whether and how to proceed the transfer of partial parameters depends on the specific model structure to be standardized. E.g., UE may only update some adaptation layers corresponding to some specific configurations of input and output parameters. However, the details may hardly be proceeded until we have more progress on the specific standardized model structure.

For the issues of the relationship of model ID, first indication, and second indication, considering the first indication corresponds to the model structure and the second indication corresponds to the parameters, they may be subject to different fields of a single model ID, or separate IDs. The study is related to the composition of the model ID, and may refer to some legacy higher layer IDs such as slice ID, TAI, SUPI, etc. The relevant study can be handled by RAN2.

Proposal 6: Regarding the study of model transfer/delivery Case z4,

- Partial parameter transfer can be discussed after more progress has been achieved for the specific standardized model structure.

- Model ID related discussion can be handled by RAN2.

  - Analysis when the delivered model is trained at UE side (Class 2)

In RAN1#116bis meeting, Case z3 has been deprioritized in Rel-19.

Conclusion From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network

Therefore, for model trained at UE side, only Case z1 is left for further study. As illustrated by Figure 2, the interaction between NW side and UE side is quite similar between Case z1 and Case z3, so the cons of these two cases are also similar.

As mentioned in the observation of TR 38.843, Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the NW, compared to Case y (w/ UE-side training). In addition, as NW side needs to maintain/store multiple UE side/UE part models subject to different UE vendors, the storage is also a challenge to NW side.

Moreover, compared to Case y, additional latency is introduced by Case z3 due to additional offline interaction between NW side and UE side.

On the other hand, we do not see clear benefits of Case z1 over Case y. Therefore, for the study of model transfer/delivery in Rel-19 where the model is trained at UE side or neutral site, we may consider Case y as baseline.

Observation 3: For model transfer/delivery where the model is trained at UE side or neutral site, the necessity of introducing Case z1 as opposed to the implementation manner of Case y is not clear:

- Case z1 incurs the burden of offline cross-vendor collaboration, compared to Case y.

- Case z1 may come with 3GPP NW side burden on model maintenance/storage compared to Case y.

- Case z1 does not bring benefits compared to Case y.

Proposal 7: For model transfer/delivery where the model is trained at UE side or neutral site, assume Case y as the baseline.

- UE side training data collection

The UE side training data collection is listed as part of the WID scope and involves both RAN2 and RAN1. From our understanding, RAN1 is obliged to provide the identified requirement of data content, type, size, latency, etc., while it is up to RAN2 to discuss and decide the detailed options (e.g., from UE to OTT server, or from CN/OAM to OTT server). Since RAN1 has fed back the identified requirements already in the replied LS in Rel-18 [3][4], we think it is not urgent for RAN1 for further discussion of this issue at the early meeting(s) of Rel-19 until there is more progress at 9.1.1 and 9.1.2 on the requirements of data.

Proposal 8: For the continued study of data collection for UE-side model training, lower the priority of the discussion at RAN1 due to the following reasons:

- The content for use cases have already been provided in the Rel-18 LS reply from RAN1.

- Discussion of UE data collection mechanisms is out of RAN1 scope.

- Conclusions

In this contribution, the 3 study objectives of model identification for two-sided models, model transfer/delivery, and UE side training data collection are discussed. Based on the discussions, we have the following observations and proposals.

Observation 1: If MI-Option 4 needs to be classified to model identification, the definition of model identification may need to be revisited.

Observation 2: For model transfer/delivery Case z4, how to align the model structure between NW side and UE side may need further study, e.g., 2 candidates are listed in below:

- Candidate 1: Offline alignment between NW side and UE side.

  - The burden of cross-vendor collaboration still exists.

  - It causes burden of maintenance/storage of multiple models to different UE vendors at the NW side.

- Candidate 2: 3GPP specified model structure.

  - Avoid the burden of cross-vendor collaboration and the burden of maintaining/storing multiple models at NW.

  - Whether it is possible to achieved agreed-upon model structure at 3GPP level may be questionable.

  - The common specified model structure may limit the upper bound of the achievable performance of the model.

Observation 3: For model transfer/delivery where the model is trained at UE side or neutral site, the necessity of introducing Case z1 as opposed to the implementation manner of Case y is not clear:

- Case z1 incurs the burden of offline cross-vendor collaboration, compared to Case y.

- Case z1 may come with 3GPP NW side burden on model maintenance/storage compared to Case y.

- Case z1 does not bring benefits compared to Case y.

Proposal 1: There is no need to further discuss MI-Option 1 in 9.1.4.2, since MI-Option 1 is applicable only to one-sided model cases, while the scope of model identification in revised WID is limit to two-sided case, for which the UE side does not need to be provided with NW-side additional condition since the data collection is performed by NW side for NW-side training or NW-first training.

Proposal 2: For the transmitted information of MI-Option 2, if the dataset is delivered from NW side to UE side, the following information may be needed:

- Content of model input and label for training the UE side CSI generation part.

- Other meta information, including at least: dataset ID, size of dataset, type/format of data samples, association between ground-truth CSI and CSI feedback subject to a data sample, scalability information, quantization method for CSI feedback, dataset split/segmentation information, and target performance information.

Proposal 3: For the procedure of MI-Option 2, the model identification, the “ID-X” could be interpreted as dataset ID.

Proposal 4: For the transmitted information of MI-Option 3, taking Case z4 for example, the following information may be needed:

- Model parameters.

- Other meta information, including at least: model ID, format of the parameters, model structure information, quantization method and parameters.

Proposal 5: For the procedure of MI-Option 3, the model identification is achieved when the model ID is delivered in together with the delivered model.

Proposal 6: Regarding the study of model transfer/delivery Case z4,

- Partial parameter transfer can be discussed after more progress has been achieved for the specific standardized model structure.

- Model ID related discussion can be handled by RAN2.

Proposal 7: For model transfer/delivery where the model is trained at UE side or neutral site, assume Case y as the baseline.

Proposal 8: For the continued study of data collection for UE-side model training, lower the priority of the discussion at RAN1 due to the following reasons:

- The content for use cases have already been provided in the Rel-18 LS reply from RAN1.

- Discussion of UE data collection mechanisms is out of RAN1 scope.

# References

RP-243245, “New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Madrid, Spain, December 9-12, 2024.

RAN1 Chair’s Notes, RAN1#119, Orlando, US, November 18-22, 2024.

- R1-2308730 “Reply LS on Data Collection Requirements and Assumptions”, Toulouse, France, August 21-25, 2023.

- R1-2310681, “Reply LS on Data Collection Requirements and Assumptions”, Xiamen, China, October 9-13, 2023.