3GPP TSG RAN WG1 #120                                           R1-2500201

Athens, Greece, February 17th – 21st, 2025

Source: 	CATT

Title:	Discussion on AI/ML-based beam management

Agenda Item:	9.1.1

Document for:	Discussion and Decision

- Introduction

One objective of Rel-19 NR work item on AI/ML is to provide specification support for AI/ML based beam management, and the specification impact for beam management including the following aspects [1].

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we will discuss the specification support for both NW-sided model and UE-sided model.

- Consistency issue

  - Consistency issue of UE-sided model

In RAN1#118, associated ID was supported for the consistency issue of NW-side additional condition across training and inference for UE-sided model [2].

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

For the applicable range of the associated ID, it was agreed that the same associated ID are consistent at least within a cell in agenda of other aspects.

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

In our view, it is difficult to use associated ID in inter-cell scenario, since it may require network vendor to share the implementation-specific information across gNBs/TRPs. Furthermore, there is a high risk of proprietary exposure. For intra-cell scenario, UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID. One FFS issue is whether/how to define similar properties of a DL Tx beam or beam set/list. We think it is difficult and unnecessary to define the similar properties represented by associated ID. One reason is that it involves several implementation properties, e.g., beam direction, beam downtilt, etc. Another reason is that it is not clear what kind of difference can be viewed as “similar” properties. The usage of associated ID is to classify data samples during the training phase and assists the UE in selecting the appropriate model or functionality during the inference phase. For intra-cell scenario, UE can assume the functionality can be applied when the associated ID indicated in inference phase matches that indicated during training phase, without explicit knowledge of the similar properties underlying the associated ID.

Proposal 1: Conclude that for UE sided model in beam management, no need to define the similar properties of a DL Tx beam or beam set/list with the associated ID.

It was agreed as a working assumption that the associated ID can be configured within CSI framework, and whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) is FFS. According to our understanding, there are three phases where the gNB sends associated ID(s) to the UE: the data collection for training phase, the phase for determining the applicable functionalities, and the inference reporting configuration phase. Since the inference reporting is based on the CSI report, it is nature to configure the associated ID within the CSI framework in inference phase. However, whether the configuration for UE-side data collection should be based on the CSI framework is still under discussion. Where to configure the associated IDs for data collection can be discussed when the procedures are more clearly defined.

Proposal 2: The associated ID can be configured within CSI framework of inference reporting.

For inference configuration, the associated ID can be configured per CSI-ReportConfig, per CSI-ResourceConfig, per CSI-ResourceSet, or per CSI-RS resource. In our view, the associated ID can represent the physical properties of beams in Set A and Set B, as well as the association between Set A and Set B. Thus, only a single associated ID is required for an association of Set A and Set B. It is unnecessary to configure the associated ID at resource set or resource level. Considering that AI/ML based beam reporting and legacy beam reporting can be associated with the same CSI-ResourceConfig, and that the associated ID is not needed for legacy beam reporting, we think it is more suitable to configure associated ID at CSI-Report level.

Proposal 3: For configuring the associated ID within CSI framework, support to configure associated ID at CSI-Report level.

If model training and model inference are performed in different cells, performance monitoring based method can be used to check the consistency of NW-side additional condition. If the performance metric meets the requirements, it can be regarded as the NW-side additional condition across training and inference are consistent. The drawback of this method is the RS overhead may be large. From the network perspective, if it sends RS for all the AI/ML functionalities of each UE to check the consistency of NW-side additional condition, the RS overhead may be large. To reduce the RS overhead, a common dataset for performance monitoring can be considered. When a UE needs to check whether the NW-side additional condition is consistent, gNB can send the dataset (including model input and the corresponding ground-truth) to UE via high layer signaling, and UE can calculate the performance metric after model inference.

Proposal 4: For the consistency of training and inference in different cells, performance monitoring based method can be supported.

  - Consistency issue of NW-sided model

In RAN1#116 meeting, for NW-sided model, whether to align the Rx information of the measurements between network and UE has been discussed and different views were expressed among companies. In Rel-18, we have evaluated the beam prediction accuracy under different Rx beam assumptions, and obtain the result in Table 1 [3].

Table 1 Beam prediction accuracy under different Rx beam assumptions

|  | Rx beam assumption of training | Rx beam assumption of inference | Top-1(%) |
| --- | --- | --- | --- |
| Case 1 | best Rx beam | best Rx beam | 84.84 |
| Case 2 | random Rx beam | best Rx beam | 29.63 |
| Case 3 | random Rx beam | random Rx beam | 64.39 |
| Case 4 | best Rx beam & random Rx beam (50%,50%) | best Rx beam | 68.98 |

From the evaluation results, it can be seen that if best Rx beam assumption is assumed in both training and inference, the best beam prediction accuracy can be achieved (Case 1). If random Rx beam assumption is assumed in training and best Rx beam assumption is assumed for model inference, the Top-1 beam prediction accuracy is only 29.63% (Case 2). Comparing the result of Case 4 and Case 1, if mixed dataset of best Rx beam assumption and random Rx beam assumption is used in model training, the degradation of beam prediction accuracy is about 16%. It can be concluded that the beam prediction accuracy will be degraded if the Rx information of the measurements is not consistent between training and inference. To ensure the Rx information consistency between training and inference, the network should be aware of the Rx information of the measurement in training and inference. Hence, it is beneficial to align the Rx information between network and UE.

Proposal 5: For NW-sided model, it is beneficial to align the Rx information of the measurements between network and UE.

In practical, to ensure satisfactory beam prediction accuracy, UE may use quasi-optimal Rx beam for each measurement. UE should use fixed Rx beam for measurement of beams in Set A or Set B. For example, when UE measures the resources of 32 beams in Set A to find the ideal Top-1 beam, UE should not vary the Rx beam when measuring the 32 beams. The specific Rx beam or the method to determining the quasi-optimal Rx beam can be up to UE.

Proposal 6: For NW-sided model, UE should use fixed Rx beam for each measured beam in Set A/Set B, and the specific Rx beam used for measurement can be up to UE.

- UE sided model

  - Model inference

Association of Set A and Set B

For model inference, the resources of Set B beams need to be measured, and the resource or index of Set A beams will only be used for reporting. The essential issue is how to represent the resource or index of Set A beams. In RAN1#116bis meeting, the following alternatives are considered for configuration of inference results reporting.

- Alt 1: one CSI-ResourceConfigId is configured for Set B

  - FFS: how UE can determine the information about set A

- Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B

  - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig

- Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

- Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId

  - FFS: how to configure/indicate separate resource set(s) for Set A

In RAN1#119 meeting, the following agreement on configuration of Set A and Set B was achieved [4] .

Agreement For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, - Two CSI-ResourceConfigId s are configured for Set A and Set B separately

The agreement corresponds to Alt 3, where the resource set(s) of Set A and the resource set(s) of Set B are configured in separate CSI-ResourceConfigId. In our view, it is also feasible that only resource set(s) of B is configured within the CSI report. To determine the information about Set A, the CSI reporting can be associated with an associated ID. As analyzed in Section 2.1, the associated ID can be configured at CSI-Report level. From the associated ID, UE can know the number of Set A beams M and the index of beams in Set A can be defined as 0 to M-1. The mapping between the reported index and physical Tx beams is only known at gNB side. UE can report the index of predicted Top-K beams of Set A to gNB based on the inference result. An example is shown in Figure 1. For associated ID #a, the numbers of beams in Set B and Set A are 8 and 32, respectively. For CSI reporting #n, UE will use the measurement of resource set #s as model input to perform model inference, and report the index of predicted Top-K beam(s) from index 0 to 31.

Figure 1 Association of Set B and Set A via an associated id.

Proposal 7: For inference of UE-sided model, at least for BM-Case1, the following alternatives can be further supported for the CSI-ReportConfig of inference results reporting:

- Alt 1: one CSI-ResourceConfigId is configured for Set B, and Set A can be determined from the associated id.

CSI configuration enhancements for BM-Case2

For the resource types for CMR of inference report, it was agreed in RAN1#118bis [5] that for BM-Case1, P CSI-RS, SP CSI-RS and AP CSI-RS are supported, and for BM-Case2, P CSI-RS and SP CSI-RS are supported, while AP CSI-RS is FFS.

Agreement For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - Aperiodic (AP) CSI-RS For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - FFS: Aperiodic (AP) CSI-RS Note: above CSI-RS resource refers to that used for beam management.

For BM-Case2, we think AP CSI-RS can also be supported. The difference of Set B measurement for BM-Case1 and BM-Case2 is that multiple measurement instances of Set B may be required for once inference in BM-Case2. The measurement duration is related to the design of the AI/ML model. If the measurement duration is long, it may affect the UL scheduling since the PUSCH out-of-order issue should be avoided. However, if the measurement duration is not long, the impact for UL scheduling will be minimal. In our view, AP CSI-RS can be supported for BM-Case2 with UE capability.

Proposal 8: For UE-sided AI/ML model inference, for BM-Case2, AP CSI-RS can be supported for CMR configuration at least for Set B.

According to RAN1#119, a conclusion was achieved that only fixed Set B across different time instance is supported for single CSI report. One issue is whether legacy P CSI-RS/SP CSI-RS/AP CSI-RS configuration mechanism can be reused for BM-Case2. An example of BM-Case2 is shown in Figure 2, where UE periodically predicts the Top-1 beam of future 3 time instances based on 3 historic measurement results.

Conclusion For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Figure 2 Example of BM-Case2

During Rel-18 AI/ML evaluation for beam management, companies provide simulation results for the AI/ML model performance for the case of Set A= Set B and for the case of Set B is the subset of Set A. In the practical network, it’s difficult for network sending Set A to enable UE-sided model to achieve RS overhead reduction. The reason is that there are various UEs within the network coverage, if Set B equals to Set A for all the UEs but the time instances of RS measurement are different for the UEs, the network still needs to send Set A most of time and thus can’t save RS overhead. Therefore, the more reasonable case is that Set B is subset of Set A. From the perspective of UE, a sliding window can be considered for measurement of CSI-RS resource as model input. In this case, BM-Case2 is similar with Rel-18 Doppler domain CSI prediction in terms that both of them are time-domain prediction based on measurements of multiple historical time instances with sliding window manner. The RS configuration of Rel-18 Doppler domain CSI prediction is as follows:

For P/SP CSI-RS,

- For CSI prediction, single resource is configured for CMR, and the measurement of one time instance in the measurement window corresponds to the measurement of one occasion of the resource.

For AP CSI-RS,

- For CSI prediction, one resource set with K ∈ {4,8,12} aperiodic resources is configured for CMR, and the measurement of one time instance in the measurement window corresponds to the measurement of one resource. The separation between two resources is m ∈ {1,2} slots.

The difference of RS measurement for Rel-18 Doppler domain CSI prediction and BM-Case2 is that, in BM-Case2, the measurement of each time instance in the measurement window contains multiple resources (corresponding to Set B) instead of one resource for AP CSI-RS or one occasion of the resource for P/SP CSI-RS. Therefore, the RS configuration mechanism of BM-Case2 can be:

For P/SP CSI-RS,

- For BM-Case2, one resource set with the resources of Set B is configured for CMR, and the separation between two consecutive measurements is the periodicity of RS set.

For AP CSI-RS,

- Option 1: For BM-Case 2, one resource set with the resource of Set B, the transmission number of the RS set, and the separation between two consecutive transmissions can be configured for one triggering instance. Multiple transmissions of the resource set are triggered by one triggering instance.

- Option 2: For BM-Case 2, multiple resource sets can be configured for one trigger state and each resource set can be configured with trigger offset separately. Multiple resource sets with different slot offsets can be triggered by one triggering instance

Proposal 9: For UE-sided AI/ML model inference of BM-Case2, for periodic or semi-static CSI-RS configuration, one RS set can be configured as CMR for the inference report and the separation between two consecutive measurements is the periodicity of RS set.

Proposal 10: For UE-sided AI/ML model inference of BM-Case2, for aperiodic CSI-RS configuration, consider two potential options as follows:

- Option 1: One resource set with the resource of Set B, the transmission number of the RS set, and the separation between two consecutive transmissions can be configured for one triggering instance. Multiple transmissions of the resource set are triggered by one triggering instance.

- Option 2: Multiple resource sets can be configured for one trigger state and each resource set can be configured with trigger offset separately. Multiple resource sets with different slot offsets are triggered by one triggering instance.

In RAN1#118[2], for UE-sided model for BM-Case 2, an agreement on inference results report was agreed to support configuring UE with N future time instance(s) for inference. Then, in RAN1#118bis [5], it achieved an agreement on three options for the reference time of the earliest time instance for the predicted results.

Agreement (RAN1#118) For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted Agreement (RAN1#118bis) For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

In Rel-18 Doppler domain CSI prediction, when UE-side prediction is assumed, the earliest of prediction time instance starts at slot l=n+δ, where n is the time slot for the report and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter. In Rel-18, the time duration between the latest measurement time instance and the earliest prediction time instance is short, i.e. slot-level. However, for UE-sided model for BM-Case2, the time duration between the latest measurement time instance and the earliest prediction time instance is a relative long duration, e.g., 40 ms or 80 ms.

- For Option 1, the earliest time instance for the predicted results can be determined by the sum of the time slot for the report and a slot offset δ‘ as shown in Figure 3. The time slot for the report may be at any time instance after the latest measurement time instance, which is up to gNB. If time offset in Option 1 is a slot-level value, the candidate value of time offset will be a large set of values, such as 0 to 320 slot in 60 KHz SCS for 80 ms.

- For Option 2, the reference time of predicted result is the CSI reference resource to the report. Since the CSI reference resource is determined based on CSI report, there is no need to use a relative resource of CSI report as reference time for prediction time instance determination.

- For Option 3, the reference time is the latest CSI-RS/SSB occasion of Set B for inference no later than the CSI report as shown in Figure 4. The earliest time instance for the predicted results can be determined based on the CSI-RS/SSB occasion of Set B for inference and a time offset δ‘. In this way, the candidate values of time offset can be some fixed values, such as 40 ms or 80 ms, which are subject to the UE-sided AI/ML model.

In general, CSI report for inference can be configured/activated/triggered on any uplink slot n. If Option 1 or Option 2 is supported, to ensure the flexibility of configuration/activated/triggered, the number of candidate values for slot offset should be a large set of multiple values for different SCSs. If Option 3 is supported, the candidate values of slot offset mainly depends on the AI/ML model or functionality of UE, which may be just a few values. Thus, compared with Option 1 or Option 2, Option 3 is a simpler and a more suitable method for UE-sided AI/ML model for BM-Case2.

Figure 3 (Option 1) Based on the time domain resource for the report

Figure 4 (Option 3) Based on the transmission occasion of the CSI-RS/SSB resource of Set B

Proposal 11: For UE-sided model for BM-Case2, for inference results report, the following option can be considered for determining the reference time of the earliest time instance for the predicted results:

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource.

If a UE is configured with a UE-sided model for BM-Case 2, assuming that the earliest predicted time instance starts at slot l, the UE is expected to report the predicted results for slot intervals l+j∙P, l+j+1∙P-1, for j=0,…N-1, where N is the number of the predicted slot interval(s) and P is duration of each slot interval. The value of N and P can be indicated to UE by implicit or explicit configuration.

Proposal 12: For UE-sided model for BM-Case2, the UE is expected to report the predicted results for slot intervals l+j∙P, l+j+1∙P-1, for j=0,…N-1, where l is the earliest predicted time instance, N is the number of the consecutive predicted slot interval(s) and P is duration of each slot interval.

Another FFS is on duration values of N slot interval(s) that can be predicted. In Rel-18 SI phase, companies provided the evaluation results for the BM-Case2 with the prediction window of 80ms/160ms/320ms /480ms/640ms/800ms/others. Based on the evaluation, it is shown that the prediction accuracy of the AI/ML model of BM-Case2 decreases as the prediction window increases. Hence, the duration values of prediction window can be at least 320ms /480ms /640ms.

Proposal 13: For UE-sided model for BM-Case2, for the duration values of the slot interval(s) that can be predicted support at least 320ms /480ms/640ms.

Inference result report

For the content of inference report, it was agreed to report the beam information on predicted Top K beam(s) among a set of beams (Opt 1) or beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams (Opt 2).

In RAN1#119 meeting, it was agreed that the beam information in inference result report is CRI/SSBRI of resource in Set A. As discussed in Section 3.1.1, if the resource set of Set A is not configured to UE, UE can report the pre-defined beam index of Set A, such as 0 to M-1 , where M is the number of beams in Set A.

Proposal 14: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported beam information can be the pre-defined beam index of the predicted Top K beam(s).

- the pre-defined beam index can be 0 to M-1 , where M is the number of beams in Set A

For the RSRP of predicted Top K beam(s) in the report of inference results, two options are agreed for further study in RAN1#116bis meeting. Option A is to report the predicted L1-RSRP for both measured beams and not measured beams, and Option B is to report predicted RSRP for not measured beams and measured RSRP for measured beams. In our view, both options can be supported and which option is used for reporting can be configured by gNB. During the discussion, some companies argued that the predicted L1-RSRP may not be accurate. In our view, if there is large difference between the predicted L1-RSRP and the ideal L1-RSRP, the performance degradation will be detected by performance monitoring procedure. Moreover, the predicted L1-RSRP can be used by network for performance monitoring, and hence the network can configure the type of RSRP for reporting.

Proposal 15: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported RSRP type of predicted Top K beam(s) can be configured by gNB with the following options:

- Option A: Predicted L1-RSRP;

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.

Another FFS is whether to report other information including the following options

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

  - FFS on the quantization method of probability information

  - Probability information is the probability of the beam to be the Top 1 or Top K beam

- Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

  - FFS on definition of reported RSRP

  - FFS on the definition and quantization method of confidence information

For Opt 3, the probability information is the probability of the beam to be the Top 1 or Top K beam, but how to get the actual probability for each model inference is not clear. For classification model, the model can output the probability of the beam to be the Top 1 beam, but whether the probability is reliable is unknown. It is possible that the probability of the beam to be the Top 1 is not high but the prediction is correct. For Opt 4, both the definition and the scheme to obtain the confidence/probability information related to the predicted result are still FFS. The motivation and benefit of Opt 4 are not clear.

In RAN1#118bis, RAN1 discussed whether to report ranking information of the predicted Top K beams for K>1 implicitly by the order of the beam information.

FL Proposal 6.1[6] For UE-sided model, at least for BM-Case1, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information. - FFS on whether/how to specify on how to obtain the ranking information.

A controversial part of the above proposal is how to obtain the ranking information. One direction is that UE determines the ranking information based on the probability information or the predicted L1-RSRP of the predicted Top K beams. Another direction is that the UE determines the ranking information based on UE implementation. From our understanding, legacy L1-RSRP reporting can convey beam information and corresponding L1-RSRP of the reported beams, which also conveys ranking information among the reported beams implicitly. For AI/ML model inference result reporting, in case that only beam information of predicted Top K beam(s) are reported, it’s better to convey ranking information of predicted Top K beams to provide auxiliary information implicitly on top of beam information for gNB. The ranking information can be obtained by UE based on the model output, i.e. the probability information of the predicted beams for classification model, or the predicted L1-RSRP of the predicted beams for regression model. If ranking information is determined by UE implementation, due to different algorithm implementation of different UEs, the gNB can’t obtain effective auxiliary information.

Proposal 16：For UE-sided mode, at least for BM-Case1, for content in the report of inference results, for Opt 1(only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K>1 can be conveyed by the order of the beam information based on the predicted probability information or the predicted L1-RSRP.

In RAN1#118[2], it was agreed to support differential RSRP reporting for UE sided model for BM-Case1 and BM-Case2. And there is still a FFS on details of differential RSRP reporting among multiple time instances for BM-Case2.

Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details

For a L1-RSRP reporting of multiple time instances, one straightforward method is to report largest L1-RSRP among multiple beams over multiple time instances and other differential L1-RSRP of multiple beams over multiple time instances with a reference to the largest L1-RSRP in a pre-defined order. In this way, the network can obtain L1-RSRP values of each beam among one or multiple time instance(s) through a largest L1-RSRP and a differential L1-RSRP. Comparing with the case of reporting largest L1-RSRP for each time instance, the reporting overhead can be further reduced.

Proposal 17: For UE-sided model for BM-Case2, for inference report, support to report largest L1-RSRP among multiple beams over multiple time instances and other differential L1-RSRP of multiple beams over multiple time instances with a reference to the largest L1-RSRP in a pre-defined order.

  - TCI state indication

In legacy TCI indication, the QCL relationship is indicated to UE via TCI indication. The indicated TCI state can be known or unknown TCI state, and the TCI state switching delay requirements for known and unknown TCI state are defined in TS 38.133[7]. If the beam(s) in Set A not in Set B is indicated to UE, the indicated TCI state may be an unknown TCI state, and the legacy requirements can be reused.

Proposal 18: From RAN1 perspective, the existing TCI state switching delay requirements can be reused for the predicted RS resource of UE sided-model.

For BM-Case2, one open issue is whether to indicate TCI states of multiple time instances using single beam indication. Before we dive into the mechanism of enhancements for TCI state indication, we should make clear the motivation and benefit. The potential advantage for indicating multiple time instances may be overhead reduction of indication signaling. But we think it only makes sense when there is no PDSCH transmission.

For example, for unified TCI framework, if there is no PDSCH transmission when the optimal beam between gNB and UE is changed, the network will indicate the corresponding TCI state to UE via a DCI without DL assignment. If a UE is moving with high speed, the optimal beam between gNB and UE can be changed frequently, and the network needs to frequently send DCIs without DL assignment to indicate the new TCI states. If TCI states of multiple time instances can be indicated by a single TCI indication signaling, the DCI without DL assignment can be send less frequently, and then the DCI overhead can be reduced. However, if there is PDSCH for transmission, the gNB will send DCI with DL assignment to UE, and the TCI field will always be present in DCI. In such a case, the DCI overhead cannot be reduced.

Another important issue to be considered is the TCI indication flexibility. If gNB indicates TCI states of multiple time instances using single beam indication, the beam application time for each TCI state should also be pre-defined or indicated to UE. Before applying the beam, the network may decide to change the Tx beam for transmissions. As shown in Figure 5, the AI/ML model can predict the optimal beam of 4 future time instances T5, T6, T7, T8 based on the measurement results of T1, T2, T3, and T4. After model inference, the network sends TCI state indication 1 to inform UE the TCI states of T5, T6, T7, and T8. If the network decides to change the TCI states of T7 and T8 before the beam application time of T7 and T8, the network will send TCI state indication 2 to update the TCI states of T7 and T8. The mechanism will be complex if allowing updating the TCI indication to ensure the flexibility. Overall, Rel-17 TCI state activation/indication mechanism can be reused for BM-Case2, the enhancement is not necessary.

Figure 5 TCI state indication of multiple time instances

Proposal 19: For TCI indication of BM-Case2, there is no need to extend the Rel-17 TCI state activation/indication signaling methods to activate/indicate TCI state indication of multiple future time instances.

  - Performance monitoring

Type 1 Option 1 monitoring

For performance monitoring of UE-sided model, it was agreed to support Type 1 performance monitoring including both Option 1 and Option 2. For Option 1, UE sends reporting to NW and NW calculates the performance metric. Both the inference result and measurement result for calculating the performance metric will be reported to the network. For the reporting for inference result and reporting for measurement result, two separate reporting can be configured to UE. In this way, per sample based reporting with CSI framework can be the baseline. The network can calculate the performance metric based on the multiple reports from the UE. .

Proposal 20: For Type 1 performance monitoring of Option 1, support per-sample based reporting with CSI framework as the baseline.

Type 1 Option 2 monitoring

Performance metric

For Type 1 Option 2 monitoring, UE calculates performance metric and reports the metric or an event to network. If UE reports performance metric to network, the reported contents are related to the performance metric. In RAN1#118 [2], it was agreed to further study four alternatives. In RAN1#118bis [5], Alt1 was supported and other alternatives are FFS.

Agreement (RAN1#118) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) Agreement (RAN1#118bis) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

In RAN1#119, the definition and reporting quantity of beam prediction accuracy was discussed. The beam prediction accuracy can be defined as Np/N, where Np is the number of “successful inference instances”, and N denotes the number of performance monitoring instances.

The first issue is whether UE reports the ratio of Np/N or reports the value of Np. In our view, reporting the ratio is preferable. For reporting the value of Np, the payload size varies with the value of N, and the payload size may be large. However, if quantized beam prediction accuracy is reported, the payload size remains consistent for both BM-Case1 and BM-Case2.

For counting the value of Np, it is essential to define what is a successful inference instance. We propose using the criteria of both Top-K/1 beam prediction accuracy and Top-K/1 beam prediction accuracy within X dB margin defined in Rel-18. Accordingly, a successful inference instance can be defined as follows, at least for the case that full set of Set A is configured as the monitoring set:

- one of the predicted Top-K beams is the Top-1 beam from the resources for monitoring

- one of the predicted Top-K beams is within X dB margin of the Top-1 beam from the resources for monitoring

Proposal 21: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Type 1 Option 2, the reporting quantity of the CSI report for monitoring is a quantized value of Np/N, where Np is the number of “successful inference instances”, and N denotes the number of performance monitoring instances.

- the successful inference instance is defined as

  - one of the predicted Top-K beams is the Top-1 beam from the resources for monitoring

  - one of the predicted Top-K beams is within X dB margin of the Top-1 beam from the resources for monitoring

For Alt2, the definitions of L1-RSRP difference are not clear. If the model output is predicted Top-K beam(s), the L1-RSRP difference can be defined as the following options.

- Alt2-1: The L1-RSRP difference between the largest L1-RSRP measurements of the Top K predicted beam(s) and the largest L1-RSRP measurements of a monitoring resource set

- Alt2-2: The mean of the absolute difference of L1-RSRP between the L1-RSRP measurements of the Top K predicted beam(s), and the Top-K largest L1-RSRP measurements of a monitoring resource set

For Alt2-1, it reflects the L1-RSRP difference between the beam with largest L1-RSRP among the predicted Top-K beam(s) and the ideal Top-1 beam. If the ideal Top-1 beam is within the predicted Top-K beam(s), the L1-RSRP difference will be zero. For Alt2-2, it represents the average L1-RSRP difference between the actual measurement of predicted Top-K beam(s) and ideal Top-K beam(s).

For Alt3, the metric is RSRP difference between the predicted RSRP and measured L1-RSRP of corresponding beam(s). We have conducted some evaluations to verify whether the L1-RSRP difference will be changed when the prediction accuracy of AI/ML model is evidently reduced. In our simulations, we consider two different generalization cases. One is AI/ML model generalized from UMi scenario to UMa scenario, and the other one is AI/ML model generalized from Set B fixed pattern 1 to Set B fixed pattern 2. The evaluation results are summarized in Table 2.

Table 2 Evaluation results for BM-Case1 with generalization

| Training data | UMi | UMi | Set B pattern 1 | Set B pattern 1 |
| --- | --- | --- | --- | --- |
| Testing data | UMi | UMa | Set B pattern 1 | Set B pattern 2 |
| Top-1(%) | 95.35 | 84.04 | 90.38 | 45.28 |
| L1-RSRP diff of Set B (dB) | 0.212 | 0.943 | 0.318 | 3.741 |
| L1-RSRP diff of predicted Top-1 beam (dB) | 0.427 | 1.344 | 0.655 | 5.542 |
| L1-RSRP diff of predicted Top-2 beams (dB) | 0.438 | 1.359 | 0.705 | 5.062 |

In Table 2, we show the KPIs of Top-1 beam prediction accuracy, L1-RSRP difference of beams in Set B, L1-RSRP difference of predicted Top-1 beam, and L1-RSRP difference of predicted Top-2 beams. It can be observed that when the Top-1 beam prediction accuracy decreased from 95.35% to 84.04%, the L1-RSRP difference increased accordingly. More obvious increase of L1-RSRP difference can be observed when the Top-1 beam prediction accuracy decreased from 90.38% to 45.28%. It indicates that the L1-RSRP difference will be increased when the beam prediction accuracy is decreased. Hence, L1-RSRP difference can be used as the metric of performance monitoring.

From our simulations, both L1-RSRP difference of Top-K beam(s) and L1-RSRP difference of Set B beams can be supported as the performance metric. For BM-Case1, if L1-RSRP difference of beams in Set B is used as the performance metric, the gNB only needs to transmit RS with Set B beams for model inference and performance monitoring, and no additional RS transmission is needed. If L1-RSRP difference of predicted Top-K beam(s) is used as the performance metric, the gNB needs to transmit RS with the predicted Top-K beams after AI/ML model inference. For BM-Case2, if L1-RSRP difference of beams in Set B or L1-RSRP difference of predicted Top-K beam(s) is used as performance metric, gNB needs to transmit the corresponding RS at time instances in the prediction window, and thus the metric with less RS transmission overhead and reporting overhead can be adopted.

For Alt4, probability information of the predicted beam is the model output. It is strange to use model output for monitoring its own performance.

Proposal 22: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Type 1 Option 2, further support the following metrics:

- Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set for monitoring

  - Alt2-1：The L1-RSRP difference between the largest L1-RSRP measurements of the Top K predicted beam(s) and the largest L1-RSRP measurements of a resource set for monitoring

  - Alt2-2：The mean of the absolute difference of L1-RSRP between the actual L1-RSRP measurements of the Top K predicted beam(s), and the Top-K largest L1-RSRP measurements of a resource set for monitoring

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring

  - The corresponding beams can be predicted Top-K beam(s) or Set B

For Alt 1and Alt 2, to obtain accurate performance metrics, the resources for monitoring needs to be Set A, but the RS overhead may be large, especially for BM-Case2. In order to reduce the overhead, it can also be considered to configure a monitoring set where the number of resources is less than the number of beams in Set A. In the following, we will discuss how to configure the full Set A and not full Set A as the monitoring resources.

Configuration of full Set A as monitoring resources

The number of beams in Set A could be large, e.g., 32, 64, or even more. In current specification, the number of SSB/CSI-RS resources configured for L1-RSRP measurement is limited by UE capability as follows [8].

BeamManagementSSB-CSI-RS ::= SEQUENCE { maxNumberSSB-CSI-RS-ResourceOneTx ENUMERATED {n0, n8, n16, n32, n64}, maxNumberCSI-RS-Resource ENUMERATED {n0, n4, n8, n16, n32, n64}, maxNumberCSI-RS-ResourceTwoTx ENUMERATED {n0, n4, n8, n16, n32, n64}, supportedCSI-RS-Density ENUMERATED {one, three, oneAndThree} OPTIONAL, maxNumberAperiodicCSI-RS-Resource ENUMERATED {n0, n1, n4, n8, n16, n32, n64} }

The definitions of the above parameters are [9]

- maxNumberSSB-CSI-RS-ResourceOneTx: The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot.

- maxNumberCSI-RS-Resource: The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP.

- maxNumberCSI-RS-ResourceTwoTx: The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot.

- maxNumberAperiodicCSI-RS-Resource: The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP.

Based on the above, both the max number of CSI-RS resources configured to measure L1-RSRP and the max number of RS to measure L1-RSRP within a slot are limited by the UE capability. For performance monitoring, it is possible that the number of Set A beams is larger than the max number of CSI-RS resources can be configured to UE for L1-RSRP measurement. In this case, how to configure the RS for Set A beams should be discussed.

Proposal 23：For Type 1 performance monitoring of Option 2, for resource configuration of Set A, study how to configure the resources when the number of beams is larger than the max number of CSI-RS resources can be configured to UE for L1-RSRP measurement.

If the number of Set A beams is less than the max number of resources configured to measure L1-RSRP, but greater than the number of resources to measure L1-RSRP within a slot, it may need to configure the RS for Set A in several slots. In legacy L1-RSRP measurement, for periodic RS or semi-persistent RS, the periodicity and slot offset of each RS in a RS set can be configured in resource level. The RS set for Set A can consist of resources with different slot offsets, and legacy P/SP resource configuration can be reused.

For aperiodic RS used for performance monitoring, the slot offset (aperiodicTriggeringOffset) is configured in resource set level. The resources for Set A may need to be transmitted in consecutive slots since the number of beams in Set A may be large. Rel-19 MIMO has supported to transmit the resources configured in a RS set within two consecutive slots for resource aggregation to attain more than 32 ports. This approach can be reused for aperiodic RS configuration for Set A. Whether there is a need of enhancement for triggering the aperiodic RS transmitted in more than two slots should be discussed.

Proposal 24：For Type 1 performance monitoring of Option 2, for resource configuration of Set A, reuse the enhancement in Rel-19 MIMO to transmit the aperiodic resources configured in a RS set within two consecutive slots.

- Study whether there is a need to trigger the aperiodic RS transmitted in more than two slots.

Configuration of not full Set A as monitoring resources

If the monitoring set is not full Set A, how to define the metric should be discussed. For example, to calculate the Top-K beam prediction accuracy, the UE needs to determine whether the Top-1 beam from the resources for monitoring is one of the Top-K predicted beams. In RAN1#119 meeting, there are two potential alternatives proposed by most of companies:

- Alt1：Performance metric is calculated (i.e., as valid instance for metric calculation) only if Top-1 or Top-K (FFS all or at least one of Top-K) predicted beam(s) are included in the subset of Set A

- Alt2: For performance metric calculation, with predefined mapping between the full resources for Set A to the monitoring resources (with a multi-to-one mapping so that all resources for Set A has a mapped resource for monitoring)

For Alt.1, the concept of the valid performance monitoring instance is introduced for performance metric calculation. A valid performance monitoring instance refers to the time instance where the predicted Top-1 or Top-K beam(s) are included in the monitoring resource set. If a time instance for performance monitoring is identified by UE as valid time instance, the measurement results of the time instance will be used to calculate performance metric; otherwise, the measurement results of the time instance are excluded from the calculation of performance metric. But regarding Alt.1, there are three issues to consider. First, the number of valid time instance for a performance metric calculation is uncertain. This is because the monitoring resource set is configured by RRC signaling, while the predicted Top-K beam(s) are inferred in real time by UE-sided model based on the measurement of Set B. Consequently, the number of samples used to calculate the performance metric is a variable value. Especially, when the predicted Top-K beam(s) are used to identify valid time instances, the number of valid time instance tends to be low, which is not benefit for the performance metric calculation.

Second, even if the predicted Top-K beam(s) are measured Top-K beam(s) within the monitoring resource set, it doesn’t mean that the predicted Top-K beams(s) are the measured Top-K beams within the resources of Set A, thus it can’t ensure the accuracy of performance monitoring for UE-sided model. As shown in Figure 6, assuming that there are 12 resources for the resource set of Set A, the predicted Top-1 beam is the RS resource 1. For the case of monitoring resource set is the subset of Set A, the measured Top-1 beam in the monitoring resource set is also RS resource 1 which is same as predicted beam. For the case of monitoring resource set is the Set A, the measured Top-1 beam in the monitoring resource set is RS resource 3 which is different from the predicted beam. In this situation, the performance monitoring metric based on subset of Set A is not accurate since an incorrect model inference result is regarded as one correct inference result.

Figure 6 Performance monitoring based on the subset of Set A

In additional, Alt.1 also needs to discuss the mapping relation between the resource of Set A and the resource of monitoring resource set to identify whether the predicted resource is within the monitoring resource set or not.

Proposal 25: For the case that the resource set(s) for monitoring is configured for a subset of Set A, the performance metric calculation based on valid time instance needs to further solve the following aspects:

- How to ensure the number of valid time instance used for a performance metric calculation;

- How to ensure the accuracy of the performance metric;

- How to define the mapping relation between the resource of Set A and the resource of monitoring resource set.

For Alt.2, the number of resources within monitoring set is less than the number of resources of Set A, and a multi-to-one mapping between the resources for Set A and the resources for monitoring is pre-defined or configured for the UE. The performance metric can be calculated based on the measurement results of monitoring set. Taken an example as shown in Figure 7 where the resources for Set A consist of 12 CSI-RS resources and the resources set for monitoring in case 1 consist of 4 SSBs, three resources in Set A are mapped to one beam within the monitoring set of case 1. Assuming that the predicted Top-1 beam is RS resource 1 within Set A and the measured Top-1 beams of case 1 is SSB#1, the model inference can be viewed as correct since the predicted beam can be mapped to the measured Top-1 beams within monitoring resource set.

Figure 7 Performance monitoring based on the set different from the Set A

For obtaining the mapping for each beam between Set A and the monitoring set, one method is to indicate the mapping to UE, and the other solution is to derive the mapping from the QCL relation of resources in the two resource sets. For example, if resource #i in the monitoring set is the QCL source of resource #j in Set A, the inference result corresponding to resource #j can be mapped to resource #i in the monitoring set.

Proposal 26: For the case that the resource set(s) for monitoring is configured for a resource set different from Set A, support using predefined mapping between the full resources for Set A and the monitoring resources when calculating the performance metric：

- The mapping between Set A and the monitoring set can be explicitly indicated to UE or derived from the QCL relations.

Association between inference report and monitoring report

To calculate the performance metric, both the inference result and measurement result of resources for monitoring is required. In RAN1#119, the configuration of monitoring report was discussed and the following agreement was achieved.

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

The ID of an inference report is configured in the dedicated monitoring report to link the inference report and monitoring report. One FFS issue is whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report. In our understanding, performance monitoring is not required for every inference instance. The frequency of reporting inference result is higher than that of reporting performance monitoring result, or performance monitoring result only reported at necessary moments. For a periodic performance monitoring report, the linked inference report should also be periodic. For a semi-persistent performance monitoring report, the linked performance monitoring report can be either semi-persistent or periodic. For an aperiodic performance monitoring report, the linked performance monitoring report can be periodic, semi-persistent, or aperiodic.

Proposal 27: For Type 1 Option 2 of UE-sided model monitoring, on the time domain behavior of the monitoring report and the linked inference report, support the following combinations:

- A periodic performance monitoring report linked to a periodic inference report;

- A semi-persistent performance monitoring report linked to a periodic or semi-persistent inference report;

- An aperiodic performance monitoring report linked to a periodic, semi-persistent, or aperiodic inference report.

Another FFS is the timing related issue of performance monitoring. To obtain performance monitoring results, the UE needs to compare the inference results with the ground-truth, and the ground-truth is the measurement result from the monitoring resource set(s). To accurately calculate the performance metric, it is necessary to establish the one-to-one linkage between the inference result and the measurement results. Before discussing how to establish the one-to-one linkage, we need to first clarify between whom the linkage is to be established. From our perspective, two candidate alternatives can be considered.

Alt 1: Establish the linkage between the inference report instance and the transmission occasion of monitoring resource.

- As an example, the CSI report for monitoring is periodic, and a periodic inference report is linked to the monitoring report. The beam prediction accuracy is calculated based on W samples, then UE would compare the inference results from W inference report instances with W measurement results derived from W transmission occasions of the monitoring resource.

Alt 2: Establish the linkage between the transmission occasion of measurement resource (Set B) of the inference report and the transmission occasion of monitoring resource.

- For instance, in BM-Case1, an inference report is linked to a CSI report for monitoring. The resource set for Set B is associated with the inference report, while the monitoring resource set is associated with the monitoring report. Since the inference result is derived based on the measurements from certain transmission occasion(s) of Set B resource, the linkage can be established between transmission occasion of Set B resource (Set B) and the transmission occasion of monitoring resource.

For both Alt 1 and Alt 2, UE can first determine the transmission occasion(s) of monitoring resource associated with each monitoring report instance, then determine the linked inference report/Set B transmission, and finally calculate the performance metric.

Proposal 28: For Type 1 Option 2 of UE-sided model monitoring, for the timing related issue, consider one-to-one linkage with the following alternatives:

- Alt 1: One-to-one linkage between the inference report instance and the transmission occasion of monitoring resource;

- Alt 2: One-to-one linkage between the transmission occasion of measurement resource (Set B) of the inference report and the transmission occasion of monitoring resource.

In Alt 1, to establish the one-to-one linkage and calculate the performance metric, the following steps can be adopted by UE.

- Step 1: Determine the transmission occasion(s) of monitoring resource for obtaining the measurement result(s)/ ground-truth

- The measurement results/ground-truth can be derived from the measurements of W consecutive transmission occasion of the monitoring resource no later than the CSI reference resource of the monitoring report, where W can be configure by the network.

- Step 2: Determine the inference report instance linked with each measurement result

- The linked inference report instance needs to meet the condition that the prediction time of the linked inference report instance is closest to the transmission occasion of the monitoring resource in the time domain. Additionally, the report time of the inference report instance should be no later than the CSI reference resource of the monitoring report.

- Step 3: Calculating the performance metric by comparing the measurement result(s) with inference result(s)

In Step 2, to determine the linked inference report instance, the prediction time of an inference report instance should be defined. For BM-Case1 and BM-Case2, it can be defined as follows, respectively.

- For BM-Case1, the prediction time of an inference report instance is the slot of the corresponding CSI reference resource.

- For BM-Case2, the prediction time of the reported inference result is future time instance(s). In Rel-18 CSI prediction in MIMO, the reported PMI indicates the precoder matrices for N4 consecutive slot intervals, each with duration of d slots. This definition can be adapted for BM-Case2, where the prediction time can be N consecutive slot intervals. The issue of earliest time instance of prediction time and the predicted slot intervals for the reported inference result is discussed in Section 3.1.2. In the case where the value of duration for each P is greater than one, the linked inference report instance can be 1) if the transmission occasion of the monitoring resource is within the slot intervals of the prediction time of an inference report instance, the linked inference report instance is the one whose prediction time includes the transmission occasion of the monitoring resource; or 2) if the transmission occasion of the monitoring resource is not within the slot intervals of the prediction time of any inference report instance, the linked inference report instance is the one whose certain slot (e.g., the first slot or the center slot) of the prediction time is closest to the transmission occasion of the monitoring resource.

Figure 8 shows an example of the one-to-one linkage between the inference report instance and the transmission occasion of monitoring resource for BM-Case1. For the monitoring report, it is assumed that in each report instance, the reported beam prediction accuracy is calculated based on comparing W=4 measurement results with 4 inference results. These 4 measurement results are derived by the most recent 4 transmission occasions of the monitoring resource, no later than CSI reference resource of the monitoring report. Since the prediction time of an inference report instance is the slot of the corresponding CSI reference resource, the linked inference report instance for each transmission occasion of the monitoring resource is determined by the CSI reference resource being closest to the transmission occasion of the monitoring resource, and the report time of the inference report instance should be no later than the CSI reference resource of the monitoring report.

Figure 8 One-to-one linkage for BM-Case1

For BM-Case2, the linked inference report instance for each transmission occasion of the monitoring resource is determined by the prediction time of the reported inference result. As shown in Figure 9, the prediction time of each inference report instance corresponds to 2 slot intervals, and the duration of each slot interval is P slots. For each transmission occasion of monitoring resource, UE can identify the slot interval that includes the transmission occasion of the monitoring resource, or the slot interval whose first slot is closest to the transmission occasion. Then, the inference report instance associated with the identified slot interval can be the linked inference report instance. The UE can calculate the performance metric based on the measurement results of the monitoring resource and the inference result of the corresponding slot intervals.

Figure 9 One-to-one linkage for BM-Case2

Proposal 29: For Type 1 Option 2 of UE-sided model monitoring, the inference report instance linked with the transmission occasion of the monitoring resource meets the following conditions:

- For BM-Case 1, the linked inference report instance is the one whose CSI reference resource being closest to the transmission occasion of the monitoring resource;

- For BM-Case 2, the linked inference report instance is the one whose predicted slot intervals include the transmission occasion of the monitoring resource, or a certain slot of the predicted slot intervals of the inference report instance is being closest to the transmission occasion of the monitoring resource;

- The report time of the linked inference report instance is no later than the CSI reference resource of the monitoring report.

For Alt 2, a similar approach can be adopted. The difference from Alt 1 is that in Step 2, to determine the linked transmission occasion of Set B resource, it is necessary to define the corresponding prediction time for the Set B measured on certain slot(s). In BM-Case1, for the transmission occasion of Set B resource in slot n, the corresponding prediction time of the Set B measurement is slot n as well. In BM-Case2, however, a slot offset exists between the transmission occasion of Set B resources and the corresponding prediction time. The slot offset is related to the design of the AI/ML model and can be reported by the UE. In this way, for BM-Case1, the linked transmission occasion(s) of Set B resource is the transmission occasion which is closest to the transmission occasion of monitoring resource. For BM-Case2, the linked transmission occasions of Set B resource are those for which the corresponding prediction time is closest to the transmission occasion of monitoring resource.

Event based reporting

For Type1 Option 2 performance monitoring, event-based reporting can also be considered. For event based reporting, there are two critical issues, one is the specific event(s) to trigger the reporting, and the other one is the uplink resource carrying the report. The specific event for performance monitoring can refer to the beam failure detection, e.g., the number of the performance metric larger than or lower than a threshold exceeds a per-defined/configured value in a period of time. For example, for performance metric Alt.1, the event can occur when the accumulated instances of Top-K/1 beam prediction accuracy lower than the threshold within a time period exceeds a configured or pre-defined value L. For the uplink resource carrying the report, both MAC-CE based and UCI based reporting can be used. For MAC-CE based reporting, the procedure is similar to beam failure reporting. For UCI based reporting, the uplink resource can be pre-configured or request by UE, which is similar as UE initiated BM in Rel-19 MIMO. Since it is more concise to have a unified framework for both event-based and non-event-based reporting, it is better to consider event-based reporting within CSI framework. Hence, UCI based reporting is preferred.

Proposal 30: For Type 1 performance monitoring of Option 2, support event-based reporting, and the specific event can be defined as:

- The accumulated instances of performance metric (e.g., Top-K/1 beam prediction accuracy) lower than or larger than the threshold within a time period exceed a configured value.

Proposal 31: For Type 1 performance monitoring of Option 2, support event-based reporting based on CSI framework, and the uplink resource carrying the report can be pre-configured or requested by UE.

Type 2 monitoring

Type 2 performance monitoring, we think it can also be supported for UE-sided AI/ML functionality. Based on TR 38.843 [10], the difference between Type 1 and Type 2 performance monitoring is whether the performance monitoring is triggered by the network or requested by UE. For UE-sided AI/ML functionality, we think performance monitoring can be requested by UE and the LCM decisions are made by the network. For the request from UE to gNB for performance monitoring, the UE can indicate the information of the AI/ML functionality and the supported performance metric to gNB. Then, gNB can configure the corresponding RS to UE for performance monitoring. For example, if UE sends the request signaling for performance monitoring and indicates Top-K beam prediction accuracy as the performance metric, gNB can configure the resources for both Set B and Set A. But if UE indicates to use the L1-RSRP difference of measured RSRP and predicted RSRP of Set B as the performance metric, gNB will only send the resources for Set B.

Proposal 32: Support Type 2 performance monitoring for UE-sided model, allowing UE to send the request signaling for performance monitoring.

  - CSI processing enhancement

In RAN1#118bis, it was agreed to use the existing CPU mechanism as a starting point for AI/ML-based CSI processing.

Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

In legacy, CPU is used to indicate the capability of UE for simultaneous CSI computations. For AI/ML-based CSI processing, it includes two steps for obtaining the inference results: RS measurement and model inference. UE may use same or different computing modules to perform RS measurement and model inference. For example, UE can use legacy modules for RS measurement and model inference. Also, UE may use legacy module for RS measurement and use GPU or NPU for model inference. If UE uses different modules, a new type of processing unit or enhanced CPU can be introduced. The UE should report the supported number or the limit of the enhanced CPU, which is separately counted from the legacy CPU pool. For an AI/ML-based CSI reporting, it may occupy both legacy CPU and enhanced CPU. Therefore, it should meet the requirements of both types of processing units.

Proposal 33: For UE side AI/ML-based CSI processing, a new type of processing unit or enhanced processing unit can be introduced if different modules are used for RS measurement and model inference. The enhance CPU is separately counted from legacy CPU pool.

Another issue is whether the new type or enhanced CPU should be shared or separately counted among different AI/ML features/functionalities. Since model inference for different features is typically performed on the same hardware module, it is natural to share the enhanced CPU among CSI related AI/ML features/functionalities.

Proposal 34: For UE side AI/ML-based CSI processing, the new type or enhanced CPU should be shared among CSI related AI/ML features/functionalities.

For legacy CSI processing criteria, the number of occupied CPUs for each CSI report is related to the report quantity, the number of resources for measurement, or the time domain behavior of the report. For AI/ML based CSI processing, for inference operation, the number of occupied processing units is related to the AI/ML model complexity (e.g., FLOPs). According to the evaluation results of AI-based BM in Rel-18 [10], the FLOPs of AI/ML model can be vastly different (e.g., from 2.7K to 222M) even for the same use case. Therefore, to determine the occupied processing units for AI/ML-based CSI processing, the AI/ML model complexity related information should be reported by UE.

Proposal 35: For UE side AI/ML-based CSI processing, UE should report the information for determining the occupied processing units.

For legacy beam reporting, the CPU occupation time includes the following:

- For P/SP report (excluding an initial semi-persistent CSI report on PUSCH after triggering), it occupies CPU from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource of latest transmission occasion no later than the corresponding CSI reference resource, until the last symbol of the PUSCH carrying the report.

- For AP report, it occupies CPU from the first symbol after the PDCCH triggering the CSI report until the last symbol of the PUSCH carrying the report.

For determining the CPU/enhanced CPU occupation time of AI/ML-based beam reporting, three aspects differ from legacy beam reporting need to be considered. First, in legacy, the occupied symbols are determined by the resources configured in the CSI report. In contrast, for AI/ML inference report, both Set B resources and Set A resources are configured in the CSI report, but the occupied symbols are only related to the resources for measurement, namely only related to Set B resources. Second, model inference is performed after channel measurements have been collected. Therefore, it is not accurate to assume that the enhanced CPU will be occupied from the beginning of the RS transmission or from the start symbol after PDCCH triggering. The specific symbols during which the enhanced CPU is occupied should be discussed. Third, for BM-Case2, the observation window may last long, such as 640ms or even longer. It is necessary to discuss whether the CPU should be counted from the beginning of the measurement window or from the last measurement instance of Set B within the observation window.

Proposal 36: For UE side AI/ML-based beam reporting, the following aspects regarding CPU/enhance CPU occupation time need to be clarified:

- The occupied symbols are determined by the resources for measurement (e.g., Set B resource);

- How to specify the symbols during which the enhanced CPU is occupied;

- How to specify the symbols during which the CPU is occupied for BM-Case2.

In legacy, when the number of required CPUs for N CSI reports occupying the same OFDM symbols is larger than the supported number of CPUs of the UE, the UE is not required to update the M CSI reports with low priority until the occupied CPUs of (N-M) CSI reports will not exceed the UE capability. One important issue is how to extend the priority rule to accommodate AI/ML-based CSI report. We believe that, on one hand, AI/ML-based CSI report for different functionalities and legacy CSI report should be differentiated in terms of priority. On the other hand, it is also necessary to determine the priorities of CSI report for different AI/ML related operations, such as data collection, model inference, and performance monitoring. For reporting the inference result, new report quantity can be introduced, e.g., predicted-RSRP. Similarly, for performance monitoring, a new report quantity, such as beam prediction accuracy, can be introduced. This approach allows defining the priority of CSI reports based on the specific report quantity.

Proposal 37: Determine the priority of CSI report for different functionalities, as well as for data collection, model inference, and performance monitoring.

- At least define the priority according to the report quantity.

- NW-sided model

  - Reporting overhead reduction

For NW-sided model, UE needs to report the measurement results of Set B beams to network. For the configuration of RS and the reporting of measurement results, the current CSI framework can be used. In RAN1#117 meeting [11], it was agreed to support Top M beam reporting which the value M can equal to the size of Set B. There are still several FFS issues to be discussed.

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

The first FSS is whether support L1-RSRP and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP. It is beneficial for overhead reduction to support reporting up to M beams within X dB gap to the largest measured value of L1-RSRP. For example, the UE can adapt dynamically the number of beams in one report based on the measurement results. Compared with the reporting with fixed number of beams, it can avoid reporting redundant beams when channel state is poor and reporting insufficient beams when channel state is good. Also, in Rel-18 SI [10], it has been observed that if the reported measurement within 14dB~20dB gap to the largest L1-RSRP, there is only about 2%~6% beam prediction accuracy degradation.

Proposal 38: For NW-sided model, for inference report, at least for BM-Case1, support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB.

The second FSS is on the maximum value of M based on UE capability. We think this issue can be discussed in the UE feature part.

Proposal 39: For NW-sided model, for inference report, the maximum value of M based on UE capability should be discussed in the UE feature.

And the third FFS is on beam information. For NW-sided model, the legacy CRI/SSBRI of a resource set can be considered as a baseline since gNB requires configuring the resources with CRI/SSBRI for the RS of Set B beams for measurement. If the CRI/SSBRI size is large due to large number of the RS form Set B, a bitmap indicating RS index of Set B can be introduced to represent the beam information of reported L1-RSRP. For example, the number of the RS of Set B beams is 16 and report content is Top-8 L1-RSRP from the RS of Set B beams. The UE can report the absolute L1-RSRP and a 3-bit indicator for largest L1-RSRP, and report other differential L1-RSRPs with a 16-bit bitmap for corresponding beam information. In this case, the total bit number for beam information in a beam report is 19 bits (3+16=19 bits). But, if legacy CRI/SSBRI is applied to beam information of differential L1-RSRP, the total bit number for beam information in a beam report is 32 bits (4×8=32 bits). Compared with legacy CRI/SSBRI, a bitmap indicating RS index of Set B beams can reduce required bit number for beam information of a beam report.

Also, if the number of beams in Set B is large, the RS of Set B beams may be consisted by more than one resource set, and then the resource set id should also be considered as parts of beam information. For example, for BM-Case2, if Set B beams equal to Set A beams and the number of beams is 256, the resources of the RS from the Set B beams could be configured at least 4 legacy resource sets. In this case, both legacy CRI/SSBRI and resource set id should be considered as beam information.

Proposal 40: For NW-sided model, the following options can be considered for the reported beam information:

- Opt 1: Legacy CRI/SSBRI of a resource set, and resource set id if multiple resource sets consists set B;

- Opt 2：The indicator for largest measured value of L1-RSRP and a bitmap indicating RS index within a resource set, and resource set id if multiple resource sets consists set B.

For NW-sided model for BM Case-2, the UE should report measurement results within measurement window for the inference of NW-sided model. It’s important to enable UE reporting measurement reports of multiple past time instances to reduce reporting overhead. For example, the UE can report L1-RSRP and beam information of largest L1-RSRP from N future time instances, and report other differential L1-RSRPs for the RS of Set B beams within the measurement window in a pre-defined time instance order.

Also, for BM-Case1, if latency requirement of beam reporting is not very urgent, e.g., for model training, the beam report method for BM-Case2 can be re-used for BM-Case1 as well for overhead reduction of beam report.

Proposal 41: For NW-sided model for BM-Case2, for inference, support to report largest L1-RSRP from N time instances and other differential L1-RSRP of N time instances within the measurement window in a pre-defined order in a beam report.

In RAN1#117, it has been agree to support differential L1-RSRP reporting with legacy quantization step and range at least for NW-sided model. There is still a FFS on whether to support larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP. According to the evaluation results captured in TR [10], a larger quantization step size, e.g. 4 dB step size for reporting differential L1-RSRP, showed only less than 5% beam prediction accuracy degradation. It’s feasible to introduce a larger quantization step size for differential L1-RSRP to reduce overhead of beam report.

Proposal 42: At least for NW-sided model, for inference, support to introduce a larger quantization step size for differential L1-RSRP reporting.

  - Content for data collection for training

In RAN1#118bis FL summary, there is a discussion on the content for data collection for training as follows.

Proposal [6]: For content for data collection for training for NW-sided model via higher layer signaling, for both BM-Case 1 and BM-Case2, for each per instance, support the following options: - Opt 1: L1-RSRPs measured based on one resource set (for Set A and Set B) [FFS: two sets] configured to UE - Opt 2: L1-RSRPs measured based on one resource set (for Set B) [FFS: two sets] configured to UE, and beam information of Top K measured based on another resource set (for Set A) configured to UE - Differential L1-RSRP reporting is supported with legacy quantization steps and ranges - FFS on whether other values for quantization steps and/or ranges can be configured by NW. - FFS on whether/how to determinate a subset of L1-RSRPs, including - Alt 1: Top M beam(s) is the beams with largest M measured values of L1-RSRPs, where M is configured by gNB - Alt 2: All beams within X dB gap to the largest measured value of L1-RSRP - FFS on whether/how to define “per time instance” - FFS on whether/how to contain time stamp information for BM-Case 2

For the sake of a brief discussion, we can start with BM-Case1. If an AI/ML model deployed at NW-side is a regression model, the L1-RSRP measurement results for both of Set A and Set B are required for NW-side. If an AI/ML model deployed at NW-side is a classification model, the L1-RSRP measurement results for Set B and Top-K beams of Set A are required for NW-side. Thus, Opt 1 can be applicable to both of regression model and classification model, and Opt 2 only can be applicable to classification model. Actually, the RS resources of Set A and Set B can be transparent to the UE. This means the UE doesn’t need to know whether the RS set corresponds to Set A/Set B. In fact, Set A or Set B can be made up of one or more resource sets. For example, the number of RS of Set A is 256, and the maximum number of RS configured for a resource set is 64, then a Set A can be made up of at least 4 resource sets. Thus, the L1-RSRP measurement and Top-K measured can be based on one or more resource set(s), which is up to gNB configuration.

Also for BM-Case2, the content for data collection for NW-sided model of each time instance can be same as one time instance of BM-Case1, and the RS configuration for BM-Case2 can be totally up to gNB configuration. In additional, since it has agreed to support differential L1-RSRP reporting with legacy quantization step and range in L1 signaling for NW-sided model, this principle can be reused for the L1-RSRP reporting via higher layer signaling.

Proposal 43： For content for data collection for training for NW-sided model via higher layer signaling, for both BM-Case1 and BM-Case2, for each per instance, support the following options:

- Opt 1: L1-RSRPs measured based on resource set(s) (for Set A and Set B) configured to UE;

- Opt 2: L1-RSRPs measured based on resource set (s) (for Set B) configured to UE, and beam information of Top K measured based on resource set(s) (for Set A) configured to UE;

- Differential L1-RSRP reporting is supported with legacy quantization steps and ranges.

  - Performance monitoring

For performance monitoring of NW-sided model, network monitors the performance metric and makes decision of LCM operations, such as model/functionality activation/deactivation/fallback operations. For calculating the performance metric, UE needs to report benchmark/reference to network. The specific reporting contents are related to the performance metric and can be configured by the network. As discussed in Section 3.3, we support using beam prediction accuracy related KPIs and the L1-RSRP difference as the performance metric. We summarize the reporting contents for calculating different performance metrics in Table 3. It can be noted that enhancement of new reporting quantity (e.g., the RS indicators corresponding to the beams within X dB margin of ideal Top-1 beam) is needed.

Table 3 Reporting contents for different performance metrics

| Performance metrics | Reporting contents of benchmark/reference |
| --- | --- |
| Top-1 beam prediction accuracy | The RS indicator corresponding to the ideal Top-1 beam |
| Top-K/1 beam prediction accuracy, | The RS indicator corresponding to the ideal Top-1 beam |
| Top-K beam prediction accuracy within X dB margin | The RS indicators corresponding to the beams within X dB margin of ideal Top-1 beam |
| L1-RSRP difference of Set B | Measured L1-RSRP of beams in Set B |
| L1-RSRP difference of predicted Top-K beams | Measured L1-RSRP of predicted Top-K beams |

For reporting the RS indicator corresponding to the ideal Top-1 or Top-K beam(s), the rule for selecting the reported beam(s) can be specified. In legacy beam reporting, the network configures the number of reported beams, but how to select the beams for reporting can be up to UE. For performance monitoring, UE should report the beams with Top-K L1-RSRP value(s) to facilitate the network obtain accurate performance metrics. In RAN1#107, this rule is already agreed for inference report of NW-sided model, it can be reused for performance monitoring.

Proposal 44: For performance monitoring of NW-sided model, support to report the RS indicators corresponding to the beams within X dB margin of ideal Top-1 beam.

Proposal 45: For performance monitoring of NW-sided model, the content in a beam report includes the beam information of Top M beam(s) with largest M measured L1-RSRP(s).

- Top-K beam measurement

As discussed in Rel-18 SI, the beam prediction accuracy of Top-K beam is the percentage of ‘Top-1 genie-aided beam is one of the predicted Top-K beams’. Consequently, the greater the value of K, the higher beam prediction accuracy can be achieved. When K>1, a second round Tx beam sweeping (P2 procedure) may be needed for both NW-sided and UE-sided model to find the best Tx beam among the predicted Top-K beams. On the other hand, it can enable UE to find a suitable Rx beam for subsequent data transmission. For instance, as shown in Figure 10, the predicted Top-4 beams of a UE-sided model are highlighted with blue, and after second round of Top-K beam sweeping, the UE is indicated to use one Tx beam from the predicted Top-4 beams for data transmission.

Figure 10 Top-K beams measurement

Regarding the Top-K beam sweeping, since the predicted Top-K beam(s) for each inference result are different, it is appropriate to conduct the Top-K beam measurement by triggering an aperiodic CSI report. In current CSI framework, network can configure the UE with a list of aperiodic trigger states, and each trigger state is linked to one or multiple CSI reports. The Resource Setting associated with each CSI report is semi-statically configured. When a trigger state is indicated via the CSI request field in DCI, the associated CSI report(s) is triggered. However, the maximum number of trigger states for aperiodic CSI reporting in current specification is 128, which is not only for the purpose of beam reporting. For example, if there are 32 beams in Set A and the model output is Top-4 predicted beam in Set A, all the possible combinations for the output of AI/ML is C324=35960. As a result, it’s impossible for network to configure all possible combinations of K beams in Set A, and it also brings complexity for network to trigger second round beam sweeping for Top-K beams. Hence, the problem of how to measure the predicted Top-K beams in the CSI framework should be solved.

Since the predicted Top-K beams are from Set A, one solution is that the configured CSI resource set in an aperiodic CSI report corresponds to Set A, and the network can indicate the active resources for measurement from the configured resource set according to the beam information of predicted Top-K beam(s). As illustrated in Figure 11, the AP CSI report#1 is associated with an aperiodic resource setting, wherein the resource set configured in the resource setting includes 32 resources. Upon obtaining the beam information of the predicted Top-K beams, the gNB can send an indication in slot n1 for indicating 4 resources used for measurement of AP CSI report #1. When the gNB triggers AP CSI report#1 in slot n2, UE will measure the 4 activated resources and subsequently report the measurement results. The indication for active resource can be conveyed by MAC-CE signaling.

Figure 11 Active resources for measurement can be indicated by the network

Another alternative is to introduce a dynamic indication of QCL relationships. In this way, K preconfigured resources can be associated with an aperiodic CSI report. The time and frequency domain locations of the K resources are configured, but the QCL information for the K resources remains unconfigured. Once the beam information of the predicted Top-K beams is obtained, the gNB can determine the QCL relationships of the K resources and indicate the QCL relations to UE. Furthermore, the gNB has the option to decide to measure only a subset of the predicted Top-K beams. In such cases, only S (where S<K) QCL relations need to be indicated.

Proposal 46：For Top-K beam measurements (P2 procedure) of NW-sided and UE-sided model, the following options can be considered:

- Option 1: NW can dynamically indicate the active resources for measurement among the pre-configured RS resources associated with the CSI report;

- Option 2: NW can dynamically indicate the QCL relationship for the pre-configured RS resources associated with the CSI report.

- RRC parameters

For NW-sided model, for inference report, the following agreement was achieved in RAN1#117 meeting [11].

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

According to the agreement, the UE is required to report the beams with largest-M L1-RSRP value(s) of a measurement resource set. In contrast, in legacy beam report, the selection of the M beams for reporting is left to UE implementation. To differentiate the beam reporting for NW-sided model from legacy reporting, an indication can be configured in the report. For example, the report quantity can be explicitly defined as Top-M beam/RSRP. This would specify the selection criteria for the reported beams and ensure the quality of the input of the NW-sided model.

Proposal 47：Introduce a new report quantity (e.g., Top-M beam/RSRP) in the CSI report to facilitate the beam reporting for NW-sided model.

For inference report of UE-sided model, to differentiate the report from legacy beam reporting, a new report quantity such as predicted RSRP can be introduced. For BM-Case2, it was agreed to report inference results of N future time instance(s)/slot interval(s). The value of N can be configured via RRC parameter.

Proposal 48：Introduce a new report quantity (e.g., predicted RSRP) in the CSI report to facilitate the inference report of UE-sided model.

Proposal 49：For BM-Case2, the value of future time instance(s)/slot interval(s) for which inference results are reported can be configured via RRC.

- Conclusion

In this contribution, we discuss the specification support for AI/ML-based beam management. The proposals are summarized as follows.

Proposal 1: Conclude that for UE sided model in beam management, no need to define the similar properties of a DL Tx beam or beam set/list with the associated ID.

Proposal 2: The associated ID can be configured within CSI framework of inference reporting.

Proposal 3: For configuring the associated ID within CSI framework, support to configure associated ID at CSI-Report level.

Proposal 4: For the consistency of training and inference in different cells, performance monitoring based method can be supported.

Proposal 5: For NW-sided model, it is beneficial to align the Rx information of the measurements between network and UE.

Proposal 6: For NW-sided model, UE should use fixed Rx beam for each measured beam in Set A/Set B, and the specific Rx beam used for measurement can be up to UE.

Proposal 7: For inference of UE-sided model, at least for BM-Case1, the following alternatives can be further supported for the CSI-ReportConfig of inference results reporting:

- Alt 1: one CSI-ResourceConfigId is configured for Set B, and Set A can be determined from the associated id.

Proposal 8: For UE-sided AI/ML model inference, for BM-Case2, AP CSI-RS can be supported for CMR configuration at least for Set B.

Proposal 9: For UE-sided AI/ML model inference of BM-Case2, for periodic or semi-static CSI-RS configuration, one RS set can be configured as CMR for the inference report and the separation between two consecutive measurements is the periodicity of RS set.

Proposal 10: For UE-sided AI/ML model inference of BM-Case2, for aperiodic CSI-RS configuration, consider two potential options as follows:

- Option 1: One resource set with the resource of Set B, the transmission number of the RS set, and the separation between two consecutive transmissions can be configured for one triggering instance. Multiple transmissions of the resource set are triggered by one triggering instance.

- Option 2: Multiple resource sets can be configured for one trigger state and each resource set can be configured with trigger offset separately. Multiple resource sets with different slot offsets are triggered by one triggering instance.

Proposal 11: For UE-sided model for BM-Case2, for inference results report, the following option can be considered for determining the reference time of the earliest time instance for the predicted results:

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource.

Proposal 12: For UE-sided model for BM-Case2, the UE is expected to report the predicted results for slot intervals l+j∙P, l+j+1∙P-1, for j=0,…N-1, where l is the earliest predicted time instance, N is the number of the consecutive predicted slot interval(s) and P is duration of each slot interval.

Proposal 13: For UE-sided model for BM-Case2, for the duration values of the slot interval(s) that can be predicted support at least 320ms /480ms/640ms.

Proposal 14: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported beam information can be the pre-defined beam index of the predicted Top K beam(s).

- the pre-defined beam index can be 0 to M-1 , where M is the number of beams in Set A

Proposal 15: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported RSRP type of predicted Top K beam(s) can be configured by gNB with the following options:

- Option A: Predicted L1-RSRP;

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.

Proposal 16：For UE-sided mode, at least for BM-Case1, for content in the report of inference results, for Opt 1(only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K>1 can be conveyed by the order of the beam information based on the predicted probability information or the predicted L1-RSRP.

Proposal 17: For UE-sided model for BM-Case2, for inference report, support to report largest L1-RSRP among multiple beams over multiple time instances and other differential L1-RSRP of multiple beams over multiple time instances with a reference to the largest L1-RSRP in a pre-defined order.

Proposal 18: From RAN1 perspective, the existing TCI state switching delay requirements can be reused for the predicted RS resource of UE sided-model.

Proposal 19: For TCI indication of BM-Case2, there is no need to extend the Rel-17 TCI state activation/indication signaling methods to activate/indicate TCI state indication of multiple future time instances.

Proposal 20: For Type 1 performance monitoring of Option 1, support per-sample based reporting with CSI framework as the baseline.

Proposal 21: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Type 1 Option 2, the reporting quantity of the CSI report for monitoring is a quantized value of Np/N, where Np is the number of “successful inference instances”, and N denotes the number of performance monitoring instances.

- the successful inference instance is defined as

  - one of the predicted Top-K beams is the Top-1 beam from the resources for monitoring

  - one of the predicted Top-K beams is within X dB margin of the Top-1 beam from the resources for monitoring

Proposal 22: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Type 1 Option 2, further support the following metrics:

- Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set for monitoring

  - Alt2-1：The L1-RSRP difference between the largest L1-RSRP measurements of the Top K predicted beam(s) and the largest L1-RSRP measurements of a resource set for monitoring

  - Alt2-2：The mean of the absolute difference of L1-RSRP between the actual L1-RSRP measurements of the Top K predicted beam(s), and the Top-K largest L1-RSRP measurements of a resource set for monitoring

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring

  - The corresponding beams can be predicted Top-K beam(s) or Set B

Proposal 23：For Type 1 performance monitoring of Option 2, for resource configuration of Set A, study how to configure the resources when the number of beams is larger than the max number of CSI-RS resources can be configured to UE for L1-RSRP measurement.

Proposal 24：For Type 1 performance monitoring of Option 2, for resource configuration of Set A, reuse the enhancement in Rel-19 MIMO to transmit the aperiodic resources configured in a RS set within two consecutive slots.

- Study whether there is a need to trigger the aperiodic RS transmitted in more than two slots.

Proposal 25: For the case that the resource set(s) for monitoring is configured for a subset of Set A, the performance metric calculation based on valid time instance needs to further solve the following aspects:

- How to ensure the number of valid time instance used for a performance metric calculation;

- How to ensure the accuracy of the performance metric;

- How to define the mapping relation between the resource of Set A and the resource of monitoring resource set.

Proposal 26: For the case that the resource set(s) for monitoring is configured for a resource set different from Set A, support using predefined mapping between the full resources for Set A and the monitoring resources when calculating the performance metric：

- The mapping between Set A and the monitoring set can be explicitly indicated to UE or derived from the QCL relations.

Proposal 27: For Type 1 Option 2 of UE-sided model monitoring, on the time domain behavior of the monitoring report and the linked inference report, support the following combinations:

- A periodic performance monitoring report linked to a periodic inference report;

- A semi-persistent performance monitoring report linked to a periodic or semi-persistent inference report;

- An aperiodic performance monitoring report linked to a periodic, semi-persistent, or aperiodic inference report.

Proposal 28: For Type 1 Option 2 of UE-sided model monitoring, for the timing related issue, consider one-to-one linkage with the following alternatives:

- Alt 1: One-to-one linkage between the inference report instance and the transmission occasion of monitoring resource;

- Alt 2: One-to-one linkage between the transmission occasion of measurement resource (Set B) of the inference report and the transmission occasion of monitoring resource.

Proposal 29: For Type 1 Option 2 of UE-sided model monitoring, the inference report instance linked with the transmission occasion of the monitoring resource meets the following conditions:

- For BM-Case 1, the linked inference report instance is the one whose CSI reference resource being closest to the transmission occasion of the monitoring resource;

- For BM-Case 2, the linked inference report instance is the one whose predicted slot intervals include the transmission occasion of the monitoring resource, or a certain slot of the predicted slot intervals of the inference report instance is being closest to the transmission occasion of the monitoring resource;

- The report time of the linked inference report instance is no later than the CSI reference resource of the monitoring report.

Proposal 30: For Type 1 performance monitoring of Option 2, support event-based reporting, and the specific event can be defined as:

- The accumulated instances of performance metric (e.g., Top-K/1 beam prediction accuracy) lower than or larger than the threshold within a time period exceed a configured value.

Proposal 31: For Type 1 performance monitoring of Option 2, support event-based reporting based on CSI framework, and the uplink resource carrying the report can be pre-configured or requested by UE.

Proposal 32: Support Type 2 performance monitoring for UE-sided model, allowing UE to send the request signaling for performance monitoring.

Proposal 33: For UE side AI/ML-based CSI processing, a new type of processing unit or enhanced processing unit can be introduced if different modules are used for RS measurement and model inference. The enhance CPU is separately counted from legacy CPU pool.

Proposal 34: For UE side AI/ML-based CSI processing, the new type or enhanced CPU should be shared among CSI related AI/ML features/functionalities.

Proposal 35: For UE side AI/ML-based CSI processing, UE should report the information for determining the occupied processing units.

Proposal 36: For UE side AI/ML-based beam reporting, the following aspects regarding CPU/enhance CPU occupation time need to be clarified:

- The occupied symbols are determined by the resources for measurement (e.g., Set B resource);

- How to specify the symbols during which the enhanced CPU is occupied;

- How to specify the symbols during which the CPU is occupied for BM-Case2.

Proposal 37: Determine the priority of CSI report for different functionalities, as well as for data collection, model inference, and performance monitoring.

- At least define the priority according to the report quantity.

Proposal 38: For NW-sided model, for inference report, at least for BM-Case1, support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB.

Proposal 39: For NW-sided model, for inference report, the maximum value of M based on UE capability should be discussed in the UE feature.

Proposal 40: For NW-sided model, the following options can be considered for the reported beam information:

- Opt 1: Legacy CRI/SSBRI of a resource set, and resource set id if multiple resource sets consists set B;

- Opt 2：The indicator for largest measured value of L1-RSRP and a bitmap indicating RS index within a resource set, and resource set id if multiple resource sets consists set B.

Proposal 41: For NW-sided model for BM-Case2, for inference, support to report largest L1-RSRP from N time instances and other differential L1-RSRP of N time instances within the measurement window in a pre-defined order in a beam report.

Proposal 42: At least for NW-sided model, for inference, support to introduce a larger quantization step size for differential L1-RSRP reporting.

Proposal 43： For content for data collection for training for NW-sided model via higher layer signaling, for both BM-Case1 and BM-Case2, for each per instance, support the following options:

- Opt 1: L1-RSRPs measured based on resource set(s) (for Set A and Set B) configured to UE;

- Opt 2: L1-RSRPs measured based on resource set (s) (for Set B) configured to UE, and beam information of Top K measured based on resource set(s) (for Set A) configured to UE;

- Differential L1-RSRP reporting is supported with legacy quantization steps and ranges.

Proposal 44: For performance monitoring of NW-sided model, support to report the RS indicators corresponding to the beams within X dB margin of ideal Top-1 beam.

Proposal 45: For performance monitoring of NW-sided model, the content in a beam report includes the beam information of Top M beam(s) with largest M measured L1-RSRP(s).

Proposal 46：For Top-K beam measurements (P2 procedure) of NW-sided and UE-sided model, the following options can be considered:

- Option 1: NW can dynamically indicate the active resources for measurement among the pre-configured RS resources associated with the CSI report;

- Option 2: NW can dynamically indicate the QCL relationship for the pre-configured RS resources associated with the CSI report.

Proposal 47：Introduce a new report quantity (e.g., Top-M beam/RSRP) in the CSI report to facilitate the beam reporting for NW-sided model.

Proposal 48：Introduce a new report quantity (e.g., predicted RSRP) in the CSI report to facilitate the inference report of UE-sided model.

Proposal 49：For BM-Case2, the value of future time instance(s)/slot interval(s) for which inference results are reported can be configured via RRC.

- References

- RP-243244, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Madrid, Spain, December 9-12, 2024.

- Chair’s Notes RAN1#118, Maastricht, NL, August 19th – 23th, 2024.

- R1-2307078, Evaluation and discussion on AI/ML for beam management, CATT, August, 2023.

- Chair’s Notes RAN1#119, Orlando, US, November 18th – 22nd, 2024.

- Chair’s Notes RAN1#118bis, Hefei, China, October 14th – 18th, 2024.

- R1-2409305, “FL summary #5 for AI/ML in beam management”, Samsung (Moderator), October 14th – 18th, 2024.

- 3GPP TS 38.133 v 18.3.0, Requirements for support of radio resource management.

- 3GPP TS 38.331 v18.1.0, Radio Resource Control (RRC) protocol specification.

- 3GPP TS 38.822 v17.1.0, User Equipment (UE) feature list.

- 3GPP TR 38.843 v18.0.0, Study on AI/ML for NR air interface.

- Chair’s Notes RAN1#117 Fukuoka, Japan, May 20th – 24th, 2024.