3GPP TSG RAN WG1 #120                                                                                   R1-2500217

Athens, Greece, February 17th – 21st, 2025

Source:	CATT

Title:	Views on enhancement for Rel-19 UE-initiated/event-driven beam management

Agenda Item:	9.2.1

Document for:	Discussion and Decision

Introduction

In RAN1 #119 meeting, agreements were achieved regarding additional trigger events, the measurement RS when only one TRS is configured in the indicated TCI state, periodicity of the current beam RS and the new beam RS, the type and one-bit indication of the first channel of mode-A and mode-B, transmission of the second channel of mode-B, PUCCH group for cross-CC beam report for UE-initiated/event-driven beam management [1]. In this contribution, remaining issues on UE-initiated/event-driven beam management are discussed, including triggering events, RS resources for beam measurement, UL report contents, UL report container and cross-CC configuration.

Discussion

  - Triggering events

According to the agreement in RAN1#117 meeting [2], one open issue for triggering event determination of Event 2 is whether the timing window for counting Event-2 instance should be captured in RAN1 or RAN2 spec: In the legacy BFR procedure, UE reports the L1 measurement results in MAC CE and the counting is performed in L2 within the time window. Thus, the legacy BFR procedure is captured in RAN2 spec. However, for UEIBM, the RS for current beam is implicitly derived from indicated TCI state and the report is via UCI. The measurement and reporting are performed in RAN1 only. Therefore, the time window and the counting should be done in RAN1 and such procedure should be captured in RAN1 spec.

Proposal 1: Regarding triggering event determination for Event-2, support to capture the timing window for counting Event-2 instance in RAN1 spec.

Another remaining issue regarding the triggering event determination for Event-2 is the evaluation periodicity for determining Event-2 instance.

Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical.

It is our view that Alt-2 should also be supported. Otherwise, gNB has to guarantee that all the RS for any new beams and the potential current beam should be the same. For UEIBM, the RS for current beam is dynamically indicated via TCI state while the RS for the new beam are configured via RRC parameter. Same periodicity for both cannot be guaranteed. If the periodicities have to be the same, it will deteriorate the scheduling flexibility for gNB. Another possible case is that the RS for the current beam is not included in the new beam set. In this case, it is not necessary to restrict that the periodicities of the new beam and the current beam are the same. Based on the above, Alt-2 should be supported. Among the sub-options of Alt 2, Alt 2-4 is preferred, which is similar as periodicity determined for radio link quality evaluation.

Proposal 2: Regarding the evaluation periodicity for determining Event-2 instance, support Alt-2_4.

  - RS resources for beam measurement

In RAN1#119 meeting, the following agreement was achieved regarding the determination for either scheme-1 or scheme-2 for Event-1 [1]:

Agreement On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement - Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced. - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

For Event-1, configuration of new beam is not needed. Therefore, option-1 is more reasonable compared to option-2.

Proposal 3: Regarding the RS measurement for Event-1, support option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced.

Regarding the triggering event determination for Event-2, one remaining issue is whether event instance(s) counting resetting is needed and the candidate condition(s) of resetting the counting. As the RS resource of the current beam is determined by indicated TCI state, it may change during the event instance(s) counting for Event-2. It is our view that resetting the counting is necessary when the RS resource for current beam is changed.

Proposal 4: Regarding the event instance(s) counting for Event-2, at least support resetting the counting when the RS resource of the current beam is changed.

Regarding RS configuration for the new beam for Event-2, it was agreed that the RS(s) for new beam(s) were explicitly configured in one RS resource set associated with one CSI reporting configuration. One open issue is whether the RS in the RS resource set can be updated by MAC-CE or not. Updating the TCI state via MAC CE can reduce the latency compared with the re-configuration of RRC. Thus, it is beneficial to support the update via MAC-CE.

Proposal 5: Regarding explicit RS configuration for new beam management for Event-2, support updating the RS resources in the RS resource set for new beam by MAC-CE.

  - UL report content

For the signaling content for reporting L1-RSRP regarding Event-2, the following agreement was made for the extension of the current reporting content in RAN1#119 meeting [1]:

Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement. Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2 FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A.

Based on the current reporting scheme, it is agreed that at least one reported CRI/SSBRI should satisfy the condition of Event-2, which is sufficient for gNB to perform beam switching. Introducing additional indication of whether the CRI/SSBRI satisfies the condition of Event-2 may lead to extra overhead and potential latency issue, which should be avoided for UEIBM. Similarly, introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window is not necessary. Reporting L1-RSRP and CRI/SSBRI is sufficient for gNB to acquire the knowledge needed for further beam switching and extra overheads should be avoided.

Proposal 6: Regarding the extension on L1-RSRP report format depending on Event-2, support Option-3: No further enhancement.

In RAN1#117 meeting [2], it was agreed that the UE initiated beam reporting can occur when a number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M. However, it is not mentioned which RSRP value is reported through the M satisfied values. As this condition is within a configured time window, the last satisfied RSRP values should be reported as it represents the latest current channel status which has the highest value in the manner of time.

Proposal 7: For event-2 in UE-initiated/event-driven beam management, when M event-2 instance(s) for one same new beam are used to trigger beam report within a configured time window, the latest RSRP value is reported for this new beam.

  - UL report container

In RAN1#119 meeting, regarding the dedicated RRC signaling for first PUCCH channel configuration, it was agreed to introduce a new UCI type for the first PUCCH channel while there are  three cases of the overlapping issues needs discussions [1]:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases: - Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH - Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain.

Although a new UCI type is introduced for the first PUCCH channel, it has been agreed that the legacy multiplexing and dropping rule of SR should be reused. Based on the principle, for case-1 for the overlapping between the new UCI for UEIBM and the SR/LRR, the priority of the new UCI is the same as the normal SR. If the 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR, the legacy multiplexing/dropping rule(s) of colliding/overlapping of two PUCCHs carrying SR should be extended to case-1. For case-2 for the overlapping between the new UCI for UEIBM and the PUSCH, the new UCI should be regarded as SR and the legacy rules for multiplexing PUCCH carrying SR with PUSCH should be re-used. For case-3, it can be avoided by the implementation of gNB.

Proposal 8: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, following solutions are considered:

- The legacy multiplexing/dropping rule(s) of colliding/overlapping of two PUCCHs carrying SR is extended to Case 1.

- For Case-2, the new UCI should be regarded as normal SR and re-use the legacy rules for multiplexing PUCCH carrying SR with PUSCH.

- Case-3 is avoided by NW’s implementation.

There is another issue whether the CSI trigger state for UEIBM is dedicated or can be associated with legacy AP-CSI configuration step-2 of Mode-A:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120 - Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration. - Option-2: A CSI trigger state can be associated with - only UE-initiated/event-driven beam reporting - or only legacy AP-CSI configuration - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration

Legacy CSI reporting framework supports maximum of 128 aperiodic trigger states, with each aperiodic trigger state associates maximum of 16 report configurations for aperiodic reporting. Each codepoint of the DCI field “CSI request” is associated with one trigger state. For UE-initiated beam reporting, one or more of the CSI trigger states can be dedicated to UE-initiated/event-driven beam reporting. This can be achieved by the following configuration:

CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState CSI-AperiodicTriggerState ::= SEQUENCE { UEIBM-AssociatedReportConfigInfo-r18 UEIBM-CSI-ReportConfigId-r18 OPTIONAL -- Need R }

Upon reception of the indication in “CSI request” of DCI field, the UE will transmit UEIBM reporting on the allocated PUSCH schedule by the DCI format 0_1/0_2.

Proposal 9: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, support using dedicated CSI trigger state(s) for UE-initiated/event-driven beam reporting, i.e., option-1.

In RAN1#118bis meeting [3], regarding Mode-B, it was agreed that the UEIBM beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. In this case, the definition of the available transmission occasion needs clarification.

Proposal 10: For the Mode-B of UL report container for UEIBM, further study the definition of the “available transmission occasion” of the second channel.

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, one pending issue is that whether the first PUCCH resource and pre-configured resource for the second UL channel can have different periodicities:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, - The UE expects that there is the same periodicity (in ms) between first PUCCH resource and pre-configured resource for second UL channel. - FFS: Whether first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms)

If the periodicity of the current beam is large than the new beam, it will lead to potential waste of the resource for the second channel as the first channel is used to notify the reporting on the second channel. If the periodicity of the current beam is smaller than the new beam, it will lead to higher latency as multiple first channel may be transmitted for one second channel. Therefore, it is not necessary to support that the first PUCCH resource and the second UL channel have different periodicities.

Proposal 11: Regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, do not support that first PUCCH resource and pre-configured resource for second UL channel can have different periodicities.

In RAN1 #119 meeting, regarding whether/how to apply prohibit-timer or maximum number of (re)transmission(s) for first PUCCH channel, two options were provided:

- Option-1: For both mode-A and mode-B: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

  - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

- Option-2: For both mode-A and mode-B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

  - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

- If the prohibit timer is running, the first PUCCH is not allowed to be transmitted.

In the legacy mechanism, upon triggering of an SR, the UE transmits the positive SR on a valid PUCCH resource when the prohibit timer is not running. Before the SR is cancelled, UE can transmit the SR until the number of transmissions reaches the maximum value. i.e., the prohibit timer is used to avoid frequent SR transmission in consecutive transmission occasions. The SR is cancelled when the uplink transmission requesting uplink resource can be transmitted on an available resource, e.g., when a MAC PDU including a MAC CE for BFR is transmitted. Similarly, in the UE-initiated beam management, similar prohibition mechanism can be re-used for the first PUCCH transmission to avoid frequent SR transmissions for Mode A and UCI indications for Mode B, respectively, in consecutive transmission occasions. The UE can start the prohibit timer at the first symbol after the end of the PUSCH transmission.

Proposal 12: On prohibit timer for Mode-A and/or Mode B for UE-initiated/event-driven beam reporting for CSI report configuration, for both mode-A and mode-B, in the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit the first PUCCH; At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer.

## 2.5 Cross-CC configuration

In RAN1#119 meeting, following agreement was achieved regarding the cross-CC framework for UEIBM [1]:

Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group. - (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability

One remaining issue is whether the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting can from different PUCCH groups. Such restriction is not necessary as it is up to gNB to schedule whether PUSCH can be from other CCs or not. This is beneficial to scheduling flexibility on NW side. In this case, the working assumption should be confirmed.

Proposal 13: Regarding cross-CC framework for UEIBM, confirm the following working assumption:

- Support the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups.

  - Subject to separate UE capability.

There are two more issues for further study: The first is whether the current beam RS and new beam RS(s) can be in the same CC or in different CCs and the second is whether the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs.

Regarding the first issue, as Event-2 is triggered by comparing RSs from the current beam and the new beam, it is fair that both the RSs are in the same CC. Regarding the second issue, in the legacy, the TCI state and the QCL-RS can be from different CC(s). For UEIBM, there is no need to restrict that the indicated TCI states and the new beam RSs should be in the same CC to deteriorate the scheduling flexibility.

Proposal 14: On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the current beam RS and new beam RS(s) should be in the same CC.

Proposal 15: On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs.

Conclusions

In this contribution, we provide our views on the enhancements for UE-initiated beam management, including triggering events, RS resources used for beam measurement, UL report contents, UL report container and cross-CC configuration. We have the following proposals:

Proposal 1: Regarding triggering event determination for Event-2, support to capture the timing window for counting Event-2 instance in RAN1 spec.

Proposal 2: Regarding the evaluation periodicity for determining Event-2 instance, support Alt-2_4.

Proposal 3: Regarding the RS measurement for Event-1, support option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced.

Proposal 4: Regarding the event instance(s) counting for Event-2, at least support resetting the counting when the RS resource of the current beam is changed.

Proposal 5: Regarding explicit RS configuration for new beam management for Event-2, support updating the RS resources in the RS resource set for new beam by MAC-CE.

Proposal 6: Regarding the extension on L1-RSRP report format depending on Event-2, support Option-3: No further enhancement.

Proposal 7: For event-2 in UE-initiated/event-driven beam management, when M event-2 instance(s) for one same new beam are used to trigger beam report within a configured time window, the latest RSRP value is reported for this new beam.

Proposal 8: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, following solutions are considered:

- The legacy multiplexing/dropping rule(s) of colliding/overlapping of two PUCCHs carrying SR is extended to Case 1.

- For Case-2, the new UCI should be regarded as normal SR and re-use the legacy rules for multiplexing PUCCH carrying SR with PUSCH.

- Case-3 is avoided by NW’s implementation.

Proposal 9: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, support using dedicated CSI trigger state(s) for UE-initiated/event-driven beam reporting, i.e., option-1.

Proposal 10: For the Mode-B of UL report container for UEIBM, further study the definition of the “available transmission occasion” of the second channel.

Proposal 11: Regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, do not support that first PUCCH resource and pre-configured resource for second UL channel can have different periodicities.

Proposal 12: On prohibit timer for Mode-A and/or Mode B for UE-initiated/event-driven beam reporting for CSI report configuration, for both mode-A and mode-B, in the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit the first PUCCH; At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer.

Proposal 13: Regarding cross-CC framework for UEIBM, confirm the following working assumption:

- Support the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups.

  - Subject to separate UE capability.

Proposal 14: On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the current beam RS and new beam RS(s) should be in the same CC.

Proposal 15: On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs.

References

- Chairman’s Notes, 3GPP RAN1 #119, Orlando, US, November,18th – 22th, 2024.

- Chairman’s Notes, 3GPP RAN1 #117, Fukuoka, Japan, May 20th – 24th, 2024.

- Chairman’s Notes, 3GPP RAN1 #118bis, Hefei, China, October,  14th – 18th, 2024.