3GPP TSG RAN WG1 #120                                                                                    		R1-2500225

Athens, Greece, February 17th – 21st, 2025

# Source:             CATT

Title:                  Ambient IoT channel coding and small frequency shift

Agenda Item:    9.4.2

Document for:  Discussion and Decision

Introduction

In RAN#106 meeting, a new WID on Ambient IoT in NR was approved [1]. The general Scope and objectives related to physical layer design for Rel-19 Ambient IoT are listed below.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this contribution, we discuss the channel coding design and small frequency shift for NR Ambient IoT communication system, including line coding, FEC, CRC, repetition and small frequency shift aspects.

Channel coding design

Based on Rel-19 A-IoT WID, R2D line code, D2R line code, D2R FEC code, D2R repetition code, CRC code for R2D/D2R link and small frequency shift techniques are discussed in this section.

R2D link channel coding

Rel-19 A-IoT WI focuses on Device 1, which does not have the capability in support of digital demodulation/decoder operating due to the limitation of the power supply.  For Device 1, only Manchester line code in TR 38.769 [2] will be specified in Rel-19 A-IoT WI with the bit-to-chip mapping of bit 0→chips{10}, bit 1→chips{01}.

For Manchester code, each coded bit is transmitted in a chip with fixed time determined by the M value of OOK-4 and OFDM symbol duration with 15k SCS, each information bit is carried in two OOK chips. The transitions at the midpoint of an information bit time interval signify the bit being 0 or 1, which can be used for information transmission and calibration of time synchronization. Furthermore, detection of the waveform transition is performed at Manchester code demodulation, which could provide better performance over amplitude threshold comparison. With Manchester line coding, the R2D information data rate can be described as 1/(2*R2D chip length) and R2D chip length  will be defined and discussed in [5].

Observation 1: With Manchester line coding, the R2D information data rate can be described as 1/(2*R2D chip length), and the R2D chip length is determined by the M value of OOK-4 and the OFDM symbol duration with 15k SCS .

D2R link channel coding

D2R convolutional code

For D2R FEC code, only convolutional code with generator polynomials as per TS 36.212 is included in Rel-19 WI scope [1] unless RAN1 decides to use other generator polynomials by RAN1#120bis.

For convolutional code, the reader can adopt effective decoding algorithm, such as soft decision based Viterbi decoding or list-decoding algorithm to improve coverage performance of NR A-IoT devices. There are two typical types of convolutional codes corresponding to different shift-register initialization/termination, i.e., tail-biting convolutional code (TBCC) and zero tail bits convolutional code (Z-TBCC). TBCC ensures that the starting state of the encoder is the same as its ending state. In comparison, Z-TBCC appends  M (e.g., M=6) zero bits to a block of data to ensure the feed-forward encoder starts from and ends in the all-zero state for each block, which results in additional overhead due to the tail bits. Considering that encoding complexity is comparable for these two types of convolutional codes and the extra overhead of Z-TBCC, CC with tail-biting shift-register termination/initialization should be specified for Rel-19 A-IoT D2R FEC code to avoid additional overheads due to the need of the tail bits transmission.

Proposal 1: Tail-biting convolutional code (TBCC) should be specified for Rel-19 D2R FEC code.

Based on the evaluation results of the SI phase [2], compared to convolutional code defined in TS 36.212, other CC generator polynomial(s) with reduction of states number or generator polynomials number will significantly degrade the BLER performance. For CC with a coding rate less than 1/3, had been studied intensely and only marginal coding gain is observed, while its complexity and memory of encoder are significantly higher than those of LTE-CC. In addition, it requires more power consumption. Thus, other convolutional code generator polynomial(s) different from LTE-CC (i.e., [133, 171,165]) should not be supported for A-IoT Device 1.

Proposal 2: Rel-19 A-IoT should only support CC generator polynomials as per TS36.212.

The rate matching of LTE-TBCC consists of interleaving the three parity-check bit streams, ,  and , followed by the collection of bits and bit selection operation as shown in Figure 1 [3].

Figure 1: Rate matching for LTE-TBCC

LTE-TBCC rate matching scheme has high implementation complexity and should not be fully reused for A-IoT Device 1 due to its 1 µW peak power consumption:

- LTE sub-block interleavers require large memory and complex addressing operation to battle the fast fading channel in the mobile communication. The sub-block interleavers are not needed for A-IoT, since it could not provide significant performance gain for A-IoT deployment scenarios with low mobility as the probability of occurrence of bust error is relatively low for indoor slow time-varying channels.

- For LTE, to match scheduling resource of gNB, bit selection is performed through puncture or repetition operation based on virtual circular buffer. Considering poor coverage performance of Device 1, it should not support the coding rate of other than 1/3.  The puncturing of CC coding bits is not needed for Rel-19 A-IoT CC coding chain.

- To reduce implementation complexity, repetition operation should be integral multiples of CC codeword. Details of the repetition scheme of CC will be described in section 2.2.4. For simplicity, bit collection should be performed based on the natural order of three encoder output parity streams, i.e., {Wk}= ,  ,.

Proposal 3: For CC in A-IoT Device 1

- Repetition operation should be integral multiples of CC codeword.

- Bit collection should be performed based on natural order of three encoder output parity streams.

- Note:

  - No need to support the sub-block interleaver(s) and puncturing operation of rate matching in LTE-TBCC.

The CC decoder is sensitive to non-ideal SFO estimation. With different residual SFO assumptions, BLER performance of PDRCH with 1/3 coding rate TBCC coding in TDL-A channel is shown in Figure 2. The performance degradation at BLER=10% and 1% compared with ideal SFO is summarized in Table 1. Severe BLER performance degradation is observed for D2R TBCC coding when the residual SFO exceeds 1000ppm. From the perspective of robustness of TBCC to SFO, D2R preamble design should ensure that residual SFO does not exceed 1000ppm.

Figure 2: BLER performance of TBCC with different SFO assumptions

Table 1:  Performance degradation with non-ideal SFO assumptions

| SFO(ppm) | SNR(dB)@BLER=10% Performance degradation(dB) | SNR(dB)@BLER=1% Performance degradation(dB) |
| --- | --- | --- |
| No SFO | 0 | 0 |
| 300 | 0.4 | 0.7 |
| 500 | 0.9 | 1.6 |
| 1000 | 2.0 | 3.5 |
| 2000 | 5.5 | 8.0 |

Proposal 4: From the perspective of robustness of TBCC to SFO, D2R preamble design should ensure that residual SFO does not exceed 1000ppm.

D2R line code

Whether Manchester line code in TR 38.769 could be used for D2R transmission is still FFS. With same coding rate, TBCC code has about 6dB coding gain than that of Manchester code at BLER=10% [4]. Thus, Manchester line code should not be supported without CC coding. By intruding line code as outer code, the diagram of TBCC and Manchester line code concatenation is illustrated in Figure 3.

Figure 3： CC concatenated with Manchester code

With different residual SFO assumptions, the BLER performance of TBCC+ Manchester code and TBCC+ repetition code with OOK and BPSK modulation for D2R transmission are shown in Figure 4. With ideal SFO assumption,                         convolutional code concatenated with Manchester code has no additional coding gain over TBCC+ repetition code for both OOK and BPSK modulation. It also can be seen that TBCC with double block-level repetition has better BLER performance that that of TBCC and Manchester code in case of SFO=1000ppm. The reason is that the error detection of D2R chip impacted by SFO could be corrected by repetition of D2R chip but error propagation will occur for Manchester code as outer code. With same SFO assumption and channel coding scheme, BLER performance of BPSK has 6dB gain over that of OOK modulation in AWGN channel. Compared with convolutional code only, convolutional code concatenated with Manchester code would introduce unnecessary redundancy, while its performance is poor than that of convolutional code with repetition operation in case of SFO=1000ppm. Based on the evaluation, Manchester line code should not be supported in D2R channel coding.

Figure 4: BLER performance of TBCC and Manchester line code

Observation 2: With same coding rate, TBCC code has about 6dB coding gain than that of Manchester code at BLER=10%.

Observation 3: Compared with CC code only, CC concatenated with Manchester code would introduce unnecessary redundancy, while its performance is poor than that of convolutional code with repetition operation in case of SFO=1000ppm.

Observation４: With same SFO assumption and channel coding scheme, BLER performance of BPSK has 6dB gain over that of OOK modulation in AWGN channel.

Proposal 5:  D2R Manchester line code should not be supported for PDRCH transmission.

Small frequency shift

To achieve small frequency shift in baseband for D2R FDM(A) among devices, the following was agreed in RAN1#119 [10].

Agreement For small frequency shifts in D2R using Manchester line codes by multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift, the time duration Tb corresponding to each information bit includes Rs number of square wave periods, where Rs = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is Rs/Tb = 1/(2 * chip length). Agreement For small frequency shifts in D2R using a square-wave corresponding to the small frequency-shift and no line codes, the time duration Tb corresponding to each information bit includes R number of square wave periods generated by 2R OOK chips [0, 1, 0, 1 …]/[1, 0, 1, 0 ..] or BPSK chips [-1, +1, -1, +1, …]/[+1, -1, +1, -1, …], such that the amount of small frequency shift in Hz is R/Tb.

Small frequency shift (SFS) can be achieved by square wave modulation (SWM) with the following two options.

- Option1: square wave modulation and no line code.

- Option2: multiplying the Manchester codeword with a square wave.

For small frequency shift in D2R using either option, the time duration Tb corresponding to an information bit should be associated with an integer number of square wave periods (e.g., K=2). As illustrated in Figure 5, in Option1, square wave modulation with multiplication/XNOR operation and XOR operation is equivalent to bit reversal of Manchester codeword repetition and the direct repetition of Manchester codeword, respectively. As shown in Figure 6, Manchester codeword and square wave multiplication operation (i.e., Option 2) is equivalent to Manchester code and square wave XNOR operation. Thus, square wave modulation with multiplication/XNOR operation can also be used for small frequency shift of D2R with Manchester line code case.

Figure 5: Square wave modulation and no line code (K=2)

Figure 6: Square wave modulated Manchester code (K=2)

Proposal 6: For small frequency shift in D2R, the time duration corresponding to each information bit should be associated with an integer number of square waves periods.

Proposal 7: Support D2R small frequency shit by XNOR/multiplication operation in the square wave modulation corresponding to the small frequency-shift.

To generate D2R signal/channel with a small frequency shift, the device needs to know the square wave period. The time duration Tb corresponding to each D2R chip could be indicated by the R2D signal/channel, which is discussed in our contributions [5], [6]. Square wave period is equal to Tb/ K, thus SFS factor K could be used for A-IoT Device 1 to determine square wave period.

Observation 5: SFS factor K could be used for A-IoT Device 1 to determine square wave period.

From spectrum perspective, the bandwidth of D2R signal/channel with small frequency shift using SWM and Manchester line code (i.e., option 2) is double of that of D2R signal/channel with small frequency shift using SWM and no line code(i.e., option 1), as shown in Figure 7. With different SFS factor values, the spectral performance of multi-device FDM for option 1 and option 2 is shown in Figure 8. We can see that with same SFS factor value, compared with the small frequency shift using SWM and no line code, the small frequency shift using SWM and Manchester line code has worse FDM multiplexing performance, as the transmission bandwidth of option 2 is significantly larger than that of option 1. Thus small frequency shifts in D2R using a square-wave modulation and no line codes should be specified for Rel-19 A-IoT.

Figure 7: Spectrum comparison of Option 1 and Option 2 with same frequency shift factor

(Left: square wave modulation and no line code)              (Right: square wave modulated Manchester codeword)

Figure 8: FDM spectrum comparison for option 1 and option 2

Observation 6: Compared with the small frequency shift without line code (Option 1), the small frequency shift with Manchester line code (Option 2) has poor FDM multiplexing performance, as Option 2 requires twice transmission bandwidth of option 1 per device.

Proposal 8: Small frequency shifts in D2R using a square-wave modulation and no line codes should be specified for Rel-19 A-IoT.

The preliminary BLER performance for coexistence of multiple D2R channels is evaluated with the evaluation assumptions provided in Table 3 of the Annex. For D2R FDM, frequency overlapping of D2R channels can be caused by small frequency shift using square wave modulation and no line code, which would degrade BLER performance of D2R transmission significantly if reader does not support effective intra-channel interference cancellation. As shown in Figure 9, due to double side frequency overlapping for SFS factors K= {3, 4, 5}, their corresponding D2R channels cannot be transmitted correctly. In contrast, for K= {2, 6}, where only single-side frequency overlapping occurs, the BLER performance is better than that of D2R channel with K={3, 4, 5}.

Figure 9: BLER performance of D2R transmission with frequency overlapping

To avoid frequency overlapping among D2R channels when Option 1 SFS is used, the difference of SFS factors between neighbour D2R channels should be no less than 2. With different SFS factors, the FDM coexistence performance of multiple D2R channels is illustrated in Figure 10 and Figure 11. As shown in Figure 10, the D2R transmissions with SFS factor 2n have similar BLER performance without FDM performance degradation; however, the number of available D2R channels is very limited. The FDM coexistence performance of multiple D2R channels with SFS factor {2, 3*2, 5*2, 7*2} is shown in Figure 11. It can be seen that compared with D2R channel with SFS factor K, BLER performance of the D2R channels with SFS factor 3*K, 5*K and 7*K would be degraded by about 3dB, 1.5 dB and 0.8dB, respectively. Therefore, if intra-channel interference cancellation is not supported by the Reader, the number of D2R channels available for effective D2R multiplexing is seriously limited.

Figure 10: BLER performance of D2R with SFS factor 2n

Figure 11:   BLER performance of D2R with SFS factor {2, 3*2, 5*2, 7*2}

Observation 7: Intra-channel interference due to frequency overlapping among D2R channels caused by small frequency shift using square wave modulation, could degrade BLER performance of D2R transmission significantly.

Observation 8: D2R transmissions with SFS factor K=2n have similar BLER performance without FDM performance degradation; however, the number of available D2R channels is reduced tremendously.  This will reduce the spectrum utilization.

Observation 9: Compared to D2R channel(s) with SFS factor K, the BLER performance of D2R channels with SFS factor 3*K, 5*K and 7*K would be degraded by about 3dB, 1.5 dB and 0.8dB, respectively without interference cancellation at the reader.   However, reader can perform interference mitigation with simple multi-tap frequency domain filtering to ensure the BLER performance.

D2R Repetition code

For repetition code, the following was agreed in RAN1#117 [7].

Agreement Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times

For bit level type 1 repetition of D2R, no repetition gain could be achieved as repetition bits are located within the same CC codeword and LLR combination cannot be performed.

For D2R transmission, bit level type 2 repetition and block level repetition have similar performance in the AWGN channel. However, block level repetition could achieve better time diversity gain than that of bit level type 2 repetition in fading channel. For block level repetition, FEC coding bits could be pre-computed and stored in the A-IoT device memory to perform repetition operation without intense processing time and power consumption. For bit level repetition, FEC coding bits and repetition bits are required to be generated in real-time, which would require more power consumption than block level repetition in A-IoT devices. With small D2R information block size, the memory requirements of A-IoT devices are comparable for bit level repetition and block level repetition. However, compared to block level repetition, bit level repetition would degrade both power efficiency and BLER performance in D2R transmission. With CC coding, block level repetition should be used for A-IoT D2R transmission.

Observation 10: For bit level repetition type 1, no repetition gain could be achieved as repetition bits are located within the same CC codeword and LLR combination cannot be performed.

Observation 11: Compared to block level repetition, bit level repetition type 2 would degrade both power efficiency and BLER performance in D2R transmission.

Proposal 9: With CC coding, block level repetition should be used for D2R transmission of A-IoT.

CRC code for D2R and R2D

TS38.212 [11] has provided 24, 16, 11 and 6 bits CRC generator polynomials for different information bits size A, as shown in Table 2.

Table 2: CRC coding for NR data and control channel

| Data channel |  | Control channel |  |  |
| --- | --- | --- | --- | --- |
| A>3824 | 24 bits CRC, FAR=2-24 | DCI |  | 24 bits CRC, FAR=2-21 |
| A<=3824 | 16bis CRC, FAR=2-16 | UCI |  | 11 bits CRC, FAR=2-8 |
|  |  |  |  | 6 bits CRC, FAR=2-3 |
|  |  |  | A<12 | No CRC |

According to the objective of A-IoT work [1], both PRDCH and PDRCH support data transmission with or without CRC. When CRC is used, it uses the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials) for 6-bit CRC and 16-bit CRC. The decision on whether to use CRC and the length of CRC will be decided in RAN1. For NR A-IoT design, the CRC bit length and the generator polynomial should be determined by false alarm rate (FAR) requirement and TB size. For RFID interrogator to tag，CRC-5 and CRC-16 are used for 16-bit commands and commands of more 16 bits, respectively. CRC-16 is used fo

r RFID Tag to interrogator [12].

The FAR performance target and supported TB size of PRDCH and PDRCH needs to be set in order to determine accurate CRC polynomials. The following principle could serve as the baseline of CRC length determination:

- CRC-16 could be used for information bits length>N, otherwise CRC-6 is applied, where N=16 or 24.

Regarding the use of CRC with different polynomials not defined in TS 38.212, as the FAR performance of NR CRC polynomials has already been carefully evaluated and ensured in NR channel coding design, there is no need to introduce other 6 bits and 16 bits CRC with different polynomials beyond those specified in TS 38.212.

Proposal 10: The following rule of CRC length determination should be adopted:

- CRC-16 should be used for information bits length>N, otherwise CRC-6 is applied, where N=16 or 24.

Proposal 11: There is no need to introduce other 6 bits and 16 bits CRC with different polynomials beyond those defined in TS 38.212.

The following agreement was made in RAN1#118bis related to CRC design for PRDCH [9]:

Agreement For the PRDCH design details, following is captured in the TR 38.769: Following design options have been studied for PRDCH: - Option 1: If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered - Note: In this option, R2D control information (if any) maybe carried by higher-layer signaling - Option 2: If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied - Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data - Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively

Regarding the CRC design for PRDCH, separate CRC (i.e., Option 2-1 in above agreement) should not be adopted in order to avoid the device processing delay and CRC overhead. Single CRC (i.e., joint CRC in Option 2-1 in above agreement) for R2D control information and R2D data should be sufficient for both the R2D control information and R2D data. The approach minimizes the large processing and power consumption at A-IoT device. Device could decode Manchester coded R2D control information to achieve TBS information before performing CRC parity-check operation,  which is similar to NR where UE achieves 6 timing bits (3bits RV version and 3bits  SSB index) of PBCH  without CRC parity-check.

Proposal 12:  Joint CRC should be applied to both R2D control information and R2D data.

Annex

Table 3: The evaluation assumptions for the BLER performance of D2R

| Parameter | Value |
| --- | --- |
| Carrier frequency | 900MHz |
| SCS | 15kHz |
| Channel model | AWGN |
| Message size | 96bits |
| Modulation | BPSK |
| Channel coding | No channel coding |
| Sampling rate | 23.52MHz |
| SFS factor | 2, 3, 4, 5, 6, 8, 10, 14 |
| SFO/STO | 0 ppm |

Conclusion

In this contribution, channel coding and small frequency shift technique for NR Ambient IoT communication system are discussed. We have the following observations and proposals:

Observation 1: With Manchester line coding, the R2D information data rate can be described as 1/(2*R2D chip length), and the R2D chip length is determined by the M value of OOK-4 and the OFDM symbol duration with 15k SCS .

Observation 2: With same coding rate, TBCC code has about 6dB coding gain than that of Manchester code at BLER=10%.

Observation 3: Compared with CC code only, CC concatenated with Manchester code would introduce unnecessary redundancy, while its performance is poor than that of convolutional code with repetition operation in case of SFO=1000ppm.

Observation４: With same SFO assumption and channel coding scheme, BLER performance of BPSK has 6dB gain over that of OOK modulation in AWGN channel.

Observation 5: SFS factor K could be used for A-IoT Device 1 to determine square wave period.

Observation 6: Compared with the small frequency shift without line code (Option 1), the small frequency shift with Manchester line code (Option 2) has poor FDM multiplexing performance, as Option 2 requires twice transmission bandwidth of option 1 per device.

Observation 7: Intra-channel interference due to frequency overlapping among D2R channels caused by small frequency shift using square wave modulation, could degrade BLER performance of D2R transmission significantly.

Observation 8: D2R transmissions with SFS factor K=2n have similar BLER performance without FDM performance degradation; however, the number of available D2R channels is reduced tremendously.  This will reduce the spectrum utilization.

Observation 9: Compared to D2R channel(s) with SFS factor K, the BLER performance of D2R channels with SFS factor 3*K, 5*K and 7*K would be degraded by about 3dB, 1.5 dB and 0.8dB, respectively without interference cancellation at the reader.   However, reader can perform interference mitigation with simple multi-tap frequency domain filtering to ensure the BLER performance.

Observation 10: For bit level repetition type 1, no repetition gain could be achieved as repetition bits are located within the same CC codeword and LLR combination cannot be performed.

Observation 11: Compared to block level repetition, bit level repetition type 2 would degrade both power efficiency and BLER performance in D2R transmission.

Proposal 1: Tail-biting convolutional code (TBCC) should be specified for Rel-19 D2R FEC code.

Proposal 2: Rel-19 A-IoT should only support CC generator polynomials as per TS36.212.

Proposal 3: For CC in A-IoT Device 1

- Repetition operation should be integral multiples of CC codeword.

- Bit collection should be performed based on natural order of three encoder output parity streams.

- Note:

  - No need to support the sub-block interleaver(s) and puncturing operation of rate matching in LTE-TBCC.

Proposal 4: From the perspective of robustness of TBCC to SFO, D2R preamble design should ensure that residual SFO does not exceed 1000ppm.

Proposal 5:  D2R Manchester line code should not be supported for PDRCH transmission.

Proposal 6: For small frequency shift in D2R, the time duration corresponding to each information bit should be associated with an integer number of square waves periods.

Proposal 7: Support D2R small frequency shit by XNOR/multiplication operation in the square wave modulation corresponding to the small frequency-shift.

Proposal 8: Small frequency shifts in D2R using a square-wave modulation and no line codes should be specified for Rel-19 A-IoT.

Proposal 9: With CC coding, block level repetition should be used for D2R transmission of A-IoT.

Proposal 10: The following rule of CRC length determination should be adopted:

- CRC-16 should be used for information bits length>N, otherwise CRC-6 is applied, where N=16 or 24.

Proposal 11: There is no need to introduce other 6 bits and 16 bits CRC with different polynomials beyond those defined in TS 38.212.

References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#106, December 9th-12th, 2024.

- TR 38.769, Study on solutions for ambient IoT (Internet of Things), V2.0.1.

- TS36.212, Multiplexing and channel coding, V13.4.0.

- R1-2409942, Discussion on general aspects of physical layer design, CATT, RAN1#119, November 18th – 22nd, 2024.

- R1-2500224, Ambient IoT physical channel design and modulation, CATT, RAN1#120, February 17th – 21st, 2025.

- R1-2500226, Ambient IoT timing and synchronization, CATT, RAN1#120, February 17th – 21st, 2025.

- 3GPP TSG RAN WG1 Meeting RAN1#117 Chairman’s Notes, May 20th – 24th, 2024.

- 3GPP TSG RAN WG1 Meeting RAN1#118 Chairman’s Notes, August 19th – 23rd, 2024.

- 3GPP TSG RAN WG1 Meeting RAN1#118bis Chairman’s Notes, October 14th – 18th, 2024.

- 3GPP TSG RAN WG1 Meeting RAN1#119 Chairman’s Notes, November 18th – 22nd, 2024.

- TS38.212, Multiplexing and channel coding, V15.5.0.

- ISO/IEC18000-61, Parameters for air interface communications at 860 MHz to 960 MHz Type A.