3GPP TSG RAN WG1 #120		                               R1-2500270

Athens, Greece, February 17th – 21st, 2025

Agenda item:		9.11.3

Source:	China Telecom

Title:	Discussion on NR-NTN uplink capacity/throughput enhancement

Document for:		Discussion

Introduction

In the RAN1 #119 meeting [1], the support of OCC techniques and signaling aspects of PUSCH with OCC were discussed. The following agreements were achieved:

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO. Observation: Option 1 Inter-slot OCC with OCC length 4 to multiplex up to 4 UEs with 2 PRBs can meet VoIP 2% BLER within 1 dB of single UE baseline for 8 slots or larger with UE grouping with similar CFO (e.g. maximum differential CFO of 200 Hz). Observation: Option 2 Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs with TBoMS can meet VoIP 2% BLER target within 1 dB of single UE baseline with 2 PRBs and 4 repetitions or larger; and with 1 PRB and 8 repetitions or larger, without need for CFO grouping. Agreement For RV cycling for OCC with DG-PUSCH, the following are considered: - Option 1: RV cycling is used across OCC groups - Note 1: RV cycling is applied when the number of repetitions is greater than the OCC length - Option 2: Fixed RV is used across OCC groups - Option 3: For OCC length 2, fixed RV is used across two OCC groups, RV cycling is used across groups of two OCC groups Agreement If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options to be considered: - Option 1: UCI is dropped - FFS: whether all UCI is dropped - Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped. - Option 3: UCI is multiplexed on PUSCH with inter-slot OCC - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC - FFS: which OCC group - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions Note: combination of the above can be considered FFS Details timeline of PUCCH and PUSCH FFS: handling of PUCCH with repetition FFS: handling of different UCI types

In this contribution, we provide our views on signalling aspects of PUSCH with OCC.

Discussion

Potential Signaling Aspects

Inter-slot OCC has a specification impact on UCI multiplexing, RV cycling across repetitions, OCC configuration, and frequency hopping. In this session, we discuss the potential signaling aspects and solutions for these.

- 2.1.1 RV cycling

As defined in clauses 6.1.2.1 and 6.1.2.3 of TS 38.214, a UE performing PUSCH repetitions will transmit a different RV with each repetition. This will complicate matters for a receiver trying to eliminate the interference caused by such a UE with inter-slot OCC transmitting on the same time-frequency resources. For this reason, for inter-slot OCC, RAN1 support that the same RV value is used in one OCC group.

At RAN1#119, RAN1 reached the following agreement

Agreement

For RV cycling for OCC with DG-PUSCH, the following are considered:

- Option 1: RV cycling is used across OCC groups

  - Note 1: RV cycling is applied when the number of repetitions is greater than the OCC length

- Option 2: Fixed RV is used across OCC groups

- Option 3: For OCC length 2, fixed RV is used across two OCC groups, RV cycling is used across groups of two OCC groups

This raises an open question: Can RV cycling be utilized across OCC groups? The number of PUSCH repetitions in one PUSCH OCC group equals OCC length. As shown in the following Figure, for 8 slots for VoIP with OCC length 4, the legacy RV cycling mechanisms can be extended to the unit of OCC group, using different RV values for different OCC groups can still meet the orthogonality requirements while obtaining coding gain from different redundancy versions. In light of this, we support using RV cycling across OCC groups.

Proposal 1: Support Option 1, i.e., RV cycling is used across OCC groups.

- 2.1.2 UCI multiplexing

UCI may consist of HARQ-ACK, CSI, and SR, and is encoded and transmitted through the PUCCH or is multiplexed on the PUSCH. As shown in Clause 6.2.7 of TS 38.212, in the case where there are more than one UL-SCH transport blocks for the PUSCH transmission, the UCI information is multiplexed only on the UL-SCH transport block with the highest IMCS (Clause 6.1.4.1 in TS 38.214) value for the initial PUSCH. In case the two transport blocks have the same IMCS value for the initial PUSCH, the UCI information is multiplexed with data only on the first transport block, which may break the orthogonality of the inter-slot time domain OCC. RAN1 may need to discuss and modify the potential enhancements for the UCI multiplexing when using the inter-slot time domain OCC.

Considering the timely requirements of UCI, to solve the aforementioned overlap issue, a straightforward solution is to drop all PUSCH repetition during one OCC period, and only PUCCH is transmitted (Option 2). Another way is that UCI can be multiplexed on PUSCH, and expanding UCI multiplexing across all the PUSCH repetitions within an OCC period (Option 3).

For Option 2, the disadvantage is that it cannot support PUSCH repetitions, which may reduce system capacity. Additionally, the gNB needs to notify UEs who are using other OCC sequences to cancel their PUSCH repetitions. For Option 3, as shown in the following figure, one more question is how to deal with the UCI multiplexing when it overlaps with the PUSCH resource in the non-first slot of PUSCH repetitions. This requires that the UE has enough time to determine whether there are any UCI overlap issues within the PUSCH transmission period after receiving the OCC configuration. The advantage is that the multiplexing of UCI does not require the gNB to notify other UEs.

To sum up, from our perspective, to solve the above overlap issue, at least the following two solutions can be considered:

- Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped.

- Option 3: UCI is multiplexed on PUSCH with inter-slot OCC

  - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC

Proposal 2: Potential enhancements for the UCI multiplexing when using the inter-slot time domain OCC are needed. At least the following two solutions can be considered, and we prefer Option 3-a.

- Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped.

- Option 3: UCI is multiplexed on PUSCH with inter-slot OCC

  - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC

- 2.1.3 OCC indication/configuration

How to indicate the OCC sequence to UE is essential for RNA1 and also needs to be discussed. As defined in clause 6.1 of TS 38.214, the dynamic grant based PUSCH transmission is scheduled by a UL grant in DCI, and the configured grant based PUSCH transmission is indicated by RRC.

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

Therefore, in our view, for dynamic grant based PUSCH transmission, OCC-related parameters can also be indicated in DCI. Similarly, for configured grant Type 1 or Type 2 PUSCH, OCC configuration can use configuredGrantConfig. Moreover, the new indication parameters should include OCC length and OCC sequence index.

Proposal 3: OCC parameters can be configured by DCI or RRC within configuredGrantConfig, which includes at least OCC length and OCC sequence index.

- 2.1.4 Frequency hopping

To ensure the orthogonality of inter-slot OCC, the time interval for frequency hopping should be extended to OCC length or OCC group length for inter-slot time-domain OCC with PUSCH repetition type A. Further consideration is needed on whether this approach can be applied across OCC groups.

Proposal 4: At least support inter-N slot frequency hopping, where N is the OCC length. It should be further considered whether to support using frequency hopping across the OCC group.

Conclusions

In this contribution, we provide our views on potential signaling aspects and have the following proposals:

Proposal 1: Support Option 1: RV cycling is used across OCC groups.

Proposal 2: Potential enhancements for the UCI multiplexing when using the inter-slot time domain OCC are needed. At least the following two solutions can be considered, and we prefer Option 3-a.

- Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped.

- Option 3: UCI is multiplexed on PUSCH with inter-slot OCC

  - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC

Proposal 3: OCC parameters can be configured by DCI or RRC within configuredGrantConfig, which includes at least OCC length and OCC sequence index.

Proposal 4: At least support inter-N slot frequency hopping, where N is the OCC length. It should be further considered whether to support using frequency hopping across the OCC group.

References

- 3GPP RAN1 Chairman’s Notes, RAN1 meeting #119, Orlando, US, November 18th - 22th, 2024.