3GPP TSG RAN WG1 #120                                                            R1- 2500337

Athens, Greece, February 17th – 21st, 2025

Source:	vivo

Title:	Specification support for beam management

Agenda Item:	9.1.1

Document for:	Discussion and Decision

Introduction

In RAN#102 meeting, WID [1] for NR AI/ML air interface was approved which includes enhancement on beam management, which is copied below for reference.

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signaling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we focus on the normative work for the use case of beam management.

- Consistency issue for UE-side model

  - Where to configure associated ID and how to define similar properties

In RAN#118 meeting, the following agreement was achieved,

Agreement

For UE sided model in beam management, support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

- FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Consider the above discussion, associated ID can be configured at least in the CSI framework to achieve consistency between training and inference. Furthermore, the associated ID must undergo interaction prior to the inference configuration if one set or multiple sets of inference related parameters for applicability report only (not for inference) are configured in Step 3.

Therefore, the Associated ID not only needs to be configured within the CSI framework, but also requires configuration in inference parameter set to address the consistency issue.

In addition, given that multiple Set B patterns may correspond to a single Set A, using just one associated ID to represent a pair of Set B and Set A is overly rigid for network configuration. It is recommended to introduce a pattern ID to specifically indicate the Set B pattern within Set B configuration. We, thus, propose,

For inference, for UE-side model, the associated ID can be configured in CSI-reportConfig.

For inference, for UE-side model, the associated ID can be configured in inference parameter set prior to CSI-reportConfig to address the consistency issue.

For inference, for UE-side model, associated ID should be mandatory configured within both inference parameter set and CSI framework.

For inference, for UE-side model, pattern ID can be configured for Set B pattern to address consistency of Set B.

Further, for the “FFS on whether/how to define similar properties of a DL Tx beam or beam set/list ” in the agreement, our understanding is that the same associated ID would imply the same network-side additional conditions including beam codebook corresponding to Tx beam boresight direction (azimuth and elevation), 3dB beam-width and mapping order of actual beams to beam indices which may corresponding to same transmission filter, for same resource IDs or same ordering in a list in Set A/Set B. We, thus, propose,

For inference, for UE-side model, similar properties of a list can be assumed that same resource IDs or through other rules (e.g. same ordering in a list) in Set A/Set B is consistent from training to inference if UE receives the same associated ID.

  - How to distinguish CSI report in option A or option B

In RAN#119 meeting, the following agreement and conclusion was achieved,

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

Given that both option A and option B have been approved and can coexist within the RRC configuration of a UE, a potential issue arises. Specifically, when the network configures a CSI-reportConfig, UE may be confused whether to send an applicability information or directly reply an RRCReconfigurationComplete. To address this ambiguity, it is recommended to add an indicator within the CSI-reportConfig to clearly specify whether the following processing follows option A or option B.

For inference, for UE-side model, an indicator should be configured within the CSI-ReportConfig to clearly specify whether following processing follows option A or option B.

  - Issues on Step 3 and Step 4

In RAN#119 meeting, the following agreement and conclusion was achieved,

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

In Step 3, the previous agreement did not include the detailed information of the inference parameter set, which poses significant challenges for RAN2 in designing the container. It is imperative that RAN1 provides comprehensive parameters in the inference parameter set.

Figure 2-1:an example of inference parameter set

In above figure, it is essential for the set to include both the associated ID and the pattern ID to address the mapping consistency issue from resources in Set B/Set A mapping to beam codebook. Additionally, the set should also encapsulate the sizes of Set B and Set A to further aid in addressing consistency issues. Considering the implementation of various AI models on the terminal, it is crucial for the UE to indicate the applicable report content in applicable report, i.e. {CRI/SSBRI reporting, or predicted RSRP-CRI/SSBRI}.

Further, for BM Case 1 and BM Case 2, even if the same associated ID is used, the UE may opt to implement only one case for a particular functionality. Consequently, the case indicator information needs to be configured within the inference parameter set. In instances where the network specifies BM-Case2 in the inference parameter set, it is also imperative to include information regarding the periodicity of Set B/Set A, the number of measurement occasions, and the number of predictable occasions. We, thus, propose,

For inference, for UE-side model, the set of inference related parameter in Step 3 at least includes,

- Associated ID

- Pattern ID

- Size of Set B

- Size of Set A

- Indicated report content, i.e. {CRI/SSBRI, or predicted RSRP-CRI/SSBRI}

- BM case indicator, i.e. {case 1, or case 2}

- If 	Case 2,

          - Periodicity of Set B

          - Periodicity of Set A

          - Number of measurement occasions

          - Number of predicted occasions

As UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters in step 4, unified applicable report should be considered for both option A and option B. A bitmap indicator is preferred to indicate which CSI report configuration or inference parameter Set is applicable or not. Furthermore, if all CSI-ReportConfig or inference parameter sets are not supported, UE can report its supported inference parameter Set(s) to aid beam-codebook selection for NW. Thus, we propose,

For inference, for UE-side model, unified applicable report should be considered for both option A and option B in Step 4, and a bitmap indicator is preferred to indicate which CSI report or inference parameter Set is applicable or not.

For inference, for UE-side model, UE can report its supported inference parameter set(s) along with the applicable report in Step 4 if all configurations for both option A and option B in Step 3 are not applicable.

  - Remaining issues on associated ID

It is necessary to discuss how associated IDs are determined or allocated as global ID or local ID. The details of global associated ID and local associated ID are provided in the following,

- Global associated ID would refer to NW side additional conditions of different NW-side implementation/configurations. Different global associated ID would refer to different Tx beam boresight direction (azimuth and elevation), 3dB beam-width and mapping of actual beams to beam indices, etc.

- Local associated ID is cell/site/region specific. NW may use different beam codebook in the daytime and night, since the UE distribution and UE requirement may be different in the daytime and night. Then NW could use different local associated ID to indicate different beam codebook.

The local associated ID requires large efforts at UE side to categorize the collected data to maintain consistency between training and inference. Depending on how local the associated ID is applied, the trained model may only be applicable to a relatively small region thus may potentially require cell/site/region specific model development. Conversely, global associated ID is useful for maintaining consistency between training and inference and help UE to train a model with good generalization performance globally and thus would be friendlier to UE side management.

However, global associated ID may potentially expose deployment choices of NW side, which results in the working assumption agreed in AI 9.1.3.3 for local associated ID to address consistency issues.

Along the line of the spirit of the working assumption in AI 9.1.3.3, local associated ID can be prioritized. On the other hand, ways to resolve UE side complexity on managing local associated IDs should be further studied. We, thus, propose,

For inference, for UE-side model, along the line of the agreed working assumption on the consistency issue in AI 9.1.3.3, support to prioritize discussion on how UE-side AI model can work with local associated ID and study how to reduce UE side complexity on managing local associated IDs.

For the case when UE moves to a cell that UE has not collected data before, we can consider the following procedures to make UE side mode more efficient.

Figure 2-2: associated ID confirmation for UE-side AI model

For data collection and model training:

- Step 1: Beam related data is collected with associated ID and corresponding cell ID(s), e.g. associated ID + {cell 1, cell 2, cell 3, …, cell N}, or associated ID + cell 1

- Step 2: AI model is trained by collected data, and mapping to the associated ID + cell ID(s)

For inference phase:

- Step 1: UE reports supported associated IDs with corresponding cell IDs to the accessed cell, e.g. associated ID + cell 1, another associated ID + another cell,

- Step 2: Supported associated ID information is processed at network.

- Step 3: The selected associated ID among supported associated IDs can be configured with RS/report configuration to UE, if the ID is also supported by the accessed cell.

The aforementioned process, i.e. introduce a cell indicator to indicate whether to interpret the associated ID as for a target cell in data collection procedure, ensures the UE-side AI model to be working with local associated ID. Further, in RAN2 128 meeting, following agreement was approved for handover,

Agreement

Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Figure 2-3: Applicable functionality reporting at handover procedure

Based on the agreement drawn by RAN2, the handover process for AI prediction can be developed as illustrated in above figure. UE initiates handover procedure after reception of the handover command from the target cell transmitted through the source cell, where handover command contains associated ID along with inference parameter set(s) or CSI-ReportConfig(s). After successful completion of the handover, UE is required to indicate the applicability of the inference parameter set(s) or CSI-ReportConfig(s) in applicable report.

It is crucial to define how to interpret associated ID in handover command as the handover command distinctly related to both the target cell and the source cell. Failing to do so would hinder the UE's ability to determine whether the associated ID in the handover command pertains to the source cell or the target cell.

when the applicable area of the associated ID is cell-specific,

- If the NW side conditions of the source cell and the target cell are different, or the target cell is a know cell, i.e., UE has been trained under the target, the handover command should provide the associated ID of the target cell to ensure UE and gNB have consistent understanding of the applicability of the target cell.

- If the network-side addition condition of the source cell and the target cell are identical, UE may be unable to interpret the associated ID defined as the target cell if it is unknow cell, i.e., the UE hasn’t been trained under the target cell but the NW-side associated conditions are identical for source cell and target cell. However, from target cell’s perspective, it can recognize that its network-side addition condition is identical to source cell. In such cases, interpreting the associated ID based on the source cell, as included in the handover command, can considerably enhance the applicability for AI prediction in target cell.

Based on the above, there are use cases for interpreting the associated ID either as for target cell or source cell. Hence, we suggest to include a new indicator to indicate whether to interpret the associated ID as for target cell or source cell, e.g., such indicator can be a cell ID to interpret the associated ID.

For inference, for UE-side model, introduce a cell indicator along with the configuration of associated ID to indicate how to interpret the associated ID for further address consistency issue.

- Performance monitoring

In RAN1#117 meeting, the following agreement was achieved for monitoring procedure,

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model:

- Support Type 1 performance monitoring, including the following two options:

  - Option 1 (NW-side performance monitoring):

    - UE sends a report to NW (for the calculation of performance metric at NW)

      - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report

      - FFS on other contents

    - The report is at least configured/triggered by NW

    - Note: this may or may not have additional spec impact

  - Option 2 (UE-assisted performance monitoring):

    - UE calculates performance metric(s)

      - FFS how to report and what to report

  - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2

- FFS Type 2 performance monitoring

The UE is required to report measurement results for a specific resource set (e.g. Set A) to enable the network (NW) to calculate performance metrics. However, it is inefficient in terms of resource utilization to report all measurement results of Set A, through either L1 signaling or higher-layer signaling.

For performance monitoring, at least support L1 beam reporting for both UE-side model and NW-side model.

  - Association between monitoring Set and inference Set

In RAN1#119meeting, the following agreement was achieved for monitoring,

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

For BM-Case1

Two sets of data involved in the calculation of KPI: one is the performance monitoring results for measurement, and the other is the inference results derived from the AI model. It has already been agreed to configure the inference report ID within the monitoring report to establish a connection with the corresponding report. However, a further decision is required on the method to associate the performance monitoring results with the inference results. Consequently, the following issues need to be addressed.

Issue 1: how to associate the reference points for monitoring Set and inference Set

We have following assumptions:

- 1st reference point (inference reference point): The reference resource (following the legacy definition) of the inference report corresponding to the monitoring report, as indicated by the orange arrow.

- 2nd reference point (monitoring reference point): The first transmission resource of the monitoring RS set corresponding to the monitoring report, as shown by the yellow arrow.

Thus, there are two methods to associate the monitoring set with the inference set:

- Option 1: Associate 2nd reference point to the 1st reference point, i.e., UE identifies 1st reference point first, and then identify the associated 2nd reference point to calculate KPI

- Option 2: Associate 1st reference point to the 2nd reference point, i.e., UE identifies 2nd reference point first, and then identify the associated 1st reference point to calculate KPI

Figure 3-1: Associate the reference points for monitoring Set and inference Set based on inference reference point

If Option 1 is used, the association steps are as follows:

- Step 1: Identify the 1st reference point based on the corresponding inference report associated with the monitoring report, e.g. the CSI reference resource (orange arrow) of the N-th inference report in above figure

- Step 2: Find the closest monitoring set from the 1st reference point, as illustrated in the figure with the M-th occasion of the monitoring set (yellow arrow)

- Step 3 (optional): If the M-th occasion of the monitoring set is too close to the monitoring report, it does not meet the UE's processing capability of generating monitoring report

  - Disadvantage-1: monitoring Set and inference Set association failure

- Step 4: Due to the Step-3 failure, re-association is required. Find the closest monitoring set with satisfying UE's processing capability based on the 1st reference point, e.g., the (M-1)-th occasion of the monitoring set

  - Disadvantage-2: Associating with an early monitoring result that is farther from the inference result, potentially leading to degraded prediction performance not attributable to AI prediction performance

  - Disadvantage-3: Associating with an early monitoring result that was already linked to a close inference result in early metric calculation, which means the monitoring result is associated twice.

- Step 5: Calculate the metric based on the N-th inference result and the (M-1)-th monitoring result

Figure 3-2: Associate the reference points for monitoring Set and inference Set based on monitoring reference point

If Option 2 is used, the association steps are as follows:

- Step 1: Identify the 2nd reference point with satisfying UE's processing capability based the monitoring report, e.g. first transmitted resource (yellow arrow) of the (M-1)-th monitoring Set in above figure

- Step 2: Find the closest CSI reference resource of inference report from the 1st reference point, as illustrated in the figure with the (N-1)-th occasion of the inference set (orange arrow)

- Step 3: Calculate the metric based on the (N-1)-th inference result and the (M-1)-th monitoring result

Thus, we have following observation and proposal,

For option 1, i.e. based on CSI reference resource of inference report corresponding to a monitoring report to associate closest reference point of monitoring set, it takes following disadvantages:

- Monitoring Set and inference Set association failure

- Associating with an early monitoring result that is farther from the inference result, potentially leading to degraded prediction performance not attributable to AI prediction performance

- Associating with an early monitoring result that was already linked to a close inference result in early metric calculation, which means the monitoring result is associated twice

For performance monitoring, for UE-side mode, for BM-case1, support to associate inference reference point to the monitoring reference point, i.e., UE identifies monitoring reference point first, and then identify the associated inference reference point to calculate KPI, where the monitoring reference point is related to monitoring set and the inference reference point is related to inference report.

Issue 2: how to define “closest” for monitoring Set and inference Set association?

Figure 3-3: One monitoring reference point associates to both “before” and “after” inference reference points

In the conventional CSI framework, the process generally involves identifying the closest point is before a reference point. If the term "closest" in AI prediction is redefined to include both "before" and "after" a reference point, it could have a substantial impact on the specification design. Furthermore, when the “closest” reference points are equidistant in both the "before" and "after" directions, this can create ambiguities in the behavior of the UE, leading to potential inconsistencies or unpredictability in its actions.

Figure 3-4: One inference reference point associates to both “before” and “after” monitoring reference points

Further, if we only specify “closest” occasion without specifying “before”, or “after”, there can be cases that an inference occasion is linked to two monitoring occasions. That is, an inference occasion appears with same time gap in between two monitoring occasions. In this case, one of the monitoring occasions is useless, which is a wasteful RS transmission.

Hence, we propose the following.

For performance monitoring, for UE-side mode, for BM-case1, "closest" should be defined as the inference reference point (pertaining to inference report) that is nearest and occurs before the monitoring reference point (pertaining to monitoring set).

Issue 3: what’s granularity for monitoring report linked to inference report?

Figure 3-5: Granularity for determining monitoring reference point for BM-Case1

Since the inference reference point corresponds to the CSI reference resource of the inference report, it can be assumed that all results of the inference set, such as all beams or top-k beams, are located at the same time instances. Consequently, there is only one inference reference point for one set of inference results. However, monitoring results require actual measurements of multiple beams, which means each RS resource in the monitoring set occupies a unique position in the time domain. In other words, if we determine the monitoring reference point for each resource based on the granularity of the resources in the monitoring set, this set will have a number of monitoring reference points equal to the number of measurement resources.

This approach could lead to different resources even within the same monitoring set being associated with different inference reference points from different occasions. As illustrated in the example from the figure, the first resource of the M-th occasion monitoring set might be associated with the (N-1)-th occasion of CSI reference resource, while a middle resource of the M-th occasion in monitoring set might be associated with the N-th occasion of CSI reference resource. Thus, based on resource granularity for determining monitoring reference point will create ambiguities on metric calculation and potential degrade monitoring accuracy.

For performance monitoring, for UE-side mode, for BM-case1, using monitoring set as granularity for determining monitoring reference point, e.g., the monitoring reference point is the first OFDM symbol of one occasion of the entire monitoring set.

For BM-Case2

Unified solution should be supported for both BM-case1 and BM-case2, we, thus, propose,

For performance monitoring, for UE-side mode, for BM-case2, support to associate inference reference point to the monitoring reference point, i.e., UE identifies monitoring reference point first, and then identify the associated inference reference point to calculate KPI, where the monitoring reference point is related to monitoring occasion set and the inference reference point is related to inference report.

For performance monitoring, for UE-side mode, for BM-case2, "closest" should be defined as the inference reference point (pertaining to inference report) that is nearest and occurs before the monitoring reference point (pertaining to monitoring set).

The most significant distinction between time-domain prediction and spatial-domain prediction is that time-domain prediction encompasses prediction results for multiple future time instances. That is, one performance monitoring contains results over multiple time instances. Consequently, the set-level granularity for monitoring reports associated with inference reports may not be adequate for capturing the intricacies of time-domain performance monitoring.

For monitoring an inference occasion set with multiple future occasions in one inference report, the monitoring RS needs to have a burst-like pattern in time domain, i.e., each monitoring occasion set needs to contain multiple monitoring occasions. Specifically, if the monitoring is period, each periodicity (a monitoring occasion set) of monitoring RS needs to contain one or multiple (e.g., 2, 3 or 4) occasions with a smaller periodicity.

Figure 3-6: Occasion level granularity for determining reference points for BM-Case2

Utilizing an association method that employs a monitoring occasion and an inference occasion as granularity, akin to the approach in spatial domain prediction, can result in different monitoring occasions within the same monitoring occasion set being linked to disparate inference occasion sets. As depicted in the preceding figure, the 1st monitoring occasion of the M-th monitoring occasion set is associated with the 2nd inference occasion of the N-th inference occasion set, whereas the 2nd monitoring occasion of the M-th monitoring occasion set is linked to the 1st inference occasion of the (N+1)-th inference occasion set. This indicates that the results within the same performance monitoring set, namely the M-th monitoring occasion set, are associated to prediction outcomes from different inference occasion sets, specifically the N-th and (N+1)-th inference occasion sets

For BM-Case2, utilizing an association method that employs a monitoring occasion and an inference occasion as granularity can result in different monitoring occasions within the same monitoring occasion set being linked to disparate inference occasion sets.

Figure 3-7: Occasion set level granularity for determining reference points for BM-Case2

In the context of time domain prediction, it is advisable to use the monitoring occasion set as the granularity for associating inference and prediction results. As illustrated in the figure above, the M-th monitoring occasion set is linked to the N-th inference occasion set. This methodology ensures that each monitoring occasion set is uniquely associated with a specific inference occasion set, thereby eliminating any potential for overlap or confusion. Additionally, each occasion within the monitoring occasion set can be uniquely mapped to an occasion in the associated inference occasion set, enabling the calculation of the corresponding metrics. This one-to-one mapping facilitates precise metric evaluation by ensuring a direct correlation between individual monitoring and inference occasions. Further, there could be cases that for the multiple inference occasions in one inference report, NW would like to flexibly monitor a subset of these occasions in one monitoring occasion set. Then how to map this inference occasion subset to the monitoring occasion set needs to be further determined. Consequently, we propose,

For performance monitoring, for UE-side mode, for BM-case2, using occasion set as granularity for determining monitoring reference point and inference reference point, e.g., the monitoring reference point is the first OFDM symbol of the first occasion of the entire monitoring occasion set.

For performance monitoring, for UE-side mode, for BM-case2, each occasion within the monitoring occasion set is uniquely mapped to an occasion in the associated inference occasion set.

- FFS：how to associate occasions between two occasion sets, if the number of monitoring occasions in a monitoring occasion set < the number of inference occasions in associated inference occasion set.

  - Monitoring type 1 option 1

Since the process of model monitoring on the NW-side AI model and NW side performance monitoring of UE-side AI model is similar, it is crucial to reduce report overhead for both. Moreover, considering the issues of data validity and real-time model monitoring, it is advisable to at least support the use of L1 signaling to feedback measurement results.

For monitoring report, support report overhead reduction for NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1), as well as for NW-side AI model, e.g., report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP.

Furthermore, it is important to note that the UE has already conducted measurements on the resources corresponding to Set A during performance monitoring, and the current beam indicated by DCI should also be encompassed within the Set A. If the report only includes the top-k measured results of Set A, there is a risk that the measured results of the current beam may not be included in the selected top-k beams, thus hindering the network side from acquiring model monitoring information for further beam indication. This scenario leads to a significant waste of UE energy consumption and resources. Consequently, it may be prudent to consider incorporating the measured results of the specified resources of Set A with the top-k measured resources in performance monitoring report.

For monitoring report, support to report measured L1-RSRP of indicated beam(s), e.g. current beams indicated by DCI, with measured L1-RSRP of top-k beams for both NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1) and NW-side AI model.

  - Monitoring type 1 option 2

Metrics

FL Proposal from summary:

For BAI calculation, considering the following with potentially down selection (at least for the case that full set of Set A can be measured)

- Option 1 (Top-1/1): The Top-1 beam with largest measured value of the resource set(s) for monitoring is Top-1 predicted beam

- Option 2 (1/Top-K): The Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams

  - K >1, or K is equal to the number of reported predicted beam configured by inference report configuration

- Option 3a (Top-K/M): The Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

  - K= 1, FFS other values

  - M is configured

  - K<=M

- Option 3b (Top-K/M): At least one of the Top-K predicted beams is among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

  - M is configured

- Option 4 (best of Top-K/1 with margin): The beam with largest measured value of L1-RSRP of Top-K predicted beams is within a margin X dB of largest measured value of L1-RSRP of the resource set(s) for monitoring

  - X=0, 1, FFS on whether to support other values ‘’

  - K=1 or K is equal to the number of reported predicted beam configured by inference report configuration, FFS other values

For the calculation of BAI, all of the above-mentioned options are viable except for option 3a. According to the calculation method outlined in the SI, Top-K/M is considered a relaxed requirement compared to the Top 1/K. Consequently, supporting option 3a is indeed unnecessary.

For monitoring metric, support Option 1 (Top-1/1), Option 2 (1/Top-K), Option 3b (Top-K/M) and Option 4 for BAI calculation.

How to report BAI

In metric definition, there are two approaches to define BAI: one based on the number of correct predictions and the other based on ratio of correct predictions. From the perspective of UE’s implementation and specification impact, using the count of correct instances is more straightforward, as it circumvents the challenges on how to quantize ratio. Further, it avoids the potential issues on how to define valid occasions when monitoring set is a subset of set A, as UE only needs to count the total number of correct predictions that fulfill the metric. Therefore, we have recommended utilizing the number of correct predictions as a basis for BAI results report. To implement this, the network must first configure the total number of monitoring instances within the monitoring CSI report. Subsequently, UE can report the number of correct instances from the total monitoring instances. We, thus, propose,

For the monitoring Type 1 Option 2 of UE-side model monitoring, for BM-Case1, support to report the number of correct predictions as BAI result.

- X (X≥1) monitoring occasions of a given CSI-ReportConfig with monitoring usage are configured to UE

- UE reports Y (0≤Y≤X) correct predictions from X associated monitoring occasions

- BAI field size in monitoring report is equal to log2(X+1)

For BM-Case2, time-domain prediction involves generating forecasting results for several future time instances, alongside monitoring resources over multiple temporal instances. Consequently, this necessitates addressing how to define granularity of performance monitoring metric within a CSI report for temporal domain prediction.

Figure 3-8: Association between monitoring and inference with sparse monitoring configuration for BM-Case2

In the figure above, predictive results for the subsequent four occasions are generated using measurement data from two historic occasions, with the assumption that the M-th monitoring occasion set is associated with the N-th inference occasion set.

To minimize performance monitoring overhead, NW may implement sparse monitoring occasions, where the number of occasions in the monitoring occasion set is less than occasions in the inference occasion set. If granularity of performance monitoring metric is per montioring occasion set, it should be aggregating these 2 performance monitoring results into a single metric. For example, a criterion might be that the monitoring results is deemed correct only if all monitoring occasion within a montioring occasion set are correct.

However, defining such criteria can profoundly influence the specification impact. Further, the accuracy of predictions typically declines along with the number of predicted occasions, leading to varying prediction accuracies across different future prediction occasions. Merging the performance monitoring results from these occasions would impede the network to accurately distinguish performance of different inference occasions within an inference occasion set.

Given these considerations, it is advisable to report the performance monitoring results for each monitoring occasion within a monitoring occasion set for BM-Case2.

For the monitoring Type 1 Option 2 of UE-side model monitoring, for BM-Case2, support to report the number of correct predictions as BAI result for each monitoring occasion within a monitoring occasion set.

- X (X≥1) monitoring occasion of a given CSI-ReportConfig with monitoring usage are configured to UE

- UE reports Y (0≤Y≤X) correct predictions from X associated monitoring occasions

- Each BAI field size in the monitoring report is equal to log2(X+1)

- The number of BAI field is equal to the number of monitoring occasions within a monitoring occasion set

Event related issues

Due to a brief mention of event-based reporting but lacked in-depth discussion in previous meeting, we discuss the difference between metric-based reporting and event-based reporting. Before delving into event-based reporting, it is imperative to establish a clear definition of metric-based reporting and event-based reporting, as well as the corresponding reporting content. As per the understanding of specification, a metric pertains to a calculated result of corresponding KPIs, whereas an event pertains to the occurrence of a specific event based on the results of corresponding metric. Therefore, based on this comprehension, the reporting content can be expanded as follows.

- Opt1: report metric

- Opt2: report event

- Opt3: based on event, report event and metric(s)

In Option 1, the UE consistently reports metrics based on a pre-configured periodicity, whereas in Option 2, the UE reports an event only when the event is triggered. Additionally, considering the UE's capability to predict comprehensive results, it can furnish more pertinent information including triggered event and associated event or metrics. For instance, when an event linked to the Top-1 beam accuracy smaller than a threshold is triggered, the UE can also report the top-1 RSRP difference to aid the network in determining whether to de-activate the model. Upon the triggering of this event, if the actual top-1 RSRP difference falls within an acceptable range, it indicates that the predicted top-1 beam and beam qualities remain favorable. Consequently, Option 3 has the potential to offer more valuable information with limited report overhead increase.

For performance monitoring, for UE-side model, support following options for further study on what/how to report,

- Opt1: report metric

- Opt2: report event

- Opt3: based on event, report event and metric(s)

- Other options cannot be precluded

  - Monitoring procedure for NW-side model

TR 38.843[2] outlines two options of Type 1 performance monitoring for UE-side models, namely NW-side performance monitoring and UE-assisted performance monitoring, where the UE sends reports to the NW for NW-side performance monitoring, or calculates performance metric(s) and reports them or an event to the NW for UE-assisted performance monitoring. The use of UE-assisted performance monitoring for the second operation can significantly reduce the report overhead for type 1 performance monitoring of UE-side model. On the other hand, performance monitoring for NW-side model can also benefit from such overhead reduction given by UE assisted performance monitoring.

Figure 3-9: UE assisted model monitoring for NW-side model

Figure 3-9 illustrates the performance monitoring procedure for UE-assisted model monitoring of NW-side models, which uses BM case 2 as an example. The procedure involves the following steps:

Step 1: The UE reports measurement results of Set B to the NW with a NW-side model for temporal domain beam prediction at occasion T.

Step 2: Temporal domain predicted labels can be obtained from the prediction results of top-N beam ID and/or corresponding L1-RSRP of future 8 occasions from occasion T + 3 to occasion T + 10. The predicted labels at occasion T +3 to occasion T + 10 are then transmitted to the UE at occasion T + 1.

Per occasion report:

Step 3: The UE receives monitoring RS resources at occasion T + 3 to obtain real measurement labels as performance monitoring reference. At the same time, the UE calculates KPI of performance metrics based on measurement labels of occasion T + 3 and predicted labels of occasion T + 3 received at step 2.

Step 4: The UE reports performance metrics or events of occasion T +3 obtained from step 3.

Step 5: The above two steps, i.e. step 3 and step 4, are cyclically executed from occasion T + 4 to occasion T + 10.

Multi-occasion results report:

Step 3: The UE receives monitoring RS resources from occasion T + 3 to occasion T + 10 to obtain real measurement labels of 8 occasions. Then, the UE calculates KPI of performance metrics based on 8 measurement labels and 8 predicted labels of occasion T + 3 ~ occasion T + 10.

Step 4: The UE reports performance metrics or events of occasion T +3 ~ occasion T + 10 obtained from step 3.

The above steps demonstrate the feasibility of UE-assisted model monitoring for NW-side models, and also can provide significant improvement on UCI overhead reduction. For example, assuming a NW-side model has 32 beams in Set A and predicts beams of 8 future occasions based on per occasion report, the following assumptions can be made to calculate UCI report overhead,

Assumption 1: Set A = 32 beams, predict 8 future occasions, report Top-4 beams information

Assumption 2: per beam ID report overhead = 5 bits per beam

Assumption 3: combination 4 beam ID report overhead = 16 bits

Assumption 4: KPI-1 = Top 4/1 beam accuracy, report overhead = 5 bits

Assumption 5: KPI-2 = Top-1 L1-RSRP difference, report overhead = 4 bits

Table 1: UCI overhead reduction of UE assisted performance monitoring for NW-side model

|  | UCI Report overhead per occasion | Total UCI report overhead | Overhead reduction |
| --- | --- | --- | --- |
| Method 1: beam ID report | Per beam ID report = 5 * 4= 20 bits Combination beam ID report = 16 bits | 16 * 8 ~ 20 * 8 = 128 bits ~ 160 bits | 68% ~ 75% |
| Method 2: beam ID of UE assisted report | Per Top-4/1 beam accuracy report = 5 bits | 5 * 8 = 40 bits |  |
| Method 3: beam ID + L1- RSRP report | Per beam ID report = 5 * 4= 20 bits Combination beam ID report = 16 bits L1-RSRP report = 7 + 4 * 3 = 19 bits | (128 bits ~ 160 bits) + (19 * 8) = 280 bits ~ 312 bits | 74%~77% |
| Method 4: beam ID + L1- RSRP of UE assisted report | Per Top-4/1 beam accuracy report = 5 bits Per Top-1 L1-RSRP accuracy report = 4 bits | 5 * 8 + 4 * 8 = 72 bits |  |

Table 1 shows the UCI overhead reduction achieved by various reporting methods. Method 1 reports only the Top-4 beam ID of 8 occasions to the NW, while method 2 reports the performance metric of KPI-1. Method 3 reports the Top-4 beam ID and corresponding L1-RSRP, whereas method 4 reports both KPI-1 and KPI-2 to the NW. It can be observed that method 2 achieves a UCI overhead reduction of 68% to 75% compared to method 1 with per beam ID report or combination beam ID report. Similarly, method 4 achieves a UCI overhead reduction of approximately 74%~77% compared to method 3, which reports the Top-4 beam ID and corresponding L1-RSRP. Please note that along with the need of having 64 or 128 set A beams, more overhead can be reduced based on method 2 or 4.

Significant UCI overhead reduction can be achieved by UE-assisted model monitoring compared to direct measurement results report for NW-side AI/ML model.

For performance monitoring, for NW-side model, support to report metrics for UCI reporting overhead reduction.

- Configuration of RS for UE-side model

  - RS configuration for data collection

A unified design of reference signals (RS) across different lifecycle management (LCM) stages, such as data collection, model inference, and performance monitoring, is crucial for the success of AI-based solutions. Therefore, it is recommended that the associated ID can be configured within CSI-ReportConfig.

Moreover, it has been agreed that for model inference, two distinct CSI-ResourceConfigIds can be configured separately for Set A and Set B. Consequently, for data collection, separate configurations for Set A and Set B should be specified within the CSI report configuration. Besides, since there is no need to report measurement data to NW for UE-side model data collection, report quantity shall be set to none.

This approach ensures alignment between data collection and inference/model monitoring framework, promoting a cohesive and efficient data management process. Thus, we proposal,

For data collection, for UE-side model, CSI-ReportConfig can used for configuring the resource for data collection purpose

- reportQuantity shall be set to none

- Associated ID can be configured in CSI-ReportConfig

- Two CSI-ResourceConfigIds are configured for Set A and Set B separately

  - Set B configuration for inference

In TR 38.843[2], following options are studied on the selection of Set B of beams (pairs):

-	Option 1: Set B is fixed across training and inference

-	Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference)

-	Opt 2A: Set B is changed following a set of pre-configured patterns

-	Opt 2B: Set B is randomly changed among pre-configured patterns

-	Opt 2C: Set B is randomly changed among Set A beams (pairs)

-	Opt 2D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C

-	The number of beams(pairs) in Set B can be fixed or variable

-	Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

-	Note: BM-Case1 and BM-Case2 may be considered for different option.

-	Note: This does not preclude the alternative that Set B is different from Set A.

It has been proved that while a single pre-defined beam subset with a fixed pattern in Set B may theoretically exhibit good performance, it lacks the necessary flexibility for practical implementation. In real-world scenarios, unexpected channel variations such as blockages can cause a particular beam or beam pair to experience performance loss or significant interference. Evaluation results from various companies in TR 38.843 [2] have highlighted the significant performance degradation that can occur when using mismatched fixed patterns in Set B during model training and inference. Therefore, it is crucial to consider the impact of channel variations and to incorporate flexibility into Set B design to ensure optimal performance in practical implementations.

Figure 4-1: Top-1 performance degradation for mismatched fixed pattern for DL Tx beam prediction

Figure 4-2: Top-2/1 performance degradation for mismatched fixed pattern for DL Tx beam prediction

Figure 4-3: Top-4/1 performance degradation for mismatched fixed pattern for DL Tx beam prediction

The simulation results presented in the three figures above were obtained from 7 companies that submitted their results in TR 38.843[2], which demonstrates beam accuracy degradation of DL Tx beam prediction performance on mis-matched fixed patterns in Set B between model training and model inference. Further, the beam accuracy degradation shown in the figures represents the difference between the beam accuracy with mis-matched fixed patterns in Set B and the beam accuracy with matched fixed patterns in Set B.

Based on these simulation results, it is observed that there is more than 65% performance degradation among different companies in the KPI of Top-1 beam prediction accuracy for DL Tx beam prediction with mis-matched fixed patterns in Set B. Similarly, a performance degradation of about 55-60% is observed in the KPI of Top-2/1 and Top4/1 beam prediction accuracy. These results highlight the importance of considering the impact of mis-matched fixed patterns in Set B and the need for flexible Set B design to ensure performance.

The simulation results from TR 38.843 show that using a wrong fixed pattern causes at least 65/60/55 percent performance degradation for the KPI of Top1, Top2/1 and Top 4/1 beam prediction accuracy, respectively.

The simulation results submitted by 10 companies in TR 38.843[2] provide a comparison between fixed patterns and pre-configured patterns in Set B. Based on these results, it is observed that there is a slightly performance degradation when using pre-configured patterns compared to fixed pattern in Set B.

Figure 4-4: Top-1 performance degradation for pre-configured patterns compared to fixed pattern

Figure 4-5: Top-2/1 performance degradation for pre-configured patterns compared to fixed pattern

Based on the simulation results presented in the figures above, it can be observed that there is an average performance decrease of less than 3% among 10 companies for the KPI of Top-1 and Top-2/1 beam prediction accuracy when using pre-configured patterns with Tx beam ID information as input, compared to fixed patterns in Set B. Further, it is important to note that approximately less than 2% performance degradation of Top-1 beam prediction accuracy and almost same Top-2/1 beam prediction accuracy of pre-configured pattern in Set B can be achieved for the majority of companies in comparison with fixed pattern in Set B. Thus, we have below observations,

Less than 3% average performance decrease among 10 companies submitted in TR 38.843 [2] for the KPI of Top1 and Top2/1 beam prediction accuracy of pre-configured patterns with Tx beam ID information compared to fixed pattern in Set B.

It can be observed from majority companies in TR38.843 [2] that only 2% performance degradation of Top-1 beam prediction accuracy and almost same performance of Top-2/1 beam prediction accuracy achieved for pre-configured patterns in Set B with Tx beam ID information as input compared to fixed pattern in Set B.

During the model training phase, the network only sends all the beams corresponding to Set A or Set B. The specific Set B patterns used for training, as well as which Set B patterns may provide better performance, are determined based on UE implementation and real channel. Consequently, during model inference phase, the network also needs to configure same Set B patterns. Therefore, UE needs to recommend some Set B patterns to prevent performance degradation due to inconsistencies in Set B patterns between the training and inference phases, and gNB to configure a resource list including multiple Set B patterns. Further, due to the time varying of channel conditions such as movement, blockage and UE rotation, gNB needs to further activate resources of one Set B pattern from the list by signaling. We, thus, propose,

For inference, for UE-side model, support UE to recommend some preferred Set B patterns which were trained during the UE-side model training phase.

For inference, for UE-side model, support gNB to configure a resource list including multiple Set B patterns and activate resources of one Set B pattern from the list by signaling during the inference phase.

  - Set A configuration for inference

For data collection, Set A should be configured for measurement as label for model training. For inference phase，although no measurement is needed for Set A, when considering beam interference management, it is important to address the following scenario: the base station decides not to use Tx beam 1 due to potential interference to other cells. Despite the gNB not utilizing Tx beam 1 for the subsequent data transmission, the beam may still be selected by the UE to report as a top-k predicted beam. Therefore, it is necessary to provide support for configuring beam subset restrictions of Set A associated with predicted beam reports to mitigate interference issues. This will help ensure that potential interference concerns are addressed and that the reporting of predicted beams takes into account the restrictions set by the base station to manage interference effectively. Thus, it is necessary to configure full and/or subset of Set A for UE-side model.

Additionally, when considering the different usage of Set A for data collection and model inference, it may be beneficial to indicate Set A or resources within Set A as a virtual set or virtual resource for inference configuration. In this context, "virtual set" or "virtual resource" refers to a legacy configuration that does not require measurements on these set A resources for the associated report. This approach can help streamline the configuration process and ensure that resources within Set A are optimized for model inference without the need for additional measurements.

For UE-side model, support to full set of Set A and subset restrictions indication of Set A for predicted beam report to address interference issue.

For inference, for UE-side model, Set A or resources in Set A should be indicated as virtual Set or virtual resource, which does not require measurement on these resources for the report.

  - How to configure RS for Set B measurement of multiple time instances for BM-case 2

Figure 4-7: same beams of Set B and different beams of Set B corresponding to multiple historical periods

For BM-case 2, the need to utilize measurement information from Set B across multiple historical periods creates the challenge of RS configuration of Set B across different historical periods. As illustrated in above figure, in method a, multiple historical periods use the same beams of Set B for measurement, corresponding to each historical period using beam 1 to beam 8. Conversely, in method b, multiple historical periods use different beams of Set B for measurement, providing more diverse beam information to assist UE for inference. This issue has been discussed in the SI, where it was noted that, under the same overhead, method b) significantly improves predictive performance compared to method a). Therefore, it is recommended to support the configuration of different historical periods corresponding to different beams of Set B for measurement. This approach can enhance the predictive performance and provide more diverse beam information for inference across multiple historical periods.

For inference, suggest to configure different beams of Set B for measurement corresponding to different historical periods for BM-Case 2.

Figure 4-8: unnecessary measurements for Set B occasions existed in the prediction window

As depicted in the above figure, the gray occasions represent the periodic RS resources, transmitted from T1, by legacy configuration method, and UE performs measurements at corresponding occasions to obtain Set B information for inference. During the 1st inference, UE uses beams from T1 to T4 as historical measurement information to infer the future prediction results from T5 to T10. However, at the next inference moment, there are two different behaviors. For option 1, UE measures T5-T8 to predict the future results from T9 to T14, without considering that T9 and T10 have already been predicted in previous prediction moment. On the other hand, for option 2, UE can measure from T7 to T10 to predict the future occasions from T11 to T16, and the Set B resources at T5 and T6 do not need to be measured. We believe that both options exist, and the benefit from Option 2 is clear as RS overhead for a number of CSI-RS occasions can be saved. gNB can even use these RS REs for scheduling other channels or UEs. Therefore, for the second configuration, further study is needed on how to avoid unnecessary measurements and RS overhead for CSI-RS occasions existed in the prediction window. We, thus, propose,

For inference, further study how to avoid unnecessary measurements and RS overhead for CSI-RS occasions existed in the prediction window for BM-Case 2.

  - Rx beam assumption for Set B

In TR 38.843[2], following observation was approved for the performance study on Quasi-Rx beam in DL Tx beam prediction,

Performance with different Rx beam assumption for DL Tx beam prediction

At least for BM-Case1 when Set B is a subset of Set A, and for DL Tx beam prediction, with the measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample, AI/ML provides the better performance than with measurements of random Rx beam(s).

-	Evaluation results from 12 sources show 20%~50% degradation with random Rx beam(s) comparing with the "best" Rx beam in terms of Top-1 prediction accuracy.

-	Evaluation results from 1 source shows 12% degradation with measurement of random Rx compared with measurement of best Rx in term of Top-1 beam prediction accuracy.

It demonstrates quasi-best Rx beam has considerable performance improvement for DL Tx beam prediction in comparison with random Rx beam. Although current specification provides implementation method to obtain Quasi-best Rx beam, it depends heavily on gNB implementation and UE coordination.

For example, a beam resource set with multiple beam resources is transmitted to UE for P3 procedure, and beam resources in a beam resource set for P2 procedure are QCLed to a beam resource specified in the P3 procedure. Thus, it can be assumed that UE receives P2 resource set by the best Rx beam searched by P3 resource set. In the current specification, a P3 procedure can only let UE acquire a best Rx beam corresponding to one Tx beam, while this best Rx beam may be different from the overall best Rx beam, which is the best Rx beam from all Tx beams. However, to acquire the best Rx beam corresponding to one Tx beam may not be sufficient for the quasi-best Rx beam acquisition for AI-based beam prediction, which is sensitive to Rx beam used in P2 resource set.

Based on evaluation in the SI, such best Rx beam corresponding to one Tx beam will lead to certain performance loss [2]. One the other hand, to search the best Rx beam for all Tx beams based on the current specification will lead to large RS overhead and large latency.  Due to time interval between P3 resource set and P2 resource set is also performed by gNB implementation, the quasi-best Rx beam obtained from P3 resource set may expire for the following P2 resource set receiving. Therefore, to trade off prediction performance, RS overhead and latency, it is desirable to perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B measurement result acquisition. Typically, to have a limited number of Tx beams can increase the prediction performance a lot. Meanwhile, it is also desirable to maintain the principle that UE does not need to report anything for P3 procedure, and the detailed Rx beam is up to UE implementation.

Figure 4-9: Quasi-best Rx beam acquisition

The figure above illustrates how to obtain the quasi-best Rx beam with two Tx beams from a Set B pattern. The first Tx beam, marked as blue, and the third beam, marked as yellow, of the Set B pattern are transmitted with 4 times repetition at T+1 and T+2, respectively. This enables the UE to obtain the quasi-best Rx beam by performing P3 based on the two Tx beams. Therefore, it is recommended to support the performance of P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for Set B measurement result acquisition.

To sum up, it is essential to support the assumption of a quasi-best Rx beam for the measurement of Set B for model inference with UE-side model. This is crucial for ensuring accurate and effective model inference, particularly in scenarios where the UE-side model is utilized to predict L1-RSRP. By supporting the quasi-best Rx beam assumption, the inference process can be optimized, leading to more reliable and precise model outcomes.

For data collection and inference, for UE-side model, support to use quasi-best Rx beam for Set A and/or Set B measurement, where quasi-best Rx beam is derived from P3 measurement on a small number of Tx beams from Set A/B.

- Configuration of RS for NW-side model

  - Set B configuration

In 4.1, we elaborate the importance of having multiple set B patterns for UE side model. The importance also holds for NW side model. If only one set B pattern is supported in NW side, and some beams in this set are blocked in fields, large performance loss is expected for AI based prediction. Hence NW side model also need to support multiple set B patterns.

In order to maintain consistency and improve flexibility in the Set B patterns used for mode inference with NW-side model, a dedicated set of beam measurement resources is established. This set supports multiple pre-configured Set B patterns and is made accessible to the UEs during the model inference procedure. This set may include relatively large number of beams, and it is not feasible to report all L1-RSRPs in UCI considering the large overhead. For example, if 4 set B patterns each with 8 beams are supported by gNB, and the union set contains 20 beams for measurement, to let UE reports all the 20 L1-RSRPs causes too much overhead. However, if UE freely selects the top N (e.g., top 8) beams from the 20 beams, the reported beam may form a pattern which hasn’t been trained by gNB, which makes the AI inference not possible.

Therefore, one way to address this issue is that the measurement results of a single Set B pattern, chosen from all the measured Set B patterns, are reported during the accessed UE report procedure. We, thus, propose,

For inference, for NW-side model, support gNB to configure a resource list including multiple Set B patterns and active resources of one Set B pattern from the list by signaling during the inference phase.

  - Set B and Set A association

During the inference phase of the NW-side model, only the measurements and feedback from Set B are taken into consideration. However, during model training and performance monitoring procedure, UE can report measurements of Set B and Set A separately. In this method, UE does not need to perceive the association between Set B and Set A. Consequently, there is no motivation to configure Set B and Set A simultaneously in one report.

For NW-side model, there is no motivation to configure Set B and Set A simultaneously in one report.

  - Rx beam assumption for Set B

For the same reasons as outlined in sub-section 4.5, it is important to support the assumption of quasi-best Rx beam for the measurement of Set A and/or Set B. Due to the inability of the UE to perceive Set B and Set A during data collection and model inference with NW-side model, specific enhancement should be focused on the configuration of beam sweeping resources to achieve quasi-best Rx beam acquisition.

For data collection and inference, for NW-side model, support to use quasi-best Rx beam for Set A measurement, where quasi-best Rx beam is derived from P3 measurement on a small number of Tx beams from Set A.

- Inference report for UE-side model

  - Report content for BM-Case 1, for inference

In RAN1#116 meeting, following agreement was approved for model inference with UE-side model,

Agreement

For UE-side model, at least for BM-Case1, for content in the report of inference results, support

- Opt 1: Beam information on predicted Top K beam(s) among a set of beams

- Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

- At least K=1 and more, FFS on max value

- FFS on beam information

- FFS on the definition of predicted Top K beam(s)

- FFS on definition of reported RSRP when applicable

- FFS on other information in the report with potential down selection among the following options

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

  - FFS on the quantization method of probability information

  - Probability information is the probability of the beam to be the Top 1 or Top K beam

- Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

  - FFS on definition of reported RSRP

  - FFS on the definition and quantization method of confidence information

- Other options are not precluded.

where the set of beams is Set A, i.e., the beams for UE prediction.

For the report content of UE-side model inference, supports one remaining issue is whether extra overhead reduction is needed. We think overhead reduction can be considered at least for L1-RSRP prediction due to the maximum number of K can be large. As the beams with RSRP lower than a certain threshold for a gNB will anyway not facilitate NW scheduling, it is not efficient to always report a fixed number of beams. Hence, we support to report M beams within X dB gap to the largest predicted L1-RSRP.

For inference, for UE-side model, support to report predicted L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest predicted value of L1-RSRP, as well as the number of reported beams.

In RAN 1#116bis meeting, following agreement and working assumption were approved for the definition of the RSRP of precited Top-k beam(s),

Agreement

For report content of inference results for UE-side model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options:

- Option A: Predicted RSRP

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement

- Where the predicted RSRP is based on AI/ML output

- Note: Support both Option A and Option B is not precluded.

In the scenario where Set B is a subset of Set A for model inference with UE-side model, the UE's predicted results may pertain to the measurements from Set B. Therefore, when the UE reports L1-RSRP of Top-k beams predicted from Set A, and the beam information of the Top-k predicted beams belongs to Set B, the UE reports the actual measured RSRP corresponding to the beam, which provides more useful and reliable information. Thus, we propose,

For inference, for UE-side model, when UE reports L1-RSRP of Top-k beams predicted from Set A, and for those that belongs to Set B, the UE reports the actually measured RSRP corresponding to the beam.

  - Report content for BM-Case 2, for inference

In RAN1#116bis meeting, following agreement was approved for BM-Case 2,

Agreement

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1

  - Note: overhead reduction is not precluded

- FFS on details

Due to predicted beam information of multiple future occasions should be contained in one beam report, time stamp information is naturally contained in this report.

For inference, for UE-side model, support time stamp information as beam content for BM-Case2.

For inference, for UE-side model, support time domain compression of beam resource indication to further reduce report overhead with a report including results of multiple occasions.

Furthermore, at least beam content in BM-Case 1 should be supported for BM-Case 2. We, thus, propose,

For inference, for UE-side model, support beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams per predicted instance for content in the report for BM-Case 2.

  - Report configuration for BM-Case 1 and BM-Case 2

Figure 6-1: legacy report configuration and predicted report configuration

In above figure, reportConfig-1 represents the conventional report configuration utilized for measurement report, while reportConfig-2 is employed for UE feedback of predictive results and is linked to Set B for measurement and Set A for prediction. Both types of report coexist for a Rel-19 UE supporting AI based beam report. Consequently, in the inference stage, it becomes imperative to contemplate how to differentiate between these two types of reports.

Since UE needs to feed back predicted results in a beam report for model inference with UE-side model, the report configuration can associate with a new report quantity to determine predicted results report without the need to define report purpose. This would minimize the impact on the existing specification.

For inference, for UE-side model, support new report quantity to differentiate between AI-based predicted report with legacy measured report.

As per the current report method, the UE reports top-k predicted L1-RSRPs and corresponding beam information, and the network may directly utilize this information for subsequent beam management. However, since the current beam, e.g. beam indicated by DCI, may not be included in the top-k predicted beams, the network cannot compare predicted beams with current beam. This situation could potentially lead to unnecessary frequent beam switching. Furthermore, if the network obtains measured L1-RSRP of the current beam through configuring additional RS resources to avoid frequent beam switching, additional measurement resources and reports for the current beam will be required, decreasing overall resource utilization of the system.

To address this issue, network can configure a beam resource indicator to force reporting the predicted L1-RSRP of an indicated beam. This will enable the network to obtain the predicted L1-RSRP of the top-k beams as well as the indicated beam, e.g. current beam indicated in TCI by gNB. By doing so, the network can avoid unnecessary beam switching or increasing resource utilization.

For inference, for UE-side model, support to report predicted L1-RSRP of indicated beam(s), e.g. current beam indicated by TCI, with predicted L1-RSRP of top-k beams in one prediction report.

  - Report overhead reduction for BM-Case 2

In RAN1#116bis meeting, following agreement was approved for BM-Case 2,

Agreement

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1

  - Note: overhead reduction is not precluded

- FFS on details

For a UE side model, UE needs to report multiple Top-N predicted beams in multiple future occasions to NW during model inference in BM-Case2. Due to beam correlation across multiple occasions, temporal domain compression of beam resource indication shall be considered to further reduce report overhead in a beam report with multiple time occasions for UE side model.

Figure 6-2: direct predicted beam resource indication

For example, a beam report includes 4 occasions where each occasion feeds back Top-8 predicted beam resource indicators from 32 total beams. If the direct predicted beam resource indication method is used in Figure 3-9, total 32 of CRI or SSBRI including in 4 occasions need to be reported in one beam report, and the corresponding report costs of CRI or SSBRI is 160 bits.

Figure 3-10: Time resource indication

Considering time correlation of beams across multiple occasions, the Top-N beams does not change too fast within a certain period. In other words, beam resource indicators may overlap between adjacent occasions. For example, unique beam resource indicators of 4 occasions in Figure 3-10 is far less than 32. Since the number of total time-domain occasions is much less than the total number of beam resources, the feedback cost of beam reports can be reduced by using TRI (time resource indicator). More specifically, first step is to determine unique predicted beam resources across multiple occasions, and then feed back corresponding time-domain indications for each unique predicted beam resource indicator. Thus, a TRI method can be considered to further reduce report overhead when multiple occasion results are included in one beam report.

Table 2: UCI overhead reduction of time resource utilization for BM-Case2

|  | Beam resource indicator | Total UCI report overhead | Overhead reduction of method 3 |
| --- | --- | --- | --- |
| Method 1: Implicit time stamp with CRI | 1st occasion: 8 * 5 = 40 bits 2nd occasion: 8 * 5 = 40 bits 3rd occasion: 8 * 5 = 40 bits 4th occasion: 8 * 5 = 40 bits Note: report 8 beams for each occasion, and beam resource indicator needs 5 bits for each reported beam indicated from total 32 beams. | 160 bits | 31.25%~45% |
| Method 2: Implicit time stamp with bitmap | 1st occasion: 32 bits 2nd occasion: 32 bits 3rd occasion: 32 bits 4th occasion: 32 bits Note: 32 bits corresponding to the total 32 beams to indicator reported beams per occasion | 128 bits |  |
| Method 3: TRI assisted report | Beam indicator: 32 bits TRI indicator: 4 * 14 = 56 bits Note: Beam indicator here is used to indicate 14 of unique predicted beam resources from 32 of total beams across multiple occasions, and each TRI indicator is used to indicated time resource for each unique beam (4-bit bitmap). | 88 bits |  |

We evaluate three types of UCI indication methods in Table 2 based on the case in Figure 3-10, where only 14 out of 32 predicted beam resources need to be indicated in one beam report for 4 occasions.

- Method 1 - Implicit time stamp with CRI: 8 CRIs each with 5 bits are reported for one occasion. 4 occasions are mapped to UCI sequentially, where time stamps are indicated implicitly based on the mapping order. Total 160 bits are required for this case.

- Method 2 - Implicit time stamp with bitmap: Similar mapping approach is used as Method 1. The difference is a fixed 32 bits bitmap is used to indicate 8 CRIs in each occasion. Total 128 bits are required.

- Method 3 - TRI assisted report: A 32-bit bitmap is firstly used to indicate the select 14 unique beams for 4 occasions. Further, 14 bitmaps are used to indicate TRI, where each bitmap contains 4 bits, used to indicate which ones from the four occasions the 14 beams locate. Total 88 bits are needed, and it achieves significant report overhead reduction ratio, i.e. 31%~45%, compared with Method 1 or Method 2.

For inference, for UE-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions.

In RAN#118 meeting, the following agreement was achieved,

Agreement

For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support

- Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting

  - For BM-Case 1, support differential RSRP report among multiple beams

  - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances

    - FFS details

Due to temporal correlation of beams across multiple occasions, it is anticipated that the predicted top-k beam IDs over multiple future time periods will be similar. Furthermore, the L1-RSRP of the top-k beams across multiple future time periods is not expected to show significant variations. Hence, when reporting TRI (Time Resource Indicator) instead of a direct predicted beam resource indication scheme, the L1-RSRPs across multiple occasions can utilize the same reference value for differential quantization. This reference value also needs to be determined through TRI and the beam index indicator indicated from a unique beam list.

For inference, for UE-side model, support one reference value for differential quantization across multiple time instances, where reference value also needs to be determined through TRI and the beam index indicator indicated from a unique beam list.

- Measurement report for NW-side model

  - Report content for data collection

During data collection procedure, network may receive UE logged measurement results from higher layer signaling if timeline of data collection is loose, while L1-layer signaling can be used for data reporting with restricted timeline. Thus, in order to enhance the flexibility of data collection for NW, we prefer to support both L1-layer signaling and high-layer signaling to contain the report of measurement data.

For data collection, for NW-side model, support to report UE measurement results via L1-layer signaling and higher-layer signaling.

In order to provide the NW with sufficient measurement results for model training, especially when a large number of beams are included in Set A and/or Set B (e.g., 64 beams in Set A), the UE needs to be able to provide feedback on these 64 L1-RSRPs or feedback on a subset of the L1-RSRP and corresponding resource indicators. It has been agreed to support the reporting of more than 4 beams in a beam report. Additionally, considering the excessive information contained in one report, it is essential to explore methods to reduce the overhead of the report.

For data collection, for NW-side model, confirming the agreement from SI phase that more than 4 beams can be reported in a beam report.

- The maximum number of reported beam related information in one report is related to UE’s capability.

For data collection, for NW-side model, it is crucial to investigate approaches to minimize the overhead of the report transmitted through L1-layer or higher-layer signaling.

In situations where the number of beams in Set A configured by the NW exceeds the maximum reported beam quantity allowed by the UE's capability, both L1-RSRPs and corresponding resource indicators should be indicated in a beam report to select a subset of beams. Furthermore, as the reporting of L1-RSRP and relative resource indicators are already supported in the current specification, the focus should be on confirming that the same report content can be utilized for data collection with the NW-side AI model.  Further, considering large size of Set A, enhancement on overhead reduction should be supported.

For data collection, for NW-side model, report content supported in current specification can be re-used for BM-Case 1 and BM-Case 2, i.e. L1-RSRPs and corresponding beam information.

For data collection, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 1.

Furthermore, due to beam correlation across multiple measurement occasions, similar as UE side model inference report in section 6.4, temporal domain compression of beam resource indication shall be considered to further reduce report overhead in a beam report with multiple time occasions for NW-side model. We, thus, propose,

For data collection and inference, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 2, e.g. consider to use time domain data compression to reduce overhead.

  - Report content for BM-Case 1, for inference

In RAN1#117 meeting, following agreement was achieved for NW-side model,

Agreement

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

- If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents)

- FFS on beam information

Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specification

It has been determined that if M represents the size of the measurement resource set, the content includes all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set. However, solely defining it in this manner may result in inconsistency understanding in the mapping order of reported L1-RSRPs to resources in the measurement resource set between UE side and NW side. This inconsistency could potentially lead to the network being unable to correctly interpret the corresponding measurement report. Therefore, further definition of the mapping order in the feedback report is necessary for the inference report for the NW-side model.

For inference, for NW-side model, if M = the size of the measurement resource set, report the L1-RSRP in a predetermined order corresponding to the measurement resource set.

Considering overhead reduction for large size of Set B, the enhancement of the quantization range directly results in numerous unnecessary resource indicators and/or corresponding L1-RSRPs being exempt from reporting. Consequently, support for an adaptive number of beams in a beam report should be considered.

For inference, for NW-side model, support to report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, as well as the number of reported beams

Furthermore, the current specification uses resource indicators to indicate which beams are included in the beam report and need to be reported. However, this design is based on the fact that only the top-1 or top-4 measured beams are reported. During model inference with a NW-side model, the UE may need to report a significantly larger number of beams than just the top-1 or top-4 beams. Therefore, it may be worth considering introducing a beam index type indicator to specify the meaning of the beam resource indicators included in a beam report.

For example, if the majority of beams are selected for reporting, the UE could report partial measured L1-RSRPs and the index of lowest-quality beams based on a beam set to the gNB, with the beam indices in the report used to indicate the lowest-quality beams. Similarly, if the minority of beams are selected for reporting, the UE could report partial measured L1-RSRPs and the index of highest-quality beams based on a beam set to the gNB, with the beam indices in the report used to indicate the highest-quality beams as legacy behavior.

For inference, for NW-side model, support to introduce a beam index type indicator to specify the meaning of resource indicator, which indicates the reported beams are the beams with highest quality or lowest quality.

  - Report content for BM-Case 2, for inference

To account for data correlation across multiple measurement periods for BM-Case 2, the report may include time stamp information if one report contains data from multiple periods. Specifically, if UE can’t report data from multiple occasions in one report, NW has to configure RS periodicity to be same as report periodicity to ensure NW has the knowledge of time information for the reported beams for training. This is restrictive. Hence the configuration of RS and report periodicities is more flexible if UE can report multiple measurement occasions in one report. Further, it can also facilitate the utilization of aperiodic report and aperiodic RS measurement.

For inference, for NW-side model, support to report of measurement results of multiple past time instances in one report for BM-Case 2.

  - Report enhancement for BM-Case 1 and BM-Case 2

The quantization overhead cost can be affected by two factors, which are quantization granularity and quantization range. The current NR specification employs 1 dB quantization granularity with a 7-bit quantization range from -140 dBm to -44 dBm for the best measurement beam, while a 2dB quantization granularity with a 4-bit quantization range is used to other beams differentially to the best beam.

It has been demonstrated in [2] that enlarging quantization steps for differential beams has minimal impact on beam prediction performance. Therefore, it may be beneficial to consider reducing report overhead from quantization perspective. Due to immoderately enlarging quantization steps may have a significant impact on prediction performance, it is important to balance report overhead reduction and beam predicted performance. In addition, considering that beams closer to the strongest beam are more important than others with lower L1-RSRP, for example, if the L1-RSRP of the first, second and third beam is -82dBm, -93dBm and -109dBm, respectively, the L1-RSRP of the first and second beams can be quantized more finely, while the L1-RSRP of the third beam can be quantized more coarsely. Thus, using multi-resolution L1-RSRP quantization, such as high-resolution quantization for a group of best RSRPs and low-resolution quantization for others, can help reduce feedback overhead as well as minimizing performance deterioration.

Additionally, existing quantization methods lead to significant performance degradation if quantization range from -140dBm to -44dBm is considered. For cases involving a quantization range, input L1-RSRPs lower than -140dBm are quantized as the lowest L1-RSRP level. However, in terms of UE’s measurement accuracy, -140dBm is far below than its’ noise floor. This implies that any values measured by the UE below the noise floor are actually invalid data. Therefore, it may be worth considering raising the lower limit of the quantization range.

For data collection and inference, for NW-side model, support enhancements on quantization range and quantization step to reduce overhead for measurement results report.

Section 6.4 offers a comprehensive explanation of how predicting multiple future beams can substantially decrease feedback overhead by leveraging on the correlation across multiple time instances. Similarly, in the context procedure with a NW-side model supporting the reporting of buffered measurement results across multiple measurement occasions, the correlation between measurement results from multiple time instances can also be leveraged to reduce feedback overhead. This approach can help streamline the feedback process and optimize the utilization of resources.

For data collection and inference, for NW-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions.

- AI processing capability

The current specification defines maximum number of CSI processing units and CPU processing timeline for CSI report. Correspondingly, for AI processing units, it is also necessary to consider how to define its timeline and quantity of AI processing units. We, thus, propose,

For UE-side model, further study whether to define AI process capability including re-use or modified the existing CSI computation time and CSI processing units.

- How to initiate data collection procedure

Figure 9-1: data collection procedure for UE-side model

For data collection procedure, where the AI model is deployed at UE side, the following steps are needed:

- In T-1: UE reports its capability information related to data collection purpose.

  - The specific contents of the capability information need to be considered with how the data collection procedure is triggered. For example, when the UE-side data collection procedure is initiated by the UE request signaling, extra information corresponding to model input/output is needed and can be carried through the request signaling. When the UE-side data collection procedure is initiated by NW, the extra information can be determined through the capability report, or through communication between UE-side OTT server and NW.

- In T-2: data collection procedure is initiated by UE request signaling or NW configuration.

- In T-3: gNB configures specific RS for data collection purpose with associated ID.

- In T-4: UE measures resources of Set A and/or Set B for model training based on a pre-measured Rx beam.

The content of UE capability reporting may be linked to the data collection process for the UE-side model, as outlined above. The specifics of data collection procedure initiation are illustrated in the following figures.

Figure 9-2: initiated data collection procedure through UE request

The figure above illustrates that the UE's OTT server sends a data collection request to the UE, and UE initiates data collection procedure via sending base station request signaling. Further, additional information pertaining to the model's input and/or output can also be signaled to NW along with the data collection request. Subsequently, the network can determine when or whether to configure/trigger the corresponding data collection resources based on the request signaling.

Figure 9-3: initiated data collection procedure through NW configuration

In Figure 9-3, the data collection request is sent by the OTT server through higher layer signaling or user plane, and then the NW can initiate the data collection procedure. Both of the aforementioned data collection methods for the UE-side model are feasible. Therefore, for the sake of flexibility, it is recommended to support both initiated behaviors for the data collection procedure of the UE-side model.

For UE-side model, data collection procedure can be initiated by gNB configuration or UE request signaling.

- Beam failure detection/report for model inference

Figure 10-1: BFD RS monitoring procedure

Currently, the UE monitors beam failure detection RS on all serving beams to evaluate whether the beam failure trigger condition is met. If all serving beam metrics meet the pre-defined conditions, the UE reports a beam failure instance indication to the UE higher layer. Conversely, if the UE physical layer determines that no beam failure instance has occurred, no indication is sent to the UE higher layer. For example, in above figure, UE-2 suffers beam failure instance and sends BFI indication to UE higher layer for BFI counter calculation.

In AI based beam prediction, an AI model are specified for model inference to predict Top-N L1-RSRPs or even all of the L1-RSPRs. Further, AI model has ability of temporal domain beam prediction for the prediction of L1-RSPRs at future time occasions. In the aforementioned scenarios, the inference side of the AI model is capable of obtaining all or Top-N predictive L1-RSRPs at multiple future occasions. As such, this can assist the detection of beam failure and no dedicated BFD RS may be needed, if AI model of L1-RSPR prediction enabled.

Figure 10-2: early cell switching

Figure 10-3: suspend transmission

More specifically, if model inference results of all predictive L1-RSPRs or Top-N predictive L1-RSPRs at one future occasion are less than or equal to pre-defined L1-RSRP threshold, UE can assume a hypothetical beam failure instance and reports to the higher layer. Consequently, an early beam failure event can be identified from UE side. Further, UE may perform early beam recovery procedure or even cell switching procedure based on time duration of hypothetical beam failure in Figure 10-2, or suspend transmission can be achieved in NW side within time duration of hypothetical beam failure from UE side in Figure 10-3. Overall, incorporating AI in the beam failure detection procedure has the potential to significantly enhance the reliability and efficiency of beam management.

Support to using AI beam prediction for beam failure detection/report enhancement.

- RRC parameters

Several RRC parameters to enable AI-based beam prediction may be potentially agreed in future meetings. Here we try to list all of them in the following table.

| Parameter name in the spec | New or existing? | Description | Parent IE | Value range |
| --- | --- | --- | --- | --- |
| InferenceParameterSet | New | Indicate inference related parameters for applicability report only (not for inference) | - | - |
| AssociatedID | New | Similar properties of a list can be assumed that same resource IDs or mapping ordering in Set A/Set B is consistent from training to inference if UE receives the same associated ID | CSI-ReportConfig, InferenceParameterSet | FFS |
| AssociatedCellIndicator | New | Indicate the associated ID belongs to the indicated cell | CSI-ReportConfig, InferenceParameterSet | FFS |
| BeamPatternID | New | Indicate beam pattern for Set B | CSI-ReportConfig, InferenceParameterSet | FFS |
| ReportQuantity | Existing | Indicate predicted CRI/SSBRI or predicted RSRP and predicted CRI/SSBRI | CSI-ReportConfig, InferenceParameterSet | {predicted-CRI/SSBRI, predicted- L1-RSRP -CRI/SSBRI} |
| VirtualResourceIndicator | New | Indicate resources in Set A shall be measured or not. | CSI-ReportConfig, | {0,1} |
| BeamSubsetRestrictionForInference | New | Indicate activated resource in Set A for inference | CSI-ReportConfig, | FFS |
| BeamSubsetRestrictionForMonitoring | New | Indicate activated resource in Set A for monitoring | CSI-ReportConfig, | FFS |
| BM-CaseIndicator | New | Indicate spatial domain prediction or temporal domain prediction | CSI-ReportConfig, InferenceParameterSet | {BM-Case1, BM-Case2} |
| NumberOfPredictedOccasions | New | Indicate the number of predicted occasions for temporal domain prediction | CSI-ReportConfig, InferenceParameterSet | FFS |
| ApplicableReportIndicator | New | Indicate applicable report following option A or option B | CSI-ReportConfig, | FFS |
| OccasionAssociatedIndicator | New | Indicate association of occasions between monitoring occasion Set and inference occasion Set | CSI-ReportConfig, | FFS |

Following RRC parameters can be introduced,

| Parameter name in the spec | New or existing? | Description | Parent IE | Value range |
| --- | --- | --- | --- | --- |
| InferenceParameterSet | New | Indicate inference related parameters for applicability report only (not for inference) | - | - |
| AssociatedID | New | Similar properties of a list can be assumed that same resource IDs or mapping ordering in Set A/Set B is consistent from training to inference if UE receives the same associated ID | CSI-ReportConfig, InferenceParameterSet | FFS |
| AssociatedCellIndicator | New | Indicate the associated ID belongs to the indicated cell | CSI-ReportConfig, InferenceParameterSet | FFS |
| BeamPatternID | New | Indicate beam pattern for Set B | CSI-ReportConfig, InferenceParameterSet | FFS |
| ReportQuantity | Existing | Indicate predicted CRI/SSBRI or predicted RSRP and predicted CRI/SSBRI | CSI-ReportConfig, InferenceParameterSet | {predicted-CRI/SSBRI, predicted- L1-RSRP -CRI/SSBRI} |
| VirtualResourceIndicator | New | Indicate resources in Set A shall be measured or not. | CSI-ReportConfig, | {0,1} |
| BeamSubsetRestrictionForInference | New | Indicate activated resource in Set A for inference | CSI-ReportConfig, | FFS |
| BeamSubsetRestrictionForMonitoring | New | Indicate activated resource in Set A for monitoring | CSI-ReportConfig, | FFS |
| BM-CaseIndicator | New | Indicate spatial domain prediction or temporal domain prediction | CSI-ReportConfig, InferenceParameterSet | {BM-Case1, BM-Case2} |
| NumberOfPredictedOccasions | New | Indicate the number of predicted occasions for temporal domain prediction | CSI-ReportConfig, InferenceParameterSet | FFS |
| ApplicableReportIndicator | New | Indicate applicable report following option A or option B | CSI-ReportConfig, | FFS |
| OccasionAssociatedIndicator | New | Indicate association of occasions between monitoring occasion Set and inference occasion Set | CSI-ReportConfig, | FFS |

- Conclusions

In this contribution, we discuss some issues on AI/ML for beam management and have the following observations:

- For option 1, i.e. based on CSI reference resource of inference report corresponding to a monitoring report to associate closest reference point of monitoring set, it takes following disadvantages:

- Monitoring Set and inference Set association failure

- Associating with an early monitoring result that is farther from the inference result, potentially leading to degraded prediction performance not attributable to AI prediction performance

- Associating with an early monitoring result that was already linked to a close inference result in early metric calculation, which means the monitoring result is associated twice

For BM-Case2, utilizing an association method that employs a monitoring occasion and an inference occasion as granularity can result in different monitoring occasions within the same monitoring occasion set being linked to disparate inference occasion sets.

Significant UCI overhead reduction can be achieved by UE-assisted model monitoring compared to direct measurement results report for NW-side AI/ML model.

The simulation results from TR 38.843 show that using a wrong fixed pattern causes at least 65/60/55 percent performance degradation for the KPI of Top1, Top2/1 and Top 4/1 beam prediction accuracy, respectively.

Less than 3% average performance decrease among 10 companies submitted in TR 38.843 [2] for the KPI of Top1 and Top2/1 beam prediction accuracy of pre-configured patterns with Tx beam ID information compared to fixed pattern in Set B.

It can be observed from majority companies in TR38.843 [2] that only 2% performance degradation of Top-1 beam prediction accuracy and almost same performance of Top-2/1 beam prediction accuracy achieved for pre-configured patterns in Set B with Tx beam ID information as input compared to fixed pattern in Set B.

- and proposals:

- For inference, for UE-side model, the associated ID can be configured in CSI-reportConfig.

For inference, for UE-side model, the associated ID can be configured in inference parameter set prior to CSI-reportConfig to address the consistency issue.

For inference, for UE-side model, associated ID should be mandatory configured within both inference parameter set and CSI framework.

For inference, for UE-side model, pattern ID can be configured for Set B pattern to address consistency of Set B.

For inference, for UE-side model, similar properties of a list can be assumed that same resource IDs or through other rules (e.g. same ordering in a list) in Set A/Set B is consistent from training to inference if UE receives the same associated ID.

For inference, for UE-side model, an indicator should be configured within the CSI-ReportConfig to clearly specify whether following processing follows option A or option B.

For inference, for UE-side model, the set of inference related parameter in Step 3 at least includes,

- Associated ID

- Pattern ID

- Size of Set B

- Size of Set A

- Indicated report content, i.e. {CRI/SSBRI, or predicted RSRP-CRI/SSBRI}

- BM case indicator, i.e. {case 1, or case 2}

- If 	Case 2,

          - Periodicity of Set B

          - Periodicity of Set A

          - Number of measurement occasions

          - Number of predicted occasions

For inference, for UE-side model, unified applicable report should be considered for both option A and option B in Step 4, and a bitmap indicator is preferred to indicate which CSI report or inference parameter Set is applicable or not.

For inference, for UE-side model, UE can report its supported inference parameter set(s) along with the applicable report in Step 4 if all configurations for both option A and option B in Step 3 are not applicable.

For inference, for UE-side model, along the line of the agreed working assumption on the consistency issue in AI 9.1.3.3, support to prioritize discussion on how UE-side AI model can work with local associated ID and study how to reduce UE side complexity on managing local associated IDs.

For inference, for UE-side model, introduce a cell indicator along with the configuration of associated ID to indicate how to interpret the associated ID for further address consistency issue.

For performance monitoring, at least support L1 beam reporting for both UE-side model and NW-side model.

For performance monitoring, for UE-side mode, for BM-case1, support to associate inference reference point to the monitoring reference point, i.e., UE identifies monitoring reference point first, and then identify the associated inference reference point to calculate KPI, where the monitoring reference point is related to monitoring set and the inference reference point is related to inference report.

For performance monitoring, for UE-side mode, for BM-case1, "closest" should be defined as the inference reference point (pertaining to inference report) that is nearest and occurs before the monitoring reference point (pertaining to monitoring set).

For performance monitoring, for UE-side mode, for BM-case1, using monitoring set as granularity for determining monitoring reference point, e.g., the monitoring reference point is the first OFDM symbol of one occasion of the entire monitoring set.

For performance monitoring, for UE-side mode, for BM-case2, support to associate inference reference point to the monitoring reference point, i.e., UE identifies monitoring reference point first, and then identify the associated inference reference point to calculate KPI, where the monitoring reference point is related to monitoring occasion set and the inference reference point is related to inference report.

For performance monitoring, for UE-side mode, for BM-case2, "closest" should be defined as the inference reference point (pertaining to inference report) that is nearest and occurs before the monitoring reference point (pertaining to monitoring set).

For performance monitoring, for UE-side mode, for BM-case2, using occasion set as granularity for determining monitoring reference point and inference reference point, e.g., the monitoring reference point is the first OFDM symbol of the first occasion of the entire monitoring occasion set.

For performance monitoring, for UE-side mode, for BM-case2, each occasion within the monitoring occasion set is uniquely mapped to an occasion in the associated inference occasion set.

- FFS：how to associate occasions between two occasion sets, if the number of monitoring occasions in a monitoring occasion set < the number of inference occasions in associated inference occasion set.

For monitoring report, support report overhead reduction for NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1), as well as for NW-side AI model, e.g., report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP.

For monitoring report, support to report measured L1-RSRP of indicated beam(s), e.g. current beams indicated by DCI, with measured L1-RSRP of top-k beams for both NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1) and NW-side AI model.

For monitoring metric, support Option 1 (Top-1/1), Option 2 (1/Top-K), Option 3b (Top-K/M) and Option 4 for BAI calculation.

For the monitoring Type 1 Option 2 of UE-side model monitoring, for BM-Case1, support to report the number of correct predictions as BAI result.

- X (X≥1) monitoring occasions of a given CSI-ReportConfig with monitoring usage are configured to UE

- UE reports Y (0≤Y≤X) correct predictions from X associated monitoring occasions

- BAI field size in monitoring report is equal to log2(X+1)

For the monitoring Type 1 Option 2 of UE-side model monitoring, for BM-Case2, support to report the number of correct predictions as BAI result for each monitoring occasion within a monitoring occasion set.

- X (X≥1) monitoring occasion of a given CSI-ReportConfig with monitoring usage are configured to UE

- UE reports Y (0≤Y≤X) correct predictions from X associated monitoring occasions

- Each BAI field size in the monitoring report is equal to log2(X+1)

- The number of BAI field is equal to the number of monitoring occasions within a monitoring occasion set

For performance monitoring, for UE-side model, support following options for further study on what/how to report,

- Opt1: report metric

- Opt2: report event

- Opt3: based on event, report event and metric(s)

- Other options cannot be precluded

For performance monitoring, for NW-side model, support to report metrics for UCI reporting overhead reduction.

For data collection, for UE-side model, CSI-ReportConfig can used for configuring the resource for data collection purpose

- reportQuantity shall be set to none

- Associated ID can be configured in CSI-ReportConfig

- Two CSI-ResourceConfigIds are configured for Set A and Set B separately

For inference, for UE-side model, support UE to recommend some preferred Set B patterns which were trained during the UE-side model training phase.

For inference, for UE-side model, support gNB to configure a resource list including multiple Set B patterns and activate resources of one Set B pattern from the list by signaling during the inference phase.

For UE-side model, support to full set of Set A and subset restrictions indication of Set A for predicted beam report to address interference issue.

For inference, for UE-side model, Set A or resources in Set A should be indicated as virtual Set or virtual resource, which does not require measurement on these resources for the report.

For inference, suggest to configure different beams of Set B for measurement corresponding to different historical periods for BM-Case 2.

For inference, further study how to avoid unnecessary measurements and RS overhead for CSI-RS occasions existed in the prediction window for BM-Case 2.

For data collection and inference, for UE-side model, support to use quasi-best Rx beam for Set A and/or Set B measurement, where quasi-best Rx beam is derived from P3 measurement on a small number of Tx beams from Set A/B.

For inference, for NW-side model, support gNB to configure a resource list including multiple Set B patterns and active resources of one Set B pattern from the list by signaling during the inference phase.

For NW-side model, there is no motivation to configure Set B and Set A simultaneously in one report.

For data collection and inference, for NW-side model, support to use quasi-best Rx beam for Set A measurement, where quasi-best Rx beam is derived from P3 measurement on a small number of Tx beams from Set A.

For inference, for UE-side model, support to report predicted L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest predicted value of L1-RSRP, as well as the number of reported beams.

For inference, for UE-side model, when UE reports L1-RSRP of Top-k beams predicted from Set A, and for those that belongs to Set B, the UE reports the actually measured RSRP corresponding to the beam.

For inference, for UE-side model, support time stamp information as beam content for BM-Case2.

For inference, for UE-side model, support time domain compression of beam resource indication to further reduce report overhead with a report including results of multiple occasions.

For inference, for UE-side model, support beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams per predicted instance for content in the report for BM-Case 2.

For inference, for UE-side model, support new report quantity to differentiate between AI-based predicted report with legacy measured report.

For inference, for UE-side model, support to report predicted L1-RSRP of indicated beam(s), e.g. current beam indicated by TCI, with predicted L1-RSRP of top-k beams in one prediction report.

For inference, for UE-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions.

For inference, for UE-side model, support one reference value for differential quantization across multiple time instances, where reference value also needs to be determined through TRI and the beam index indicator indicated from a unique beam list.

For data collection, for NW-side model, support to report UE measurement results via L1-layer signaling and higher-layer signaling.

For data collection, for NW-side model, confirming the agreement from SI phase that more than 4 beams can be reported in a beam report.

- The maximum number of reported beam related information in one report is related to UE’s capability.

For data collection, for NW-side model, it is crucial to investigate approaches to minimize the overhead of the report transmitted through L1-layer or higher-layer signaling.

For data collection, for NW-side model, report content supported in current specification can be re-used for BM-Case 1 and BM-Case 2, i.e. L1-RSRPs and corresponding beam information.

For data collection, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 1.

For data collection and inference, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 2, e.g. consider to use time domain data compression to reduce overhead.

For inference, for NW-side model, if M = the size of the measurement resource set, report the L1-RSRP in a predetermined order corresponding to the measurement resource set.

For inference, for NW-side model, support to report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, as well as the number of reported beams

For inference, for NW-side model, support to introduce a beam index type indicator to specify the meaning of resource indicator, which indicates the reported beams are the beams with highest quality or lowest quality.

For inference, for NW-side model, support to report of measurement results of multiple past time instances in one report for BM-Case 2.

For data collection and inference, for NW-side model, support enhancements on quantization range and quantization step to reduce overhead for measurement results report.

For data collection and inference, for NW-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions.

For UE-side model, further study whether to define AI process capability including re-use or modified the existing CSI computation time and CSI processing units.

For UE-side model, data collection procedure can be initiated by gNB configuration or UE request signaling.

Support to using AI beam prediction for beam failure detection/report enhancement.

Following RRC parameters can be introduced,

| Parameter name in the spec | New or existing? | Description | Parent IE | Value range |
| --- | --- | --- | --- | --- |
| InferenceParameterSet | New | Indicate inference related parameters for applicability report only (not for inference) | - | - |
| AssociatedID | New | Similar properties of a list can be assumed that same resource IDs or mapping ordering in Set A/Set B is consistent from training to inference if UE receives the same associated ID | CSI-ReportConfig, InferenceParameterSet | FFS |
| AssociatedCellIndicator | New | Indicate the associated ID belongs to the indicated cell | CSI-ReportConfig, InferenceParameterSet | FFS |
| BeamPatternID | New | Indicate beam pattern for Set B | CSI-ReportConfig, InferenceParameterSet | FFS |
| ReportQuantity | Existing | Indicate predicted CRI/SSBRI or predicted RSRP and predicted CRI/SSBRI | CSI-ReportConfig, InferenceParameterSet | {predicted-CRI/SSBRI, predicted- L1-RSRP -CRI/SSBRI} |
| VirtualResourceIndicator | New | Indicate resources in Set A shall be measured or not. | CSI-ReportConfig, | {0,1} |
| BeamSubsetRestrictionForInference | New | Indicate activated resource in Set A for inference | CSI-ReportConfig, | FFS |
| BeamSubsetRestrictionForMonitoring | New | Indicate activated resource in Set A for monitoring | CSI-ReportConfig, | FFS |
| BM-CaseIndicator | New | Indicate spatial domain prediction or temporal domain prediction | CSI-ReportConfig, InferenceParameterSet | {BM-Case1, BM-Case2} |
| NumberOfPredictedOccasions | New | Indicate the number of predicted occasions for temporal domain prediction | CSI-ReportConfig, InferenceParameterSet | FFS |
| ApplicableReportIndicator | New | Indicate applicable report following option A or option B | CSI-ReportConfig, | FFS |
| OccasionAssociatedIndicator | New | Indicate association of occasions between monitoring occasion Set and inference occasion Set | CSI-ReportConfig, | FFS |

References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

3GPP TR 38.843