3GPP TSG RAN WG1 #120		R1-2500352

Athens, Greece, February 17th – 21st, 2025

Source:	vivo

Title:	Discussion on other aspects for Rel-19 Ambient IoT

Agenda Item:	9.4.4

Document for:	Discussion and Decision

- Introduction

In RAN#106 meeting, the work item on Solutions for Ambient IoT (Internet of Things) in NR has been approved with following RAN1 scope [1]:

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this contribution, we discuss other aspects including resource allocation for multiplexing/multiple access, scheduling information and timing relationships.

- Multiplex/Multiple access

According to the WID, D2R uses only TDMA and FDMA for multiplexing/multiple access. Before detailing TDMA and FDMA resource allocation during the A-IoT random access procedure, we use the term "Access occasion" from TR 38.769 where an Access occasion is defined as an opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device). A set of access occasion(s) for different A-IoT device(s) is scheduled via the R2D message triggering random access by the reader as shown in Figure 6.3.4-1 of TR 38.769 [2].

  - Resource allocation for Msg1

For Msg1 resource allocation, the following agreements were made in RAN1#118 and #118bis meetings:

RAN1#118 meeting: Agreement Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following - How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 - How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1 RAN1#118bis meeting: Agreement RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). - The study includes - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain - Determination of the X time domain resource(s) by the device - Addressing timing errors for adjacent time domain resources due to residual SFO of the device Agreement@RAN1#117 Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

Frequency domain resource allocation for a D2R transmission

A R2D transmission triggering random access can schedule multiple i.e., Y, where Y1 access occasions multiplexed in frequency domain for Msg.1 transmission. Remaining issues for frequency domain resource allocation are following

- Issue#1: How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1

- Issue#2: How a device determines the frequency domain resource for the D2R transmissions for Msg.1

For the 1st issue, it is also related to the scheduling information needed for a D2R transmission, which is discussed in section 3.2. One frequency domain resource for a D2R transmission can be identified by the frequency domain location i.e., FSx and the transmission bandwidth, i.e, Btx,D2R.

- Frequency domain location for each resource FSx can be derived by the indication of Tb and R, FSx = R/Tb. For example,

  - FSx = R/Tb=1/(2 × chip length), where the time duration Tb corresponds to an information bit, R is a codeword repetition number for small frequency shift with Manchester line code Option1 or R is the number of square wave periods for small frequency shift with Manchester line code Option2 or no line codes

- Transmission bandwidth, Btx,D2R, no need to indicate the transmission bandwidth since the transmission bandwidth is natural consequence of the data rate and transparent to the device. The transmission bandwidth can be derived by Btx,D2R = 2/Tb

- About the maximum number of resources that can be FDMed, i.e., determining the maximum value of Y, based on our companion contribution [3], following factors should be considered for determination of Y.

  - SFO impacts on the D2R transmission bandwidth and small frequency shift

  - Avoid odd order harmonics, which makes R adopt the values of 2n and reduce impacts of CW interference etc.

  - D2R data rate is comparable with RFID, e.g., from ~5kbps to ~640kbps

Proposal 2.1-1: For Y (Y1) frequency domain resource(s) for D2R transmission(s) for Msg1 that determined by a R2D transmission triggering random access,

- the frequency location is determined by the parameter related to small frequency shift, i.e., R and chip/information bit duration

- No need to indicate the transmission bandwidth

The frequency domain location for each resource is determined by D2R information bit or chip duration and R.  About indicating the R and/or the number of FDMed access occasion(s) i.e., Y in the R2D control information, following options can be considered.

Option 1: Two indication fields for Y and R.

- The field length for Y can be fixed for indicating the number of FDMed resources, e.g., log2Ymax bits to accommodating the maximum value of Y, i.e., Ymax; and based on the indicated Y, the total bits for indicating Y number of R can be further determined.

Option 2: One joint indication of Y and R.

- Specification can define the association between the Y and R, based on the number of associations, the field length for the joint indication can be determined.

- For example, the number of associations can be 8, 3-bit is sufficient for the joint indication and can be defined as following Table 2.1-1.

Table 2.1-1: Example for defining the joint indication for Y and Ri values

| Joint indication field: 3 bits (b2,b1,b0) | Y: the number of FDMed access occasions | Ri: {R0,R1,…,RY-1} |
| --- | --- | --- |
| 000 | 1 | {1} |
| 001 | 2 | {1,4} |
| 010 | 2 | {2,8} |
| 011 | 3 | {1, 4, 8} |
| 100 | 3 | {1, 4, 8, 16} |
| 101 | 4 | {2, 4, 8, 16} |
| 110 | 5 | {1, 4, 8, 16, 32} |
| 111 | 6 | {1, 4, 8, 16, 32, 64, 128} |

Option 3: One indication field in the form of bitmap for the set of candidate R values only, e.g., {Ri}, where each Ri value is associated with one bit location, the bit set to 1 indicates that the Ri is used/present; the bit set to 0 indicates that the Ri is NOT used/absent for frequency domain resource allocation.

- For example: the candidate Ri values in the set {R0, R1, …,R6} can be defined in the specification as {1, 2, 4, 8, 16, 32, 64}, then 7-bit bitmap as shown in Table 2.1-2, where the leftmost bit corresponds to R6 and the rightmost bit corresponds to R0 can be used to indicate whether Ri is present. Following table represents the number of FDMed access occasions is 4, and the frequency locations Ri=1, 4, 8, 16

Table 2.1-2 bitmap based Ri indication

| Bitmap field for Ri indication | R6 | R5 | R4 | R3 | R2 | R1 | R0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 1 | 1 | 1 | 0 | 1 |

Above three options, each has pros and cons. Option 1 has the benefits of the full flexibility on the number of Y and selection of Ri values compared to Option 2. While Option 3 can also achieve the full flexibility with the fixed overhead. However, with larger value of Y, Option 1 requires much more overhead than option 3. For example, assuming the maximum value of Y is 8, and the number of candidate Ri values is also 8, only when Y=1, the overhead of Option 1 is 6 bits which is smaller than Option 3 of 8 bits. The overhead for Option 2 depends on the extent of the indication flexibility on the FDMed access occasions. In summary, Option 2 or Option 3 can achieve the better tradeoff between the flexibility and overhead, thus is preferred.

Proposal 2.1-2: For indicating the frequency domain locations for FDMA of Y where Y1 access occasions, following Option 2 and 3 can be down-selected considering R2D overhead and the flexibility for FDMed access occasion(s) indication.

- Option 2: One joint indication of Y and R, that the specification can define the association between the Y and R, based on the number of associations, the field length for the joint indication can be determined.

- Option 3: One indication field in the form of bitmap for the set of candidate R values only, e.g., {Ri}, where each Ri value is associated with one bit location, the bit set to 1 indicates that the Ri is used/present; the bit set to 0 indicates that the Ri is NOT used/absent for frequency domain resource allocation.

For 2nd issue, RAN2 agreed that for CBRA, at least for TDMA case, the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader. It needs to be further discussed if this is applicable to FDMA case. From RAN1 perspective, device should have the same ability for small frequency shift, hence the same resource selection rule as for TDMA case can be used for FDMA case.

Proposal 2.1-3: From RAN1 perspective, for CBRA and for FDMA case, the device randomly selects one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader.

Time domain resource allocation for a D2R transmission

A R2D transmission triggering random access can schedule multiple i.e., X, where X1 access occasions multiplexed in time domain for Msg.1 transmission. Remaining issues for time domain resource allocation are following:

- Issue#1: How the time domain resources are allocated for the TDMA of D2R transmissions for Msg.1

- Issue#2: Determination of maximum value of X

For issue#1, one time domain resource for a D2R transmission can be identified by the starting time and the ending time/transmission length for each resource, which is also discussed in section 3.2 of scheduling information for D2R transmission.

- For the starting time for Msg1 transmission, there are two options agreed in RAN1#117. Option 1 is based on a time window by TR2Dmin and TR2Dmax, similar to RFID; Option 2 is based on transmission timing indication, similar to NR k2 indication for NR PUSCH. For TDMA of X where X>1 Msg1 transmissions, using a fixed time window [TR2Dmin to TR2Dmax] works for the 1st Msg1 transmission but becomes inefficient for subsequent transmissions. This is especially true as timing uncertainty increases with each additional Msg1 due to SFO impacts. Option 2 is more straightforward, efficient and can be a unified solution to determine the transmission timing for the TDMA of X D2R transmissions.

- For the ending time/transmission length, we think it is derived by the PDRCH TBS, D2R information bit duration, small frequency shift, the number of repetitions and FEC if used.

Proposal 2.1-4: For X (X1) time domain resource(s) for D2R transmission(s) for Msg1 that determined by a R2D transmission triggering random access,

- the starting time TR2D is indicated by the control information in the R2D transmission, where TR2D  TR2D_min (i.e., Option 2)

- No need to indicate the time length for PDRCH transmission. It is derived by the PDRCH TBS, D2R information bit duration, small frequency shift, the number of repetitions and FEC if used.

For issue#2 of determining the maximum value of X, the factors of device implementation complexity, power consumption and the resource usage efficiency and inventory latency affected by SFO need to be considered. In the following, we give analysis for determining the maximum value of X from the perspective of resource usage efficiency and inventory latency. Due to SFO, the clock at device side is not stable and may be faster or slower compared with ideal clock without SFO. With the increase of the X, the gap between adjacent Msg1 transmissions and the Msg1 transmission length also increases for the Msg1/Msg3 transmitted later in time. An illustration is given in Figure 2.1-1.

Figure .2.1-1. D2R transmission for Msg1 with and without SFO

As shown in Figure 2.1-1, the transmission duration/length of Msg1 is Tmsg1 which should be confined within the time duration of Ts for an access occasion. With SFO= fe, the actual transmission length of Tmsg1 would be in the range of [Tmsg1*1-fe,  Tmsg1*1+fe]. For the ith access occasion, the starting time Ti_start is i*Ts*(1- fe) with a fast clock or i*Ts*(1+fe) with a slow clock, and the ending time Ti_end >= Ti_start+Tmsg1.

Additionally, to ensure no overlapping between the adjacent Msg1 transmissions, the the starting time of the current access occasion with a fast clock should be after the ending time of the previous access occasion with a slow clock, i.e., Ti_start≥Ti-1_end.

- Ti_start≥Ti-1_end≥Ti-1_start+Tmsg1*1+fe                                         (1)

From (1), it is further obtained

- i*Ts*1-fe≥i-1*Ts*1+fe+Tmsg1*1+fe                                (2)

Then the time duration for the ith access occasion is derived as Ts≥Tmsg1*(1+fe)1-2*i*fe+fe (3) and the resource usage efficiency is defined as (1XTmsg1)/(1XTs)  (4).

Apparently, the gap/guard time between adjacent Msg1 transmissions becomes larger with the increase of X due to the time error cumulation. To determine the X values, following options are compared, the evaluation  assumptions are listed in Table 2.1-3.

- Option 1(baseline which is without TDMA): Each D2R transmission for Msg1 is scheduled by a R2D.

- Option 2 (with TDMA and X access occasions with equal time duration): X D2R transmission for Msg1 is scheduled by a R2D and the time duration for the X access occasions is the same, which is determined by the maximum value of X.

  - the guard time between adjacent Msg1 transmission is the maximum to accommodate the last Msg1 transmission.

- Option 3 (with TDMA and X access occasions with unequal time duration): X D2R transmission for Msg1 is scheduled by a R2D and the time duration for the X access occasions is different. It depends on the current access occasion is which one of the X access occasions.

  - the guard time between adjacent Msg1 transmission is the minimum required time gap between previous and current Msg1 transmissions without considering later Msg1 transmissions.

- Option 4 (with TDMA with updating the timing every X1 access occasions): X D2R transmission for Msg1 is scheduled by a R2D and every X1 access occasions, reader sends a R2D preamble to help device re-sync  clock. X1 <X and is determined when the time error cumulation due to SFO exceeds one threshold.

Table 2.1-3: Assumption for comparison of the four options for TDMA of X access occasions

| SFO | 0.1/0.05 |
| --- | --- |
| Transmission time of R2D | 0.5 ms |
| Transmission time duration of Msg1 | 1 ms |
| TD2R/ TR2D | 100us/ 200us |
| X1 for Option 4 | 3 |

The simulation results for the four options are provided in Figure 2.1-2 and Figure 2.1-3 for 10% and 5% SFO respectively, in terms of the required total guard time and the resource usage efficiency which is defined as (1XTmsg1)/(1XTs). In option 4, reader sends a R2D preamble after every 3 D2R transmissions for Msg1.

Figure 2.1-2. The total guard time and the resource usage efficiency varies with X values for 10% SFO

Figure 2.1-2. The total guard time and the resource usage efficiency varies with X values for 5% SFO

Figures 2.1-1 and 2.1-2 show that total guard time for all options increases as the number (X) of time domain resources grows, but at different rates. For option 2, where X access occasions with the same time duration for Msg1 transmission are scheduled by an R2D, the growth rate accelerates because larger X values increase the gap/guard time between Msg1 transmissions. Option 3 improves resource efficiency over option 2 since each transmission occasion has a different duration; later access occasions last longer due to time error accumulation. With a 10% SFO, compared to the baseline option 1 without TDMA, option 2 is more efficient when 1< X 4, option 3 when 1< X 6, and option 4 is always more efficient when X>1. Compared to Option 2, Option 3 and Option 4 always have better resource usage efficiency. While compared between Option 3 and option 4, option 3 is better when X4 and Option 4 is better when X is larger than 4, thanks to the additional R2D preamble to help the device that has not sent Msg1 to adjust its counting samples more accurately.

From figure 2.1-2, it can be observed that the gap/guard time between adjacent D2R transmission for Msg1 is decreased as SFO is decreased form 10% to 5%. For 5% SFO, compared to the baseline option 1 without TDMA, option 2 is more efficient when 1< X 7, option 3 and option 4 is always more efficient compared to option 1. Compared to option 2, option 3 and option 4 always performs better. While compared between Option 3 and option 4, option 3 is better when X7 and Option 4 is better when X is larger than 7.

In addition, it is also observed that for option 1 and option 4, time resource efficiency will eventually tend towards a stable value which will be close to 52% and 60% for 10% SFO, 55% and 70% for 5% SFO thanks to additional R2D transmission to for timing acquisition.

Observation 2.1-1: For 10% SFO,

- Option 2: Total guard time increases from 0.23ms to 31.84ms and the range of resource usage efficiency variation is from 90% to 22% as X increases from 2 to 9.

- Option 3: Total guard time increases from 0.23ms to 13.89ms and the range of resource usage efficiency variation is from 90% to 39% as X increases from 2 to 9.

- Option 4: Total guard time increases from 0.23ms to 5.42ms and the range of resource usage efficiency variation is from 90% to 60% as X increases from 2 to 9.

Observation 2.1-2: For 5% SFO,

- Option 2: Total guard time increases from 0.11ms to 9.84ms and the range of resource usage efficiency variation is from 95% to 48% as X increases from 2 to 9.

- Option 3: Total guard time increases from 0.11ms to 4.89ms and the range of resource usage efficiency variation is from 95% to 65% as X increases from 2 to 9.

- Option 4: Total guard time increases from 0.11ms to 3.29ms and the range of resource usage efficiency variation is from 95% to 70% as X increases from 2 to 9.

Observation 2.1-3: Compared to the option 2, for both 10% and 5% SFO, option 3 and option 4 always achieve better resource usage efficiency.

Observation 2.1-4: Compared to the option 3,

- For 10% SFO, Option 4 can always achieve less total guard time and better resource usage efficiency when X is larger than 4.

- For 5% SFO, Option 4 can always achieve less total guard time and better resource usage efficiency when X is larger than 7.

Based on above observations, following is proposed:

Proposal 2.1-5: The maximum value of X can be determined based on following options:

- Option 2: X is up to [4], where the time duration for the X access occasions including the guard time due to SFO is the same.

- Option 3: X is up to [6], where the time duration for the X access occasions including the guard time due to SFO is different.

- Option 4: X can be any number in theory, but up to [10] considering device complexity, where there is one R2D transmission to facilitate device re-sync after per X1 where 1< X1 <= [3] Msg1 transmissions.

For TDMed and/or FDMed multiple of Msg1 access occasions, due to residual SFO of the device, how to address timing errors for adjacent time domain resources and/or frequency inaccuracy for adjacent frequency domain resources need to be discussed. In our view, it should be up to reader to reserve the sufficient gap between the adjacent D2R transmissions in the time-domain and/or frequence-domain.

Proposal 2.1-6: For TDMA and/or FDMA of multiple access occasions for D2R transmission(s), it is up to reader to reserve the sufficient gap between the adjacent D2R transmissions in the time and/or frequency domain due to residual SFO of the device.

  - Msg2 Response to Msg1

The following agreements was made from RAN1#118 to 119 meetings for Msg2 monitoring and reception.

Agreement@RAN1#118 When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices Agreement@RAN1#118bis RAN1 studies the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices Agreement@RAN1#119 For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies the following for the starting time for Msg2 monitoring - For Option 1, where a PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device, - Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource. - Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources. - For Option 2, where a PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources. Agreement@RAN1#119 For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options for a device to determine the reference time for the starting time for Msg2 monitoring - Option 1: End of time domain resource where the device transmitted the Msg1 - Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s) - Option 3: End of the R2D transmission triggering random access - Option 4: End of a R2D transmission carrying Msg2 - Note the reference time is used to determine the starting time for Msg2 monitoring, it does not mean that the starting time always equals the reference time. Agreement@RAN1#119 For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options to determine the time duration for Msg2 monitoring - Option 1: Predefined - Option 2: Indicated in the R2D transmission triggering random access - Note: Option 1 and Option 2 may not be mutually exclusive.

    - A-IoT Msg2 format(s): one Msg2 for one and/or multiple Msg1

For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, two options were studied. Option 1 is one Msg2 for one Msg1 and Option 2 is one Msg2 for multiple Msg1. The decision on option 1 and/or option 2 would have impacts on the Msg2 format(s) design and the device behaviour on Msg2 monitoring. In our view, both options can be supported and which option to use for Msg2 transmission should be decided by the reader. While the flexibility provided to the reader should also take into device’s monitoring complexity and power consumption account. Therefore, the Msg2 format should be designed or Msg2 content should include the information to help device to identify the number of echoed Msg1 responses contained in present Msg2.

Proposal 2.2.1: For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, support a PRDCH for Msg2 transmission corresponds to one or multiple A-IoT Msg1 received from one device or different devices.

- From the RAN1 perspective, Msg2 formats and/or contents should be designed to minimize device monitoring complexity and power consumption.

    - Starting time for A-IoT Msg2 monitoring

- Separate or Common starting time for Msg 2 monitoring

About the starting time for Msg 2 monitoring, considering reader can decide the number of responses to Msg1 the Msg2 carriers, there is a case that the reader sends multiple Msg2 in sequence with a time interval between adjacent Msg2 and each Msg2 contains one and/or more than one responses to Msg1, see Figure 2.2 below.

Figure 2.2: Illustration for back-to back PRDCH for Msg2 transmissions

Two options i.e., separate or common starting time for Msg2 monitoring can be considered.

Separate starting time is beneficial to reduce device power consumption, while it may introduce additional overhead in the R2D triggering the random access and/or specification efforts. For example, the association rule between the Msg1 resource and starting time for Msg2 monitoring, reference time for the starting time for Msg2 monitoring need to be defined. In addition, separate starting time also requires pre-scheduling before reader successfully receives the actual Msg1 transmissions, in which case, the time duration reserved for Msg2 may be wasted as shown in Figure 2.2.

Common starting time is simple at the cost of device power consumption. However, the power consumption may not be an issue for device 1 and it only happens to the unlucky device that cannot identify its own response that monitors the Msg2 until the end of the Msg2 monitoring window. Compared to the complex separate starting time, some simple enhancement can be considered, e.g., Msg2 can inform the device whether this is the last Msg2 transmission in the Msg2 monitoring window. Therefore, common starting time is preferred.

Proposal 2.2.2-1: For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

- FFS enhancement to reduce device power consumption for Msg2 monitoring.

Reference time for the starting time for Msg2 monitoring

For a device to determine the reference time for the starting time for Msg2 monitoring, there are four options.

•	Option 1: End of time domain resource where the device transmitted the Msg1

•	Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s)

•	Option 3: End of the R2D transmission triggering random access

•	Option 4: End of a R2D transmission carrying Msg2

In general, for Msg1 transmission with or without FDMA/TDMA, or for Msg2 contains one or multiple responses for Msg1 with TDMA and/or FDMA, a unified definition for the reference time is preferred for the starting time for Msg2 monitoring. Therefore, Option 4 is not preferred since it works only for the case where Msg1 transmission with FDMA and/or TDMA and more than one Msg2 would be transmitted to response the received multiple    Msg1 transmission. Then for Option 4, the starting time for Msg2 monitoring is separate, for example, the reference time for the subsequent Msg2 monitoring except the 1st Msg2 is the end of the previous Msg2. While the reference time for the starting time for the 1st Msg2 still needs to select from above option 1, 2 or 3. The benefit for option 4 was claimed as power saving for Msg2 monitoring, while it is not clear how much power it can be saved since each device still needs to monitor previous Msg2 to get the information about the ending time of the previous Msg2.

For Option 1 that the reference time is defined as the end of time domain resource where the device transmitted the Msg1, then it cannot directly be used for the common starting time for Msg2 monitoring. Additional signalling is needed to inform each device about the time interval between the reference time and the starting time for Msg2 monitoring, which results in large overhead in the R2D transmission triggering the random access.

Both Option 2 and Option 3 can work well for the Msg2 monitoring with a common starting time without additional signalling, e.g., the time interval between the reference time and the starting time for Msg2 monitoring can be defined in the specification. Even if the time interval between the reference time and the starting time for Msg2 monitoring needs to be indicated in the R2D triggering random access, both options can work and option 2 can have smaller overhead compared to option 3. In addition, it was agreed in RAN1#118 to study to define a minimum time TD2R_min and a maximum time TD2R_max between the D2R transmission for A-IoT Msg1 and the corresponding R2D transmission for A-IoT Msg2 following, so that the Msg2 transmission timing is expected to be within [TD2R_min, TD2R_max]. From this perspective, option 2 seems more aligned with such definition.

Observation 2.2.2-1: Option 4 is not a unified solution, and option 1 causes either high signaling overhead or increased device power consumption, making it less preferred.

Observation 2.2.2-2: both option 2 and option 3 work well. Option 2 has smaller overhead and more aligned with definition

Proposal 2.2.2-2: For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, support option 2 that the reference time for determination of the starting time for Msg2 monitoring is the end of the last of X time domain resource(s) for Msg1 transmission(s), where X1.

    - Time duration for A-IoT Msg2 monitoring

About the time duration for A-IoT Msg2 monitoring, it is preferred to be predefined in the specification to reduce the R2D overhead. As discussed in section 2.2.2, although the starting time for Msg2 is common for all devices that transmit the Msg1 and the time duration is also common and predefined in the specification, early terminating of Msg2 monitoring can be considered to help device reduce the power consumption.

Proposal 2.2.2-3: For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, the time duration for Msg2 monitoring is predefined.

- FFS the number and value(s) of the time duration.

  - Resource allocation for Msg3

Following agreement was made for Msg3 transmission:

Agreement@RAN1#118bis Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following - How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3

Different from Msg1 transmission which is contention-based transmission, Msg3 transmission can be treated as contention-free or scheduling-based transmission. Because reader already identified the device during the contention resolution process in Msg2. Therefore, the time and/or frequency domain resource that different from Msg1 can be assigned to each device for Msg3 transmission. To reduce the PRDCH overhead for Msg2 transmission, following options were proposed during SI:

- Option 1: Msg3 uses the same time resource and frequency resource as Msg1

- Option 2: Msg3 uses the same frequency resource as Msg1, the time resource for Msg3 is provided in the corresponding Msg2.

- Option 3: Msg3 uses the same time resource as Msg1, the frequency resource for Msg3 is provided in the corresponding Msg2.

- None of three options above works reliably and efficiently for Msg3 transmission. Since some of the allocated time and frequency domain resource for Msg1 transmission may not be used or has collisions, which cannot be further used by Msg3. For option 1 and option 3, considering large TBS of Msg3 compared to Msg1, the data rate would be increased if same time domain resource for Msg1 is used, which may result in Msg3 decoding failure. Therefore, following is proposed.

Proposal 2.3: The time and frequency domain resource for Msg3 transmission is indicated by the corresponding PRDCH for Msg2 transmission.

- Scheduling related aspects

  - Scheduling information for R2D

According to the WID, R2D exclusively supports TDMA for multiplexing and OOK-4 modulation. It only uses the Manchester line code and does not support FEC or physical-layer repetition transmission. Additionally, for PRDCH reception, following agreements made during the SI should be considered as the starting point.

Agreement@116bis To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information. Agreement@117 For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration - FFS: Pattern design to support determination of chip duration Agreement@118 For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) - FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

Given above, our views on the necessary information for PRDCH reception is following:

- PRDCH chip length

- PRDCH payload size e.g., TBS-like information

- Device ID(s)

For Device ID, RAN2 studied the following cases on the identifier that may be required to identify the device/group of devices in the A-IoT paging message (e.g., for the case of reaching a single or a group of devices):

- Case 1: The A-IoT paging message containing an identifier of a single A-IoT device

- Case 2: The A-IoT paging message containing a group ID that maps to multiple A-IoT devices

- Case 3: The A-IoT paging message that does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond

- Case 4: The A-IoT paging message containing multiple identifiers of A-IoT devices. The need for this use case is still to be confirmed/dependent according to the conclusion in 3GPP TR 23.700. From RAN2 perspective, it is feasible to support paging multiple identifiers of A-IoT devices, pending on TB size and multiplexing design of A-IoT paging message.

Therefore, Device ID related information should be better to be determined by RAN2 in coordination with SA2 on the use cases. From RAN1 perspective, device ID related information is better to be located in the front of the PRDCH, so that the device can determine whether it is the target device for reception of the R2D transmission.

Proposal 3.1-1: Device ID related information should be better to be determined by RAN2 in coordination with SA2 on the use cases. From RAN1 perspective, prefer to be placed in front.

For PRDCH chip length indication, it is straightforward and sufficient to use the clock-acquisition part of R2D timing acquisition signal (e.g., R2D preamble) preceding the PRDCH to indicate the information of chip length used for the following PRDCH transmission.

Proposal 3.1-2: For R2D, the clock-acquisition part of the R2D time acquisition signal preceding the PRDCH is used to determine the OOK chip duration used for the following PRDCH transmission.

For PRDCH payload size, e.g., TBS-like information, we provide our views for option 1 of R2D postamble and option 2 of R2D control information.

The main benefit of using postamble i.e., Option 1 to identify the end of R2D transmission is to achieve the most variable and flexible R2D TBS, e.g., the payload size can be as fine as 1-bit granularity. However, for the intended Rel-19 indoor inventory and command use cases, such flexibility is not necessary given the following reasons. For inventory and command use cases, it is expected that “command code”-like information will be defined for the R2D transmissions. For some command codes, the PRDCH TBS can be fixed, which means by the command code, the device can identify the PRDCH TBS. For some command codes, the PRDCH TBS can be variable, while the TBS can still be determined by “length”-like information similar as RFID “Length field” in Select/QueryX command or “WordCount” field in Read command or similar as NR “L” the Length field in the MAC subheader indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. In addition, based on following analysis, option 1 of R2D postamble either complicates/prolongs the device decoding PRDCH process or being unnecessary.

When receiving and decoding PRDCH at the device side,

- If the size of the PRDCH transmission block (TBS) is known by R2D control information, the device can directly input the received data and CRC into the CRC decoder with the appropriate CRC bit length for verification. Data integrity is confirmed when the output from the CRC register matches the known bit sequence.

- If the device uses R2D postamble to determine the TBS, it needs to first buffer all bits received before the postamble if it has one CRC decoder working at one time. Next, the device determines the CRC length. Finally, it inputs the data and CRC into the CRC decoder with the correct bit length for verification. This method complicates and prolongs the decoding process.

- Alternatively, the device can utilize parallel CRC decoders with different bit length (e.g., 6-bit and 16-bit CRC), eliminating the need to buffer all data and CRC bits. The device can continuously input received bits into both CRC decoders and monitor their outputs. When a matching bit sequence is detected by one of the CRC decoders, data integrity is confirmed. Consequently, a R2D postamble becomes redundant, as the device only needs to verify the output of the CRC decoders to identify the matching sequence.For R2D data size e.g., TBS24 bits using the 16-bit CRC, the error for early termination of the data decoding wronly is only 1/216; while for R2D data size e.g., TBS<24 bits using the e.g., 6-bit CRC, the error for early termination of the data decoding wrongly is 1/26, around 1.6% which is small for 10% BLER target.

Therefore, if R2D postamble serves the purpose of the PRDCH TBS indication, it is not necessary and option 2 offers a more unified and efficient solution by defining the command code and/or length-like information/fields of the R2D control information.

Proposal 3.1-3: The end of PRDCH transmission that is determined or derived by the R2D control information.

  - Scheduling information for D2R

According to the WID, D2R supports only TDMA and FDMA for multiplexing, supports only OOK and BPSK modulations for backscattering, and supports either Manchester line code or no line code (one to be down-selected), only convolutional code with generator polynomials, D2R supports physical layer repetition transmission. Additionally, for PDRCH transmission, following agreements made during the SI should be considered as the starting point.

Agreement@RAN1#116bis For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information Agreement@RAN1#118 For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

Given above, we provide our views on the necessary information for PDRCH transmission in the following Table based on above agreement:

Table 1: Necessary information for PDRCH transmission

| Scheduling information | Contents | Our Views |
| --- | --- | --- |
| ID associated with device(s) | - A single ID or a group ID - No any ID, i.e., indicating all A-IoT devices - Multiple device ID(s) | - Same as for PRDCH reception, it is up to RAN2 and SA2 to decide. |
| Time domain resources | - Number of TDMA resources, e.g, X - Starting time for each resource - [the ending time or the transmission length] | - 1X[6], up to [2 or 3]-bit indication is sufficient if codepoint is used or [X]-bit indication is sufficient if bitmap is used. - Starting time for each resource, option 2 that indication based for D2R transmission timing TR2D in the R2D control information is preferred as unified solution. The bit length can be FFS after the processing time needed for A-IoT operation is decided. - About the ending time or the transmission length for each resource, no specific indication is needed, it is derived by PDRCH TBS, chip or information bit duration, small frequency shift, repetition number etc.. |
| Chip duration | - D2R chip duration | The chip duration can be selected from a set of predefined values, considering the supported minimum and maximum data rate, and the frequency shift magnitude |
| Frequency domain resources | - Number of FDMA resources, e.g., Y - Small frequency shift, e.g., R, a codeword repetition number for Manchester line code Option1 or the number of square wave periods for Manchester line code Option2 or no line codes | - Either Option 2 of One joint indication of Y and R or Option 3 of One indication field in the form of bitmap for the set of candidate R values is used for indicating the Y and frequency domain resources - The candidate set of R values can be found in our companion contribution [4] |
| Information related to MCS | - [OOK and BPSK] - [Manchester line code or no line code] - With or without convolutional code | - For OOK and BPSK, which one to use can be left to device implementation, no need of explicit indication - No need to indicate the code scheme since Manchester line code or no line code will be down-selected - May need 1-bit indication on whether to use FEC |
| Information related to Repetitions | - Repetition type - Block-level repetition - Bit-level repetition type 1 and type 2 - Number of repetitions | - Support only block repetition [3], hence no indication on the repetition type is needed - Indication on the number of repetition is needed |
| Other information | - PDRCH TBS - D2R midamble | - PDRCH TBS is indicated by R2D control information. - FFS the bit length for PDRCH TBS indication, considering the potential supported minimum and maximum D2R TBS - D2R midamble, if agreed to be supported, FFS indication at least the density and location(s) [6] |

Proposal 3.2-1: Take Table 1 as starting point for discussing the necessary information for PDRCH transmission, including the signaling details and bit length for the identified necessary information that need to be indicated by R2D control information.

For indication of the PDRCH payload size, using D2R postamble implies that the A-IoT device should decide the PDRCH TBS. Then it is difficult for the Reader to make scheduling decisions and control the resource usage. For using control information to indicate the end of PDRCH transmission, there can be three cases:

- Case 1: the control information is R2D control information with implicit PDRCH TBS indication.

- Case 2: the control information is R2D control information with explicit PDRCH TBS indication.

- Case 3: the control information is D2R control information, which may serve some special purposes such as error reply in case the device encounters an error.

From RAN1 perspective, Case 1 and Case 2 are obvious. For example, during the contention based random access, without indicate the PDRCH TBS, the device knows the TBS for Msg1 transmission is 16 bits by detection of A-IoT paging. Case 2 can be used for PDRCH transmission with variable TBS. It may be beneficial to support Case 3, while this should be further determined by RAN2 for the procedure of A-IoT data transmission, including data (re-)transmission for failure handling.

Proposal 3.2-2: Reader acquires the end of PDRCH transmission based on R2D control information with explicit or implicit TBS indication.

About the candidate values of the TBS indication, it needs to be discussed for both PRDCH and PDRCH. It depends on the supported minimum and maximum TBS and can be decided later.

  - Higher-layer signaling and/or L1 R2D control signaling

For transmission of R2D control information for R2D reception and D2R scheduling, following two options are studied and captured in the TR 38.769:

- Option 1: L1-control signaling

- Option 2: Higher-layer signaling

About the formats/container for transmission of R2D control information for R2D reception and D2R scheduling, our views are following

- No separate CRC is needed for R2D control information since it is meaningless to have different reliability for R2D control information and data

- R2D control information is better to be placed in front of the corresponding data to enable early identification of target devices and obtain the necessary information required for data reception/decoding and transmission/preparation

- To keep the R2D control information overhead small, the bit string carried by R2D control information is no need to be byte aligned (i.e. multiple of 8 bits) in length, especially when the PRDCH carries only R2D control information for PDRCH transmission.

As long as the formats/container can be designed to meet above points, either L1-control signaling and higher layer signaling can be used to transmit the R2D control information for R2D reception and D2R scheduling, as there is no fundamental difference between the 2 options. Unified container (either L1-control or higher layer signaling) to carry the R2D control information should be strived for simplicity and efficiency. It is not preferred to discuss case by case which field/parameter should be carried by L1-control signaling or by higher layer signaling, considering the tight time schedule and specification efforts.

Proposal 3.3-1: Support the following for R2D control information:

- A separate CRC is not required.

- R2D control information should be placed in the earlier part of the PRDCH.

- The bit string carried by R2D control information does not need to be byte-aligned (i.e., a multiple of 8 bits) in length.

Proposal 3.3-2: For R2D control information, it is not preferred to discuss case by case which field/parameter should be carried by L1-control signaling or by higher layer signaling. Unified container (either L1-control or higher layer signaling) is used to carry the R2D control information.

- Timing relationships

During SI, following agreements were made for A-IoT scheduling and timing relations:

Agreement@RAN1#116 For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device. - The study should consider at least following aspects - Implementation restrictions for the existing BS/UE - [Processing time is common or different for different A-IoT devices] - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)] - FFS other timing aspects Agreement@RAN1#118bis - The start indicator part of the R2D time acquisition signal is not included in TD2R_min.

The exact time point for the timing relations, for example the time interval between “the end” of the R2D transmission and “the start” of the corresponding D2R transmission following it and vice versa should be discussed. Our views are present as follows:

For the time interval between a R2D transmission and the corresponding D2R transmission following it, including TR2D_min, TR2D_max and/or TR2D, “the end of the R2D transmission” and “the start of the corresponding D2R transmission” should be defined.

For the end of the R2D transmission, following options can be considered:

- Option 1: the end of the R2D transmission is the last chip of the R2D transmission

- Option 2: the end of the R2D transmission is the last OFDM symbol of the R2D transmission

Above a R2D transmission also needs discussion on whether it includes a R2D postamble that depends on the decision on whether and how a R2D postamble is supported e.g., always included in a PRDCH or only included in a PRDCH under some conditions. This point can be further clarified once the decision is made for R2D postamble. While regardless of whether/how to support R2D postamble, the R2D waveform generation uses OFDM-based transmitter, the end of the R2D transmission should be aligned with the last OFDM symbol boundary. In RAN1#118bis meeting, it was agreed to at least use padding when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol from the transmitter perspective. From the receiver i.e., device perspective, actually the most CP handling methods that works for both small and large values of M, the OFDM symbol boundary should be identified by the device [5]. Therefore, Option 2 can also be considered for determining “the end” of the R2D transmission. Compared to option 1, option 2 may increase a bit of delay for D2R transmission if many padding chips are used for the last OFDM symbol.

For the start of the corresponding D2R transmission, it is reasonable to define it as the first chip for the D2R preamble.

Proposal 4-1: For the time interval between a R2D transmission and the corresponding D2R transmission following it,

- “the end of the R2D transmission” is defined by one of the following options

  - Option 1: the last chip of the R2D transmission

  - Option 2: the last OFDM symbol of the R2D transmission

- Note that whether the R2D transmission includes a R2D postamble can be determined after the decision on the whether/how to support the R2D postamble is made.

- “the start of the corresponding D2R transmission” is defined as the first chip of the D2R preamble.

For the time interval between a D2R transmission and the corresponding R2D transmission following it, including TD2R_min, TD2R_max , “the end of the D2R transmission” and “the start of the corresponding R2D transmission” should be defined.

For the end of the D2R transmission, it should be defined as the last chip of the D2R transmission. Similar as for the R2D transmission, whether the D2R transmission includes a D2R postamble can be determined after the decision on the whether/how to support the D2R postamble is made.

For the start of the corresponding R2D transmission, since the OFDM-based waveform is used, it was agreed in RAN1#116bis meeting that the start of R2D transmission from reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP), it means that the start, the first chip of the R2D preamble should be aligned with boundary of the first OFDM symbol for the R2D transmission. The exact time point for the start of the R2D transmission is the same regardless of the first OFDM symbol or the first chip for the R2D preamble is used. Given above, following proposal is made:

Proposal 4-2: For the time interval between a D2R transmission and the corresponding R2D transmission following it,

- “the end of the D2R transmission” is defined as the last chip of the D2R transmission

  - Note that whether the D2R transmission includes a D2R postamble can be determined after the decision on the whether/how to support the D2R postamble is made.

- “the start of the corresponding R2D transmission” is defined as the first chip or the first OFDM symbol of the R2D preamble.

  - Time unit for A-IoT timing relations

About whether and how to define the time unit for A-IoT operation, let’s first take NR as an example.

In NR,

- The basic timing unit Tc is defined to be the shortest/smallest sampling time corresponding to the widest subcarrier spacing in Rel-15 NR specification, i.e., Tc = 1/(fmax∙Nf), where fmax=480KHz, Nf=4096.

- For time-domain resource allocation, a.k.a SLIV, basically the smallest time unit is 1 OFDM symbol for PDCCH/CSI-RS/PUSCH/PUCCH/SRS while is 2 OFDM symbols for PDSCH. The SLIV and MCS is used to calculate the TBS.

- For timing indication such as k0, k1 and k2, basically the smallest time unit is 1 slot.

In RFID,

- The basic or reference time interval for a data-0 in Interrogator-to-Tag signaling is Tari, with the value range of 6.25 µs to 25 µs.

- In time-domain, for Interrogator-to-Tag link, a data-0 symbol using Tari and a data-1 symbol length is (RTcal-Tari), where the RTcal is 2.5 Tari  RTcal  3.0 Tari equals to the length of a data-0 symbol plus the length of a data-1 symbol. For the Tag-to-Interrogator link, the Interrogator determines the Tag’s BLF using the TRcal with the value range of 1.1 RTcal  TRcal  3 RTcal and divide ratio (DR) carried in payload of Query command. Unlike NR, the TBS for RFID is indicated implicitly by the command code or explicitly by the Length (L) field in the Interrogator-to-Tag command.

- For the link timings, the time unit for defining the time window is based on RTcal and Tpri, where Tpri denotes either the commanded period of an FM0 symbol or the commanded period of a single subcarrier cycle, as shown in Table 6-16 of [4] (copy below).

Table 6-16: Link timing parameters (in RFID C1G2 Standard)

| Para. | Minimum | Nominal | Maximum | Description |
| --- | --- | --- | --- | --- |
| T1 | MAX(RTcal,10Tpri) × (1 – ¦FrT¦) – 2μs | MAX(RTcal,10Tpri) | MAX(RTcal,10Tpri) × (1 + ¦FrT¦) + 2μs | Immediate reply time from Interrogator transmission to Tag reply. Specifically, the time from the last rising edge of the last bit of the Interrogator transmission to the first rising edge of the Tag reply for an immediate Tag reply, measured at the Tag’s antenna terminals. |
| T2 | 3.0Tpri |  | 20.0Tpri | Interrogator reply time if a Tag is to demodulate the Interrogator signal, measured from the end of the last (dummy) bit of the Tag reply to the first falling edge of the Interrogator transmission |
| T3 | 0.0Tpri |  |  | Time an Interrogator waits, after T1, before it issues another command |
| T4 | 2.0 RTcal |  | 20ms | Minimum time between Interrogator commands |
| T5 | MAX(RTcal,10Tpri) × (1 – ¦FrT¦) – 2μs |  | 20ms | Delayed reply time from Interrogator transmission to Tag reply. Specifically, the time from the last rising edge of the last bit of the Interrogator transmission to the first rising edge of the Tag reply for a delayed Tag reply, measured at the Tag’s antenna terminals. |
| T6 | MAX(RTcal,10Tpri) × (1 – ¦FrT¦) – 2μs |  | 20ms | In-process reply time from Interrogator transmission to the first Tag reply. Specifically, the time from the last rising edge of the last bit of the Interrogator transmission to the first rising edge of the first Tag reply indicating that the Tag is either (a) still working, or (b) is done, measured at the Tag’s antenna terminals |
| T7 | MAX(250μs, T2(max)) |  | 20ms | In-process reply time between Tag replies. Specifically, the time from the end of the last (dummy) bit of the Tag’s prior transmission indicating that the Tag is still working to the first rising edge of the current Tag reply indicating that the Tag is either (a) still working, or (b) is done, measured at the Tag’s antenna terminals |
| FrT is the frequency tolerance specified in Table 6-9. |  |  |  |  |

Then, regarding the time unit for A-IoT operation, following concept can be discussed.

The basic time unit can be defined to be the shortest/smallest chip length that A-IoT system supports, e.g., minimum (Smallest R2D chip length, Smallest D2R chip length) assuming R2D chip length and D2R chip length have an integer multiple relationship.

Regarding to the unit for time-domain resource allocation, it seems not necessary for A-IoT operation. In NR, the time-domain resource allocation (SLIV) is needed for calculating the TBS. While unlike NR, the device does not need to calculate the R2D and D2R TBS, the TBS is directly indicated by the R2D command in implicit or explicit manner.

Observation 4.1: The time unit for time domain resource allocation is not needed for A-IoT operation, since the R2D and D2R TBS is directly given by the R2D transmission in explicit or implicit manner.

For timing relations, following options can be considered. Note that similar to RFID, where both RTcal and Tpri determine link timings (see Table 6-16), different options or a combination of these options for a R2D reply time (TD2R_min, TD2R_max , the time interval between a D2R and the corresponding R2D following it) and a D2R reply time (TR2D_min, TR2D_max and/or TR2D, the time interval between a R2D and the corresponding D2R following it) are also viable.

- Option 1-a: the time duration corresponding to a R2D chip used for PRDCH that schedules the PDRCH.

- Option 1-b: the time duration corresponding to a D2R chip used for PDRCH transmission.

- Option 2-a: the time duration corresponding to a R2D information bit that schedules the PDRCH.

- Option 2-b: the time duration corresponding to a D2R information bit, Tb for D2R.

- Option 3: the basic chip length that is defined as the shortest/smallest chip length supported by A-IoT system.

- Option 4: the time duration corresponding to a OFDM symbol, e.g., 71.43us.

For a D2R reply time (TR2D_min, TR2D_max and/or TR2D, the time interval between a R2D and the corresponding D2R following it) that is related to the device’s processing time including the time used for decoding the PRDCH the preparing the PDRCH, similar as RFID, the combination of Option 1-a and Option 1-b can be considered and preferred.

For a R2D reply time (TD2R_min, TD2R_max , the time interval between a D2R and the corresponding R2D following it) that is related to the reader’s processing time, some companies proposed to consider e.g. ms-level time considering the impact on the existing BS implementation. While to be competitive with RFID, OFDM symbol level i.e., Option 4 can also be considered.

Proposal 4.1: Consider defining following time unit for A-IoT operation.

- Basic time unit: the shortest/smallest chip length that A-IoT system supports, e.g., Minimum (Smallest R2D chip length, Smallest D2R chip length).

- Unit for timing relations: the time duration used to determine at least the time interval between the R2D and corresponding D2R following it, the D2R and corresponding R2D following it.

  - For D2R response to a R2D

Two options for the time interval between a R2D transmission and corresponding D2R response was studied in SI.

Considering the support of the TDMA for D2R transmissions, option 2 that timing indication by the R2D transmission is preferred as we proposed in Proposal 2.1-4.

Regardless option 1 and/or option 2 is/are selected, the remaining issues for the timing of D2R response to R2D are the following

- Whether/how to address timing error for a D2R transmission caused by the residual SFO of the scheduled device

- Determination of the minimum value(s) of TR2D_min and the maximum value(s) of TR2D_max or TR2D, including same or different minimum and maximum value(s) is/are defined for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)

On the timing error (initial/residual SFO) handling for a D2R transmission, similar as for addressing the timing and/or frequency errors for adjacent time and/or frequency domain resources, it should be handled by the reader and no RAN1 specification impacts. In detail

- For Option 1, the device is required to transmit the D2R signal within its own clock-based time range [TR2D_min, TR2D_max] and the Reader starts detecting the D2R signal no later than the time TR2D_min*(1-clock error)

- For Option 2, the device is required to transmit the D2R signal based on its own clock-based time TR2D and TR2D*(1-clock error) should be larger than or equal to TR2D_min.

About the processing time for different traffic types/command types (e.g., DT or DO-DTT) and/or different use case (e.g., Inventory or Command), we think different processing time for different command types may be needed since different commands may result in different processing time. For example, in RFID, both immediate reply time and delayed reply time are defined. Because for executing the commands like Write, Lock, BlockErase etc, the tag requires more time. For A-IoT inventory and command use cases, different commands may also require different minimum time for processing. Therefore, following is proposed:

Proposal 4.2: Discuss whether it is necessary to define different minimum processing time for different commands, for example, whether device requires more processing time for certain time-consuming commands.

  - For R2D response to D2R

In RAN1#118 meeting, the following agreement was made for R2D response timing, more specifically Msg2 to a Msg1 transmission.

Agreement When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices

As noticed in the following RAN2 agreement in RAN2#127bis, the expected R2D response for a A-IoT device is not only for Msg1, it is also expected for Msg3 for re-access purpose.

RAN2 agreement @RAN2#127bis meeting 2. Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data.

Therefore, we propose to extend the above agreement to cover all general cases where the feedback from R2D is expected for a device. It is beneficial to define the maximum R2D response time that can be used by device to decide the next action in case the expected response is not received within the maximum time.

Proposal 4.3: For the case(s) where a R2D transmission in response to a D2R transmission is expected, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max].

About the minimum and maximum value for a R2D reply time (TD2R_min, TD2R_max), further discussion is needed and it is desirable to have short reply time while with less BS implementation impacts.

  - Others

About the time between two different consecutive R2D transmissions to the same A-IoT device, the terminology of TR2D_R2D_min is defined as the minimum time between two different consecutive R2D transmissions to the same A-IoT device, we think this time interval is needed in case the R2D transmission for selection of the target devices meeting some criterion and the R2D transmission for triggering those selected devices to join the inventory procedure or group-type command operation. While it is not yet clear whether it is necessary to define a maximum time between two different consecutive R2D transmissions to the same A-IoT device.

Proposal 4.4-1: the minimum time between two different consecutive R2D transmissions to the same A-IoT device needs to be defined.

- No need to define the maximum time between two different consecutive R2D transmissions to the same A-IoT device unless the necessity is identified.

About the time between two different consecutive D2R transmissions from the same A-IoT device, there may be a need for following use cases

- Use case 1. Similar as in-process reply in RFID

- For in-process reply, a device may need more time to execute a command, and to notify the reader on a periodic basis that it is still processing the command.

- Use case 2. Different D2R segments for a higher layer data

Use case 1 is specified in RFID that allows a Tag to spend longer than T5(max) (Table 6-16) executing a command such as authenticated kill command, and to notify the Interrogator on a periodic basis that it is still processing the command. An in-process reply may include multiple backscatter transmissions from Tag to Interrogator. The first transmission shall meet the T6 limits specified in Table 6-16; subsequent transmissions (if any) shall meet T7. A Tag shall backscatter a transmission at least once every T7(max) while processing the command.

Use case 2 is related to D2R segmentation which will be supported based on the WID. Therefore, discussion is needed on whether there is a need to specify the time gap between consecutive D2R segments to facilitate reader and/or device side processing.

Proposal 4.4-2: Discuss whether to define the the minimum time between two consecutive D2R transmissions from the same A-IoT device, considering following use cases.

- Use case 1. Similar as in-process reply in RFID

- Use case 2. Different D2R segments for a higher layer data

- Conclusion

In this contribution, we provide our views on other aspects including resource allocation for multiplexing/multiple access, scheduling information and timing relationships for A-IoT operation. The observations and proposals are summarized as follows.

For Multiplex/Multiple access

Observation 2.1-1: For 10% SFO,

- Option 2: Total guard time increases from 0.23ms to 31.84ms and the range of resource usage efficiency variation is from 90% to 22% as X increases from 2 to 9.

- Option 3: Total guard time increases from 0.23ms to 13.89ms and the range of resource usage efficiency variation is from 90% to 39% as X increases from 2 to 9.

- Option 4: Total guard time increases from 0.23ms to 5.42ms and the range of resource usage efficiency variation is from 90% to 60% as X increases from 2 to 9.

Observation 2.1-2: For 5% SFO,

- Option 2: Total guard time increases from 0.11ms to 9.84ms and the range of resource usage efficiency variation is from 95% to 48% as X increases from 2 to 9.

- Option 3: Total guard time increases from 0.11ms to 4.89ms and the range of resource usage efficiency variation is from 95% to 65% as X increases from 2 to 9.

- Option 4: Total guard time increases from 0.11ms to 3.29ms and the range of resource usage efficiency variation is from 95% to 70% as X increases from 2 to 9.

Observation 2.1-3: Compared to the option 2, for both 10% and 5% SFO, option 3 and option 4 always achieve better resource usage efficiency.

Observation 2.1-4: Compared to the option 3,

- For 10% SFO, Option 4 can always achieve less total guard time and better resource usage efficiency when X is larger than 4.

- For 5% SFO, Option 4 can always achieve less total guard time and better resource usage efficiency when X is larger than 7.

Observation 2.2.2-1: Option 4 is not a unified solution, and option 1 causes either high signaling overhead or increased device power consumption, making it less preferred.

Observation 2.2.2-2: both option 2 and option 3 work well. Option 2 has smaller overhead and more aligned with definition

Observation 4.1: The time unit for time domain resource allocation is not needed for A-IoT operation, since the R2D and D2R TBS is directly given by the R2D transmission in explicit or implicit manner.

Proposal 2.1-1: For Y (Y1) frequency domain resource(s) for D2R transmission(s) for Msg1 that determined by a R2D transmission triggering random access,

- the frequency location is determined by the parameter related to small frequency shift, i.e., R and chip/information bit duration

- No need to indicate the transmission bandwidth

Proposal 2.1-2: For indicating the frequency domain locations for FDMA of Y where Y1 access occasions, following Option 2 and 3 can be down-selected considering R2D overhead and the flexibility for FDMed access occasion(s) indication.

- Option 2: One joint indication of Y and R, that the specification can define the association between the Y and R, based on the number of associations, the field length for the joint indication can be determined.

- Option 3: One indication field in the form of bitmap for the set of candidate R values only, e.g., {Ri}, where each Ri value is associated with one bit location, the bit set to 1 indicates that the Ri is used/present; the bit set to 0 indicates that the Ri is NOT used/absent for frequency domain resource allocation.

Proposal 2.1-3: From RAN1 perspective, for CBRA and for FDMA case, the device randomly selects one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader.

Proposal 2.1-4: For X (X1) time domain resource(s) for D2R transmission(s) for Msg1 that determined by a R2D transmission triggering random access,

- the starting time TR2D is indicated by the control information in the R2D transmission, where TR2D  TR2D_min (i.e., Option 2)

- No need to indicate the time length for PDRCH transmission. It is derived by the PDRCH TBS, D2R information bit duration, small frequency shift, the number of repetitions and FEC if used.

Proposal 2.1-5: The maximum value of X can be determined based on following options:

- Option 2: X is up to [4], where the time duration for the X access occasions including the guard time due to SFO is the same.

- Option 3: X is up to [6], where the time duration for the X access occasions including the guard time due to SFO is different.

- Option 4: X can be any number in theory, but up to [10] considering device complexity, where there is one R2D transmission to facilitate device re-sync after per X1 where 1< X1 <= [3] Msg1 transmissions.

Proposal 2.1-6: For TDMA and/or FDMA of multiple access occasions for D2R transmission(s), it is up to reader to reserve the sufficient gap between the adjacent D2R transmissions in the time and/or frequency domain due to residual SFO of the device.

Proposal 2.2.1: For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, support a PRDCH for Msg2 transmission corresponds to one or multiple A-IoT Msg1 received from one device or different devices.

- From the RAN1 perspective, Msg2 formats and/or contents should be designed to minimize device monitoring complexity and power consumption.

Proposal 2.2.2-1: For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

- FFS enhancement to reduce device power consumption for Msg2 monitoring.

Proposal 2.2.2-2: For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, support option 2 that the reference time for determination of the starting time for Msg2 monitoring is the end of the last of X time domain resource(s) for Msg1 transmission(s), where X1.

Proposal 2.2.2-3: For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, the time duration for Msg2 monitoring is predefined.

- FFS the number and value(s) of the time duration.

Proposal 2.3: The time and frequency domain resource for Msg3 transmission is indicated by the corresponding PRDCH for Msg2 transmission.

For Scheduling related aspects

Proposal 3.1-1: Device ID related information should be better to be determined by RAN2 in coordination with SA2 on the use cases. From RAN1 perspective, prefer to be placed in front.

Proposal 3.1-2: For R2D, the clock-acquisition part of the R2D time acquisition signal preceding the PRDCH is used to determine the OOK chip duration used for the following PRDCH transmission.

Proposal 3.1-3: The end of PRDCH transmission that is determined or derived by the R2D control information.

Proposal 3.2-1: Take Table 1 as starting point for discussing the necessary information for PDRCH transmission, including the signaling details and bit length for the identified necessary information that need to be indicated by R2D control information.

Proposal 3.2-2: Reader acquires the end of PDRCH transmission based on R2D control information with explicit or implicit TBS indication.

Proposal 3.3-1: Support the following for R2D control information:

- A separate CRC is not required.

- R2D control information should be placed in the earlier part of the PRDCH.

- The bit string carried by R2D control information does not need to be byte-aligned (i.e., a multiple of 8 bits) in length.

Proposal 3.3-2: For R2D control information, it is not preferred to discuss case by case which field/parameter should be carried by L1-control signaling or by higher layer signaling. Unified container (either L1-control or higher layer signaling) is used to carry the R2D control information.

For Timing relationships

Proposal 4-1: For the time interval between a R2D transmission and the corresponding D2R transmission following it,

- “the end of the R2D transmission” is defined by one of the following options

  - Option 1: the last chip of the R2D transmission

  - Option 2: the last OFDM symbol of the R2D transmission

- Note that whether the R2D transmission includes a R2D postamble can be determined after the decision on the whether/how to support the R2D postamble is made.

- “the start of the corresponding D2R transmission” is defined as the first chip of the D2R preamble.

Proposal 4-2: For the time interval between a D2R transmission and the corresponding R2D transmission following it,

- “the end of the D2R transmission” is defined as the last chip of the D2R transmission

  - Note that whether the D2R transmission includes a D2R postamble can be determined after the decision on the whether/how to support the D2R postamble is made.

- “the start of the corresponding R2D transmission” is defined as the first chip or the first OFDM symbol of the R2D preamble.

Proposal 4.1: Consider defining following time unit for A-IoT operation.

- Basic time unit: the shortest/smallest chip length that A-IoT system supports, e.g., Minimum (Smallest R2D chip length, Smallest D2R chip length).

- Unit for timing relations: the time duration used to determine at least the time interval between the R2D and corresponding D2R following it, the D2R and corresponding R2D following it.

Proposal 4.2: Discuss whether it is necessary to define different minimum processing time for different commands, for example, whether device requires more processing time for certain time-consuming commands.

Proposal 4.3: For the case(s) where a R2D transmission in response to a D2R transmission is expected, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max].

Proposal 4.4-1: the minimum time between two different consecutive R2D transmissions to the same A-IoT device needs to be defined.

- No need to define the maximum time between two different consecutive R2D transmissions to the same A-IoT device unless the necessity is identified.

Proposal 4.4-2: Discuss whether to define the the minimum time between two consecutive D2R transmissions from the same A-IoT device, considering following use cases.

- Use case 1. Similar as in-process reply in RFID

- Use case 2. Different D2R segments for a higher layer data

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR3GPP RAN1#116bis meeting, Chairman’s note, RAN1 Vice-chair (Huawei)

- 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (Release 19), 2024-12

- R1-2500350, Discussion on line coding, FEC, CRC and repetition for A-IoT, vivo

- EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard, Release 3.0, Ratified, Jan 2024

- R1-2500349, Discussion on Modulation Aspects of Physical Channels Design, vivo

- R1-2500351, Discussion on Timing acquisition and synchronization for AIoT, vivo