3GPP TSG RAN WG1 #120                             	                                     	               R1-2500371

Athens, Greece, February 17th – 21st, 2025

Source: 	vivo

Title:	Discussion on enhancement of multi-cell PUSCH/PDSCH scheduling with a single DCI

Agenda Item:	9.12.1

Document for:	Discussion and Decision

- Introduction

In RAN#105, a new WID for multi-carrier enhancements for NR Phase 2 is approved [1]. The objectives of the Rel-19 MC (multi-carrier) work item are shown as follows. In this contribution, the views on the multi-cell PUSCH/PDSCH scheduling with a single DCI are provided.

Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs per scheduled cell is [4 or 8]. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

- Discussion

  - Scenarios

    - Different SCSes/carrier types among co-scheduled cells by the single DCI

Agreement - Consider at least the case that up to two different SCS can be scheduled by a DCI format 0_3/1_3 in Rel-19. - Consider at least the following cases for scheduled cells in Rel-19: - Case 1: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR1 licensed TDD cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 2: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 3: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2. - SCS1 can be same or different to SCS2. - Case 4: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with different SCS. - Case 5: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with different SCS. - Case 6: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR2-1 cell(s) with different SCS.

Due to, e.g., TU limitation, the Rel-18 MC WI only supports to co-schedule multiple carriers of the same type with the same SCS. In the Rel-19, it is intended to enhance the MC scheduling for the co-scheduled cells with different SCSes and/or carrier types. Considering the work load and the limited TU, it is important to clarify the scenarios to be supported in Rel-19, e.g., intended combination of carrier types and/or SCS. In the RAN1#118bis meeting, the above six cases are considered.

For case 1, the TDD and FDD duplex modes with individual SCS are already deployed in different bands. Comparing with TDD band, the FDD band has better coverage but narrower frequency range. Furthermore, the carrier aggregation for FDD and TDD has been supported in NR, so that a single DCI co-scheduling the TDD band and FDD band in FR1 should be supported to save control signaling overhead. For case 2 and case 3, FR1 + FR2-1 have high commercial needs. For example, the DCI in FR1 with better coverage schedules cells in FR2 with wider bandwidth. For case 4/5/6, the different SCSes on FR1 TDD/FR2 FDD/FR2-1 scheduled cells can be supported for more flexible scheduling by extending the Rel-18 multi-carrier scheduling.

Proposal 1: The case 1 to case 6 for the scheduled cells can be supported in Rel-19 MCE.

Besides the above cases, the cases of scheduled cells on unlicensed bands can also be considered. Rel-18 MC supports to schedule multiple carriers by a DCI format 0_3/1_3, where all the carriers are deployed in licensed band or unlicensed band with the same carrier type. It is worth noting that in DCI format 0_3/1_3, the DCI fields related to unlicensed band, i.e., ChannelAccess-CPext-CAPC field and frequency domain resource assignment field, can be simply re-used to support scheduling the carriers on both a licensed band and an unlicensed band, with none or minor spec impact.

Proposal 2: In Rel-19, if there is no additional spec impact, support the following cases for scheduled cells:

- Case 7: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR1 unlicensed TDD cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 8: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with SCS1 and FR1 unlicensed TDD cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 9: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 unlicensed TDD cell(s) with different SCSes.

It has been agreed to consider at least the case that up to two different SCSes can be scheduled by a DCI format 0_3/1_3 in Rel-19. In our view, more than two different SCSes can be also considered in Rel-19 with minor or no additional design effort. The cross-carrier scheduling has been supported for 15khz~960khz in current specification. It seems no or little additional effort is required to extend the support to MC scheduling. A typical case can be a single DCI scheduling a carrier with 15khz/30khz SCS and a carrier with SCS up to 960khz, which is similar as case 3.

Proposal 3: Support at least the case that up to two different SCSes can be scheduled by a DCI format 0_3/1_3 in Rel-19. If there is no additional spec impact, support more than two different SCSes from 15kHz to 960kHz among the cells co-scheduled by a DCI format 0_3/1_3.

    - One or multiple PUSCHs/PDSCHs per cell scheduled by the single DCI

Proposal in FL summary[2] - Multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to at least 120kHz SCS in Rel-19. - Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR1 cells and FR2-1 cells.

The scenarios for multi-cell multi-PDSCH and Multi-cell multi-PUSCH were discussed in the RAN1#118bis meeting.  In Rel-16 and Rel-17, multi-PDSCH and multi-PUSCH scheduling in one cell are supported. In our view, in Rel-19, the multi-cell multi-PDSCH scheduling and multi-cell multi-PUSCH scheduling by DCI format 1_3 and 0_3 can base on the applicable scenarios for the multi-PDSCH and multi-PUSCH scheduling in one cell in the current spec. Specifically, the multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to 120kHz SCS for FR2-1 cells and FR2-2 cells. Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR1/FR2-1/FR2-2 cells.

Proposal 4: In Rel-19 MCE, consider the following scenarios:

- Multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to 120kHz SCS for FR2-1 cells.

- Multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to FR2-2 cells.

- Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR1 cells.

- Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to 120kHz SCS for FR2-1 cells.

- Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR2-2 cells.

  - Different SCSes/carrier types among co-scheduled cells by the single DCI

    - PUCCH resource timing

In the RAN1#118bis meeting, one question was raised [2] that the UE PDSCH process time may be affeted when a DCI format 1_3 schedules the co-scheduled cells with different SCSes, because the Tproc,1​ determined based on the last ending PDSCH may not be the maximum Tproc,1​ value among the co-scheduled PDSCHs. However, in our understanding, it is not problematic. The Tproc,1​ is clearly defined in the spec for each co-scheduled PDSCH, thus, the gNB can select a suitable PUCCH resource that meets the timing requirement for all the Tproc,1​ values, including the maximum Tproc,1​ value among co-scheduled PDSCHs.

Observation 1：It is up to gNB implementation to make sure the indicated PUCCH resource satisfying the maximum Tproc,1​ value among the co-scheduled PDSCHs.

In the RAN1#119 meeting, the issue of PUCCH resource timing was raised (the related specification in TS 38213 is quoted below), and the following proposal 3-1 for this issue was discussed. If the UE is provided subslotLengthForPUCCH, according to the specification, n is the last UL slot for PUCCH transmission that overlaps with a PDSCH reception, thus, even for scheduled PDSCHs with different SCSes by a DCI format 1-3, the slot n for PUCCH transmission can be determined by the Rel-18 operation without spec change. On other hand, if the UE is not provided subslotLengthForPUCCH, according to the specification, n is the last UL slot for PUCCH transmission that overlaps with the DL slot nD for the PDSCH reception, where nD is the DL slot in which PDSCH receptions ending. In another word, the DL slot nD is the DL slot ending last amongst the DL slots where the PDSCH receptions are scheduled by the DCI format 1_3. Thus, the current spec already applies for the case that scheduled PDSCHs with different SCSes by a DCI format 1-3 if the UE is not provided subslotLengthForPUCCH.

- Proposal 3-1 rev2[3]: - For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation, i.e., - If the UE is not provided subslotLengthForPUCCH, the DL slot 𝑛𝐷 is the DL slot ending last, amongst the DL slots where the PDSCH receptions are scheduled by the DCI format 1_3. - FFS: RAN1 spec impact - If the UE is provided subslotLengthForPUCCH, no spec change is necessary. - Note: Specification of this feature shall not impact the existing UE processing PDSCH timeline requirement for any individual PDSCH, as specified in 5.3.1 of TS38.214. TS 38213 Section 9.2.3 UE procedure for reporting HARQ-ACK [4] The following apply to the PCell if the UE is provided pucch-sSCellPattern; otherwise, the following apply to the serving cell of the PUCCH transmission. If the UE is provided subslotLengthForPUCCH, n is the last UL slot for PUCCH transmission that overlaps with a PDSCH reception or with a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception; otherwise, n is the last UL slot for PUCCH transmission that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception in case of a DCI format that triggers a HARQ-ACK information report and does not schedule a PDSCH reception. For a SPS PDSCH reception ending in DL slot nD, the UE transmits the PUCCH in UL slot n+k where k is provided by the PDSCH-to-HARQ_feedback timing indicator field, if present, in a DCI format activating the SPS PDSCH reception. If the UE detects a DCI format that does not include a PDSCH-to-HARQ_feedback timing indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k where k is provided by dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-v1700. If the UE detects a DCI format scheduling a number of PDSCH receptions ending in DL slot nD or if the UE detects a DCI format generating a HARQ-ACK information bit and does not schedule a PDSCH reception through a PDCCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-v1700.

Proposal 5: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation, i.e.,

- If the UE is not provided subslotLengthForPUCCH, the DL slot 𝑛𝐷 is the DL slot ending last, amongst the DL slots where the PDSCH receptions are scheduled by the DCI format 1_3, no spec change is necessary.

- If the UE is provided subslotLengthForPUCCH, no spec change is necessary.

Note: Specification of this feature shall not impact the existing UE processing PDSCH timeline requirement for any individual PDSCH, as specified in 5.3.1 of TS38.214.

    - The number of unicast DCI(s)

Some additional issues may need to be resolved if different SCSes/carrier types among the co-scheduled cells are supported. For example, in the case of combination of FR1/FR2-1 + FR2-2, the number of unicast DCI(s) per N consecutive slots of scheduling cell to process for a set of cells need to be discussed. In Rel-18 MC, N is based on pair of (scheduling CC’s SCS, scheduled CC’s SCS) for high-to-low SCS scheduling, N equals 1 for low-to-high SCS scheduling. In Rel-19 MC, if different SCSes/carrier types among the co-scheduled cells are supported, the following cases occur:

- Case 1: low to multiple cells with different higher SCSes are simultaneously scheduled by a DCI format 0_3 or 1_3

- Case 2: high to multiple cells with different lower SCSes are simultaneously scheduled by a DCI format 0_3 or 1_3

- Case 3: both high-to-low SCS and low-to-high SCS are simultaneously scheduled by a DCI format 0_3 or 1_3.

For case 1, the N can be equal to 1. For case 2, the N can be based on pair of (scheduling CC’s SCS, scheduled CC’s SCS), the scheduled CC SCS can be the lowest or the highest or the reference SCS. For case 3, how to determine the value of N should be decided, for example, N=1 or based on the on pair of (scheduling CC’s SCS, scheduled CC’s SCS). One straightforward way is to determine N based on pair of (scheduling CC’s SCS, scheduled CC’s SCS). However, unlike the case in Rel-18, there can be multiple different SCSes among the scheduled cells. Thus, it should decide which SCS should be used as the scheduled CC’s SCS, e.g., the lowest, the highest or the reference SCS. In our opinion, the N can be based on the scheduled CC with the lowest SCS so that the DCI processing capability is accommodating for the other SCSes.

Proposal 6: For multi carrier scheduling with mix SCSes, the number of unicast DCI(s) to be monitored is defined per N consecutive slots, where the N is based on the lowest SCS among the cells.

  - One or multiple PUSCHs/PDSCHs per cell scheduled by the single DCI

    - DCI design

Agreement - For NDI indication in DCI format 0_3/1_3 for TB1, further study - Option 1: For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3. - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell. - Option 2a: Total number of bits of the NDI field is equal to the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells. - The number of bits of a block of NDI field corresponding to a scheduled cell is equal to the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell. - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells. - Other options are not precluded. - Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of NDI field for TB1, same as in Rel-18. Agreement - For RV indication in DCI format 0_3/1_3 for TB1, further study - Option 1: For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell. - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell. - Option 2a: Total number of bits of the RV field is determined based on the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells. - The number of bits of a block of RV field corresponding to a scheduled cell is determined based on the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell and number of bits for RV configured for the cell. - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells. - Other options are not precluded. - Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of RV field for TB1, same as in Rel-18.

In the RAN1#119 meeting, it was agreed to further study option 1 and option 2a for the NDI and RV indication for TB1. For both options, DCI format 0_3/1_3 contains NcellUL or NcellDL blocks of NDI field and RV field for TB1, consistent with that in Rel-18. In Rel-18 MC, if the scheduled cells are indicated by FDRA, the number of blocks NcellUL or NcellDL in DCI format 0_3/1_3 corresponds to the number of cells in the cell set. Otherwise, if the scheduled cells are indicated by scheduled cell indicator, the number of blocks NcellUL or NcellDL in DCI format 0_3/1_3 corresponds to the number of cells in the cell combination indicated by the scheduled cell indicator.

Nevertheless, the overhead is different between these options. For evaluation, we compare the total DCI payload size between them, based on the following assumptions:

- A cell set consists of 4 cells (NcellUL-max or NcellDL-maxequals to 4), each with 20 MHz bandwidth operating with an SCS of 30 kHz, which translates to 50 RBs per cell. [5]

- The maximum number of schedulable PUSCHs/PDSCHs per cell is 8.

- The maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells, denoted by NPUSCHmax or NPDSCHmax , is either 8, 16, 24 or 32, which is determined jointly by the TDRA table and scheduled cell combination table configurations. It is noted that if the scheduled cells are indicated by FDRA indicator, the NPUSCHmax or NPDSCHmax is determined by the TDRA table assuming all the cells in the cell set are scheduled.

Based on the Table 5 and Table 6 provided in appendix, the total DCI payload of DCI format 0_3/1-3 can be calculated using the following equation:

- For Option 1

  - Total DCI payload of DCI format 0_3

    - 22+NMCS*NcellUL-max+NNDI*NcellUL-max*8+NRV*NcellUL-max*8+NHPN*NcellUL-max+Nscheduledcellindicator

  - Total DCI payload of DCI format 1_3

    - 24+ NMCS*NcellDL-max+NNDI*NcellDL-max*8+NRV*NcellDL-max*8+NHPN*Ncell  DL-max+Nscheduledcellindicator

- For Option 2a

  - Total DCI payload of DCI format 0_3

    - 22+NMCS*NcellUL-max+NNDI*NPUSCHmax+NRV*NPUSCHmax+NHPN*NcellUL-max+Nscheduledcellindicator

  - Total DCI payload of DCI format 1_3

    - 24+NMCS*NcellDL-max+NNDI*NPDSCHmax+NRV*NPDSCHmax+NHPN*NcellDL-max+Nscheduledcellindicator

If the scheduled cells are indicated by FDRA, Nscheduledcellindicator  is 0 bit, otherwise, Nscheduledcellindicator  is 2 bits. Furthermore, the following assumptions apply:

- NNDI is 1 bit per PDSCH/PUSCH.

- NRV is 1 bit per PDSCH/PUSCH.

- NHPN is 5bits per cell.

- NMCS is 5 bits per cell.

Using these assumptions, the DCI payload for DCI format 0-3 and 1-3 are calculated and illustrated in Table 1 and Table 2.

Table 1 DCI payload of DCI format 0-3 for Option 1 and Option 2a

| FDRA based |  | Scheduled cell indicator indicates NcellUL |  |
| --- | --- | --- | --- |
| Option1 | 126bits | Option1 | 128bits |
| Option2a (NPUSCHmax=32) | 126bits | Option2a (NPUSCHmax=8) | 80bits |
|  |  | Option2a (NPUSCHmax=16) | 96bits |
|  |  | Option2a (NPUSCHmax=24) | 112bits |
|  |  | Option2a (NPUSCHmax=32) | 128bits |

Table 2 DCI payload of DCI format 1-3 for Option 1 and Option 2a

| FDRA based |  | Scheduled cell indicator indicates NcellDL |  |
| --- | --- | --- | --- |
| Option1 | 128bits | Option1 | 130bits |
| Option2a (NPDSCHmax=32) | 128bits | Option2a (NPDSCHmax=8) | 82bits |
|  |  | Option2a (NPDSCHmax=16) | 98bits |
|  |  | Option2a (NPDSCHmax=24) | 114bits |
|  |  | Option2a (NPDSCHmax=32) | 130bits |

In Table 1 and Table 2, the color-coded remarks provide the following comparisons between Option 2a and Option 1 in terms of DCI payload size:

- Yellow: Option 2a has a smaller DCI payload size than Option 1.

- Green: Option 2a has an equal DCI payload size to Option 1.

For FDRA based indication, the DCI payload size is same between option 2a and option1. For scheduled-cell-indicator based indication, when NPUSCHmax=8, 16, 24, the option 2a generally results in a DCI payload size that is smaller than option 1. Specifically, the DCI payload for DCI format 0-3 for option 2a reduces by 37.5%, 25%, 12.5% compared with option 1, and the DCI payload for DCI format 1-3 for option 2a reduces by 36.9%, 24.6%, 12.3% compared with option 1. When NPUSCHmax=32, option 2a has the same DCI payload size as option1. However, NPUSCHmax=32 corresponds to a corner case where 4 cells in a cell set are scheduled and each cell having 8 PDSCH/PUSCHs. This is highly specific and rare scheduling scenario.  Thus, option 2a is preferred.

Observation 2: For FDRA based indication, the DCI payload size is same between option 2a and option1.

Observation 3: For scheduled-cell-indicator based indication, the option 2a can reduce the DCI payload size by ~37%, ~25%, or ~12% when the maximum number of schedulable TBs across all the co-schedulable cells is 8, 16, or 24.

Proposal 7: For NDI and RV indication in DCI format 0_3/1_3 for TB1, option2a is recommended for adoption.

    - Type 2 HARQ-ACK codebook

Agreement - Time-domain HARQ-ACK bundling is configured per cell as Rel-17. Agreement - Type-2 HARQ-ACK codebook is generated by concatenating a first sub-codebook and a second sub-codebook. - The first sub-codebook comprises HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single PDSCH, or each scheduling a single cell with multiple PDSCHs on it and nrofHARQ-BundlingGroups configured as 1, and HARQ-ACK information bit(s) for DCI(s) having associated HARQ-ACK information without scheduling PDSCH reception. - The second sub-codebook comprises HARQ-ACK information bits for PDSCHs scheduled by DCI(s) with each scheduling more than one cell, or each scheduling a single cell with multiple PDSCHs on it without nrofHARQ-BundlingGroups or nrofHARQ-BundlingGroups configured larger than 1. - Separate DAI counting is applied for DCI(s) associated with the first sub-codebook and DCI(s) associated with the second sub-codebook as Rel-18. - Note: For providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication as Rel-18. Agreement - For the second sub-codebook, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, to select one of the following options. - Option 1: M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration. - Option 2: M is explicitly configured by RRC parameter for the PUCCH group. - Other options are not precluded.

The number of HARQ-ACK information bits for each DCI format 1_3

In the RAN1#119 meeting, it was agreed to further study the option1 and option2 for the number of HARQ-ACK information bits for each DCI format 1_3(M) of the second sub-codebook. Option 1 aligns with the legacy method for determining the number of HARQ-ACK information bits for each DCI format 1_3. Option 2 requires introducing a moderation value for all possible scenarios which may increase the signalling overhead. For simplification, option 1 is preferred over option 2.

For option 1, several sub-options can be considered:

- Option 1a: max⁡(Xi*Y) across all the schedulable cell, where:

  - Xi: Maximum number of HARQ-ACK bits for each scheduled cell i.

  - Y: Number of schedulable cells in a cell set.

  - i=0,1…… Y-1.

- Option 1b: 0 Y-1Xi, where:

  - Xi: Maximum number of HARQ-ACK bits for each scheduled cell i,

  - Y: Number of schedulable cells in a cell set.

  - i=0,1…… Y-1.

- Option 1c: max (i=0 Mj-1Ni) across all the schedulable cell combinations, where:

  - Mj:Number of cells for a schedulable cell combination j, configured by scheduledCellComboListDCI-1-3.

  - Ni: Number of HARQ-ACK bits for each scheduled cell i in the cell combination.

  - j=0,1……P-1, where P is the total number of cell combinations configured.

  - i=0,1……Mj-1.

- Option 1d: max (i=0 Mj-1Xi) across all the schedulable cell combinations, where:

  - Mj:Number of cells for a schedulable cell combination j, configured by scheduledCellComboListDCI-1-3.

  - Xi: Maximum number of HARQ-ACK bits for each scheduled cell i.

  - j=0,1……P-1, where P is the total number of cell combinations configured.

  - i=0,1……Mj-1.

Table 3 Scheduled cell combinations

| Combination 0 | Cell 0+Cell 1 |
| --- | --- |
| Combination 1 | Cell 0+Cell 1+Cell 2 |

Table 4 TDRA table for the active BWP in each cell

| Cell 0 | Cell 1 | Cell 2 |
| --- | --- | --- |
| 1 SLIVs | 2 SLIVs | 4 SLIVs |
| 2 SLIVs | 4 SLIVs | 2 SLIVs |

For evaluation the overhead, we compare the options based on the Table 3 and Table 4, and the following parameters:

- harq-ACK-SpatialBundlingPUCCH or nrofHARQ-BundlingGroups is configured, with maxNrofCodeWordsScheduledByDCI=1 for each serving cell.

- The number of HARQ-ACK bits for each serving cell equals the configured SLIVs.

- A cell set contains three serving cells: X0=2, X1=4, X2=4 (The maximum number of HARQ-ACK bits for cells 0, 1, and 2)

The calculation for each sub-option is as follows:

- Option1a: max⁡(Xi*Y) across all the schedulable cell

- maxX0*Y,X1*Y,X2*Y=max2*3, 4*3, 4*3=12.

- Option1b: 0 Y-1Xi

- X0+X1+X2=2+4+4=10.

- Option1c: max (i=0 Mj-1Ni) across all the schedulable cell combinations

  - Cell combination 0:

  - M0=2 (cell0 and cell1)

  - Ni=1,2 or 2,4 for (cell 0, cell 1).

  - Cell combination 1:

  - M1=3 (cell0, cell1 and cell2)

  - Ni=1,2,4 or 2,4,2 for (cell 0, cell 1, cell2).

- max (i=0 Mj-1Ni) = max (1+2, 2+4, 1+2+4, 2+4+2) =8.

- Option1d: max (i=0 Mj-1Xi) across all the schedulable cell combinations

  - Cell combination 0:

  - M0==2 (cell0 and cell1)

  - Xi=2,4 for (cell 0, cell 1).

  - Cell combination 1:

  - M1=3 (cell0, cell1 and cell2)

  - Xi=2,4,4 for (cell 0, cell 1, cell2).

- max (i=0 Mj-1Xi)=(2+4, 2+4+4) =10

In this example, Option 1a and Option 1b are simpler but result in larger HARQ codebook size, leading to increased padding bits. Option 1c is the most effective method to optimize signaling overhead especially when scheduledCellComboListDCI-1_3 is configured.

When the scheduled cells are indicated using FDRA (without pre-configured cell combinations in the RRC parameter), Option 1c and Option 1d are applied only based on the jointly configured TDRA table, in this case, the HARQ codebook size may not be reduced compare with Option 1a and Option 1b. As a result, in this case, Option 1a and Option 1b are preferred. Moreover, compare with Option 1a, Option 1b provides better signaling overhead reduction.

Therefore, in our view, if the scheduledCellComboListDCI-1_3 is configured, Option 1c is applied to minimize signaling overhead, otherwise, use Option 1b for FDRA-based indication (without pre-configured cell combinations in the RRC parameter) to balance simplicity and signaling overhead reduction.

Proposal 8: For type 2 HARQ codebook generation, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration (Option 1).

Proposal 9: For the number of HARQ-ACK information bits for each DCI format 1_3 which is equal to M, M is implicitly derived based on following rule:

- If scheduledCellComboListDCI-1_3 is configured, apply Option 1c:

- Option 1c: max (i=0 Mj-1Ni) across all the schedulable cell combinations, where:

    - Mj:Number of cells for a schedulable cell combination j, configured by scheduledCellComboListDCI-1-3.

    - Ni: Number of HARQ-ACK bits for each scheduled cell i in the cell combination.

    - j=0,1……P-1, where P is the total number of cell combinations configured.

    - i=0,1……Mj-1.

- If scheduledCellComboListDCI-1_3 is not configured, apply Option 1b:

- Option 1b: 0 Y-1Xi, where:

    - Xi: Maximum number of HARQ-ACK bits for each scheduled cell i,

    - Y: Number of schedulable cells in a cell set.

    - i=0,1…… Y-1.

For each serving cell, the HARQ-ACK bits is determined based on the configuration of HARQ bundling in spatial domain and time domain, for example, whether the harq-ACK-SpatialBundlingPUCCH or nrofHARQ-BundlingGroups is configured. Thus, the following proposal is proposed.

Proposal 10: The number of HARQ-ACK bits for each serving cell is determined based on the configurations of HARQ bundling in the spatial and time domains. The logic is defined as follows:

- If harq-ACK-SpatialBundlingPUCCH is configured,

  - If nrofHARQ-BundlingGroups is configured, the number of HARQ-ACK bits is NHARQ-ACKTBG,max

  - If nrofHARQ-BundlingGroups is not configured, the number of HARQ-ACK is the number of configured PDSCHs

- If harq-ACK-SpatialBundlingPUCCH is not configured,

  - If maxNrofCodeWordsScheduledByDCI=2 for the serving cell

    - If nrofHARQ-BundlingGroups is configured, the number of HARQ-ACK bits is NHARQ-ACKTBG,max*2

    - If nrofHARQ-BundlingGroups is not configured, the number of HARQ-ACK is the number of configured PDSCHs * 2

  - If maxNrofCodeWordsScheduledByDCI=1 for the serving cell

    - If nrofHARQ-BundlingGroups is configured, the number of HARQ-ACK bits is NHARQ-ACKTBG,max

    - If nrofHARQ-BundlingGroups is not configured, the number of HARQ-ACK is the number of configured PDSCHs

The order for HARQ-ACK information bits

In RAN1 119 meeting, for Rel-17 multi-PDSCHs scheduling by DCI format 1_1, there are two interpretations:

- Interpretation 1: First, in ascending order of codeword index for each PDSCH, second, in ascending order of PDSCH reception starting time.

  - Example order: (TB1 of PDSCH1, TB2 of PDSCH1, TB1 of PDSCH2, TB2 of PDSCH2)

- Interpretation 2: First, in ascending order of PDSCH reception starting time, second, in ascending order of codeword index for each PDSCH.

  - Example order: (TB1 of PDSCH1, TB1 of PDSCH2, TB2 of PDSCH1, TB2 of PDSCH2)

It can be observed that interpretation 1 is more aligned with the specification of TS 38213. Firstly, for a single PDSCH, the HARQ-ACK bits for the two codewords are tightly connected according to the pseudocode. Secondly, when multi-PDSCHs are scheduled, the pseudocode implements each PDSCH following the yellow remarks. Thus, HARQ-ACK information bits for co-scheduled PDSCHs on one cell by a DCI format 1_1 is ordered firstly on the codeword index, and then in order of the PDSCH reception starting time.

TS 38213[4] 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell < Unchanged parts are omitted > If a UE is provided pdsch-TimeDomainAllocationListForMultiPDSCH and neither provided nrofHARQ-BundlingGroups nor harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport blocks for PDSCH receptions. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NPDSCH,c HARQ-ACK information bits for the first TBs in the ascending order of the starting of PDSCH receptions and, if applicable, generates NPDSCH,c HARQ-ACK information bits for the second TBs in the ascending order of the starting of PDSCH receptions. For a PDSCH reception that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the first TB and, if applicable, generates a NACK value for the second TB in the PDSCH reception. If NPDSCH,c<NPDSCH,cmax, the UE generates a NACK value for the last NTB,cDL∙NPDSCH,cmax-NTB,cDL∙NPDSCH,c HARQ-ACK information bits where NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c and NPDSCH,cmax is determined by the maximum number of SLIVs amongst all rows of the TDRA table configured by pdsch-TimeDomainAllocationListForMultiPDSCH. < Unchanged parts are omitted >

Observation 4：For the order methods for Rel-17 multi-PDSCHs scheduling by DCI format 1_1, the specification in TS 38213 is interpreted as follows: first, in ascending order of codeword index for a PDSCH, second, in ascending order of starting reception time for a PDSCH.

Additionally, in Rel-18, HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_3 is ordered based on serving cell indices associated with co-scheduled PDSCHs. As a result, for multi-cell multi-PDSCH scheduling by a DCI format 1_3 in Rel-19, the HARQ-ACK information bits for co-scheduled PDSCHs can be derived based on the Rel-17 multi-PDSCHs scheduling ordering together with the Rel-18 MC ordering. More specially, first, in ascending order of codeword index for a PDSCH, second, in ascending order of the PDSCH reception starting time, third, in ascending order of serving cell index.

Proposal 11: For multi-cell multi-PDSCH scheduling by a DCI format 1_3 in Rel-19, the HARQ-ACK information bits for co-scheduled PDSCHs can be ordered by:

- First, in ascending order of codeword index for a PDSCH.

- Second, in ascending order of the PDSCH reception starting time.

- Third, in ascending order of serving cell index.

HARQ-ACK Feedback for Invalid PDSCH

Regarding the HARQ-ACK feedback for the invalid PDSCH due to collision with semi-static TDD DL/UL configuration, in Rel-18 MC scheduling, the HARQ-ACK bit for an invalid PDSCH is set to NACK. In Rel-17 multi-PDSCH scheduling by DCI format 1_1, the HARQ-ACK bit for an invalid PDSCH is set to NACK if HARQ bundling in time domain is not configured. Moreover, if HARQ bundling in time domain is configured, for a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bit for the bundling group is also set to NACK.

These mechanisms defined in Rel-17 and Rel-18 can be reused for Rel-19 MC scheduling, ensuring continuity and predictability in HARQ-ACK feedback behavior. If HARQ bundling in time domain is not configured for a serving cell, the HARQ-ACK bit for an invalid PDSCH is set to NACK. If HARQ bundling in time domain is configured for a serving cell, Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case. If the group is empty or filled with only invalid PDSCH(s), HARQ-ACK bit for the bundling group is set to NACK.

Proposal 12: For the HARQ-ACK feedback for the invalid PDSCH due to collision with semi-static TDD DL/UL configuration, the mechanisms defined in Rel-17 and Rel-18 can be reused:

- If HARQ bundling in time domain is not configured for a serving cell, the HARQ-ACK bit for an invalid PDSCH is set to NACK.

- If HARQ bundling in time domain is configured for a serving cell, Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case.

- If the group is empty or filled with only invalid PDSCH(s), HARQ-ACK bit for the bundling group is set to NACK

HARQ-ACK feedback for SCell dormancy indication

Another issue is the HARQ-ACK for SCell dormancy indication in the second sub-codebook. In Rel-18, if a DCI format 1_3 is transmitted with fields repurposed for SCell dormancy indication and schedules one or more PDSCHs, HARQ-ACK information for the SCell dormancy indication is mapped to HARQ-ACK bit position for the serving cell with the smallest cell index with invalid FDRA and included in the second Type-2 HARQ-ACK sub-codebook. The UE assumes that it receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication, and sets the HARQ-ACK bit for the PDSCH TB1 to ACK. For Rel-19 mc, the same way can also be applied. If there are multiple SLIVs in the serving cell with smallest cell index with invalid FDRA, the HARQ-ACK bits for the remaining SLIVs are assumed with NACK.

Proposal 13: If a DCI format 1_3 is transmitted with fields repurposed for SCell dormancy indication and schedules one or more PDSCHs, and if there are multiple SLIVs in the serving cell with smallest cell index with invalid FDRA, the HARQ-ACK bits for SCell dormancy indication is ACK for TB1. The remaining SLIVs are assumed with NACK.

The reference PDSCH for PUCCH timing

In Rel-18, for determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_3 among the set of co-scheduled PDSCHs. In Rel-17, for determining the timing of the HARQ feedback in the case of multi-PDSCHs scheduling, the reference PDSCH is the last PDSCH among the co-scheduled PDSCHs by DCI format 1_1 even this PDSCH is invalid due to collision with uplink symbol. In the case of multiple PDSCHs in multi-cell scheduled by DCI format 1_3, the determination of reference PDSCH also need to be specified. In our view, the definition for reference PDSCH in Rel-18 can be reused. Specifically, the reference PDSCH is the PDSCH ending last among the co-scheduled PDSCHs among the co-scheduled cells. Even if the reference PDSCH is an invalid PDSCH due to collision with uplink symbol, it can also be used to determine the PUCCH timing since the it is simple and having less standard effort.

Proposal 14: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs scheduled by a DCI format 1_3, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_3 among the set of co-scheduled PDSCHs including the invalid PDSCH due to collision with uplink symbol.

- Conclusion

In this contribution, the following observations and proposals are summarized as follows:

Observation 1：It is up to gNB implementation to make sure the indicated PUCCH resource satisfying the maximum Tproc,1​ value among the co-scheduled PDSCHs.

Observation 2: For FDRA based indication, option 2a generally results in a DCI payload equal to option1.

Observation 3: For scheduled-cell-indicator based indication, the option 2a can reduce the DCI payload size by ~37%, ~25%, or ~12% when the maximum number of schedulable TBs across all the co-schedulable cells is 8, 16, or 24.

Proposal 1: The case 1 to case 6 for the scheduled cells can be supported in Rel-19 MCE.

Proposal 2: In Rel-19, if there is no additional spec impact, support the following cases for scheduled cells:

- Case 7: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR1 unlicensed TDD cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 8: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with SCS1 and FR1 unlicensed TDD cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 9: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 unlicensed TDD cell(s) with different SCSes.

Proposal 3: Support at least the case that up to two different SCSes can be scheduled by a DCI format 0_3/1_3 in Rel-19. If there is no additional spec impact, support more than two different SCSes from 15kHz to 960kHz among the cells co-scheduled by a DCI format 0_3/1_3.

Proposal 4: In Rel-19 MCE, consider the following scenarios:

- Multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to 120kHz SCS for FR2-1 cells.

- Multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to FR2-2 cells.

- Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR1 cells.

- Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to 120kHz SCS for FR2-1 cells.

- Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR2-2 cells

Proposal 5: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation, i.e.,

- If the UE is not provided subslotLengthForPUCCH, the DL slot 𝑛𝐷 is the DL slot ending last, amongst the DL slots where the PDSCH receptions are scheduled by the DCI format 1_3, no spec change is necessary.

- If the UE is provided subslotLengthForPUCCH, no spec change is necessary.

Proposal 6: For multi carrier scheduling with mix SCSes, the number of unicast DCI(s) to be monitored is defined per N consecutive slots, where the N is based on the lowest SCS among the cells.

Proposal 7: For NDI and RV indication in DCI format 0_3/1_3 for TB1, option2a is recommended for adoption.

Proposal 8: For type 2 HARQ codebook generation, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration (Option 1).

Proposal 9: For the number of HARQ-ACK information bits for each DCI format 1_3 which is equal to M, M is implicitly derived based on following rule:

- If scheduledCellComboListDCI-1_3 is configured, apply Option 1c:

- Option 1c: max (i=0 Mj-1Ni) across all the schedulable cell combinations, where:

    - Mj:Number of cells for a schedulable cell combination j, configured by scheduledCellComboListDCI-1-3.

    - Ni: Number of HARQ-ACK bits for each scheduled cell i in the cell combination.

    - j=0,1……P-1, where P is the total number of cell combinations configured.

    - i=0,1……Mj-1.

- If scheduledCellComboListDCI-1_3 is not configured, apply Option 1b:

- Option 1b: 0 Y-1Xi, where:

    - Xi: Maximum number of HARQ-ACK bits for each scheduled cell i,

    - Y: Number of schedulable cells in a cell set.

    - i=0,1…… Y-1.

Proposal 10: The number of HARQ-ACK bits for each serving cell is determined based on the configurations of HARQ bundling in the spatial and time domains. The logic is defined as follows:

- If harq-ACK-SpatialBundlingPUCCH is configured,

  - If nrofHARQ-BundlingGroups is configured, the number of HARQ-ACK bits is NHARQ-ACKTBG,max

  - If nrofHARQ-BundlingGroups is not configured, the number of HARQ-ACK is the number of configured PDSCHs

- If harq-ACK-SpatialBundlingPUCCH is not configured,

  - If maxNrofCodeWordsScheduledByDCI=2 for the serving cell

    - If nrofHARQ-BundlingGroups is configured, the number of HARQ-ACK bits is NHARQ-ACKTBG,max*2

    - If nrofHARQ-BundlingGroups is not configured, the number of HARQ-ACK is the number of configured PDSCHs * 2

  - If maxNrofCodeWordsScheduledByDCI=1 for the serving cell

    - If nrofHARQ-BundlingGroups is configured, the number of HARQ-ACK bits is NHARQ-ACKTBG,max

    - If nrofHARQ-BundlingGroups is not configured, the number of HARQ-ACK is the number of configured PDSCHs

Proposal 11: For multi-cell multi-PDSCH scheduling by a DCI format 1_3 in Rel-19, the HARQ-ACK information bits for co-scheduled PDSCHs can be ordered by:

- First, in ascending order of codeword index for a PDSCH.

- Second, in ascending order of the PDSCH reception starting time.

- Third, in ascending order of serving cell index.

Proposal 12: For the HARQ-ACK feedback for the invalid PDSCH due to collision with semi-static TDD DL/UL configuration, the mechanisms defined in Rel-17 and Rel-18 can be reused:

- If HARQ bundling in time domain is not configured for a serving cell, the HARQ-ACK bit for an invalid PDSCH is set to NACK.

- If HARQ bundling in time domain is configured for a serving cell, Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case.

- If the group is empty or filled with only invalid PDSCH(s), HARQ-ACK bit for the bundling group is set to NACK

Proposal 13: If a DCI format 1_3 is transmitted with fields repurposed for SCell dormancy indication and schedules one or more PDSCHs, and if there are multiple SLIVs in the serving cell with smallest cell index with invalid FDRA, the HARQ-ACK bits for SCell dormancy indication is ACK for TB1. The remaining SLIVs are assumed with NACK.

Proposal 14: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs scheduled by a DCI format 1_3, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_3 among the set of co-scheduled PDSCHs including the invalid PDSCH due to collision with uplink symbol.

# Reference

- 3GPP TSG RAN Meeting #105, Melbourne, Australia, Sep 9-12 2024, New WID: Multi-carrier enhancements for NR Phase 2, RP-242408.

- 3GPP TSG RAN WG1 Meeting #118bis, Hefei, China, October 14th – 18th, 2024, Feature lead summary #5 on multi-cell scheduling with a single DCI, R1-2409164.

- R1-2410732, Feature lead summary #4 on multi-cell scheduling with a single DCI Moderator, Orlando, US, November 18th – 22nd, 2024

- 3GPP TS 38.213 V18.5.0 (2024-12), Physical layer procedures for control

- R1-2410500, Multi-cell PUSCH/PDSCH scheduling with a single DCI, Qualcomm Incorporated

# Appendix

Table 5: Number of bits for each field in DCI format 0_3 for a schedulable cell in Rel-18 for example[5]

| - Fields | - Number of bits |
| --- | --- |
| - Identifier for DCI formats | - 1 |
| - Scheduled cell set indicator | - 1 |
| - Scheduled cells indicator | - Nscheduledcellindicator |
| - Bandwidth part indicator | - 0 |
| - Frequency domain resource assignment | - NFDRA |
| - Time domain resource assignment | - 5 |
| - Frequency hopping flag | - 1 |
| - Modulation and coding scheme | - NMCS |
| - New data indicator | - NNDI |
| - Redundancy version | - NRV |
| - HARQ process number | - NHPN |
| - 1st Downlink assignment index | - 1 |
| - 2nd Downlink assignment index | - 0 |
| - TPC for scheduled PUSCH | - 8 |
| - SRS resource indicator | - 1 (Type1A) |
| - Precoding information and number of layers | - 0 (Type1A) |
| - Antenna port(s) | - 2 (Type1A) |
| - SRS request | - 0 |
| - SRS offset indicator | - 0 |
| - CSI request | - 0 |
| - PTRS-DMRS association | - 0 |
| - Beta_offset indicator | - 0 |
| - DMRS sequence initialization | - 1 |
| - UL-SCH indicator | - 1 |
| - ChannelAccess-CPext-CAPC | - 0 |
| - Open-loop power control parameter set indication | - 0 |
| - Priority indicator | - 0 |
| - Minimum applicable scheduling offset indicator | - 0 |
| - SCell dormancy indication | - 0 |
| - PDCCH monitoring adaptation indication | - 0 |
|  | - Total = 22+NMCS+NNDI+NRV+NHPN+Nscheduledcellindicator |

Table 6: Number of bits for each field in DCI format 1_3 for a schedulable cell in Rel-18 for example[5]

| - Fields | - Number of bits |
| --- | --- |
| - Identifier for DCI formats | - 1 |
| - Scheduled cell set indicator | - 1 |
| - Scheduled cells indicator | - Nscheduledcellindicator |
| - Bandwidth part indicator | - 0 |
| - Frequency domain resource assignment | - NFDRA |
| - Time domain resource assignment | - 5 |
| - VRB-to-PRB mapping | - 0 |
| - PRB bundling size indicator | - 0 |
| - Rate matching indicator | - 1 |
| - ZP CSI-RS trigger | - 1 |
| - Modulation and coding scheme | - NMCS |
| - New data indicator | - NNDI |
| - Redundancy version | - NRV |
| - HARQ process number | - NHPN |
| - Downlink assignment index | - 4 |
| - TPC for scheduled PUCCH | - 2 |
| - PUCCH resource indicator | - 3 |
| - PDSCH-to-HARQ_feedback timing indicator | - 1 |
| - One-shot HARQ-ACK request | - 0 |
| - Enhanced Type 3 codebook indicator | - 0 |
| - HARQ-ACK retransmission indicator | - 0 |
| - Antenna port(s) | - 2 (Type1A) |
| - Transmission configuration indication | - 0 |
| - SRS request | - 1 |
| - SRS offset indicator | - 1 |
| - DMRS sequence initialization | - 1 |
| - Priority indicator | - 0 |
| - ChannelAccess-CPext | - 0 |
| - Minimum applicable scheduling offset indicator | - 0 |
| - SCell dormancy indication | - 0 |
| - PDCCH monitoring adaptation indication | - 0 |
| - PUCCH Cell indicator | - 0 |
|  | - Total = 24+NMCS+NNDI+NRV+NHPN+ Nscheduledcellindicator |