3GPP TSG RAN WG1 #120	R1-2500397

Athens, Greece, February 17th - 21st, 2025

Source:	Nokia

Title:	AIoT Other aspects incl. multiplexing/multiple access, scheduling information, and timing relationships

Agenda item:	9.4.4

Document for:	Discussion and Decision

# Introduction

At RAN#106, a new work item “Solutions for Ambient IoT (Internet of Things) in NR” (Ambient_IoT_Solutions) was approved [1].

The following objective(s) is/are relevant for the present agenda item:

- PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769.

# Discussion

## Scheduling of D2R transmission

In our understanding, full scheduling of D2R transmissions applies to the following cases (see Figure 6.3.1-1 in [2]):

- transmission of Msg1 in contention-free random access (A-IoT paging message schedules Msg1)

- transmission of Msg3 (Msg2 schedules Msg3)

- transmission of D2R data transmission in Step C2 (scheduled by R2D data transmission in Step C1)

Transmission of Msg1 in contention-based random access, on the other hand, is not fully scheduled since the device selects transmission resources randomly within constraints imposed by specification, system configuration and potentially signaling by the reader.

Regarding scheduling of D2R transmissions, the following two agreements have been reached during the study item:

Agreement Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

Agreement For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

Some of the potential information fields depend on decisions in other agenda items:

- MCS-like information

- Repetitions

### Contents

In the following we’ll discuss the remaining information items one by one.

#### Time domain resources

Regarding time domain, the following agreement was reached during the study item:

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

If pure option 1 is adopted, then indication of time domain resources by signaling may not be needed. However, this would restrict the reader’s scheduling flexibility. We therefore propose

Proposal 1: For D2R scheduling, time domain resources shall be indicated by signaling in the corresponding PRDCH.

#### Frequency domain resources, chip duration

Observation 1: According to the WID, both TDMA and FDMA are supported for D2R.

It follows that the device has to determine the frequency domain resources for its D2R transmission. To provide maximum scheduling flexibility to the reader, this should be based on signaling by the reader.

Proposal 2: For D2R scheduling, frequency domain resources shall be indicated by signaling in the corresponding PRDCH.

FDMA will be based on the device performing a small frequency shift. This small frequency shift can be characterized by a shift factor according to the following agreement reached during the study item:

Agreement Regarding small frequency shift factor, capture in the TR that: - For study purposes - For BPSK and OOK, the small-frequency-shift factor is defined as the ratio of bit length to two times the chip length - The value of small-frequency-shift factor depends on details of design, which according to proposals considered could be e.g. - The repetition number of Manchester codeword within one information bit duration - The M value of Miller in Figure 6-12 of UHF RFID standard - The number of square-wave periods within one information bit duration

Indication of an FDMA frequency domain resource is therefore equivalent to indication of a small frequency shift factor.

According to the WID, only BPSK and OOK modulations are supported for D2R, therefore the following definition from this agreement applies:

small-frequency-shift factor is defined as the ratio of bit length to two times the chip length

It follows that an FDMA frequency domain resource can be indicated indirectly e.g. by signaling the chip length (provided the bit length is known to the device).

Observation 2: For D2R scheduling, the FDMA frequency domain resource can be indicated indirectly by signaling the chip duration and vice versa (assuming the bit duration is known).

#### ID associated with device(s)

Such identification information is potentially needed not just for D2R scheduling, but also for R2D reception, according to the following agreement reached during the study item:

Agreement For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) - FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

The question then arises whether the “ID associated with device(s)” for a D2R transmission is always identical to the “ID associated with device(s)” for reception of the corresponding R2D transmission. If these IDs are always identical for a given R2D transmission, then separate indication for D2R scheduling is not required.

If the R2D transmission targets a single device then it is presumably the same device which is expected to respond on the scheduled resources. If, on the other hand, the R2D transmission targets multiple devices then it is conceivable that a single ID in the R2D transmission identifies a group of devices intended to receive the R2D while the same R2D transmission schedules transmissions of multiple devices by indicating dedicated time and frequency resources along with an ID for each device that is expected to respond.

Proposal 3: RAN1 to discuss whether for a given R2D transmission, “ID associated with device(s)” for R2D reception and for D2R scheduling is identical.

#### D2R “amble” configuration

Regarding the inclusion of midamble and postamble in D2R transmissions, the following agreement was made during the study item:

Agreement - The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options: - Option 1: D2R preamble only - Option 2: D2R preamble + X midamble(s), where X 1 - Option 3: D2R preamble + postamble - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 - For the above options, companies are encouraged to report at least the following: - Purpose(s) of the preamble, midamble and postamble - Whether companies assume multiple options can be supported

If multiple configurations are supported (e.g. presence/number of midamble(s) depends on factors such message size/message duration, SFO of the device) and the selection of the configuration is performed by the reader then the selected configuration will need to be indicated by the reader in the D2R scheduling information.

Proposal 4: For D2R scheduling, if multiple D2R “amble” configurations are supported and selection between them is up to the reader then amble configuration shall be indicated by signaling in the corresponding PRDCH.

### Signaling layer

One of the open issues is “whether higher-layer signaling and/or L1 R2D control signaling is used” for indicating D2R scheduling information. During the study item discussions, two potential benefits were advanced for L1 R2D control signaling:

- If NAK is supported to indicate a device’s failure to decode R2D data then a device which successfully decodes L1 R2D control but fails to decode R2D data can use resources indicated in the L1 R2D control to transmit NAK.

- The device can start earlier to prepare itself for the D2R transmission, namely after decoding L1 R2D control and before decoding the R2D data.

However, neither of these advantages seems compelling at this point. In our understanding, such a NAK mechanism has not been agreed, and it is not clear what exact preparations the device could start earlier.

On the other hand, including D2R scheduling information in L1 R2D control signaling, if supported, would increase the size of the L1 R2D control and hence the probably that its decoding by the device fails.

Proposal 5: D2R scheduling information is indicated by higher layer signaling.

## Timing Relationships

### D2R response for R2D transmissions

RAN1 defined the following terminologies for the discussions, which is captured from [2]

A-IoT processing time aspects are studied in terms of the following timing relationships: TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device.

TD2R_max would not necessarily be limited to the timing relationship between A-IoT Msg2 response to A-IoT Msg1. This maximum time could be between a D2R transmission response to a R2D transmission. Also, RAN1 studied the time interval between a R2D transmission and its corresponding R2D transmission with two options [2].

Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

In our understanding, the two options are not exclusive to each other. Both options would be necessary. From the reader side, it should take action if it does not receive any response from a target A-IoT device after a R2D transmission. If the reader does not receive responses, the reader may retransmit R2D signal. After a maximum time, the reader needs to decide that the A-IoT device failed to receive the R2D, or the D2R signal power was too weak to be detected. Thus, option 1 should be supported.

Observation 3: The reader cannot wait until it receives a response D2R message after a transmission of R2D signal requiring the response from the device.

Proposal 6: RAN1 should define a maximum time between a R2D transmission and its corresponding D2R reception so that the reader can decide the failure of either R2D reception at the device or D2R reception at the reader.

Option 2 is somewhat related to scheduling feature. For the D2R transmissions, NR should support scheduling mechanism so that the reader can indicate the device to transmit data at a specific timing although it is not aligned with NR subframes and/or symbols. In RFID communications, it should be noted that the tag does not always immediately transmit data after receiving signals from a reader. For example, a low power delayed circuit or low power MCU can be used to support a delayed transmission. In case the device is completely passive without any additional functionality, a reader control the transmission timing based on Slotted-Aloha. The reader pulses energy at each slot, and each device backscatters its selected slot. In order for this, the device should be silent except for the selected slot even though it receives a RF signal. It is implemented by using a pseudo-random number generator to select a random slot number, slot counter, and a state machine to be silent at unselected slots by counting slots and process at the selected slot. RAN1 needs to discuss the baseline implementations to discuss scheduling issues.

### R2D/D2R ending time

RAN1 studied ending time of R2D reception at the device based on two different options from the following agreements, which are also captured in [2].

Agreement

To determine or derive the end of PRDCH transmission, study at least following options:

- Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH.

- Option 2: Based on R2D control information.

Option 1 can provide more flexibility to the reader as the payload size could be variable up to the reader’s decision especially for the consecutive transmissions of PRDCHs. Also, the device can use the postamble for further timing adjustments, which may improve the PRDCH decoding performance.

Proposal 7: RAN1 supports R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH

Also, RAN1 studied the ending time indication for D2R reception based on the following agreement.

Agreement

For the reader to acquire the end of PDRCH transmission, study at least following options:

- Option 1: D2R postamble immediately follows the PDRCH

- Option 2: Based on control information

In contrast to the R2D reception, D2R data transmission resource should be well controlled by the reader, so the reader should be able to know the ending time without any further information provided by the device. Thus, the justification on the necessity of this feature is unclear at the moment.

# Conclusion

In this contribution we have made the following observations and proposals:

Proposal 1: For D2R scheduling, time domain resources shall be indicated by signaling in the corresponding PRDCH.

Observation 1: According to the WID, both TDMA and FDMA are supported for D2R.

Proposal 2: For D2R scheduling, frequency domain resources shall be indicated by signaling in the corresponding PRDCH.

Observation 2: For D2R scheduling, the FDMA frequency domain resource can be indicated indirectly by signaling the chip duration and vice versa (assuming the bit duration is known).

Proposal 3: RAN1 to discuss whether for a given R2D transmission, “ID associated with device(s)” for R2D reception and for D2R scheduling is identical.

Proposal 4: For D2R scheduling, if multiple D2R “amble” configurations are supported and selection between them is up to the reader then amble configuration shall be indicated by signaling in the corresponding PRDCH.

Proposal 5: D2R scheduling information is indicated by higher layer signaling.

Observation 3: The reader cannot wait until it receives a response D2R message after a transmission of R2D signal requiring the response from the device.

Proposal 6: RAN1 should define a maximum time between a R2D transmission and its corresponding D2R reception so that the reader can decide the failure of either R2D reception at the device or D2R reception at the reader.

Proposal 7: RAN1 supports R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH

# References

- RP-243326, ” New WID on Rel-19 Ambient IoT”, RAN#106

- TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”