3GPP TSG RAN WG1 Meeting #120	R1-2500409

Athens, Greece, February 17th – 21st, 2025

Source:	Intel Corporation

Title:	Discussion on channel modeling verification for 7-24 GHz

Agenda item:	9.8.1

Document for:	Discussion

# Introduction

In RAN Plenary #102, study for channel modeling verification for 7 – 24 GHz was approved [1]. The Study includes two objectives as described below.

- Validate using measurements the channel model of TR38.901 at least for 7-24 GHz

  - Note: Only stochastic channel model is considered for the validation.

  - Note: The validation may consider all existing scenarios: UMi-street canyon, UMa, Indoor-Office, RMa and Indoor-Factory.

- Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios:

  - Near-field propagation (with consideration being given to consistency between near-field and far-field)

  - Spatial non-stationarity

Note 1: Continuity of the channel model in the frequency domain below 7 GHz and above 24 GHz shall be ensured.

Note 2: Mathematical and/or theoretical aspects (if any) may be studied before results of measurement campaigns are available. While measurement results may be available and submitted at any time, the study of measurement results may start later (e.g., Q3 2024).

In this document, we discuss the potential aspects that may require channel modeling validation effort for frequencies from 7 to 24 GHz.

# Discussion of modeling parameters

In RAN1 #119, RAN1 has made series of observations and conclusions on various modeling parameters. In this section, we provide comments and views on various modeling parameters.

Conclusions on Pathloss

Conclusion - Based on measurement data provided, RAN1 concludes pathloss models in TR38.901 for the following scenarios are validated and no updates to TR are made. - InH-Office LOS - InH-Office NLOS - RMa LOS - RMa NLOS - InF LOS - InF NLOS Agreement - For the following scenarios, further study path loss model parameters for at least 6 – 24 GHz frequency range. - UMi LOS - UMi NLOS - UMa LOS - UMa NLOS

Based on measurement results that we have provided in previous meeting in [3] and measurement data provided by other companies in [4], we believe RAN1 could additional conclude that pathloss modeling for UMi Street Canyon LOS/NLOS do not need to be updated.

Proposal 1:

- Confirm that no pathloss updates are needed for UMi Street Canyon LOS/NLOS.

Conclusions on Delay Spread

Observation - For the following scenarios, some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR. - InF LOS - InF NLOS Conclusion - For the following scenarios, there is no consensus to update delay models due to lack of consistent and significant observed difference between model and measurements. - InF LOS - InF NLOS Agreement - For the following scenarios, further study delay spread parameters for at least 6 – 24 GHz frequency range. - UMi LOS - UMi NLOS - InH-Office LOS - InH-Office NLOS - UMa LOS - UMa NLOS

Based on measurement results that we have provided in [3] and measurement data provided by other companies in[4], at least UMa NLOS scenarios seem to be somewhat off compared to parameters defined in TR38.901. Measurement reported in [3] shows average of -7.746 in logarithmic scale, other companies have provided average values such as -7.53, -7.695 for NLOS cases which is lower value compared to parameters specified in TR38.901.

The delay spread values have impact to system by changing the frequency selectivity of the system and impacting choice of the subcarrier spacing feasible for systems that are to be deployed in the frequencies of interest. Therefore, important to get the value correct.

Based on results provided, we recommend reviewing the need to reducing the delay spread value for UMa LOS and NLOS scenario.

Proposal 2:

- Update the delay spread of UMa NLOS scenarios for 7 to 24 GHz based on measurements provided in R1-2407251.

  - FFS how to handle the frequency continuity beyond the 7 to 24 GHz

Parameter Updates

For some parameters identified to be updated. RAN1 must also discuss how to achieve frequency continuity while providing the updates to the modeling parameters for 7-24 GHz. We think there are three main resolutions to this issue.

- Option 1) Assuming measurements are available for the entire frequency range or at least available for FR1 and 7-24GHz range, update the value for the entire frequency range. If measurements are not available for other frequency ranges, do not update (keep as it is in the current TR).

- Option 2) Try to check if some smoothing or inter/extrapolation of the parameters is feasible, if feasible create new set of parameters that maintain frequency continuity

- Option 3) Make an exception and update the parameters for a specific frequency range.

From our understanding option 3, making an exception to frequency continuity, is something that should be only considered for truly exceptional cases with consensus, and SID should be updated to incorporate those specific exceptions. It might be possible to apply Option 3 for SMa scenario, as it is a scenario currently not defined in TR38.901, therefore does not have the issue with frequency continuity. Careful discussion is needed on whether Option 3 could be applied beyond SMa parameters.

An example of option 2 would be either extrapolating or interpolating measurement samples. One hypothetical example is if we had measurement of logarithmic mean delay spread of -7.74 at 10 GHz, the existing logarithmic mean delay spread was defined as -6.28 – 0.204*log10(fc), then we could update the delay spread for values from 6 to 24 GHz and interpolate the values for the transient frequencies. Figure 2-1 shows an illustration of interpolation scenario of Option 2. It can be easily seen that the results do not yield great frequency continuity (even if the values are somewhat continuous). Therefore, we believe Option 2 could be only applicable for very specific cases, where the interpolation seems reasonable.

Figure 2-1. Illustration of interpolation of measurement samples for frequencies of interest

This leaves RAN1 with Option 1. The main issue with Option 1 is that update of the parameters is only feasible if companies are able to bring in sufficient data set to update the entire frequency range. This can be quite challenging.

Proposal 3:

RAN1 to further discuss how to update channel modeling parameters if need for updating parameter(s) has been determined. Discuss among the following options:

- Option 1) Assuming measurements are available for the entire frequency range or at least available for FR1 and 7-24GHz range, update the value for the entire frequency range. If measurements are not available for other frequency ranges, keep as it is in the current TR.

- Option 2) Try to check if some smoothing or inter/extrapolation of the parameters is feasible, if feasible create new set of parameters that maintain frequency continuity

- Option 3) Make an exception and update the parameters for a specific frequency range.

- Note: applicability of which option may need to be discussed for each parameter currently under study

# Discussion on other channel modeling aspects

In RAN1 #119, the following on updates to angle calculation for CDL model was agreed.

Agreement - To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles. - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A. - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180]. - Down-select among options. - Option 1) - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model, - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel). - Option 2a) - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model, - s=minx≥0{x: ASx=ASdesired}. - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn - Option 2b) - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model, - s is chosen such that AS(ϕn,intermediate) = AS_desired - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn - Option 3) - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model, - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread. - FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

Among the two issues discussed in our previous Tdoc [3], issue #1 regarding separation of mean component has been addressed by the agreement. Issue #2 has 4 options for RAN1 to select from. Option 3 is what was present in the original TR and has the same problems that have been identified in our previous Tdoc. Option 1, 2a, and 2b all equally seem reasonable resolutions to the issue identified. Option 2b seems to be an editorial update of Option 1 and yield the same results. Among the three viable options, our preference would be Option 2a, which allows same implementation to be used by all companies, followed by Option 2b.

Proposal 4:

- RAN1 to agree to either Option 2a or Option 2b for handling of angles for MIMO simulation extension for CDL as part of the 7 – 24 GHz channel model validation SI.

# Discussion on Calibration for Channel Model Updates

During the original channel modeling study in Release 14, calibration of channel model was conducted among companies. While complete alignment of calibration results was not a requirement for successful completion of the SI, the calibration served useful tool for companies to cross check modeling implementation and potentially identify any issues with the model.

Similarly, we suggest RAN1 to conduct a similar calibration effort for Rel-19 channel modeling enhancement for 7-24 GHz SI. The calibration would be best effort work among companies of interest and alignment of simulated data should not be a requirement for successful completion of the SI. The calibration effort would merely aid companies to cross check implementation and potentially be a reference point for future discussions and study.

We suggest splitting the calibration into two categories, large scale calibration and full calibration, just as it was performed for Release 14. RAN1 could re-use the simulation parameters provided in [5] and [6]. The following are the simulation assumptions suggested for calibration of channel modeling.

Proposal 5:

- Interest companies in RAN1 to perform large scale and full calibration based on the following simulation assumptions.

Large scale calibration:

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon, Indoor-office, SMa |
| Sectorization | 3 sectors per cell site – 30, 150 and 270 degrees |
| BS antenna configurations | Mg = Ng = 1; (M,N,P) = (10,1,1), dV = 0.5lambda |
| BS port mapping | The 10 elements maps to one CRS port |
| BS antenna downtilt | 102 degrees for Uma and UMi Street Canyon 110 degrees for indoor |
| Antenna virtualization | DFT precoding according to TR 36.897 with application of panning and tilting angles |
| BS Tx power | 44 dBm for UMi-Street Canyon, 49 for Uma at 6GHz 35 dBm at 30GHz, 60 and 70 GHz for Uma and UMi-Street canyon 24 dBm for Indoor for all carrier frequencies |
| Bandwidth | 20MHz for 6GHz, 8GHz, and 100MHz for 15GHz, 30Ghz, 60 and 70 GHz |
| MS antenna configurations | 1 element (vertically polarized), Omnidirectional |
| Handover margin (for calibration) | 0dB |
| UE distribution | Following TR36.873 for UMa and UMi, 2D dropping uniform dropping for indoor with minimum distance of 0 m |
| UE attachment | Based on pathloss considering LOS angle |
| UE noise figure | 9 dB |
| Fast fading channel | Fast fading channel is not modelled |
| O2I penetration loss | 50% low loss and 50% high loss |
| Carrier Frequency | 6 Ghz, 8GHz, 15GHz 30 Ghz, 70GHz |
| Wrapping method for Uma and UMi | geographical distance based wrapping (mandatory) radio distance (optional) |
| Metrics | 1) Coupling loss – serving cell (based on LOS pathloss) |
|  | 2) Geometry (based on LOS pathloss) with and without white noise |

Full calibration: unless otherwise stated, the agreed large-scale calibration parameters are reused:

| Parameter | Values |
| --- | --- |
| Scenarios | 3D-UMa, 3D-UMi-street Canyon, Indoor-office |
| Carrier Frequency | 6 Ghz, 30 Ghz, 60GHz, 70GHz |
| BS antenna configurations | Config 1: M=4,N=4,P=2, Mg=1, Ng =2, dH = dV = 0.5lambda, dH,g=dV,g=2.5lambda … calibration metrics 1), 2), 3) are calibrated Config 2: Mg=Ng=1, M=N=2, P = 1 … calibration metrics 1), 2), 4) are calibrated |
| BS port mapping | Config 1: all 16 elements for each polarization on each panel maps to one CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations Config 2: each antenna maps to one CRS port |
| MS antenna configurations | Mg=Ng=1, M=N=1, P=2 |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Polarized antenna modelling | Model-2 in TR36.873 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UE antenna pattern | Omnidirectional |
| Metrics | 1) Coupling loss – serving cell |
|  | 2) Wideband SIR before receiver without noise |
|  | 3) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) from the serving cell (according to circular angle spread definition of TR 25.996) |
|  | 4) CDF of largest (1st) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale CDF of smallest (2nd) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale CDF of the ratio between the largest PRB singular value and the smallest PRB singular value (serving cell) at t=0 plotted in 10*log10 scale Note: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. |

# Conclusions

In this contribution, we discussed the potential aspects that may require channel modeling validation effort for frequencies from 7 to 24 GHz. The following is a summary of proposals and observations made in this contribution.

Proposal 1:

- Confirm that no pathloss updates are needed for UMi Street Canyon LOS/NLOS.

Proposal 2:

- Update the delay spread of UMa NLOS scenarios for 7 to 24 GHz based on measurements provided in R1-2407251.

  - FFS how to handle the frequency continuity beyond the 7 to 24 GHz

Proposal 3:

RAN1 to further discuss how to update channel modeling parameters if need for updating parameter(s) has been determined. Discuss among the following options:

- Option 1) Assuming measurements are available for the entire frequency range or at least available for FR1 and 7-24GHz range, update the value for the entire frequency range. If measurements are not available for other frequency ranges, keep as it is in the current TR.

- Option 2) Try to check if some smoothing or inter/extrapolation of the parameters is feasible, if feasible create new set of parameters that maintain frequency continuity

- Option 3) Make an exception and update the parameters for a specific frequency range.

- Note: applicability of which option may need to be discussed for each parameter currently under study

Proposal 4:

- RAN1 to agree to either Option 2a or Option 2b for handling of angles for MIMO simulation extension for CDL as part of the 7 – 24 GHz channel model validation SI.

Proposal 5:

- Interest companies in RAN1 to perform large scale and full calibration based on the following simulation assumptions.

Large scale calibration:

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon, Indoor-office, SMa |
| Sectorization | 3 sectors per cell site – 30, 150 and 270 degrees |
| BS antenna configurations | Mg = Ng = 1; (M,N,P) = (10,1,1), dV = 0.5lambda |
| BS port mapping | The 10 elements maps to one CRS port |
| BS antenna downtilt | 102 degrees for Uma and UMi Street Canyon 110 degrees for indoor |
| Antenna virtualization | DFT precoding according to TR 36.897 with application of panning and tilting angles |
| BS Tx power | 44 dBm for UMi-Street Canyon, 49 for Uma at 6GHz 35 dBm at 30GHz, 60 and 70 GHz for Uma and UMi-Street canyon 24 dBm for Indoor for all carrier frequencies |
| Bandwidth | 20MHz for 6GHz, 8GHz, and 100MHz for 15GHz, 30Ghz, 60 and 70 GHz |
| MS antenna configurations | 1 element (vertically polarized), Omnidirectional |
| Handover margin (for calibration) | 0dB |
| UE distribution | Following TR36.873 for UMa and UMi, 2D dropping uniform dropping for indoor with minimum distance of 0 m |
| UE attachment | Based on pathloss considering LOS angle |
| UE noise figure | 9 dB |
| Fast fading channel | Fast fading channel is not modelled |
| O2I penetration loss | 50% low loss and 50% high loss |
| Carrier Frequency | 6 Ghz, 8GHz, 15GHz 30 Ghz, 70GHz |
| Wrapping method for Uma and UMi | geographical distance based wrapping (mandatory) radio distance (optional) |
| Metrics | 1) Coupling loss – serving cell (based on LOS pathloss) |
|  | 2) Geometry (based on LOS pathloss) with and without white noise |

Full calibration: unless otherwise stated, the agreed large-scale calibration parameters are reused:

| Parameter | Values |
| --- | --- |
| Scenarios | 3D-UMa, 3D-UMi-street Canyon, Indoor-office |
| Carrier Frequency | 6 Ghz, 30 Ghz, 60GHz, 70GHz |
| BS antenna configurations | Config 1: M=4,N=4,P=2, Mg=1, Ng =2, dH = dV = 0.5lambda, dH,g=dV,g=2.5lambda … calibration metrics 1), 2), 3) are calibrated Config 2: Mg=Ng=1, M=N=2, P = 1 … calibration metrics 1), 2), 4) are calibrated |
| BS port mapping | Config 1: all 16 elements for each polarization on each panel maps to one CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations Config 2: each antenna maps to one CRS port |
| MS antenna configurations | Mg=Ng=1, M=N=1, P=2 |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Polarized antenna modelling | Model-2 in TR36.873 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UE antenna pattern | Omnidirectional |
| Metrics | 1) Coupling loss – serving cell |
|  | 2) Wideband SIR before receiver without noise |
|  | 3) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) from the serving cell (according to circular angle spread definition of TR 25.996) |
|  | 4) CDF of largest (1st) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale CDF of smallest (2nd) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale CDF of the ratio between the largest PRB singular value and the smallest PRB singular value (serving cell) at t=0 plotted in 10*log10 scale Note: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. |

# References

- RP-234018, “New SID: Study on channel modelling enhancements for 7-24GHz for NR”, Nokia, Nokia Shanghai Bell, RAN#102, December 2023.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”, V17.1.0 (2023-12).

- R1-2409738, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation, RAN1 #119, November 2024.

- R1-2410631, “Data source descriptions for 7 – 24 GHz SI,” Moderator (Intel Corporation), RAN1 #119, November 2024.

- R1-163641, “WF on simulation assumption for large-scale calibration,” Samsung, Nokia, RAN1 #84bis, April, 2016.

- R1-163642, “WF on simulation assumption for full calibration,” Samsung, Nokia, RAN1 #84bis, April, 2016.