3GPP TSG RAN WG1 #120		R1-2500428

Athens, Greece, February 17th – 21st, 2025

Source:	OPPO

Title:	Discussion on LS on UL 8Tx

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, RAN2 sent a LS [1] on UL 8Tx transmission with two TBs to RAN1 with following issue:

1. To support UL 8Tx in the MAC specification, RAN2 made the following assumption regarding how to model UL 8Tx operation from MAC point of view to minimize specification impacts.

For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.

2. For UL 8Tx 2TB transmission scheduled by one DCI, if MAC entity only generates a single TB due to the uplink skipping, would there be any issue for PHY layer?

To RAN1

ACTION:   RAN2 respectfully asks RAN1 to provide feedback, if any, on the modelling assumption in the first item and answer the question in the second item.

Discussion

Regarding issue 1, the modeling assumption for UL 8Tx 2TB transmission by one DCI in RAN2 is the same as that for downlink 2TB transmission. It is consistent with current RAN1 specification and seems no issue from RAN1 perspective.

Regarding issue 2, from physical layer perspective, the UE can only transmit one TB (i.e. partial layers) if another TB is skipped by MAC entity. From RAN1 procedure, the transmission of one TB would not be impacted by the skipping of another TB, e.g. the UE may transmit less layers than that scheduled by DCI. There is even potential gain from power boosting on remaining layers in some case. Though gNB would still detect all the layers and the skipped TB may be treated as NACK resulting in HARQ retransmission, it is an issue in higher layer rather than physical layer.

Proposal: For issue 2 in the RAN2 LS, if MAC entity only generates a single TB due to the uplink skipping, there is no issue for transmission of one TB in PHY layer.

Conclusion

In this contribution, we discuss the issues in RAN2 LS and also the possible response to RAN2.

Reference

[1] R2-2411220, LS on UL 8Tx, RAN2(Samsung), RAN2#128 meeting