3GPP TSG RAN WG1 #120                                         R1-2500455

Athens, Greece, February 17th – 21st, 2025

Source:	OPPO

Title:	Discussion on SBFD random access operation

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

In RAN #104 meeting, the WID of “Evolution of NR duplex operation: Sub-band full duplex (SBFD)” was revised including the following objectives:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this contribution, we discuss the potential enhancements for random access in SBFD symbols in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode.

- PRACH transmission

## PRACH configuration

Regarding to the PRACH configurations for SBFD-aware UE, the following agreements were made in previous RAN1 meetings with details for further study:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. Agreement For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols. - Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon). - FFS the details FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s).

In legacy RACH resource configuration, the time domain resource and preamble domain resource are configured via RRC parameter prach-ConfigurationIndex, which points to the random access configuration tables defined in TS 38.211. The PRACH frequency resource allocations are configured via RRC parameter msg1-FrequencyStart and msg1-FDM, where msg1-FrequencyStart indicates the offset of the lowest PRACH transmission occasion in frequency domain with respective to PRB 0 and msg1-FDM configures the number of PRACH transmission occasions FDMed in one time instance. When msg1-FDM is larger than 1, the PRACH transmission occasions are continuously distributed in the frequency domain. Below, we discuss further details for option 1 and option 2.

### RACH configuration option 1:

One remaining issue to be discussed is how to perform separate power control for PRACH transmission in additional ROs and legacy ROs. In legacy, the PRACH transmission power determination employs open loop power control mechanism with power ramping among PRACH retransmissions as following:

PPRACH,b,f,c(i)=minPCMAX,f,c(i), PPRACH,target,f,c+PLb,f,c [dBm],

Where PPRACH,target,f,c is the PRACH target reception power provided by higher layer as:

PREAMBLE_RECEIVED_TARGET_POWER=preambleReceivedTargetPower+DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP. preambleReceivedTargetPower is the target power level at the network receiver side. Once the maximum number of PRACH attempt (preambleTransMax) is reached, random access failure is declared.

The following agreement was made in last RAN1 meeting:

Agreement For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

Regarding to the FFS on whether other parameters besides preambleReceivedTargetPower need to be separately configured or not, the following agreements made in RAN2 #127bis meeting need to be taken into consideration. The following agreement allows RO type switch between additional ROs and legacy ROs in one RACH procedure and the switching condition is “after certain (configured) number of times of RACH attempt in SBFD RACH occasions”. To achieve this mechanism for RO switch, the preambleTransMax needs to be separately configured, e.g., preambleTransMax_additionalRO defines the maximum number of RACH attempt in additional RO and preambleTransMax_total represents the maximum number of RACH attempt in both additional RO and legacy RO.

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

Regarding to whether powerRampingStep needs to be separately configured or not, we do not have strong preference. If considering the different CLI status in additional RO and legacy RO, the powerRampingStep can be separately discussed for flexibility. And in such a case, when changing from additional RO to legacy RO, an additional power offset needs to be added to PRACH transmission power to compensate the power ramping difference between additional RO and legacy RO, which is similar to the mechanism of 2-step RACH switching to 4-step RACH.

Proposal 1: For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate configuration of preambleTransMax and powerRampingStep for additional ROs and legacy ROs.

Proposal 2: If separate configuration of powerRampingStep for additional ROs and legacy ROs is supported, when switching PRACH transmissions from additional RO to legacy RO, an additional power offset needs to added to PRACH transmission power to compensate the power ramping difference between additional ROs and legacy ROs, i.e.,

- Set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower+DELTA_PREAMBLE+(PREAMBLE_POWER_RAMPING_COUNTER-1)*PREAMBLE_POWER_RAMPING_STEP+POWER_OFFSET_ additionalRO, where POWER_OFFSET_additionalRO=(PREAMBLE_POWER_RAMPING_COUNTER-1)*(additionalRO _ PREAMBLE_POWER_RAMPING_STEP- PREAMBLE_POWER_RAMPING_STEP).

### RACH configuration option 2:

Option 2 aims to use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration. Regarding on PRACH configuration table for RACH configuration option 2, the following agreement was made in last RAN1 meeting:

Agreement For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, down-select from the following options: - Alt-1: introduce a subframe/slot offset (similar as prach-ConfigurationSOffset-IAB) to determine the subframe/slot number for ROs in SBFD symbols for FR1/FR2 - FFS: details of the subframe/slot offset - Alt-2: introduce a new parameter indicating from an entry from a pre-defined/configured table to derive additional subframe/slot numbers. - Details are FFS. - Alt-3: introduce a bitmap indicating or puncturing subframe(s)/slot(s) for additional ROs in SBFD symbols. - Details are FFS. - Alt-4: no enhancement

In current specification, the PRACH configuration tables are given by Table 6.3.3.2-2 to Table 6.3.3.2-4 in TS 38.211 which configures the PRACH resources (time domain and preamble domain) for FR1 FDD, FR1 TDD and FR2 respectively. From the RACH configuration table, it can be observed that for PRACH format 0/3/A/B/C, the time domain resource is relatively flexible for SBFD operation. In last RAN1 meeting, companies supporting to introduce new parameter(s) to determine the slot/subframe number for ROs in SBFD symbols argue that PRACH format 0/3 with periodicity of larger than 10ms can be located only within subframe numbers 4 or 9 which are UL subframes in DDDSUDDSUU and DDDDDDDSUU. However, the goal to configure RO in SBFD symbols is to achieve more RACH occasions for PRACH transmission and in such a case, why do we need to configure RO periodicity larger than 10ms? We can simply shorten the RO periodicity. For PRACH format 1/2, the choices of time domain resources that can be configured from the table are limited, i.e., PRACH format 1 can be configured only in subframe 7 and PRACH format 2 can be configured only in subframe 6, which lacks of flexibility to support PRACH occasion in SBFD symbols. From another perspective, the duration of PRACH format 1/2 is 3ms/3.5ms, which is long enough and can be used for large and deep coverage. The motivation for further enhancement of PRACH format 1/2 in SBFD symbols is not strong. Some companies may argue that there would be a vast SBFD ROs overhead when using some PRACH configuration index to configure ROs in non-UL slots, however, as the argument itself says, this limitation appears only in some PRACH configuration index, not all indexes, why do we need such much flexibility for each PRACH configuration index? Anyway, the frame structure, the SBFD time/frequency domain configuration as well as the PRACH configuration index are configured by gNB, we can trust gNB to make the reasonable choice. With the above consideration, it seems that reusing the existing PRACH configuration table for TDD in FR1 and FR2 is enough.

Proposal 3: For RACH configuration Option 2 and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, support Alt 4: no enhancement.

For the SSB-RO mapping in option 2, the following agreement was made in last RAN1 meeting:

Agreement For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

For the case that additional ROs overlap with legacy ROs, similar issue is observed in legacy for the legacy ROs and additional ROs configured for other feature combinations, so this is not particular issue for SBFD and it is preferred to handle it by gNB configuration and do not pursue further optimization.

Proposal 4: For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt 1: do not optimize this case.

## Msg3 PUSCH

### Msg 3 power control

The following agreement was made in last RAN1 meeting for Msg 3 power control:

Agreement Study how to determine the Msg3 PUSCH power control in SBFD symbols and non-SBFD symbols based on at least the following parameters. - preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, TPC Command in RAR

In legacy, Msg 3 transmission power is determined based on the transmission power of the corresponding PRACH transmission, that is, for a PUSCH transmission occasion i, UE determines its transmission power as following:

PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l   [dBm]

Where,

- for a PUSCH (re)transmission corresponding to a RAR UL grant, , , and , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or  dB if msg3-DeltaPreamble is not provided, for carrier  of serving cell

- For

-	For ,  is a value of msg3-Alpha, when provided; otherwise,

- For closed-loop power control, if the UE receives a random access response message in response to a PRACH transmission on active UL BWP b of carrier f of serving cell c, then:

-	fb,f,c0,l=∆Prampup,b,f,c+δmsg2,b,f,c, where l=0 and

-	δmsg2,b,f,c is a TPC command value indicated in a random access response grant of the random access response message corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant of the random access response message corresponding to a MsgA transmission according to Type-2 random access procedure with RAR message(s) for fallbackRAR, on active UL BWP b of carrier f of serving cell c, and

-	 and ∆Prampup_requested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last random access preamble for carrier f in the serving cell c, MRB,b,f,cPUSCH(0) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for the first PUSCH transmission on active UL BWP b of carrier f of serving cell c, and ∆TF,b,f,c0 is the power adjustment of first PUSCH transmission on active UL BWP b of carrier f of serving cell c.

In addition, it has already been agreed to support separate configuration of preambleReceivedTargetPower for additional ROs and legacy ROs. For determination of Msg3 PUSCH power control in SBFD symbols and non-SBFD symbols, it is related with whether Msg3 PUSCH transmission should follow the same symbol type with the latest PRACH transmission, specifically,

- Case 1: If the latest PRACH transmission is on legacy RO, then gNB does not know whether the UE is SBFD aware UE or not, so gNB can only schedule Msg3 PUSCH in non-SBFD symbols as in legacy.

- Case 2: If the latest PRACH transmission is on additional RO, then gNB knows that the UE is SBFD aware UE, and here comes the question that whether to restrict the corresponding Msg3 PUSCH scheduling in SBFD symbols to align with the latest PRACH transmission?

  - Considering that the transmission power of Msg3 PUSCH is determined based on the power control parameter of PRACH (preambleReceivedTargetPower, total power ramp-up from the first to the last preamble), restrict Msg3 PUSCH transmission to be scheduled in SBFD symbols can make the power control mechanism simple and accurate. Otherwise if Msg3 PUSCH is scheduled in non-SBFD symbols, gNB would have limited knowledge on the power offset between PRACH in additional RO and Msg3 PUSCH in non-SBFD symbols and further optimizations would be needed.

Proposal 5: Msg3 PUSCH transmission follow the same symbol type with the latest PRACH transmission, that is:

- If the latest PRACH transmission is on legacy RO, then Msg3 PUSCH can be only scheduled in non-SBFD symbols;

- If the latest PRACH transmission is on additional RO, then Msg3 PUSCH can be only scheduled in SBFD symbols.

Under the above premise that Msg3 PUSCH transmission follow the same symbol type with the latest PRACH transmission, then for the following parameters:

- preambleReceivedTargetPower: since this parameter has already been agreed to be separately configured for additional RO and legacy RO, so for Msg3 PUSCH in SBFD symbols, its transmission power is determined based on preambleReceivedTargetPower for additional RO, otherwise, preambleReceivedTargetPower for legacy RO is used;

- msg3-DeltaPreamble/deltaPreamble: considering that preambleReceivedTargetPower has already been agreed to be separately configured for additional RO and legacy RO to cope with different CLI in SBFD symbols and non-SBFD symbols and msg3-DeltaPreamble/deltaPreamble only represents the power offset between Msg3 PUSCH and the corresponding PRACH, so it seems not necessary to separate configure msg3-DeltaPreamble/deltaPreamble for Msg3 PUSCH in SBFD symbols and non-SBFD symbols

- msg3-Alpha: considering that separate configuration of P0AlphaSets for PUSCH/PUCCH/SRS transmission in SBFD symbols and non-SBFD symbols has already been supported in AI9.3.1, it is natural to support separate configuration of msg3-Alpha for Msg3 PUSCH transmission in SBFD symbols and non-SBFD symbols

Agreement: For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

- TPC Command in RAR: Since Msg3 PUSCH can be only on SBFD symbols or non-SBFD symbols and is not allowed to across symbol type, so if gNB schedules Msg3 PUSCH in SBFD symbols, gNB can indicate one TPC command which is suitable for SBFD symbols, while if gNB schedules Msg3 PUSCH in non-SBFD symbols, gNB can indicate another TPC command which is more appropriate for non-SBFD symbols. With that, it seems not necessary to separate closed-loop power control for Msg3 PUSCH in two symbol types.

Proposal 6: For Msg3 PUSCH in SBFD symbols, its transmission power is determined based on preambleReceivedTargetPower for additional RO; While for Msg3 PUSCH in non-SBFD symbols, its transmission power is determined based on preambleReceivedTargetPower for legacy RO.

Observation 1: Considering that preambleReceivedTargetPower has already been agreed to be separately configured for additional RO and legacy RO to cope with different CLI levels in SBFD symbols and non-SBFD symbols so it is not necessary to separate configure msg3-DeltaPreamble/deltaPreamble for Msg3 PUSCH in SBFD symbols and non-SBFD symbols

Proposal 7: Support to separate configure msg3-Alpha for Msg3 PUSCH in SBFD symbols and non-SBFD symbols.

Observation 2: Considering that Msg3 PUSCH can be only on SBFD symbols or non-SBFD symbols and is not allowed to across symbol type, it is not necessary to separate closed-loop power control for Msg3 PUSCH in SBFD symbols and non-SBFD symbols.

### Msg 3 resource allocation

In current specification, for determining the frequency domain resource allocation for Msg3 PUSCH, the following is used:

TS 38.213 8.3 PUSCH scheduled by RAR UL grant If useInterlacePUCCH-PUSCH is not provided by BWP-UplinkCommon and BWP-UplinkDedicated, for determining the frequency domain resource allocation for the PUSCH transmission within the active UL BWP - if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the initial UL BWP is used - else, the RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

One issue is raised in last RAN1 meeting, that is if the frequency domain resource allocation for Msg3 PUSCH is still based on the RB numbering starting from the first RB of the active UL BWP, then some of the PRBs in UL usable PRBs in SBFD symbols may be not addressable. As shown in the following figure, for the case where active UL BWP is not initial BWP, since the UL usable PRB is the intersection of active UL BWP and UL subband, then there may exist some cases where all the PRBs of UL usable PRB are outside of FDRA range for Msg3 PUSCH and no PRB is addressable for Msg3 PUSCH. However, from our perspective, this issue can be solved by proper gNB configuration, e.g., configure the UL subband a bit lower to the bottom of the active UL BWP to ensure that there is addressable PRB for Msg3 PUSCH, or configure active UL BWP to include all RBs of the initial UL BWP and configure same SCS and same CP length for the active UL BWP and initial BWP to ensure all the UL usable PRBs are addressable for Msg3 PUSCH.

Proposal 8: For determining the frequency domain resource allocation for Msg3 PUSCH transmission in SBFD symbols, support to use legacy rule for RB numbering and FDRA interpretation.

- Conclusions

This contribution concludes with the following observations and proposals:

Proposal 1: For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate configuration of preambleTransMax and powerRampingStep for additional ROs and legacy ROs.

Proposal 2: If separate configuration of powerRampingStep for additional ROs and legacy ROs is supported, when switching PRACH transmissions from additional RO to legacy RO, an additional power offset needs to added to PRACH transmission power to compensate the power ramping difference between additional ROs and legacy ROs, i.e.,

- Set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower+DELTA_PREAMBLE+(PREAMBLE_POWER_RAMPING_COUNTER-1)*PREAMBLE_POWER_RAMPING_STEP+POWER_OFFSET_ additionalRO, where POWER_OFFSET_additionalRO=(PREAMBLE_POWER_RAMPING_COUNTER-1)*(additionalRO _ PREAMBLE_POWER_RAMPING_STEP- PREAMBLE_POWER_RAMPING_STEP).

Proposal 3: For RACH configuration Option 2 and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, support Alt 4: no enhancement.

Proposal 4: For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt 1: do not optimize this case.

Proposal 5: Msg3 PUSCH transmission follow the same symbol type with the latest PRACH transmission, that is:

- If the latest PRACH transmission is on legacy RO, then Msg3 PUSCH can be only scheduled in non-SBFD symbols;

- If the latest PRACH transmission is on additional RO, then Msg3 PUSCH can be only scheduled in SBFD symbols.

Proposal 6: For Msg3 PUSCH in SBFD symbols, its transmission power is determined based on preambleReceivedTargetPower for additional RO; While for Msg3 PUSCH in non-SBFD symbols, its transmission power is determined based on preambleReceivedTargetPower for legacy RO.

Observation 1: Considering that preambleReceivedTargetPower has already been agreed to be separately configured for additional RO and legacy RO to cope with different CLI levels in SBFD symbols and non-SBFD symbols so it is not necessary to separate configure msg3-DeltaPreamble/deltaPreamble for Msg3 PUSCH in SBFD symbols and non-SBFD symbols

Proposal 7: Support to separate configure msg3-Alpha for Msg3 PUSCH in SBFD symbols and non-SBFD symbols.

Observation 2: Considering that Msg3 PUSCH can be only on SBFD symbols or non-SBFD symbols and is not allowed to across symbol type, it is not necessary to separate closed-loop power control for Msg3 PUSCH in SBFD symbols and non-SBFD symbols.

Proposal 8: For determining the frequency domain resource allocation for Msg3 PUSCH transmission in SBFD symbols, support to use legacy rule for RB numbering and FDRA interpretation.

- References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- TS 38.213 v18.2.0 Physical layer procedures for control (Release 18)