3GPP TSG RAN WG1 #120		                                    R1-2500513

Athens, Greece, February 17th – 21st, 2025

Source:	Ofinno

Title:	On UE-initiated/event-driven beam management

Agenda Item:	9.2.1

Document for:	Discussion and Decision

# Introduction

The revised WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#105 meeting. The objectives related to UE-initiated/event-driven beam management are listed below:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we present our views on the remaining issues of UE-initiated/event-driven beam management.

# Trigger event detection

In RAN1#118b, the following agreement has been reached regarding the conditions to reset the event instance counting for Event 2.

Agreement Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

When a counter used for counting event instances of a new beam reaches a maximum value within a time window, the UE triggers a UE-initiated CSI report. If the condition(s) to reset the counter is not defined, the UE may continuously trigger UE-initiated CSI reports as long as the counter is greater than or equal to the maximum value. This may lead to unnecessary PUCCH and/or PUSCH transmissions of UE-initiated CSI reporting in Mode A and Mode B.

Some candidate conditions for resetting the counter were discussed during the last meeting. Among those, the following conditions may be considered for resetting the counter for Event-2 for UE initiated beam reporting:

- Reconfiguration/update of the new beams: After the UE receives an RRC reconfiguration message updating the list of new beams, if a new beam in the old beam RS list (e.g., before receiving the RRC reconfiguration message) is no longer in the new beam RS list, the counter associated with this new beam is reset. There is no need to reset the counter of a new beam which is configured in both the old beam RS list and the new beam RS list.

- Update of the indicated TCI state:

  - In scheme 1, if the QCL-RSs in the new indicated TCI state and in the old indicated TCI state are different, the counter associated with any new beam is reset.

  - In scheme 2, if the SSB which is QCL-ed with the QCL-RS in the new indicated TCI state and the SSB which is QCL-ed with the QCL-RS in the old indicated TCI state are different, the counter associated with any new beam is reset. Otherwise, the counter associated with any new beam is NOT reset.

- Transmission of UE-initiated CSI report: When the UE transmits UE-initiated CSI report indicating at least one new beam fulfilling the triggered condition, the counter(s) of the at least one new beam is reset. There is no need to reset the counter of the other new beams that the triggered condition is not satisfied. The counter(s) of those new beams can be reset, as proposed in the previous condition, if the base station decides to update the indicated TCI state by transmitting a DCI/MAC-CE.

- Expiration of the time window: Similar to BFD procedure, where the BFR counter is reset if the BFD timer (e.g., beamFailureDetectionTimer) expires, the counter of a new beam is reset if the time window associated with the new beam expires.

- Reconfiguration of threshold: In legacy systems, if any parameter (e.g., beamFailureDetectionTimer, beamFailureInstanceMaxCount) used to detect beam failure is reconfigured, the UE resets the BFI_COUNTER to zero. Similarly, in UE initiated beam reporting, if any parameter (e.g. threshold) used to detect the condition of the event is reconfigured, the counter associated with any new beam is reset.

In addition to the candidate conditions above, the following condition may also be considered for resetting the counter for Event-2 for UE initiated beam reporting:

- Reconfiguration of time window and maximum event instance count: Similar to legacy BFR counters, counters associated with UE initiated beam report should be reset, when configuration parameters, e.g., time window and max count, related to the counters are reconfigured.

- Regarding the triggering event determination for Event 2, the following conditions can be considered for resetting the counter for Event-2:

- Reconfiguration/update of the new beams

- Update of the indicated TCI state

- Transmission of UE-initiated CSI report

- Expiration of the time window

- Reconfiguration of threshold

- Reconfiguration of time window and/or maximum event instance count

In addition to candidate conditions for resetting the counter, the conditions to cancel triggered UE-initiated beam reporting are also needed. In some cases, there may be no longer a need to continue triggered UE-initiated beam reporting, and canceling the triggered UE-initiated beam reporting in such cases prevent unnecessary signaling (e.g., the first PUCCH and the second PUSCH transmission) of the UE-initiated beam reporting. This is similar to cancellation of triggered BFR procedure. Some candidate conditions to cancel a triggered UE-initiated beam reporting may be update of the indicated TCI state, transmission of the UE-initiated CSI report, deactivation of the SCell, BWP switch, and MAC reset if RAN2 is involved in UE-initiated beam reporting, and so on.

- The following conditions can be considered for cancellation of a triggered UE-initiated/event-driven beam reporting for Event 2:

- Update of the indicated TCI state

- Transmission of UE-initiated CSI report

- BWP switching

Figure 1 shows an existing beam failure recovery procedure. When a UE detects a beam failure on a cell (e.g., PCell, SCell), the indicated TCI state (e.g., TCI state 1) of the cell is likely to have a low radio link quality. When the indicated TCI state has a low radio link quality, the conditions of Event 1 and/or Event 2 are satisfied frequently during the ongoing beam failure recovery (e.g., between t1 and t2 in Figure 1), and the counter used for counting event instances of a new beam for UE-initiated beam reporting may reach the maximum value within the time window quickly. In another words, the wireless device may keep triggering the UE-initiated/event-driven beam report and transmit the first PUCCH and/or the second PUSCH multiple times leading to increased signaling. In our view, the ongoing beam failure recovery procedure is already trying to recover (or indicate) the potential blockage, and there is no need for a UE-initiated CSI report during the ongoing beam failure recovery. Especially, in the case of no carrier aggregation (e.g., only PCell is configured), when there is a beam failure on the PCell, the uplink channels are also likely to fail (the reason for resetting the uplink channels after beam failure recovery), so the first PUCCH and the second PUSCH for the UE-initiated beam reporting may fail.

In the legacy systems, 28 symbols after a beam failure recovery is completed, the UE applies the candidate reference signal identified for the beam failure recovery to PDCCH/PDSCH/A-CSI-RS receptions until application of a new indicated TCI state (e.g., between t2 and t5 in Figure 1). However, since the indicated TCI state for the cell is still TCI state 1 between t2 and t5 though it is not used, the conditions of Event 1 and/or Event 2 are still satisfied frequently even after successful completion of a beam failure recovery, and thus the UE triggers UE-initiated/event-driven beam report. One solution can be to restrict evaluation of the conditions for Event 1 and/or Event 2 after beam failure recovery and until a new TCI state is indicated (e.g., no trigger-event condition evaluation between t2 and t5), or another solution can be to evaluate the conditions Event 1 and/or Event 2 based on the candidate reference signal instead of the indicated TCI state.

- : Beam failure recovery procedure

- Specify necessary enhancements to address beam failure recovery procedure when UE is configured with UE-initiated/event-driven beam report procedure at least based on Event -1 or Event-2.

- For example, the UE-initiated/event-driven beam report procedure is not performed during an ongoing beam failure recovery procedure. (e.g., no condition evaluation, no first PUCCH and/or second PUSCH transmission)

- Study whether UE-initiated/event-driven beam report procedure (e.g., condition evaluation, first PUCCH transmission, second PUSCH transmission, and so on) is impacted after a beam failure recovery procedure is successfully completed.

- Option 1: The UE-initiated/event-driven beam report procedure is restricted until a new TCI state is indicated.

- Option 2: The RS for current beam used for trigger-event condition is the candidate reference signal identified for the beam failure recovery.

In RAN1#119, the following agreements [2] have been reached regarding RS measurement in Event 1.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement - Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced. - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

As mentioned by the agreement above, the additionally supported events (e.g., Event 1 and Event 7) will reuse the same design as Event 2. Therefore, an RS resource set for new beams should be configured for Event 1. Moreover, when the radio link quality of the current beam is low (e.g., the condition of Event 1 is satisfied), reporting up to N new beams to the NW may help the NW decide on a new TCI state. If the RS resource set for new beam is configured, an implicit manner for enabling one of either scheme-1 or scheme-2 can be used, similar to Event 2. In particular, if the new beam RS is CSI-RS for beam management, the current beam is QCL RS in the indicated TCI state. Otherwise, the current beam is the SSB which is QCL-ed with the QCL RS in the indicated TCI state. Thus, there is no need for an explicit selection among scheme-1 and scheme-2.

- On UE-initiated/event-driven beam reporting for Event 1, support Option-2 for RS measurement:

- Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

  - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

# Uplink signaling content

In RAN1#119, the following agreement [2] has been reached regarding the L1-RSRP report format for Event 2.

Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement. Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2 FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A.

Event 2: When the UE does not report the L1-RSRP of the current beam in the CSI report, an additional indication is required to specify whether a reported beam meets the event-triggering condition. Without this information, the network may be unable to determine which CRI/SSBRI(s) in the CSI report satisfy the Event 2 condition and, as a result, may fail to effectively use UE-initiated beam reporting for fast TCI state updates. This would require to always include L1-RSRP of the current beam regardless of quality of the current beam, which may require also changes in current CSI report.

Even when the L1-RSRP of the current beam is configured to be included in the report, there is still a risk that the L1-RSRP of an unqualified beam, whose event-instance counter has not reached a maximum value within a time window, exceeds the threshold. This may lead the network to use the unqualified beam for updating the TCI state. This could reduce the effectiveness of event-instance counting and/or the time window. We believe this issue cannot be fully resolved through L1 filtering alone.

Therefore, we support providing additional indications to identify which CRI/SSBRI(s) meet the Event 2 condition. Between Option 1 and Option 2, the additional benefits of Option 2 with additional overhead is unclear.

- On UE-initiated/event-driven beam reporting, support Option-1 as an extension on L1-RSRP report format depending on Event 2.

- Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured.

Event 7: Unlike Event 2, where the NW has the information of the indicated TCI state, in Event 7, the NW may not have the information of the activated TCI state with the Q-th best quality as the activated TCI state with Q-th best quality is determined by the UE based on its own measurements. Therefore, an additional field indicating the TCI codepoint of the activated TCI state with the Q-th best quality needs to be reported to the NW. Otherwise, if the activated TCI state with Q-th best quality is not known at the NW, the NW may not use the beam report to update the correct activated TCI states.

- On UE-initiated/event-driven beam reporting, for L1-RSRP report format of Event-7, support to include an additional field to indicate the codepoint of the activated beam with Q-th best quality.

In RAN1#118, the following agreement has been reached regarding the number of beams to report when N is not configured.

Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4} - FFS: additional candidate value(s) of {5, 6, 7, 8} - FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default.

In the legacy systems, when nrofReportedRS is not configured, only one L1-RSRP and CRI/SSBRI are reported by default (see the text copied below from 38.331). We prefer to have the same default behavior for UE-initiated/event-driven beam reporting

nrofReportedRS The number (N) of measured RS resources to be reported per report setting in a non-group-based report. N <= N_max, where N_max is either 2 or 4 depending on UE capability. (see TS 38.214 [19], clause 5.2.1.4) When the field is absent the UE applies the value 1

- On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format of Event-2, if N is not configured by RRC, only one L1-RSRP and CRI/SSBRI are reported by default.

# Remaining issues on the first PUCCH

In RAN1#119, the following agreements [2] have been reached regarding the container of first PUCCH.

Agreement Confirm the following working assumption in RAN1#117: On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

For the first PUCCH transmission in UE-initiated/event-driven beam reporting, it has been agreed to introduce a new UCI type for the periodic PUCCH resource configuration. In this case, there may be a limit on the number of periodic PUCCH resource configurations for the first PUCCH transmission, similar to the limitation on SR configuration (e.g., up to 8).

As the periodic PUCCH resource configurations for the first PUCCH transmission may be limited, using the same periodic PUCCH resource configuration for multiple UE-initiated CSI reporting configurations improves resource efficiency. However, whether those multiple UE-initiated CSI reporting configurations are from the same CC or from multiple CCs have to be clarified.

In Mode B, it has been agreed to carry the beam report in the CC where the corresponding CSI report configuration is configured. When a CSI report configuration 1 is configured on CC1 and a CSI report configuration 2 is configured on CC2, where the CSI report configuration 1 and the CSI report configuration 2 share the same PUCCH resource for the first PUCCH transmission, if the NW receives the first PUCCH transmission via the PUCCH resource, the NW may not know whether to monitor the CC1 or the CC2 for the PUSCH transmission carrying the beam report. In the worst case, the NW monitors both CCs increasing the blind detection complexity. Therefore, in Mode B, multiple UE-initiated CSI reporting configurations from different CCs should not use the same PUCCH resource (or the same periodic PUCCH resource configuration for the first PUCCH transmission).

In Mode A, since there is no agreement, so far, restricting to carry the beam report in the CC where the corresponding CSI report configuration is configured as in Mode B, multiple UE-initiated CSI reporting configurations sharing the same periodic PUCCH resource configurations for the first PUCCH transmission can be from the same CC or from multiple CCs. However, to have a unified design as in Mode B discussed in the previous paragraph, we support multiple UE-initiated CSI reporting configurations only from the same CC using the same PUCCH resource (or using the same periodic PUCCH resource configuration for the first PUCCH transmission).

- For UE-initiated/event-driven beam reporting in Mode-A and Mode-B, support that the periodic PUCCH resource configuration for the first PUCCH transmission can be associated with multiple CSI report configurations, at least associated with a same event, in one CC.

It was agreed that the dedicated RRC parameter for the periodic PUCCH resource configuration comprises at least periodicityAndOffset and PUCCH-ResourceID. Moreover, it was agreed to reuse multiplexing/dropping rule(s) of SR as baseline. In PHY prioritization/multiplexing handling of an SR resource, the RRC parameter phy-PriorityIndex indicating whether this SR resource has a high or low priority is used. Similarly, the periodic PUCCH resource configuration of UE-initiated/event-driven beam reporting should comprise a PHY prioritization parameter to handle PHY prioritization/multiplexing of a PUCCH resource used for the first PUCCH transmission.

- Periodic PUCCH resource configuration (e.g. firstPUCCHResourceConfig-UEIBR) is configured with a priority index to handle PHY prioritization/multiplexing of the first PUCCH transmission.

It is still an FFS whether/how to support multi-bits indication in the first PUCCH. On one hand, multi-bits indication in the first PUCCH allows the UE to transmit more information in the first indication. This may allow the gNB to allocate smaller number of PUCCHs with the same number of configured events per UE. If the multi-bits indication is used to indicate multiple events, e.g., one bit for one event, the gNB may be able to understand which one or more events are triggering the UEI/ED report before receiving the second PUSCH that carries the UEI/ED measurements.

On the other hand, there are several drawbacks for multi-bits indication in the first PUCCH. Firstly, a maximum payload size of 2 bits of PUCCH format 0/1 allows limited flexibility comparing to one-bit indication. Considering there may be more events to be defined in the future, e.g., mTRP or even AI/ML triggered events, carrying maximum two bits in the first PUCCH may still lack of extensibility. Secondly, introducing a higher PUCCH format for the first PUCCH to have enough extensibility to support more than 2 bits may need to ensure that false alarm is still acceptable.

- Support of multi-bits indication in the first PUCCH does not have clear benefits to support multi-events indication.

- Do not support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved:

- UE transmits the first PUCCH if one or more events are satisfied.

- FFS second PUSCH if more than one event is satisfied.

Adopting a new UCI type for the first PUCCH would require a new IE to configure PUCCH resource for the first PUCCH. A first parameter that may require optimization is periodicityAndOffset. As opposed to the semantics of a scheduling request, which means “UE has data to transmit”, a UEIBR means “UE has observed a change in downlink measurements”. There seems to be a lack of motivation to have a periodicity of the first PUCCH shorter than the shorter periodicity of the current beam or the new beam. Having said that, a CSI-ResourcePeriodicityAndOffset may be a better starting point to define periodicityAndOffset for UEIBR. Among the available periodicities of CSI-ResourcePeriodicityAndOffset, smaller periodicities may be more suitable for configuring the RSs of current beam and new beam for UEIBR. For a very large periodicity, e.g., above 100 slots, a UE may be in a static channel environment. The gNB may configure those large periodicities just to receive a periodic “sign-of-life” signal. There is probably not much to save if the UE transmits an event-triggered measurement.

For candidate values for periodicityAndOffset, two existing values may be considered. One from SR periodicity and the other from CSI resource periodicity (CSI-ResourcePeriodicityAndOffset). Given this resource is based on CSI measurement, we feel utilizing CSI resource periodicity, as a baseline, is more suitable.

- Periodicity for the first PUCCH of UEIBR is based on periodicity values of CSI-ResourcePeriodicityAndOffset. FFS whether all values in current CSI-ResourcePeriodicityAndOffset are supported.

In RAN1#119, the following agreement [2] has been reached regarding the multiplexing/dropping of the first PUCCH transmission.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases: - Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH - Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain.

In our view, beam failure recovery is more urgent, and thus has a higher priority than UE-initiated beam reporting. Therefore, in Case 1, when the 1-bit first PUCCH is collided/overlapped with a PUCCH with LRR, the PUCCH resource with the LRR is valid. When the 1-bit first PUCCH is collided/overlapped with a PUCCH with normal SR, the 1-bit first PUCCH is valid, if they have the same priority index.

When a PUSCH is transmitted without UL-SCH, the PUSCH can carry an aperiodic CSI report or a semi-persistent CSI report. In our view, an aperiodic CSI report can carry more information than the UE-initiated CSI report (or may already carry the beam information that will be reported by the UE-initiated CSI report). Therefore, in Case 2, for a fast reporting of the CSI report (either the aperiodic CSI report or the semi-persistent CSI report), the 1-bit first PUCCH can be dropped and the PUSCH without UL-SCH can be transmitted. When PUSCH is transmitted with UL-SCH (and no CSI report), for a timely UE-initiated beam reporting, in our view, 1-bit first PUCCH has a higher priority than uplink data transmission. In this case, PUSCH can be dropped, and 1-bit first PUCCH is transmitted.

Case 3 can be resolved by UE implementation or the CSI configuration with the highest CSI report configuration ID can be prioritized.

- On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH:

- Case 1: The priority of the first PUCCH is higher than legacy/normal SR and lower than LRR.

- Case 2: If the PUSCH is transmitted with a legacy CSI report (e.g., aperiodic CSI report), the 1-bit first PUCCH is dropped; otherwise, the PUSCH transmission is dropped.

- Case 3: This can be up to UE implementation or the CSI configuration with the highest CSI report configuration ID can be prioritized.

# Remaining issues on the second PUSCH

In RAN1#119, the following agreement [2] has been reached regarding the triggering procedure in Step-2 of Mode-A.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120 - Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration. - Option-2: A CSI trigger state can be associated with - only UE-initiated/event-driven beam reporting - or only legacy AP-CSI configuration - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration

In our view, regardless of whether Option-1 or Option-2 is selected, only one CSI trigger state should be associated with all CSI report configurations used for UE-initiated beam reporting. The difference between two options should be whether to add an aperiodic CSI report configuration to this single CSI trigger state or not. For example, if CSI trigger state 1 is associated with CSI report configuration 1 used for UE-initiated beam reporting and CSI trigger state 2 is associated with CSI report configuration 2 used for UE-initiated beam reporting (plus CSI report configuration 3 used for aperiodic beam reporting if Option 2 is selected), when the NW receives the first PUCCH, the NW may not know which CSI report configuration triggered the UE-initiated beam reporting, unless a dedicated/specific PUCCH resource for first PUCCH is configured per CSI report configuration or the first PUCCH indicates the CSI report configuration triggering the UE-initiated beam reporting. In this case, if the NW does not know which CSI report configuration triggered the UE-initiated beam reporting, the NW may not indicate the correct CSI trigger state by CSI request field in DCI.

In option 2, when a CSI trigger state is associated with both the UE-initiated CSI reporting and aperiodic CSI reporting, and after transmission of the 1-bit first PUCCH, if a CSI request field in DCI indicates this CSI trigger state, both the UE-initiated CSI report and aperiodic CSI report are transmitted in the same PUSCH. However, NW may not need the legacy aperiodic CSI report. Reporting both CSI reports all the time may be unnecessary. Moreover, the delay/timeline of UE-initiated CSI reporting and legacy aperiodic CSI reporting could be different. To transmit only one of the UE-initiated CSI report and the aperiodic CSI report, a one bit field indicating to transmit the UE-initiated CSI report or the aperiodic CSI report can be included in the scheduling DCI. Or in an implicit determination, if the UE receives the DCI after transmitting a 1-bit first PUCCH transmission, the UE transmits the UE-initiated CSI report, otherwise an aperiodic CSI report is transmitted.

We are fine with both options, however, a dedicated CSI trigger state for UE-initiated/event-driven beam reporting is a simpler option. Therefore, we prefer a dedicated CSI trigger state for the UE initiated beam UE-initiated/event-driven beam reporting.

- On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, Support Option-1.

- Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

# Latency reduction of beam application time

In RAN1 #119, there was a discussion on latency reduction of beam application time (BAT) [3]. The three alternatives below are agreed to be further studied.

Agreement

Study the following to reduce beam application latency after a UEI/ED beam report is sent

  - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

    - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

    - update TCI state(s) with the reported new beam(s)

    - activate new beam(s) without additional SSB reception

  - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

    - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

    - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

  - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

    - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

  - Other alternatives are not precluded

In our view, the main benefit of a UE-initiated beam management is being able to achieve small beam report latency with reduced feedback overhead. However, the overall beam application latency, e.g., from a new beam emerged to the new beam applied, may still be increased if no QCL assumption is available when the gNB received the event-driven measurements. In legacy, UE can be activated with up to 8 TCI states and/or pairs of TCI states. UE must have stored the QCL information of those activated TCI states. As such the UE is QCL-ready to decode a PDCSH with an indicated TCI state out of activated TCI states.

Under UE-initiated beam management, it has been agreed that the new beams are RRC configured. During the evaluation of relevant events, e.g., Event-2/7, it is reasonable to assume that UE has stored at least QCL-TypeD properties for the new beams. Whether UE has stored other QCL properties, e.g., QCL-TypeA, is specific to UE implementation. For example, a UE may implement a simple RSRP estimation for the new beams without requiring QCL-TypeA property. Or a UE may have used QCL-TypeA property to estimate RSRP but not stored.

For the three alternatives above, Alt-1/3 requires that the UE to store QCL property after sending the UEI/ED report. Alt-2 allows the UE to have more freedom of storing the QCL property for the new beams at a cost of an additional synchronized indicator field in the UCI. Alt-2 sounds like a UE with a weaker capability than Alt-1/3. Among Alt-1 and Alt-3, Alt-1 only allows the new set of activated TCI states being driven by the UEIBR report. The gNB can only confirm switching to the new set of activated TCI states or stay with the existing set of activated TCI states. A confirmation message may be just a DCI. There may be a risk that the confirmation message is sent but lost over the air. This may result in an unsynchronized set of activated TCI states. Alt-3 always requires an explicit MAC CE activation of TCI states. There should be no risk of an unsynchronized set of activated TCI states. But it may be worth clarifying how long the UE should wait for the MAC CE activating before dropping the stored QCL properties for the new beams. If such an MAC CE does not come as expected, UE should be allowed to drop the stored QCL properties as UE may continue performing UEI/ED measurement and event evaluation based on the existing set of activated TCI states.

- Alt-2 may require less capability of UE to store QCL properties than Alt-1/3.

- Alt-1 may have a risk of an unsynchronized set of activated TCI states between the UE and the gNB.

- It may be worth clarifying a time window that UE expects to receive a MAC CE after a UEIBR report has been transmitted.

- Alt-3 is preferred to Alt-1 with an expected time window to receive the MAC CE. UE can discard the QCL properties if no MAC CE is received within the time window.

- Alt-2 can be considered if the UE does not have the capability to store QCL properties of all new beams.

# Conclusion

In this contribution, we make the following proposals:

- Regarding the triggering event determination for Event 2, the following conditions can be considered for resetting the counter for Event-2:

- Reconfiguration/update of the new beams

- Update of the indicated TCI state

- Transmission of UE-initiated CSI report

- Expiration of the time window

- Reconfiguration of threshold

- Reconfiguration of time window and/or maximum event instance count

- The following conditions can be considered for cancellation of a triggered UE-initiated/event-driven beam reporting for Event 2:

- Update of the indicated TCI state

- Transmission of UE-initiated CSI report

- BWP switching

- Specify necessary enhancements to address beam failure recovery procedure when UE is configured with UE-initiated/event-driven beam report procedure at least based on Event -1 or Event-2.

- For example, the UE-initiated/event-driven beam report procedure is not performed during an ongoing beam failure recovery procedure. (e.g., no condition evaluation, no first PUCCH and/or second PUSCH transmission)

- Study whether UE-initiated/event-driven beam report procedure (e.g., condition evaluation, first PUCCH transmission, second PUSCH transmission, and so on) is impacted after a beam failure recovery procedure is successfully completed.

- Option 1: The UE-initiated/event-driven beam report procedure is restricted until a new TCI state is indicated.

- Option 2: The RS for current beam used for trigger-event condition is the candidate reference signal identified for the beam failure recovery.

- On UE-initiated/event-driven beam reporting for Event 1, support Option-2 for RS measurement:

- Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

  - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

- On UE-initiated/event-driven beam reporting, support Option-1 as an extension on L1-RSRP report format depending on Event 2.

- Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured.

- On UE-initiated/event-driven beam reporting, for L1-RSRP report format of Event-7, support to include an additional field to indicate the codepoint of the activated beam with Q-th best quality.

- On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format of Event-2, if N is not configured by RRC, only one L1-RSRP and CRI/SSBRI are reported by default.

- For UE-initiated/event-driven beam reporting in Mode-A and Mode-B, support that the periodic PUCCH resource configuration for the first PUCCH transmission can be associated with multiple CSI report configurations, at least associated with a same event, in one CC.

- Periodic PUCCH resource configuration (e.g. firstPUCCHResourceConfig-UEIBR) is configured with a priority index to handle PHY prioritization/multiplexing of the first PUCCH transmission.

- Support of multi-bits indication in the first PUCCH does not have clear benefits to support multi-events indication.

- Do not support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved:

- UE transmits the first PUCCH if one or more events are satisfied.

- FFS second PUSCH if more than one event is satisfied.

- Periodicity for the first PUCCH of UEIBR is based on periodicity values of CSI-ResourcePeriodicityAndOffset. FFS whether all values in current CSI-ResourcePeriodicityAndOffset are supported.

- On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH:

- Case 1: The priority of the first PUCCH is higher than legacy/normal SR and lower than LRR.

- Case 2: If the PUSCH is transmitted with a legacy CSI report (e.g., aperiodic CSI report), the 1-bit first PUCCH is dropped; otherwise, the PUSCH transmission is dropped.

- Case 3: This can be up to UE implementation or the CSI configuration with the highest CSI report configuration ID can be prioritized.

- On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, Support Option-1.

- Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

- Alt-2 may require less capability of UE to store QCL properties than Alt-1/3.

- Alt-1 may have a risk of an unsynchronized set of activated TCI states between the UE and the gNB.

- It may be worth clarifying a time window that UE expects to receive a MAC CE based on a UEI/ED report it has transmitted.

- Alt-3 is preferred to Alt-1 with an expected time window to receive the MAC CE. UE can discard the QCL properties if no MAC CE is received within the time window.

- Alt-2 can be considered if some UE may not have the capability to store QCL properties of all new beams.

# References

- RP-242394, WID revision: NR MIMO Phase 5, Samsung, RAN#105, Sept. 2024

- Chair notes RAN1 #119, 3GPP, RAN1 #119, Nov. 2024

- R1-2410872, Moderator (ZTE), RAN1 #119, Nov. 2024