3GPP TSG RAN WG1 #120	R1-2500536

Athens, Greece, February 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on transmission power reduction for STxMP in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to clause 7.1 and 7.2 in TS 38.213, in case of STxMP, the UE should determine the PUCCH/PUSCH transmission power for the first and second TCI-State or TCI-UL-State based on the maximum output power configured for the respective TCI-State or TCI-UL-State. For scenarios involving two uplink carriers, carrier aggregation and LTM, transmission power reduction operation is specified in clause 7.5 in TS 38.213 if the total UE transmit power for uplink transmissions on serving cells or on a candidate cell in a respective transmission occasion would exceed the linear value of maximum output power in the transmission occasion. Considering that, for a STxMP scenario, the sum of transmission power of the simultaneous uplink transmissions may exceed the UE configured maximum output power for a given carrier of a given serving cell, the transmission power reduction operation specified in clause 7.5 should be applied for STxMP cases as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a condition of applying transmission power reduction operation for STxMP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmission power reduction operation remains unclear when transmission power of STxMP PUCCH/PUSCH transmissions exceeds the UE configured maximum output power for a given carrier of a given serving cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has no isolated impact on network and UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of this CR. |  |  |  |  |  |  |  |  |

7.5	Prioritizations for transmission power reductions

For single cell operation with two uplink carriers or for operation with carrier aggregation or for operation with a candidate cell configured by LTM-Config or for operation with multiple panel simultaneous uplink transmission configured by multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel or multipanelSFN-Scheme, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cell(s) or on a candidate cell, if any, in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cell(s) or on a candidate cell, if any, in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i. If the UE transmits SRS on multiple SRS resources according to Clause 6.2.1.4 of [6, TS 38.214], the UE allocates power so that all REs of the SRS transmission have same power.

<Unchanged part is omitted>