3GPP TSG-RAN WG1 Meeting #120	Tdoc R1-2500540

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	5

Source:	Ericsson

Title:	Discussion of RAN2 LS on 8 Tx UL Transmission

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we consider two questions that RAN2 raise on 8 Tx UL MIMO transmission [1] regarding 1) RAN2 assumptions on how to model UL 8 Tx two transport block operation from a MAC point of view and 2) if only one TB is generated by the MAC if there is any issue for the physical layer.  We propose answers to RAN2 according to the discussion.

# 2	Discussion

In [1], RAN2 ask RAN1 for feedback on their assumption on how 8 Tx UL transmission should be modelled and if there is a problem in the physical layer if MAC generates a single TB due to uplink skipping:

1. Overall Description:

1. To support UL 8Tx in the MAC specification, RAN2 made the following assumption regarding how to model UL 8Tx operation from MAC point of view to minimize specification impacts.

For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.

2. For UL 8Tx 2TB transmission scheduled by one DCI, if MAC entity only generates a single TB due to the uplink skipping, would there be any issue for PHY layer?

2. Actions:

To RAN1: RAN2 respectfully asks RAN1 to provide feedback, if any, on the modelling assumption in the first item and answer the question in the second item.

Firstly, regarding RAN2’s assumption, there may be some confusion with respect to the term ‘grant’.  The DCI carried in a PDCCH that grants transmission of one or two TBs in uplink 8 Tx MIMO operation may be referred to as one ‘grant’. Noting that the intention in RAN1 for 8 Tx was to reuse the downlink handling of multiple TBs as much as possible, we observe that the MAC for the downlink has e.g.

For each received TB and associated HARQ information, the HARQ process shall: 1> if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

and so considers HARQ on a TB by TB basis, not a DCI by DCI (i.e. a per ‘grant’) basis.

Furthermore, there is a single HARQ process ID indicated to the UE in the DCI of one PDCCH, and the DCI may indicate that either one of two TBs is disabled.  From a physical layer viewpoint, whether soft bits for a TB are flushed can be determined through the associated HARQ process ID and TB disabling indications.

Given the understanding that ‘grant’ refers to if a PUSCH TB is scheduled rather than the DCI scheduling up to two PUSCH TBs, we think the RAN2 assumption is consistent with RAN1 operation.

Given the understanding that ‘grant’ refers to if a PUSCH TB is scheduled rather than the DCI scheduling up to two PUSCH TBs, the RAN2 assumption “For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.” is consistent with RAN1 operation.

Reply to RAN2 that the RAN2 assumption “For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.” is consistent with RAN1 operation.

Regarding the question on uplink skipping, we understand that uplink skipping is supported for both dynamically scheduled PUSCH and for configured grant.  Configured grant is not supported for 8 Tx with two TB transmission according to a conclusion in RAN1#113 (copied below), and so only skipping for dynamic grants needs to be considered.

Conclusion

In Rel-18, there is no consensus to support CG transmission with dual CW PUSCH by an 8TX UE.

Configured grant is not supported for 8 Tx with two TB transmission, and so only skipping for dynamic grants needs to be considered.

When skipping is configured for dynamically scheduled PUSCH, if a UE does not have data to transmit, subject to some additional constraints, it should not transmit the MAC PDU, as stated in 38.321:

1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

Not transmitting a MAC PDU is captured in the RAN1 specs in 38.214 with the following:

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1, 0_2 or 0_3 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321].

To be consistent with the MAC specification (and with the concept of skipping), if a TB is not generated, it should not be transmitted on the physical layer.

Given this observation, then if the UE generates one TB, but is scheduled for two, it should not follow the DCI, but instead transmit only the layers that contain the TB that is generated.  Therefore, the text in 38.214 as written is not correct: the UE does generate a transport block in the single TB case, but should not follow the DCI.  We note the behavior for UL skipping was unclear for a time during Rel-15 and Rel-16, and this was corrected for Rel-16 with the highlighted yellow text with a CR (R1-2007337). However, the CR was written when only one TB was supported for the PUSCH, and in our understanding corrections for dynamic skipping after adding two TBs to PUSCH in Rel-18 has not been considered.  The behavior to support skipping with two TBs could be captured with something like the following:

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1, 0_2 or 0_3 transmit the transport blocks and corresponding layers, DM-RS ports, and PT-RS ports of the PUSCH as indicated by that DCI unless the UE does not generate a the transport blocks as described in [10, TS38.321].

The DCI indicates the number of layers from which the number of TBs is determined. Since a TB is mapped to only a portion of the layers, when a TB is skipped, those layers corresponding to the skipped TB are not transmitted. This requires that the corresponding DMRS and PTRS ports for the unoccupied layers are not transmitted; otherwise the receiver will have an incorrect estimate of the SNR of the data REs, which should be zero rather than non-zero.

This behavior for dynamic skipping in the two TB case is therefore more complex than the single codeword / TB case in UL MIMO up to Rel-17; for single codeword the entire PUSCH is dropped, whereas for the two TB case, portions of the PUSCH are dropped.

The use cases for dynamic skipping seems rather fewer than for skipping with configured grant, given that the network is more likely to be aware of the UE’s buffer status when the UE is dynamically scheduled.  If more than 5 layers are transmitted, which is needed for two TBs, this may not be for long periods of time, since the channel conditions for rank 5 and higher could disappear quickly if the UE moves or interference appears, and dynamic scheduling is therefore much more desirable than configured grant in scenarios supporting two TBs.  Therefore, the need for dynamic skipping does not seem high to us.

The RAN1 spec is not clear for dynamic UL skipping in the two transport block case, since it says a PUSCH should be transmitted according to DCI if even one TB is generated and that two codewords are transmitted when the number of layers (indicated by DCI) is greater than 4 even when only one TB is available from MAC.

The RAN1 spec can be corrected to support skipping of one of two TBs, but this requires that portions of a PUSCH (subsets of layers, DM-RS, and PT-RS ports) are not transmitted, whereas Rel-17 skipping (with at most one TB) simply drops the entire PUSCH.

Given that the network is more likely to track the UE’s buffer status due to the use of dynamic grants for 8 Tx and the relatively rapid time variation of high rank transmission where two TBs are transmitted, the use cases for dynamic skipping for 8 Tx seem limited.

Reply to RAN2 that uplink skipping is not supported for 8 Tx, including both configured grant and dynamic skipping.

# 3	Conclusion

In this contribution, we considered two questions that RAN2 raise on 8 Tx UL MIMO transmission [1] regarding 1) RAN2 assumptions on how to model UL 8 Tx two transport block operation from a MAC point of view and 2) if only one TB is generated by the MAC if there is any issue for the physical layer.  We made the following observations:

Observation 1	Given the understanding that ‘grant’ refers to if a PUSCH TB is scheduled rather than the DCI scheduling up to two PUSCH TBs, the RAN2 assumption “For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.” is consistent with RAN1 operation.

Observation 2	Configured grant is not supported for 8 Tx with two TB transmission, and so only skipping for dynamic grants needs to be considered.

Observation 3	To be consistent with the MAC specification (and with the concept of skipping), if a TB is not generated, it should not be transmitted on the physical layer.

Observation 4	The RAN1 spec is not clear for dynamic UL skipping in the two transport block case, since it says a PUSCH should be transmitted according to DCI if even one TB is generated and that two codewords are transmitted when the number of layers (indicated by DCI) is greater than 4 even when only one TB is available from MAC.

Observation 5	The RAN1 spec can be corrected to support skipping of one of two TBs, but this requires that portions of a PUSCH (subsets of layers, DM-RS, and PT-RS ports) are not transmitted, whereas Rel-17 skipping (with at most one TB) simply drops the entire PUSCH.

Observation 6	Given that the network is more likely to track the UE’s buffer status due to the use of dynamic grants for 8 Tx and the relatively rapid time variation of high rank transmission where two TBs are transmitted, the use cases for dynamic skipping for 8 Tx seem limited.

Based on the observations, we propose the following:

Proposal 1	Reply to RAN2 that the RAN2 assumption “For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.” is consistent with RAN1 operation.

Proposal 2	Reply to RAN2 that uplink skipping is not supported for 8 Tx, including both configured grant and dynamic skipping.

# 4	References

RAN2 (Samsung), “R2-2411220, LS on UL 8Tx”, 3GPP TSG RAN2 Meeting #128, Orlando, US, November 18th - 22nd, 2024.