3GPP TSG RAN WG1 #120		R1-2500548

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.3.1

Source:	Google

Title:	ML based CSI compression

Document for:	Discussion/Decision

# Introduction

In RAN1 #119 meeting, the following agreements on ML based CSI compression were achieved.

Conclusion - For Direction A 4-1 and 3a-1, send LS to RAN2 for the study of feasibility of parameter / dataset exchange Conclusion Above LS, additionally CC SA2, SA3, SA5 Agreement - For Direction A Option 3a-1 and Direction C, study the feasibility of scalable model structure specification over numbers of Tx ports, CSI feedback payload sizes, and bandwidths, number of slots. - Feasibility of Direction A Option 3a-1 is contingent on the feasibility of the scalable model structure specification. - Feasibility of Direction C is contingent on the feasibility of the scalable model structure specification and model parameters specification. - Note: Angular/Delay/Doppler domain conversion and quantization are considered to be part of feasibility study Agreement For studying the standardized model structure, - For temporal domain Case 0, - In case of spatial-frequency domain input, adopt Transformer as the backbone structure. - Further study angular-frequency and angular-delay domain input - For temporal domain Case 2, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation with additional layers - Conv-LSTM - LSTM - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots - Note: other options are not precluded - Further study angular-frequency and angular-delay domain input - For temporal domain Case 3, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation - Option 2: Latent adaptation - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input. - Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure. - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered. - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used. Agreement For NW to collect data for training, study following spec impacts - Data format: codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction. - FFS if enhancement is needed - FFS number of samples in the report. - FFS whether channel or precoder is needed for temporal Cases 3 - Configuration of rank/layer, number of subbands - Mechanism for ground-truth reporting - FFS: Report additional information regarding the samples, e.g., data quality, FFS the definition of data quality and corresponding parameters. - FFS if enhancements in CSI-RS and SRS configuration is needed. - FFS: Report associated information that captures UE side additional condition - FFS: Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI - FFS: details of CSI measurement For UE to collect data for training, study following spec impacts - NW configuration or UE request, e.g., RS configuration/transmission for data collection - Whether enhancements in CSI-RS configuration is needed. - Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI - FFS: Need of configuration of ID, and configuration of ID Agreement To capture all observations in section 2 of R1-2410725 to TR38.843. Agreement Draft LS R1-2410915 with the following updates: Revising “The size becomes 600K * (2000 bits) / (8bits/Byte) = 150 MB+ 11.6 MB = 161.6MB if we assume N1=N2.” To “The size becomes N2 * (2000 bits) / (8bits/Byte) + 11.6 MB” , and Revising Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI} to Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI} 3. Deleting “RAN1 respectfully requests RAN2’s feedback on the feasibility of standardized signaling, considering the size of dataset / encoder parameters provided above.” 4. Deleting “The size is based on Case 0 of two-sided CSI compression; the size may or may not be larger for Case 2 and Case 3.” from “For Option 4-1: sharing {target CSI, CSI feedback} dataset:” Agreement Final LS R1-2410922 is endorsed. Agreement For temporal domain aspects Case 3, study LCM aspects and specification impacts, consider the following options for training data collection - Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s). - Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s). - Note: During inference, the input to the CSI generation part is derived based on the predicted CSI. consider following options for the monitoring labels - Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s). - CSI prediction output is used as input to CSI generation part. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. - Option 2: The monitoring label is derived based on the measured CSI of the future slot(s) - Option 2a: CSI prediction output is used as input to CSI generation part. - Note: This corresponds to end-to-end monitoring of CSI prediction and compression. - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose. - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately. Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

In this contribution, we provide some discussion on AI/ML based CSI compression.

# Discussion

## CSI Report content

In RAN1 #118, the following has been agreed.

Agreement For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3. Agreement For studying the standardization of model structure RAN1, RAN1 assumes at least the following: - Precoding matrix as an input (as opposed to raw channel matrix) - Per-layer processing, with common structure across ranks and layers (corresponding to Option 2-1, 2-2, 3-1, or 3-2 for handling rank ≥ 1) - For temporal domain aspects Case 2 and Case 3, strive to reuse the model structure of Case 0 where appropriate, with additional layers or operations either at the input/output domain or at the latent domain. - For Case 0, use precoding matrix, e.g., eigen vector in spatial-frequency domain, and/or angular, delay domain representation such as eType-II W2. - For Case 2 and Case 3, use precoding matrix, e.g., spatial-frequency domain for each CSI observation instance, and/or angular, delay, and Doppler domain representation such as eType-II W2. - Consider scalability approaches over numbers of Tx ports, CSI feedback payload sizes, and bandwidths.

For case 2, both UE and NW use the past CSI information to identify the CSI for present slot. For case 3, UE uses the past CSI information and NW does not use the past CSI information to identify the CSI for future slot. Thus, case 2 could potentially require the NW to perform CSI prediction and case 3 could require the UE to perform the CSI prediction. Then for both case 2 and case 3, the UE should report a W1 and compressed W2. The W2 is compressed based on the AI/ML. The difference is whether the W2 is for the past slots or future slots.

UE can report the W1 and W2 based on a configured RI. Then after receiving the W1 and W2, the network can transmit a set of precoded CSI-RS resources for the UE to measure RI and CQI, where the network applies the channel eigenvector based on the received W1 and reconstructed W2 to the precoded CSI-RS.

Proposal 1: For case 2 and case 3, support the UE to report W1 and compressed W2 for a configured rank

- The compressed W2 is calculated based on the AI/ML

- NW can configure the UE to report RI and CQI based on precoded CSI-RS resources after receiving the W1 and compressed W2

## Priority of AI/ML based CSI report

Usually, the non-ML based CSI could provide more stable CSI, which can be used for performance monitoring. For example, the gNB can configure a non-ML based CSI with higher overhead to monitor the performance for the ML based CSI. Therefore, the priority for the non-ML based CSI should be higher than the priority of the ML based CSI.

Proposal 2: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report.

## CSI Processing Unit

The ML based CSI measurement should include the following two steps.

- Step 1: Channel estimation and pre-processing

- Step 2: Compression/prediction based on the ML (Inference)

The two steps may be handled by different hardware. Thus, the CPU occupancy rule should define two types of units: one is measurement processing unit (MPU) which is used for step 1 related aspects and the other one is the inference processing unit (IPU), which is used for the step 2 related aspects.

Proposal 3: Support the CPU occupancy rule for ML based CSI based on two types processing unit

- Type1 CPU: a measurement processing unit (MPU) used for channel estimation and pre-processing

- Type2 CPU: an inference processing unit (IPU) used for inference for ML based CSI

## AI/ML model monitoring

In RAN1 #117, the following on model monitoring has been agreed.

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

For NW-side monitoring, the NW can configure the UE to report CSI based on a high-resolution codebook or can trigger the UE to transmit the SRS for performance monitoring.

For CSI report based performance monitoring, the NW can configure the UE to report PMI only. The NW can configure a rank for the PMI calculation. Then it can compare the reported ground-truth CSI and the ML based CSI. The NW can further configure the UE to report subband L1-SINR. Then based on the received PMI and subband L1-SINR, it can also select the precoder and MCS for downlink transmission. Further, for joint ML based CSI compression and prediction, the NW can configure whether the UE should perform the CSI predication based on ML or non-ML and the CSI quantization based on ML or non-ML for separate performance monitoring for ML based CSI prediction and ML based CSI compression. In addition, for the CSI-RS with low SINR, the UE is unable to measure the CSI accurately. One state of the CSI report should be used to indicate the reported CSI is invalid for performance monitoring.

For SRS based performance monitoring, the NW can configure the SRS to be linked with a CSI-RS or CSI report configuration for ML based CSI. Then, the UE can apply the same ports including the antenna visualization scheme to transmit the SRS and to receive the CSI-RS. Without such linkage, UE may use different antenna visualization schemes and there can be some mismatch between the UL and DL channel. Further, for the coverage limited UE, it is unable to transmit the SRS with wider bandwidth. Then the NW cannot get the wideband channel for performance monitoring. Therefore, it is necessary to introduce a burst like approach for SRS transmission with frequency hopping. Then the NW can get the wideband channel based on multiple transmission occasions of the SRS.

For UE-side model monitoring, the first issue could be to identify the KPI. The SCS and hypothetical BLER could be the possible KPI for model monitoring. However, sometimes the SCS cannot reflect the performance status. Figure 1 illustrates the simulation results on the SCS and corresponding SE offset for two precoders. It can be observed that low SCS does not always produce large performance gap. Therefore, although the SCS can be considered as an intermediate KPI, it should not be used for model performance monitoring. Therefore, a KPI other than SCS can be considered. Currently, the hypothetical BLER is used for RLM/BFD, which can be considered as one KPI for model monitoring.

Figure 1: Simulation results on SCS vs SE offset for two precoders

The next issue should be the procedure for the UE-side model monitoring. The hypothetical BLER is usually measured based on CSI-RS. As shown in Figure 2, the NW can transmit the precoded CSI-RS for model monitoring. The UE can calculate the hypothetical BLER based on a non-ML CQI and the CSI-RS for model monitoring. The non-ML CQI is measured based on a CSI-RS for CQI acquisition and existing codebook. If the hypothetical BLER is above the BLER threshold for CQI selection, a model performance failure can be declared.

Figure 2: Procedure for model monitoring

Proposal 4: For CSI report based performance monitoring, the following spec impact should be considered:

- New report quantity PMI only should be introduced, where UE reports the PMI based on a configured rank

  - Support a further enhancement to report subband L1-SINR in addition to the PMI to facilitate the precoder and MCS selection for PDSCH

- With regard to measurement accuracy in low SINR case, support the UE to report a state of CSI indicating the CSI is invalid for performance monitoring

- With regard to joint ML based CSI compression and prediction, support to configure whether the UE should perform the CSI predication based on ML or non-ML and the CSI quantization based on ML or non-ML for separate performance monitoring for ML based CSI prediction and ML based CSI compression.

Proposal 5: For SRS based performance monitoring, the following spec impact should be considered:

- Support to configure the SRS linked with a CSI-RS report configuration for ML based CSI, where the UE uses the same ports including antenna virtualization scheme to transmit the SRS and to receive the CSI-RS for ML based CSI

- Support burst based SRS with frequency hopping to facilitate the performance monitoring for wideband channel for coverage-limited UE

Proposal 6: For UE-side monitoring, the following spec impact should be considered:

- Introduce the hypothetical BLER as the metric for performance calculation

- Configuration of precoded CSI-RS for hypothetical BLER calculation

## NW side data collection

In RAN1 #119, the following on NW side data collection is agreed.

Agreement For NW to collect data for training, study following spec impacts - Data format: codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction. - FFS if enhancement is needed - FFS number of samples in the report. - FFS whether channel or precoder is needed for temporal Cases 3 - Configuration of rank/layer, number of subbands - Mechanism for ground-truth reporting - FFS: Report additional information regarding the samples, e.g., data quality, FFS the definition of data quality and corresponding parameters. - FFS if enhancements in CSI-RS and SRS configuration is needed. - FFS: Report associated information that captures UE side additional condition - FFS: Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI - FFS: details of CSI measurement

The first issue is whether some enhancement for current codebook is needed. The purpose of the data collection is to get high resolution based CSI. Therefore, some enhancement for current codebook to improve the resolution could be necessary. For example, the oversampling factors O1 and O2, and additional codebook parameter combinations can be considered especially for rank 3 and 4.  Further, for the codebook for PMI prediction, the data collection should be based on the measured CSI instead of the reported CSI, therefore, the codebook configuration should allow the UE to report the measured CSIs. In addition, the number of DD basis can also be increased to improve the accuracy.

The configuration of the number of layers for the CSI report is necessary for the NW to get the CSI for a certain number of layers instead of CSI based on a UE-determined layers. The number of layers can be configured based on the RI restriction. Current RI restriction can already provide such functionality. The potential spec impact could be introducing a constraint for the RI restriction to configure one RI for the CSI report.

Another remaining issue is the additional information for the CSI report. One simple way is to report the L1-SINR. Then the NW can identify the accuracy for the CSI measurement. Further, UE can also report the timing information for the transmission occasion(s) of the CSI-RS used for the CSI measurement.

In addition, the NW may perform the data collection based on the SRS. With regard to the overhead and capacity of SRS, the SRS may be shared with other functionality, e.g., SRS for codebook or antenna switching. Different functionalities may require different target received power. Then one potential enhancement is to configure multiple uplink power control parameter sets for SRS and indicate which one is applied for the SRS transmission dynamically.

Proposal 7: For NW-side data collection, support the following enhancement for Rel-16 eType2 codebook and Rel-18 eType2 codebook for PMI prediction

- Additional O1 and O2 value

- Additional codebook parameter combinations at least for rank 3 and 4

- UE reports the measured CSIs instead of predicted CSI for Rel-18 eType2 codebook for PMI prediction

- Additional number of DD basis for Rel-18 eType2 codebook for PMI prediction

Proposal 8: For NW-side data collection, the potential spec impact for the configured number of layers can be based on introducing a constraint for the RI restriction to configure one RI for the CSI report.

Proposal 9: For NW-side data collection, support the UE to report the following additional information

- L1-SINR measured based on the CSI-RS to indicate the measurement quality

- Timing for the transmission occasion(s) of the CSI-RS for the measured CSI

Proposal 10: Support to configure multiple uplink power control parameter sets for SRS

- The SRS can be used for data collection and other functionalities, where different uplink power control parameter sets can be used for different functionalities

- The NW can indicate which one is applied for the SRS transmission dynamically

## UE side data collection

In RAN1 #119, the following on NW side data collection is agreed.

Agreement For UE to collect data for training, study following spec impacts - NW configuration or UE request, e.g., RS configuration/transmission for data collection - Whether enhancements in CSI-RS configuration is needed. - Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI - FFS: Need of configuration of ID, and configuration of ID

The UE side data collection may be based on the status in both NW and UE side. If there is additional resource in the NW side, it can configure some CSI-RS for the UE to perform the data collection. Similarly, if there is additional resource, e.g., additional power or memory, in the UE side, it can also require the CSI-RS for data collection. Therefore, both NW configuration and UE request based CSI-RS for data collection should be necessary. For the UE-side data collection, since UE needs to perform CSI calculation, corresponding number of CPUs should be occupied. Further, the associated ID is necessary for the UE to identify the additional condition for the CSI measurement.

Proposal 11: Support both NW-configured and UE-requested based CSI-RS for UE-side data collection

- Corresponding number of CPUs should be occupied for the CSI-RS

- Associated ID should be provided

## Hybrid AI/ML based and non-AI/ML based CSI measurement and report

The AI/ML based CSI compression can provide accurate CSI with smaller payload size than eType2 CSI. Usually the NW schedules MU-MIMO operation for low rank case and schedules SU-MIMO operation for high rank case. For MU-MIMO operation, the CSI quantization error could have significant impact on the performance. For SU-MIMO operation, the CSI quantization error could not be critical, especially for high rank case. Therefore, with regard to the UE complexity and CSI report overhead, one possible way is to hybrid AI/ML based and non-AI/ML based CSI, where the UE reports the CSI based on AI/ML if it reports a small RI and the UE can report the CSI based on Type1 codebook if it reports a large RI.

Proposal 12: Support hybrid AI/ML based and non-AI/ML based CSI measurement and report

- UE reports the CSI based on AI/ML if it reports a small RI and the UE can report the CSI based on Type1 codebook if it reports a large RI

## Inter-vendor collaboration

Direction A (Sharing parameter/reference model/dataset)

For direction A, the following options have been agreed for further study. It is also concluded that there is no proprietary information concern for both options below. Since RAN1 sent an LS to RAN2 about the feasibility for both 4-1 and 3a-1, the potential down-selection can be deferred until RAN2 provides the responses.

Agreement For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Proposal 13: Defer further discussion on 4-1 and 3a-1 for Direction A until receiving RAN2’s response

Direction B (Sharing NW side encoder parameter)

Sharing NW side encoder parameter could be helpful to avoid the model mismatch. But it could require the NW to disclose some proprietary information. Such parameter sharing should be done offline. Therefore, there should be no overhead concern. UE may identify the UE model based on the NW side model, and it is unnecessary to use a common encoder across UEs.

Proposal 14: For direction B, it should not require a common encoder across UEs.

Direction C (fully standardized reference model(s))

In RAN1 #119, the following on fully standardized reference model has been agreed.

Agreement For studying the standardized model structure, - For temporal domain Case 0, - In case of spatial-frequency domain input, adopt Transformer as the backbone structure. - Further study angular-frequency and angular-delay domain input - For temporal domain Case 2, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation with additional layers - Conv-LSTM - LSTM - Option 2: Latent adaptation on top of Case 0 model structure - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots - Note: other options are not precluded - Further study angular-frequency and angular-delay domain input - For temporal domain Case 3, - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation: - Option 1: Input/output adaptation - Option 2: Latent adaptation - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input. - Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure. - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered. - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used.

The remaining issue is the adaptation for Case 2 and Case 3. Option 1 requires additional layers and Option 2 reuses the same structure and change the feedback payload over different slots. For temporal domain CSI prediction, one use case is to measure one CSI and predict one CSI, which is similar to Case 0. Therefore, it is better to have a unified model structure for all cases. Compared to Option 1, Option 2 is preferred.

Proposal 15: For direction C, Option 2 (Latent adaptation) is preferred compared to Option 1 (Input/output adaptation with additional layers)

# Conclusion

In this contribution, we provided discussion on AI/ML based CSI compression. Based on the discussion, the following proposals are provided.

Proposal 1: For case 2 and case 3, support the UE to report W1 and compressed W2 for a configured rank

- The compressed W2 is calculated based on the AI/ML

- NW can configure the UE to report RI and CQI based on precoded CSI-RS resources after receiving the W1 and compressed W2

Proposal 2: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report.

Proposal 3: Support the CPU occupancy rule for ML based CSI based on two types processing unit

- Type1 CPU: a measurement processing unit (MPU) used for channel estimation and pre-processing

- Type2 CPU: an inference processing unit (IPU) used for inference for ML based CSI

Proposal 4: For CSI report based performance monitoring, the following spec impact should be considered:

- New report quantity PMI only should be introduced, where UE reports the PMI based on a configured rank

  - Support a further enhancement to report subband L1-SINR in addition to the PMI to facilitate the precoder and MCS selection for PDSCH

- With regard to measurement accuracy in low SINR case, support the UE to report a state of CSI indicating the CSI is invalid for performance monitoring

- With regard to joint ML based CSI compression and prediction, support to configure whether the UE should perform the CSI predication based on ML or non-ML and the CSI quantization based on ML or non-ML for separate performance monitoring for ML based CSI prediction and ML based CSI compression.

Proposal 5: For SRS based performance monitoring, the following spec impact should be considered:

- Support to configure the SRS linked with a CSI-RS report configuration for ML based CSI, where the UE uses the same ports including antenna virtualization scheme to transmit the SRS and to receive the CSI-RS for ML based CSI

- Support burst based SRS with frequency hopping to facilitate the performance monitoring for wideband channel for coverage-limited UE

Proposal 6: For UE-side monitoring, the following spec impact should be considered:

- Introduce the hypothetical BLER as the metric for performance calculation

- Configuration of precoded CSI-RS for hypothetical BLER calculation

Proposal 7: For NW-side data collection, support the following enhancement for Rel-16 eType2 codebook and Rel-18 eType2 codebook for PMI prediction

- Additional O1 and O2 value

- Additional codebook parameter combinations at least for rank 3 and 4

- UE reports the measured CSIs instead of predicted CSI for Rel-18 eType2 codebook for PMI prediction

- Additional number of DD basis for Rel-18 eType2 codebook for PMI prediction

Proposal 8: For NW-side data collection, the potential spec impact for the configured number of layers can be based on introducing a constraint for the RI restriction to configure one RI for the CSI report.

Proposal 9: For NW-side data collection, support the UE to report the following additional information

- L1-SINR measured based on the CSI-RS to indicate the measurement quality

- Timing for the transmission occasion(s) of the CSI-RS for the measured CSI

Proposal 10: Support to configure multiple uplink power control parameter sets for SRS

- The SRS can be used for data collection and other functionalities, where different uplink power control parameter sets can be used for different functionalities

- The NW can indicate which one is applied for the SRS transmission dynamically

Proposal 11: Support both NW-configured and UE-requested based CSI-RS for UE-side data collection

- Corresponding number of CPUs should be occupied for the CSI-RS

- Associated ID should be provided

Proposal 12: Support hybrid AI/ML based and non-AI/ML based CSI measurement and report

- UE reports the CSI based on AI/ML if it reports a small RI and the UE can report the CSI based on Type1 codebook if it reports a large RI

Proposal 13: Defer further discussion on 4-1 and 3a-1 for Direction A until receiving RAN2’s response

Proposal 14: For direction B, it should not require a common encoder across UEs.

Proposal 15: For direction C, Option 2 (Latent adaptation) is preferred compared to Option 1 (Input/output adaptation with additional layers)