3GPP TSG RAN WG1 #120		R1-2500549

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.3.3

Source:	Google

Title:	ML Model and Data

Document for:	Discussion/Decision

# Introduction

In RAN1 #119 meeting, the following agreements on ML model and data have been agreed.

Agreement For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, - ID-X can be used for pairing the UE-part and the NW-part of a two-sided model - FFS: other information needed for pairing Agreement Regarding the relationship of model ID, first indication, and second indication for model transfer/delivery Case z4, further study the following options: - Opt.1: model ID consists of the information of the first indication and the second indication - E.g., model ID is a combination of the first and second indications - Opt.2: The second indication is assumed as the model ID - Opt.3: Model ID is assigned by network and is separated from the first indication and the second indication Conclusion Regarding MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, from RAN1 perspective, how to construct the dataset, including whether a dataset constructed from one cell or from multiple cells is up to network implementation. Conclusion For the study of model delivery/transfer Case z4, if the model delivery/transfer is directly used for inference, the following options are identified as the candidate solutions to determine the readiness of AI model with the transferred parameters for inference (either or combination of the following options) - Option 1: UE sends signaling to network to notify that the AI model with the transferred parameters is ready for activation - Option 2: The AI model with the transferred parameters can be assumed ready for activation from a minimum applicable time after the completion of model delivery/transfer

In this contribution, we provide some discussion on AI/ML model and data.

# Model Identification

The model identification was studied during Rel-18. Model identification is defined as follows according to 3GPP TR 38.843.

Model identification: A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.

In RAN1 #116 meeting, the following on MI has been agreed.

Agreement - To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases. - To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose - Note: other options are not precluded Observation The other options are proposed for model identification type B by companies during the discussion: - MI-Option 4. Model identification via standardization of reference models. (for CSI compression) - MI-Option 5. Model identification via model monitoring Agreement - Regarding MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) of model identification type B, RAN1 further study the following aspects: - Relationship between model ID and data collection related configuration(s) and/or indication(s) - Information transmitted from NW to UE (if any) - Information transmitted from UE to NW (if any) - The associated procedure - Usage/Applicable use case(s) of MI-Option 1 Note: whether MI-Option 1 is needed or not is a separate discussion

For MI type A, the model is identified without OTA signaling. Therefore, the model may be identified based on inter-vendor collaboration or non-3GPP signaling. Then in this case, the indication of a model ID can be assumed to be known by both NW and UE. There is no additional specification work required to exchange the communication of the indication of a model ID.

Proposal 1: For MI type A, it is assumed that the indication of a model ID is known by the NW and UE after UE connected to the NW.

- No additional specification work is required to maintain the same understanding between the NW and UE on the indication of a model ID.

For MI type B, the model is identified via OTA signaling, which can be initiated by the UE or NW. During the data collection procedure, to maintain the same understanding between the NW and UE only on some property for a model could be necessary. For UE-side model, the NW needs to know the property for the model input to configure or schedule the corresponding downlink signals for the model inference. The NW also needs to know the property for the model output, so that it can configure or schedule the corresponding uplink resource for model output report. Therefore, during the data collection procedure, the MI should assist the NW and UE to maintain the same understanding for the property of model input and model output. Thus, the MI-Option 1 should be necessary. For MI-Option 2 and MI-Option 3, since the dataset transfer and model transfer may not be essential steps, such options can be deprioritized.

Proposal 2: MI-Option 1 is necessary to assist the NW and UE to maintain the same understanding for the property of model input and model output, so that the NW can configure corresponding DL RS for the UE to identify the model input and configure corresponding UL resource for model output report.

# UE data collection

For UE data collection, the first issue is whether the UE data collection should be transparent or not. The UE data collection takes some computing cycles and memories from UE. For example, the UE data collection for AI/ML based CSI compression and prediction could take some CPUs. The UE data collection for AI/ML based BM could require the UE to receive some SSBs with Rx beam sweeping. Then the UE may not be able to receive other downlink signal in the same symbols with SSBs, although the SSBs are not configured with a functionality, e.g., RLM/BFD. Therefore, the NW and UE should maintain the same understanding on when the UE can perform data collection.

In RAN1 #117 meeting, the following has been agreed.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

An open issue is the UE assumption for the associated ID across cells. Cell-specific association ID could require higher complexity in UE side, since the UE may need to prepare different models for different cells. Therefore, it is necessary to consider cross-cell association for the associated ID. Thus, the NW can configure whether the associated ID for different cells can indicate the same additional conditions. Then it is possible for the UE to apply the same model for such cells.

Proposal 3: Support the NW and UE to maintain the same understanding on when the UE can perform data collection.

Proposal 4: Support the NW to configure whether the associated ID in different cells indicating the same additional conditions or not.

# Connections for model ID, ID-X (dataset ID) and associated ID

Currently several IDs have been agreed: model ID, ID-X (dataset ID) and associated ID. It is necessary to clarify the connection and use cases for these IDs. The ID-X is introduced in the following agreement in RAN1 #118b and RAN1 #119.

Agreement Regarding the study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, ID (denoted as ID-X) can be transmitted from network/network-side to UE/UE-side for the dataset. - Note: The notation “ID-X” is used for discussion purpose Agreement For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, - ID-X can be used for pairing the UE-part and the NW-part of a two-sided model - FFS: other information needed for pairing

Since the ID-X is used for model identification as well, but it is used to indicate a certain dataset for the model training. Therefore, its functionality is still to identify a model, which is the same as model ID. Then it is unnecessary to introduce a new ID for the same functionality. Thus, the ID-X should be model ID.

Further, for the inference configuration for two-side model, model-ID is already sufficient for the NW and UE to maintain the alignment for the models. It is unnecessary to provide associated ID for the inference configuration for two-side model. But for UE-side data collection, such associated ID could be helpful for the UE to identify the additional conditions in NW side to classify the data. For one-side model, the model-ID is unnecessary since the NW-side model and UE-side model are transparent. But the associated ID is necessary for both inference configuration and data collection.

Proposal 5: The ID-X for MI-Option2 is model ID.

Proposal 6: For two-side model, support the following configurations for the agreed IDs

- Support to configure the model ID in CSI report configuration for inference

- Support to configure the associated ID for CSI-RS resource set for CSI acquisition for UE-side data collection

Proposal 7: For one-side model, support the following configurations for the agreed IDs

- Support to configure the associated ID in CSI report configuration for inference

- Support to configure the associated ID for SSB/CSI-RS resource set for BM or CSI-RS resource set for CSI acquisition for UE-side data collection

- Model ID is not applicable for one-side model

# Conclusion

In this contribution, we provided discussion on AI/ML model and data. Based on the discussion, the following proposals are provided.

Proposal 1: For MI type A, it is assumed that the indication of a model ID is known by the NW and UE after UE connected to the NW.

- No additional specification work is required to maintain the same understanding between the NW and UE on the indication of a model ID.

Proposal 2: MI-Option 1 is necessary to assist the NW and UE to maintain the same understanding for the property of model input and model output, so that the NW can configure corresponding DL RS for the UE to identify the model input and configure corresponding UL resource for model output report.

Proposal 3: Support the NW and UE to maintain the same understanding on when the UE can perform data collection.

Proposal 4: Support the NW to configure whether the associated ID in different cells indicating the same additional conditions or not.

Proposal 5: The ID-X for MI-Option2 is model ID.

Proposal 6: For two-side model, support the following configurations for the agreed IDs

- Support to configure the model ID in CSI report configuration for inference

- Support to configure the associated ID for CSI-RS resource set for CSI acquisition for UE-side data collection

Proposal 7: For one-side model, support the following configurations for the agreed IDs

- Support to configure the associated ID in CSI report configuration for inference

- Support to configure the associated ID for SSB/CSI-RS resource set for BM or CSI-RS resource set for CSI acquisition for UE-side data collection

- Model ID is not applicable for one-side model