3GPP TSG RAN WG1 #120	   	 	  	                R1-2500569

Athens, Greece, February 17th – 21st, 2025

Agenda item:		9.2.1

Source:		LG Electronics

Title:			UE-initiated beam management

Document for:	Discussion and Decision

Introduction

RAN1 made the following agreements in RAN1#119 and in this contribution we discuss several remaining issues.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported: - Option-2: Type-1 CG PUSCH can NOT be transmitted Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group. - (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability Conclusion On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the case that the first PUCCH and the second PUSCH are from the different CG is NOT supported. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, down-select one of Alt-1 and Alt-2 for both mode-A and mode-B. - Alt-1 (dedicated SR): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR/reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - FFS: whether/how to enhance legacy SR to support notification of beam report transmission in Mode-B - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission for mode A and mode B will be separately discussed. Agreement On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation: - Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality. - Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality. - Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs Note: For Event-2, we have the following definition for scheme-1 and scheme-2 - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. Agreement On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement - Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced. - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1. Agreement Confirm the following working assumption in RAN1#117: On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. Agreement On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured - Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window. - The presence of this field is enabled by RRC with subjective to UE capability. - The presence of this field is only for the case that N > 1 and the time window and M are configured. - Option-3: No further enhancement. Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2 FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120 - Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration. - Option-2: A CSI trigger state can be associated with - only UE-initiated/event-driven beam reporting - or only legacy AP-CSI configuration - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases: - Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR - Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH - Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain. Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical. Send an LS to RAN4. Final LS in R1-2410914.

Discussion

  - First PUCCH transmission

Regarding first PUCCH transmission, there are several remaining issues. The first issue is the reliability of first PUCCH. For Mode A, if gNB fails to receive first PUCCH, UE cannot report UEI beam, so the reliability of first PUCCH should be ensured first. More importantly, for Mode B, if gNB fails to decode event report in first PUCCH, unexpected UL interference would be incurred from the UL transmission in the second PUSCH. This is because UE would transmit the second PUSCH but gNB is not aware of the transmission and may allocate the PUSCH RBs for other UL channel. Therefore, reliable reception of first PUCCH should be ensured and first PUCCH should be able to retransmit. To this end, like legacy SR retransmission mechanism, the prohibit timer can be used. Specifically, if the prohibit timer is not running, the UE can transmit first PUCCH and after transmitting first PUCCH, UE starts the prohibit timer. In addition, it is beneficial to start the prohibit timer after second PUSCH transmission. This is because, depending on network situation, gNB can updates beams, not right after UEI beam is reported. Therefore, UE should wait for gNB’s beam updates, for certain amount of time after reporting UEI beams. If UE transmit first PUCCH again without waiting gNB’s beam updates, it just results in the waste of UL resource and UE Tx power.

Proposal#1: For first PUCCH (re)transmission, introduce prohibit timer; one starts at the first symbol after the end of the PUCCH transmission and another starts at the first symbol after the end of the PUSCH transmission.

- if the both prohibit timers are NOT running, the UE can transmit first PUCCH

Another issue for first PUCCH is how to support multi-CC or multi-event case. One way to address this issues is to use multi-bits first PUCCH. By using multi-bits, gNB can distinguish which CC or which event occurs and prepare following second PUSCH reception for the CC or the event. Another way is to associate a 1-bit first PUCCH with multiple CSI report configurations corresponding to multi-CC or multi-event. For example, one CSI report configuration is used for Event 2 and another CSI report configuration is used for Event 7, and they share the same 1-bit PUCCH. In this case, for mode A, since gNB cannot distinguish which event is triggered between Event 2 and 7, it may trigger AP CSI for both events, resulting in UL resource waste. For mode B, UE needs to report beams for both events in order to avoid gNB blind decoding for variable reporting payload size, which causes UL resource waste as well.

Proposal#2: Support multi-bits first PUCCH for multi-CC or multi-event case.

Regarding the multiplexing/dropping rule(s) of first PUCCH, there are three cases. Case-1 is the first PUCCH is collided with a PUCCH carrying normal SR and/or a PUCCH with normal LRR. In this case LRR should have the top priority since beam failure is most urgent event. Given that beam quality affects throughput and coverage of DL/UL channel, it is desirable that first PUCCH related to beam quality has higher priority than normal SR. Case-2 is when the 1-bit first PUCCH is collided with a PUSCH. In this case, instead of dropping one of the two channel, it is beneficial to Piggyback 1-bit indication of first PUCCH into the PUSCH. Case-3 is when the 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated beam reporting are overlapping in the time domain. In this case, priority can be determined based on CSI report configuration but if Event 1 is not associated with CSI report configuration it is not available option. Also, given that Event 1 is a light version of BFD event since event definition is the same but event evaluation metric is simpler, it makes sense that first PUCCH for event 1 has higher priority than other events.

Proposal#3: For the multiplexing/dropping rule(s) of first PUCCH, support the following priority:

Case-1: LRR>first PUCCH>normal SR

Case-2: Support piggyback 1-bit indication of first PUCCH into the PUSCH

Case-3: First PUCCH for Event 1 has higher priority than first PUCCH for other events

  - Second PUSCH transmission

For mode A, legacy AP beam report mechanism can be reused and there is no reason to support other UL channel formats. Also, there is no reason to introduce dedicated CSI trigger state for UEI BM. It is up to gNB implementation whether to use one CSI trigger state which contains UEI beam report and legacy beam report or contains UEI beam report only.

Proposal#4: Step 2 and Step3 of Mode A are same as those for legacy AP beam report on PUSCH, i.e. reusing legacy AP CSI request field and CSI trigger state without limitation.

CSI reporting priority of second PUSCH could be set higher than normal CSI/beam due to its event-driven nature. Also, if multiple events can be configured to a UE, CSI priority may be set differently per event.

Proposal#5: CSI reporting priority of second PUSCH should be set higher than normal CSI/beam report considering its event-driven nature.

UCI multiplexing needs to be considered especially when multiple events are associated with a 1-bit PUCCH. Specifically, even if first PUCCH is reported because an event occurs, some events may not occur since multiple events shares the first PUCCH. In that case, contents and payload size of second PUSCH is not clear since gNB does not know which event occurs. Also, in that case, how to multiplex this second PUSCH payload with another UCI also should be discussed.

Proposal#6: If multiple events are associated with a 1-bit first PUCCH, how to determine contents and payload size of second PUSCH should be discussed.

In the last meeting, how to reduce beam application latency after second PUSCH transmission is shortly discussed and three alternatives are studied. In our view, reducing beam application time is also important to reduce overall latency of beam management. This latency comes from RRC/MAC-CE signaling, so it is good to apply new beam without those signaling.

Proposal#7: To reduce beam application latency after second PUSCH transmission, Alt-1 can be considered.

Conclusion

In this contribution, the following proposals are provided.

Proposal#1: For first PUCCH (re)transmission, introduce prohibit timer; one starts at the first symbol after the end of the PUCCH transmission and another starts at the first symbol after the end of the PUSCH transmission.

- if the both prohibit timers are NOT running, the UE can transmit first PUCCH

Proposal#2: Support multi-bits first PUCCH for multi-CC or multi-event case.

Proposal#3: For the multiplexing/dropping rule(s) of first PUCCH, support the following priority:

Case-1: LRR>first PUCCH>normal SR

Case-2: Support piggyback 1-bit indication of first PUCCH into the PUSCH

Case-3: First PUCCH for Event 1 has higher priority than first PUCCH for other events

Proposal#4: Step 2 and Step3 of Mode A are same as those for legacy AP beam report on PUSCH, i.e. reusing legacy AP CSI request field and CSI trigger state without limitation.

Proposal#5: CSI reporting priority of second PUSCH should be set higher than normal CSI/beam report considering its event-driven nature.

Proposal#6: If multiple events are associated with a 1-bit first PUCCH, how to determine contents and payload size of second PUSCH should be discussed.

Proposal#7: To reduce beam application latency after second PUSCH transmission, Alt-1 can be considered.