3GPP TSG RAN WG1 #120												  R1-2500599

Athens, Greece, February 17th – 21st, 2025

Agenda item:	5

Source:	NEC

Title:              	Discussion on RAN2 LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP

Document for:	Discussion and Decision

- Introduction

In RAN2 LS [1], differentiation of sDCI mTRP, mDCI mTRP and sTRP was discussed.

In this contribution, we showed our understanding on the issue.

- Discussion

sDCI mTRP/sTRP vs mDCI mTRP

First remaining issue is the 3-bit scaling factor for RI>1.

RAN2 LS simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list.. Question 1a: RAN2 would like to ask if RAN1 has any concern on the above highlight sentence? Question 1b: If there is any concern on Question 1a, what’s RAN1’s suggestion on how to separate the cells operated as sDCI mTRP/sTRP and mDCI mTRP?

Based on current TS, coresetPoolIndex can be configured (value 0 or 1) or not configured (value 0 applied) for a CORESET (as shown in TS 38.331, if coresetPoolIndex is not configured, the value 0 is applied).

coresetPoolIndex The index of the CORESET pool for this CORESET as specified in TS 38.213 [13] (clauses 9 and 10) and TS 38.214 [19] (clauses 5.1 and 6.1). If the field is absent, the UE applies the value 0.

In RAN1 TS, mDCI based multi-TRP is actually based on two values of CORESETPoolIndex, in other words, if there is only one value for coresetPoolIndex, it’s not mDCI based multi-TRP. Note that if coresetPoolIndex is not configured for all CORESETs, it’s same feature (not mDCI based multi-TRP) as coresetPoolIndex configured with a single value for all CORESETs.

TS38.213 If a UE can support - a first set of Ncells,0DL serving cells where the UE is either not provided coresetPoolIndex or is provided coresetPoolIndex with a single value for all CORESETs on all DL BWPs of each scheduling cell from the first set of serving cells, and - a second set of Ncells,1DL serving cells where the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with a value 0 for a first CORESET, and with a value 1 for a second CORESET on any DL BWP of each scheduling cell from the second set of serving cells the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, pdcch-BlindDetectionCA1, and pdcch-BlindDetectionCA3, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is a value reported by the UE.

For each DL BWP configured to a UE in a serving cell, the UE can be provided by higher layer signalling with - P≤3 CORESETs if coresetPoolIndex is not provided, or if a value of coresetPoolIndex is same for all CORESETs if coresetPoolIndex is provided - P≤5 CORESETs if coresetPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET For each CORESET, the UE is provided the following by ControlResourceSet: - a CORESET index p, by controlResourceSetId or by controlResourceSetId-v1610, where - 0<p<12 if coresetPoolIndex is not provided, or if a value of coresetPoolIndex is same for all CORESETs if coresetPoolIndex is provided; - 0<p<16 if coresetPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET;

If a UE is provided two coresetPoolIndex values 0 and 1 for first and second CORESETs, or is not provided coresetPoolIndex value for first CORESETs and is provided coresetPoolIndex value of 1 for second CORESETs, respectively, a MAC CE command activating TCI states for the first or second CORESETs [11, TS 38.321] can include coresetPoolIndex value 0 or 1 - if the UE is provided SSB_MTC_AdditionalPCI, the activated TCI states for the first and/or the second CORESETs are for physCellId from ServingCellConfigCommon and the activated TCI states for either the first or the second CORESETs can be for physCellId from additionalPCI.

Based on this, following two examples leading to different number of coresetPoolIndexes, but with same feature of mDCI based multi-TRP

- coresetPoolIndex is not configured for all CORESETs in a first cell; coresetPoolIndex configured with a single value for all CORESETs in a second cell, both the two cells are not mDCI based multi-TRP, while in this example, the number of coresetPoolIndexes is actually different for the two cells (0 vs 1).

- coresetPoolIndex is not configured for first CORESETs, and coresetPoolIndex configured with value 1 for second CORESETs in a first cell (it’s mDCI based multi-TRP case, and the number of coresetPoolIndex is actually 1); coresetPoolIndex is configured with value 0 for first CORESETs, and coresetpoolIndex configured with value 1 for second CORESETs in second cell (it’s mDCI based multi-TRP case, and the number of coresetPoolIndexes is 2)

Another example leading to same number of coresetPoolIndex, but different features.

- coresetPoolIndex is not configured for first CORESETs, and coresetPoolIndex configured with value 1 for second CORESETs in a first cell (it’s mDCI based multi-TRP case, and the number of coresetPoolIndex is actually 1); coresetPoolIndex configured with a single value for all CORESETs in a second cell (it’s not mDCI based multi-TRP case, but the number of coresetPoolIndex is also 1).

Table 1. Examples for corestPoolIndex and mDCI based multi-TRP

| Configuration of coresetPoolIndex | Number of values for coresetPoolIndex | Number of coresetPoolIndex | mDCI based multi-TRP or not |
| --- | --- | --- | --- |
| Not configured for all CORESETs | Can be regarded as 1 | 0 | No |
| Configured with single value for all CORESETs | 1 | 1 | No |
| Not configured for some CORESETs, configured with 0 for other CORESETs | 1 | 1 | No |
| Not configured for first CORESETs, configured with 1 for second CORESETs | 2 | 1 | Yes |
| Configured with 0 for first CORESETs, configured with 1 for second CORESETs | 2 | 2 | Yes |

As shown in the Table 1, we can see only based on number of coresetPoolIndexes can not well differentiate whether it’s mDCI based multi-TRP or not, instead, the number of values for coresetPoolIndex can be applied. So to be more accurate, we prefer to differentiate the mDCI based multi-TRP feature based on number of values of coresetPoolIndex.

Proposal 1: Differential on mDCI based multi-TRP based on number of values of coresetPoolIndex.

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of values of coresetPoolIndexes in the same list.

sTRP vs sDCI mTRP

RAN2 LS simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list. For each list, the network ensures that either applyIndicatedTCI-State is configured for all ControlResourceSets in all BWPs of all serving cells or applyIndicatedTCI-State is not configured in any ControlResourcesSet of any BWP of any serving cell.. Question 2a: RAN2 would like to ask if RAN1 has any concern on the above highlight sentence? Question 2b: If there is any concern on Question 2a, what’s RAN1’s suggestion on how to separate the cells operated as sTRP and sDCI mTRP?

Based on current TS, followUnifiedTCI-state is configured per ControlResourceSet, and applyIndicatedTCI-State is conditioned on followUnifiedTCI-state.

TS 38.331 ControlResourceSet ::= SEQUENCE { controlResourceSetId ControlResourceSetId, frequencyDomainResources BIT STRING (SIZE (45)), duration INTEGER (1..maxCoReSetDuration), cce-REG-MappingType CHOICE { interleaved SEQUENCE { reg-BundleSize ENUMERATED {n2, n3, n6}, interleaverSize ENUMERATED {n2, n3, n6}, shiftIndex INTEGER(0..maxNrofPhysicalResourceBlocks-1) OPTIONAL -- Need S }, nonInterleaved NULL }, precoderGranularity ENUMERATED {sameAsREG-bundle, allContiguousRBs}, tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB-initialBWP tci-StatesPDCCH-ToReleaseList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId OPTIONAL, -- Cond NotSIB-initialBWP tci-PresentInDCI ENUMERATED {enabled} OPTIONAL, -- Need S pdcch-DMRS-ScramblingID INTEGER (0..65535) OPTIONAL, -- Need S ..., [[ rb-Offset-r16 INTEGER (0..5) OPTIONAL, -- Need S tci-PresentDCI-1-2-r16 INTEGER (1..3) OPTIONAL, -- Need S coresetPoolIndex-r16 INTEGER (0..1) OPTIONAL, -- Need S controlResourceSetId-v1610 ControlResourceSetId-v1610 OPTIONAL -- Need S ]], [[ followUnifiedTCI-State-r17 ENUMERATED {enabled} OPTIONAL -- Need R ]], [[ applyIndicatedTCI-State-r18 ENUMERATED {first, second, both, none} OPTIONAL -- Cond FollowUTCI ]] }

In our understanding, not all CORESETs need to be configured with followUnifiedTCI-State, especially for those associated with CSS. And even some CORESETs not configured with followUnifiedTCI-State, it doesn’t impact on the feature of single-TRP or multi-TRP.

So we prefer to make following update:

Proposal 2: Update the description with:

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list. For each list, the network ensures that either applyIndicatedTCI-State is configured for all ControlResourceSets that configured with followUnifiedTCI-State in all BWPs of all serving cells or applyIndicatedTCI-State is not configured in any ControlResourcesSet of any BWP of any serving cell.

- Conclusion

In this contribution, we provided our views on the LS from RAN2, and we propose:

Proposal 1: Differential on mDCI based multi-TRP based on number of values of coresetPoolIndex.

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of values of coresetPoolIndexes in the same list.

Proposal 2: Update the description with:

simultaneousU-TCI-UpdateList1, simultaneousU-TCI-UpdateList2, simultaneousU-TCI-UpdateList3, simultaneousU-TCI-UpdateList4 List of serving cells for which the Unified TCI States Activation/Deactivation MAC CE applies simultaneously, as specified in TS 38.321 [3] clause 6.1.3.47. The different lists shall not contain same serving cells. Network only configures in these lists serving cells that are configured with unifiedTCI-StateType. Network should not configure serving cells that are configured with a BWP with different number of coresetPoolIndexes in the same list. For each list, the network ensures that either applyIndicatedTCI-State is configured for all ControlResourceSets that configured with followUnifiedTCI-State in all BWPs of all serving cells or applyIndicatedTCI-State is not configured in any ControlResourcesSet of any BWP of any serving cell.

- References

- R1-2500013, LS on differentiation of sDCI mTRP, mDCI mTRP and sTRP, RAN2, Chair’s notes, RAN1#120, Athens, Greece, February 17th – 21st, 2025