3GPP TSG RAN WG1 #120		R1-2500603

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.6.2

Source:	NEC

Title:              	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode

Document for:	Discussion and Decision

Introduction

In the RAN1#119, the following agreements and conclusion on LP-WUS operation in RRC-IDLE/INACTIVE mode were achieved:

Agreement Confirm the following working assumption with the addition of the note at the bottom: Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption. Agreement At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams. Conclusion Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams. Conclusion No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams. Agreement For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1 Agreement At least for the 1:1 LO to PO mapping, - The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4. Conclusion There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping. Agreement Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. Working Assumption If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration. Conclusion From RAN1 perspective, there is no consensus on the following proposal: UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if the UE has not received LP-WUS for a long time.

In this contribution, we provide our views on LP-WUS operations in RRC IDLE/INACTIVE mode.

Discussion

The LP-WUS configuration should at least comprise the resource configuration information for the LP-WUS. As indicated in the WID, duty-cycle based LP-WUS monitoring is supported, so a periodicity and offset of LO can be comprised in the LP-WUS configuration. In addition, in order to improve the flexibility of LP-WUS resource configuration and to increase the number of supported UE subgroups, it is beneficial to support more than one LP-WUS frequency bands in a carrier.

Fig.1 an example of LP-WUS resources

Proposal 1: for RRC-IDLE/INACTIVE, the LP-WUS configuration indicates one or more LP-WUS frequency bands in a carrier, and for each LP-WUS frequency band, a periodicity and an offset of LOs are comprised in the configuration.

Fig.2 an example of association between LOs and POs

In general, the wake-up indication should be UE subgroup based in order to control the overhead, this means a wake-up indication may cause a subgroup of UEs to wake up. However, a paging false alarm may still occur for a UE if it belongs to a subgroup however the paging is intended to another UE in the same subgroup. In order to reduce the paging false alarm probability, it is beneficial to support a relatively tight subgroup with a small number of UEs, in other words, a relatively large number of UE subgroups should be supported.

On the other hand, in order to improve the coverage, the LP-WUS may have a relatively small payload size, e.g., several bits to tens of bits. Therefore, the number of subgroups indicated in a single LP-WUS will be limited. In our view, time/frequency resources for LP-WUS can also be used as a subgrouping method. Specifically, gNB may configure relatively dense LOs in the time and frequency domain, and UE may select a subset of the LOs based on its UE-ID or a PF/PO index it is intended to monitor. Through this method, UEs are firstly assigned into different groups for different LO subsets, and then a finer subgrouping can be applied within the UEs in the same group, e.g., the wake-up indication in an LP-WUS content can be based on the subgroup.

An open issue is whether an explicit association between an LO and a PO (e.g., by a time offset) should be defined. In our view, it is true that the preparation time (i.e., offset) exists, however it does not mean gNB cannot transmit the paging message before the end of preparation time, because gNB may be not aware of whether the target UE is monitoring LP-WUS (it may depend on the MR/LR measurement results), so gNB may transmit the paging information earlier for a UE which is assumed not monitoring LP-WUS. In addition, considering gNB can retransmit the paging message several times if the target UE does not respond, therefore the paging message will not be missed by the target UE even gNB start to transmit paging message before the preparation time.

In RAN1#117, three options for the linkage between PO and LP were agreed. In our view, both the three options can be supported, and it depends on gNB configuration. From UE perspective, it is not necessary to be aware which option is used by gNB, UE can determine its own LO based on gNB configuration, and monitor a PO which is associated with the LO.

Observation 1: explicit association between an LO and a PO (e.g., by a time offset) is not necessary, UE can determine the association based on its UE-ID implicitly.

Proposal 2: for the association between LO and PO, support option 1, option 2 and option 3, UE determine which option is used based on gNB configuration.

Proposal 3: For the offset value(s) between an LO and a reference PO/PF, support configurability between Option 1-1 and Option 1-2.

Conclusion

In this contribution, we discuss the LP-WUS operations in RRC IDLE/INACTIVE modes, and the following observations and proposals are made:

Proposal 1: for RRC-IDLE/INACTIVE, the LP-WUS configuration indicates one or more LP-WUS frequency bands in a carrier, and for each LP-WUS frequency band, a periodicity and an offset of LOs are comprised in the configuration.

Observation 1: explicit association between an LO and a PO (e.g., by a time offset) is not necessary, UE can determine the association based on its UE-ID implicitly.

Proposal 2: for the association between LO and PO, support option 1, option 2 and option 3, UE determine which option is used based on gNB configuration.

Proposal 3: For the offset value(s) between an LO and a reference PO/PF, support configurability between Option 1-1 and Option 1-2.

References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC, RAN meeting #102, Dec. 11-15, 2023