3GPP TSG RAN WG1 #120		                                                    R1-2500609

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.11.4

Source:	NEC

Title:	IoT-NTN uplink capacity/throughput enhancement

Document for:	Discussion

- Introduction

In RAN1#119, the following agreements on IoT-NTN uplink capacity/throughput enhancement have been established[1]:

Agreement For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2 for connected mode. Agreement For single tone 15kHz SCS Slot-level OCC for NPUSCH format 1, OCC length larger than 2 is not supported. Agreement For NPUSCH Format 1 single-tone 15kHz SCS, RAN1 studies at least the following options for CDM DMRS with legacy pattern for down-selection: - Option 1: DMRS symbols are spread before the OCC is applied, e.g. according to the formula: ru,OCCn=ru(nM)q(n mod M) Where: M is the OCC length, q is the assigned OCC codeword for the UE and run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1 - Option 2: DMRS symbols are not spread before the OCC is applied. - Option 2_1: OCC is applied to the legacy complex-valued DMRS symbol used in slot 1 and slot 2, e.g. according to the formula: ru,OCCn=ru(n)q(n mod M) - Option 2_2: OCC is applied to the complex-valued DMRS symbol used in slot 1 and slot 2. Depending on the OCC codeword, different DMRS sequence is used. - Option 3: DMRS symbols are not spread and OCC is not applied. - Legacy complex-valued DMRS symbol is used in slots corresponding to an OCC codeword of NPUSCH. Different DMRS sequences are used for multiplexed UEs. Agreement For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot. - Note: whether RU length is extended or not after applying OCC is a separate discussion. Agreement For support of single-tone OCC for NPUSCH format 1 for connected mode, the parameters that need to be signalled are: - OCC sequence index - Enabling of OCC feature - FFS: whether signaling is explicit or implicit

In this contribution, we provide further views on the uplink capacity enhancement for IoT-NTN.

- NPUSCH

15 kHz SCS DMRS

The agreed down-selected options for CDM DMRS with legacy pattern are illustrated in Fig. 1.

Fig. 1 CMD DMRS Options

Option 1 employs slot-level spreading where NPUSCH symbols and DMRS symbols are jointly spread across the slot, followed by separate OCC applications in each slot. This straightforward approach maintains DMRS orthogonality between paired UEs through slot-level OCC operations on both symbol types.

Option 2-1 implements separate spreading processes for NPUSCH and DMRS symbols, eliminating slot-level spreading for DMRS. This option only involves OCC codeword multiplexing the symbols at the slot level. This option might not work for all the DMRS sequences in the current specification.

Option 2-2 shares the same non-spreading characteristic for DMRS symbols as Option 2-1, but introduces an additional requirement for distinct DMRS sequences across different slots. As specified in Clause 10.1.4.1.1 of [7], the single-tone DMRS sequence for NPUSCH format 1 is:

run=run

Where run is :

ru(n)=121+j1-2cnwnmod16,0≤n<MrepNPUSCHNslotsULNRU

The M-sequence initialization (c_init = 35 at NPUSCH transmission start) and w(n) values from Table 10.1.4.1.1-1 in [7] (where u=NIDNcellmod16 when group hopping is disabled for NPUSCH format 1) result in a fixed DMRS sequence for single-tone NPUSCH. Implementing different sequences for different DMRS symbols would consequently require substantial specification modifications.

Option 3 similarly omits slot-level spreading for DMRS symbols like Option 2 variants, but further removes slot-level multiplexing operations. The requirement for distinct DMRS sequences between slots introduces specification impacts equivalent to those described in Option 2-2.

Observation 1: Option 3 and Option 2-2 introduce substantial specification impacts due to their requirements for dynamic DMRS sequence variations, Option 2-1  fails to guarantee full-scenario compatibility, as its non-spread DMRS implementation compromises orthogonality between paired UEs under certain DMRS sequences defined in current specification.

Observation 2: Option1 could ensure:

- Backward compatibility with legacy DMRS designs.

- Orthogonality preservation through unified OCC operation.

- Minimal specification impact by leveraging existing sequence determination and slot-level OCC operation frameworks.

Proposal 1: Support DMRS symbols are spread and multiplexed with OCC together with NPUSCH symbols for 15 kHz SCS CDM DMRS with legacy pattern

SCS 3.75 kHz

For the single carrier NPUSCH with SCS 3.75 kHz, where 5 slots are a frame, the slot duration is 61440 Ts or 2 ms, and each slot contains 7 symbols [2]. Thus, the gap between any two DMRS symbols will be an integer multiplied by 2/7 ms.

Based on the agreements in RAN1#116-bis, the frequency error is a uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs, and the carrier frequency is 2 GHz [1]. Thus, the maximum frequency error is 400 Hz.

To ensure the gNB could estimate the frequency error based on the DMRS symbols without phase rotation ambiguity, the maximum phase rotation between two DMRS symbols should be smaller than 2π. As the maximum frequency error is 400Hz, the gap between two DMRS symbols should be smaller than 2.5 ms, which is 8.75 symbols for SCS 3.75 KHz and 35 symbols for 15 KHz. Thus, a maximum distance between two DMRS symbols of 8 symbols for 3.75 KHz and 35 symbols for 15 KHz could meet the CFO estimation requirement of the current simulation assumption.

Observation 3: To meet the CFO estimation requirement of the current simulation assumption, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 2.5 ms.

Observation 4: For the single carrier, NPUSCH with 3.75 KHz SCS, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 8 symbols to meet the CFO estimation requirement.

Proposal 2: For the single carrier, NPUSCH with 3.75 KHz SCS, CDM DMRS with a new pattern should be studied to meet the CFO estimation requirement for OCC length 2, symbol-level.

The two-level gap is proposed in [3], which is similar to the NPRACH “two-level hopping”. Within a DMRS cluster, the gap between two multiplexed consecutive DMRS symbols is x1. Between two DMRS clusters, the gap between two multiplexed consecutive DMRS symbols is x2. For symbol-level OCC of 3.75 KHz SCS NPUSCH, the x1 is strictly constrained to meet the CFO estimation requirement, and the x2 is constrained to retain the same DMRS overhead as the legacy.

Proposal 3: For 3.75 kHz symbol-level OCC, the multiplexed DMRS pattern contains the first level gap between two consecutive DMRS symbols within a DMRS cluster and the second level gap between two consecutive DMRS symbols across two DMRS clusters, where the first level gap should be smaller or equal to 8 symbols and the second level gap should keep the DMRS overhead same as the legacy NPUSCH without OCC.

Based on clause 10.1.4.2 of TS 36.211, the DMRS is mapped to the physical resource with an explicit location for NPUSCH. Thus, following the legacy mapping procedure, the OCC length 2 CDM/TDM DMRS location for NPUSCH can be similarly specified.  Following the first- and second-level gap rule, some potential CDM DMRS locations for NPUSCH are presented in Fig. 1.

Fig. 2 Potential CDM DMRS locations for NPUSCH

Proposal 4: Study the CDM DMRS locations for NPUSCH with OCC length 2 for 3.75 kHz symbol-level OCC for NPUSCH format 1.

- NPRACH

In RAN2#127bis, companies discussed the working point for CB-msg3 EDT [4], and RAN2 focus on a working point where the PLR is lower than 10% for CB-msg3 EDT [5]. For DSA or SA, when the PLR is lower than 10%, the active user number is limited which is lower than 30% of the orthogonal transmission opportunity number[6]. For CB-msg3 EDT, 70% of the UL resources are wasted to ensure limited users can take some advantage to access the network with reduced delay.

Observation 5: CB-msg3 EDT could only work under a user overloading factor below 30%, which will waste the limited uplink resource and could not enhance the uplink capacity or throughput.

For NB-IoT, only type-B half-duplex FDD operation is supported [7]. In this type of operation, guard periods (GP, illustrated in Fig. 2), each referred to as a half-duplex guard subframe, are created by the UE by

-	not receiving a downlink subframe immediately preceding an uplink subframe from the same UE, and

-	not receiving a downlink subframe immediately following an uplink subframe from the same UE.

Fig. 3 Type-B half-duplex FDD

For a terminal using a UL subframe to transmit NPRACH, it can not transmit NPUSCH or receive any DL signal. To better utilize the limited UL resource, reduce the UL subframe for NPRACH, and enhance the transmission opportunities for NPUSCH and NPDSCH, NPRACH enhancement with OCC is necessary.

Proposal 5: NPRACH enhancement with OCC is essential to improve IoT NTN uplink capacity and throughput in R-19.

For the legacy NPRACH preamble component, each preamble repetition contains P symbol groups, each group contains one CP + N symbols as illustrated in Fig. 3 and Fig. 4.

Fig. 4 NPRACH symbol

Fig. 5 NPRACH symbol group

For intra-symbol group OCC, additional CP might be required to ensure the orthogonal property. In an NPRACH symbol group, all the symbols are identical. Thus, the third symbol of NPRACH Format 1 can be treated as the second CP for the NPRACH symbols multiplexed with the second OCC code word as illustrated in Fig. 5. for OCC length = 2 cases.

Fig. 6 NPRACH intra-symbol group OCC

From the UE perspective, UE multiplex the S1 and S2 with the first code word; S3, S4, and S5 with the second code word, and then added the CP as legacy. There is no need to specify a new NPRACH symbol group structure or add new procedures for processing the additional CP.

From the gNB perspective, for any NPRACH with intra-symbol group OCC, whether treat the third symbol of an NPACH symbol group as a CP or NPRACH symbol is left to the network implementation.

As illustrated in Fig. 5, the legacy terminals can use the spreading code word [+1, +1], which is the same as the legacy PRACH transmission.

Observation 6: Enhancing NPRACH with OCC-2 has no backward compatibility issue.

Proposal 6: Adopting the same OCC sequence with NPUSCH for NPRACH.

Proposal 7: For NPRACH with OCC, the intra-symbol group OCC should be supported at least with the OCC length =2 cases.

For PUCCH with OCC, the codebook index is configured by the Network. During a random access procedure, the codebook index will be selected randomly.  The UEs might select the same codebook index resulting in codebook index conflicts.

Proposal 8: Support study of the performance of different codebook index hopping mechanisms for NPRACH with OCC in uplink capacity enhancement for IoT-NTN.

The legacy Random Access procedure is performed on the anchor carrier or one of the non-anchor carriers for which the PRACH resource has been configured in system information. For a legacy NB-IoT UE, when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities [8]:

- the selection probability of the anchor carrier PRACH resource for the given enhanced coverage level, nprach-ProbabilityAnchor, is given by the corresponding entry in nprach-ProbabilityAnchorList

- the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

Proposal 9: Support study if the anchor carrier and non-anchor carrier selection probability could be enhanced to multiplex NPRACH with OCC and legacy NPRACH  in uplink capacity enhancement for IoT-NTN.

- Conclusion

From the discussion, we have the following observations and proposals.

Observation 1: Option 3 and Option 2-2 introduce substantial specification impacts due to their requirements for dynamic DMRS sequence variations, Option 2-1  fails to guarantee full-scenario compatibility, as its non-spread DMRS implementation compromises orthogonality between paired UEs under certain DMRS sequences defined in current specification.

Observation 2: Option1 could ensure:

- Backward compatibility with legacy DMRS designs.

- Orthogonality preservation through unified OCC operation.

- Minimal specification impact by leveraging existing sequence determination and slot-level OCC operation frameworks.

Observation 3: To meet the CFO estimation requirement of the current simulation assumption, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 2.5 ms.

Observation 4: For the single carrier, NPUSCH with 3.75 KHz SCS, the distance between two corresponding consecutive DMRS symbols should be equal to or smaller than 8 symbols to meet the CFO estimation requirement.

Observation 5: CB-msg3 EDT could only work under a user overloading factor below 30%, which will waste the limited uplink resource and could not enhance the uplink capacity or throughput.

Observation 6: Enhancing NPRACH with OCC-2 has no backward compatibility issue.

Proposal 1: Support DMRS symbols are spread and multiplexed with OCC together with NPUSCH symbols for 15 kHz SCS CDM DMRS with legacy pattern

Proposal 2: For the single carrier, NPUSCH with 3.75 KHz SCS, CDM DMRS with a new pattern should be studied to meet the CFO estimation requirement for OCC length 2, symbol-level.

Proposal 3: For 3.75 kHz symbol-level OCC, the multiplexed DMRS pattern contains the first level gap between two consecutive DMRS symbols within a DMRS cluster and the second level gap between two consecutive DMRS symbols across two DMRS clusters, where the first level gap should be smaller or equal to 8 symbols and the second level gap should keep the DMRS overhead same as the legacy NPUSCH without OCC.

Proposal 4: Study the CDM DMRS locations for NPUSCH with OCC length 2 for 3.75 kHz symbol-level OCC for NPUSCH format 1.

Proposal 5: NPRACH enhancement with OCC is essential to improve IoT NTN uplink capacity and throughput in R-19.

Proposal 6: Adopting the same OCC sequence with NPUSCH for NPRACH.

Proposal 7: For NPRACH with OCC, the intra-symbol group OCC should be supported at least with the OCC length =2 cases.

Proposal 8: Support study of the performance of different codebook index hopping mechanisms for NPRACH with OCC in uplink capacity enhancement for IoT-NTN.

Proposal 9: Support study if the anchor carrier and non-anchor carrier selection probability could be enhanced to multiplex NPRACH with OCC and legacy NPRACH  in uplink capacity enhancement for IoT-NTN.

References

- Chair’s Notes RAN1 #119

- TS 36.211

- R1-2405175, “IOT-NTN uplink capacity/throughput enhancement”, Qualcomm Incorporated

- R2-2409231

- RAN2#127 chairman notes

- R2-2408413

- TS 36.211

- TS 36.321