3GPP TSG RAN WG1 #120	R1-2500612

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.4.4

Source:	NEC

Title:	Discussion on control and other aspects of ambient IoT

Document for:	Discussion and Decision

1	Introduction

In RAN#106 meeting, a new WID (RP-243326)[1] is proposed with RAN1 details below.

The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this paper, we discuss and give our views on the aspects of multiple access, scheduling information and timing relationship of ambient IoT physical channel design.

2	Discussion

2.1	Scheduling information

2.1.1 Time domain resource indication

In TR 38.769[2], the time domain resource indication is discussed at part of scheduling information.

- 6.1.2.8 Scheduling of D2R For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions - Midamble (if supported) related information

In 5G NR, the time domain resource of PUSCH is indicated by a TDRA table, and each entry of the TDRA table represent a time duration with start symbol and length within a slot. For D2R transmission, the definition of slot no longer exists, and time domain resource indication should be re-designed.

In our opinion, there are two options to indicate the time domain resources. Option 1 is to indicate the chips or modulated symbol of the D2R transmission. To save indication overhead, the granularity of one unit could be one chip, M chips, groups of chips. Option 2 is to indicate a TB size of D2R transmission, and allow the device to derive the time domain resource based on the TB size. Option 2 seems simple, but it requires setting all rules for chip calculation. For example, rules to determine midamble need to be predetermined.

Proposal 1: Two options are discussed to indicate the time domain resource.

- Option 1: granularity of one chip, M chip or groups of chips;

- Option 2: TB size for device to calculate the time domain resource.

Besides indication for one D2R transmission, reader may need to schedule X time resource for Msg1 transmission. Such a case is mentioned in TR 38.769[2] as below.

- 6.1.4 Random access A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). The study includes: - X=1 and X>1 and X>=1. The maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain. - Determination of the X time domain resource(s) by the device. - Addressing timing errors for adjacent time domain resources due to residual SFO of the device.

To save standardization efforts, the time domain resource indication should be reused to indicate X without introducing another control format. Since Msg1 only contains 16-bit ID, and chips of one Msg1 transmission should be a fixed number TMsg1. The time domain resource indication can carry a value related to X×TMsg1. And the device can derive the value of X based on the indicated value and the fixed number TMsg1.

Proposal 2: The time domain resource indication is reused to indicate X time domain resources for Msg1 scheduling.

2.1.2 Time domain resource indication of Msg3

It is captured in TR 38.769 [2] that, for Msg2 transmission in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified:

Option 1: A PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device.

Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource.

Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources.

Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

Regarding different options of Msg2 transmission, resource allocation of Msg3 may vary from each other. To be specific, for Msg2 transmission with option 1, it is preferred to reuse resource allocation scheme of Msg1 for the resource allocation of single Msg3. With the size of Msg3 probably larger than that of Msg1, if index-based resource block is applied for Msg1, multiple indices of resource block would be applied for Msg3 resource allocation. Alternatively, if Msg1 resource allocation is reused directly, then an additional dynamic extension coefficient could be indicated for Msg3 resource allocation.

Proposal 3: For Msg2 transmission with option 1, following two options can be considered for resource allocation of Msg3.

- Option 1: Multiple indices of resource block indication.

- Option 2: Extension coefficient indication.

For Msg2 transmission with option 2, one Msg2 will contain multiple D2R grant corresponding to multiple devices, large TBS of Msg2 is inevitable. Therefore, a compact design/indication of resource allocation for a reduced TBS is preferred in this case. To achieve this, less resource allocation indication together with a pre-defined rule for extension can be considered. Specifically, assuming Msg2 transmission with option 2 support multiple Msg3 with only TDMA or FDMA mode. As shown in Fig.1(a), with one Msg3 resource indication and pre-defined rule on extension by a fixed gap automatically in time domain for TDMA mode, multiple TDM resources can be derived by device. Similarly, in Fig.1(b), with one Msg3 resource indication and pre-defined rule on extension by a fixed gap automatically in frequency domain for FDMA mode, multiple FDM resources can be derived by device. After that, with the same order of device ID related information indication in Msg2, multiple resources can be mapped to corresponding device accordingly. Through this manner, less resource allocation with a mode indication are sufficient for multiple Msg3 resource allocation. It is worth noting that, there would be multiple Msg2 in one round of random access. Even only TDMA or FDMA mode of Msg3 is supported for option 2 of Msg2 transmission, together with Msg3 resources indication by other Msg2(s) (either with option 1 or option 2), all Msg3 in one round work in TDMA and FDMA mode is achieved. However, with this separation on Msg2 transmission with option2 only support TDMA or FDMA mode, TBS of Msg2 can be compact designed for signalling overhead reduction.

- Multiple Msg3 with TDMA mode

- Multiple Msg3 with FDMA mode

Fig. 1 Resource allocation of multiple Msg3 corresponding to Msg2 transmission with option2

Proposal 4: For Msg2 transmission with option2, only TDMA or FDMA mode of Msg3 is supported.

Proposal 5: For Msg2 transmission with option2, support compact design on resource allocation of Msg3 for Msg2 TBS reduction.

2.1.3 Timing relations between Msg2 and Msg3

In RAN1#119 meeting, aspects related to timing relations between Msg2 and Msg3 is discussed and captured in FL summary [3] as below.

FL2/FL3 High priority proposal 5.2-4a: RAN1 identifies the timing relations for the following transmissions after multiple Msg1 triggered by the same R2D transmission triggering the random access - Option 1: device transmits the Msg3 after the last Msg2 transmission in case a PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: device transmits the Msg3 after the corresponding Msg2 transmission FL4: based in Wednesday online discussion, this proposal should be reformulated from Reader perspective. Discussion can be left to the potential normative phase.

In option 1, the device will transmit Msg3 after reception of the last Msg2. In this case, an additional bit flagging whether this Msg2 is the last or not would be needed in Msg2 indication. Moreover, the device is required to monitor the last Msg2 for final timing relation acquisition. This will bring another issue if the device did not monitor/receive the last Msg2 successfully, how does device determine when to transmit Msg3. Furthermore, monitoring of multiple Msg2 for each device will introduce large power consumption at device side. In contrast, option 2 will have relatively less power consumption by monitoring corresponding Msg2. Therefore, option 2 is preferred for timing relations between Msg2 and Msg3 from our perspective.

Proposal 6: Support device transmitting Msg3 after the corresponding Msg2 reception.

2.1.4 Small frequency offset indication for data transmission

In WID (RP-243326)[1], small frequency offset has been mentioned in RAN1 scope.

The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769.

In order for FDMA D2R transmission, D2R scheduling should contain small frequency offset indication. Since each device only use one frequency resource, the bitwidth of the indication may depend on frequency domain resource of the D2R transmission. For example, similar to R2D transmission with upto 4 PRBs, the indication can be confined within 3 or 4 PRBs, and the bitwidth of the indication should be applicable with scheduled D2R frequency PRB size.

Proposal 7: D2R scheduling information should contain a field to indicate small frequency offset.

Proposal 8: The bitwidth of the field of small frequency offset depends on the range of frequency resource which the D2R transmission could occupy.

To avoid interference among different devices and from carrier wave(CW), some frequency gap should be pre-set. For example, to avoid interference between two devices, their SFS should have a gap of 15kHz or 30kHz. For device 1, strong single at the frequency resource of CW is expected, and neighboring SFS should be at least 15kHz from the frequency resource of CW. Both gaps should be carefully designed.

Proposal 9: A minimum gap between two SFS is required, and a minimum gap between a SFS and CW is required.

2.1.5 Small frequency offset indication for Msg1/Msg3

In TR 38.769[2], the frequency domain resource indication is discussed at part of scheduling information.

- 6.1.2.8 Scheduling of D2R For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions - Midamble (if supported) related information

Since D2R transmission support FDMA, the scheduling information should contain frequency domain resource, i.e., indication of small frequency shift offsets. However, for Msg1 scheduling, the number of devices is unknown to the reader, and the indication of small frequency offset is shared by all devices inventoried. Hence, for the frequency domain resource of Msg1, the indication should be related to the maximum frequency resource which could be used by device(s), and device(s) can randomly select frequency domain resource from the maximum frequency resource. Similarly, the small frequency offset indication for Msg3 should also be the maximum frequency resource which could be used by device(s).

Proposal 10: For Msg1 or Msg3 scheduling, a common small frequency offset indicates the maximum frequency resource which could be used by device(s).

2.1.6 Midamble indication

In RAN1#119[4], midamble related information is agreed as follows:

Agreement For D2R scheduling, midamble (if supported) related information can be explicitly/implicitly indicated via corresponding PRDCH.

In previous discussion, plenty of companies has mentioned the midamble can be benefit for medium or large payload size[2]. These results are reasonable, since the timing error is accumulated along with transmission time. The transmission time increases with payload size, and the accumulated timing error can be expected as the transmission time multiplying SFO. Therefore, the timing error of receiving PDRCH would be dependent on payload size, repetition, MCS and SFO. When the timing error increases to a certain degree, for example half a chip, it may affect the detection result, and at least one midamble would be needed.

Observation 1: For PDRCH, the timing error would depend on transmission time, which is derived based on payload size, repetition number, MCS, and SFO.

As in our companion contribution [5], with regard to multiple purposes/functionalities of D2R midamble, multiple midamble sequences design would be beneficial. In this case, the midamble sequence indication is needed for D2R scheduling.

Observation 2: D2R midamble sequence could be the D2R midamble related information if multiple D2R midamble sequences are supported.

Consider that in some cases, the predefined fixed-length midamble sequence may not be sufficient to combat the channel conditions. For example, assuming there is a well-designed midamble sequence as midamble#1 with length x. However, in some real transmission cases with worse channel condition, a longer midamble sequence midamble#2 with length y is better to support efficient PDRCH transmission. With a predefined rule by repeating the first/last (y-x) length of midamble#1 to the end of midamble#1, the sequence of midamble#2 can be derived from midamble#1. Similarly, for some transmission cases with better channel condition, a shorter midamble sequence midamble#3 with length z would be preferred for efficient PDRCH transmission. With a predefined rule by shortening the first/last (x-z) length of midamble#1, the sequence of midamble#3 can be generated.

Observation 3: Length of midamble sequence could be the D2R midamble related information.

Observation 4: Variable-length D2R midamble sequence design is of benefit to support flexible PDRCH scheduling.

As captured in TR 38.769 [2], companies provide simulations on the performance (e.g., BLER, D2R timing accuracy) of PDRCH reception with regard to different number of D2R midambles. Generally speaking, more midambles will contribute to better performance in some cases but with larger signalling overhead accordingly. As for the cases where multiple midambles are needed for one PDRCH transmission, the location of each midamble should also be indicated explicitly or implicitly by reader. Therefore, the location of D2R midamble would be another D2R midamble related information.

Observation 5: Location of D2R midamble could be the D2R midamble related information.

Proposal 11: Midamble related information includes payload size, repetition number, MCS and SFO, midamble sequence, midamble length and midamble location.

To avoid detection error, the determination of midamble can be made by either a threshold on transmission time or a threshold on expected timing error. For one way, a threshold could be made on the chip/symbol for PDRCH conveying information bits. When the amount of chips exceeds a threshold, a midamble should be added. For another way, a threshold could be set as half of a chip, and when the expected accumulated error reaches the threshold, a midamble should be added. By either way, a midamble can be implicitly indicated.

In TR 38.769 [2], potential indication of repetitions for D2R scheduling is agreed and captured. With repetitions indication in D2R scheduling, another potential option for midamble location indication could be the boundary of each repetition.

Proposal 12: Midamble can be implicitly indicated by one of the following options:

- Option 1: pre-defined chip number/TBS threshold;

- Option 2: pre-defined timing error threshold;

- Option 3: the boundary of each repetition when repetition indicated;

2.2	Timing relationship

In TR 38.769[2], the timing relationship is discussed as below.

- 6.1.3 Timing relationships A-IoT processing time aspects are studied in terms of the following timing relationships: TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device.

For Msg2 monitoring, the following options are captured in TR 38.769[2].

- 6.1.4 Random access For a device to determine the reference time for the starting time for monitoring Msg2 in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: Option 1: End of time domain resource where the device transmitted the Msg1 Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s) Option 3: End of the R2D transmission triggering random access Option 4: End of a R2D transmission carrying Msg2

In our understanding, Option 1 and Option 2 can be regarded as one general option with X value from 1.

In RAN1#116bis[6], the following agreement regarding to R2D transmission was given.

Agreement For R2D transmission, if OFDM-based waveform is used, the start of R2D transmission from reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation.

Hence, when determining TD2R_max for monitoring Msg2, the OFDM symbol duration should be considered. For example,  TD2R_max should be at least not shorter than one OFDM symbol duration.

For Msg2 monitoring, if Option 1 is adopted, [TD2R_min, TD2R_max] should be determined at least base on a reference time of processing, e.g., TD2R and maximum sampling frequency offset, and TD2R_max further need be prolonged to at least one OFDM symbol duration.

For Msg2 monitoring, if Option 3 is adopted, [TD2R_min, TD2R_max] should be determined at least base on a reference time TD2R, a reference time TR2D, transmission duration of Msg1 with or without repetition, and maximum sampling frequency offset. Also, TD2R_max further need be prolonged to at least one OFDM symbol duration. Furthermore, if X time resources are scheduled, [TD2R_min, TD2R_max] need to further derived based on X.

Proposal 13: For Msg2 monitoring, [TD2R_min, TD2R_max] need to be determined at least base on a reference time of processing TD2R, maximum sampling frequency offset, one OFDM symbol duration.

- If Option 3 is adopted, transmission duration of Msg1 and repetition number are needed for determination;

- If Option 3 is adopted and X time resources are scheduled for Msg1, X should be considered for determination.

For power saving, if no R2D transmission is detected during [TD2R_min, TD2R_max], the device should turn to inactive mode, and wait for next round of scheduling or inventory. However, the detection of R2D transmission depends on the detection of start-indicator part of preamble, and it takes a few chips to detect. Therefore, either those chips are included into TD2R_max, or the device should further take a few chips after TD2R_max to determine whether R2D transmission exists and decide whether to turn to inactive mode.

Proposal 14: The device may take a few chips beyond TD2R_max to determine whether R2D transmission exists.

2.3	TB size determination

In TS 38.214[7], the procedure of TB size determination of PDSCH is captured as follows:

- 5.1.3.2 Transport block size determination 1) The UE shall first determine the number of REs (NRE) NRE)within the slot. - A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1, 1_2 or 1_3 or as described for format 1_0 in Clause 5.1.6.2, and …… 2) Unquantized intermediate variable (Ninfo) TBStemp) is obtained by TBStemp= NRE*R*Qm*ʋ. If Use step 3 as the next step of the TBS determination else Use step 4 as the next step of the TBS determination end if 3) When , TBS is determined as follows - quantized intermediate number of information bits , where . - use Table 5.1.3.2-1 find the closest TBS that is not less than .

Due to limited time, the procedure of NR should be captured as much as possible to obtain the TB size of PRDCH and PDRCH. Unlike PDSCH, which is scheduled with slot structure, the basic time unit of PRDCH or PDRCH transmission is chip/symbol, and for single tone waveform, only one carrier or frequency resource could be used by each PRDCH or PDRCH. Hence, the determination of TB size would depend on the amount of chip.

For a PRDCH or PDRCH with the amount of chips as Nchip, the chip number of functionality other than conveying the information bit should be exclude as Nchip-Namble, and then use code rate of channel code/line code to obtain information bit Ninfo. For example, assuming PDRCH with convolutional code rate Rchannel=1/3 and Manchester code Rline=1/2, Ninfo=(Nchip-Namble)×Rchannel×Rline. And then some quantization can be performed on Ninfo to obtain the TB size.

Proposal 15: TB size determination should be based on chips to convey information bits.

- The chips to convey information bits could be obtained by excluding chips of ambles;

- If convolution code is used, its code rate should be used to derive the TB size;

- If channel code is used, its code rate should be used to derive the TB size.

2.4	Signalling for control information

Both higher-layer signaling and L1 signaling for control information indication were discussed, wherein higher-layer indication saves overhead introduced by dedicated control part, and L1 signaling provides more time for device to process the receiving data, or provides a possibility for device to terminate receiving/processing in time to save power if no control information is decoded successfully. However, by considering the kinds of services, kinds of device with different capability, a fixed, single, common signaling for all the devices, all the services isn’t efficient. Then a flexible indication signaling is preferred.

The indication signaling may be determined by the TBS of the payload of data part, for example, when the TBS is less than a value, then the control information indicated by the higher-layer signaling is more suitable with limited time delay for decoding. By considering different device have different buffering capability, the control information, the signaling for control information indicating should be different. For example, if a device can’t buffer the data part whose length is larger than a value, then control information indicated by L1 signaling is necessary.

Proposal 16: Signaling for control information indication should be determined according to the service, capability of the device.

3	Conclusion

In this contribution, we give our views on control and other aspects of ambient IoT. We propose that:

Observation 1: For PDRCH, the timing error would depend on transmission time, which is derived based on payload size, repetition number, MCS, and SFO.

Observation 2: D2R midamble sequence could be the D2R midamble related information if multiple D2R midamble sequences are supported.

Observation 3: Length of midamble sequence could be the D2R midamble related information.

Observation 4: Variable-length D2R midamble sequence design is of benefit to support flexible PDRCH scheduling.

Observation 5: Location of D2R midamble could be the D2R midamble related information.

Proposal 1: Two options are discussed to indicate the time domain resource.

- Option 1: granularity of one chip, M chip or groups of chips;

- Option 2: TB size for device to calculate the time domain resource.

Proposal 2: The time domain resource indication is reused to indicate X time domain resources for Msg1 scheduling.

Proposal 3: For Msg2 transmission with option 1, following two options can be considered for resource allocation of Msg3.

- Option 1: Multiple indices of resource block indication.

- Option 2: Extension coefficient indication.

Proposal 4: For Msg2 transmission with option2, only TDMA or FDMA mode of Msg3 is supported.

Proposal 5: For Msg2 transmission with option2, support compact design on resource allocation of Msg3 for Msg2 TBS reduction.

Proposal 6: Support device transmitting Msg3 after the corresponding Msg2 reception.

Proposal 7: D2R scheduling information should contain a field to indicate small frequency offset.

Proposal 8: The bitwidth of the field of small frequency offset depends on the range of frequency resource which the D2R transmission could occupy.

Proposal 9: A minimum gap between two SFS is required, and a minimum gap between a SFS and CW is required.

Proposal 10: For Msg1 or Msg3 scheduling, a common small frequency offset indicates the maximum frequency resource which could be used by device(s).

Proposal 11: Midamble related information includes payload size, repetition number, MCS and SFO, midamble sequence, midamble length and midamble location.

Proposal 12: Midamble can be implicitly indicated by one of the following options:

- Option 1: pre-defined chip number/TBS threshold;

- Option 2: pre-defined timing error threshold;

- Option 3: the boundary of repetition when repetition indicated;

Proposal 13: For Msg2 monitoring, [TD2R_min, TD2R_max] need to be determined at least base on a reference time of processing TD2R, maximum sampling frequency offset, one OFDM symbol duration.

- If Option 3 is adopted, transmission duration of Msg1 and repetition number are needed for determination;

- If Option 3 is adopted and X time resources are scheduled for Msg1, X should be considered for determination.

Proposal 14: The device may take a few chips beyond TD2R_max to determine whether R2D transmission exists.

Proposal 15: TB size determination should be based on chips to convey information bits.

- The chips to convey information bits could be obtained by excluding chips of ambles;

- If convolution code is used, its code rate should be used to derive the TB size;

- If channel code is used, its code rate should be used to derive the TB size.

Proposal 16: Signaling for control information indication should be determined according to the service, capability of the device.

References

[1]	RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106, December 9-12, 2024.

[2]	3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things)

[3]	R1-2410866, Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT. RAN1#119.

[4]	Report of 3GPP TSG RAN WG1 #119

[5]	R1-2500625, Discussion on timing acquisition and synchronization. RAN1#120.

[6]	Report of 3GPP TSG RAN WG1 #116bis

[7]	3GPP TS 38.214 v18.4.0, Physical layer procedures for data.