3GPP TSG-RAN WG1 Meeting #120	R1-2500615

Athens, Greece, February 17-21, 2025

Agenda Item:	9.14

Source:	Ericsson

Title:	On TEI-19

Document for:	Discussion

# Introduction

This document provides our views on the TEI proposals on counting of CSI-RS resources. Note that we have decided not to resubmit the Ericsson TEI-19 proposal on link adaptation.

# 2 TEI discussion on Counting of active CSI-RS resources

In RAN1#119, the issue of active resource counting with a proposal to introduce an enhancement as a TEI item in Rel-19.  In the second round of discussion (see TEI proposal #5’ of R1-2410891), some additional restrictions were raised.  We provide some views on a few of these restrictions below:

- Restriction of NZP CSI-RS (CMR) and CSI-IM (IMR) has the same periodicity: typically, in order for the UE to compute CSI, the needs to perform both channel measurements on the CMR and interference measurements on IMR.  In NR, the periodicity and the slot offset of CMR and IMR are independently configurable as part of NZP-CSI-RS IE and CSI-IM IE, respectively.  However, such flexibility may not be necessary, we are open to consider introducing an assumption that both CMR and IMR have the same periodicity.

- Regarding slot offset of NZP CSI-RS (CMR) and CSI-IM (IMR): in RAN1#119’, it was proposed by some companies to introduce a threshold duration which is the maximum offset between a paired CMR and IMR, where the threshold duration is subject to UE capability.  In our view, we don’t see a need to define such a threshold duration and introduce additional UE capabilities for this.  It should be possible to configure either the same offset or a slot offset that differs by a maximum value.  Hence, we don’t support the introduction of a threshold duration on slot offset between the paired CMR and IMR.

- On whether to limit the new active resource counting feature to only non-CRI based CSI reporting: in the case with CRI based reporting, the pairs of CMR and IMR corresponding to different CRI values may have different periodicities and/or slot offsets.  As such, introducing active resource counting durations (as proposed in this TEI) may be come complicated as multiple active resource counting durations may need to be defined for the different pairs of CMR and IMR.  Hence, our preference is to limit this feature to only non-CRI based CSI reporting.