3GPP TSG RAN WG1 Meeting #120		R1-2500643

Athens, Greece, February 17th – 21st 2025

Agenda Item: 	9.1.2

Source: 	Sony

Title: 	Support for AI/ML for positioning accuracy enhancement

Document for: 	Discussion & Decision

# Introduction

In RANP#105 meeting, a Rel-19 work item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was revised [1]. However, there is no impact to the objective on positioning accuracy enhancements. The work item is basically based on the study item outcome as reported in TR 38.843 [2]. One of the objectives is to provide specification support for the AI/ML use-case for positioning accuracy enhancements. The scope is described as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

The following agreements were made in RAN1#119 meeting [3]

Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion. For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements

This contribution provides our views on various aspects to support AI/ML for positioning accuracy enhancements, including data collections, model monitoring and updates, and positioning measurement enhancements by AI/ML.

# Discussion

The AI/ML framework for NR air interface has been developed during the study item phase [2]. The purpose is to facilitate the AI/ML operation for NR air interface in various use-cases, such as beam management, CSI feedback enhancements, and positioning. The aim of the framework is to cover a general architecture addressing the whole AI model life cycle, Life Cycle Management, (LCM), including data collection, model training, model inference, management, etc. The connection between different function blocks in LCM is illustrated Figure 1 taken from [2].

Figure 1: Functional framework for AI/ML for NR [2]

Data Collection

Data collection is a fundamental step in development and training of AI/ML model. Data collection is a process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics, and inference. It gathers relevant information and observations that are used to train, validate, and test the AI/ML model and its algorithms. In the context of AI-based positioning, the LMF can be responsible for data collection. For example, to develop an AI/ML model for positioning purpose, the LMF needs to collect data from UE and/or gNB. These data can be raw measurements taken from the radio reference signal, or some features extracted from the raw measurements.

Additionally, it is essential to label the collected data. The data labeling ensures that each piece of data is associated with the label itself, such as the ground truth location of the UE. This linkage between information data and ground truth location is the key in training AI models sufficiently and in improving positioning estimation accuracy.

Data collection relies on cooperation between information generation entity (e.g., UE, gNB) and the server/management functions (e.g., LMF). During the whole process, the information generation entity (UE, gNB) provides positioning measurements and other data needed in model training. Subsequently. it reports the data to LMF. LMF collects the information data and proceed with data analysis, post-processing and model training.

Unlike the legacy positioning scheme, AI/ML positioning may require massive data collection, which requires UE or gNB to report Channel Impulse Response (CIR) measurements to the LMF. CIR measurements contain rich information, such as the amplitude, phase, and delay which reflects the channel conditions between the UE and gNB. This information can be exploited for AI/ML model training and/or inference. Beyond that, the LMF needs to also be aware of the ground truth label associated with each CIR measurement report. The CIR measurements and the labels normally are generated from the UE, PRU or gNB depending on the signaling direction (DL or UL). These information needs to be reported over the air to the LMF.  While as of today, there is no mechanism of reporting it.

Proposal 1: For data collection, support radio channel characteristics reporting in a form of channel impulse response (CIR) (e.g., power, time, and phase information) for AI/ML positioning.

The data collection operation differs depending on the positioning techniques and/or the AI/ML positioning cases. In DL -based operation, gNB transmits DL-PRS to UE. UE measures the received DL-PRS and estimates the Channel Impulse Response (CIR). Then it reports CIR measurements and the ground truth label to the LMF. It should be noted that in the DL case, CIR measurement and GT label are reported from the same entity. In UL-based operation, gNB measures SL-SRS signal and reports CIR to LMF. The GT label is reported by the UE. The illustration of the DL and UL operation is shown in the following figures.

Figure 2: Illustration of data collection for AI-based positioning in DL (left) and in UL (right)

Reporting raw CIR measurement requires heavy signaling overhead. CIR measurement is normally implemented by UE in DL positioning or gNB in UL positioning. One raw CIR measurement should at least cover the wireless channel of interest, with duration dependent on the deployment scenario (outdoor or indoor) and the environment’s delay spread. The report size may depend on the measurement samples per UE-gNB link. It can be impractical to report a large data packet every positioning occasion. Thus, it becomes important to consider report size reduction without compromising the positioning accuracy.

Proposal 2: Support configurable CIR measurement report (e.g., report size, measurement window size) in an effort to reduce the signalling overhead.

The labelling aspect as described earlier is commonly used for AI/ML supervised model training. The concept is to learn the relationship between the model input and the expected model output using an AI/ML. In the case of model training for positioning, LMF requires model input (e.g., CIR measurement data) and the corresponding labels (as expected AI model output). Thus, besides the model input, UE also needs to convey the label and the association between CIR measurement and the corresponding label. In RAN1#116bis, it was agreed that the training data collection for AI/ML positioning, the collected data sample can include two parts, namely: part A and part B [3]. Furthermore, the association of part A and part B can be discussed further [3]. We consider the association between part A and part B can be based on the time information or location information. The time information can utilize the time-stamp information.

Proposal 3: The association of part A and part B in the collected data sample (for training data collection of AI/ML based positioning) can be based on timing information (e.g., time stamp) or location information.

Two cases of AI/ML assisted positioning with different priorities have been listed as part of the normative work of AI/ML positioning. The first priority is the case of NG-RAN node assisted positioning with gNB-side model (Case 3a) and as second priority is the case of UE-assisted/LMF-based positioning with UE-side model (Case 2a).

Data collection is used for the model training so that the AI/ML model can be produced. The AI/ML model can be created in a network node, such as AI/ML data analytic function or LMF and then later to be used at the UE or gNB depending on the AI/ML cases. For simplicity, we can consider LMF as the network node which performs the model training. We expect the LMF as a network node to have better computational power and better power capacity/source than the UE. Furthermore, LMF may receive information or data collection from many nodes (UEs, gNBs, etc) as the input to create, train, re-train the AI/ML model. Once the AI/ML model has been created/ updated, the AI/ML model can be provided to the UE for AI/ML UE-side model or gNB for AI/ML gNB-side model. Subsequently, the UE or gNB performs AI/ML model inference.

The AI/ML model being transferred shall have a predefined structure and the model parameters. A complex structure can be provided in the case when high positioning accuracy is required and also for the case when the UE has high computational power capability. A specific model structure may be required for different types of AI/ML positioning modes (e.g., assisted or direct) and for different positioning techniques. Further study is required to define the detailed model structure and parameters for model transfer. AI/ML model structure and parameters can consider at least the following aspects:

- The inference model can be partially or fully provided.

- The time validity of the inference model.

- The structure (i.e., number and types of layers, interconnections, etc.) of the predictive neural network as part of the inference model.

- The measurements that the UE should perform associated with the provided inference model.

Proposal 4: Support model transfer operation from network node (e.g., LMF) to UE/gNB to support AI/ML-assisted positioning.

Model Input

RAN1 has made some progress on AI/ML model input aspect.. Further refinement, such as the agreements on the definition of sample-based and path-based, were agreed.. Subsequently it was discussed particularly on identifying the definition and the details of each approach. The model input can be in a form of the obtained time domain channel measurements. Two alternatives were identified, such as the sample-based and path-based measurements.

We consider sample-based measurement has some advantages for the AI/ML model input over the path-based measurements. The sample-based measurement is information rich. This would provide more freedom in utilizing the model input and it can lead to improvement of the positioning estimation accuracy. Furthermore, we consider that the sample-based measurement could still be further processed to generate the timing information for each path via post-processing at the LMF side.

One can argue that sample-based measurement may have larger signaling overhead (i.e., measurement size) than the path-based measurement. However, the richness of the information carried by the sampled-based measurements should be the main consideration. Furthermore, the reporting size of sampled-based measurement can be configured depending on the need of the model training. For example, the window size of the sampled-based measurement can be adjusted.

Depending on the positioning techniques, the time-domain channel measurement is expected to be generated by the UE/gNB. For example, for downlink-based positioning, the information is provided by the UE and for uplink-based positioning the measurement information is provided by the gNB. In terms of implementation, the legacy UE is in principle capable of performing the sample-based computation for positioning measurements. We expect this has no significant implementation changes in the legacy UE in order to support sample-based measurement.

In RAN1#119 meeting, there have been some progress that the sample based measurement has been supported for AI/ML case 3b. In our view, the same approach can also be applied to the other similar case, i.e., Case 2b.

Proposal 5: Support sample-based time domain channel measurements as the AI/ML model input for Case 2b.

Model Output

For AI/ML assisted positioning case 2a and case 3a, positioning measurements (e.g., timing, power) and LOS/NLOS indicator is supported for reporting. For LOS/NLOS indicator reporting, it is also agreed that the indicator can be reported as soft indicator or hard indicator as defined in TS 38.214. It means the LOS/NLOS indicator reporting format is reusing the legacy NR positioning. For these two cases, it would be beneficial for the LMF to be aware whether the reported LOS/NLOS indicator is based on the legacy measurement or based on AI/ML computation. The UE or gNB provides extra efforts in producing LOS/NLOS indicator with AI/ML computation. The LMF can expect the LOS/NLOS indicator with AI/ML computation can be more accurate in comparison to the legacy NR positioning. We propose that the report is also accompanied with an indication whether the LOS/NLOS indicator is generated based on AI/ML or not. It is not guaranteed that the AI/ML computation will always improve the accuracy. At least, the LMF should be aware that the reported LOS/NLOS indicator is computed differently than the legacy NR positioning (i.e., with AI/ML computation). Subsequently, the LMF can trust LOS/NLOS indicator obtained from AI/ML computation than the LOS/NLOS indicator from the legacy NR positioning.

Proposal 6: For AI/ML assisted positioning case 2a and case 3a, support an indication whether the positioning measurement is based on AI/ML computation or not.

Model Inference Operation

For case 1, case 2a, and case 3a, the AI/ML model (i.e., AI/ML model inference) is performed in the radio network node, such as at the UE or gNB. The UE/gNB uses a trained AI/ML model to produce a set of outputs based on a set of inputs. In the context of AI/ML positioning, the model input can be the positioning measurement, such as positioning measurement (e.g., the legacy NR positioning measurements based on Time Different of Arrival (TDOA), signal / power strength measurement, phase measurement (Angle of Arrival (AoA), Angle of Departure (AOD)). These positioning measurements are calculated based on the received reference signals, such as DL-PRS. The training / generation of AI/ML can be performed at the radio node itself or in another node. The trained AI/ML model is created based on some measurements, in which the measurement is also based on the received reference signal, including DL-PRS. The model output of AI/ML model inference can be the positioning estimation of the UE (This is known as direct AI/ML positioning) or an improved positioning measurement results or a new set of measurements (an accurate TDOA measurements, improve LOS/NLOS identification, etc). This operation is illustrated in Figure 3.

Figure 3: AI/ML model Inference operation

Ideally, the inputs to the training data collection and the inputs to be used for AI/ML model inference are using the same set of parameters and/or the same characteristics. For example, the inputs of these two operations are based on the received reference signals (e.g., DL-PRS) with the same characteristics, such as the same transmit power, the same bandwidth, the same TRPs, the same signal quality, etc. However, the radio channel propagation can be changed over time. Furthermore, the UE may also move from one location to another. Performing AI/ML model inference with the data collection input based on DL-PRS that has different characteristics than the DL-PRS characteristics for the AI/ML model generation may result in the degradation on the AI/ML model output. We consider it is important to maintain consistency between AI/ML training and AI/ML inference. This can be facilitated by identifying the reference signal characteristics (e.g., DL-PRS) and associate to the corresponding AI/ML process (e.g., AI/ML training or AI/ML inference). The reference signal characteristics can be represented by a set of parameters of the reference signals.

Proposal 7: Define a set of parameters (e.g., part/all of DL-PRS configuration, received signal quality, etc) representing reference signal characteristics to be used for AI/ML positioning.

Proposal 8: For ensuring the AI/ML model consistency between the training and inference operation, support the association of the reference signal (e.g., DL-PRS) characteristics to the trained AI/ML model and AI/ML model inference operation.

The association of the reference signal (e.g., DL-PRS) characteristics to the trained AI/ML model and the association of the reference signal characteristics to the AI/ML model inference operation could facilitate the node performing A/ML model to evaluate / compare whether the trained AI/ML model (to be used for AI/ML model inference) and AI/ML model inference are based on the same reference signal characteristics or not. The AI/ML model inference operation with a consistent reference signal characteristics could lead an accurate AI/ML model output.

Proposal 9: Further study the signalling procedure to ensure the consistency between AI/ML training and AI/ML inference.

The assistance data provided from LMF to UE for AIML Positoning Case 1 was discussed in RAN1#119. The assistance data is to be used for ensuring consistency between training and inference.

The discussion points and agreements made in RAN1#119 are related to the assistance data  AD-IE-Group1 (i.e., assistance data needed to support reception of the DL PRS). Furthermore, for ensuring consistency purpose, AD-IE-Group2 (information related to NW-side additional condition) shall also be provided to the UE. For example, this could be related to the Tx power when the gNB transmits DL-PRS.

Proposal 10: For case 1, support LMF to provide the UE with AD-IE-Group2 (NW-side additional condition).

Model Performance Monitoring & Update

It can be observed in Figure 1 that the Model Management is the core of LCM, connecting to all the other components. The objective of model management function is to automatically monitor the model performance or the model validity at different model deployment entities (UEs or gNBs). Consequently, it is also tasked to constantly deliver the up-to-date models, to guarantee the best model performance. In our view, Model Management function mainly triggers two procedures: Model Switching and Model Updating.

In many cases, AI/ML model may only be valid or can achieve good performance in some designated areas or under certain conditions. However, these areas or conditions may be subject to environment change or UE behavior change. When there are some changes but UE/gNB does not adapt to an up-to-date model, the AI/ML positioning accuracy could be comprised. For instance, consider a scenario in a production line where the configuration/constellation of machines might change. This rearrangement of the environment can impact channel conditions and propagation characteristics, such as LOS.  Continuously employing an outdated model in the UE/gNB may result in compromised positioning performance.

Therefore, it is essential to implement updating mechanism so that accurate positioning estimation can still be guaranteed. It is worth noting that based on simulation carried out in the SI, an inaccurate / obsolete AI/ML model could lead to positioning estimates even worse than the calculation without AI/ML (i.e., legacy NR positioning).

As for the AI/ML positioning, the UE or gNB receives the AI/ML model from a server (e.g., LMF or dedicated AI/ML server). The AI/ML model is used to produce a set of outputs based on a set of inputs, so called AI/ML inference. AI/ML model inference is performed by UE or gNB depending on the AI/ML cases, for example:

-	Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

-	Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

-	Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

In certain conditions/scenarios, the obtained AI/ML model requires some updates. However, the AI/ML model update mechanisms involve various nodes, and various positioning techniques. A solution is that the UE or gNB which performs AI/ML model inference can determine the validity of AI/ML model and can trigger model updating. For Case 1, this option is known as Option A (The target UE performs monitoring metric calculation). Several sub-options were identified, such as:

- Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE.

  - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label.

- Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE.

- Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE.

- Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server).

In RAN1#118 meeting, it was concluded that Option A-4 can be realized via implementation.

In above options, some information is provided to the UE. Apart from that, we consider the operation at UE shall also be controlled / managed by the network. The details of the performance monitoring operation can be up to UE implementation, however the performance monitoring output shall be reliable.. In order to achieve this, LMF can provide criteria/conditions which comprises model updating criteria/condition(s) to the UE or gNB, in assisting UE/gNB to assess whether the deployed model fits the current environment such as whether the model is up-to-date. Furthermore, there should also be a possibility where the LMF can provide the reference measurements as input for the performance monitoring and the expected output. Then, the UE can make evaluate, for example, the deviation between the estimated output by processing the reference measurements (input) and the expected output. The UE can report the deviation to LMF. Then, LMF can make further action. Based on this, we consider option A-1, A-2, and A3 can be supported. A-3 is the special case when the PRU data is available (PRU positioning measurement and PRU location info) and relevant to the target UE. It is up to LMF to make the association between the target UE and the PRU.

Proposal 11: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-1, A-2, A-3 are supported in Rel-19.

For case 3a, there was some progress that the following options were feasible, at least from RAN1 perspective [3].

Option A. NG-RAN node performs monitoring metric calculation for its own model.

Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node.

The detailed of the above may be discussed within RAN3. However, the concept of the operation can still be discussed within RAN1. For the option B above, once the LMF has performed the monitoring for the model located at the NG-RAN node and, for example, identify that the applied model is no longer feasible, the LMF provides an indication to the NG-RAN node.

Proposal 12: For case 3a and with option B, support LMF to provide an indication of AI/ML model validity (e.g., based on the monitoring outcome) to NG-RAN node.

For case 1 and case 3a with option A, The UE or NG-RAN node (e.g., gNB) performs model monitoring, including computation and measurement to assess the validity of the AI/ML model. Once the assessment is done, the UE or gNB can send an indication to the AI/ML server/management (e.g., LMF) to trigger retraining of the given AI/ML model. In such case, the current protocol, such as LPP and NRPPa, can carry the indication to LMF and gNB, respectively.

Proposal 13: For case 1 and case 3a with option A, UE or gNB to provide an indication of AI/ML model validity (e.g., based on the monitoring outcome)  to the AI/ML server/management (e.g., LMF).

# Conclusion

In this contribution, we provide our views various aspects to support AI/ML for positioning accuracy enhancements. The proposals are listed below.

Proposal 1: For data collection, support radio channel characteristics reporting in a form of channel impulse response (CIR) (e.g., power, time, and phase information) for AI/ML positioning.

Proposal 2: Support configurable CIR measurement report (e.g., report size, measurement window size) in an effort to reduce the signalling overhead.

Proposal 3: The association of part A and part B in the collected data sample (for training data collection of AI/ML based positioning) can be based on timing information (e.g., time stamp) or location information.

Proposal 4: Support model transfer operation from network node (e.g., LMF) to UE/gNB to support AI/ML-assisted positioning.

Proposal 5: Support sample-based time domain channel measurements as the AI/ML model input for Case 2b.

Proposal 6: For AI/ML assisted positioning case 2a and case 3a, support an indication whether the positioning measurement is based on AI/ML computation or not.

Proposal 7: Define a set of parameters (e.g., part/all of DL-PRS configuration, received signal quality, etc) representing reference signal characteristics to be used for AI/ML positioning.

Proposal 8: For ensuring the AI/ML model consistency between the training and inference operation, support the association of the reference signal (e.g., DL-PRS) characteristics to the trained AI/ML model and AI/ML model inference operation.

Proposal 9: Further study the signalling procedure to ensure the consistency between AI/ML training and AI/ML inference.

Proposal 10: For case 1, support LMF to provide the UE with AD-IE-Group2 (NW-side additional condition).

Proposal 11: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-1, A-2, A-3 are supported in Rel-19.

Proposal 12: For case 3a and with option B, support LMF to provide an indication of AI/ML model validity (e.g., based on the monitoring outcome) to NG-RAN node.

Proposal 13: For case 1 and case 3a with option A, UE or gNB to provide an indication of AI/ML model validity (e.g., based on the monitoring outcome)  to the AI/ML server/management (e.g., LMF).

# References

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RANP #105, September 2024

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” V18.0.0

- Chairman’s notes RAN1#119, Orlando, November 2024.

- R1-2410921, “Final Summary AI/ML Positioning”, Ericsson, Orlando, November 2024.