3GPP TSG RAN WG1 #120									R1-2500652

Athens, Greece.  17-21 February 2025

Agenda Item 	:	9.4.3

Source 	:	Sony

Title 	:	Timing acquisition and synchronisation for Ambient IoT

Document for 	:	Discussion

# Introduction

In RANP#106 Madrid, a work item [1] on Ambient IoT was agreed, including the following RAN1-related objectives:

This WI is to standardize RAN aspects of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The work shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope: The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this document, we consider the following aspects of Ambient IoT:

- R-TAS

- D-TAS

- D2R Segmentation

# R2D timing and clock acquisition

In TR 38.769 [2] section 6.1.1.6.1 the R2D timing acquisition signal (R-TAS) is described. The R-TAS is immediately preceding the transmission of PRDCH and is included at least for timing acquisition and indicating the start of R2D transmission in the time domain. It is studied to have an R-TAS with two parts, a start-indicator preceding a clock acquisition part.

For the start indicator part two options have been studied with observations presented in TR 38.769 [2]

- Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration - Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation

And for the clock acquisition part (CAP) two options have been studied with the following observations, presented in TR 38.769 [2].

The clock-acquisition part is based on OOK without line coding, and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration. The following options for design of the clock-acquisition part are studied: - Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M. - Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant

We would like to point out that while the R-TAS needs to both indicate the start of the transmission and provide synchronization, it is not necessary to assign two separate parts for either of the purposes. By selecting option 2, i.e., a preamble from sequences with good correlation properties such as m-sequences or PN sequences, the detected peak can be used both for synchronization and indication of the start of the transmission.

Furthermore, by suitable design of the R2D signals and its reception technique, we can avoid the need for instance to determine OOK rising/falling threshold setting. This can easily be done using Manchester coded OOK for both preamble control/data transmission where an energy comparator, with zero threshold is used to detect the OOK bits.

Observation 1 – Selecting a preamble from sequences with good auto- and cross-correlation properties, the detected peak can be used both for synchronization and as an indicator of the start of the transmission, eliminating the need for a separate start-indicator in the R-TAS.

- D2R timing and clock acquisition

In TR 38.769 [2] section 6.1.2.6 the D2R timing acquisition signal (D-TAS) is described. The D-TAS is preceding each PDRCH and is included at least for timing acquisition and indicating the start of D2R transmission in the time domain. The D-TAS has been studied for correlation properties, sequence type, line coding, SFO estimation, CFO estimation, channel estimation, and interference estimation.

We would like to point out that an off period can be included prior to the preamble. The OFF period may also be utilized for interference mitigation. The carrier wave emission will show up as a strong interference at the reader when decoding the D2R backscattered data. This is especially the case when there is no or a small frequency shift performed at the A-IoT device when modulating the data. An OFF period, with known length in time at the reader, will open up for interference cancellation at the reader where the path from the carrier wave emitter to reader are cancelled from the received signal. If the OFF period is configured prior to the preamble, the preamble can be made shorter resulting from interference mitigation being applied to the preamble.

Proposal 1: For purpose of interference mitigation, consider the option to use an OFF period known at the reader during the PDRCH transmission.

Observation 2: Introducing an OFF period at the beginning of the PDRCH preamble allows for shorter preamble.

# D2R segmentation

In TR 38.769 [2] section 6.1.2.6 also potential midamble and the D2R end timing is described. For the D2R end timing two options are listed as studied,

- Option 1) D2R postamble immediately follows the PDRCH,

- Option 2) based on the information in control channel.

For midamble it is indicated it is studied “for the purpose of performing timing/frequency tracking or channel estimation or interference estimation”.  A trade off analysis has been carried out where the following options have been studied as combinations of the above ambles.

- Option 1: D2R preamble only - Option 2: D2R preamble + X midamble(s), where X≥1 - Option 3: D2R preamble + postamble - Option 4: D2R preamble + Y midamble(s) + postamble, where Y≥1

Regarding device (un)availability, the objective above indicates Direction 1 from 38.379 to be further studied.

Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable.

As of the Direction 1, a device might become unavailable without any indication from the reader. The unavailability could be due to the A-IoT device running out of power or performing other tasks for which the Reader is not aware of. It might also be possible that the A-IoT device runs out of power and needs to pause its transmission. In particular a device using BPSK (in comparison to ASK) might consume more power since there is no longer absorption periods of the signal where the A-IoT device may harvest energy.

Under these circumstances we see it as beneficial to have the option to segment the data in D2R as described in section 6.3.5 of TR 38.379 [2] A-IoT Data Transmission. From TR 38.379  the TB size is under study:

- In both R2D and D2R directions, there is no lower bound on the minimum TB size, and a maximum TB size of around 1000 bits can be supported. - How large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc.

And for the message size indication the following

- Option 1: The estimate of expected following D2R message size from the CN to the reader; - Advantages/disadvantages of option 1: If this information is available from CN, it saves the overhead/latency caused by Option 2 in AS layer. It can be also used by the reader as early as for the first D2R message, if needed. - Option 2: The simple message size indication from the device to the reader. - Advantages/disadvantages of option 2: It is useful in case the CN is not able to provide or does not provide the expected D2R message size. Limited granularity is supported considering the AS layer overhead. It requires D2R message to carry this information in advance, before the reader can actually use this size information for scheduling.

In our view, the length of the data can be transmitted in the control information and therefore introduction of control information for this purpose is beneficial. However, there may be scenarios where the A-IoT device cannot fulfil the data transmission according to the data size indicated in the control information. For instance when:

  - the A-IoT device may run out of power earlier than expected and it needs to pause its transmission without any need to re-transmit the earlier part of its data.

  - the total size of data available for transmission is smaller than that scheduled by the reader and the A-IoT device does not want to perform any zero padding to the end of its data transmission.

  - the A-IoT device autonomously segments its data transmission to be able to harvest energy for transmission of each data segment.

Under these conditions, the A-IoT devices need to transmit a postamble or an end-delimiter or a midamble with a configured gap to indicate a pause or end of the data transmissions.

Proposal 2 – Control information prior to data transmission is needed to indicate the total size or length of the data transmission.

Proposal 3 – Support transmission of a postamble as an end-delimiter of data transmission and or a midamble with a gap configuration to support all transport block sizes under the possible user scenarios.

# Conclusion

This document has considered R-TAS, D-TAS and segmentation. The following proposals and observations are made:

Observation 1 – Selecting a preamble from sequences with good auto- and cross-correlation properties, the detected peak can be used both for synchronization and as an indicator of the start of the transmission, eliminating the need for a separate start-indicator in the R-TAS.

Proposal 1: For purpose of interference mitigation, consider the option to use an OFF period known at the reader during the PDRCH transmission.

Observation 2: Introducing an OFF period at the beginning of the PDRCH preamble allows for shorter preamble.

Proposal 2 – Control information prior to data transmission is needed to indicate the total size or length of the data transmission.

Proposal 3 – Support transmission of a postamble as an end-delimiter of data transmission and or a midamble with a gap configuration to support all transport block sizes under the possible user scenarios.

References

[1]	RP-243326. “Solutions for Ambient IoT (Internet of Things) in NR”. RANP#106. Madrid, Spain. December 2024.

[2]	TR 38.769 V19.0.0. “Study on solutions for Ambient IoT (Internet of Things) in NR”.