3GPP TSG RAN WG1 #120			R1-2500683

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.5.3

Source:	KT Corp.

Title:	Discussion on Adaptation of common signal/channel transmissions

Document for:	Discussion and decision

1. Introduction

During the study period, numerous techniques for achieving energy savings in NR BS were thoroughly examined [1]. It was found that reducing SSB significantly improves energy efficiency, especially in near-zero load situations. Previously, Release 18 supported SSB-less SCells for this purpose. In Release 19, a new approach is being standardized: adaptation of common signal/channel. This aims to enhance the energy efficiency not only for SCell, however, there was no consensus to support the adaptation in PCell, so this also aims SCell consequently.

Over the past year, significant progress has been made on this agenda, although only critical issues have been addressed for SSB adaptation. In this contribution, we present our views on the agenda for the final steps.

2. Adaptation of SSB

In the previous meeting [2], the following conclusion was made.

Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Since there was no consensus on supporting the adaptation of SSB at PCell, as the gain is marginal even for SSBs not on the sync raster, given the presence of always-on CD-SSB on the sync raster with a maximum 20ms period. The adaptation is only valuable when the cell intends to use SSB frequently with a period of less than 20ms.

Proposal 1: For the adaptation of CD-SSB on sync raster (B1), if it is supported, the maximum period is limited to 20ms.

Another issue for the adaptation is about the adaptation type other than period. In the previous meeting [2], the following agreement for the reply LS to RAN4 was made.

Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Since the operation for PCell is not assumed, the main role of SSB adaptation can be measurement and resulting radio link management. The pattern adaptation, e.g., changing ssb-PositionsInBurst,  can be meaningful for some cases like scenarios with the situation that some SSB indices are frequently accessed indices while oothers are rarely accessed.

Proposal 2: For adaptation of SSB in time-domain, Option 2 is supported in addition to Option 1, and the separate configuration contains at least ssb-Periodicity and ssb-PositionsInBurst.

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

We give another issue is about the measurement and collision, similar with the issues arised in the On-demand SSB SCell agenda. In the last moderator summary of the agenda [3], some proposals and questions are described as follows and still in discussion.

### 9.5.1 On-demand SSB SCell operation ### Proposal #7-1 (Measurement configuration): - For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), support at least Option 2 and further study whether to additionally support Option 1. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether/how to differentiate always-on SSB and on-demand SSB ### Proposal #7-2 (Measurement report): - For a cell supporting on-demand SSB SCell operation and for L1 measurement based on on-demand SSB, - For on-demand SSB indicated by RRC, - Periodic, semi-persistent, and aperiodic CSI report are supported. - For on-demand SSB indicated by MAC-CE, - Semi-persistent and aperiodic CSI report are supported. - Periodic CSI report is not supported. ### Proposal #7-3 (Beam failure based on on-demand SSB): Study whether/how to support beam failure based on on-demand SSB for Case #1 and/or Case #2 Q#9-1 (Collision): At least 5 companies suggested to handle the collision issue between SSB and other signals/channels (e.g., RACH occasions). Do you agree to handle this issue? Q#9-2 (Rate-matching): At least 3 companies suggested to handle the rate-matching issue when SSB is collided with DL/UL data channels. Do you agree to handle this issue?

In our view, similar issues can be discussed with SSB adaptation operation.

Proposal 3:  For a cell supporting SSB adaptation  operation, support at least Option 2 and further study whether to additionally support Option 1.

- Option 1: A CSI report configuration is associated with both of SSB after and before adaptation

- Option 2: A CSI report configuration is associated with one of SSB after and before adaptation

Proposal 4: For a cell supporting SSB adaptation operation and for L1 measurement based on adaptive SSB, further study whether periodic, semi-persistent, or aperiodic CSI report can be supported.

- If MAC-CE based triggering is supported, semi-persistent, or aperiodic CSI report is supported and periodic CSI report is not supported.

Proposal 5: Collision handling with another signal and channel is further studied at least for the region of SSB transmission that is not transmitted after adaptation, including collision with CSI-RS and PDSCH.

3. Adaptation of PRACH

In this section, we give our views for some remained issues descussed in [3]. The first is about the PRACH usage for CBRA/CFRA. Since performing timing for CFRA has ambiguity, supporting may need additional signalling.

Proposal 6: For the additional PRACH resources by adaptation, only CBRA is supported.

The second is about the triggering of PRACH adaptation. In the previious RAN1 meetings, the followings are agreed [2,3] for tha adaptation indication mechnisms. Agreements updated later are omitted.

### RAN1 #116 Agreement For the adaptation mechanisms of PRACH in time-domain - Support at least PRACH adaptation provided by gNB without UE trigger - FFS: PRACH adaptation with UE trigger - Note: UE trigger means UE requests adaptation of PRACH - Study at least the following, - Dynamic signaling and/or semi-static signaling of PRACH adaptation - Adaptation of PRACH transmission according to certain condition - Applicability to idle/inactive and/or connected mode UEs - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs) ### RAN1 #117 Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX RAN1 #118 Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format. Agreement For DCI-based adaptation for additional PRACH resources, - Select from the following DCI format(s) to carry the adaptation indication. - DCI format 1_0 - DCI format 2_7 - DCI format 2_9 - FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format RAN1 #119 Working Assumption For DCI-based adaptation for additional PRACH resources, - Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI - Option 1: Use reserved bits in the DCI format - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format - Option 2: Use Bits 5-8 within the Short Message (from upper layers) - Note: Availability should be confirmed by checking with RAN2. - Option 3: Use bits available for both Option 1 and Option 2 FFS: Payload size for adaptation for additional PRACH resources Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details Agreement Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used

For these issues, we see that UE triggering is beneficial but hard to be operated, since SR or MSG1-like blind detection is required. We do not see the need of semi-static signalling in addition to DCI-based triggering. So we give our proposal as follows.

Proposal 7: For triggering/indicating PRACH adaptation, only DCI-based adaption is supported.

- UE triggering is not supported

4. Conclusion

We give our proposals in this contribution as follows:

Proposal 1: For the adaptation of CD-SSB on sync raster (B1), if it is supported, the maximum period is limited to 20ms.

Proposal 2: For adaptation of SSB in time-domain, Option 2 is supported in addition to Option 1, and the separate configuration contains at least ssb-Periodicity and ssb-PositionsInBurst.

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

Proposal 3:  For a cell supporting SSB adaptation  operation, support at least Option 2 and further study whether to additionally support Option 1.

- Option 1: A CSI report configuration is associated with both of SSB after and before adaptation

- Option 2: A CSI report configuration is associated with one of SSB after and before adaptation

Proposal 4: For a cell supporting SSB adaptation operation and for L1 measurement based on adaptive SSB, further study whether periodic, semi-persistent, or aperiodic CSI report can be supported.

- If MAC-CE based triggering is supported, semi-persistent, or aperiodic CSI report is supported and periodic CSI report is not supported.

Proposal 5: Collision handling with another signal and channel is further studied at least for the region of SSB transmission that is not transmitted after adaptation, including collision with CSI-RS and PDSCH.

Proposal 6: For the additional PRACH resources by adaptation, only CBRA is supported.

Proposal 7: For triggering/indicating PRACH adaptation, only DCI-based adaption is supported.

- UE triggering is not supported

5. References

[1] 3GPP TR 38.864 V18.0.0, Study on network energy savings for NR.

[2] RAN1 #119 Chairman’s note, final.

[3] R1-2410912, Summary #5 of on-demand SSB for NES, Moderator (LG Electronics).