3GPP TSG RAN WG1 #120		  R1-2500697

Athens, Greece, February 17th – 21st, 2025

Source:	Panasonic

Title: 	Discussion on SBFD TX/RX/measurement procedures

Agenda Item:		9.3.1

Document for:	Discussion

Introduction

A work item on Evolution of NR duplex operation has been approved for Rel.19 [1]. This document provides our view on SBFD TX/RX/measurement procedures.

Discussion

SBFD subband indication

Link direction indication

In RAN1#116, the following agreement was reached on the link direction indication for SBFD-aware UE in SBFD symbol.

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded.

For Option 2, the two benefits were argued. One is collision handling, i.e., the collision between UL transmission and DL reception can be resolved by the dynamic indication or semi-static configuration of the link direction. The other is UE power saving, i.e., UE power consumption can be reduced by avoiding the frequent switching of the link direction and/or the PDCCH monitoring.

For collision handling, in RAN1#117, it was agreed that collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) is assumed as error case if link direction indication is not supported nor provided as follows. For half-duplex FDD, collision Case 3 is also assumed as an error case, and the link direction indication is not introduced, but there would be no critical issue. Thus, the link direction indication would not be essential to handle the collision.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases

For UE power saving, if needed, gNB can control the link direction by scheduling/configuring UE's transmission/reception on SBFD symbols, and can reuse Rel.17 PDCCH monitoring reduction schemes (i.e., search space set group switching, PDCCH skipping specified in TS38.213 [2] section 10.4). Therefore, the explicit indication of link direction would not be essential.

Proposal 1: For link direction for SBFD-aware UE in SBFD symbol, support Option 1.

Guard period indication

In RAN4#112, the following agreements were made on transient period length between SBFD and non-SBFD symbols  [3].

- Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1.

In RAN1, whether to explicitly indicate the guard period between SBFD and non-SBFD symbols was discussed. In legacy TDD, although F symbol can be used to absorb the transition period between transmission and reception by gNB implementation, the transition period is not limited to F symbol. Even in DL symbol or UL symbol, with proper gNB scheduling (i.e., transmission/reception are not allocated in the transition period), the transition period can be absorbed in DL or UL symbols.

Considering that the legacy TDD supports dynamic TDD and UE-specific TDD-UL-DL configuration, proper scheduling for SBFD would not be difficult compared to TDD because dynamic SBFD and UE-specific time domain location of SBFD subband were precluded in Rel.19. Therefore, even if explicit indication of guard periods is not supported, gNB implementation can avoid UL/DL transmission on the transition period as legacy.

Proposal 2: Not support explicit indication of guard periods.

Interaction between SBFD symbol and TDD-UL-DL-ConfigDedicated

In RAN1#119, the following agreement was reached on the interaction between SBFD symbols and SFI.

Agreement For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols - Note: no additional specification efforts are expected to support joint operation of SBFD and dynamic SFI.

The interaction between SBFD symbols and TDD-UL-DL-ConfigDedicated is not concluded yet. The following options were discussed in RAN1#119 [4].

- Option 1: An SBFD aware UE does not expect to be provided with TDD-UL-DL-ConfigDedicated for SBFD symbols

- Option 2: TDD-UL-DL-ConfigDedicated is not applicable to SBFD symbols. An SBFD aware UE interprets TDD-UL-DL-ConfigDedicated as legacy for non-SBFD symbols.

- Option 3: TDD-UL-DL-ConfigDedicated is applicable to both non-SBFD and SBFD symbols.

Option 1 does not work for the slot which consists of both SBFD symbols and non-SBFD symbols because TDD-UL-DL-ConfigDedicated is configured per slot basis. Option 2 is to apply the same handling with SFI in the above agreement (i.e., TDD-UL-DL-ConfigDedicated is ignored for SBFD symbol). Option 3 can be used for the link direction indication in SBFD symbols. However, as discussed above, we don't think that the link direction indication is essential. Therefore, our preference is Option 2.

Proposal 3: TDD-UL-DL-ConfigDedicated is not applicable to SBFD symbols. An SBFD aware UE interprets TDD-UL-DL-ConfigDedicated as legacy for non-SBFD symbols.

UE-specific frequency locations of SBFD subband

In RAN2#127, the following agreement was made. Whether to support UE specific dedicated RRC configuration needs to be discussed from RAN1 perspective.

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

The main motivation for supporting UE-specific frequency locations is to support UE-specific guard band. We think that the necessity of UE-specific guard band depends on UE filter implementation. If UE is not required to have specific filter corresponding to SBFD subband or UE's guard bands can be always within gNB’s signalled guardbands, UE-specific guard band would not be needed. Otherwise, UE-specific guard band would be needed. UE filter aspects should be discussed in RAN4. Therefore, we propose to send LS to RAN4.

Proposal 4: Send LS to RAN4 regarding the necessity of UE-specific guard band.

TX/RX/measurement procedures

Separate configuration for PUCCH

In RAN1#118, the following agreement were reached on separate configurations for PUCCH

Agreement Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

For UE behaviour when no separate configuration is provided for SBFD symbols (described as FFS in the agreement), the following options were discussed in RAN#119 [4].

- Option 1: PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected

- Option 2: Configurations for non-SBFD symbols are applied for SBFD symbols

- Option 3: If PUCCH transmissions partially or entirely fall outside UL usable RBs in SBFD symbols, PUCCH transmissions in SBFD symbols for this pucch-ResourceId is dropped. If PUCCH transmissions entirely fall within UL usable RBs in SBFD symbols, configurations for non-SBFD symbols are applied for SBFD symbols.

For Option 1, signaling overhead would be increased, but it would not be critical. Option 2 and Option 3 would also work and which one is more suitable as the default depends each company's assumption, which would be difficult to be converged.  As these enhancements are not essential, instead of debating option 2 or option 3, to conclude Option 1 can save the meeting time. Therefore, we support Option 1.

Proposal 5: For UE behaviour when no separate configuration is provided for SBFD symbols, PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected.

CSI reporting

In RAN1#117, the following agreement was reached on CSI report associated with periodic/semi-persistent CSI-RS.

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols.

For Option A, specification impacts can be reduced compared to Option B. However, the maximum number of CSI-ReportConfig can be an issue. The number of supported CSI-ReportConfig depends on UE capability, i.e., up to four CSI-ReportConfig can be configured for each periodic, aperiodic, and semi-persistent. If UE supports less than two CSI-ReportConfig, CSI report for SBFD and non-SBFD symbol would not work.

For Option B, the number of CSI-ReportConfig does not increase due to separate CSI reports for SBFD symbols because sub-configuration can be used. However, NES functionality cannot jointly be used with SBFD.

From our perspective, the issue of maximum number of CSI-ReportConfig is not critical. If SBFD aware UE supports two or more CSI-ReportConfigs, Option A can work. In addition, Option A makes independent of NES. Therefore, our preference is Option A.

Proposal 6: For CSI report associated with periodic/semi-persistent CSI-RS, support Option A.

TBS determination for Configuration 2

The following agreement were reached on TBS determination for PDSCH for Configuration 2. How to determine TBS for SPS PDSCH is still FFS.

Working Assumption (RAN1#118) For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only. Agreement (RAN1#118bis) - For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs. Agreement (RAN1#119) For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

For SPS PDSCH, the main purpose is that the same TBS is used and repeated like voice service. Different TBSs based on DL usable PRBs with the same coding rate are not useful for this purpose. When the link quality is not sufficient, retransmission is carried out as usual operation of SPS. One packet of 20 ms voice data is segmented to multiple PDSCHs for the reason of reduced TBS by SBFD would actually increase the latency than the same TBS with the possibility of more retransmissions because of higher coding rate.

In addition, one of the SPS usages would be XR. For XR, fractional periodicity like 16.667 ms was discussed to handle jitter. If the fractional periodicity is applied to SPS, it would be difficult to align the periodicity with SBFD slot/symbol pattern. This means that it is not deterministic which symbol type is used. Figure 1 shows an example of the relationship between the fractional periodicity of 16.6667 ms and SBFD slot configuration (DXXXU). In this sense, it would be simple that TBS determination is based on assigned PRBs.

Proposal 7: For SPS PDSCH for Configuration 2, TBS determination is based on assigned PRBs.

Figure 1 Relationship between fractional periodicity (16.6667 ms) and SBFD slot configuration (DXXXU)

DMRS bundling

Whether/how to support DMRS bundling for SBFD symbol is not concluded yet. DMRS bundling is used to improve UL channel estimation. For DMRS bundling, gNB can perform joint channel estimation in an actual TDW. Since one of the motivations of SBFD is UL coverage enhancement, to support DMRS bundling in SBFD symbols would be desirable to improve UL coverage. If DMRS bundling is supported for SBFD symbol, how to determine actual TDW needs to be clarified especially for the boundary between SBFD symbol and non-SBFD symbol. The following options can be considered.

- Option 1: Actual TDW is not across SBFD and non-SBFD symbols.

- Option 2: Actual TDW can be across SBFD and non-SBFD symbols.

For Option 1, the boundary between SBFD and non-SBFD symbols is always considered as an event. Option 1 can reduce UE complexity because UE does not needs to maintain power consistency and phase continuity between SBFD and non-SBFD symbol (e.g., if UE filter is adjusted between SBFD and non-SBFD symbols, UE cannot maintain phase continuity). Figure 2 (a) shows an example of Option 1. PUSCH repetitions are transmitted from slot #1 to slot #4. The boundary between slot #3 and slot #4 is considered as an event. Then, there are two actual TDWs for the PUSCH repetitions.

For Option 2, the boundary between SBFD and non-SBFD symbols is not always considered as an event. Option 2 can improve channel estimation performance if some conditions are met (e.g., the same frequency domain resources are used across SBFD and non-SBFD symbols, the same UE filter is applied between SBFD and non-SBFD symbols, etc.). Figure 2 (b) shows an example of Option 2. In contrast to Option 1, the boundary between slot #3 and slot #4 is not considered as an event. Then, there is one actual TDW for the PUSCH repetitions. Channel estimation performance can be improved compared to Option 1. The details of when DMRS bundling can be applied across SBFD and non-SBFD symbols would need to be further discussed (e.g., whether to apply separate handling between Configuration 1 and Configuration 2).

Proposal 8: Discuss whether/how to support DMRS bundling for SBFD symbols.

Figure 2 Example of actual TDW

Collision handling

In RAN1#119, the following FL proposal was discussed on collision handling in SBFD symbols [4].

Proposal 3-2 Proposed Agreement: Collision handling in SBFD symbols follows the following steps: - Step 1: Resolve collision between transmissions/receptions and unusable time/frequency resources - E.g. transmissions/receptions in invalid symbol type for Configuration 1 etc. - Step 2: Resolve collision between transmissions with same transmission direction - Step 3: Resolve collision between transmissions and receptions

One discussion point was whether SBFD specific handling (Step 1) is performed before the legacy handling or after the legacy handling (Step 2 and Step 3 are the legacy handling). Step 1 would be kind of the determination whether symbol type is valid for transmission/reception. In the current specification, such determination is specified in 38.213 section 11.1 [2] (e.g., UE does not receive DL channel/signals in UL symbols, and UE does not transmit UL channel/signals in DL symbols).

38.213, section 11.1 For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive DL PRS in the set of symbols of the slot, if the UE is not provided with a measurement gap. For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.

In the current specification, the above handling (i.e., whether symbol type is valid) is performed before Step 2 and Step 3. Therefore, it would be natural that SBFD specific handling is performed before the legacy handling.

Proposal 9: For collision handling in SBFD symbol, SBFD specific handling is performed before the legacy handling.

Conclusion

In this contribution, we made the following observations and proposals.

Proposal 1: For link direction for SBFD-aware UE in SBFD symbol, support Option 1.

Proposal 2: Not support explicit indication of guard periods.

Proposal 3: TDD-UL-DL-ConfigDedicated is not applicable to SBFD symbols. An SBFD aware UE interprets TDD-UL-DL-ConfigDedicated as legacy for non-SBFD symbols.

Proposal 4: Send LS to RAN4 regarding the necessity of UE-specific guard band.

Proposal 5: For UE behaviour when no separate configuration is provided for SBFD symbols, PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected.

Proposal 6: For CSI report associated with periodic/semi-persistent CSI-RS, support Option A.

Proposal 7: For SPS PDSCH for Configuration 2, TBS determination is based on assigned PRBs.

Proposal 8: Discuss whether/how to support DMRS bundling for SBFD symbols.

Proposal 9: For collision handling in SBFD symbol, SBFD specific handling is performed before the legacy handling.

- Reference

- RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)", Huawei

- 3GPP TS 38.213 V18.5.0 (2024-12)

- R4-2413514, "Way Forward for [112][307] NR_duplex_evo_General", Samsung

- R1-2410694, "Summary #3 of SBFD TX/RX/measurement procedures", Moderator (CATT)