3GPP TSG RAN WG1 #120		R1-2500705

Athens, Greece, February 17th – 21st, 2025

Title:	Draft Reply LS on UL 8Tx

Response to:	R2-2411220

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	Lenovo

To:	RAN2

Cc:

Contact Person:

#### Name:

E-mail Address:	< >

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Discussion

1. To support UL 8Tx in the MAC specification, RAN2 made the following assumption regarding how to model UL 8Tx operation from MAC point of view to minimize specification impacts.

For UL 8Tx 2TB transmission scheduled by one DCI, MAC entity considers two uplink grants for two TBs in two HARQ processes with one HARQ process ID and two HARQ buffers.

2. For UL 8Tx 2TB transmission scheduled by one DCI, if MAC entity only generates a single TB due to the uplink skipping, would there be any issue for PHY layer?

Answer:

RAN1 confirms that the modelling assumption in the first item is correct.

Regarding the second item, RAN1 spec TS38.214 specifies the behavior that one of the two transport blocks can be disabled, which can support the mentioned behavior.

2. Actions:

RAN1 kindly requests RAN2 to take the above information into account.

3. Dates of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #120bis	07 - 11 Apr 2025	China

TSG RAN WG1 Meeting #121	19 - 23 May 2025	Malta